Department of Jobs, Precincts and Regions

kangaroo flat dust monitoring report q4 2021

28 MARCH 2022

CERTIFICATE OF APPROVAL FOR ISSUE OF DOCUMENTS

Client:	Department of Jobs, Precincts and Regions

Project Title:	Kangaroo Flat Dust Monitoring Report Q4 2021

Title:		Kangaroo Flat Dust Monitoring Report Q4 2021

Project: 	MV214940

Document Status:	Final

Date of Issue:		 28/03/2022

|  | Position | Name | Initials | Date |
| --- | --- | --- | --- | --- |
| Prepared by: | Northern Victorian Regional Manager | Rohan Oliver | RO | 09/03/22 |
| Technical Review by: | Victorian Data Manager | Karen Bootland | KB | 09/03/22 |
| Client: | Department of Jobs, Precincts and Regions | Earth Resources Regulation | ERR | 28/03/22 |

For further information on this report, contact:

Name:		Rohan Oliver
Title:		Northern Victorian Regional Manager
Address:	94 Koondrook Rd, Kerang VIC 3579
Mobile:		+61 4 3664 4539
E-mail:		Rohan.Oliver@alsglobal.com

© ALS Water and Hydrographics Pty Ltd

This report is prepared for the Client, Department of Jobs, Precincts and Regions, for the purpose of monitoring dust at the Kangaroo Flat Mine site which is currently in care and maintenance. An unrestricted copy of this report will be shared with the public via the Earth Resources Regulation website.

Disclaimer

This document has been prepared for the Client above and is to be used only for the purposes for which it was commissioned. While all due skill and attention has been taken in collecting, validating and providing the data contained in this report, ALS shall not be liable in any way for loss of any kind including damages, costs, interest, loss of profits or special loss or damage, arising from any error, inaccuracy, incompleteness or other defect in this information. No warranty is given as to its suitability for any other purpose.

ALS Water and Hydrographics Pty Ltd ABN 94 105 060 320

ALS Australia Pty Ltd trading as ALS Hydrographics

Table of Contents

Executive Summary	4

1	Introduction	6

2	Background	6

2.1	Scope and Objective	6

2.2	Regulatory Environment	7

2.3	What is Dust	7

3	Measurement of Dust	8

3.1	Site Locations	8

3.2	Deposited Matter	10

3.3	Metals/Metalloids Monitoring in Dust	11

3.4	Ash Content	11

3.5	Dust Monitoring Standards	12

3.6	Dust Monitoring Program	12

4	Dust Deposition Results	13

4.1	Kangaroo Flat Mine – Q4 2021	13

4.1.1 Deposition of Total Insoluble Matter	13

4.1.2 Mineral Content in Dust	14

4.1.3 Metals/Metalloids in Dust	17

4.2	Quality Assurance and Quality Control	19

5	Summary of Results	20

6	Further Considerations	20

7	References	21

8	Tables	22

9	Supplementary Data	25

9.1	Arsenic	25

9.2	Barium	27

9.3	Manganese	29

Appendix A – Supplementary Information	31

Appendix B – Laboratory Reports	34

Table of Figures

Figure 1 - Kangaroo Flat - Monitoring Locations	8

Figure 2 - Kangaroo Flat - Control Site Location KF5 (BG)	9

Figure 3 - Kangaroo Flat - Control Site Gauge KF5 (BG)	9

Figure 4 - Kangaroo Flat - Dust Deposition Gauge KF1 (W)	11

Figure 5 - Kangaroo Flat - Total Insoluble Matter (TIM) - Q4 2021	13

Figure 6 - Kangaroo Flat - Total Insoluble Matter (TIM) - 2021	14

Figure 7 - Kangaroo Flat - Total Ash Content - Q4 2021	15

Figure 8 - Kangaroo Flat - Total Ash Content - 2021	15

Figure 9 - Kangaroo Flat - Ash Content/TIM Ratio - Q4 2021	16

Figure 10 - Kangaroo Flat - Ash Content/TIM Ratio - 2021	16

Figure 11 - Kangaroo Flat - Total Arsenic/Total Solids - Q4 2021	17

Figure 12 - Kangaroo Flat - Total Arsenic/Total Solids - 2021	18

Figure 13 - Kangaroo Flat - Arsenic in Soluble Fraction - Q4 2021	25

Figure 14 - Kangaroo Flat - Arsenic in Insoluble Fraction - Q4 2021	25

Figure 15 - Kangaroo Flat - Arsenic in Soluble Fraction - 2021	26

Figure 16 - Kangaroo Flat - Arsenic in Insoluble Fraction - 2021	26

Figure 17 - Kangaroo Flat - Barium in Soluble Fraction - Q4 2021	27

Figure 18 - Kangaroo Flat - Barium in Insoluble Fraction - Q4 2021	27

Figure 19 - Kangaroo Flat - Barium in Soluble Fraction - 2021	28

Figure 20 - Kangaroo Flat - Barium in Insoluble Fraction - 2021	28

Figure 21 - Kangaroo Flat - Manganese in Soluble Fraction - Q4 2021	29

Figure 22 - Kangaroo Flat - Manganese in Insoluble Fraction - Q4 2021	29

Figure 23 - Kangaroo Flat - Manganese in Soluble Fraction - 2021	30

Figure 24 - Kangaroo Flat - Manganese in Insoluble Fraction - 2021	30

Table of Tables

Table 1 - Kangaroo Flat - Dust Deposition Data	22

# Executive Summary

ALS was engaged by the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Kangaroo Flat Mine from the start of January 2021. This report provides results and analysis of data collected from sampling in Q4 2021 for the Kangaroo Flat Mine site.

In April 2021, Earth Resources Regulation (ERR) inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its operator, Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust.

This report analyses the data gathered in Q4 2021 for the Kangaroo Flat Mine site by comparing the levels to accepted standards (where available) and previous dust monitoring results. The results of dust monitoring in the first half of 2021 were published in a report by CDM Smith. During Q3 and Q4, ALS completed the collection of samples/monitoring, analysis of lab results (Springvale ALS Laboratory - NATA accredited) and drafted the dust monitoring reports.

The Kangaroo Flat Mine site (including dust monitoring sites) is located approximately 4 km south from the centre of Bendigo. The Kangaroo Flat Mine site includes:

- fine tailings dam currently covered in dust suppression polymer

- course tailings dam which has been covered with waste rock

- two water dams which capture rainwater

- various other smaller ponds for stormwater management.

This site is currently under care and maintenance. To keep dust levels down, ERR is applying a range of dust control measures at the site, including applying water and dust suppression polymer and ensuring vegetation cover. Daily inspections during business days are carried out to make sure that dust levels are kept minimised.

To maintain continuity and comparability of data, monitoring was undertaken at the same locations, and a similar methodology and analysis process was followed in this report as done before.

The original scope of dust monitoring at Kangaroo Flat was to operate and maintain four depositional dust gauges. ERR has since asked ALS to add a further monitoring site for the purpose gathering background data. Data from this control site will be used for comparative background analysis.

The purpose of dust monitoring is to check that dust issues are being minimised and managed effectively including testing the effectiveness of dust control measures. Another objective of the dust monitoring program at Kangaroo Flat is to gather baseline data ahead of rehabilitation works. These results will inform decision making on dust control measures required during site rehabilitation, to ensure works meet air quality standards (where available).

The results for Q4 2021 were consistent to previous results recorded by ALS and published in the Report for Q3 2021. Samples taken from all monitoring locations around the Kangaroo Flat site recorded Total Insoluble Matter (TIM) figures below the 4 g/m2/month criteria. However, one of the samples taken at the background monitoring location KF5 (BG) in October 2021 recorded a higher level (i.e. 9.4 g/m2/month) than the 4 g/m2/month criteria.

The elevated reading at KF5 (BG) in October 2021 has most likely been influenced by a local source, such as agricultural (e.g. ploughing), road maintenance, construction, industrial or environmental influences. For all other months however, this background location results have been representative of background dust levels in the Bendigo area. This can be seen in Figure 5 and Figure 6.

As done previously in the CDM Smith Report, the following questions are answered for Q4 2021:

- Did the dust deposition rates recorded in Q4 2021, exceed the adopted dust deposition criteria of 4 g/m2/month, measured as Total Insoluble Matter (TIM)?

In Q4 2021, there was no instance where the dust level exceeded over the 4 g/m2/month criteria at the mine monitoring sites, although the background site KF5 (BG) was above the 4 g/m2/month criteria (i.e. 9.4 g/m2/month)  in October 2021 as explained above. As this reading is from the background location, this cannot be considered an exceedance due to the source i.e. Kangaroo Flat site.

- Are dust deposition rates recorded in Q4 2021, comparable to available historic results in this area?

Overall results for Q4 2021 were comparable to previous results (Kralcopic Report in 2018, CDM Smith Report for the first half of 2021 and ALS Report for Q3 2021) and the source data remained steady and below the 4 g/m2/month criteria.

- Are arsenic concentrations recorded in deposited dust in Q4 2021, comparable to available historic results in this area?

Arsenic, barium and manganese were generally within the historical ranges, and the results here in Q4 2021 are comparable for the four Kangaroo Flat monitoring sites. Some results were lower than results reported in the previous quarters of 2021.

The only exception to this is that a high value of arsenic concentration which was recorded for KF2 (E) in November 2021 (i.e. 133.3 mg/kg). See Figure 11. This reading however has been biased by a low value for total solids (0.6 mg/kg) effecting the calculation. The low total solids value has led to a high arsenic concentration being calculated. This low total sample size can be visually observed at a glance in the November 2021 figures in Figure 5.

Continued monitoring at the newly introduced background monitoring location KF5 (BG) will provide additional data to which the source data can be compared to in future reports. Furthermore, ERR has confirmed that the focus here is to keep the dust levels minimised, which is why the fines tailings dam has been fully covered with dust suppression polymer.

# Introduction

ALS was engaged by the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Kangaroo Flat Mine site from the start of January 2021. This report provides results and analysis of data collected from sampling in Q4 2021. ALS undertook the field component of maintaining dust gauges and replacing sample containers, as well as laboratory analysis of samples collected. Samples were collected on a routine monthly basis from locations specified by DJPR. These locations are displayed in Section 3.1, Figure 1 and Figure 2 of this report.

# Background

In April 2021, ERR inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its operator, Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust. This report analyses the data gathered in Q3 2021 for the Kangaroo Flat site by comparing the levels to accepted standards (where available) and previous dust monitoring results published in:

- ALS report for Q3 2021;

- CDM Smith report for the first half of 2021; and

- Kralcopic report for Q4 2018.

The reports listed above are referenced in this document. To maintain continuity and comparability of results, this document has been prepared in a similar format to the CDM Smith report.

The Kangaroo Flat Mine is located to the east of Kangaroo Flat, approximately 4 km south from the centre of Bendigo. The site includes decommissioned mine tunnels and shafts as well as processing plants and tailings dams. The site is currently in care and maintenance. To keep dust levels down, ERR is applying a range of dust control measures at the Bendigo Mine sites, including applying water and dust suppression polymer and ensuring vegetation cover. Daily inspections during business days are carried out to make sure that dust levels are kept minimised.

To maintain continuity and comparability of data, monitoring was undertaken at the same locations as done previously, and similar methodology and analysis used. This report builds on the previous results and provides baseline information to potentially inform decisions around some aspects of rehabilitation.

## Scope and Objective

The scope of dust monitoring at Kangaroo Flat is to operate and maintain five depositional dust gauges.

All depositional dust gauges except for the background gauge KF5 (BG) are located at their historic locations with the following naming conventions since the dates specified below:

| - KF1 (W) Jan 2021 - KF2 (E) Jan 2021 - KF5 (BG) Sep 2021 | - KF3 (N) Jan 2021 - KF4 (S) Jan 2021 |
| --- | --- |

Laboratory analysis requirements were provided by DJPR and were in line with historical requirements during the previous monitoring program to maintain continuity and comparability of results.

The purpose of dust monitoring is to check that dust issues are being minimised and managed effectively including testing the effectiveness of dust control measures. Another objective of the dust monitoring program at Kangaroo Flat is to gather baseline data ahead of rehabilitation works. These results will inform decision making on dust control measures required during site rehabilitation to ensure works meet air quality standards (where available).

As done previously, the following considerations were included in assessment of Q4 2021 data:

- Did the dust deposition rates recorded in Q4 2021, exceed the adopted dust deposition criteria of 4 g/m2/month, measured as Total Insoluble Matter (TIM)?

- Are dust deposition rates recorded in Q4 2021, comparable to available historic results in this area?

- Are arsenic concentrations recorded in deposited dust in Q4 2021, comparable to available historic results in this area?

## Regulatory Environment

The Environment Protection Authority (EPA) has overarching responsibility for the air quality in Victoria. The EPA administers the Environment Protection Act 2017 and the Environment Protection Regulations 2021 under which Environmental Reference Standards (ERS) and Guidelines are provided.

The purpose of the ERS is to support the protection of human health and the environment from pollution and waste by providing benchmarks to assess and report on environmental conditions in the whole or any part of Victoria.

The ERS seeks to achieve this purpose by:

- identifying environmental values to be achieved or maintained in the whole or any part of Victoria; and

- specifying indicators and objectives to be used to measure, determine or assess whether those environmental values are being achieved, maintained or threatened.

The ERS provides guidance, objectives and levels on values where the environment may be impacted and incorporates the Ambient Air Quality National Environment Protection Measure (NEPM AAQ).

The main guidance document used for the purpose of this report is EPA Publication 1191 – Protocol for Environmental Management – Mining and Extractive Industries (Mining PEM). The criteria in this guidance used for this report has been carried over to the EPA’s latest Guideline for Assessing and Minimising Air Pollution in Victoria, Publication 1961, February 2022.

Under the new EPA legislation, duty holders have responsibility under the general environmental duty to apply controls to eliminate or minimise risks to human health and the environment. This means that the primary focus is on applying effective elimination, minimisation and control measures. Monitoring is for the purpose of determining the effectiveness of control measures.

## 2.3	What is Dust

Dust is not typically classified according to its composition, but instead on its particle size, as follows:

- Deposited matter - any particles that fall out of suspension in the atmosphere.

- Total Suspended Particles (TSP) - particles suspended or entrained in the air. Typically, this is particles of 30 µm (0.03 mm) equivalent aerodynamic diameter or less. Larger particles tend not to become suspended.

- PM10 - particles 10 µm equivalent aerodynamic diameter or less.

- PM2.5 - particles 2.5 µm equivalent aerodynamic diameter or less.

Dust particle size is an important consideration influencing dispersion and transport in the atmosphere and potential effects on human health. Human activities (e.g. energy use, transport, industrial activities etc) can affect the air quality, in particular airborne dust.

Potential sources of dust are:

- natural sources such as dust storms, agricultural dust, bushfires, vegetation, pollen and fungi; and

- anthropogenic sources such as mines sites, industry, roads and vehicles, construction sites, domestic and diffuse sources (CDM Smith, 2021).

# Measurement of Dust

## 3.1	Site Locations

Locations of depositional dust gauges around the Kangaroo Flat Mine are presented in Figure 1.

Figure 1 - Kangaroo Flat - Monitoring Locations

Location of background gauge and image are presented in Figure 2 and Figure 3.

Figure 2 - Kangaroo Flat - Control Site Location KF5 (BG)

Figure 3 - Kangaroo Flat - Control Site Gauge KF5 (BG)

Samples were collected in accordance with the Standard AS/NZS 3580.10.1 - Methods for sampling and analysis of ambient air – Determination of particulate matter – Deposited matter – Gravimetric method and ALS Dust Sampling and Depositional Dust Gauge Work Procedure MAT-MAP1000 (See Appendix A – Supplementary Information).

ALS Laboratory in Springvale analysed the samples for the following:

- Total deposited matter

- Total ash content

- Metals/metalloids, in the soluble, insoluble and ash content fraction of dust.

## 3.2	Deposited Matter

The Standard (AS/NZS 3580.10.1:2016) sets out a method (used in this report) for sampling particulate matter that is deposited from the atmosphere. This Standard also sets out procedures for the gravimetric (i.e. by weighing) determination of the mass deposition rate of insoluble/soluble solids, ash/combustible matter, and total solids from ambient air.

The method provides an estimate of the mean surface concentration of deposited matter settling from the air over a sampling period, typically of one month. Particulate matter deposition rates of 0.1 g/m2/month and above may be determined using a monthly sampling period. The sample obtained by the sampling procedure specified may be subjected to physical or chemical analysis.

Over a given sampling period, particles that settle from the ambient air are collected in a vessel and retained together with any rainwater. The sample is passed through a sieve to remove any extraneous matter (e.g. leaves, insects), and the sieved sample containing the deposited matter is transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of the dried insoluble solids is gravimetrically determined.

The ash and combustible matter content are determined by loss on ignition of the insoluble solids. Soluble solids are determined from the filtrate. The total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass deposition rate of deposited matter is then calculated from the mass of solids obtained, the funnel cross-sectional area and the exposure period. (AS/NZS 3580.10.1:2016)

Figure 4 - Kangaroo Flat - Dust Deposition Gauge KF1 (W)

## 3.3	Metals/Metalloids Monitoring in Dust

The inclusion of the analysis of metals/metalloids in the dust samples assist in identifying possible sources of contamination if present.

Analysis for insoluble matter, soluble matter and ash content was undertaken for the following:

- Arsenic

- Barium

- Manganese

It is understood that there are no specific threshold values for metals and metalloids in deposited dust either in the former Mining Licence or within the ERS. (CDM Smith, 2021)

## 3.4	Ash Content

Ash Content is the remaining material after the sample has been combusted in the laboratory. Ash content provides an indication of the mineral content (or soil dust) of the sample. The mineral content may be attributable to onsite contributions, but may also be attributable to other sources such as agriculture, unsealed roads, etc.

## 3.5	Dust Monitoring Standards

Results were compared against relevant dust deposition rate criteria described by EPA Publication 1191 (2007), as shown below:

The following points apply to the criteria:

- Results of monitoring should not exceed 4 g/m2/month (no more than 2 g/m2/month above background) as a monthly average.

- The 2 g/m2/month criteria are used when baseline data on deposited dust levels exist, while the 4 g/m2/month criteria is used when no baseline data exists.

- The criteria refer to all sources of deposited matter (including sources from mines, agriculture, unsealed roads, etc) and cumulative impacts.

- The criteria states that in some cases, a mine may increase deposited dust levels by up to 2 g/m2/month. However, the total deposited dust level (including sources from mines, agriculture, unsealed roads, etc) must not exceed 4 g/m2/month.

Deposited matter (dust) can be used as an indicator of the effectiveness of site management practices and the potential for offsite nuisance (fugitive dust). Deposited dust monitoring, when conducted over a set time period, is useful for examining trends and evaluating deviation from long term trends with respect to site activities.

## 3.6	Dust Monitoring Program

The current method of undertaking depositional dust monitoring is considered sufficient as the site is in care and maintenance i.e. no new pollution is being introduced to the site from industry. There is a considerable body of knowledge about the contamination level at the site, so all that is required at this stage is to determine whether this contamination is becoming air borne and becoming a problem to the nearby landholders. Depositional dust monitoring provides a range of useful information to be able to understand what’s happening at this site and its surrounds.

Depositional dust monitoring can provide the following invaluable information:

- test the effectiveness of dust minimisation, control and management measures

- identify key problematic sources, or groups of sources on larger more complex sites

- identify where dust sensitivities may occur

- characterise temporal or spatial trends

- metallic/metalloid concentrations useful for considering impacts on nearby community.

# Dust Deposition Results

## 4.1	Kangaroo Flat Mine – Q4 2021

Dust deposition analysis results are presented in Table 1 and laboratory results reports are attached in Appendix B, all at the end of this document.

Where results were reported as “less than” (<) a specified number (laboratory reporting limit) the number reported was adopted for use in the graphical displays. This approach adopts a conservative value for the number being reported.

## 4.1.1 Deposition of Total Insoluble Matter

Total Insoluble Matter (TIM) met the criteria of less than 4 g/m2/month on 14 of 15 measurements during Q4 2021. See Figure 5. The single exceedance occurred at the background site KF5 (BG) in October 2021 with a level of 9.4 g/m2/month being recorded. This result was not recorded at any of the sites in the vicinity of the Kangaroo Flat Mine, so is not considered to be an exceedance of the criteria.

Throughout Q4 results, none of the readings exceeded the background plus 2 g/m2/month criteria. These readings verify the monitored sites are within the requirements for Mining PEM criteria.

Aside from the high value recorded at KF5 (BG) in October 2021, dust deposition rate at site KF3 (N) was the highest value for the period during October 2021 recording a result of 2.4 g/m2/month. When the data from all source monitoring sites were averaged for Q4 2021, an average of 1.1 g/m2/month was recorded. This value is below the inferred background level suggested within the Mining PEM. Both exceedance criteria (4 g/m2/month and background plus 2 g/m2/month) have been considered when reviewing data sets. ALS found no exceedances were recorded or observed during Q4, 2021 at the source.

Figure 5 - Kangaroo Flat - Total Insoluble Matter (TIM) - Q4 2021

Red dotted line is the criteria used for the purpose of this report.

Figure 6 - Kangaroo Flat - Total Insoluble Matter (TIM) - 2021

Figure 6 shows TIM from throughout 2021, comparison of Q4 data with the first three quarters (excluding the exceedance at KF5 (BG) show that no exceedances of the recommended criteria level occurred during this period with values being recorded generally similar for Q4 2021 as with other quarters in 2021. Overall, average value of TIM for Q4 at the source is aligned with the rest of the year’s data with the average being at 1.1 g/m2/month.

## 4.1.2 Mineral Content in Dust

The fraction of dust measured as ash content is displayed below in Figure 7. The values recorded during October and November were 0.2 g/m2/month or less. During September values recorded were elevated with KF5 (BG) recording a value of 1.1 g/m2/month. This reading and other elevated readings during September correspond with higher values in TIM (in Figure 5). This suggests increases in TIM are influenced by elevated levels of mineral dust.

The elevated reading at KF5 (BG) in October 2021 has most likely been influenced by a local source, such as agricultural (e.g. ploughing), road maintenance, construction, industrial or environmental influences.

The rest of the data at KF5 (BG) location are representative of background figures in the area, so there is no need to change the location of the control site, because at any control site, you can expect to get some high figures from time to time.

Figure 7 - Kangaroo Flat - Total Ash Content - Q4 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 8 - Kangaroo Flat - Total Ash Content - 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 8 provides graphical summary of total ash content throughout 2021. As can be seen from this graph, most of the data was below 0.3 g/m2/month with only the KF3 (N) and KF5 (BG) recording higher figures than 0.6 g/m2/month, but all total ash content levels recorded were within acceptable levels with the exception of the background level recorded at KF5 (BG) for October 2021 which showed higher levels.

To gain a general understanding of what proportion of the sample is ash content i.e. mineral dust, the ratio of total ash content and TIM has been plotted in Figure 9. This graph shows that ash content in the samples throughout Q4 2021 were generally low, with higher readings at KF2 (E) for November 2021 and KF5 (BG) in December 2021. This shows that quite a proportion of the samples consisted of leafy biological content as opposed to mineral dust content. This is typical for areas where there are reasonable levels of vegetation in its surrounds. Figure 10 shows that this explanation holds true for a greater part of the samples with a few exceptions recorded in June, July and September 2021 where the mineral dust content has dominated the sample.

Figure 9 - Kangaroo Flat - Ash Content/TIM Ratio - Q4 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 10 - Kangaroo Flat - Ash Content/TIM Ratio - 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## 4.1.3 Metals/Metalloids in Dust

Graphical presentations of soluble and insoluble metals (arsenic, barium and manganese) are provided in Section 9 of this report. Majority of arsenic, barium and manganese concentrations for Q4 2021 were consistent with previously recorded historical data sets. Historical data sets from Q1, Q2, and Q3 have been reported and results were within historical ranges.

In reference to the graphs presented in Section 9, metal content for KF5 (BG) during October 2021 were elevated as were other analytes previously discussed, but still within historical trends. This was because the sample bottle at this location gathered an unusually large sample of particulate dust, and so the total metal content of that dust sample would also be higher than the normal background levels typically recorded at this location.

To provide an understanding of total arsenic concentrations measured in dust relative to concentrations measured in soils within the region, total arsenic has been displayed below Figure 11 as mg/kg. This is calculated by adding soluble and insoluble components of arsenic in µg/m2/month and dividing by the total solids g/m2/month.

This ratio is being plotted because this provides a figure which could be compared to the figures quoted in the guideline, Contaminated Sites Management Series – Assessment Levels for Soil, Sediment and Water. The assessment level for soil for arsenic is 100 mg/kg for standard residential areas in this guideline.

Figure 11 - Kangaroo Flat - Total Arsenic/Total Solids - Q4 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Total arsenic concentrations for Q4 ranged from 0.7 to 133.3 mg/kg with a mean of 22.6 mg/kg across all depositional monitoring sites. This is well below the 100 mg/kg assessment level mentioned above and comparable to Q3 mean total arsenic concentration which was 20.9 mg/kg.

A high value was recorded for KF2 (E) during November 2021 (i.e. 133.3 mg/kg), this reading has been biased by a low value for total solids (0.6 mg/kg) effecting the calculation. The low total solids value has led to a high arsenic concentration being calculated. This low total sample size can be visually observed at a glance in the November 2021 figures in Figure 5.

Overall, concentrations recorded for Q4 remained consistent with previous historical readings during other quarters in 2021. See Figure 12 below.

When comparing arsenic concentrations to those recorded at the background site KF5 (BG), it can be seen that levels recorded are similar or slightly elevated, but not significantly higher (except for samples taken at KF2 (E) for October and November 2021). The predominant wind directions in October, November and December are southerly and westerly so in these months, it could be possible that the dust sample gathered at KF2 (E) consisted of dust coming from the Kangaroo Flat site. However, although October and November showed higher arsenic concentrations at this location, the results for December did not show the same as would be expected if the dust gathered at this gauge is primarily from the Kangaroo Flat site. So further monitoring would be required to confirm that the Kangaroo Flat site is in fact the contributor to these higher arsenic concentration levels recorded.

There is no definitive pattern across the monitoring sites to indicate that the arsenic concentrations are from a single source. Highest concentrations through the period were observed at KF2 (E) (mean 69.8 mg/kg) and KF3 (N) (mean 21.1 mg/kg). These figures are still below the 100 mg/kg assessment level in the contaminated sites guideline. KF2 (E) is located slightly closer to the mine although land usage varies at each monitoring site.

Figure 12 - Kangaroo Flat - Total Arsenic/Total Solids - 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

By comparing the graphs in Figure 11 and Figure 12 above, it can be concluded that these results are trending in line with previous results. Results recorded for Q4 2021 were overall lower than the values recorded in the first three quarters of 2021.

## 4.2	Quality Assurance and Quality Control

Field sampling and analysis was undertaken in accordance with the following Standards:

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

Sampling methodology and locations are consistent with sampling previously undertaken to maintain continuity and comparability of results.

ALS Environmental Laboratory has NATA accreditation for the following Ambient Air Parameters:

The NATA accreditation process involves confirming that the relevant Australian Standards are being followed in the company procedures. ALS has followed the relevant Australian Standards in sample gathering, analysis and reporting of results which is satisfactory. Nevertheless ALS is reviewing any gaps in their accreditation and will be rectifying these gaps in due course.

Internal laboratory quality control measures note the following:

- No Method Blank value outliers occurred.

- No laboratory control outliers occurred.

- No Matrix Spike (MS) or Matrix Spike Duplicate (MSD) results reported.

- No laboratory duplicate results reported.

- No holding time outliers occurred.

- No quality control sample frequency outliers occurred (based on NEPM 2013 B3 quality control specification).

# Summary of Results

Results are discussed in detail throughout the relevant sections in this report. In summary, the overall results for Q4 2021 were comparable to previous results or less than what was observed in the first three quarters of 2021.

The key findings for Q4 2021 are as follows:

- There were no exceedances of the 4 g/m2/month (or the background plus 2 g/m2/month) criteria at depositional gauges monitoring the Kangaroo Flat mine in Q4 2021.

- However, in October 2021 at the background monitoring location KF5 (BG), a depositional dust level of 9.4 g/m2/month was recorded, which cannot be considered an exceedance, because this reading was not taken from one of the source locations at Kangaroo Flat mine.

- Consistent with the result above, high ash content was also observed at the background monitoring location KF5 (BG) in October 2021, which has most likely been influenced by local sources, such as agricultural (e.g. ploughing), road maintenance, construction, industrial or environmental influences.

- Arsenic, barium and manganese were generally within the historical ranges to the first three quarters of 2021, with results being comparable and at times lower than the first three quarters of 2021.

- An elevated arsenic concentration was recorded at KF2 (E) in November 2021 (i.e. 133.3 mg/kg). This reading however has been biased by a low value for total solids (0.6 mg/kg) affecting the calculation. The low total solids value has led to a high arsenic concentration being calculated. However, the overall  total arsenic concentrations mean values for Q4 were well below the 100 mg/kg assessment level.

# 6	Further Considerations

ERR has advised that they will continue to monitor the site for dust levels and ensure that a range of dust control measures are applied during this care and maintenance phase of the rehabilitation project to minimise dust nuisance. ERR will be endeavouring to make continuous improvements to dust suppression and monitoring from the findings of the dust monitoring work.

# 7	References

- ALS Hydrographics Pty Ltd (2021), Kangaroo Flat Dust Monitoring Q3 2021 Report. (ALS Report)

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

- CDM Smith (2021), Kangaroo Flat Mine - Dust Monitoring Report - January to June 2021. (CDM Smith Report)

- Department of Conservation (2010), Contaminated Sites Management Series, Assessment Levels for Soil, Sediment and Water, February 2010. (Contaminated Sites Assessment Levels)

- EPA (Vic) Publication 1191 (2007), Protocol for Environmental Management - Mining and Extractive Industries. (Mining PEM)

- EPA Publication 1961 (2022), Guideline for Assessing and Minimising Air Pollution in Victoria, February 2021. (Assessing and Minimising Guideline)

- EPA Publication 440.1 (2002), A Guide to the Sampling and Analysis of Air Emissions and Air Quality, December 2002. (Sampling and Analysis Guideline)

- Kralcopic Pty Ltd (2018), Quarterly Environmental Monitoring Report – Kangaroo Flat Dust Monitoring, October to December 2018. (Kralcopic Report)

- National Environment Protection Council (1998), National Environment Protection (Ambient Air Quality) Measure, July 2018. (NEPM AAQ)

- Victorian Government (2001), Statement of Environment Protection Policy (Air Quality Management), Victorian Government Gazette, No S 240, 21 December 2001. (SEPP AQM)

- Victorian Government (2016), State Environment Protection Policy (Ambient Air Quality), Original Policy including Two Variations, Victorian Gazette Nos S19, S240 and G30, published on 9 February 1999, 21 December 2001 and 28 July 2016, respectively. (SEPP AAQ)

- Victorian Government (2021), Environment Reference Standard, under the Environment Protection Act 2017, Victorian Government Gazette, No. S 245, 26 May 2021. (ERS)

# 8	Tables

Table 1 - Kangaroo Flat - Dust Deposition Data

# 9	Supplementary Data

Graphs for Metals/Metalloids in Insoluble and Soluble Fractions.

## 9.1	Arsenic

Figure 13 - Kangaroo Flat - Arsenic in Soluble Fraction - Q4 2021

Figure 14 - Kangaroo Flat - Arsenic in Insoluble Fraction - Q4 2021

Figure 15 - Kangaroo Flat - Arsenic in Soluble Fraction - 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 16 - Kangaroo Flat - Arsenic in Insoluble Fraction - 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## 9.2	Barium

Figure 17 - Kangaroo Flat - Barium in Soluble Fraction - Q4 2021

Figure 18 - Kangaroo Flat - Barium in Insoluble Fraction - Q4 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 19 - Kangaroo Flat - Barium in Soluble Fraction - 2021

Figure 20 - Kangaroo Flat - Barium in Insoluble Fraction - 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## 9.3	Manganese

Figure 21 - Kangaroo Flat - Manganese in Soluble Fraction - Q4 2021

Figure 22 - Kangaroo Flat - Manganese in Insoluble Fraction - Q4 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 23 - Kangaroo Flat - Manganese in Soluble Fraction - 2021

Figure 24 - Kangaroo Flat - Manganese in Insoluble Fraction - 2021

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

# Appendix A – Supplementary Information

# Appendix B – Laboratory Reports