Department of Energy, Environment, and Climate Action

Woodvale dust monitoring report q3 2022

20 April 2023

CERTIFICATE OF APPROVAL FOR ISSUE OF DOCUMENTS

Client:	Department of Energy, Environment and Climate Action

Project Title:	Woodvale Dust Monitoring Report Q3 2022

Title:		Woodvale Dust Monitoring Report Q3 2022

Project: 	MV214940

Document Status:	Version 1 Final

Date of Issue:		 25/1/2023

|  | Position | Name | Initials | Date |
| --- | --- | --- | --- | --- |
| Prepared by: | Northern Victorian Regional Manager | Rohan Oliver | RO | 12/12/22 |
| Technical Review by: | Victorian Data Manager | Karen Bootland | KB (Reviewed V1) | 14/04/23 |
| Client: | Department of Energy, Environment and Climate Action (DEECA) | Michael Fitzgerald | DEECA | 20/04/25 |

For further information on this report, contact:

Name:		Rohan Oliver
Title:		Northern Victorian Regional Manager
Address:	94 Koondrook Rd, Kerang VIC 3579
E-mail:		Rohan.Oliver@alsglobal.com

© ALS Water and Hydrographics Pty Ltd

This report is prepared for the client, Department of Energy, Environment and Climate Action, for the purpose of monitoring dust at the former Bendigo Mine Woodvale site, which is currently in care and maintenance. An unrestricted copy of this report will be shared with the public via the Earth Resources Regulation website.

Disclaimer

This document has been prepared for the client above and is to be used only for the purposes for which it was commissioned. While all due skill and attention has been taken in collecting, validating and providing the data contained in this report, ALS shall not be liable in any way for loss of any kind, including damages, costs, interest, loss of profits or special loss or damage, arising from any error, inaccuracy, incompleteness or other defect in this information. No warranty is given as to its suitability for any other purpose.

ALS Water and Hydrographics Pty Ltd ABN 94 105 060 320

ALS Australia Pty Ltd trading as ALS Hydrographics

Table of Contents

Executive Summary	4

1.	Introduction	7

2.	Background	7

2.1	Scope and Objective	8

2.2	Regulatory Environment	9

2.3	What is Dust	9

3.	Measurement of Dust	10

3.1	Site Locations	10

3.2	Deposited Matter	11

3.3	Directional Deposited Dust Monitoring	13

3.4	Metals/Metalloids Monitoring in Dust	14

3.5	Ash Content	14

3.6	Dust Monitoring Standards	14

3.7	Dust Monitoring Program	15

4.	Dust Deposition Results	16

4.1	Woodvale Evaporation Ponds – Q3 2022	16

4.1.1 Deposition of Total Insoluble Matter	16

4.1.2 Mineral Content in Dust	18

4.1.3 Metals/Metalloids in Dust	19

4.1.4 Woodvale Directional Dust Deposition	21

4.2	Review of Quality Assurance and Quality Control	24

5.	Summary of Results	25

6.	Further Considerations	25

7.	References	26

8.	Tables	27

9.	Supplementary Data	45

9.1	Arsenic	45

9.2	Barium	47

9.3	Manganese	49

9.4	Trend Analysis Graphs	51

10.	Appendix A – Supplementary Information	59

11.	Appendix B – Laboratory Reports	61

Table of Figures

Figure 1 - Monitoring Locations at Woodvale Ponds	10

Figure 2 - Control Site Location WVDD07 (BG)	11

Figure 3 - Dust Deposition Gauge WVDD01	12

Figure 4 - Directional Dust Deposition Gauge WVDG06	13

Figure 5 - Woodvale - Total Insoluble Matter (TIM) - Q3 2022	17

Figure 6 - Woodvale - Total Ash Content – Q3 2022	18

Figure 7 - Woodvale - Ash Content/TIM Ratio – Q3 2022	18

Figure 8 - Woodvale - Total Arsenic/Total Solids – Q3 2022	19

Figure 9 - Woodvale - Directional Dust Deposition Results - Total Solids (g/m2/month) – Q3 2022 (North/South)	21

Figure 10 - Woodvale - Directional Dust Results - Arsenic Concentrations – Q3 2022 (North/South)	22

Figure 11 - Woodvale - Directional Dust Deposition Results - Total Solids (g/m2/month)  – Q3 2022 (East/West)	22

Figure 12 - Woodvale - Directional Dust Results - Arsenic Concentrations – Q3 2022 (East/West)	23

Figure 13 - Woodvale - Arsenic in Soluble Fraction – Q3 2022	45

Figure 14 - Woodvale - Arsenic in Insoluble Fraction – Q3 2022	45

Figure 15 - Woodvale - Arsenic in Soluble Fraction Year Review (2021-22)	46

Figure 16 - Woodvale - Arsenic in Insoluble Fraction Year Review (2021-22)	46

Figure 17 - Woodvale - Barium in Soluble Fraction – Q3 2022	47

Figure 18 - Woodvale - Barium in Insoluble Fraction – Q3 2022	47

Figure 19 - Woodvale - Barium in Soluble Fraction Year Review (2021-22)	48

Figure 21 - Woodvale - Manganese in Soluble Fraction – Q3 2022	49

Figure 22 - Woodvale - Manganese in Insoluble Fraction – Q3 2022	49

Figure 23 - Woodvale - Manganese in Soluble Fraction Year Review (2021-22)	50

Figure 24 - Woodvale - Manganese in Insoluble Fraction Year Review (2021-22)	50

Figure 25 - Woodvale - Total Insoluble Matter (TIM) - 2021-2022	51

Figure 26 - Woodvale - Total Ash Content - 2021-2022	52

Figure 27 - Woodvale - Ash Content/TIM Ratio - 2021-2022	53

Figure 28 - Woodvale - Total Arsenic/Total Solids - 2021-2022	54

Figure 29 - Woodvale - DDG Total Solids Comparison North/South - 2021-2022	55

Figure 30 - Woodvale - Arsenic/Total Solids Comparison - North/South 2021-2022	56

Figure 31 - Woodvale - DDG Total Solids Comparison East/West - 2021-2022	57

Figure 32 - Woodvale - Arsenic/Total Solids Comparison - East/West - 2021-2022	58

Table of Tables

Table 1 - Woodvale Dust Deposition Analysis Results Q3, 2022	27

Table 2 - Woodvale Directional Dust Deposition Analysis Results Q3, 2022	32

# Executive Summary

ALS was engaged by the Department of Energy, Environment and Climate Action (DEECA) to undertake dust monitoring at the Woodvale Evaporation Ponds Complex from the start of January 2021. This report provides results and analysis of data collected from sampling in Q3 (July-September 2022).

Earth Resources Regulation (ERR) inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites as the owner, Kralcopic Pty Ltd, went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Gold Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust, and in particular the arsenic content of the dust.

This report analyses the data gathered in Q3 2022 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results. The results of dust monitoring in the first half of 2021 were published in a report by CDM Smith. During Q3 and Q4, ALS completed the collection of samples/monitoring, analysis of lab results (Springvale ALS Laboratory – NATA accredited) and drafted the dust monitoring reports.

The Woodvale Evaporation Ponds Complex is located 13 km north of Bendigo. The ponds were formerly used to hold groundwater extracted from dewatering historic underground mine workings under Bendigo. The site is currently in care and maintenance while ERR develops a rehabilitation program for the site. ERR is continuing to visually monitor the site by daily site visits, keeping an eye on weather conditions, and proactively applying a range of dust control measures. The evaporation ponds are the main source of dust at this site as not all ponds are kept wet. Dust suppression measures at the site include covering ponds with water, vegetation cover, mulch application and keeping a water/polymer spray truck on standby, should the need arise to use such control measures.

To maintain continuity and comparability of data, dust monitoring was undertaken at the same locations and a similar methodology and analysis process was followed in this report as per other quarters. Upon routine monthly monitoring, ALS field staff observed all sites where in good working order with no obvious variations to sites that would reflect abnormalities in results.

The original scope of dust monitoring works at Woodvale was to operate and maintain six depositional dust gauges and two directional dust deposition gauges. ERR has since asked ALS to add a further three monitoring sites to the dust monitoring program at Woodvale, i.e. one control site to obtain background data and two directional dust deposition gauges at the east and west of the Woodvale site, to provide more complete data from directional dust monitoring. These newly added sites have been in operation since 1 October 2021.

The purpose of dust monitoring is to ensure that dust issues are being minimised and managed effectively. The objective of the dust monitoring program at Woodvale is to gather baseline data and report exceedances when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will inform decision making on rehabilitation requirements and measures required to ensure works meet air quality standards (where available).

The results for Q3 2022 were slightly elevated to previous results supplied (Kralcopic Report in 2018, CDM Smith Report for the first half of 2021, and ALS Reports for Q3 2021, Q4 2021 and Q1 2022) for some results collected.

As done previously in historic reports, the following questions are answered for Q3 2022:

- Did the dust deposition rates recorded in Q3 2022 exceed the adopted dust deposition criteria of 
4 g/m2/month, measured as Total Insoluble Matter (TIM)?

In Q3 2022, there were ten instances where the dust levels from all monitoring sites were over the 
4 g/m2/month criteria used in this report (refer below).

- WVDD04 (E), during July 2022 (5.6 g/m2/month)

- WVDD05 (S), during July 2022 (6.2 g/m2/month)

- WVDD06 (W), during July 2022 (18.0 g/m2/month)

- WVDD07(BG), during July 2022 (8.3 g/m2/month)

- WVDD02 (N), during August 2022 (5.0 g/m2/month)

- WVDD05 (S), during August 2022 (5.1 g/m2/month)

- WVDD07(BG), during August 2022 (12.7 g/m2/month)

- WVDD02 (N), during September 2022 (10.9 g/m2/month)

- WVDD06 (W), during September 2022 (12.7 g/m2/month)

- WVDD07 (BG), during September 2022 (12.4 g/m2/month)

Three of these occurrences were at the background site WVDD07 (BG). Those occurrences cannot be attributed to the Woodvale Evaporation Ponds Complex. When the data from all source monitoring sites, excluding WVDD07 (BG) is averaged for Q3 2022, 4.1 g/m2/month was recorded. This value is marginally above the 4 g/m2/month criteria, and the inferred background plus 2 g/m2/month criteria in the Mining PEM.

High levels of dust were recorded at the background site WVDD07 (BG) during July, August and September 2022.

High dust levels were also recorded at WVDD06 (W) that, at the time of monitoring, was located on a neighbouring farm property in close proximity to a sheep feedlot. It is possible that the activity at the sheep feedlot contributes to the dust measured at this location. This observation is reinforced by the background site also recording a higher dust level in July and September. In addition, the ponds on the west side of the Woodvale site (where WVDD01 (S) and WVDD06 (W) are located) contained water in all ponds as a result of the significant rain events across Victoria through Spring.

The surrounding land use to the north, south and west is predominantly agricultural farmland with reserved parklands to the east of the Woodvale site. ERR conducts site inspections to regularly monitor dust control measures (water cover, dampness, foliage) are adequate.

- Are dust deposition rates recorded in Q3 2022, comparable to available historic results in this area?

Overall, results for Q3 2022 were slightly higher than previous results recorded in the Kralcopic Report for Q4 2018, CDM Smith Report for the first half of 2021, and ALS Reports for Q3 2021, Q4 2021 and Q1 2022.

- Are arsenic concentrations recorded in deposited dust in Q3 2022, comparable to available historic results in this area?

Arsenic, barium and manganese were within the historical ranges (with some results a little over the mid-range).

It is noted that in addition to the results above, the directional dust deposition gauge results for Q3 2022 showed no clear overall net gain in dust or arsenic concentrations resulting from the Woodvale Evaporation Ponds Complex site, with results fluctuating from month to month showing gains and losses.

ALS was unable to report insoluble metals for the month of August. This is because ALS laboratory staff used the entire filter (used to collect the sample) to determine the soluble metal content when half should have been used to determine soluble metal content, and the other half used to determine insoluble matter. This is required as the two processes to measure soluble and insoluble content are different.

ALS has identified any graphs where this may have been compromised and added comments below the chart results.

# Introduction

ALS was engaged by the Earth Resources Regulation team in the then Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Woodvale Evaporation Ponds Complex from the start of January 2021. On 1 January 2023, ERR joined a new Victorian Government department, the Department of Energy, Environment and Climate Action (DEECA). This report provides results and analysis of data collected from sampling in Q3 2022. ALS undertook the field component of maintaining dust gauges and replacing sample containers, as well as laboratory analysis of samples collected. Samples were collected from locations specified by DJPR on a routine monthly basis. These locations are displayed in Section 3.1 of this report.

# Background

In April 2021, ERR inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites as the owner, Kralcopic Pty Ltd, went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Gold Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust, and in particular the arsenic content of the dust. This report analyses the data gathered in Q3 2022 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results published in:

- ALS Report for Q3 2022 – New agreement with ALS

- ALS Report for Q2 2022

- ALS Report for Q1 2022

- ALS Report for Q4 2021

- ALS Report for Q3 2021

- CDM Smith Report for the first half of 2021

- Kralcopic Report for Q4 2018.

All reports are referenced in this document. To maintain continuity and comparability of results, this document has been prepared in a similar format to the CDM Smith Report.

The Woodvale Evaporation Ponds Complex is located 13 km north of Bendigo. The ponds were formerly used to hold groundwater extracted from dewatering historic underground mines, including Kangaroo Flat and New Moon mines. The site is currently in care and maintenance while ERR develops the rehabilitation plan for the site. ERR is continuing to visually monitor the site by regular site visits, keeping an eye on weather conditions, and proactively applying a range of dust control measures. Dust suppression measures include ensuring water or vegetation cover, application of mulch and keeping a water/polymer spray truck on standby, should the need arise to use such control measures.

To maintain continuity and comparability of data, monitoring was undertaken at the same locations as previously, and similar methodology and analysis used. This report builds on the previous results and provides baseline information to potentially inform decisions around some aspects of site rehabilitation.

## Scope and Objective

The scope of dust monitoring works at Woodvale is to operate and maintain seven depositional dust gauges and four directional dust deposition gauges, including gathering samples and sending off to the laboratory for analysis, analysing the data from the laboratory and producing quarterly dust monitoring reports consolidating results and comparing to historic and guideline levels.

All depositional dust gauges except for the background gauge (WVDD07 (BG)) are located at their historic locations with the following naming conventions since the dates specified below:

| - WVDD01 (S) Jan 2021 - WVDD02 (N) Jan 2021 - WVDD03 (N) Jan 2021 - WVDD04 (E) Jan 2021 | - WVDD05 (S) Jan 2021 - WVDD06 (W) Jan 2021 - WVDD07 (BG) Sep 2021 |  |
| --- | --- | --- |

The location of the six depositional dust gauges at the Woodvale site can be seen in Figure 1. The location of the background depositional dust gauge is in Figure 2 which serves as a control site.

Similarly, four directional dust deposition gauges were installed since the dates specified below at the following locations:

- WVDG03 (N) Jan 2021

- WVDG04 (E) Oct 2021

- WVDG05 (S) Jan 2021

- WVDG06 (W) Oct 2021

These directional gauges are located next to the depositional dust gauges: WVDD03 (N), WVDD04 (E), WVDD05 (S) and WVDD006 (W).

Laboratory analysis requirements were provided by DJPR and were in line with historical requirements during the previous monitoring program undertaken by Kralcopic to meet its legislative requirements. This alignment maintains continuity and comparability of results.

The purpose of dust monitoring is to check that dust issues are managed effectively, including testing the effectiveness of dust control measures. Another objective of the dust monitoring program at Woodvale is to gather baseline data ahead of rehabilitation works. These results will inform decision making on dust control measures required during site rehabilitation, to ensure works meet air quality standards (where available).

As done previously, the following considerations were included in assessment of Q3 2022 data:

- Did the dust deposition rates recorded in Q3 2022 exceed the adopted dust deposition criteria of 
4 g/m2/month, measured as Total Insoluble Matter (TIM)?

- Are dust deposition rates recorded in Q3 2022 comparable to available historic results in this area?

- Are arsenic concentrations recorded in deposited dust in Q3 2022 comparable to available historic results in this area?

## Regulatory Environment

The Environment Protection Authority (EPA) has overarching responsibility for administering legislation related to air quality across Victoria. The EPA administers the Environment Protection Act 2017 (EP Act 2017) and the Environment Protection Regulations 2021 under which Environmental Reference Standards (ERS) and related guidelines are provided.

The purpose of the ERS is to support the protection of human health and the environment from harms posed by pollution and waste. The ERS does this by providing benchmarks to assess and report on environmental conditions in the whole or any part of Victoria.

The ERS seeks to achieve this purpose by:

- identifying environmental values to be achieved or maintained in the whole or any part of Victoria; and

- specifying indicators and objectives to be used to measure, determine or assess whether those environmental values are being achieved, maintained or threatened.

The ERS provides guidance, objectives and levels on values where the environment may be impacted and incorporates the Ambient Air Quality National Environment Protection Measure (NEPM AAQ).

The main guidance document used for the purpose of this report is EPA Publication 1191: Protocol for Environmental Management – Mining and Extractive Industries (Mining PEM). The criteria in the Mining PEM used for this report has been carried over to the EPA’s latest Guideline for Assessing and Minimising Air Pollution in Victoria, Publication 1961, February 2022. As stated in the PEM, results of monitoring should not exceed 4 g/m2/month (no more than 2 g/m2/month above background) as a monthly average.

Under the EP Act 2017, duty holders have responsibility under the general environmental duty to apply controls to eliminate or minimise risks to human health and the environment as far as reasonably practicable. This means that the primary focus is on applying effective elimination, minimisation and control measures. Monitoring is for the purpose of determining the effectiveness of control measures.

## What is Dust?

Dust is typically not classified due to its composition but based on its particle size, as follows:

- Deposited matter – any particles that fall out of suspension in the atmosphere

- Total Suspended Particles (TSP) – particles suspended or entrained in the air. Typically, these are particles of 30 µm (0.03 mm) equivalent aerodynamic diameter or less. Larger particles tend not to become suspended

- PM10 – particles 10 µm equivalent aerodynamic diameter or less

- PM2.5 – particles 2.5 µm equivalent aerodynamic diameter or less.

Dust particle size is an important consideration influencing dispersion and transport in the atmosphere and potential effects on human health. Human activities (e.g. energy use, transport, industrial activities etc.) can affect the air quality, in particular airborne dust.

Potential sources of dust are:

- natural sources such as dust storms, agricultural dust, bushfires, vegetation, pollen and fungi; and

- anthropogenic sources such as mines sites, industry, roads and vehicles, construction sites, domestic and diffuse sources (CDM Smith, 2021).

# Measurement of Dust

## Site Locations

Locations of depositional dust gauges are as presented in Figure 1 and Figure 2. Directional depositional dust gauges WVDG03 (N), WVDG04 (E), WVDG05 (S) and WVDG06 (W) have been installed next to the respective depositional gauges as mentioned above.

Figure 1 - Monitoring Locations at Woodvale Ponds

Figure 2 - Control Site Location WVDD07 (BG)

Samples were collected in accordance with the Standard AS/NZS 3580.10.1: Methods for sampling and analysis of ambient air – Determination of particulate matter – Deposited matter – Gravimetric method and ALS Dust Sampling and Depositional Dust Gauge Work Procedure MAT-MAP1000 (See Appendix A).

ALS Laboratory in Springvale analysed the samples for the following:

- Total deposited matter

- Total ash content

- Metals/metalloids, in the soluble, insoluble and ash content fraction of dust.

## Deposited Matter

The Standard (AS/NZS 3580.10.1:2016) sets out a method (used in this report) for sampling particulate matter that is deposited from the atmosphere. This Standard also sets out procedures for the gravimetric (i.e. by weighing) determination of the mass deposition rate of insoluble/soluble solids, ash/combustible matter, and total solids from ambient air.

The method provides an estimate of the mean surface concentration of deposited matter settling from the air over a sampling period, typically of one month. Particulate matter deposition rates of 0.1 g/m2/month and above may be determined using a monthly sampling period. The sample obtained by the sampling procedure specified may be subjected to physical or chemical analysis.

Over a given sampling period, particles that settle from the ambient air are collected in a vessel (see sampling bottle in Figure 3 below) and retained together with any rainwater. The sample is passed through a sieve to remove any extraneous matter (e.g. leaves, insects), and the sieved sample containing the deposited matter is transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of the dried insoluble solids is gravimetrically determined.

The ash and combustible matter content are determined by loss on ignition of the insoluble solids. Soluble solids are determined from the filtrate. The total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass deposition rate of deposited matter is then calculated from the mass of solids obtained, the funnel cross-sectional area and the exposure period. (AS/NZS 3580.10:2016)

Figure 3 - Dust Deposition Gauge WVDD01

## Directional Deposited Dust Monitoring

The Standard (AS/NZS 3580.10.2:2013) states that over a given period, particles which impinge on the vertical, internal surface of four collecting cylinders of the directional dust gauge are sampled in collecting pots at the bases of the cylinders. Refer to Figure 4.

The samples are passed through a sieve to remove any extraneous matter (e.g. leaves, insects), with the sieved sample containing the impinged matter subsequently transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of dried insoluble solids determined gravimetrically. Soluble solids are determined by evaporation of the filtrate and gravimetric analysis.

The ash and combustible matter content are determined by loss on combustion of the insoluble solids. Total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass flux rate of impinged matter for each collecting cylinder is calculated from the mass of solids obtained, the slot cross-sectional area and the exposure period.

A directional dust gauge consists of four vertical collecting cylinders, each with a slot of dimensions 
45 mm by 340 mm, commencing approximately 10 mm from the top of the cylinder. The edges of each slot shall be at right angles to the width of the slot and not radial (see Figure 4). The cylinders shall be mounted at 90o to each other with the slots facing outwards. An easily detachable collecting pot shall be fitted to the bottom of each collecting cylinder. The combined length of a collecting cylinder and its attached collecting pot shall be at least 640 mm, with the collecting pot having a capacity of approximately 1 L.

The collecting cylinders and pots shall be constructed of chemically resistant material such as polyvinyl chloride. The external diameter of the collecting cylinders shall be 75 mm. The collecting cylinders shall be located with their axes at the corners of a 150 mm square mounting plate. (AS/NZS 3580.10.2:2013)

Figure 4 - Directional Dust Deposition Gauge WVDG06

## Metals/Metalloids Monitoring in Dust

The inclusion of the analysis of metals/metalloids in the dust samples assists in identifying possible sources of contamination if present.

Analysis for insoluble matter, soluble matter and ash content was undertaken for the following:

- Arsenic

- Barium

- Manganese

It is understood that there are no specific threshold values for metals and metalloids in deposited dust, either in the former Mining Licence or within the ERS. (CDM Smith, 2021)

## Ash Content

Ash content is the remaining material after the sample has been combusted in the laboratory. Ash content provides an indication of the mineral content (or soil dust) of the sample. The mineral content may be attributable to on site contributions, but may also be attributable to other sources such as agriculture, unsealed roads, etc.

## Dust Monitoring Standards

Results were compared against relevant dust deposition rate criteria described by EPA Publication 1191 (2007) as shown below, which has been adopted in EPA Publication 1961 (2022).

The following points apply to the criteria:

- Results of monitoring should not exceed 4 g/m2/month (no more than 2 g/m2/month above background) as a monthly average.

- The 2 g/m2/month criterion is used when baseline data on deposited dust levels exist, while the 
4 g/m2/month criterion is used when no baseline data exists.

- If dust levels exceed this value, then site management practices should be reviewed and dust controls implemented to reduce dust levels to within these guidelines.

- The criteria refer to all sources of deposited matter (including sources from mines, agriculture, unsealed roads, etc) and cumulative impacts.

- The criteria state that in some cases, a mine may increase deposited dust levels by up to 
2 g/m2/month. However, the total deposited dust level (including sources from mines, agriculture, unsealed roads, etc) must not exceed 4 g/m2/month.

Deposited matter (dust) can be used as an indicator of the effectiveness of site management practices and the potential for offsite nuisance (fugitive dust). Deposited dust monitoring, when conducted over a set time period, is useful for examining trends and evaluating deviation from long term trends with respect to site activities.

## Dust Monitoring Program

The current method of undertaking depositional dust monitoring is considered sufficient as the site is in care and maintenance, i.e. no new pollution is being introduced to the site from industry. There is a considerable body of knowledge about the contamination level at the site, so all that is required at this stage is to determine whether this contamination is becoming air borne and becoming a health hazard or problem to the nearby landholders. Depositional dust monitoring provides a range of useful information to be able to understand what’s happening at this site and its surrounds.

Depositional dust monitoring can provide the following invaluable information:

- test the effectiveness of dust minimisation, control and management measures

- identify key problematic sources, or groups of sources on larger more complex sites

- identify where dust sensitivities may occur

- characterise temporal or spatial trends

- metallic/metalloid concentrations are useful for health effect considerations.

The environmental conditions during Q3 2022 across Bendigo remained consistent for July with rainfall increasing during August and September. Water levels in ponds located at Woodvale were not continuously monitored by ALS, although site observations from days where staff were on site to collect dust samples observed the ponds were in a wet state. Therefore, it is unlikely any dust was emitted from these ponds during the monitoring period. A new weather station was installed by ALS in 2023, which will help (in future reporting) to investigate predominant wind direction and speed. Weather station data during Q3 has been sourced from the BOM daily weather observations from site location, Bendigo Airport (station 081123).

# Dust Deposition Results

## Woodvale Evaporation Ponds Complex – Q3 2022

Dust deposition analysis results are presented in Table 1 – Woodvale Dust Deposition Analysis Results and the accompanying Laboratory Certificates of Analysis are presented in Appendix B – Laboratory Reports.

Where results were reported as “less than” (<) a specified number (laboratory reporting limit), the number reported was adopted for use in the graphical displays. This approach adopts a conservative value for the number being reported.

## 4.1.1 Deposition of Total Insoluble Matter

Total Insoluble Matter (TIM) met the recommended criteria of less than 4 g/m2/month on 11 of 21 samples during Q3 2022. See Figure 5 below.

Dust deposition rate exceeded the assessment criteria at the sites below:

- WVDD04 (E), during July 2022 (5.6 g/m2/month)

- WVDD05 (S), during July 2022 (6.2 g/m2/month)

- WVDD06 (W), during July 2022 (18.0 g/m2/month)

- WVDD07(BG), during July 2022 (8.3 g/m2/month)

- WVDD02 (N), during August 2022 (5.0 g/m2/month)

- WVDD05 (S), during August 2022 (5.1 g/m2/month)

- WVDD07(BG), during August 2022 (12.7 g/m2/month)

- WVDD02 (N), during September 2022 (10.9 g/m2/month)

- WVDD06 (W), during September 2022 (12.7 g/m2/month)

- WVDD07 (BG), during September 2022 (12.4 g/m2/month)

When the data from all source monitoring sites except WVDD07 (BG) (the background reference location) were averaged for Q3 2022, an average of 4.1 g/m2/month was recorded. This value is marginally above the 4 g/m2/month criteria and the inferred background plus 2 g/m2/month criteria in the Mining PEM as referenced in section 2.2 of this report. The results at the WVDV07 (BG) site were also recorded as exceeding the PEM criteria during July 2022, August 2022 and September 2022. WVDD06 exceeded the PEM criteria during July 2022.

Figure 5 - Woodvale - Total Insoluble Matter (TIM) - Q3 2022

Red dotted line is the criteria used for the purpose of this report.

## 4.1.2 Mineral Content in Dust

The fraction of dust measured as ash content is displayed below in Figure 6. As expected, the increased levels in TIM (Figure 5) generally correspond with total ash content in Figure 6. Some of the peaks recorded in Figure 5 for July-August also generally appear in Figure 6.

Figure 6 - Woodvale - Total Ash Content – Q3 2022

To gain a general understanding of what proportion of the sample is ash content (i.e. mineral dust), the ratio of total ash content and TIM has been plotted in Figure 7.

Figure 7 - Woodvale - Ash Content/TIM Ratio – Q3 2022

## 4.1.3 Metals/Metalloids in Dust

Graphical presentations of soluble and insoluble metals (arsenic, barium and manganese) are provided in Section 9 of this report. Arsenic, barium and manganese concentrations for Q3 2022 were within historical ranges and results comparable to results reported in the CDM Smith Report for the first half of 2021 and in the ALS Reports for Q3 2021, Q4 2021 and Q1 2022. These can be found in Sections 9.1, 9.2 and 9.3 of this report.

Lab results for insoluble metals were unavailable for August due to the sample being ashed. EM2217582 The results calculated in Figure 8 have used results = zero for insoluble metals for August 2022.

Lab results where zero has been used for insoluble metals has been denoted on graphs with. Further details are available on page 6 of this report.

To provide an understanding of total arsenic concentrations measured in dust relative to concentrations measured in soils within the region, total arsenic concentration has been displayed below Figure 8 in mg/kg. This is calculated by adding soluble and insoluble arsenic µg/m2/month divided by the total solids g/m2/month. August results have been calculated using zero for insoluble metals.

This ratio has been plotted as this provides a value which could be compared to the figures quoted in the guideline, Contaminated Sites Management Series – Assessment Levels for Soil, Sediment and Water. The assessment level for soil for arsenic is 100 mg/kg for standard residential areas in this guideline.

Figure 8 - Woodvale - Total Arsenic/Total Solids – Q3 2022

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

The has been placed over the sample where no lab result was available. This occurred in August 2022 for Arsenic.

Total arsenic concentrations for Q3 ranged from 0 to 38.3 mg/kg with a mean of 12.1 mg/kg for July 2022, no result calculated for August, and a mean of 6.8 for September 2022 across all depositional monitoring sites (excluding the background monitoring site). The mean is well below the 100 mg/kg assessment level mentioned above.

Highest concentrations were recorded in July 2022 – 38.3 mg/kg – at WVDD05 (S) and again in August 2022 – 18.8 mg/kg – at WVDD05 (S). These values are well below the 100 mg/kg assessment level mentioned above.

## 4.1.4 Woodvale Directional Dust Deposition

Results for directional dust deposition gauges are summarised in Table 2 – a Woodvale Directional Dust Deposition Analysis Results, with Laboratory Analytical Results attached in Appendix B. As mentioned previously, for the purpose of continuity, the same approach for review of dust from the directional dust deposition gauges as in the CDM Smith Report has been applied here for Q3 2022 results.

The results from the northerly and southerly facing quadrants at the two directional dust deposition gauges WVDG03 (N) and WVDG05 (S) have been compared in Figure 9. Figure 9 shows the net gain or loss in dust deposition downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

To undertake this analysis, the northern quadrant dust gauge at WVDG03 is assumed to measure dust coming onto the site in northerly winds. This is then compared to the northern quadrant dust gauge at WVDG05. The variance is then used to indicate dust contributions to the air from the site. A positive variance indicates an increase in dust to the atmosphere from the ponds. A negative variance indicates the ponds have removed more dust from the air than they contribute. This theoretical analysis is repeated for the site from each axis for directional dust gauge locations.

From the graph below (Figure 9), it can be seen that there is a small net gain in dust observed through the Q3 2022 monitoring period.

Such small gains can be expected and can be influenced by both the Woodvale site and environmental conditions. Note that there is agricultural farmland to the north, south and west of the Woodvale site and reserved parklands to the east of the Woodvale site, which can all contribute to the dust levels at these sampling locations. Furthermore, the predominant winds during this quarter were from the south, which would have also influenced these results.

Figure 9 - Woodvale - Directional Dust Deposition Results - Total Solids (g/m2/month) – Q3 2022 (North/South)

Similarly, the concentration of arsenic was compared using the same methodology as above, and it was found that low net gains and losses occurred for five of the six values recorded during Q2 2022, with these levels being comparable to what was observed in 2021 and Q1 2022.

Figure 10 presents the net gain and loss in total arsenic, downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

Figure 10 - Woodvale - Directional Dust Results - Arsenic Concentrations – Q3 2022 (North/South)

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

August results influenced by no lab results for insoluble metals.

Figure 10 – No recorded high figures were seen in any of the two directional gauges or six depositional dust gauges. Figure 11 below displays the net gain or loss both upwind and downwind from east and west. No clear pattern is identified during Q3 2022 with both gains and losses measured throughout the period. July west facing gauge recorded zero variance.

Figure 11 - Woodvale - Directional Dust Deposition Results - Total Solids (g/m2/month)  – Q3 2022 (East/West)

Figure 12 presents the variances in total arsenic downwind of the site compared to upwind of the site, for both east and west facing quadrants.

For the purpose of providing an understanding of total arsenic concentrations measured in dust relative to arsenic concentrations measured in soils in the assessment region, total arsenic is presented as mg/kg (based on the sum of soluble and insoluble arsenic µg/m2.month divided by total solids g/ m2.month).

Figure 12 - Woodvale - Directional Dust Results - Arsenic Concentrations – Q3 2022 (East/West)

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

## Review of Quality Assurance and Quality Control

Field sampling was undertaken in accordance with the following Standards:

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air – Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air – Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

Sampling methodology and locations are consistent with sampling previously undertaken to maintain continuity and comparability of results.

ALS Environmental Laboratory has NATA accreditation for the following Ambient Air Parameters:

ALS currently does not have NATA accreditation for metals analysis of ambient air but is in the process of gaining accreditation. Nevertheless, ALS is operating as though this accreditation is in place and expects to finalise the accreditation process in 2022.

Internal laboratory quality control measures note the following:

- No Method Blank value outliers occurred

- No laboratory control outliers occurred

- No Matrix Spike (MS) or Matrix Spike Duplicate (MSD) results reported

- No laboratory duplicate results reported

- No holding time outliers occurred

- No quality control sample frequency outliers occurred (based on NEPM 2013 B3 quality control specification).

# Summary of Results

Results are discussed in detail throughout the relevant sections. In summary, the overall results for Q3 2022 were comparable to previous results.

The key findings for Q3 2022 are as follows:

- There were ten instances where the dust level exceeded the 4 g/m2/month criteria used in this report.  Three of these instances occurred at the background site (WVDD07 (BG)) and are not attributed to the Woodvale Evaporation Ponds Complex.

- WVDD04 (E), during July 2022 (5.6 g/m2/month)

- WVDD05 (S), during July 2022 (6.2 g/m2/month)

- WVDD06 (W), during July 2022 (18.0 g/m2/month)

- WVDD07(BG), during July 2022 (8.3 g/m2/month)

- WVDD02 (N), during August 2022 (5.0 g/m2/month)

- WVDD05 (S), during August 2022 (5.1 g/m2/month)

- WVDD07(BG), during August 2022 (12.7 g/m2/month)

- WVDD02 (N), during September 2022 (10.9 g/m2/month)

- WVDD06 (W), during September 2022 (12.7 g/m2/month)

- WVDD07 (BG), during September 2022 (12.4 g/m2/month)

- Arsenic, Barium and Manganese were within the historical ranges previously reported. Barium and Manganese recorded higher readings in September 2022 at WVDD06.

- The background site (WVDD07) results from Q3 2022 support the previous observation that the arsenic levels recorded in the dust samples from the site are generally slightly higher than the background arsenic levels in the area but are still much lower than the Contaminated Sites Guideline criteria.

- The results from the directional dust monitoring gauges showed no clear patterns during the Q3 period.

# Further Considerations

ERR has advised that they will continue to monitor the site with regular visits and ensure that a range of dust control measures are applied during the current care and maintenance phase of this rehabilitation project. ERR is also reviewing the effectiveness of dust control measures and monitoring on an ongoing basis and adopting a continuous improvement approach to dust minimisation and monitoring.

The installation of an onsite weather monitoring station will be completed in the coming months. This will allow ALS and ERR to investigate any future results.

# References

- ALS Hydrographics Pty Ltd (2021), Woodvale Dust Monitoring Q3 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2021), Woodvale Dust Monitoring Q4 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q1 2022 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q2 2022 Report. (ALS Report)

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Deposited Matter - Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Impinged Matter - Gravimetric Method. (AS/NZS 3580.10.2)

- CDM Smith (2021), Woodvale Evaporation Ponds - Dust Monitoring Report - January to June 2021. (CDM Smith Report)

- West Australian Department of Conservation (2010), Contaminated Sites Management Series, Assessment Levels for Soil, Sediment and Water, February 2010. (Contaminated Sites Assessment Levels)

- EPA Vic Publication 1191 (2007), Protocol for Environmental Management - Mining and Extractive Industries. (Mining PEM)

- EPA NSW Publication (2017), Approved Methods for the Modelling and Assessment of Air Pollutants in NSW, January 2017.

- EPA Vic Publication 1961 (2022), Guideline for Assessing and Minimising Air Pollution in Victoria, February 2022. (Assessing and Minimising Guideline)

- EPA Vic Publication 440.1 (2002), A Guide to the Sampling and Analysis of Air Emissions and Air Quality, December 2002. (Sampling and Analysis Guideline)

- Kralcopic Pty Ltd (2018), Quarterly Environmental Monitoring Report - Woodvale Dust Monitoring, October to December 2018. (Kralcopic Report)

- National Environment Protection Council (1998), National Environment Protection (Ambient Air Quality) Measure, July 2018. (NEPM AAQ)

- Peter J Ramsay and Associates (2018), Woodvale Evaporation Ponds Complex, Environmental Audit of the Risk of any Possible Harm or Detriment to the Environment Pursuant to Part IXD (Section 53V) of the Environment Protection Act 1970, 29 June 2018. (Environmental Audit Report)

- Victorian Government (2001), Statement of Environment Protection Policy (Air Quality Management), Victoria Government Gazette, No S 240, 21 December 2001. (SEPP AQM)

- Victorian Government (2016), State Environment Protection Policy (Ambient Air Quality), Original Policy including Two Variations, Victorian Gazette Nos S19, S240 and G30, published on 9 February 1999, 21 December 2001 and 28 July 2016, respectively. (SEPP AAQ)

- Victorian Government (2021), Environment Reference Standard, under the Environment Protection Act 2017, Victoria Government Gazette, No. S 245, 26 May 2021. (ERS)

# Tables

Table 1 - Woodvale Dust Deposition Analysis Results Q3, 2022

|  |  |  |  |  | Arsenic | Barium | Manganese | Total Solids | Total Solids (mg) | Total Insoluble Matter | Total Insoluble Matter (mg) | Total Soluble Matter | Total Soluble Matter (mg) | Combustible Matter | Combustible Matter (mg) | Ash Content | Ash Content (mg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ug/m2 month | ug/m2 month | ug/m2 month | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg |
| EQL |  |  |  |  |  |  |  |  | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 |
| Description | Location | Quarter | Month | Lab Report Number |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WVDD01 (Insoluble) | WVDD01 | Q3 2022 | Jul-22 | EM2214938 | 1.51 | 129 | 38.4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 5.43 | 132 | 87.2 |  |  |  |  |  |  |  |  |  |  |
| WVDD01 (Soluble) | WVDD01 | Q3 2022 | Jul-22 | EM2214938 | 0.76 | 102 | 174 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 3.36 | 133 | 201 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 0 | 234 | 352 |  |  |  |  |  |  |  |  |  |  |
| WVDD01 (Ash) | WVDD01 | Q3 2022 | Jul-22 | EM2214938 | 1.51 | 129 | 27 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 2.19 | 76.9 | 62.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 3.21 | 105 | 60.8 |  |  |  |  |  |  |  |  |  |  |
| WVDD01 | WVDD01 | Q3 2022 | Jul-22 | EM2214938 |  |  |  | 1.3 | 25 | 1 | 19 | 0.3 | 6 | 1 | 18 | 0.1 | 2 |
|  |  |  | Aug-22 | EM2217582 |  |  |  | 1.9 | 37 | 0.8 | 16 | 1.1 | 21 | 0.3 | 6 | 0.5 | 10 |
|  |  |  | Sep-22 | EM2219329 |  |  |  | 1.6 | 27 | 1.2 | 20 | 0.4 | 7 | 0.9 | 15 | 0.3 | 5 |
| WVDD02 (Insoluble) | WVDD02 | Q3 2022 | Jul-22 | EM2214938 | 1.64 | 44.7 | 32.6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 38.2 | 1110 | 1022 |  |  |  |  |  |  |  |  |  |  |
| WVDD02 (Soluble) | WVDD02 | Q3 2022 | Jul-22 | EM2214938 | 1.75 | 35.3 | 167 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 53.6 | 866 | 3130 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 62.8 | 1230 | 5070 |  |  |  |  |  |  |  |  |  |  |
| WVDD02 (Ash) | WVDD02 | Q3 2022 | Jul-22 | EM2214938 | 1.41 | 41 | 32.3 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 23.4 | 581 | 462 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 28.3 | 1110 | 810 |  |  |  |  |  |  |  |  |  |  |
| WVDD02 | WVDD02 | Q3 2022 | Jul-22 | EM2214938 |  |  |  | 1.9 | 35 | 1.1 | 20 | 0.8 | 15 | 1.1 | 19 | 0.1 | 2 |
|  |  |  | Aug-22 | EM2217582 |  |  |  | 11.2 | 213 | 5 | 95 | 6.2 | 118 | 2.2 | 42 | 2.8 | 53 |
|  |  |  | Sep-22 | EM2219329 |  |  |  | 21.5 | 354 | 10.9 | 179 | 10.6 | 175 | 7.8 | 128 | 3.1 | 51 |
| WVDD03 (Insoluble) | WVDD03 | Q3 2022 | Jul-22 | EM2214938 | 2.15 | 57.5 | 37.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 2.9 | 94.7 | 85.6 |  |  |  |  |  |  |  |  |  |  |
| WVDD03 (Soluble) | WVDD03 | Q3 2022 | Jul-22 | EM2214938 | 6.48 | 218 | 252 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 0.16 | 208 | 607 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 0.65 | 1950 | 1030 |  |  |  |  |  |  |  |  |  |  |
| WVDD03 (Ash) | WVDD03 | Q3 2022 | Jul-22 | EM2214938 | 2.15 | 57.5 | 35.4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 17.1 | 216 | 142 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 2.81 | 94.7 | 81.1 |  |  |  |  |  |  |  |  |  |  |
| WVDD03 | WVDD03 | Q3 2022 | Jul-22 | EM2214938 |  |  |  | 1.9 | 34 | 1.1 | 20 | 0.8 | 14 | 1.1 | 20 | 0.1 | 2 |
|  |  |  | Aug-22 | EM2217582 |  |  |  | 3.2 | 61 | 1.8 | 34 | 1.4 | 27 | 0.8 | 14 | 1 | 20 |
|  |  |  | Sep-22 | EM2219329 |  |  |  | 4.9 | 81 | 1.3 | 22 | 3.6 | 59 | 1.1 | 19 | 0.2 | 3 |
| WVDD04 (Insoluble) | WVDD04 | Q3 2022 | Jul-22 | EM2214938 | 23 | 381 | 509 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 7.69 | 109 | 108 |  |  |  |  |  |  |  |  |  |  |
| WVDD04 (Soluble) | WVDD04 | Q3 2022 | Jul-22 | EM2214938 | 170 | 392 | 562 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 80.2 | 268 | 686 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 55.3 | 550 | 1210 |  |  |  |  |  |  |  |  |  |  |
| WVDD04 (Ash) | WVDD04 | Q3 2022 | Jul-22 | EM2214938 | 23 | 381 | 509 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 8.69 | 135 | 132 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 3.49 | 83 | 87.3 |  |  |  |  |  |  |  |  |  |  |
| WVDD04 | WVDD04 | Q3 2022 | Jul-22 | EM2214938 |  |  |  | 8.9 | 162 | 5.6 | 102 | 3.3 | 60 | 4.5 | 81 | 1.1 | 21 |
|  |  |  | Aug-22 | EM2217582 |  |  |  | 5.2 | 99 | 2 | 38 | 3.2 | 61 | 1.2 | 24 | 0.8 | 14 |
|  |  |  | Sep-22 | EM2219329 |  |  |  | 2.9 | 47 | 0.5 | 8 | 2.4 | 39 | 0.4 | 6 | 0.1 | 2 |
| WVDD05 (Insoluble) | WVDD05 | Q3 2022 | Jul-22 | EM2214938 | 65.3 | 579 | 883 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 0.92 | 39.7 | 29.6 |  |  |  |  |  |  |  |  |  |  |
| WVDD05 (Soluble) | WVDD05 | Q3 2022 | Jul-22 | EM2214938 | 413 | 459 | 1150 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 192 | 564 | 1840 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 0 | 285 | 433 |  |  |  |  |  |  |  |  |  |  |
| WVDD05 (Ash) | WVDD05 | Q3 2022 | Jul-22 | EM2214938 | 49.1 | 579 | 883 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 17.1 | 423 | 601 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 0.92 | 39.7 | 29.6 |  |  |  |  |  |  |  |  |  |  |
| WVDD05 | WVDD05 | Q3 2022 | Jul-22 | EM2214938 |  |  |  | 12.5 | 229 | 6.2 | 114 | 6.3 | 115 | 5.2 | 96 | 1 | 18 |
|  |  |  | Aug-22 | EM2217582 |  |  |  | 10.2 | 192 | 5.1 | 96 | 5.1 | 96 | 3.2 | 61 | 1.9 | 35 |
|  |  |  | Sep-22 | EM2219329 |  |  |  | 5.2 | 85 | 0.2 | 3 | 5 | 82 | 0.2 | 3 | 0.1 | 2 |
| WVDD06 (Insoluble) | WVDD06 | Q3 2022 | Jul-22 | EM2214938 | 39.7 | 2130 | 1440 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 50.6 | 2070 | 1470 |  |  |  |  |  |  |  |  |  |  |
| WVDD06 (Soluble) | WVDD06 | Q3 2022 | Jul-22 | EM2214938 | 45.7 | 449 | 2640 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 0.05 | 332 | 259 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 166 | 2930 | 6080 |  |  |  |  |  |  |  |  |  |  |
| WVDD06 (Ash) | WVDD06 | Q3 2022 | Jul-22 | EM2214938 | 39.7 | 2130 | 1440 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 1.64 | 64.9 | 33.4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 49.6 | 2070 | 1470 |  |  |  |  |  |  |  |  |  |  |
| WVDD06 | WVDD06 | Q3 2022 | Jul-22 | EM2214938 |  |  |  | 18.5 | 339 | 18 | 329 | 0.5 | 10 | 15.3 | 279 | 2.7 | 50 |
|  |  |  | Aug-22 | EM2217582 |  |  |  | 1.8 | 34 | 0.5 | 10 | 1.3 | 24 | 0.2 | 4 | 0.3 | 6 |
|  |  |  | Sep-22 | EM2219329 |  |  |  | 28.6 | 472 | 12.7 | 209 | 15.9 | 263 | 8.7 | 142 | 4 | 67 |
| WVDD07BG (Insoluble) | WVDD07BG | Q3 2022 | Jul-22 | EM2214938 | 17.3 | 882 | 1100 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 37.6 | 1160 | 2180 |  |  |  |  |  |  |  |  |  |  |
| WVDD07BG (Soluble) | WVDD07BG | Q3 2022 | Jul-22 | EM2214938 | 37 | 549 | 1440 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 53.7 | 930 | 2310 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 38.5 | 531 | 1220 |  |  |  |  |  |  |  |  |  |  |
| WVDD07BG (Ash) | WVDD07BG | Q3 2022 | Jul-22 | EM2214938 | 15.9 | 882 | 980 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Aug-22 | EM2217582 | 30.6 | 865 | 864 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Sep-22 | EM2219329 | 35 | 1160 | 2090 |  |  |  |  |  |  |  |  |  |  |
| WVDD07BG | WVDD07BG | Q3 2022 | Jul-22 | EM2214938 |  |  |  | 11.6 | 212 | 8.3 | 151 | 3.3 | 61 | 6.1 | 110 | 2.2 | 41 |
|  |  |  | Aug-22 | EM2217582 |  |  |  | 20.2 | 381 | 12.7 | 240 | 7.5 | 141 | 2.7 | 52 | 10 | 188 |
|  |  |  | Sep-22 | EM2219329 |  |  |  | 21 | 347 | 12.4 | 205 | 8.6 | 142 | 7.8 | 129 | 4.6 | 76 |

Table 2 - Woodvale Directional Dust Deposition Analysis Results Q3, 2022

|  |  |  | Arsenic | Barium | Manganese | Total Solids | Total Solids (mg) | Total Insoluble Matter | Total Insoluble Matter (mg) | Total Soluble Matter | Total Soluble Matter (mg) | Combustible Matter | Combustible Matter (mg) | Ash Content | Ash Content (mg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ug/m2 month | ug/m2 month | ug/m2 month | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg |
| EQL |  |  |  |  |  |  | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 |
| Description | Month | Lab Report Number |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WVDG03E (Insoluble) | Jul-22 | EM2214938 | 1.27 | 55.1 | 17.9 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 22.7 | 16.8 |  |  |  |  |  |  |  |  |  |  |
| WVDG03E (Soluble) | Jul-22 | EM2214938 | 0.77 | 30.8 | 15.5 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.28 | 12.1 | 6.74 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 13 | 14.3 |  |  |  |  |  |  |  |  |  |  |
| WVDG03E (Ash) | Jul-22 | EM2214938 | 1.27 | 36.2 | 17.9 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.26 | 16.7 | 7.3 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 14.3 | 8.91 |  |  |  |  |  |  |  |  |  |  |
| WVDG03E | Jul-22 | EM2214938 |  |  |  | 1 | 14 | 0.8 | 12 | 0.2 | 2 | 0.7 | 10 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.5 | 8 | 0.6 | 9 | 0.1 | 2 | 0.6 | 8 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.8 | 13 | 0.8 | 12 | 0.1 | 2 | 0.8 | 12 | 0.1 | 2 |
| WVDG03N (Insoluble) | Jul-22 | EM2214938 | 2.76 | 1310 | 15.7 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.11 | 84.6 | 28.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG03N (Soluble) | Jul-22 | EM2214938 | 0.27 | 27 | 3.4 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.05 | 7.31 | 1.39 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 7.6 | 7.33 |  |  |  |  |  |  |  |  |  |  |
| WVDG03N (Ash) | Jul-22 | EM2214938 | 1.94 | 469 | 9.43 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.11 | 15.5 | 2.52 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 32.9 | 10.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG03N | Jul-22 | EM2214938 |  |  |  | 1.3 | 20 | 1.3 | 20 | 0.1 | 2 | 1.3 | 20 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.6 | 9 | 0.6 | 9 | 0.1 | 2 | 0.5 | 8 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
| WVDG03S (Insoluble) | Jul-22 | EM2214938 | 0.66 | 25.5 | 20.1 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.27 | 24.6 | 35 |  |  |  |  |  |  |  |  |  |  |
| WVDG03S (Soluble) | Jul-22 | EM2214938 | 0.33 | 19.5 | 10.1 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.36 | 21.8 | 9.54 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 1.58 | 7.98 | 27.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG03S (Ash) | Jul-22 | EM2214938 | 0.66 | 25.5 | 20.1 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.05 | 3.62 | 3.48 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.12 | 17.5 | 18.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG03S | Jul-22 | EM2214938 |  |  |  | 0.9 | 15 | 0.8 | 13 | 0.1 | 2 | 0.7 | 11 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.5 | 8 | 0.5 | 8 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 1.1 | 15 | 0.9 | 13 | 0.2 | 2 | 0.9 | 13 | 0.1 | 2 |
| WVDG03W (Insoluble) | Jul-22 | EM2214938 | 0.25 | 18.4 | 7.66 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.12 | 23.3 | 21.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG03W (Soluble) | Jul-22 | EM2214938 | 0.17 | 20.9 | 8.41 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.28 | 6.33 | 5.18 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.05 | 5.23 | 6.3 |  |  |  |  |  |  |  |  |  |  |
| WVDG03W (Ash) | Jul-22 | EM2214938 | 0.25 | 12.7 | 7.66 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.05 | 1.26 | 1.15 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 |  | 0.11 | 23 | 15 |  |  |  |  |  |  |  |  |  |  |
| WVDG03W | Jul-22 | EM2214938 |  |  |  | 1.2 | 19 | 1.1 | 18 | 0.1 | 2 | 1.1 | 18 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.2 | 5 | 0.2 | 4 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.2 | 3 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
| WVDG05E (Insoluble) | Jul-22 | EM2214938 | 0.423 | 12.9 | 7.88 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.23 | 20.5 | 18.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG05E (Soluble) | Jul-22 | EM2214938 | 0.26 | 4.28 | 3.36 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 2.99 | 41.2 | 46.1 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.37 | 6.15 | 7.49 |  |  |  |  |  |  |  |  |  |  |
| WVDG05E (Ash) | Jul-22 | EM2214938 | 0.39 | 12.9 | 5.69 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 1.54 | 78.7 | 35 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.11 | 20.5 | 18.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG05E | Jul-22 | EM2214938 |  |  |  | 1.3 | 20 | 1.3 | 20 | 0.1 | 2 | 1.3 | 20 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 1 | 16 | 0.6 | 10 | 0.4 | 6 | 0.3 | 4 | 0.3 | 6 |
|  | Sep-22 | EM2219329 |  |  |  | 0.2 | 4 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
| WVDG05N (Insoluble) | Jul-22 | EM2214938 | 0.36 | 8.21 | 15.5 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 1.32 | 18.9 | 15.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG05N (Soluble) | Jul-22 | EM2214938 | 0.45 | 15.8 | 8.02 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.08 | 17.5 | 10.3 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.17 | 5.36 | 5.01 |  |  |  |  |  |  |  |  |  |  |
| WVDG05N (Ash) | Jul-22 | EM2214938 | 0.36 | 8.21 | 15.5 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 6.94 | 12.1 | 14.2 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.65 | 18.8 | 14.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG05N | Jul-22 | EM2214938 |  |  |  | 0.8 | 12 | 0.7 | 11 | 0.1 | 2 | 0.7 | 11 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.5 | 8 | 0.4 | 6 | 0.1 | 2 | 0.4 | 5 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.3 | 4 | 0.2 | 3 | 0.1 | 2 | 0.2 | 2 | 0.1 | 2 |
| WVDG05S (Insoluble) | Jul-22 | EM2214938 | 0.55 | 10.9 | 9.5 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.31 | 26.1 | 21.8 |  |  |  |  |  |  |  |  |  |  |
| WVDG05S (Soluble) | Jul-22 | EM2214938 | 0.44 | 6.52 | 8.33 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.07 | 17.4 | 10.1 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 3.77 | 4.25 |  |  |  |  |  |  |  |  |  |  |
| WVDG05S (Ash) | Jul-22 | EM2214938 | 0.55 | 9.94 | 7.13 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.05 | 0.4 | 0.2 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.31 | 26.1 | 21.8 |  |  |  |  |  |  |  |  |  |  |
| WVDG05S | Jul-22 | EM2214938 |  |  |  | 1.1 | 17 | 1.1 | 17 | 0.1 | 2 | 1.1 | 17 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.2 | 2 | 0.1 | 2 | 0.1 | 2 | 0.2 | 2 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
| WVDG05W (Insoluble) | Jul-22 | EM2214938 | 0.767 | 8.57 | 5.05 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.28 | 22.1 | 28.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG05W (Soluble) | Jul-22 | EM2214938 | 0.05 | 0.95 | 0.74 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.21 | 23.9 | 13.1 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 5.01 | 10.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG05W (Ash) | Jul-22 | EM2214938 | 0.069 | 5.49 | 1.6 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.36 | 43.2 | 8.77 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.16 | 22.1 | 21.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG05W | Jul-22 | EM2214938 |  |  |  | 0.7 | 11 | 0.7 | 11 | 0.1 | 2 | 0.7 | 11 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.3 | 5 | 0.1 | 2 | 0.2 | 4 | 0.1 | 2 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.4 | 6 | 0.1 | 2 | 0.3 | 4 | 0.1 | 2 | 0.1 | 2 |
| WVDG04E (Insoluble) | Jul-22 | EM2214938 | 0.97 | 37.3 | 12.5 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 22.1 | 16.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG04E (Soluble) | Jul-22 | EM2214938 | 0.07 | 1.71 | 1.1 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 1.02 | 13.3 | 19.7 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 1.48 | 36.2 | 52.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG04E (Ash) | Jul-22 | EM2214938 | 0.97 | 37.3 | 12.4 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 1.07 | 36.3 | 32.8 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 13.8 | 14.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG04E | Jul-22 | EM2214938 |  |  |  | 1.1 | 18 | 1.1 | 18 | 0.1 | 2 | 1.1 | 18 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.3 | 5 | 0.4 | 6 | 0.1 | 2 | 0.3 | 4 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.5 | 8 | 0.2 | 3 | 0.3 | 5 | 0.2 | 2 | 0.1 | 2 |
| WVDG04N (Insoluble) | Jul-22 | EM2214938 | 0.61 | 20.7 | 11.8 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.82 | 31.7 | 21.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG04N (Soluble) | Jul-22 | EM2214938 | 0.48 | 6.73 | 7.31 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 1.8 | 38.1 | 22.7 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 7.27 | 15.6 | 47.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG04N (Ash) | Jul-22 | EM2214938 | 0.61 | 17.3 | 11.8 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 3.21 | 159 | 88 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.82 | 31.7 | 20.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG04N | Jul-22 | EM2214938 |  |  |  | 1.3 | 20 | 1.2 | 19 | 0.1 | 2 | 1.2 | 19 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 1 | 17 | 1 | 17 | 0.1 | 2 | 0.4 | 7 | 0.6 | 10 |
|  | Sep-22 | EM2219329 |  |  |  | 0.5 | 7 | 0.3 | 4 | 0.2 | 3 | 0.3 | 4 | 0.1 | 2 |
| WVDG04S (Insoluble) | Jul-22 | EM2214938 | 0.36 | 15.3 | 8.49 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 17.4 | 22.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG04S (Soluble) | Jul-22 | EM2214938 | 0.05 | 1.73 | 1.18 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 1.04 | 11.5 | 12.7 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 3.18 | 14.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG04S (Ash) | Jul-22 | EM2214938 | 0.36 | 10.5 | 6.46 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.86 | 30 | 28 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 14 | 13.9 |  |  |  |  |  |  |  |  |  |  |
| WVDG04S | Jul-22 | EM2214938 |  |  |  | 0.9 | 15 | 0.9 | 15 | 0.1 | 2 | 0.9 | 15 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.4 | 6 | 0.4 | 6 | 0.1 | 2 | 0.3 | 4 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.4 | 5 | 0.4 | 5 | 0.1 | 2 | 0.4 | 5 | 0.1 | 2 |
| WVDG04W (Insoluble) | Jul-22 | EM2214938 | 0.75 | 14.4 | 11.3 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.23 | 18.4 | 20.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG04W (Soluble) | Jul-22 | EM2214938 | 1.36 | 17.4 | 14.3 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.61 | 12.1 | 11.2 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.93 | 12.8 | 31.3 |  |  |  |  |  |  |  |  |  |  |
| WVDG04W (Ash) | Jul-22 | EM2214938 | 0.75 | 14.4 | 10.5 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.93 | 23.2 | 14.9 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.23 | 15.4 | 14.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG04W | Jul-22 | EM2214938 |  |  |  | 0.8 | 14 | 0.8 | 13 | 0.1 | 2 | 0.8 | 13 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.6 | 11 | 0.6 | 10 | 0.1 | 2 | 0.5 | 9 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.7 | 10 | 0.4 | 6 | 0.3 | 4 | 0.4 | 6 | 0.1 | 2 |
| WVDG06E (Insoluble) | Jul-22 | EM2214938 | 0.38 | 17.4 | 10.8 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 20.6 | 22.3 |  |  |  |  |  |  |  |  |  |  |
| WVDG06E (Soluble) | Jul-22 | EM2214938 | 0.05 | 1.56 | 2.42 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.42 | 8.21 | 15.9 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 10.2 | 23.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG06E (Ash) | Jul-22 | EM2214938 | 0.38 | 12 | 9 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.58 | 14.2 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 20.6 | 22.3 |  |  |  |  |  |  |  |  |  |  |
| WVDG06E | Jul-22 | EM2214938 |  |  |  | 0.9 | 14 | 0.9 | 14 | 0.1 | 2 | 0.9 | 14 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.2 | 2 | 0.1 | 2 | 0.2 | 2 | 0.1 | 2 | 0.1 | 2 |
| WVDG06N (Insoluble) | Jul-22 | EM2214938 | 1.16 | 35.8 | 24.2 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 16.8 | 17.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG06N (Soluble) | Jul-22 | EM2214938 | 2.13 | 24.9 | 46 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 1.51 | 10.1 | 19.6 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 4.53 | 7.41 |  |  |  |  |  |  |  |  |  |  |
| WVDG06N (Ash) | Jul-22 | EM2214938 | 1.16 | 35.8 | 24.2 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 7.88 | 46.3 | 29.3 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 16.8 | 17.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG06N | Jul-22 | EM2214938 |  |  |  | 1.5 | 24 | 1 | 16 | 0.5 | 8 | 0.9 | 15 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.7 | 11 | 0.6 | 10 | 0.1 | 2 | 0.3 | 5 | 0.3 | 5 |
|  | Sep-22 | EM2219329 |  |  |  | 0.2 | 3 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
| WVDG06S (Insoluble) | Jul-22 | EM2214938 | 0.59 | 116 | 56.3 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.39 | 35.3 | 109 |  |  |  |  |  |  |  |  |  |  |
| WVDG06S (Soluble) | Jul-22 | EM2214938 | 0.16 | 2 | 2.18 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.61 | 7.71 | 15.9 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.37 | 12.1 | 25.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG06S (Ash) | Jul-22 | EM2214938 | 0.59 | 116 | 56.3 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.49 | 18 | 17.4 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.13 | 21.9 | 25 |  |  |  |  |  |  |  |  |  |  |
| WVDG06S | Jul-22 | EM2214938 |  |  |  | 0.9 | 14 | 0.9 | 14 | 0.1 | 2 | 0.8 | 12 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.2 | 3 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 0.2 | 3 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
| WVDG06W (Insoluble) | Jul-22 | EM2214938 | 0.464 | 12.1 | 12.1 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 40.4 | 16.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG06W (Soluble) | Jul-22 | EM2214938 | 0.87 | 13.8 | 19.4 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.26 | 4.94 | 8.27 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0.11 | 3.97 | 10.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG06W (Ash) | Jul-22 | EM2214938 | 0.33 | 8.76 | 7.37 |  |  |  |  |  |  |  |  |  |  |
|  | Aug-22 | EM2217582 | 0.27 | 13.6 | 11.1 |  |  |  |  |  |  |  |  |  |  |
|  | Sep-22 | EM2219329 | 0 | 40.4 | 16.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG06W | Jul-22 | EM2214938 |  |  |  | 0.8 | 13 | 0.7 | 11 | 0.1 | 2 | 0.7 | 11 | 0.1 | 2 |
|  | Aug-22 | EM2217582 |  |  |  | 0.2 | 4 | 0.2 | 3 | 0.1 | 2 | 0.2 | 3 | 0.1 | 2 |
|  | Sep-22 | EM2219329 |  |  |  | 1.2 | 17 | 1.1 | 16 | 0.1 | 2 | 1.1 | 16 | 0.1 | 2 |

# Supplementary Data

Graphs for Metals/Metalloids in Insoluble and Soluble Fractions including trends are presented below.

## Arsenic

Figure 13 - Woodvale - Arsenic in Soluble Fraction – Q3 2022

Figure 14 - Woodvale - Arsenic in Insoluble Fraction – Q3 2022

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

Figure 15 - Woodvale - Arsenic in Soluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

Figure 16 - Woodvale - Arsenic in Insoluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

Arsenic lab results unable to be calculated have been left blank.

## Barium

Figure 17 - Woodvale - Barium in Soluble Fraction – Q3 2022

Figure 18 - Woodvale - Barium in Insoluble Fraction – Q3 2022

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

No lab results available for August.

Figure 19 - Woodvale - Barium in Soluble Fraction Year Review (2021-22)

.

## Manganese

Figure 20 - Woodvale - Manganese in Soluble Fraction – Q3 2022

Figure 21 - Woodvale - Manganese in Insoluble Fraction – Q3 2022

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

No lab results available for August.

Figure 22 - Woodvale - Manganese in Soluble Fraction Year Review (2021-22)

Figure 23 - Woodvale - Manganese in Insoluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

No lab results available for August.

## Trend Analysis Graphs

Figure 24 - Woodvale - Total Insoluble Matter (TIM) - 2021-2022

Figure 24 shows TIM for all months in 2021 and the first three quarters of 2022. The review of Q3 2022 data shows slightly higher trends across WVDD05 and WVDD06, which is consistent with the background site. In Q3 2022, there were multiple sites where elevated levels were recorded for July 2022, Aug 2022 and Sept 2022. This may have been influenced by environmental conditions including, but not limited to, strong wind conditions. The overall average from all monitoring locations was higher, being 4.1 g/m2/month for Q3 2022 (excluding the background control site), compared to 2.5 g/m2/month for Q2 in 2022. During the Q3 2022 field visits, ALS observed no variation in site conditions while undertaking monthly site visits, which could have contributed to higher levels recorded.

Figure 25 - Woodvale - Total Ash Content - 2021-2022

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

Figure 25 shows the Total Ash Content for all months in 2021and the first two quarters of 2022 with Q2 monthly figures being at similar levels to the levels observed in Q2 2021. Q3 has shown an increase in all three months.

Figure 26 - Woodvale - Ash Content/TIM Ratio - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 26 shows all results collected and sampled for 2021 as well as results for Q1, Q2 and Q3 of 2022.

Figure 27 - Woodvale - Total Arsenic/Total Solids - 2021-2022

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

August 2022 could not be calculated as per other months due to lab sample results being unavailable.

Figure 27 above presents the ratio of Total Arsenic/Total Solids. Q3 Arsenic results have been given a     over the sample as lab results were compromised.

Figure 28 - Woodvale - DDG Total Solids Comparison North/South - 2021-2022

Figure 28 above displays the results from the 2021 to present. No definitive pattern is apparent throughout this period with small net gains and losses from both the upwind and downwind quadrants being observed. The southerly facing quadrant does have greater gains in 15 of 21 months and the northerly facing quadrant has gains in 10 of 21 months. However, the overall gains from the site are showing to be very low levels.

Figure 29 - Woodvale - Arsenic/Total Solids Comparison - North/South 2021-2022

An asterisk (*) has been placed over the samples recorded as less than (<) the value x displayed on the graph.

No results for arsenic (insoluble) available for August 2022.

Figure 29 shows the net gains and losses in total arsenic concentration for the north and south quadrants from January 2021 to September 2022. No definitive pattern is apparent in this period with net gains and losses from both the upwind and downwind quadrants being observed.

Figure 30 - Woodvale - DDG Total Solids Comparison East/West - 2021-2022

Figure 30 above displays the net gain or loss both upwind and downwind from east and west for total solids. No clear pattern is identified during the 12 months with both gains and losses occurring throughout the period.

Figure 31 - Woodvale - Arsenic/Total Solids Comparison - East/West - 2021-2022

Figure 31 shows the net gains and losses in total arsenic concentration for the east/west quadrants from the previous 12 months. No definitive pattern is apparent in this period with net gains and losses from both the upwind and downwind quadrants being observed.

# Appendix A – Supplementary Information

# Appendix B – Laboratory Reports

Environmental

CERTIFICATE OF ANALYSIS

Work Order	: EM2214938

Page	: 1 of 19

Client

Contact

Address

: ALS WATER AND HYDROGRAPHICS PTY LTD

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Bronwyn Sheen

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +6138549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 05-Aug-2022 13:38 |
| Order number | : ---- | Date Analysis Commenced | : 10-Aug-2022 |
| C-O-C number Sampler Site | : ---- : ---- : ---- | Issue Date | : 18-Aug-2022 15:43 |
| Quote number No. of samples received No. of samples analysed | : ME/ : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Jarwis Nheu	Non-Metals Team Leader	Melbourne Inorganics, Springvale, VIC

R I G H T  S O L U T I O N S | R I G H T  P A R T N E R

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- Sampling period: 01/07/2022-01/08/22.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- EA120+EA125+EA139+EA141+EA142: EM2214938 #6 & #7: Samples were visually observed to be turbid.

- EA120+EA125+EA139+EA141+EA142: EM2214938 #1-#12: Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- EA120I+EA125I+EA139I+EA141I+EA142I: EM2214938 #14-#16, #20, #21, #25, #28: Sample container was received with no liquid. 100ml of DI water was used to dissolve contents, perform analysis and for calculation of results.

- EA120I+EA125I+EA139I+EA141I+EA142I: EM2214938 #13-#28: Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 11:25 | 01-Aug-2022 11:35 | 01-Aug-2022 11:50 | 01-Aug-2022 12:15 | 01-Aug-2022 12:05 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2214938-001 | EM2214938-002 | EM2214938-003 | EM2214938-004 | EM2214938-005 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result 1.1 | Result 1.0 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.0 | <2 1.1 | <2 1.1 | 21 4.5 | 18 5.2 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 18 0.3 | 19 0.8 | 20 0.8 | 81 3.3 | 96 6.3 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 6 1.0 | 15 1.1 | 14 1.1 | 60 5.6 | 115 6.2 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 19 1.3 | 20 1.9 | 20 1.9 | 102 8.9 | 114 12.5 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 25 <1.51 | 35 1.41 | 34 <2.15 | 162 <23.0 | 229 49.1 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <129 | 41.0 | <57.5 | <381 | <579 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 27.0 | 32.3 | 35.4 | <509 | <883 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD06 | WVDD07BG | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 11:10 | 01-Aug-2022 10:45 | 01-Aug-2022 13:35 | 01-Aug-2022 13:20 | 01-Aug-2022 13:25 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2214938-006 | EM2214938-007 | EM2214938-008 | EM2214938-009 | EM2214938-010 |
|  |  |  | Result 2.7 | Result 2.2 | Result <0.1 | Result <0.1 | Result 0.3 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 50 15.3 | 41 6.1 | <2 1.8 | <2 1.3 | 6 2.2 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 279 0.5 | 110 3.3 | 32 0.2 | 23 0.3 | 39 0.5 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 10 18.0 | 61 8.3 | 3 1.8 | 5 1.3 | 9 2.5 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 329 18.5 | 151 11.6 | 32 2.0 | 24 1.6 | 45 3.0 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 339 <39.7 | 212 15.9 | 35 <2.72 | 29 5.67 | 54 77.8 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <2130 | <882 | <46.9 | <44.3 | <1790 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <1440 | 980 | <36.4 | 28.8 | <489 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 12:15 | 01-Aug-2022 13:05 | 01-Aug-2022 11:50 | 01-Aug-2022 11:50 | 01-Aug-2022 11:50 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2214938-011 | EM2214938-012 | EM2214938-013 | EM2214938-014 | EM2214938-015 |
|  |  |  | Result <0.1 | Result <0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | <2 ---- | <2 ---- | ---- <0.1 | ---- 0.1 | ---- 0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 1.6 | ---- 1.2 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 29 ---- | 21 ---- | ---- 1.3 | ---- 0.7 | ---- 0.7 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.2 | ---- 0.3 | 20 ---- | 10 ---- | 11 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 3 ---- | 6 ---- | ---- <0.1 | ---- 0.2 | ---- 0.1 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 1.6 | ---- 1.2 | <2 ---- | 2 ---- | 2 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 29 ---- | 22 ---- | ---- 1.3 | ---- 0.8 | ---- 0.8 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 1.8 | ---- 1.5 | 20 ---- | 12 ---- | 13 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 32 ---- | 28 ---- | ---- 1.3 | ---- 1.0 | ---- 0.9 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- <19.5 | ---- <4.47 | 20 1.94 | 14 <1.27 | 15 <0.660 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <74.6 | <46.0 | 469 | 36.2 | <25.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <38.2 | 30.2 | 9.43 | <17.9 | <20.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 11:50 | 01-Aug-2022 12:15 | 01-Aug-2022 12:15 | 01-Aug-2022 12:15 | 01-Aug-2022 12:15 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2214938-016 | EM2214938-017 | EM2214938-018 | EM2214938-019 | EM2214938-020 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.1 | <2 1.2 | <2 1.1 | <2 0.9 | <2 0.8 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 18 0.1 | 19 0.1 | 18 <0.1 | 15 <0.1 | 13 <0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 1.1 | <2 1.2 | <2 1.1 | <2 0.9 | <2 0.8 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 18 1.2 | 19 1.3 | 18 1.1 | 15 0.9 | 13 0.8 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 19 <0.250 | 20 <0.610 | 18 <0.970 | 15 <0.360 | 14 <0.750 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 12.7 | 17.3 | <37.3 | 10.5 | <14.4 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <7.66 | <11.8 | 12.4 | 6.46 | 10.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG06N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 12:05 | 01-Aug-2022 12:05 | 01-Aug-2022 12:05 | 01-Aug-2022 12:05 | 01-Aug-2022 11:10 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2214938-021 | EM2214938-022 | EM2214938-023 | EM2214938-024 | EM2214938-025 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.7 | <2 1.3 | <2 1.1 | <2 0.7 | <2 0.9 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 11 0.1 | 20 <0.1 | 17 <0.1 | 11 <0.1 | 15 0.5 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.7 | <2 1.3 | <2 1.1 | <2 0.7 | 8 1.0 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 11 0.8 | 20 1.3 | 17 1.1 | 11 0.7 | 16 1.5 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 12 <0.360 | 20 0.390 | 17 <0.550 | 11 0.069 | 24 <1.16 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <8.21 | <12.9 | 9.94 | 5.49 | <35.8 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <15.5 | 5.69 | 7.13 | 1.60 | <24.2 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06E | WVDG06S | WVDG06W | WVDD01 Soluble metals | WVDD02 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 11:10 | 01-Aug-2022 11:10 | 01-Aug-2022 11:10 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2214938-026 | EM2214938-027 | EM2214938-028 | EM2214938-029 | EM2214938-030 |
|  |  |  | Result <0.1 | Result 0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.9 | <2 0.8 | <2 0.7 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 14 <0.1 | 12 <0.1 | 11 0.1 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.9 | <2 0.9 | 2 0.7 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 14 0.9 | 14 0.9 | 11 0.8 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 14 <0.38 | 14 <0.59 | 13 ---- | ---- ---- | ---- ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | ---- | ---- | 0.330 | 0.760 | 1.75 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 12.0 | <116 | 8.76 | 102 | 35.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 9.00 | <56.3 | 7.37 | 174 | 167 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble metals | WVDD04 Soluble metals | WVDD05 Soluble metals | WVDD06 Soluble metals | WVDD07BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2214938-031 | EM2214938-032 | EM2214938-033 | EM2214938-034 | EM2214938-035 |
|  |  |  | Result 6.48 | Result 170 | Result 413 | Result 45.7 | Result 37.0 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 218 | 392 | 459 | 449 | 549 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 252 | 562 | 1150 | 2640 | 1440 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble metals | KF2 Soluble metals | KF3 Soluble metals | KF4 Soluble metals | KF5BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2214938-036 | EM2214938-037 | EM2214938-038 | EM2214938-039 | EM2214938-040 |
|  |  |  | Result 2.70 | Result 9.02 | Result 33.9 | Result 5.09 | Result 2.30 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 77.6 | 55.0 | 174 | 91.8 | 57.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 129 | 85.2 | 401 | 106 | 127 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N Soluble metals | WVDG03E Soluble metals | WVDG03S Soluble metals | WVDG03W Soluble metals | WVDG04N Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2214938-041 | EM2214938-042 | EM2214938-043 | EM2214938-044 | EM2214938-045 |
|  |  |  | Result 0.270 | Result 0.770 | Result 0.330 | Result 0.170 | Result 0.480 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 27.0 | 30.8 | 19.5 | 20.9 | 6.73 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 3.40 | 15.5 | 10.1 | 8.41 | 7.31 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E Soluble metals | WVDG04S Soluble metals | WVDG04W Soluble metals | WVDG05N Soluble metals | WVDG05E Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2214938-046 | EM2214938-047 | EM2214938-048 | EM2214938-049 | EM2214938-050 |
|  |  |  | Result 0.070 | Result 0.050 | Result 1.36 | Result 0.450 | Result 0.260 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 1.71 | 1.73 | 17.4 | 15.8 | 4.28 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 1.10 | 1.18 | 14.3 | 8.02 | 3.36 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble metals | WVDG05W Soluble metals | WVDG06N Soluble metals | WVDG06E Soluble metals | WVDG06S Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2214938-051 | EM2214938-052 | EM2214938-053 | EM2214938-054 | EM2214938-055 |
|  |  |  | Result 0.440 | Result 0.050 | Result 2.13 | Result 0.050 | Result 0.160 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 6.52 | 0.95 | 24.9 | 1.56 | 2.00 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 8.33 | 0.74 | 46.0 | 2.42 | 2.18 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06W Soluble metals | WVDD01 Insoluble metals | WVDD02 Insoluble metals | WVDD03 Insoluble metals | WVDD04 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2214938-056 | EM2214938-057 | EM2214938-058 | EM2214938-059 | EM2214938-060 |
|  |  |  | Result ---- | Result <1.51 | Result 1.64 | Result <2.15 | Result <23.0 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 0.870 | ---- | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 13.8 | <129 | 44.7 | <57.5 | <381 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 19.4 | 38.4 | 32.6 | 37.5 | <509 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD05 Insoluble metals | WVDD06 Insoluble metals | WVDD07BG Insoluble metals | KF1 Insoluble metals | KF2 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2214938-061 | EM2214938-062 | EM2214938-063 | EM2214938-064 | EM2214938-065 |
|  |  |  | Result 65.3 | Result <39.7 | Result 17.3 | Result <2.72 | Result 10.4 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <579 | <2130 | <882 | <46.9 | <44.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <883 | <1440 | 1100 | <36.4 | 35.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 Insoluble metals | KF4 Insoluble metals | KF5BG Insoluble metals | WVDG03N Insoluble metals | WVDG03E Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2214938-066 | EM2214938-067 | EM2214938-068 | EM2214938-069 | EM2214938-070 |
|  |  |  | Result 79.7 | Result <19.5 | Result <4.47 | Result 2.76 | Result <1.27 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <1790 | <74.6 | <46.0 | 1310 | 55.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <489 | <38.2 | 31.9 | 15.7 | <17.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Insoluble metals | WVDG03W Insoluble metals | WVDG04N Insoluble metals | WVDG04E Insoluble metals | WVDG04S Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2214938-071 | EM2214938-072 | EM2214938-073 | EM2214938-074 | EM2214938-075 |
|  |  |  | Result <0.660 | Result <0.250 | Result <0.610 | Result <0.970 | Result <0.360 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <25.5 | 18.4 | 20.7 | <37.3 | 15.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <20.1 | <7.66 | <11.8 | 12.5 | 8.49 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Insoluble metals | WVDG05N Insoluble metals | WVDG05E Insoluble metals | WVDG05S Insoluble metals | WVDG05W Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2214938-076 | EM2214938-077 | EM2214938-078 | EM2214938-079 | EM2214938-080 |
|  |  |  | Result <0.750 | Result <0.360 | Result 0.423 | Result <0.550 | Result 0.767 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <14.4 | <8.21 | <12.9 | 10.9 | 8.57 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 11.3 | <15.5 | 7.88 | 9.50 | 5.05 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06N Insoluble metals | WVDG06E Insoluble metals | WVDG06S Insoluble metals | WVDG06W Insoluble metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | 01-Aug-2022 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2214938-081 | EM2214938-082 | EM2214938-083 | EM2214938-084 | -------- |
|  |  |  | Result <1.16 | Result <0.380 | Result <0.590 | Result 0.464 | ---- ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <35.8 | 17.4 | <116 | 12.1 | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <24.2 | 10.8 | <56.3 | 12.1 | ---- |

Environmental

CERTIFICATE OF ANALYSIS

Work Order	: EM2217582

Page	: 1 of 14

Client

Contact

Address

: ALS WATER AND HYDROGRAPHICS PTY LTD

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Bronwyn Sheen

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +6138549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 06-Sep-2022 10:10 |
| Order number | : ---- | Date Analysis Commenced | : 12-Sep-2022 |
| C-O-C number Sampler Site | : ---- : R. Oliver : ALS KERANG DUST | Issue Date | : 20-Sep-2022 20:43 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 56 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Nikki Stepniewski	Senior Inorganic Instrument Chemist	Melbourne Inorganics, Springvale, VIC

R I G H T  S O L U T I O N S | R I G H T  P A R T N E R

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- EM2217582 #7; Sample was observed to be very turbid.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- EG020TUG: EM2217582#1 to #28 metals analysis were conducted on the whole filter paper.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- EM2217582 # 15, 21, 23, 24; Sample container was received with no liquid. 100ml of DI water was used to dissolve contents, perform analysis and for calculation of results.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 11:15 | 02-Sep-2022 11:25 | 02-Sep-2022 11:40 | 02-Sep-2022 11:55 | 02-Sep-2022 12:08 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2217582-001 | EM2217582-002 | EM2217582-003 | EM2217582-004 | EM2217582-005 |
|  |  |  | Result 0.5 | Result 2.8 | Result 1.0 | Result 0.8 | Result 1.9 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 10 0.3 | 53 2.2 | 20 0.8 | 14 1.2 | 35 3.2 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 6 1.1 | 42 6.2 | 14 1.4 | 24 3.2 | 61 5.1 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 21 0.8 | 118 5.0 | 27 1.8 | 61 2.0 | 96 5.1 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 16 1.9 | 95 11.2 | 34 3.2 | 38 5.2 | 96 10.2 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | 37 2.19 | 213 23.4 | 61 17.1 | 99 8.69 | 192 17.1 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 76.9 | 581 | 216 | 135 | 423 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 62.1 | 462 | 142 | 132 | 601 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD06 | WVDD007BG | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 10:40 | 02-Sep-2022 10:20 | 02-Sep-2022 13:35 | 02-Sep-2022 13:25 | 02-Sep-2022 13:30 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2217582-006 | EM2217582-007 | EM2217582-008 | EM2217582-009 | EM2217582-010 |
|  |  |  | Result 0.3 | Result 10.0 | Result 0.3 | Result 0.2 | Result 0.7 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 6 0.2 | 188 2.7 | 5 0.5 | 3 0.4 | 13 1.1 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 4 1.3 | 52 7.5 | 11 0.1 | 9 <0.1 | 20 3.1 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 24 0.5 | 141 12.7 | 2 0.8 | <2 0.6 | 58 1.8 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 10 1.8 | 240 20.2 | 16 0.9 | 12 0.6 | 33 4.9 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | 34 1.64 | 381 30.6 | 18 3.78 | 12 6.97 | 91 42.9 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 64.9 | 865 | 61.9 | 36.3 | 264 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 33.4 | 864 | 35.3 | 33.3 | 160 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 13:15 | 02-Sep-2022 13:00 | 02-Sep-2022 11:40 | 02-Sep-2022 11:40 | 02-Sep-2022 11:40 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2217582-011 | EM2217582-012 | EM2217582-013 | EM2217582-014 | EM2217582-015 |
|  |  |  | Result <0.1 | Result 0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | <2 ---- | 3 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 0.6 | ---- 0.6 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 12 ---- | 10 ---- | ---- 0.5 | ---- 0.6 | ---- <0.1 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- <0.1 | ---- 0.4 | 8 ---- | 8 ---- | <2 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | <2 ---- | 7 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.7 | ---- 0.7 | <2 ---- | <2 ---- | <2 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 13 ---- | 13 ---- | ---- 0.6 | ---- 0.6 | ---- 0.5 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 0.5 | ---- 1.1 | 9 ---- | 9 ---- | 8 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 10 ---- | 20 ---- | ---- 0.6 | ---- 0.5 | ---- 0.5 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | ---- 2.16 | ---- 1.90 | 9 0.110 | 8 0.260 | 8 <0.050 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 35.1 | 28.6 | 15.5 | 16.7 | 3.62 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 25.7 | 26.5 | 2.52 | 7.30 | 3.48 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 11:40 | 02-Sep-2022 11:55 | 02-Sep-2022 11:55 | 02-Sep-2022 11:55 | 02-Sep-2022 11:55 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2217582-016 | EM2217582-017 | EM2217582-018 | EM2217582-019 | EM2217582-020 |
|  |  |  | Result <0.1 | Result 0.6 | Result 0.1 | Result 0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 <0.1 | 10 0.4 | 2 0.3 | 2 0.3 | <2 0.5 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | <2 <0.1 | 7 <0.1 | 4 <0.1 | 4 <0.1 | 9 <0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.2 | <2 1.0 | <2 0.4 | <2 0.4 | <2 0.6 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 4 0.2 | 17 1.0 | 6 0.3 | 6 0.4 | 10 0.6 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | 5 <0.050 | 17 3.21 | 5 1.07 | 6 0.860 | 11 0.930 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 1.26 | 159 | 36.3 | 30.0 | 23.2 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 1.15 | 88.0 | 32.8 | 28.0 | 14.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG06N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 12:08 | 02-Sep-2022 12:08 | 02-Sep-2022 12:08 | 02-Sep-2022 12:08 | 02-Sep-2022 10:40 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2217582-021 | EM2217582-022 | EM2217582-023 | EM2217582-024 | EM2217582-025 |
|  |  |  | Result <0.1 | Result 0.3 | Result <0.1 | Result <0.1 | Result 0.3 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.4 | 6 0.3 | <2 0.2 | <2 <0.1 | 5 0.3 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 5 0.1 | 4 0.4 | 2 0.1 | <2 0.2 | 5 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 2 0.4 | 6 0.6 | <2 0.1 | 4 0.1 | <2 0.6 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 6 0.5 | 10 1.0 | <2 0.2 | <2 0.3 | 10 0.7 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | 8 6.94 | 16 1.54 | 2 <0.050 | 5 0.360 | 11 7.88 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 12.1 | 78.7 | 0.400 | 43.2 | 46.3 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 14.2 | 35.0 | 0.200 | 8.77 | 29.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06E | WVDG06S | WVDG06W | WVDD01 Soluble metals | WVDD02 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 10:40 | 02-Sep-2022 10:40 | 02-Sep-2022 10:40 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2217582-026 | EM2217582-027 | EM2217582-028 | EM2217582-029 | EM2217582-030 |
|  |  |  | Result 0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 <0.1 | <2 <0.1 | <2 0.2 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | <2 <0.1 | <2 0.1 | 3 <0.1 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.1 | <2 0.1 | <2 0.2 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | <2 0.1 | 2 0.2 | 3 0.2 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | 2 0.580 | 3 0.490 | 4 0.270 | ---- 3.36 | ---- 53.6 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 14.2 | 18.0 | 13.6 | 133 | 866 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 20.0 | 17.4 | 11.1 | 201 | 3130 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble metals | WVDD04 Soluble metals | WVDD05 Soluble metals | WVDD06 Soluble metals | WVDD007BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2217582-031 | EM2217582-032 | EM2217582-033 | EM2217582-034 | EM2217582-035 |
|  |  |  | Result 0.160 | Result 80.2 | Result 192 | Result <0.050 | Result 53.7 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 208 | 268 | 564 | 332 | 930 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 607 | 686 | 1840 | 259 | 2310 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble metals | KF2 Soluble metals | KF3 Soluble metals | KF4 Soluble metals | KF5BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2217582-036 | EM2217582-037 | EM2217582-038 | EM2217582-039 | EM2217582-040 |
|  |  |  | Result 23.8 | Result 4.52 | Result 132 | Result 2.51 | Result <0.050 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 93.0 | 96.4 | 334 | 79.7 | 70.2 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 171 | 126 | 787 | 109 | 239 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N Soluble metals | WVDG03E Soluble metals | WVDG03S Soluble metals | WVDG03W Soluble metals | WVDG04N Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2217582-041 | EM2217582-042 | EM2217582-043 | EM2217582-044 | EM2217582-045 |
|  |  |  | Result <0.050 | Result 0.280 | Result 0.360 | Result 0.280 | Result 1.80 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 7.31 | 12.1 | 21.8 | 6.33 | 38.1 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 1.39 | 6.74 | 9.54 | 5.18 | 22.7 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E Soluble metals | WVDG04S Soluble metals | WVDG04W Soluble metals | WVDG05N Soluble metals | WVDG05E Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2217582-046 | EM2217582-047 | EM2217582-048 | EM2217582-049 | EM2217582-050 |
|  |  |  | Result 1.02 | Result 1.04 | Result 0.610 | Result 0.080 | Result 2.99 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 13.3 | 11.5 | 12.1 | 17.5 | 41.2 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 19.7 | 12.7 | 11.2 | 10.3 | 46.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble metals | WVDG05W Soluble metals | WVDG06N Soluble metals | WVDG06E Soluble metals | WVDG06S Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 | 02-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2217582-051 | EM2217582-052 | EM2217582-053 | EM2217582-054 | EM2217582-055 |
|  |  |  | Result 0.070 | Result 0.210 | Result 1.51 | Result 0.420 | Result 0.610 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 17.4 | 23.9 | 10.1 | 8.21 | 7.71 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 10.1 | 13.1 | 19.6 | 15.9 | 15.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06W Soluble metals | ---- | ---- | ---- | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 02-Sep-2022 00:00 | ---- | ---- | ---- | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2217582-056 | -------- | -------- | -------- | -------- |
|  |  |  | Result 0.260 | ---- ---- | ---- ---- | ---- ---- | ---- ---- |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 4.94 | ---- | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 8.27 | ---- | ---- | ---- | ---- |

Environmental

CERTIFICATE OF ANALYSIS

Work Order	: EM2219329

Page	: 1 of 19

Client

Contact

Address

: ALS WATER AND HYDROGRAPHICS PTY LTD

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Bronwyn Sheen

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +6138549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 04-Oct-2022 10:40 |
| Order number | : ---- | Date Analysis Commenced | : 06-Oct-2022 |
| C-O-C number Sampler Site | : ---- : R. Oliver : ALS KERANG DUST | Issue Date | : 09-Nov-2022 11:45 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Arenie Vijayaratnam	Senior Inorganic Chemist	Melbourne Inorganics, Springvale, VIC

Eric Chau	Metals Team Leader	Melbourne Inorganics, Springvale, VIC

R I G H T  S O L U T I O N S | R I G H T  P A R T N E R

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- EA120-EA142: EM2219329 #2,6; Samples were visually observed to be turbid.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- Sampling Period: 02/09/22 - 30/09/22.

- Sample exposure period is XX days which is outside the typical exposure period of 30+/-2 days as per AS3580.10.1

- EA120I-EA142I: EM2219329 #14; Sample container was received with no liquid. 100ml of DI water was used to dissolve contents, perform analysis and for calculation of results.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 11:05 | 30-Sep-2022 11:20 | 30-Sep-2022 11:30 | 30-Sep-2022 11:45 | 30-Sep-2022 12:00 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2219329-001 | EM2219329-002 | EM2219329-003 | EM2219329-004 | EM2219329-005 |
|  |  |  | Result 0.3 | Result 3.1 | Result 0.2 | Result 0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 5 0.9 | 51 7.8 | 3 1.1 | <2 0.4 | <2 0.2 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 15 0.4 | 128 10.6 | 19 3.6 | 6 2.4 | 3 5.0 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 7 1.2 | 175 10.9 | 59 1.3 | 39 0.5 | 82 0.2 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 20 1.6 | 179 21.5 | 22 4.9 | 8 2.9 | 3 5.2 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 27 3.21 | 354 28.3 | 81 2.81 | 47 3.49 | 85 <0.92 |
| ø Barium 7440-39-3 | 0.05 | µg | 105 | ---- | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | <1110 | <94.7 | 83.0 | <39.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 60.8 | 810 | 81.1 | 87.3 | <29.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD06 | WVDD007BG | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 10:40 | 30-Sep-2022 10:10 | 30-Sep-2022 13:20 | 30-Sep-2022 13:05 | 30-Sep-2022 13:10 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2219329-006 | EM2219329-007 | EM2219329-008 | EM2219329-009 | EM2219329-010 |
|  |  |  | Result 4.0 | Result 4.6 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 67 8.7 | 76 7.8 | <2 0.4 | <2 0.8 | <2 0.8 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 142 15.9 | 129 8.6 | 7 1.4 | 14 0.7 | 13 1.7 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 263 12.7 | 142 12.4 | 23 0.5 | 12 0.8 | 29 0.8 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 209 28.6 | 205 21.0 | 8 1.9 | 14 1.5 | 14 2.5 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 472 49.6 | 347 35.0 | 31 <4.99 | 26 0.60 | 43 <21.0 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <2070 | <1160 | <29.9 | 8.14 | <59.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <1470 | 2090 | <24.3 | 7.60 | 49.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 12:55 | 30-Sep-2022 12:50 | 30-Sep-2022 11:30 | 30-Sep-2022 11:30 | 30-Sep-2022 11:30 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2219329-011 | EM2219329-012 | EM2219329-013 | EM2219329-014 | EM2219329-015 |
|  |  |  | Result <0.1 | Result <0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | <2 ---- | <2 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 0.2 | ---- 0.7 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 4 ---- | 11 ---- | ---- 0.1 | ---- 0.8 | ---- 0.9 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 1.8 | ---- 0.8 | 2 ---- | 12 ---- | 13 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 30 ---- | 14 ---- | ---- <0.1 | ---- <0.1 | ---- 0.2 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.2 | ---- 0.7 | <2 ---- | <2 ---- | 2 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 4 ---- | 11 ---- | ---- 0.1 | ---- 0.8 | ---- 0.9 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 2.0 | ---- 1.5 | 2 ---- | 12 ---- | 13 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 34 ---- | 25 ---- | ---- 0.1 | ---- 0.8 | ---- 1.1 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- 5.50 | ---- 0.91 | 2 0 | 13 0 | 15 0.12 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <43.6 | 21.8 | 32.9 | 14.3 | 17.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 27.5 | 18.5 | 10.2 | 8.91 | 18.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 11:30 | 30-Sep-2022 11:45 | 30-Sep-2022 11:45 | 30-Sep-2022 11:45 | 30-Sep-2022 11:45 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2219329-016 | EM2219329-017 | EM2219329-018 | EM2219329-019 | EM2219329-020 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.1 | <2 0.3 | <2 0.2 | <2 0.4 | <2 0.4 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.1 | 4 0.2 | 2 0.3 | 5 <0.1 | 6 0.3 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.1 | 3 0.3 | 5 0.2 | <2 0.4 | 4 0.4 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 2 0.2 | 4 0.5 | 3 0.5 | 5 0.4 | 6 0.7 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 3 0.11 | 7 <0.82 | 8 0 | 5 0 | 10 <0.23 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 23.0 | <31.7 | 13.8 | 14.0 | 15.4 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 15.0 | 20.6 | 14.1 | 13.9 | 14.4 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG06N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 12:00 | 30-Sep-2022 12:00 | 30-Sep-2022 12:00 | 30-Sep-2022 12:00 | [30-Sep-2022] |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2219329-021 | EM2219329-022 | EM2219329-023 | EM2219329-024 | EM2219329-025 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result 0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.2 | <2 0.1 | <2 <0.1 | <2 <0.1 | <2 0.1 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 2 0.1 | 2 0.1 | <2 <0.1 | <2 0.3 | 2 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.2 | 2 0.1 | <2 0.1 | 4 0.1 | <2 0.1 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 3 0.3 | 2 0.2 | 2 0.1 | 2 0.4 | 2 0.2 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 4 0.65 | 4 0.11 | 2 <0.31 | 6 0.16 | 3 0 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 18.8 | <20.5 | <26.1 | <22.1 | <16.8 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 14.2 | <18.1 | <21.8 | 21.7 | <17.7 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06E | WVDG06S | WVDG06W | WVDD01 soluble metals | WVDD02 soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 10:40 | 30-Sep-2022 10:40 | 30-Sep-2022 10:40 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2219329-026 | EM2219329-027 | EM2219329-028 | EM2219329-029 | EM2219329-030 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 <0.1 | <2 0.1 | <2 1.1 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.2 | 2 0.1 | 16 0.1 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 2 <0.1 | <2 0.1 | <2 1.1 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | <2 0.2 | 2 0.2 | 16 1.2 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 2 0 | 3 0.13 | 17 0 | ---- 0 | ---- 62.8 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <20.6 | 21.9 | <40.4 | 234 | 1230 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <22.3 | 25.0 | <16.6 | 352 | 5070 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 soluble metals | WVDD04 soluble metals | WVDD05 soluble metals | WVDD06 soluble metals | WVDD007BG soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-031 | EM2219329-032 | EM2219329-033 | EM2219329-034 | EM2219329-035 |
|  |  |  | Result 0.65 | Result 55.3 | Result 0 | Result 166 | Result 38.5 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 1950 | 550 | 285 | 2930 | 531 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 1030 | 1210 | 433 | 6080 | 1220 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 soluble metals | KF2 soluble metals | KF3 soluble metals | KF4 soluble metals | KF5BG soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-036 | EM2219329-037 | EM2219329-038 | EM2219329-039 | EM2219329-040 |
|  |  |  | Result 0 | Result 0 | Result 0 | Result 0 | Result 0 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 92.4 | 66.5 | 87.9 | 169 | 75.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 182 | 113 | 200 | 148 | 135 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N soluble metals | WVDG03E soluble metals | WVDG03S soluble metals | WVDG03W soluble metals | WVDG04N soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-041 | EM2219329-042 | EM2219329-043 | EM2219329-044 | EM2219329-045 |
|  |  |  | Result 0 | Result 0 | Result 1.58 | Result 0.05 | Result 7.27 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 7.60 | 13.0 | 7.98 | 5.23 | 15.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 7.33 | 14.3 | 27.4 | 6.30 | 47.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E soluble metals | WVDG04S soluble metals | WVDG04W soluble metals | WVDG05N soluble metals | WVDG05E soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-046 | EM2219329-047 | EM2219329-048 | EM2219329-049 | EM2219329-050 |
|  |  |  | Result 1.48 | Result 0 | Result 0.93 | Result 0.17 | Result 0.37 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 36.2 | 3.18 | 12.8 | 5.36 | 6.15 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 52.6 | 14.1 | 31.3 | 5.01 | 7.49 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S soluble metals | WVDG05W soluble metals | WVDG06N soluble metals | WVDG06E soluble metals | WVDG06S soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-051 | EM2219329-052 | EM2219329-053 | EM2219329-054 | EM2219329-055 |
|  |  |  | Result 0 | Result 0 | Result 0 | Result 0 | Result 0.37 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 3.77 | 5.01 | 4.53 | 10.2 | 12.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 4.25 | 10.4 | 7.41 | 23.5 | 25.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06W soluble metals | WVDD01 insoluble metals | WVDD02 insoluble metals | WVDD03 insoluble metals | WVDD04 insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-056 | EM2219329-057 | EM2219329-058 | EM2219329-059 | EM2219329-060 |
|  |  |  | Result 0.11 | Result 5.43 | Result 38.2 | Result 2.90 | Result 7.69 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 3.97 | 132 | <1110 | <94.7 | 109 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 10.1 | 87.2 | 1022 | 85.6 | 108 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD05 insoluble metals | WVDD06 insoluble metals | WVDD007BG insoluble metals | KF1 insoluble metals | KF2 insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-061 | EM2219329-062 | EM2219329-063 | EM2219329-064 | EM2219329-065 |
|  |  |  | Result <0.92 | Result 50.6 | Result 37.6 | Result <4.99 | Result 7.05 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <39.7 | <2070 | <1160 | <29.9 | 25.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <29.6 | <1470 | 2180 | <24.3 | 29.0 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 insoluble metals | KF4 insoluble metals | KF5BG insoluble metals | WVDG03N insoluble metals | WVDG03E insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-066 | EM2219329-067 | EM2219329-068 | EM2219329-069 | EM2219329-070 |
|  |  |  | Result <21.0 | Result 9.03 | Result 1.25 | Result 0.11 | Result 0 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <59.0 | <43.6 | 27.6 | 84.6 | 22.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 59.7 | 29.2 | 78.8 | 28.1 | 16.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S insoluble metals | WVDG03W insoluble metals | WVDG04N insoluble metals | WVDG04E insoluble metals | WVDG04S insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-071 | EM2219329-072 | EM2219329-073 | EM2219329-074 | EM2219329-075 |
|  |  |  | Result 0.27 | Result 0.12 | Result <0.82 | Result 0 | Result 0 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 24.6 | 23.3 | <31.7 | 22.1 | 17.4 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 35.0 | 21.6 | 21.1 | 16.6 | 22.4 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W insoluble metals | WVDG05N insoluble metals | WVDG05E insoluble metals | WVDG05S insoluble metals | WVDG05W insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-076 | EM2219329-077 | EM2219329-078 | EM2219329-079 | EM2219329-080 |
|  |  |  | Result <0.23 | Result 1.32 | Result 0.23 | Result <0.31 | Result 0.28 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 18.4 | 18.9 | <20.5 | <26.1 | <22.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 20.1 | 15.7 | <18.1 | <21.8 | 28.7 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06N insoluble metals | WVDG06E insoluble metals | WVDG06S insoluble metals | WVDG06W insoluble metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | 30-Sep-2022 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2219329-081 | EM2219329-082 | EM2219329-083 | EM2219329-084 | -------- |
|  |  |  | Result 0 | Result 0 | Result 0.39 | Result 0 | ---- ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <16.8 | <20.6 | 35.3 | <40.4 | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <17.7 | <22.3 | 109 | <16.6 | ---- |