Department of Jobs, Precincts and Regions

kangaroo flat dust monitoring report q1 2022

30 MAY 2022

CERTIFICATE OF APPROVAL FOR ISSUE OF DOCUMENTS

Client:	Department of Jobs, Precincts and Regions

Project Title:	Kangaroo Flat Dust Monitoring Report Q1 2022

Title:		Kangaroo Flat Dust Monitoring Report Q1 2022

Project: 	MV214940

Document Status:	Draft

Date of Issue:		 30/05/2022

|  | Position | Name | Initials | Date |
| --- | --- | --- | --- | --- |
| Prepared by: | Northern Victorian Regional Manager | Rohan Oliver | RO | 16/05/2022 |
| Technical Review by: | Victorian Data Manager | Karen Bootland | KB | 16/05/2022 |
| Client: | Department of Jobs, Precincts and Regions | Earth Resources Regulation | ERR | 30/05/2022 |

For further information on this report, contact:

Name:		Rohan Oliver
Title:		Northern Victorian Regional Manager
Address:	94 Koondrook Rd, Kerang VIC 3579
Mobile:		+61 4 3664 4539
E-mail:		Rohan.Oliver@alsglobal.com

© ALS Water and Hydrographics Pty Ltd

This report is prepared for the Client, Department of Jobs, Precincts and Regions, for the purpose of monitoring dust at the Kangaroo Flat Mine site which is currently in care and maintenance. An unrestricted copy of this report will be shared with the public via the Earth Resources Regulation website.

Disclaimer

This document has been prepared for the Client above and is to be used only for the purposes for which it was commissioned. While all due skill and attention has been taken in collecting, validating and providing the data contained in this report, ALS shall not be liable in any way for loss of any kind including damages, costs, interest, loss of profits or special loss or damage, arising from any error, inaccuracy, incompleteness or other defect in this information. No warranty is given as to its suitability for any other purpose.

ALS Water and Hydrographics Pty Ltd ABN 94 105 060 320

ALS Australia Pty Ltd trading as ALS Hydrographics

Contents

Executive Summary	4

1	Introduction	6

2	Background	6

2.1	Scope and Objective	6

2.2	Regulatory Environment	7

2.3	What is Dust	8

3	Measurement of Dust	9

3.1	Site Locations	9

3.2	Deposited Matter	11

3.3	Metals/Metalloids Monitoring in Dust	12

3.4	Ash Content	12

3.5	Dust Monitoring Standards	13

3.6	Dust Monitoring Program	13

4	Dust Deposition Results	14

4.1	Kangaroo Flat Mine – Q1 2022	14

4.1.1 Deposition of Total Insoluble Matter	14

4.1.2 Mineral Content in Dust	15

4.1.3 Metals/Metalloids in Dust	16

4.2	Quality Assurance and Quality Control	18

5	Summary of Results	19

6	Further Considerations	19

7	References	20

8	Tables	21

9	Supplementary Data	24

9.1	Arsenic	24

9.2	Barium	26

9.3	Manganese	28

9.4	Trend Graphs	30

Appendix A – Supplementary Information	34

Appendix B – Laboratory Reports	37

Table of Figures

Figure 1 - Kangaroo Flat - Monitoring Locations	9

Figure 2 - Kangaroo Flat - Control Site Location KF5 (BG)	10

Figure 3 - Kangaroo Flat - Control Site Gauge KF5 (BG)	10

Figure 4 - Kangaroo Flat - Dust Deposition Gauge KF1 (W)	12

Figure 5 - Kangaroo Flat - Total Insoluble Matter (TIM) - Q1 2022	14

Figure 6 - Kangaroo Flat - Total Ash Content - Q1 2022	15

Figure 7 - Kangaroo Flat - Ash Content/TIM Ratio - Q1 2022	15

Figure 8 - Kangaroo Flat - Total Arsenic/Total Solids - Q1 2022	16

Figure 9 - Kangaroo Flat - Arsenic in Soluble Fraction - Q1 2022	24

Figure 10 - Kangaroo Flat - Arsenic in Insoluble Fraction - Q1 2022	24

Figure 11 - Kangaroo Flat - Arsenic in Soluble Fraction - 2021-2022	25

Figure 12 - Kangaroo Flat - Arsenic in Insoluble Fraction - 2021-2022	25

Figure 13 - Kangaroo Flat - Barium in Soluble Fraction - Q1 2022	26

Figure 14 - Kangaroo Flat - Barium in Insoluble Fraction - Q1 2022	26

Figure 15 - Kangaroo Flat - Barium in Soluble Fraction - 2021-2022	27

Figure 16 - Kangaroo Flat - Barium in Insoluble Fraction - 2021-2022	27

Figure 17 - Kangaroo Flat - Manganese in Soluble Fraction - Q1 2022	28

Figure 18 - Kangaroo Flat - Manganese in Insoluble Fraction - Q1 2022	28

Figure 19 - Kangaroo Flat - Manganese in Soluble Fraction - 2021-2022	29

Figure 20 - Kangaroo Flat - Manganese in Insoluble Fraction - 2021-2022	29

Figure 21 - Kangaroo Flat - Total Insoluble Matter (TIM) – 2021-2022	30

Figure 22 - Kangaroo Flat - Total Ash Content - 2021-2022	31

Figure 23 - Kangaroo Flat - Ash Content/TIM Ratio - 2021-2022	32

Figure 24 - Kangaroo Flat - Total Arsenic/Total Solids - 2021-2022	33

Table of Tables

Table 1 - Kangaroo Flat - Dust Deposition Data	21

# Executive Summary

ALS was engaged by the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the former Kangaroo Flat Mine from the start of January 2021. This report provides results and analysis of data collected from sampling in Q1 2022 for the Kangaroo Flat Mine site.

In April 2021, Earth Resources Regulation (ERR) inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its operator, Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust.

This report analyses the data gathered in Q1 2022 for the Kangaroo Flat Mine site by comparing the levels to accepted standards (where available) and previous dust monitoring results. The results of dust monitoring in the first half of 2021 were published in a report by CDM Smith. During Q3 and Q4, ALS completed the collection of samples/monitoring, analysis of lab results (Springvale ALS Laboratory - NATA accredited) and drafted the dust monitoring reports.

The Kangaroo Flat Mine site (including dust monitoring sites) is located approximately 4 km south from the centre of Bendigo. The Kangaroo Flat Mine site includes:

- fine tailings dam currently covered in dust suppression polymer

- course tailings dam which has been covered with waste rock

- two water dams which capture rainwater

- various other smaller ponds for stormwater management.

This site is currently under care and maintenance. To keep dust levels down, ERR is applying a range of dust control measures at the site, including applying water and dust suppression polymer and ensuring vegetation cover. Daily inspections during business days are carried out to make sure that dust levels are kept minimised.

To maintain continuity and comparability of data, monitoring was undertaken at the same locations, and a similar methodology and analysis process was followed in this report as done before.

The original scope of dust monitoring at Kangaroo Flat was to operate and maintain four depositional dust gauges. ERR has since asked ALS to add a further monitoring site for the purpose gathering background data. Data from this control site will be used for comparative background analysis.

The purpose of dust monitoring is to check that dust issues are being minimised and managed effectively including testing the effectiveness of dust control measures. Another objective of the dust monitoring program at Kangaroo Flat is to gather baseline data ahead of rehabilitation works. These results will inform decision making on dust control measures required during site rehabilitation, to ensure works meet air quality standards (where available).

The results for Q1 2022 were consistent to previous results recorded by ALS and published in the CDM Smith Report for the first half of 2021, ALS Reports for Q3 and Q4 2021.

As done previously in the CDM Smith Report, the following questions are answered for Q1 2022:

- Did the dust deposition rates recorded in Q1 2022, exceed the adopted dust deposition criteria of 4 g/m2/month, measured as Total Insoluble Matter (TIM)?

In Q1 2022, there was no instance where the dust level exceeded over the 4 g/m2/month criteria at any of the monitoring sites.

- Are dust deposition rates recorded in Q1 2022, comparable to available historic results in this area?

Overall results for Q1 2022 were comparable to previous results (Kralcopic Report in 2018, CDM Smith Report for the first half of 2021, ALS Reports for Q3 and Q4 of 2021).

- Are arsenic concentrations recorded in deposited dust in Q1 2022, comparable to available historic results in this area?

Although there were higher readings for arsenic in the soluble fraction, when these readings were used to calculate the total arsenic concentration, the results were 96.6 mg/kg for KF1 in Jan-22 and 35.8 mg/kg for KF2 in Feb-22 which were both below the 100 mg/kg assessment level from the Contaminated Sites Guideline. These results however are higher than the background levels which is why it is important for the dust suppression polymer cover to be maintained at the fine tailings dam.

Barium and manganese were generally within the historical ranges previously recorded, with results being comparable and at times lower than 2021 results.

Continued monitoring at the newly introduced background monitoring location KF5 (BG) will provide additional data to which the source data can be compared to, in future reports. Furthermore, ERR has confirmed that the focus here is to keep the dust levels minimised, which is why the fines tailings dam has been fully covered with dust suppression polymer.

# Introduction

ALS was engaged by the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the former Kangaroo Flat Mine site from the start of January 2021. This report provides results and analysis of data collected from sampling in Q1 2022. ALS undertook the field component of maintaining dust gauges and replacing sample containers, as well as laboratory analysis of samples collected. Samples were collected on a routine monthly basis from locations specified by DJPR. These locations are displayed in Section 3.1, Figure 1 and Figure 2 of this report.

# Background

In April 2021, ERR inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its operator, Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust. This report analyses the data gathered in Q1 2022 for the Kangaroo Flat site by comparing the levels to accepted standards (where available) and previous dust monitoring results published in:

- ALS report for Q4 2021;

- ALS report for Q3 2021;

- CDM Smith report for the first half of 2021; and

- Kralcopic report for Q4 2018.

The reports listed above are referenced in this document. To maintain continuity and comparability of results, this document has been prepared in a similar format to the CDM Smith report.

The Kangaroo Flat Mine is located to the east of Kangaroo Flat, approximately 4 km south from the centre of Bendigo. The site includes decommissioned mine tunnels and shafts as well as processing plants and tailings dams. The site is currently in care and maintenance. To keep dust levels down, ERR is applying a range of dust control measures at the former the Kangaroo Flat Mine site, including applying water and dust suppression polymer and ensuring vegetation cover. Daily inspections carried out during business days ensure dust levels are minimised.

To maintain continuity and comparability of data, monitoring was undertaken at the same locations as done previously, and similar methodology and analysis used. This report builds on the previous results and provides baseline information to potentially inform decisions around some aspects of rehabilitation.

## Scope and Objective

The scope of dust monitoring at Kangaroo Flat is to operate and maintain five depositional dust gauges.

All depositional dust gauges except for the background gauge KF5 (BG) are located at their historic locations with the following naming conventions since the dates specified below:

| - KF1 (W) Jan 2021 - KF2 (E) Jan 2021 - KF5 (BG) Sep 2021 | - KF3 (N) Jan 2021 - KF4 (S) Jan 2021 |
| --- | --- |

Laboratory analysis requirements were provided by DJPR and were in line with historical requirements during the previous monitoring program to maintain continuity and comparability of results.

The purpose of dust monitoring is to check that dust issues are being minimised and managed effectively including testing the effectiveness of dust control measures. Another objective of the dust monitoring program at Kangaroo Flat is to gather baseline data ahead of rehabilitation works. These results will inform decision making on dust control measures required during site rehabilitation to ensure works meet air quality standards (where available).

As done previously, the following considerations were included in the assessment of Q1 2022 data:

- Did the dust deposition rates recorded in Q1 2022, exceed the adopted dust deposition criteria of 4 g/m2/month, measured as Total Insoluble Matter (TIM)?

- Are dust deposition rates recorded in Q1 2022, comparable to available historic results in this area?

- Are arsenic concentrations recorded in deposited dust in Q1 2022, comparable to available historic results in this area?

## Regulatory Environment

The Environment Protection Authority (EPA) has overarching responsibility for the air quality in Victoria. The EPA administers the Environment Protection Act 2017 and the Environment Protection Regulations 2021 under which Environmental Reference Standards (ERS) and Guidelines are provided.

The purpose of the ERS is to support the protection of human health and the environment from pollution and waste by providing benchmarks to assess and report on environmental conditions in the whole or any part of Victoria.

The ERS seeks to achieve this purpose by:

- identifying environmental values to be achieved or maintained in the whole or any part of Victoria; and

- specifying indicators and objectives to be used to measure, determine or assess whether those environmental values are being achieved, maintained or threatened.

The ERS provides guidance, objectives and levels on values where the environment may be impacted and incorporates the Ambient Air Quality National Environment Protection Measure (NEPM AAQ).

The main guidance document used for the purpose of this report is EPA Publication 1191 – Protocol for Environmental Management – Mining and Extractive Industries (Mining PEM). The criteria in this guidance used for this report has been carried over to the EPA’s latest Guideline for Assessing and Minimising Air Pollution in Victoria, Publication 1961, February 2022.

Under the new EPA legislation, duty holders have responsibility under the general environmental duty to apply controls to eliminate or minimise risks to human health and the environment. This means that the primary focus is on applying effective elimination, minimisation and control measures. Monitoring is for the purpose of determining the effectiveness of control measures.

## 2.3	What is Dust

Dust is not typically classified according to its composition, but instead on its particle size, as follows:

- Deposited matter - any particles that fall out of suspension in the atmosphere.

- Total Suspended Particles (TSP) - particles suspended or entrained in the air. Typically, this is particles of 30 µm (0.03 mm) equivalent aerodynamic diameter or less. Larger particles tend not to become suspended.

- PM10 - particles 10 µm equivalent aerodynamic diameter or less.

- PM2.5 - particles 2.5 µm equivalent aerodynamic diameter or less.

Dust particle size is an important consideration influencing dispersion and transport in the atmosphere and potential effects on human health. Human activities (e.g. energy use, transport, industrial activities etc) can affect the air quality, in particular airborne dust.

Potential sources of dust are:

- natural sources such as dust storms, agricultural dust, bushfires, vegetation, pollen and fungi; and

- anthropogenic sources such as mines sites, industry, roads and vehicles, construction sites, domestic and diffuse sources (CDM Smith, 2021).

# Measurement of Dust

## 3.1	Site Locations

Locations of depositional dust gauges around the Kangaroo Flat Mine are presented in Figure 1.

Figure 1 - Kangaroo Flat - Monitoring Locations

Location of background gauge and image are presented in Figure 2 and Figure 3 below.

Figure 2 - Kangaroo Flat - Control Site Location KF5 (BG)

Figure 3 - Kangaroo Flat - Control Site Gauge KF5 (BG)

Samples were collected in accordance with the Standard AS/NZS 3580.10.1 - Methods for sampling and analysis of ambient air – Determination of particulate matter – Deposited matter – Gravimetric method and ALS Dust Sampling and Depositional Dust Gauge Work Procedure MAT-MAP1000 (See Appendix A – Supplementary Information).

ALS Laboratory in Springvale analysed the samples for the following:

- Total deposited matter

- Total ash content

- Metals/metalloids, in the soluble, insoluble and ash content fraction of dust.

## 3.2	Deposited Matter

The Standard (AS/NZS 3580.10.1:2016) sets out a method (used in this report) for sampling particulate matter that is deposited from the atmosphere. This Standard also sets out procedures for the gravimetric (i.e. by weighing) determination of the mass deposition rate of insoluble/soluble solids, ash/combustible matter, and total solids from ambient air.

The method provides an estimate of the mean surface concentration of deposited matter settling from the air over a sampling period, typically of one month. Particulate matter deposition rates of 0.1 g/m2/month and above may be determined using a monthly sampling period. The sample obtained by the sampling procedure specified may be subjected to physical or chemical analysis.

Over a given sampling period, particles that settle from the ambient air are collected in a vessel and retained together with any rainwater. The sample is passed through a sieve to remove any extraneous matter (e.g. leaves, insects), and the sieved sample containing the deposited matter is transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of the dried insoluble solids is gravimetrically determined.

The ash and combustible matter content are determined by loss on ignition of the insoluble solids. Soluble solids are determined from the filtrate. The total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass deposition rate of deposited matter is then calculated from the mass of solids obtained, the funnel cross-sectional area and the exposure period. (AS/NZS 3580.10.1:2016)

Figure 4 - Kangaroo Flat - Dust Deposition Gauge KF1 (W)

## 3.3	Metals/Metalloids Monitoring in Dust

The inclusion of the analysis of metals/metalloids in the dust samples assist in identifying possible sources of contamination if present.

Analysis for insoluble matter, soluble matter and ash content was undertaken for the following:

- Arsenic

- Barium

- Manganese

It is understood that there are no specific threshold values for metals and metalloids in deposited dust either in the former Mining Licence or within the ERS. (CDM Smith, 2021)

## 3.4	Ash Content

Ash Content is the remaining material after the sample has been combusted in the laboratory. Ash content provides an indication of the mineral content (or soil dust) of the sample. The mineral content may be attributable to onsite contributions, but may also be attributed to other sources such as agriculture, unsealed roads, etc.

## 3.5	Dust Monitoring Standards

Results were compared against relevant dust deposition rate criteria described by EPA Publication 1191 (2007), as shown below which has been adopted in EPA Publication 1961 (2022):

The following points apply to the criteria:

- Results of monitoring should not exceed 4 g/m2/month (no more than 2 g/m2/month above background) as a monthly average.

- The 2 g/m2/month criteria are used when baseline data on deposited dust levels exist, while the 4 g/m2/month criteria is used when no baseline data exists.

- The criteria refer to all sources of deposited matter (including sources from mines, agriculture, unsealed roads, etc) and cumulative impacts.

- The criteria states that in some cases, a mine may increase deposited dust levels by up to 2 g/m2/month. However, the total deposited dust level (including sources from mines, agriculture, unsealed roads, etc) must not exceed 4 g/m2/month.

Deposited matter (dust) can be used as an indicator of the effectiveness of site management practices and the potential for offsite nuisance (fugitive dust). Deposited dust monitoring, when conducted over a set time period, is useful for examining trends and evaluating deviation from long term trends with respect to site activities.

## 3.6	Dust Monitoring Program

The current method of undertaking depositional dust monitoring is considered sufficient as the site is in care and maintenance i.e. no new pollution is being introduced to the site from industry. There is a considerable body of knowledge about the contamination level at the site, so all that is required at this stage is to determine whether this contamination is becoming air borne and becoming a problem to the nearby landholders. Depositional dust monitoring provides a range of useful information to be able to understand what’s happening at this site and its surrounds.

Depositional dust monitoring can provide the following invaluable information:

- test the effectiveness of dust minimisation, control and management measures

- identify key problematic sources, or groups of sources on larger more complex sites

- identify where dust sensitivities may occur

- characterise temporal or spatial trends

- metallic/metalloid concentrations useful for considering impacts on nearby community.

# Dust Deposition Results

## 4.1	Kangaroo Flat Mine – Q1 2022

Dust deposition analysis results are presented in Table 1 and laboratory results reports are attached in Appendix B, all at the end of this document.

Where results were reported as “less than” (<) a specified number (laboratory reporting limit) the number reported was adopted for use in the graphical displays. This approach adopts a conservative value for the number being reported.

## 4.1.1 Deposition of Total Insoluble Matter

Total Insoluble Matter (TIM) met the criteria of less than 4 g/m2/month on 15 of 15 measurements during Q1 2022. See Figure 5 below.

Throughout Q1 results, none of the readings exceeded the background plus 2 g/m2/month criteria. These readings verify the monitored sites are within the requirements for Mining PEM criteria.

Dust deposition rate at site KF5 (BG) was the highest value for the period during March 2022 recording a result of 2.4 g/m2/month. When the data from all source monitoring sites were averaged for Q1 2022, an average of 1.3 g/m2/month was recorded. This value is below the inferred background level suggested within the Mining PEM. Both exceedance criteria (4 g/m2/month and background plus 2 g/m2/month) have been considered when reviewing data sets. ALS found no exceedances were recorded during Q1, 2022.

Figure 5 - Kangaroo Flat - Total Insoluble Matter (TIM) - Q1 2022

Red dotted line is the criteria used for the purpose of this report.

## 4.1.2 Mineral Content in Dust

The fraction of dust measured as ash content is displayed below in Figure 6. The values recorded during Q1 were generally 0.2 g/m2/month or less. Slightly elevated readings at KF2 (E) (0.3 g/m2/month), KF4 (N) (0.4 g/m2/month) during January and KF3 (S) (0.3 g/m2/month) in February were recorded with these values at or below historical values.

Figure 6 - Kangaroo Flat - Total Ash Content - Q1 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

To gain a general understanding of what proportion of the sample is ash content i.e. mineral dust, the ratio of total ash content and TIM has been plotted in Figure 7.

Figure 7 - Kangaroo Flat - Ash Content/TIM Ratio - Q1 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 7 shows that ash content in the samples throughout Q1 2022 were generally low, with higher readings at KF4 (S) and KF2 (E) for January 2021 and KF3 (N) in February 2022. This shows that quite a proportion of the samples consisted of leafy biological content as opposed to mineral dust content. This is typical for areas where there are reasonable levels of vegetation in its surrounds.

Figure 23 in Section 9.4 shows that this explanation holds true for a greater part of the samples with a few exceptions recorded in wetter periods seen in June, July and September 2021 where the mineral dust content has dominated the sample.

## 4.1.3 Metals/Metalloids in Dust

Graphical presentations of soluble and insoluble metals (arsenic, barium and manganese) are provided in Section 9 of this report. Majority of arsenic, barium and manganese concentrations for Q1 2022 were consistent with previously recorded historical data sets. Historical data sets from 2021 have been reported and results were within historical ranges.

In the Q1 2022 results, ALS identified higher readings at KF1 in January 2022, KF2 in February 2022 for Arsenic. ALS field staff observed no major changes onsite that could have contributed to the higher results. Results also indicated a rise in the soluble fraction of barium in Jan-22 and Mar-22 for KF4 (S). Barium concentration thresholds are a couple of orders of magnitude higher than arsenic concentration thresholds in the Contaminated Sites Guidelines, so this is not a major concern.

To provide an understanding of total arsenic concentrations measured in dust relative to concentrations measured in soils within the region, total arsenic concentration has been displayed below in Figure 8 in mg/kg. This is calculated by adding soluble and insoluble components of arsenic in µg/m2/month and dividing by the total solids g/m2/month.

This ratio is being plotted because this provides a figure which could be compared to the figures quoted in the guideline, Contaminated Sites Management Series – Assessment Levels for Soil, Sediment and Water. The assessment level for soil for arsenic is 100 mg/kg for standard residential areas in this guideline.

Figure 8 - Kangaroo Flat - Total Arsenic/Total Solids - Q1 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Total arsenic concentrations for Q1 ranged from 1.3 to 96.6 mg/kg with a mean of 22.8 mg/kg across all source monitoring sites. This is well below the 100 mg/kg assessment level mentioned above and comparable to Q3 (20.9 mg/kg) and Q4 (26.4 mg/kg) mean total arsenic concentrations.

Overall, concentrations recorded for Q1 remained consistent with previous historical readings during 2021. See Figure 24 in Section 9.4.

There is no definitive pattern across the monitoring sites to indicate that the arsenic concentrations are from a single source. Highest concentrations through the period were observed at KF1 (W) (mean 35.8 mg/kg) and KF3 (N) (mean 23.6 mg/kg). These figures are still below the 100 mg/kg assessment level in the Contaminated Sites Guideline.

## 4.2	Quality Assurance and Quality Control

Field sampling and analysis was undertaken in accordance with the following Standards:

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

Sampling methodology and locations are consistent with sampling previously undertaken to maintain continuity and comparability of results.

ALS Environmental Laboratory has NATA accreditation for the following Ambient Air Parameters:

The NATA accreditation process involves confirming that the relevant Australian Standards are being followed in the company procedures. ALS has followed the relevant Australian Standards in sample gathering, analysis and reporting of results which is satisfactory. Nevertheless ALS is reviewing any gaps in their accreditation and will be rectifying these gaps in due course.

Internal laboratory quality control measures note the following:

- No Method Blank value outliers occurred.

- No laboratory control outliers occurred.

- No Matrix Spike (MS) or Matrix Spike Duplicate (MSD) results reported.

- No laboratory duplicate results reported.

- No holding time outliers occurred.

- No quality control sample frequency outliers occurred (based on NEPM 2013 B3 quality control specification).

# Summary of Results

Results are discussed in detail throughout the relevant sections in this report. In summary, the overall results for Q1 2022 were comparable to previous results or less than what was observed in 2021.

The key findings for Q1 2022 are as follows:

- There were no exceedances of the 4 g/m2/month (or the background plus 2 g/m2/month) criteria at depositional gauges monitoring the Kangaroo Flat mine in Q1 2022.

- Although there were higher readings for arsenic in the soluble fraction, when these readings were used to calculate the total arsenic concentration, the results were 96.6 mg/kg for KF1 in Jan-22 and 35.8 mg/kg for KF2 in Feb-22 which were both below the 100 mg/kg assessment level from the Contaminated Sites Guideline. These results however are higher than the background levels which is why it is important for the dust suppression polymer cover to be maintained at the fine tailings dam.

- Barium and manganese were generally within the historical ranges previously recorded, with results being comparable and at times lower than 2021 results.

# 6	Further Considerations

ERR has advised that they will continue to monitor the site for dust levels and ensure that a range of dust control measures are applied during this care and maintenance phase of the rehabilitation project to minimise dust nuisance. ERR will be endeavouring to make continuous improvements to dust suppression and monitoring from the findings of the dust monitoring work.

# 7	References

- ALS Hydrographics Pty Ltd (2021), Kangaroo Flat Dust Monitoring Q3 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2021), Kangaroo Flat Dust Monitoring Q4 2021 Report. (ALS Report)

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

- CDM Smith (2021), Kangaroo Flat Mine - Dust Monitoring Report - January to June 2021. (CDM Smith Report)

- Department of Conservation (2010), Contaminated Sites Management Series, Assessment Levels for Soil, Sediment and Water, February 2010. (Contaminated Sites Assessment Levels)

- EPA (Vic) Publication 1191 (2007), Protocol for Environmental Management - Mining and Extractive Industries. (Mining PEM)

- EPA Publication 1961 (2022), Guideline for Assessing and Minimising Air Pollution in Victoria, February 2022. (Assessing and Minimising Guideline)

- EPA Publication 440.1 (2002), A Guide to the Sampling and Analysis of Air Emissions and Air Quality, December 2002. (Sampling and Analysis Guideline)

- Kralcopic Pty Ltd (2018), Quarterly Environmental Monitoring Report – Kangaroo Flat Dust Monitoring, October to December 2018. (Kralcopic Report)

- National Environment Protection Council (1998), National Environment Protection (Ambient Air Quality) Measure, July 2018. (NEPM AAQ)

- Victorian Government (2001), Statement of Environment Protection Policy (Air Quality Management), Victorian Government Gazette, No S 240, 21 December 2001. (SEPP AQM)

- Victorian Government (2016), State Environment Protection Policy (Ambient Air Quality), Original Policy including Two Variations, Victorian Gazette Nos S19, S240 and G30, published on 9 February 1999, 21 December 2001 and 28 July 2016, respectively. (SEPP AAQ)

- Victorian Government (2021), Environment Reference Standard, under the Environment Protection Act 2017, Victorian Government Gazette, No. S 245, 26 May 2021. (ERS)

- EPA NSW Publication, Approved Methods for the Modelling and Assessment of Air Pollutants in NSW, (2017)

# 8	Tables

Table 1 - Kangaroo Flat - Dust Deposition Data

# 9	Supplementary Data

Graphs for Metals/Metalloids in Insoluble and Soluble Fractions including trends are presented below.

## 9.1	Arsenic

Figure 9 - Kangaroo Flat - Arsenic in Soluble Fraction - Q1 2022

Figure 10 - Kangaroo Flat - Arsenic in Insoluble Fraction - Q1 2022

Figure 11 - Kangaroo Flat - Arsenic in Soluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 12 - Kangaroo Flat - Arsenic in Insoluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## 9.2	Barium

Figure 13 - Kangaroo Flat - Barium in Soluble Fraction - Q1 2022

Figure 14 - Kangaroo Flat - Barium in Insoluble Fraction - Q1 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 15 - Kangaroo Flat - Barium in Soluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 16 - Kangaroo Flat - Barium in Insoluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## 9.3	Manganese

Figure 17 - Kangaroo Flat - Manganese in Soluble Fraction - Q1 2022

Figure 18 - Kangaroo Flat - Manganese in Insoluble Fraction - Q1 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 19 - Kangaroo Flat - Manganese in Soluble Fraction - 2021-2022

Figure 20 - Kangaroo Flat - Manganese in Insoluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## 9.4	Trend Graphs

Figure 21 - Kangaroo Flat - Total Insoluble Matter (TIM) – 2021-2022

Figure 21 shows TIM from throughout 2021 and no exceedances in the first quarter of 2022. The chart shows no exceedances of the recommended criteria level occurred during this period with values being recorded generally similar for Q1 2022 as with other quarters in 2021. Overall, average value of TIM for Q1 at the source is aligned with the rest of the year’s data with the average being at 1.3 g/m2/month.

Figure 22 - Kangaroo Flat - Total Ash Content - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 22 provides graphical summary of total ash content throughout 2021 and the first quarter of 2022. As can be seen from this graph, most of the data for Q1 2022 was below 0.3 g/m2/month with only KF4 (N) recording a higher figure of 0.4 g/m2/month. All total ash content levels recorded for Q1 2022 were within acceptable levels and comparable to recorded values throughout 2021.

Figure 23 - Kangaroo Flat - Ash Content/TIM Ratio - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 24 - Kangaroo Flat - Total Arsenic/Total Solids - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

By comparing the graphs in Figure 23 and Figure 24 above, it can be concluded that these results for Q1 2022 are trending in line with previous results. Results recorded for Q1 2022 were overall similar to the values recorded in 2021.

# Appendix A – Supplementary Information

# Appendix B – Laboratory Reports