Department of Energy, Environment and Climate Action

Q4 October-December 2022

30th June 2023

CERTIFICATE OF APPROVAL FOR ISSUE OF DOCUMENTS

Client:	Department of Energy, Environment and Climate Action

Project Title:	Kangaroo Flat Dust Monitoring Report Q4 2022

Title:		Kangaroo Flat Dust Monitoring Report Q4 2022

Project: 	MV214940

Document Status:	Version 1 Final

Date of Issue:		 30/06/2023

|  | Position | Name | Initials | Date |
| --- | --- | --- | --- | --- |
| Prepared by: | Northern Victorian Regional Manager | Rohan Oliver | RO | 20/02/2023 |
| Technical Review by: | Victorian Data Manager | Karen Bootland | KB | 30/06/2023 |
| Client: | Department of Energy, Environment and Climate Action | Michael Fitzgerald | DEECA |  |

For further information on this report, contact:

Name:		Rohan Oliver
Title:		Northern Victorian Regional Manager
Address:	94 Koondrook Rd, Kerang VIC 3579
Mobile:		+61 4 3664 4539
E-mail:		Rohan.Oliver@alsglobal.com

© ALS Water and Hydrographics Pty Ltd

This report is prepared for the Client, Department of Energy, Environment and Climate Action, for the purpose of monitoring dust at the Kangaroo Flat Mine site which is currently in care and maintenance. An unrestricted copy of this report will be shared with the public via the Earth Resources Regulation website.

Disclaimer

This document has been prepared for the Client above and is to be used only for the purposes for which it was commissioned. While all due skill and attention has been taken in collecting, validating and providing the data contained in this report, ALS shall not be liable in any way for loss of any kind including damages, costs, interest, loss of profits or special loss or damage, arising from any error, inaccuracy, incompleteness or other defect in this information. No warranty is given as to its suitability for any other purpose.

ALS Water and Hydrographics Pty Ltd ABN 94 105 060 320

ALS Australia Pty Ltd trading as ALS Hydrographics

Table of Contents

Executive Summary	4

1	Introduction	6

2	Background	6

2.1	Scope and Objectives	7

2.2	Regulatory Environment	7

2.3	What is Dust	8

3	Measurement of Dust	9

3.1	Site Locations	9

3.2	Deposited Matter	11

3.3	Metals/Metalloids Monitoring in Dust	12

3.4	Ash Content	12

3.5	Dust Monitoring Standards	13

3.6	Dust Monitoring Program	13

4	Dust Deposition Results	14

4.1	Kangaroo Flat Mine – Q4 2022	14

4.1.1 Deposition of Total Insoluble Matter	14

4.1.2 Mineral Content in Dust	15

4.1.3 Metals/Metalloids in Dust	16

4.2	Quality Assurance and Quality Control	18

5	Summary of Results	19

6	Further Considerations	19

7	References	20

8	Tables	21

9	Supplementary Data	25

9.1	Arsenic	25

9.2	Barium	27

9.3	Manganese	29

9.4	Trend Graphs	31

Appendix A - Supplementary Information	35

Appendix B - Laboratory Reports	38

Table of Figures

Figure 1 - Kangaroo Flat - Monitoring Locations	9

Figure 2 - Kangaroo Flat - Control Site Location KF5 (BG)	10

Figure 3 - Kangaroo Flat - Control Site Gauge KF5 (BG)	10

Figure 4 - Kangaroo Flat - Dust Deposition Gauge KF1 (W)	12

Figure 5 - Kangaroo Flat - Total Insoluble Matter (TIM) – Q4 2022	15

Figure 6 - Kangaroo Flat - Total Ash Content – Q4 2022	15

Figure 7 - Kangaroo Flat - Ash Content/TIM Ratio – Q4 2022	16

Figure 8 - Kangaroo Flat - Total Arsenic/Total Solids – Q4 2022	17

Figure 9 - Kangaroo Flat - Arsenic in Soluble Fraction – Q4 2022	25

Figure 10 - Kangaroo Flat - Arsenic in Insoluble Fraction – Q4 2022	25

Figure 11 - Kangaroo Flat - Arsenic in Soluble Fraction - 2021-2022	26

Figure 12 - Kangaroo Flat - Arsenic in Insoluble Fraction - 2021-2022	26

Figure 13 - Kangaroo Flat - Barium in Soluble Fraction – Q4 2022	27

Figure 14 - Kangaroo Flat - Barium in Insoluble Fraction – Q4 2022	27

Figure 15 - Kangaroo Flat - Barium in Soluble Fraction - 2021-2022	28

Figure 16 - Kangaroo Flat - Barium in Insoluble Fraction - 2021-2022	28

Figure 17 - Kangaroo Flat - Manganese in Soluble Fraction – Q4 2022	29

Figure 18 - Kangaroo Flat - Manganese in Insoluble Fraction – Q4 2022	29

Figure 19 - Kangaroo Flat - Manganese in Soluble Fraction - 2021-2022	30

Figure 20 - Kangaroo Flat - Manganese in Insoluble Fraction - 2021-2022	30

Figure 21 - Kangaroo Flat - Total Insoluble Matter (TIM) - 2021-2022	31

Figure 22 - Kangaroo Flat - Total Ash Content - 2021-2022	32

Figure 23 - Kangaroo Flat - Ash Content/TIM Ratio - 2021-2022	33

Figure 24 - Kangaroo Flat - Total Arsenic/Total Solids - 2021-2022	34

Table of Tables

Table 1 - Kangaroo Flat - Dust Deposition Data	21

# Executive Summary

ALS is engaged by the Department of Energy, Environment and Climate Action , Earth Resources Regulation (DEECA(ERR)) to undertake dust monitoring at the former Kangaroo Flat Mine. This report provides results and analysis of data collected from sampling in Q4 2022 for the Kangaroo Flat Mine site.

DEECA(ERR) have responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its operator, Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, DEECA(ERR) engaged ALS to undertake depositional dust monitoring at their former Kangaroo Flat and Woodvale sites.

This report analyses the data gathered in Q4 2022 for the Kangaroo Flat Mine site by comparing the levels to accepted standards (where available) and previous dust monitoring results. The results of dust monitoring in the first half of 2021 were published in a report by CDM Smith. ALS has completed the collection of samples/monitoring, analysis of lab results (Springvale ALS Laboratory - NATA accredited) and drafted the dust monitoring reports since Q3 2021.

The Kangaroo Flat Mine site (including dust monitoring sites) is located approximately 4 km south from the centre of Bendigo. The Kangaroo Flat Mine site includes:

- fine tailings dam currently covered in dust suppression polymer

- course tailings dam which has been covered with waste rock

- two water dams which capture rainwater

- various other smaller ponds for stormwater management

- process plant

- mine administration buildings and various sheds.

This site is currently under care and maintenance. To keep dust levels down, DEECA(ERR) is applying a range of dust control measures at the site, including applying water and dust suppression polymer and ensuring vegetation cover, and minimising vehicle movements across the site. Inspections are carried out regularly to make sure that dust levels are minimised.

To maintain continuity and comparability of data, dust monitoring was undertaken at the same locations and a similar methodology and analysis process was followed in this report as per other quarters. Upon routine monthly monitoring, ALS field staff observed all monitoring sites where in good working order with no obvious variations to sites that would reflect abnormalities in results.

The original scope of dust monitoring at Kangaroo Flat was to operate and maintain four depositional dust gauges and a background site. Data from this control site will be used for comparative background analysis.

The purpose of dust monitoring is to assure the local community that dust issues are being minimised and managed effectively. The objective of the dust monitoring program at Kangaroo Flat is to gather baseline data and report exceedances when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will inform decision making on rehabilitation requirements and measures required to ensure works meet air quality standards (where available).

As done previously in the CDM Smith Report, the following questions are answered for Q4 2022:

- Did the dust deposition rates recorded in Q4 2022, exceed the adopted dust deposition criteria of 4 g/m2/month, measured as Total Insoluble Matter (TIM)?

In Q4 2022, there was no instance where the dust level exceeded over the 4 g/m2/month criteria at any of the monitoring sites.

- Are dust deposition rates recorded in Q4 2022, comparable to available historic results in this area?

Overall results for Q4 2022 were comparable to previous results (Kralcopic Report in 2018, CDM Smith Report for the first half of 2021, ALS Reports for Q3 2021, Q4 2021 and Q1, Q2 and Q3 2022).

- Are arsenic concentrations recorded in deposited dust in Q4 2022, comparable to available historic results in this area?

Yes. Figure 24 shows the trends for arsenic. The overall total arsenic mean value for all source monitoring sites for Q4 2022 was 21.1 mg/kg. This value is slightly bias due to an elevated sample collected at KF-2 with a value of 100.9mg/kg included in the mean. The mean is well below the 100 mg/kg assessment level. Barium and manganese were generally within the historical ranges previously recorded, with results being comparable and at times lower than 2021 and Q1/Q2/Q3 2022 results. The only outlier can be seen at the background site for Manganese in soluble fraction for Dec 2022. A similar outlier occurred at the same site back in October 2021.

Continued monitoring at the background monitoring location KF5 (BG) provides additional data to which the site data can be compared.

# Introduction

ALS was engaged by the Earth Resource Regulation (ERR) team in the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Kangaroo Flat mine from the start of January 2021. On 1st January 2023 ERR joined a new Government department, the Department of Energy, Environment and Climate Action (DEECA). The ongoing engagement of ALS is now with DEECA Earth Resources Regulation (DEECA(ERR)). This report provides results and analysis of data collected from sampling in Q4 2022. ALS undertook the field component of maintaining dust gauges and replacing sample containers, as well as laboratory analysis of samples collected.  Samples were collected from locations specified by DJPR on a routine monthly basis (Figure 1 and Figure 2). These locations are displayed in Section 3.1 of this report.

# Background

In April 2021, ERR inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites as its operator, Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at their former Kangaroo Flat and Woodvale sites. This report analyses the data gathered in Q4 2022 for the Kangaroo Flat site by comparing the levels to accepted standards (where available) and previous dust monitoring results published in:

- ALS Report for Q3 2022

- ALS Report for Q2 2022

- ALS Report for Q1 2022

- ALS Report for Q4 2021

- ALS Report for Q3 2021

- CDM Smith report for the first half of 2021

- Kralcopic Report for Q4 2018.

All reports are referenced in this document. To maintain continuity and comparability of results, this document has been prepared in a similar format to the CDM Smith Report.

The Kangaroo Flat Mine site is located to the east of Kangaroo Flat, approximately 4 km south of the centre of Bendigo. The site includes decommissioned mine tunnels and shafts as well as processing plants and tailings dams. The site is currently in care and maintenance. To keep dust levels down, DEECA(ERR) is applying a range of dust control measures at the former the Kangaroo Flat Mine site, including applying water and dust suppression polymer, ensuring vegetation cover where practical, and minimising vehicle movements. Inspections are also carried out regularly to make sure dust levels are being maintained.

To ensure continuity and comparability of data, monitoring was undertaken at the same previous locations, and similar methodology and analysis was used. This report builds on the previous results and provides baseline information to potentially inform decisions around some aspects of rehabilitation.

## Scope and Objectives

The scope of dust monitoring at Kangaroo Flat is to operate and maintain five depositional dust gauges.

All depositional dust gauges are located at their historic locations with the following naming conventions since the dates specified below:

| - KF1 (W) Jan 2021 - KF2 (E) Jan 2021 - KF5 (BG) Sep 2021 | - KF3 (N) Jan 2021 - KF4 (S) Jan 2021 |
| --- | --- |

Laboratory analysis requirements were provided by DEECA(ERR) and were in line with historical requirements during the previous monitoring program undertaken by Kralcopic to meet it’s legislative requirements. This alignment maintains continuity and comparability of results.

The purpose of dust monitoring is to check that dust issues are managed effectively including testing the effectiveness of dust control measures. Another objective of the dust monitoring program at Kangaroo Flat is to gather baseline data ahead of rehabilitation works. These results will inform decision making on dust control measures required during site rehabilitation, to ensure works meet air quality standards (where available).

As done previously, the following considerations were included in the assessment of Q4 2022 data:

- Did the dust deposition rates recorded in Q4 2022, exceed the adopted dust deposition criteria of 4 g/m2/month, measured as Total Insoluble Matter (TIM)?

- Are dust deposition rates recorded in Q4 2022, comparable to available historic results in this area?

- Are arsenic concentrations recorded in deposited dust in Q4 2022, comparable to available historic results in this area?

## Regulatory Environment

The Environment Protection Authority (EPA) has overarching responsibility for administering legislation related to the air quality across Victoria. The EPA administers the Environment Protection Act 2017 and the Environment Protection Regulations 2021 under which Environmental Reference Standards (ERS) and related Guidelines are provided.

The purpose of the ERS is to support the protection of human health and the environment from harms posed by pollution and waste. The ERS does this by providing benchmarks to assess and report on environmental conditions in the whole or any part of Victoria.

The ERS seeks to achieve this purpose by:

- identifying environmental values to be achieved or maintained in the whole or any part of Victoria; and

- specifying indicators and objectives to be used to measure, determine or assess whether those environmental values are being achieved, maintained or threatened.

The ERS provides guidance, objectives and levels on values where the environment may be impacted and incorporates the Ambient Air Quality National Environment Protection Measure (NEPM AAQ).

The main guidance document used for the purpose of this report is EPA Publication 1191 – Protocol for Environmental Management – Mining and Extractive Industries (Mining PEM). The criteria in the Mining PEM used for this report has been carried over to the EPA’s latest Guideline for Assessing and Minimising Air Pollution in Victoria, Publication 1961, February 2022. As stated in PEM, Results of monitoring should not exceed 4g/m2/month (no more than 2g/m2/month above background) as a monthly average.

Under the new EPA legislation, duty holders have responsibility under the general environmental duty to apply controls to eliminate or minimise risks of harm to human health and the environment as far as reasonably practicable. This means that the primary focus is on applying effective elimination, minimisation, and control measures. Monitoring is for the purpose of determining the effectiveness of control measures.

## 2.3	What is Dust

Dust is not typically classified according to its composition, but instead on its particle size, as follows:

- Deposited matter - any particles that fall out of suspension in the atmosphere.

- Total Suspended Particles (TSP) - particles suspended or entrained in the air. Typically, this is particles of 30 µm (0.03 mm) equivalent aerodynamic diameter or less. Larger particles tend not to become suspended.

- PM10 - particles 10 µm equivalent aerodynamic diameter or less.

- PM2.5 - particles 2.5 µm equivalent aerodynamic diameter or less.

Dust particle size is an important consideration influencing dispersion and transport in the atmosphere and potential effects on human health. Human activities (e.g. energy use, transport, industrial activities etc) can affect the air quality, in particular airborne dust.

Potential sources of dust are:

- natural sources such as dust storms, agricultural dust, bushfires, vegetation, pollen and fungi; and

- anthropogenic sources such as mines sites, industry, roads and vehicles, construction sites, domestic and diffuse sources (CDM Smith, 2021).

# Measurement of Dust

## 3.1	Site Locations

Locations of depositional dust gauges around the Kangaroo Flat Mine are presented in Figure 1.

Figure 1 - Kangaroo Flat - Monitoring Locations

Location of background gauge and image are presented in Figure 2 and Figure 3 below.

Figure 2 - Kangaroo Flat - Control Site Location KF5 (BG)

Figure 3 - Kangaroo Flat - Control Site Gauge KF5 (BG)

Samples were collected in accordance with the Standard AS/NZS 3580.10.1 - Methods for sampling and analysis of ambient air - Determination of particulate matter - Deposited matter - Gravimetric method and ALS Dust Sampling and Depositional Dust Gauge Work Procedure MAT-MAP1000 (See Appendix A - Supplementary Information).

ALS Laboratory in Springvale analysed the samples for the following:

- Total deposited matter

- Total ash content

- Metals/metalloids, in the soluble, insoluble and ash content fraction of dust.

## 3.2	Deposited Matter

The Standard (AS/NZS 3580.10.1:2016) sets out a method (used in this report) for sampling particulate matter that is deposited from the atmosphere. This Standard also sets out procedures for the gravimetric (i.e., by weighing) determination of the mass deposition rate of insoluble/soluble solids, ash/combustible matter, and total solids from ambient air.

The method provides an estimate of the mean surface concentration of deposited matter settling from the air over a sampling period, typically of one month. Particulate matter deposition rates of 0.1 g/m2/month and above may be determined using a monthly sampling period. The sample obtained by the sampling procedure specified may be subjected to physical or chemical analysis.

Over a given sampling period, particles that settle from the ambient air are collected in a vessel (Figure 4) and retained together with any rainwater. The sample is passed through a sieve to remove any extraneous matter (e.g. leaves, insects), and the sieved sample containing the deposited matter is transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of the dried insoluble solids is gravimetrically determined.

The ash and combustible matter content are determined by loss on ignition of the insoluble solids. Soluble solids are determined from the filtrate. The total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass deposition rate of deposited matter is then calculated from the mass of solids obtained, the funnel cross-sectional area and the exposure period. (AS/NZS 3580.10.1:2016)

Figure 4 - Kangaroo Flat - Dust Deposition Gauge KF1 (W)

## 3.3	Metals/Metalloids Monitoring in Dust

The inclusion of the analysis of metals/metalloids in the dust samples assist in identifying possible sources of contamination if present.

Analysis for insoluble matter, soluble matter and ash content was undertaken for the following:

- Arsenic

- Barium

- Manganese

It is understood that there are no specific threshold values for metals and metalloids in deposited dust either in the former Mining Licence or within the ERS. (CDM Smith, 2021)

## 3.4	Ash Content

Ash Content is the remaining material after the sample has been combusted in the laboratory. Ash content provides an indication of the mineral content (or soil dust) of the sample. The mineral content may be attributable to mine site contributions, but may also be attributed to other sources such as agriculture, unsealed roads, etc.

## 3.5	Dust Monitoring Standards

Results were compared against relevant dust deposition rate criteria described by EPA Publication 1191 (2007), as shown below which has been adopted in EPA Publication 1961 (2022):

The following points apply to the criteria:

- Results of monitoring should not exceed 4 g/m2/month (no more than 2 g/m2/month above background) as a monthly average.

- The 2 g/m2/month criteria are used when baseline data on deposited dust levels exist, while the 4 g/m2/month criteria is used when no baseline data exists.

- If dust levels exceed these values, then site management practices should be reviewed, and additional dust controls investigated.

- The criteria refer to all sources of deposited matter (including sources from mines, agriculture, unsealed roads, etc) and cumulative impacts.

- The criteria states that in some cases, a mine may increase deposited dust levels by up to 2 g/m2/month. However, the total deposited dust level (including sources from mines, agriculture, unsealed roads, etc) must not exceed 4 g/m2/month.

Deposited matter (dust) can be used as an indicator of the effectiveness of site management practices and the potential for offsite nuisance dust (fugitive dust). Deposited dust monitoring, when conducted over a set time, is useful for examining trends and evaluating deviation from long term trends with respect to site activities.

## 3.6	Dust Monitoring Program

The current method of undertaking depositional dust monitoring is considered sufficient as the site is in care and maintenance i.e., no new pollution is being introduced to the site from industry. There is a considerable body of knowledge about the contamination level at the site, so all that is required at this stage is to determine whether this contamination is becoming air borne and becoming a health hazard or problem to the nearby landholders. Depositional dust monitoring provides a range of useful information to be able to understand what’s happening at this site and its surrounds.

Depositional dust monitoring can provide the following invaluable information:

- test the effectiveness of dust minimisation, control and management measures

- identify key problematic sources, or groups of sources on larger more complex sites

- identify where dust sensitivities may occur

- characterise temporal or spatial trends

- metallic/metalloid concentrations are useful for health effect considerations.

The environmental conditions during Q4 2022 across Bendigo saw increased rainfall during October 2022. The tailings dam level located at Kangaroo Flat Mine is not continuously monitored by ALS.

A new weather station was installed by ALS in 2023 which will help in future reporting investigate predominant wind direction and speed. Weather station data during Q4 has been sourced from the BOM daily weather observations from site location, Bendigo Airport (station 081123).

# Dust Deposition Results

## 4.1	Kangaroo Flat Mine – Q4 2022

Dust deposition analysis results are presented in Table 1 and the accompanying Laboratory Certificates of Analysis are presented in Appendix B.

Where results were reported as “less than” (<) a specified number (laboratory reporting limit) the number reported was adopted for use in the graphical displays. This approach adopts a conservative value for the number being reported.

## 4.1.1 Deposition of Total Insoluble Matter

Total Insoluble Matter (TIM) met the criteria of less than 4 g/m2/month on 15 of 15 measurements during Q4 2022. See Figure 5 below.

Dust deposition rate at site KF2 (E) was the highest value for the period during December 2022 recording a result of 3.1 g/m2/month. When the data from all source monitoring sites was averaged for Q4 2022(excluding the background site), an average of 1.5 g/m2/month was recorded. This value is below the inferred background level suggested within the Mining PEM.

Both exceedance criteria (4 g/m2/month and background plus 2 g/m2/month) have been considered when reviewing data sets. No exceedances were recorded during Q4, 2022.

Figure 5 - Kangaroo Flat - Total Insoluble Matter (TIM) g/m2/month – Q4 2022

Red dotted line is the criteria used for the purpose of this report.

## 4.1.2 Mineral Content in Dust

The fraction of dust measured as ash content is displayed below in Figure 6. The values recorded during Q4 were all between <0.1 g/m2/month and 0.2 g/m2/month.

Figure 6 - Kangaroo Flat - Total Ash Content g/m2/month – Q4 2022

To gain a general understanding of what proportion of the sample is ash content i.e., mineral dust, the ratio of total ash content and TIM has been plotted in Figure 7.

Figure 7 - Kangaroo Flat - Ash Content/TIM Ratio – Q4 2022

Figure 7 shows that ash content in the samples throughout Q4 2022 were generally low. The results above are also representative of the background site.

## 4.1.3 Metals/Metalloids in Dust

Graphical presentations of soluble and insoluble metals (arsenic, barium and manganese) are provided in Section 9 of this report. Arsenic, Barium, and Manganese results for Q4 2022 were within historical ranges. Manganese at the background site KF-5BG recorded one result outside the previous results although this reading was similar to that recorded in Oct 2021. Historical data sets from 2021 and Q1/Q2/Q3 2022 have been reported and results were within historical ranges.  This can be found in Sections 9.1,9.2 and9.3 of this report.

To provide an understanding of total arsenic concentrations measured in dust relative to concentrations measured in soils within the region, total arsenic concentration has been displayed below Figure 8 in mg/kg. This is calculated by adding soluble and insoluble arsenic µg/m2/month dividing by the total solids g/m2/month.

This ratio is being plotted because this provides a figure which could be compared to the figures quoted in the guideline, Contaminated Sites Management Series - Assessment Levels for Soil, Sediment and Water. The assessment level for soil for arsenic is 100 mg/kg for standard residential areas in this guideline.

Total arsenic concentrations for Q4 ranged from 0.80 to 100.90 mg/kg with a mean of 21.1mg/kg across all source monitoring sites excluding the KF5 (BG) site. The mean is well below the 100 mg/kg assessment level. Due to an elevated result at KF-2 during December of 100.9mg/kg the mean is higher than Q3.

Figure – - Kangaroo Fl–t - Total Arsenic/Total Solids(mg/kg)  – Q4 2022

The elevated reading at KF2 (E) has historically been thought to have been influenced by its proximity to the fine tailings dam which can contain high arsenic levels. Wind direction and speed may have influenced the elevated level of organic matter collected at KF2. This effect is the likely cause of the elevated arsenic level. The readings for Q4 (average 21.1 mg/kg not including BG site) are still much lower than that of Q2 (average 33.4mg/kg) yet similar to that of Q1 2022 (average 22.8 mg/kg).

DEECA(ERR) undertakes regular inspections to identify any dust issues at site. This assists with promptly identifying and addressing issues (e.g., by the maintenance and enhancement of dust control measures). However, at times climatic conditions such as wind speed, direction and rainfall can cause elevated  readings.

## 4.2	Quality Assurance and Quality Control

Field sampling and analysis was undertaken in accordance with the following Standards:

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Deposited Matter - Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Impinged Matter - Gravimetric Method. (AS/NZS 3580.10.2)

Sampling methodology and locations are consistent with sampling previously undertaken to maintain continuity and comparability of results.

ALS Environmental Laboratory has NATA accreditation for the following Ambient Air Parameters:

ALS currently does not have NATA accreditation for metals analysis of ambient air but is in the process of gaining accreditation. Nevertheless, ALS is operating as though this accreditation is in place and expects to finalise the accreditation process in 2022.

Internal laboratory quality control measures note the following:

- No Method Blank value outliers occurred.

- No laboratory control outliers occurred.

- No Matrix Spike (MS) or Matrix Spike Duplicate (MSD) results reported.

- No laboratory duplicate results reported.

- No holding time outliers occurred.

- No quality control sample frequency outliers occurred (based on NEPM 2013 B3 quality control specification).

# Summary of Results

Results are discussed in detail throughout the relevant sections in this report. In summary, the overall results for Q4 2022 were comparable to previous results and similar to the results observed in the same quarter 2021 and the first quarter of 2022.

The key findings for Q4 2022 are as follows:

- There were no exceedances of the 4 g/m2/month (or the background plus 2 g/m2/month) criteria at depositional gauges monitoring the Kangaroo Flat former mine site in Q4 2022.

- The overall total arsenic mean value for all source monitoring sites for Q4 2022 was 21.1 mg/kg and well below the 100 mg/kg assessment level.The value is slightly bias due to the high sample collected at KF-2 with a value of 100.9mg/kg included in the mean.

- Barium and Manganese were within the historical ranges previously recorded, with results being comparable to 2021 results. The only exception occurred at the background site KF -5 BG in December 2022.

# 6	Further Considerations

DEECA has advised that they will continue to monitor the site for dust emissions and ensure that a range of dust control measures are applied during this care and maintenance phase of the rehabilitation project. DEECA is also reviewing the effectiveness of dust control measures and monitoring on an ongoing basis and adopting a continuous improvement approach to dust minimisation and monitoring.

The installation of an onsite weather monitoring station has been completed and will provide information to ALS and DEECA(ERR) to support dust monitoring and management at the site. Weather information applicable to the results will be included in future reports.

# 7	References

- ALS Hydrographics Pty Ltd (2021), Kangaroo Flat Dust Monitoring Q3 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2021 Kangaroo Flat Dust Monitoring Q4 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Kangaroo Flat Dust Monitoring Q1 2022 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Kangaroo Flat Dust Monitoring Q2 2022 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Kangaroo Flat Dust Monitoring Q3 2022 Report. (ALS Report)

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Deposited Matter - Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Impinged Matter - Gravimetric Method. (AS/NZS 3580.10.2)

- CDM Smith (2021), Kangaroo Flat Mine - Dust Monitoring Report - January to June 2021. (CDM Smith Report)

- Department of Conservation (2010), Contaminated Sites Management Series, Assessment Levels for Soil, Sediment and Water, February 2010. (Contaminated Sites Assessment Levels)

- EPA (Vic) Publication 1191 (2007), Protocol for Environmental Management - Mining and Extractive Industries. (Mining PEM)

- EPA Publication 1961 (2022), Guideline for Assessing and Minimising Air Pollution in Victoria, February 2022. (Assessing and Minimising Guideline)

- EPA Publication 440.1 (2002), A Guide to the Sampling and Analysis of Air Emissions and Air Quality, December 2002. (Sampling and Analysis Guideline)

- Kralcopic Pty Ltd (2018), Quarterly Environmental Monitoring Report – Kangaroo Flat Dust Monitoring, October to December 2018. (Kralcopic Report)

- National Environment Protection Council (1998), National Environment Protection (Ambient Air Quality) Measure, July 2018. (NEPM AAQ)

- Victorian Government (2001), Statement of Environment Protection Policy (Air Quality Management), Victorian Government Gazette, No S 240, 21 December 2001. (SEPP AQM)

- Victorian Government (2016), State Environment Protection Policy (Ambient Air Quality), Original Policy including Two Variations, Victorian Gazette Nos S19, S240 and G30, published on 9 February 1999, 21 December 2001, and 28 July 2016, respectively. (SEPP AAQ)

- Victorian Government (2021), Environment Reference Standard, under the Environment Protection Act 2017, Victorian Government Gazette, No. S 245, 26 May 2021. (ERS)

- EPA NSW Publication, Approved Methods for the Modelling and Assessment of Air Pollutants in NSW, (2017)

# 8	Tables

Table 1 - Kangaroo Flat - Dust Deposition Data

|  |  | Arsenic | Barium | Manganese | Total Solids | Total Solids (mg) | Total Insoluble Matter | Total Insoluble Matter (mg) | Total Soluble Matter | Total Soluble Matter (mg) | Combustible Matter | Combustible Matter (mg) | Ash Content | Ash Content (mg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ug/m2 month | ug/m2 month | ug/m2 month | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg |
| EQL |  |  |  |  |  | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 |
| Description | Lab Report Number |  |  |  |  |  |  |  |  |  |  |  |  |  |
| KF1 (Insoluble) | EM2221964-064_V1 | 4.8 | 27.4 | 34.3 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-064_V1 | 26.4 | 83.2 | 85.1 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-064 | 8.4 | 68.6 | 82.7 |  |  |  |  |  |  |  |  |  |  |
| KF1 (Soluble) | EM2221964-036_V1 | 2.19 | 136 | 200 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-036_V1 | 6.5 | 208 | 462 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-036 | 4.6 | 94.6 | 328 |  |  |  |  |  |  |  |  |  |  |
| KF1 (Ash) | EM2221964-008_V1 | 4.8 | 27.4 | 29.8 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-008_V1 | 26.4 | 83.2 | 55.2 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-008 | 7.27 | 68.6 | 61.3 |  |  |  |  |  |  |  |  |  |  |
| KF1 | EM2221964-008_V1 |  |  |  | 6.2 | 113 | 0.9 | 16 | 5.3 | 97 | 0.9 | 15 | 0.1 | 2 |
|  | EM2223849-008_V1 |  |  |  | 5.9 | 97 | 1.6 | 26 | 4.3 | 71 | 1.4 | 24 | 0.2 | 2 |
|  | EM2300219-008 |  |  |  | 4.8 | 88 | 2.5 | 46 | 2.3 | 42 | 2.3 | 43 | 0.2 | 3 |
| KF2 (Insoluble) | EM2221964-065_V1 | 6.6 | 26 | 43.9 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-065_V1 | 77.5 | 82.8 | 113 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-065 | 367 | 96.1 | 141 |  |  |  |  |  |  |  |  |  |  |
| KF2 (Soluble) | EM2221964-037_V1 | 8.26 | 40.4 | 124 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-037_V1 | 16.4 | 128 | 298 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-037 | 66.7 | 105 | 382 |  |  |  |  |  |  |  |  |  |  |
| KF2 (Ash) | EM2221964-009_V1 | 6.60 | 26 | 30.10 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-009_V1 | 51.3 | 82.8 | 65.1 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-009 | 209 | 96.1 | 70.5 |  |  |  |  |  |  |  |  |  |  |
| KF2 | EM2221964-009_V1 |  |  |  | 1.9 | 35 | 0.9 | 17 | 1 | 18 | 0.9 | 17 | 0.1 | 2 |
|  | EM2223849-009_V1 |  |  |  | 6.2 | 102 | 1.4 | 23 | 4.8 | 79 | 1.3 | 21 | 0.1 | 2 |
|  | EM2300219-009 |  |  |  | 4.3 | 80 | 3.1 | 57 | 1.2 | 23 | 2.9 | 54 | 0.2 | 3 |
| KF3 (Insoluble) | EM2221964-066_V1 | 20.3 | 117.0 | 78.9 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-066_V1 | 15.9 | 73.1 | 112 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-066 | 73 | 81.4 | 92.7 |  |  |  |  |  |  |  |  |  |  |
| KF3 (Soluble) | EM2221964-038_V1 | 12.8 | 65.8 | 160 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-038_V1 | 2.61 | 142 | 237 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-038 | 17.1 | 70.2 | 225 |  |  |  |  |  |  |  |  |  |  |
| KF3 (Ash) | EM2221964-010_V1 | 17.7 | 117.0 | 54.8 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-010_V1 | 15.9 | 73.1 | 70.8 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-010 | 73.0 | 81.4 | 63.6 |  |  |  |  |  |  |  |  |  |  |
| KF3 | EM2221964-010_V1 |  |  |  | 2.4 | 43 | 1.5 | 27 | 0.9 | 16 | 1.3 | 23 | 0.2 | 4 |
|  | EM2223849-010_V1 |  |  |  | 5.9 | 97 | 1.3 | 21 | 4.6 | 76 | 1.1 | 18 | 0.2 | 3 |
|  | EM2300219-010 |  |  |  | 1.6 | 29 | 1.0 | 18 | 0.6 | 11 | 0.8 | 15 | 0.2 | 3 |
| KF4 (Insoluble) | EM2221964-067_V1 | 39.4 | 34 | 44.1 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-067_V1 | 32.8 | 211 | 159 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-067 | 40.0 | 57.4 | 92.6 |  |  |  |  |  |  |  |  |  |  |
| KF4 (Soluble) | EM2221964-039_V1 | 4.59 | 136 | 151 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-039_V1 | 23.9 | 336 | 708 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-039 | 6.92 | 153 | 276 |  |  |  |  |  |  |  |  |  |  |
| KF4 (Ash) | EM2221964-011_V1 | 3.02 | 34 | 29.3 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-011_V1 | 32.8 | 211 | 159 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-011 | 27.7 | 57.4 | 49.3 |  |  |  |  |  |  |  |  |  |  |
| KF4 | EM2221964-011_V1 |  |  |  | 1.9 | 34 | 1.1 | 20 | 0.8 | 14 | 1.1 | 20 | 0.1 | 2 |
|  | EM2223849-011_V1 |  |  |  | 11.5 | 189 | 2.1 | 34 | 9.4 | 155 | 1.9 | 30 | 0.2 | 4 |
|  | EM2300219-011 |  |  |  | 1.0 | 17 | 0.9 | 16 | 0.1 | 2 | 0.8 | 14 | 0.1 | 2 |
| KF5BG (Insoluble) | EM2221964-068_V1 | 5.2 | 75.5 | 35.1 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-068_V1 | 4.44 | 78.1 | 65.8 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-068 | 8.2 | 61.6 | 154 |  |  |  |  |  |  |  |  |  |  |
| KF5BG (Soluble) | EM2221964-040_V1 | 2.89 | 55.7 | 163 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-040_V1 | 4.53 | 179 | 331 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-040 | 30.7 | 175 | 2521 |  |  |  |  |  |  |  |  |  |  |
| KF5BG (Ash) | EM2221964-012_V1 | 5.2 | 75.5 | 35.1 |  |  |  |  |  |  |  |  |  |  |
|  | EM2223849-012_V1 | 3.13 | 55.1 | 46.4 |  |  |  |  |  |  |  |  |  |  |
|  | EM2300219-012 | 7.2 | 61.6 | 103 |  |  |  |  |  |  |  |  |  |  |
| KF5BG | EM2221964-012_V1 |  |  |  | 1 | 20 | 1.0 | 19 | 0.1 | 2 | 1.0 | 19 | 0.1 | 2 |
|  | EM2223849-012_V1 |  |  |  | 6.6 | 110 | 1.3 | 22 | 5.3 | 88 | 1.2 | 21 | 0.1 | 2 |
|  | EM2300219-012 |  |  |  | 4.8 | 88 | 1.0 | 19 | 3.8 | 69 | 0.9 | 17 | 0.1 | 2 |

# 9	Supplementary Data

Graphs for Metals/Metalloids in Insoluble and Soluble Fractions including trends are presented below.

## 9.1	Arsenic

Figure 9 - Kangaroo Flat - Arsenic in Soluble Fraction – Q4 2022

Figure 10 - Kangaroo Flat - Arsenic in Insoluble Fraction – Q4 2022

Figure 11 - Kangaroo Flat - Arsenic in Soluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 12 - Kangaroo Flat - Arsenic in Insoluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## 9.2	Barium

Figure 13 - Kangaroo Flat - Barium in Soluble Fraction – Q4 2022

Figure 14 - Kangaroo Flat - Barium in Insoluble Fraction – Q4 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 15 - Kangaroo Flat - Barium in Soluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 16 - Kangaroo Flat - Barium in Insoluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## 9.3	Manganese

Figure 17 - Kangaroo Flat - Manganese in Soluble Fraction – Q4 2022

Figure 18 - Kangaroo Flat - Manganese in Insoluble Fraction – Q4 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 19 - Kangaroo Flat - Manganese in Soluble Fraction - 2021-2022

Figure 20 - Kangaroo Flat - Manganese in Insoluble Fraction - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## 9.4	Trend Graphs

Figure 21 - Kangaroo Flat - Total Insoluble Matter (TIM) - 2021-2022

Figure 21 shows TIM from throughout 2021 and 2022. The chart shows no exceedances of the recommended PEM criteria occurred during the Q4 2022 period. Overall average of reported TIM for 2021 was 0.3 g/m2/month and average for 2022 recorded 0.2 g/m2/month. This is well below the exceedance criteria. (excludes the background data.)

Figure 22 - Kangaroo Flat - Total Ash Content - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 22 provides graphical summary of total ash content throughout 2021 and 2022. As can be seen from this graph, most of the data for Q4 2022 was between 0.1 and 0.2 g/m2/month.

Figure 23 - Kangaroo Flat - Ash Content/TIM Ratio - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 23 shows the Ash Content/TIM Ratio for all months in 2021 and 2022. The ratio values for Q4 2022 are at lower when compared to other quarters, in 2021 and 2022.

Figure 24 - Kangaroo Flat - Total Arsenic/Total Solids - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

By comparing the graphs in Figure 23 and Figure 24 above, it can be concluded that these results for Q4 2022 are trending in line with previous results.

# Appendix A - Supplementary Information

# Appendix B - Laboratory Reports

Environmental

CERTIFICATE OF ANALYSIS

Work Order	: EM2221964

Page	: 1 of 19

Amendment

Client

Contact

Address

: 1

: ALS WATER AND HYDROGRAPHICS PTY LTD

# :

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Bronwyn Sheen

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +6138549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 07-Nov-2022 10:40 |
| Order number | : ---- | Date Analysis Commenced | : 15-Nov-2022 |
| C-O-C number Sampler Site | : ---- : RO : ---- | Issue Date | : 28-Feb-2023 20:26 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Dilani Fernando	Laboratory Coordinator	Melbourne Inorganics, Springvale, VIC

Eric Chau	Metals Team Leader	Melbourne Inorganics, Springvale, VIC

R I G H T  S O L U T I O N S | R I G H T  P A R T N E R

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- EA120-EA142: EM2221964 #1; Sample was observed to be very turbid.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- Amendment (28/02/2023): This report has been amended following a change to the metal (EG020TUG) results reported for sample EM2221964 #1-#28 due to a transcription error. All details are recorded in client query 23MECC076 and a full investigation will be detailed in corrective action request 23MEC033.

- Sampling Period: 30/09/2022-31/10/2022.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 10:35 | 31-Oct-2022 10:50 | 31-Oct-2022 11:15 | 31-Oct-2022 11:25 | 31-Oct-2022 11:45 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-001 | EM2221964-002 | EM2221964-003 | EM2221964-004 | EM2221964-005 |
|  |  |  | Result 4.1 | Result 0.2 | Result 1.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 76 10.7 | 4 1.2 | 21 3.0 | <2 1.2 | <2 1.0 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 194 8.4 | 22 6.7 | 54 0.8 | 22 1.2 | 18 0.9 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 154 14.8 | 123 1.4 | 14 4.1 | 22 1.3 | 16 1.0 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 270 23.2 | 26 8.1 | 75 4.9 | 24 2.5 | 18 1.9 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 424 36.5 | 149 2.27 | 89 <19.6 | 46 1.41 | 34 <1.70 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <1630 | <52.0 | <560 | <33.2 | <22.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 1140 | 31.5 | <318 | 26.3 | 20.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD06 | WVDD007BG | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 10:15 | 31-Oct-2022 09:45 | 31-Oct-2022 13:05 | 31-Oct-2022 12:55 | 31-Oct-2022 13:00 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-006 | EM2221964-007 | EM2221964-008 | EM2221964-009 | EM2221964-010 |
|  |  |  | Result 2.7 | Result 0.1 | Result <0.1 | Result <0.1 | Result 0.2 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 49 7.0 | 2 1.2 | <2 0.9 | <2 0.9 | 4 1.3 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 129 11.0 | 21 3.4 | 15 5.3 | 17 1.0 | 23 0.9 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 201 9.7 | 62 1.3 | 97 0.9 | 18 0.9 | 16 1.5 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 178 20.7 | 23 4.7 | 16 6.2 | 17 1.9 | 27 2.4 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 379 27.6 | 85 1.49 | 113 <4.80 | 35 <6.60 | 43 17.7 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <781 | <33.2 | <27.4 | <26.0 | <117 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <478 | 30.0 | 29.8 | 30.1 | 54.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 12:45 | 31-Oct-2022 12:35 | 31-Oct-2022 11:15 | 31-Oct-2022 11:15 | 31-Oct-2022 11:15 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-011 | EM2221964-012 | EM2221964-013 | EM2221964-014 | EM2221964-015 |
|  |  |  | Result <0.1 | Result <0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | <2 ---- | <2 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 1.1 | ---- 1.0 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 20 ---- | 19 ---- | ---- 1.0 | ---- 1.3 | ---- 1.3 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.8 | ---- <0.1 | 16 ---- | 21 ---- | 20 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 14 ---- | <2 ---- | ---- 1.0 | ---- 0.3 | ---- 0.8 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 1.1 | ---- 1.0 | 15 ---- | 5 ---- | 12 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 20 ---- | 19 ---- | ---- 1.0 | ---- 1.3 | ---- 1.3 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 1.9 | ---- 1.0 | 16 ---- | 21 ---- | 20 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 34 ---- | 20 ---- | ---- 2.0 | ---- 1.6 | ---- 2.1 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- 3.02 | ---- <5.20 | 31 <0.50 | 26 <1.10 | 32 <1.60 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <34.0 | <75.5 | 175 | <35.5 | <32.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 29.3 | <35.1 | 10.6 | <35.1 | 38.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 11:15 | 31-Oct-2022 11:25 | 31-Oct-2022 11:25 | 31-Oct-2022 11:25 | 31-Oct-2022 11:25 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-016 | EM2221964-017 | EM2221964-018 | EM2221964-019 | EM2221964-020 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result 0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 <0.1 | <2 1.2 | <2 1.0 | <2 1.4 | <2 1.2 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 21 0.1 | 20 0.3 | 17 0.2 | 22 0.2 | 18 0.3 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 1.3 | 4 1.3 | 3 1.1 | 4 1.5 | 5 1.3 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 21 1.4 | 21 1.6 | 18 1.3 | 23 1.7 | 20 1.6 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 22 <0.50 | 25 1.02 | 21 <1.10 | 27 1.47 | 25 <2.20 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 18.3 | <35.1 | 31.9 | 40.8 | <31.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 23.8 | 30.4 | 46.0 | <74.1 | <102 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG06N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 11:45 | 31-Oct-2022 11:45 | 31-Oct-2022 11:45 | 31-Oct-2022 11:45 | 31-Oct-2022 10:15 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-021 | EM2221964-022 | EM2221964-023 | EM2221964-024 | EM2221964-025 |
|  |  |  | Result <0.1 | Result 0.3 | Result <0.1 | Result 0.5 | Result 0.2 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.9 | 5 1.4 | <2 1.0 | 7 2.0 | 3 1.4 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 15 <0.1 | 22 1.3 | 17 0.3 | 32 0.1 | 22 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.9 | 21 1.7 | 4 1.1 | 2 2.5 | 2 1.6 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 15 0.9 | 27 3.0 | 18 1.4 | 39 2.6 | 25 1.7 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 15 0.69 | 48 <2.70 | 22 <1.10 | 41 3.52 | 27 4.47 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <17.8 | <119 | <27.0 | <226 | <51.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <23.9 | 116 | <52.2 | 53.8 | 32.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06E | WVDG06S | WVDG06W | WVDD01 Soluble metals | WVDD02 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 10:15 | 31-Oct-2022 10:15 | 31-Oct-2022 10:15 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-026 | EM2221964-027 | EM2221964-028 | EM2221964-029 | EM2221964-030 |
|  |  |  | Result 0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.5 | <2 1.2 | <2 1.5 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 23 0.8 | 20 <0.1 | 23 1.0 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 13 1.6 | <2 1.3 | 15 1.5 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 25 2.4 | 21 1.3 | 23 2.5 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 38 1.43 | 22 0.81 | 38 <0.92 | ---- 99.9 | ---- <0.05 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <31.0 | <17.2 | <15.9 | 2280 | 108 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <28.8 | 27.0 | 21.6 | 3120 | 203 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble metals | WVDD04 Soluble metals | WVDD05 Soluble metals | WVDD06 Soluble metals | WVDD007BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-031 | EM2221964-032 | EM2221964-033 | EM2221964-034 | EM2221964-035 |
|  |  |  | Result 67.4 | Result 3.40 | Result 0.33 | Result 84.7 | Result 1.98 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 646 | 61.6 | 48.9 | 1010 | 102 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 2330 | 143 | 135 | 4640 | 206 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble metals | KF2 Soluble metals | KF3 Soluble metals | KF4 Soluble metals | KF5BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-036 | EM2221964-037 | EM2221964-038 | EM2221964-039 | EM2221964-040 |
|  |  |  | Result 2.19 | Result 8.26 | Result 12.8 | Result 4.59 | Result 2.89 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 136 | 40.4 | 65.8 | 136 | 55.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 200 | 124 | 160 | 151 | 163 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N Soluble metals | WVDG03E Soluble metals | WVDG03S Soluble metals | WVDG03W Soluble metals | WVDG04N Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-041 | EM2221964-042 | EM2221964-043 | EM2221964-044 | EM2221964-045 |
|  |  |  | Result 2.47 | Result 2.72 | Result 5.73 | Result 0.57 | Result 1.33 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 115 | 76.9 | 53.6 | 8.08 | 46.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 47.2 | 221 | 211 | 17.1 | 71.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E Soluble metals | WVDG04S Soluble metals | WVDG04W Soluble metals | WVDG05N Soluble metals | WVDG05E Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-046 | EM2221964-047 | EM2221964-048 | EM2221964-049 | EM2221964-050 |
|  |  |  | Result 0.70 | Result 1.13 | Result 0.94 | Result 0.31 | Result 6.77 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 14.4 | 37.0 | 27.7 | 0.65 | 271 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 33.9 | 57.6 | 57.8 | 1.02 | 111 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble metals | WVDG05W Soluble metals | WVDG06N Soluble metals | WVDG06E Soluble metals | WVDG06S Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-051 | EM2221964-052 | EM2221964-053 | EM2221964-054 | EM2221964-055 |
|  |  |  | Result 1.42 | Result 0.73 | Result 1.59 | Result 26.7 | Result 0.28 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 14.5 | 29.7 | 17.7 | 32.2 | 4.93 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 86.8 | 13.2 | 38.0 | 149 | 8.20 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06W Soluble metals | WVDD01 Insoluble metals | WVDD02 Insoluble metals | WVDD03 Insoluble metals | WVDD04 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-056 | EM2221964-057 | EM2221964-058 | EM2221964-059 | EM2221964-060 |
|  |  |  | Result 2.25 | Result 40.7 | Result 2.56 | Result <19.6 | Result 1.51 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 35.0 | <160 | <52.0 | <560 | <33.2 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 145 | 1210 | 50.9 | <318 | 36.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD05 Insoluble metals | WVDD06 Insoluble metals | WVDD007BG Insoluble metals | KF1 Insoluble metals | KF2 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-061 | EM2221964-062 | EM2221964-063 | EM2221964-064 | EM2221964-065 |
|  |  |  | Result <1.70 | Result 36.0 | Result 1.56 | Result <4.80 | Result <6.60 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <22.1 | <78.1 | <33.2 | <27.4 | <26.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 32.7 | <478 | 57.1 | 34.3 | 43.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 Insoluble metals | KF4 Insoluble metals | KF5BG Insoluble metals | WVDG03N Insoluble metals | WVDG03E Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-066 | EM2221964-067 | EM2221964-068 | EM2221964-069 | EM2221964-070 |
|  |  |  | Result 20.3 | Result 39.4 | Result <5.20 | Result <0.50 | Result <1.10 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <117 | <34.0 | <75.5 | 211 | <35.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 78.9 | 44.1 | <35.1 | 21.0 | <35.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Insoluble metals | WVDG03W Insoluble metals | WVDG04N Insoluble metals | WVDG04E Insoluble metals | WVDG04S Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-071 | EM2221964-072 | EM2221964-073 | EM2221964-074 | EM2221964-075 |
|  |  |  | Result <1.60 | Result <0.50 | Result 1.03 | Result <1.10 | Result 1.69 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <32.5 | 21.7 | <35.1 | 39.7 | 63.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 40.1 | 31.5 | 34.9 | 46.5 | <74.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Insoluble metals | WVDG05N Insoluble metals | WVDG05E Insoluble metals | WVDG05S Insoluble metals | WVDG05W Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-076 | EM2221964-077 | EM2221964-078 | EM2221964-079 | EM2221964-080 |
|  |  |  | Result <2.20 | Result 0.79 | Result <2.70 | Result <1.10 | Result 3.90 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <31.3 | <17.8 | <119 | <27.0 | <226 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <102 | <23.9 | 117 | <52.2 | 59.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06N Insoluble metals | WVDG06E Insoluble metals | WVDG06S Insoluble metals | WVDG06W Insoluble metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-081 | EM2221964-082 | EM2221964-083 | EM2221964-084 | -------- |
|  |  |  | Result 6.08 | Result 1.93 | Result 0.90 | Result <0.92 | ---- ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <51.0 | <31.0 | <17.2 | <15.9 | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 41.1 | <28.8 | 37.7 | 24.1 | ---- |

Environmental

CERTIFICATE OF ANALYSIS

Work Order	: EM2223849

Page	: 1 of 19

Amendment

Client

Contact

Address

: 1

: ALS WATER AND HYDROGRAPHICS PTY LTD

# :

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Bronwyn Sheen

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +6138549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 30-Nov-2022 16:15 |
| Order number | : ---- | Date Analysis Commenced | : 06-Dec-2022 |
| C-O-C number Sampler Site | : ---- : ROHAN OLIVER : ALS KERANG DUST | Issue Date | : 02-Mar-2023 18:41 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Arenie Vijayaratnam	Senior Inorganic Chemist	Melbourne Inorganics, Springvale, VIC

Eric Chau	Metals Team Leader	Melbourne Inorganics, Springvale, VIC

R I G H T  S O L U T I O N S | R I G H T  P A R T N E R

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- EM2223849 #1,6: Sample was observed to be turbid.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- Amendment (02/03/2023): This report has been amended following a change to the EG020TUG arsenic result reported for sample EM2223849 #27, 47, 51, 52, 53, 55, 56, 81 & 83 and barium results for #55 due to a transcription error. All details are recorded in client query 23MECC080 and a full investigation will be detailed in corrective action request 23MEC008.

- Dust sampled from 31/10/22 to 28/11/22.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- EM2223849 #1-15, 18-19, 21-28: Sample container was received with no liquid. 100ml of DI water was used to dissolve contents, perform analysis and for calculation of results.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 10:40 | 28-Nov-2022 10:50 | 28-Nov-2022 11:05 | 28-Nov-2022 11:25 | 28-Nov-2022 11:40 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-001 | EM2223849-002 | EM2223849-003 | EM2223849-004 | EM2223849-005 |
|  |  |  | Result 4.2 | Result 0.2 | Result <0.1 | Result 0.3 | Result 0.3 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 69 24.5 | 4 2.5 | <2 1.2 | 6 2.4 | 4 2.9 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 404 15.7 | 41 13.2 | 21 7.2 | 39 6.3 | 48 4.7 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 259 28.7 | 218 2.7 | 119 1.3 | 104 2.7 | 78 3.2 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 473 44.4 | 45 15.9 | 22 8.5 | 45 9.0 | 52 7.9 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 732 <33.5 | 263 3.91 | 141 <2.84 | 149 <5.22 | 130 <3.07 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <1900 | <110 | <138 | <88.9 | <84.2 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <2260 | 128 | 47.6 | <122 | <144 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD06 | WVDD007BG | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 10:10 | 28-Nov-2022 09:50 | 28-Nov-2022 13:10 | 28-Nov-2022 12:55 | 28-Nov-2022 13:00 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-006 | EM2223849-007 | EM2223849-008 | EM2223849-009 | EM2223849-010 |
|  |  |  | Result 3.1 | Result 0.2 | Result 0.2 | Result 0.1 | Result 0.2 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 51 6.8 | 3 1.4 | 2 1.4 | 2 1.3 | 3 1.1 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 112 9.6 | 24 6.4 | 24 4.3 | 21 4.8 | 18 4.6 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 158 9.9 | 105 1.6 | 71 1.6 | 79 1.4 | 76 1.3 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 163 19.5 | 27 8.0 | 26 5.9 | 23 6.2 | 21 5.9 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 321 <50.3 | 132 2.15 | 97 <26.4 | 102 51.3 | 97 <15.9 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <1480 | <64.8 | <83.2 | <82.8 | <73.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <778 | 55.1 | 55.2 | 65.1 | 70.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 12:45 | 28-Nov-2022 12:30 | 28-Nov-2022 11:05 | 28-Nov-2022 11:05 | 28-Nov-2022 11:05 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-011 | EM2223849-012 | EM2223849-013 | EM2223849-014 | EM2223849-015 |
|  |  |  | Result 0.2 | Result <0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | 4 ---- | <2 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 1.9 | ---- 1.2 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 30 ---- | 21 ---- | ---- 1.2 | ---- 0.8 | ---- 0.8 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 9.4 | ---- 5.3 | 17 ---- | 12 ---- | 12 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 155 ---- | 88 ---- | ---- 0.8 | ---- 0.6 | ---- 0.2 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 2.1 | ---- 1.3 | 12 ---- | 9 ---- | 2 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 34 ---- | 22 ---- | ---- 1.2 | ---- 0.8 | ---- 0.8 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 11.5 | ---- 6.6 | 17 ---- | 12 ---- | 12 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 189 ---- | 110 ---- | ---- 2.0 | ---- 1.4 | ---- 1.0 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- <32.8 | ---- 3.13 | 29 <0.21 | 21 <0.38 | 14 <0.15 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <211 | 55.1 | <893 | 50.3 | 23.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <159 | 46.4 | <12.4 | 10.7 | 7.37 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 11:05 | 28-Nov-2022 11:25 | 28-Nov-2022 11:25 | 28-Nov-2022 11:25 | 28-Nov-2022 11:25 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-016 | EM2223849-017 | EM2223849-018 | EM2223849-019 | EM2223849-020 |
|  |  |  | Result 0.2 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 2 1.9 | <2 1.7 | <2 1.7 | <2 1.5 | <2 1.4 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 28 0.1 | 24 <0.1 | 24 <0.1 | 22 0.1 | 20 0.4 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 2.1 | <2 1.7 | <2 1.7 | <2 1.5 | 5 1.4 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 30 2.2 | 24 1.7 | 24 1.7 | 22 1.6 | 20 1.8 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 31 3.54 | 24 <0.62 | 24 0.08 | 23 <0.35 | 25 <6.53 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <49.1 | <29.0 | 24.3 | 23.0 | 38.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 30.4 | 14.4 | 9.83 | <15.6 | <23.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG06N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 11:40 | 28-Nov-2022 11:40 | 28-Nov-2022 11:40 | 28-Nov-2022 11:40 | 28-Nov-2022 10:10 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-021 | EM2223849-022 | EM2223849-023 | EM2223849-024 | EM2223849-025 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.2 | <2 1.5 | <2 1.9 | <2 1.9 | <2 2.2 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 17 1.2 | 22 <0.1 | 27 0.1 | 27 <0.1 | 31 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 18 1.3 | <2 1.6 | <2 1.9 | <2 2.0 | <2 2.2 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 18 2.5 | 23 1.6 | 27 2.0 | 28 2.0 | 31 2.3 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 36 0.28 | 24 <0.28 | 28 0.15 | 29 <0.24 | 32 0.09 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 31.1 | 19.2 | 21.7 | <27.3 | 28.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 8.10 | 7.99 | 11.1 | 9.52 | 7.44 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06E | WVDG06S | WVDG06W | WVDD01 Soluble metals | WVDD02 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 10:10 | 28-Nov-2022 10:10 | 28-Nov-2022 10:10 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-026 | EM2223849-027 | EM2223849-028 | EM2223849-029 | EM2223849-030 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.1 | <2 1.1 | <2 1.7 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 16 0.1 | 16 <0.1 | 24 <0.1 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 2 1.2 | <2 1.1 | <2 1.7 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 17 1.3 | 16 1.1 | 24 1.7 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 19 0.05 | 17 ---- | 25 0.21 | ---- 118 | ---- 49.9 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | ---- | <0.050 | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 27.3 | 4.06 | 37.5 | 966 | 264 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <9.15 | 0.61 | 9.80 | 795 | 1430 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble metals | WVDD04 Soluble metals | WVDD05 Soluble metals | WVDD06 Soluble metals | WVDD007BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-031 | EM2223849-032 | EM2223849-033 | EM2223849-034 | EM2223849-035 |
|  |  |  | Result 4.41 | Result 30.4 | Result 13.6 | Result 136 | Result 2.13 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 853 | 154 | 152 | 1210 | 179 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 432 | 759 | 852 | 5100 | 1020 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble metals | KF2 Soluble metals | KF3 Soluble metals | KF4 Soluble metals | KF5BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-036 | EM2223849-037 | EM2223849-038 | EM2223849-039 | EM2223849-040 |
|  |  |  | Result 6.50 | Result 16.4 | Result 2.61 | Result 23.9 | Result 4.53 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 208 | 128 | 142 | 336 | 179 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 462 | 298 | 237 | 708 | 331 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N Soluble metals | WVDG03E Soluble metals | WVDG03S Soluble metals | WVDG03W Soluble metals | WVDG04N Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-041 | EM2223849-042 | EM2223849-043 | EM2223849-044 | EM2223849-045 |
|  |  |  | Result 1.29 | Result 0.28 | Result 0.50 | Result 0.30 | Result 0.20 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 103 | 28.4 | 27.1 | 4.05 | 4.59 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 13.3 | 14.6 | 15.1 | 4.22 | 7.18 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E Soluble metals | WVDG04S Soluble metals | WVDG04W Soluble metals | WVDG05N Soluble metals | WVDG05E Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-046 | EM2223849-047 | EM2223849-048 | EM2223849-049 | EM2223849-050 |
|  |  |  | Result 0.23 | Result ---- | Result 1.93 | Result 0.59 | Result 0.31 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | ---- | <0.050 | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 24.0 | 21.8 | 11.0 | 24.4 | 18.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 67.8 | 15.1 | 34.1 | 80.9 | 12.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble metals | WVDG05W Soluble metals | WVDG06N Soluble metals | WVDG06E Soluble metals | WVDG06S Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-051 | EM2223849-052 | EM2223849-053 | EM2223849-054 | EM2223849-055 |
|  |  |  | Result ---- | Result ---- | Result ---- | Result 0.16 | Result ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | <0.050 | <0.050 | <0.050 | ---- | <0.050 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 17.2 | 13.9 | 17.3 | 23.3 | 15.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 23.3 | 3.24 | 6.98 | 14.0 | 8.89 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06W Soluble metals | WVDD01 Insoluble metals | WVDD02 Insoluble metals | WVDD03 Insoluble metals | WVDD04 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-056 | EM2223849-057 | EM2223849-058 | EM2223849-059 | EM2223849-060 |
|  |  |  | Result ---- | Result <33.5 | Result 4.37 | Result <2.84 | Result <5.22 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | <0.050 | ---- | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 19.9 | <1900 | <110 | <138 | <88.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 9.79 | <2260 | 151 | 56.1 | <122 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD05 Insoluble metals | WVDD06 Insoluble metals | WVDD007BG Insoluble metals | KF1 Insoluble metals | KF2 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-061 | EM2223849-062 | EM2223849-063 | EM2223849-064 | EM2223849-065 |
|  |  |  | Result <3.07 | Result <50.3 | Result 2.55 | Result <26.4 | Result 77.5 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <84.2 | <1480 | <64.8 | <83.2 | <82.8 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <144 | <778 | 72.6 | 85.1 | 113 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 Insoluble metals | KF4 Insoluble metals | KF5BG Insoluble metals | WVDG03N Insoluble metals | WVDG03E Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-066 | EM2223849-067 | EM2223849-068 | EM2223849-069 | EM2223849-070 |
|  |  |  | Result <15.9 | Result <32.8 | Result 4.44 | Result <0.21 | Result <0.38 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <73.1 | <211 | 78.1 | <893 | 53.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 112 | <159 | 65.8 | <12.4 | 13.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Insoluble metals | WVDG03W Insoluble metals | WVDG04N Insoluble metals | WVDG04E Insoluble metals | WVDG04S Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-071 | EM2223849-072 | EM2223849-073 | EM2223849-074 | EM2223849-075 |
|  |  |  | Result <0.15 | Result 4.83 | Result <0.62 | Result 0.21 | Result <0.35 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 25.7 | <49.1 | <29.0 | 25.2 | 27.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 10.2 | 36.0 | 14.8 | 18.2 | <15.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Insoluble metals | WVDG05N Insoluble metals | WVDG05E Insoluble metals | WVDG05S Insoluble metals | WVDG05W Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-076 | EM2223849-077 | EM2223849-078 | EM2223849-079 | EM2223849-080 |
|  |  |  | Result <6.53 | Result 0.93 | Result <0.28 | Result 0.35 | Result <0.24 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 52.6 | 31.4 | 22.6 | 26.2 | <27.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <23.1 | 9.35 | 12.7 | 13.7 | 11.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06N Insoluble metals | WVDG06E Insoluble metals | WVDG06S Insoluble metals | WVDG06W Insoluble metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-081 | EM2223849-082 | EM2223849-083 | EM2223849-084 | -------- |
|  |  |  | Result ---- | Result 0.18 | Result ---- | Result 0.22 | ---- ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | <0.050 | ---- | <0.050 | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 43.6 | 62.3 | 26.1 | 39.1 | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 8.03 | <9.15 | 9.69 | 14.6 | ---- |

Environmental

CERTIFICATE OF ANALYSIS

Work Order	: EM2300219

Page	: 1 of 19

Client

Contact

Address

: ALS WATER AND HYDROGRAPHICS PTY LTD

# :

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Bronwyn Sheen

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +6138549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 11-Jan-2023 10:35 |
| Order number | : ---- | Date Analysis Commenced | : 18-Jan-2023 |
| C-O-C number Sampler Site | : ---- : RO : ---- | Issue Date | : 25-Jan-2023 15:57 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Dilani Fernando	Laboratory Coordinator	Melbourne Inorganics, Springvale, VIC

R I G H T  S O L U T I O N S | R I G H T  P A R T N E R

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- Sampling Period:28/11/2022-29/12/2022.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- EM2300219 #13-28; Sample container was received with no liquid. 100ml of DI water was used to dissolve contents, perform analysis and for calculation of results.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 12:20 | 29-Dec-2022 12:30 | 29-Dec-2022 12:35 | 29-Dec-2022 13:00 | 29-Dec-2022 13:25 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-001 | EM2300219-002 | EM2300219-003 | EM2300219-004 | EM2300219-005 |
|  |  |  | Result 0.3 | Result <0.1 | Result 0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 6 8.6 | <2 2.1 | 2 3.0 | <2 2.6 | <2 2.1 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 156 6.2 | 40 4.2 | 54 2.1 | 48 1.4 | 39 0.1 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 113 8.9 | 77 2.2 | 38 3.1 | 26 2.7 | 3 2.2 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 162 15.1 | 41 6.4 | 56 5.2 | 50 4.1 | 41 2.3 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 275 <6.00 | 118 <3.00 | 94 <4.00 | 76 <10.5 | 44 <3.50 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <383 | <47.3 | <70.5 | <59.5 | <61.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 250 | 45.5 | 52.1 | 42.2 | <45.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD06 | WVDD007BG | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 11:50 | 29-Dec-2022 11:35 | 29-Dec-2022 15:15 | 29-Dec-2022 15:40 | 29-Dec-2022 15:25 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-006 | EM2300219-007 | EM2300219-008 | EM2300219-009 | EM2300219-010 |
|  |  |  | Result 0.1 | Result 0.4 | Result 0.2 | Result 0.2 | Result 0.2 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 3 2.5 | 8 7.1 | 3 2.3 | 3 2.9 | 3 0.8 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 45 1.7 | 131 5.1 | 43 2.3 | 54 1.2 | 15 0.6 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 32 2.6 | 93 7.6 | 42 2.5 | 23 3.1 | 11 1.0 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 48 4.3 | 139 12.7 | 46 4.8 | 57 4.3 | 18 1.6 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 80 <3.10 | 232 <9.50 | 88 7.27 | 80 209 | 29 <73.0 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <73.5 | <325 | <68.6 | <96.1 | <81.4 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <82.2 | <383 | 61.3 | 70.5 | 63.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 15:30 | 29-Dec-2022 15:00 | 29-Dec-2022 12:40 | 29-Dec-2022 12:45 | 29-Dec-2022 12:50 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-011 | EM2300219-012 | EM2300219-013 | EM2300219-014 | EM2300219-015 |
|  |  |  | Result 0.1 | Result 0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | 2 ---- | 2 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 0.8 | ---- 0.9 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 14 ---- | 17 ---- | ---- 2.5 | ---- 2.7 | ---- 2.5 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.1 | ---- 3.8 | 39 ---- | 42 ---- | 40 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | <2 ---- | 69 ---- | ---- 0.2 | ---- 0.2 | ---- 0.2 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.9 | ---- 1.0 | 4 ---- | 4 ---- | 3 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 16 ---- | 19 ---- | ---- 2.5 | ---- 2.7 | ---- 2.6 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 1.0 | ---- 4.8 | 40 ---- | 43 ---- | 41 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 17 ---- | 88 ---- | ---- 2.7 | ---- 2.9 | ---- 2.8 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- 27.7 | ---- 7.20 | 44 0.20 | 47 <0.70 | 44 0.70 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <57.4 | <61.6 | 149 | 50.3 | 31.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 49.3 | 103 | 4.00 | 19.9 | 15.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 12:55 | 29-Dec-2022 13:05 | 29-Dec-2022 13:10 | 29-Dec-2022 13:15 | 29-Dec-2022 13:20 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-016 | EM2300219-017 | EM2300219-018 | EM2300219-019 | EM2300219-020 |
|  |  |  | Result 0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 2 2.7 | <2 2.6 | <2 2.7 | <2 2.3 | <2 2.2 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 42 0.2 | 41 0.3 | 42 0.2 | 36 0.5 | 34 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 4 2.8 | 5 2.6 | 4 2.7 | 8 2.3 | <2 2.2 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 44 3.0 | 41 2.9 | 42 2.9 | 36 2.8 | 35 2.3 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 48 1.90 | 46 0.19 | 46 <0.50 | 44 <0.80 | 36 1.14 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 30.6 | 11.6 | 18.0 | <18.3 | <18.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 23.7 | 7.40 | 17.7 | <58.3 | <18.4 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG06N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 13:30 | 29-Dec-2022 13:35 | 29-Dec-2022 13:40 | 29-Dec-2022 13:45 | 29-Dec-2022 12:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-021 | EM2300219-022 | EM2300219-023 | EM2300219-024 | EM2300219-025 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.6 | <2 0.8 | <2 0.7 | <2 2.5 | <2 2.3 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 25 0.5 | 13 0.1 | 11 0.3 | 40 0.1 | 37 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 8 1.6 | <2 0.9 | 6 0.7 | 2 2.5 | <2 2.4 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 26 2.1 | 14 1.0 | 11 1.0 | 40 2.6 | 38 2.5 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 34 <0.35 | 15 0.84 | 17 <0.70 | 42 0.30 | 39 <1.20 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <21.3 | <23.5 | <21.3 | 20.6 | <31.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <14.8 | <24.0 | <19.7 | 8.82 | <23.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06E | WVDG06S | WVDG06W | WVDD01 Soluble metals | WVDD02 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 12:05 | 29-Dec-2022 11:55 | 29-Dec-2022 12:10 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-026 | EM2300219-027 | EM2300219-028 | EM2300219-029 | EM2300219-030 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 3.0 | <2 3.0 | <2 2.6 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 48 0.7 | 48 0.5 | 41 0.4 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 11 3.1 | 8 3.1 | 6 2.7 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 49 3.8 | 49 3.6 | 42 3.1 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 60 <0.80 | 57 1.10 | 48 <2.70 | ---- 24.9 | ---- 6.40 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <20.5 | 24.3 | <40.8 | 228 | 90.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 13.6 | 31.2 | <49.0 | 945 | 266 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble metals | WVDD04 Soluble metals | WVDD05 Soluble metals | WVDD06 Soluble metals | WVDD07BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-031 | EM2300219-032 | EM2300219-033 | EM2300219-034 | EM2300219-035 |
|  |  |  | Result 47.0 | Result 7.30 | Result 4.07 | Result 36.0 | Result 7.30 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 389 | 102 | 86.0 | 227 | 183 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 414 | 395 | 268 | 1120 | 998 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble metals | KF2 Soluble metals | KF3 Soluble metals | KF4 Soluble metals | KF5BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-036 | EM2300219-037 | EM2300219-038 | EM2300219-039 | EM2300219-040 |
|  |  |  | Result 4.60 | Result 66.7 | Result 17.1 | Result 6.92 | Result 30.7 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 94.6 | 105 | 70.2 | 153 | 175 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 328 | 382 | 225 | 276 | 2521 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N Soluble metals | WVDG03E Soluble metals | WVDG03S Soluble metals | WVDG03W Soluble metals | WVDG04N Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-041 | EM2300219-042 | EM2300219-043 | EM2300219-044 | EM2300219-045 |
|  |  |  | Result 1.20 | Result 1.80 | Result 1.15 | Result 1.15 | Result 0.40 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 130 | 36.5 | 23.7 | 27.6 | 19.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 42.0 | 45.0 | 54.0 | 30.0 | 28.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E Soluble metals | WVDG04S Soluble metals | WVDG04W Soluble metals | WVDG05N Soluble metals | WVDG05E Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-046 | EM2300219-047 | EM2300219-048 | EM2300219-049 | EM2300219-050 |
|  |  |  | Result 0.40 | Result 1.00 | Result 1.20 | Result 0.60 | Result 0.60 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 22.0 | 18.5 | 19.5 | 22.5 | 18.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 34.0 | 37.2 | 19.5 | 30.5 | 41.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble metals | WVDG05W Soluble metals | WVDG06N Soluble metals | WVDG06E Soluble metals | WVDG06S Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-051 | EM2300219-052 | EM2300219-053 | EM2300219-054 | EM2300219-055 |
|  |  |  | Result 0.40 | Result 0.50 | Result 0.50 | Result 3.20 | Result 3.80 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 19.0 | 18.0 | 19.0 | 22.1 | 41.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 30.0 | 26.3 | 41.2 | 43.4 | 44.2 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06W Soluble metals | WVDD001 Insoluble metals | WVDD002 Insoluble metals | WVDD003 Insoluble metals | WVDD004 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-056 | EM2300219-057 | EM2300219-058 | EM2300219-059 | EM2300219-060 |
|  |  |  | Result 2.80 | Result <6.00 | Result <3.00 | Result <4.00 | Result <10.5 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 18.8 | <383 | <47.3 | <70.5 | <59.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 36.2 | 285 | 47.3 | 53.6 | 55.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD005 Insoluble metals | WVDD006 Insoluble metals | WVDD007BG Insoluble metals | KF1 Insoluble metals | KF2 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-061 | EM2300219-062 | EM2300219-063 | EM2300219-064 | EM2300219-065 |
|  |  |  | Result <3.50 | Result <3.10 | Result <9.50 | Result 8.40 | Result 367 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <61.3 | <73.5 | <325 | <68.6 | <96.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <45.8 | <82.2 | <383 | 82.7 | 141 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 Insoluble metals | KF4 Insoluble metals | KF5BG Insoluble metals | WVDG03N Insoluble metals | WVDG03E Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-066 | EM2300219-067 | EM2300219-068 | EM2300219-069 | EM2300219-070 |
|  |  |  | Result <73.0 | Result 40.0 | Result 8.20 | Result 0.60 | Result <0.70 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <81.4 | <57.4 | <61.6 | 2074 | 80.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 92.7 | 92.6 | 154 | 28.7 | 25.2 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Insoluble metals | WVDG03W Insoluble metals | WVDG04N Insoluble metals | WVDG04E Insoluble metals | WVDG04S Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-071 | EM2300219-072 | EM2300219-073 | EM2300219-074 | EM2300219-075 |
|  |  |  | Result 2.30 | Result 9.20 | Result 0.40 | Result <0.50 | Result <0.80 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 47.1 | 37.7 | 19.1 | 21.2 | <18.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 37.0 | 29.4 | 13.4 | 19.3 | <58.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Insoluble metals | WVDG05N Insoluble metals | WVDG05E Insoluble metals | WVDG05S Insoluble metals | WVDG05W Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-076 | EM2300219-077 | EM2300219-078 | EM2300219-079 | EM2300219-080 |
|  |  |  | Result 5.40 | Result <0.35 | Result 5.46 | Result <0.70 | Result 0.46 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <18.6 | <21.3 | <23.5 | <21.3 | 22.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <18.4 | <14.8 | <24.0 | <19.7 | 11.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06N Insoluble metals | WVDG06E Insoluble metals | WVDG06S Insoluble metals | WVDG06W Insoluble metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-081 | EM2300219-082 | EM2300219-083 | EM2300219-084 | -------- |
|  |  |  | Result <1.20 | Result <0.80 | Result 1.11 | Result <2.70 | ---- ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <31.5 | <20.5 | 26.8 | <40.8 | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <23.5 | 18.7 | 33.6 | <49.0 | ---- |