Department of Energy, Environment and Climate Action

Woodvale dust monitoring report q4 2022

30 th June 2023

CERTIFICATE OF APPROVAL FOR ISSUE OF DOCUMENTS

Client:	Department of Energy, Environment and Climate Action

Project Title:	Woodvale Dust Monitoring Report Q4 2022

Title:		Woodvale Dust Monitoring Report Q4 2022

Project: 	MV214940

Document Status:	Version 1

Date of Issue:		 30/06/2023

|  | Position | Name | Initials | Date |
| --- | --- | --- | --- | --- |
| Prepared by: | Northern Victorian Regional Manager | Rohan Oliver | RO | 05/04/23 |
| Technical Review by: | Victorian Data Manager | Karen Bootland | KB | 30/06/23 |
| Client: | Department of Energy, Environment and Climate Action (DEECA) | Michael Fitzgerald | DEECA |  |

For further information on this report, contact:

Name:		Rohan Oliver
Title:		Northern Victorian Regional Manager
Address:	94 Koondrook Rd, Kerang VIC 3579
E-mail:		Rohan.Oliver@alsglobal.com

© ALS Water and Hydrographics Pty Ltd

This report is prepared for the Client, Department of Energy, Environment and Climate Action for the purpose of monitoring dust at the former Bendigo Mine Woodvale site which is currently in care and maintenance. An unrestricted copy of this report will be shared with the public via the Earth Resources Regulation website.

Disclaimer

This document has been prepared for the Client above and is to be used only for the purposes for which it was commissioned. While all due skill and attention has been taken in collecting, validating and providing the data contained in this report, ALS shall not be liable in any way for loss of any kind including damages, costs, interest, loss of profits or special loss or damage, arising from any error, inaccuracy, incompleteness or other defect in this information. No warranty is given as to its suitability for any other purpose.

ALS Water and Hydrographics Pty Ltd ABN 94 105 060 320

ALS Australia Pty Ltd trading as ALS Hydrographics

Table of Contents

Executive Summary	4

1.	Introduction	6

2.	Background	6

2.1	Scope and Objective	7

2.2	Regulatory Environment	8

2.3	What is Dust	8

3.	Measurement of Dust	9

3.1	Site Locations	9

3.2	Deposited Matter	10

3.3	Directional Deposited Dust Monitoring	12

3.4	Metals/Metalloids Monitoring in Dust	13

3.5	Ash Content	13

3.6	Dust Monitoring Standards	13

3.7	Dust Monitoring Program	14

4.	Dust Deposition Results	15

4.1	Woodvale Evaporation Ponds – Q4 2022	15

4.1.1 Deposition of Total Insoluble Matter	15

4.1.2 Mineral Content in Dust	16

4.1.3 Metals/Metalloids in Dust	17

4.1.4 Woodvale Directional Dust Deposition	18

4.2	Review of Quality Assurance and Quality Control	21

5.	Summary of Results	22

6.	Further Considerations	22

7.	References	23

8.	Tables	25

9.	Supplementary Data	45

9.1	Arsenic	45

9.2	Barium	47

9.3	Manganese	49

9.4	Trend Analysis Graphs	51

10.	Appendix A – Supplementary Information	59

11.	Appendix B – Laboratory Reports	61

12.	: ROHAN OLIVER	16

Table of Figures

Figure 1 - Monitoring Locations at Woodvale Ponds	9

Figure 2 - Control Site Location WVDD07 (BG)	10

Figure 3 - Dust Deposition Gauge WVDD01	11

Figure 4 - Directional Dust Deposition Gauge WVDG06	12

Figure 5 - Woodvale - Total Insoluble Matter (TIM) g/m2/month – Q4 2022	15

Figure 6 - Woodvale - Total Ash Content g/m2/month – Q4 2022	16

Figure 7 - Woodvale - Ash Content/TIM Ratio – Q4 2022	16

Figure 8 - Woodvale - Total Arsenic/Total Solids mg/kg – Q4 2022	17

Figure 9 - Woodvale - Directional Dust Deposition Results - Total Solids g/m2/month – Q4 2022 (North/South)	18

Figure 10 - Woodvale - Directional Dust Results – Arsenic (mg)/dust(kg) – Q4 2022 (North/South)	19

Figure 11 - Woodvale - Directional Dust Deposition Results - Total Solids g/m2/month – Q4 2022 (East/West)	19

Figure 12 - Woodvale - Directional Dust Results - Arsenic (mg/dustkg) – Q4 2022 (East/West)	20

Figure 13 - Woodvale - Arsenic in Soluble Fraction – Q4 2022	45

Figure 14 - Woodvale - Arsenic in Insoluble Fraction – Q4 2022	45

Figure 15 - Woodvale - Arsenic in Soluble Fraction Year Review (2021-22)	46

Figure 16 - Woodvale - Arsenic in Insoluble Fraction Year Review (2021-22)	46

Figure 17 - Woodvale - Barium in Soluble Fraction – Q4 2022	47

Figure 18 - Woodvale - Barium in Insoluble Fraction – Q4 2022	47

Figure 19 - Woodvale - Barium in Soluble Fraction Year Review (2021-22)	48

Figure 20 - Woodvale - Barium in Insoluble Fraction Year Review (2021-22)	48

Figure 21 - Woodvale - Manganese in Soluble Fraction – Q4 2022	49

Figure 22 - Woodvale - Manganese in Insoluble Fraction – Q4 2022	49

Figure 23 - Woodvale - Manganese in Soluble Fraction Year Review (2021-22)	50

Figure 24 - Woodvale - Manganese in Insoluble Fraction Year Review (2021-22)	50

Figure 25 - Woodvale - Total Insoluble Matter (TIM) - 2021-2022	51

Figure 26 - Woodvale - Total Ash Content - 2021-2022	52

Figure 27 - Woodvale - Ash Content/TIM Ratio - 2021-2022	53

Figure 28 - Woodvale - Total Arsenic/Total Solids - 2021-2022	54

Figure 29 - Woodvale - DDG Total Solids Comparison North/South - 2021-2022	55

Figure 30 - Woodvale - Arsenic/Total Solids Comparison - North/South 2021-2022	56

Figure 31 - Woodvale - DDG Total Solids Comparison East/West - 2021-2022	57

Figure 32 - Woodvale - Arsenic/Total Solids Comparison - East/West - 2021-2022	58

Table of Tables

Table 1 - Woodvale Dust Deposition Analysis Results Q4, 2022	25

Table 2 - Woodvale Directional Dust Deposition Analysis Results Q4, 2022	32

# Executive Summary

ALS is engaged by the Department of Energy, Environment and Climate Action, Earth Resources Regulation (DEECA(ERR)) to undertake dust monitoring at the Woodvale Evaporation Ponds Complex. This report provides results and analysis of data collected from sampling in Q4 October - December 2022.

DEECA(ERR) have responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its owner Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, DEECA engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites at Kangaroo Flat and Woodvale.

This report analyses the data gathered in Q4 2022 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results. The results of dust monitoring in the first half of 2021 were published in a report by CDM Smith. For reporting since Q3 2021, ALS completed the collection of samples/monitoring, analysis of lab results (Springvale ALS Laboratory - NATA accredited) and drafted the dust monitoring reports.

The Woodvale Evaporation Ponds are located 13 km north of Bendigo. The ponds were formerly used to hold groundwater extracted from dewatering historic underground mine workings at Kangaroo Flat. The site is currently in care and maintenance. DEECA(ERR) is continuing to visually monitor the site by regular site visits, keeping an eye on weather conditions, and proactively applying a range of dust control measures. These measures include covering ponds with water, vegetation cover, mulch application and keeping a water/polymer spray truck on standby, should the need arise to use such control measures.

To maintain continuity and comparability of data, dust monitoring was undertaken at the same locations and a similar methodology and analysis process was followed in this report as per other quarters. Upon routine monthly monitoring, ALS field staff observed all monitoring sites where in good working order with no obvious variations to sites that would reflect abnormalities in results.

The original scope of dust monitoring works at Woodvale was to operate and maintain six depositional dust gauges and two directional dust deposition gauges. ALS has added a further three monitoring sites to the dust monitoring program at Woodvale, i.e. one control site to obtain background data, and two additional directional dust deposition gauges at the east and west of the Woodvale site, to provide more complete data from directional dust monitoring. These sites have been in operation since 1 October 2021.

The purpose of dust monitoring is to assure the local community that dust issues are being minimised and managed effectively. The objective of the dust monitoring program at Woodvale is to gather baseline data and report exceedances when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will inform decision making on rehabilitation requirements and measures required to ensure works meet air quality standards (where available).

The results for Q4 2022 recorded slightly elevated results in comparison to previous reports supplied (Kralcopic Report in 2018, CDM Smith Report for the first half of 2021 and ALS Reports for Q3/Q4 2021, and Q1/Q2/Q3 2022) for sites WVDD01 and WVDD06. These results are surprising as the ponds were wet because of the significant rainfall events that occurred through this monitoring period. This indicates that they were most likely influenced by agricultural activities located nearby the monitoring sites.

As done previously in historic reports, the following questions are answered for Q4 2022:

- Did the dust deposition rates recorded in Q4 2022, exceed the adopted dust deposition criteria of 4g/m2/month, measured as Total Insoluble Matter (TIM)?

In Q4 2022, there were seven instances where the dust levels were over the 4 g/m2/month criteria used in this report as follows:

- WVDD01 (S), during October 2022 (14.8 g/m2/month)

- WVDD03 (N), during October 2022 (4.1 g/m2/month)

- WVDD06 (W), during October 2022 (9.7 g/m2/month)

- WVDD01(S), during November 2022 (28.7 g/m2/month)

- WVDD06 (W), during November 2022 (9.9 g/m2/month)

- WVDD01 (S), during December 2022 (8.9 g/m2/month)

- WVDD07(BG), during December 2022 (7.6 g/m2/month)

However, it should be noted that one of these occurrences was at the background site WVDD07 (BG) and therefore cannot be attributed to the Woodvale Evaporation Ponds. When the data from all source monitoring sites excluding WVDD07 (BG) were averaged for Q4 2022, an average of 5.7g/m2/month was recorded. This value is above the 4 g/m2/month criteria, and the inferred background plus 2 g/m2/month criteria in the Mining PEM.

The dust levels recorded at WVDD01(S) and WVDD06 (W) exceeded in October and November and WVDD01again in December are most likely influenced by agricultural activities located nearby to the monitoring sites as the ponds were wet through this monitoring period. WVDD06 will be relocated in coming months.

The ponds within the site contained water in all ponds because of the significant rain events across Victoria through Spring/Summer months.

The surrounding land use to the north, south and west is predominantly agricultural farmland with reserved parklands to the east of the Woodvale site. ERR conduct daily site inspections to ensure dust control measures (water cover, dampness, foliage) are adequate.

- Are dust deposition rates recorded in Q4 2022 comparable to available historic results in this area?

Overall, results for Q4 were higher at WVDD01 during Q4. WVDD06 was over the 4.0 g/m2/month although it was less than results collected in Q3.

Overall, results for Q4 2022 were higher than previous results recorded in the Kralcopic Report for Q4 2018, CDM Smith Report for the first half of 2021, and ALS Reports for Q3 2021, Q4 2021 and Q1/Q2 and Q3 2022.

- Are arsenic concentrations recorded in deposited dust in Q4 2022, comparable to available historic results in this area?

Yes they are comparable. When comparing sample location WVDD06 it was observed that arsenic was elevated for the measure of soluble fraction, which was not represented at the background site.

It is noted that in addition to the results above, the directional dust deposition gauge results for Q4 2022 showed that, there is no clear overall net gain in dust or arsenic concentrations resulting from the Woodvale Evaporation Ponds site.

# Introduction

ALS was engaged by the Earth Resource Regulation team in the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Woodvale Evaporation Ponds Complex from the start of January 2021. On 1 January 2023 ERR joined a new Government department, the Department of Energy, Environment and Climate Action (DEECA). The ongoing engagement of ALS is now with DEECA Earth Resources Regulation (DEECA(ERR)). This report provides results and analysis of data collected from sampling in Q4 2022. ALS undertook the field component of maintaining dust gauges and replacing sample containers, as well as laboratory analysis of samples collected.  Samples were collected from locations specified by DJPR on a routine monthly basis. These locations are displayed in Section 3.1 of this report.

# Background

In April 2021, ERR inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its owner Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites at Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust, and in particular the arsenic content of the dust. This report analyses the data gathered in Q4 2022 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results published in:

- ALS Report for Q3 2022

- ALS Report for Q2 2022

- ALS Report for Q1 2022

- ALS Report for Q4 2021

- ALS Report for Q3 2021

- CDM Smith Report for the first half of 2021

- Kralcopic Report for Q4 2018.

All reports are referenced in this document. To maintain continuity and comparability of results, this document has been prepared in a similar format to the CDM Smith Report.

The Woodvale Evaporation Ponds are located 13 km north of Bendigo. The ponds were formerly used to hold groundwater extracted from dewatering historic underground mines including Kangaroo Flat and New Moon mines. The site is currently in care and maintenance while ERR develop the rehabilitation plan for the site. DEECA(ERR) is continuing to visually monitor the site by regular site visits, keeping an eye on weather conditions, and proactively applying a range of dust control measures. Dust suppression measures include ensuring water or vegetation cover, application of mulch and keeping a water/polymer spray truck on standby, should the need arise to use such control measures.

To maintain continuity and comparability of data, monitoring was undertaken at the same locations as previously, and similar methodology and analysis used. This report builds on the previous results and provides baseline information to potentially inform decisions around some aspects of site rehabilitation.

## Scope and Objective

The scope of dust monitoring works at Woodvale is to operate and maintain seven depositional dust gauges and four directional dust deposition gauges including gathering samples and sending off to the laboratory for analysis, analysing the data from the laboratory and producing quarterly dust monitoring reports consolidating results and comparing to historic and guideline levels.

All depositional dust gauges are located at their historic locations with the following naming conventions since the dates specified below:

| - WVDD01 (S) Jan 2021 - WVDD02 (N) Jan 2021 - WVDD03 (N) Jan 2021 - WVDD04 (E) Jan 2021 | - WVDD05 (S) Jan 2021 - WVDD06 (W) Jan 2021 - WVDD07 (BG) Sep 2021 |  |
| --- | --- | --- |

The location of the six depositional dust gauges at the Woodvale site can be seen in Figure 1. The location of the background depositional dust gauge is in Figure 2 which serves as a control site.

Similarly, four directional dust deposition gauges were installed since the dates specified below at the following locations:

- WVDG03 (N) Jan 2021

- WVDG04 (E) Oct 2021

- WVDG05 (S) Jan 2021

- WVDG06 (W) Oct 2021

These directional gauges are located next to the depositional dust gauges: WVDD03 (N), WVDD04 (E), WVDD05 (S) and WVDD006 (W).

Laboratory analysis requirements were provided by DEECA(ERR) and were in line with historical requirements during the previous monitoring program undertaken by Kralcopic to meet it’s legislative requirements. This alignment maintains continuity and comparability of results.

The purpose of dust monitoring is to assist ERR monitor the effectiveness of dust minimisation actions and to also identify if dust leaving the site is at unacceptable levels and subsequently manage dust risks if they arise. The objective of the dust monitoring program at Woodvale is to gather baseline data when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will potentially inform decisions around some aspects of site rehabilitation and any measures required to ensure works meet air quality standards (where available).

As done previously, the following considerations were included in assessment of Q4 2022 data:

- Did the dust deposition rates recorded in Q4 2022, exceed the adopted dust deposition criteria of 4g/m2/month, measured as Total Insoluble Matter (TIM)?

- Are dust deposition rates recorded in Q4 2022, comparable to available historic results in this area?

- Are arsenic concentrations recorded in deposited dust in Q4 2022, comparable to available historic results in this area?

## Regulatory Environment

The Environment Protection Authority (EPA) has overarching responsibility for administering legislation related to the air quality across Victoria. The EPA administers the Environment Protection Act 2017 and the Environment Protection Regulations 2021 under which Environmental Reference Standards (ERS) and related Guidelines are provided.

The purpose of the ERS is to support the protection of human health and the environment from harms posed by pollution and waste. The ERS does this by providing benchmarks to assess and report on environmental conditions in the whole or any part of Victoria.

The ERS seeks to achieve this purpose by:

- identifying environmental values to be achieved or maintained in the whole or any part of Victoria; and

- specifying indicators and objectives to be used to measure, determine or assess whether those environmental values are being achieved, maintained or threatened.

The ERS provides guidance, objectives and levels on values where the environment may be impacted and incorporates the Ambient Air Quality National Environment Protection Measure (NEPM AAQ).

The main guidance document used for the purpose of this report is EPA Publication 1191 – Protocol for Environmental Management – Mining and Extractive Industries (Mining PEM). The criteria in the Mining PEM used for this report has been carried over to the EPA’s latest Guideline for Assessing and Minimising Air Pollution in Victoria, Publication 1961, February 2022. As stated in PEM, Results of monitoring should not exceed 4g/m2/month (no more than 2g/m2/month above background) as a monthly average.

Under the new EPA legislation, duty holders have responsibility under the general environmental duty to apply controls to eliminate or minimise risks of harm to human health and the environment as far as reasonably practicable. This means that the primary focus is on applying effective elimination, minimisation and control measures. Monitoring is for the purpose of determining the effectiveness of control measures.

## What is Dust

Dust is typically not classified due to its composition but based on its particle size, as follows:

- Deposited matter - any particles that fall out of suspension in the atmosphere.

- Total Suspended Particles (TSP) - particles suspended or entrained in the air. Typically, this is particles of 30 µm (0.03 mm) equivalent aerodynamic diameter or less. Larger particles tend not to become suspended.

- PM10 - particles 10 µm equivalent aerodynamic diameter or less.

- PM2.5 - particles 2.5 µm equivalent aerodynamic diameter or less.

Dust particle size is an important consideration influencing dispersion and transport in the atmosphere and potential effects on human health. Human activities (e.g. energy use, transport, industrial activities etc) can affect the air quality, in particular airborne dust.

Potential sources of dust are:

- natural sources such as dust storms, agricultural dust, bushfires, vegetation, pollen and fungi; and

- anthropogenic sources such as mines sites, industry, roads and vehicles, construction sites, domestic and diffuse sources (CDM Smith, 2021).

# Measurement of Dust

## Site Locations

Locations of depositional dust gauges are as presented in Figure 1 and Figure 2. Directional depositional dust gauges WVDG03 (N), WVDG04 (E), WVDG05 (S) and WVDG06 (W) have been installed next to the respective depositional gauges as mentioned above.

Figure 1 - Monitoring Locations at Woodvale Ponds

Figure 2 - Control Site Location WVDD07 (BG)

Samples were collected in accordance with the Standard AS/NZS 3580.10.1 - Methods for sampling and analysis of ambient air - Determination of particulate matter - Deposited matter - Gravimetric method and ALS Dust Sampling and Depositional Dust Gauge Work Procedure MAT-MAP1000 (See Appendix A).

ALS Laboratory in Springvale analysed the samples for the following:

- Total deposited matter

- Total ash content

- Metals/metalloids, in the soluble, insoluble and ash content fraction of dust.

## Deposited Matter

The Standard (AS/NZS 3580.10.1:2016) sets out a method (used in this report) for sampling particulate matter that is deposited from the atmosphere. This Standard also sets out procedures for the gravimetric (i.e. by weighing) determination of the mass deposition rate of insoluble/soluble solids, ash/combustible matter, and total solids from ambient air.

The method provides an estimate of the mean surface concentration of deposited matter settling from the air over a sampling period, typically of one month. Particulate matter deposition rates of 0.1 g/m2/month and above may be determined using a monthly sampling period. The sample obtained by the sampling procedure specified may be subjected to physical or chemical analysis.

Over a given sampling period, particles that settle from the ambient air are collected in a vessel (see sampling bottle in Figure 3 below) and retained together with any rainwater. The sample is passed through a sieve to remove any extraneous matter (e.g. leaves, insects), and the sieved sample containing the deposited matter is transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of the dried insoluble solids is gravimetrically determined.

The ash and combustible matter content are determined by loss on ignition of the insoluble solids. Soluble solids are determined from the filtrate. The total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass deposition rate of deposited matter is then calculated from the mass of solids obtained, the funnel cross-sectional area and the exposure period. (AS/NZS 3580.10:2016)

Figure 3 - Dust Deposition Gauge WVDD01

## Directional Deposited Dust Monitoring

The Standard (AS/NZS 3580.10.2:2013) states that over a given period, particles which impinge on the vertical, internal surface of four collecting cylinders of the directional dust gauge are sampled in collecting pots at the bases of the cylinders. Refer to Figure 4.

The samples are passed through a sieve to remove any extraneous matter (e.g. leaves, insects), with the sieved sample containing the impinged matter subsequently transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of dried insoluble solids determined gravimetrically. Soluble solids are determined by evaporation of the filtrate and gravimetric analysis.

The ash and combustible matter content are determined by loss on combustion of the insoluble solids. Total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass flux rate of impinged matter for each collecting cylinder is calculated from the mass of solids obtained, the slot cross-sectional area and the exposure period.

A directional dust gauge consists of four vertical collecting cylinders, each with a slot of dimensions 45 mm by 340 mm, commencing approximately 10 mm from the top of the cylinder. The edges of each slot shall be at right angles to the width of the slot and not radial (see Figure 4). The cylinders shall be mounted at 90o to each other with the slots facing outwards. An easily detachable collecting pot shall be fitted to the bottom of each collecting cylinder. The combined length of a collecting cylinder and its attached collecting pot shall be at least 640 mm, with the collecting pot having a capacity of approx. 1 L.

The collecting cylinders and pots shall be constructed of chemically resistant material such as polyvinyl chloride. The external diameter of the collecting cylinders shall be 75 mm. The collecting cylinders shall be located with their axes at the corners of a 150 mm square mounting plate. (AS/NZS 3580.10.2:2013)

Figure 4 - Directional Dust Deposition Gauge WVDG06

## Metals/Metalloids Monitoring in Dust

The inclusion of the analysis of metals/metalloids in the dust samples assist in identify possible sources of contamination if present.

Analysis for insoluble matter, soluble matter and ash content was undertaken for the following:

- Arsenic

- Barium

- Manganese

It is understood that there are no specific threshold values for metals and metalloids in deposited dust either in the former Mining Licence or within the ERS. (CDM Smith, 2021)

## Ash Content

Ash Content is the remaining material after the sample has been combusted in the laboratory. Ash content provides an indication of the mineral content (or soil dust) of the sample. The mineral content may be attributable to mine site contributions, but may also be attributable to other sources such as agriculture, unsealed roads, etc.

## Dust Monitoring Standards

Results were compared against relevant dust deposition rate criteria described by EPA Publication 1191  (2007) as shown below which has been adopted in EPA Publication 1961 (2022):

The following points apply to the criteria:

- Results of monitoring should not exceed 4 g/m2/month (no more than 2 g/m2/month above background) as a monthly average.

- The 2 g/m2/month criteria are used when baseline data on deposited dust levels exist, while the 4 g/m2/month criteria is used when no baseline data exists.

- If dust levels exceed these values, then site management practices should be reviewed, and additional dust controls investigated

- The criteria refer to all sources of deposited matter (including sources from mines, agriculture, unsealed roads, etc) and cumulative impacts.

- The criteria states that in some cases, a mine may increase deposited dust levels by up to 2 g/m2/month. However, the total deposited dust level (including sources from mines, agriculture, unsealed roads, etc) must not exceed 4 g/m2/month.

Deposited matter (dust) can be used as an indicator of the effectiveness of site management practices and the potential for offsite nuisance dust (fugitive dust). Deposited dust monitoring, when conducted over a set time period, is useful for examining trends and evaluating deviation from long term trends with respect to site activities.

## Dust Monitoring Program

The current method of undertaking depositional dust monitoring is considered sufficient as the site is in care and maintenance i.e. no new pollution is being introduced to the site from industry. There is a considerable body of knowledge about the contamination level at the site, so all that is required at this stage is to determine whether this contamination is becoming air borne and becoming a health hazard or problem to the nearby landholders. Depositional dust monitoring provides a range of useful information to be able to understand what’s happening at this site and its surrounds.

Depositional dust monitoring can provide the following invaluable information:

- test the effectiveness of dust minimisation, control and management measures

- identify key problematic sources, or groups of sources on larger more complex sites

- identify where dust sensitivities may occur

- characterise temporal or spatial trends

- metallic/metalloid concentrations are useful for health effect considerations.

The environmental conditions during Q4 2022 across Bendigo resulted in high rainfall totals in October. This was not just localised to Bendigo, but high rainfall had occurred across most of Victoria. Rainfall during November and December were more representative of summer totals. Water levels in ponds located at Woodvale are not continuously monitored by ALS, although site observations from days where staff were on site to collect dust samples observed ponds were in a wet state. Therefore, it is unlikely any dust was emitted from these ponds during the monitoring period. A new weather station was installed by ALS in 2023 which will help in future analysis predominantly by providing wind direction and speed. Weather station data during Q4 has been sourced from the BOM daily weather observations from site location, Bendigo Airport (station 081123).

# Dust Deposition Results

## Woodvale Evaporation Ponds – Q4 2022

Dust deposition analysis results are presented in Table 1 - Woodvale Dust Deposition Analysis Results and the accompanying Laboratory Certificates of Analysis are presented in Appendix B - Laboratory Reports.

Where results were reported as “less than” (<) a specified number (laboratory reporting limit) the number reported was adopted for use in the graphical displays. This approach adopts a conservative value for the number being reported.

## 4.1.1 Deposition of Total Insoluble Matter

Total Insoluble Matter (TIM) met the recommended criteria of less than 4 g/m2/month on 7 of 21 samples during Q4 2022. See Figure 5 below.

Dust deposition rate exceeded the assessment criteria at the sites below:

- WVDD01 (S), during October 2022 (14.8 g/m2/month)

- WVDD03 (N), during October 2022 (4.1 g/m2/month)

- WVDD06 (W), during October 2022 (9.7 g/m2/month)

- WVDD01(S), during November 2022 (28.7 g/m2/month)

- WVDD06 (W), during November 2022 (9.9 g/m2/month)

- WVDD01 (S), during December 2022 (8.9 g/m2/month)

- WVDD07(BG), during December 2022 (7.6 g/m2/month)

When the data from all source monitoring sites except WVDD07 (BG) (the background reference location) were averaged for Q4 2022, an average of 5.7g/m2/month was recorded. This value is above the 4 g/m2/month criteria and the inferred background plus 2 g/m2/month criteria in the Mining PEM. The average of 5.7g/m2/month is biased by the high figure from WVDD001result of 28.7 g/m2/month. The results at the WVDV07 (BG) site were also recorded to exceed during December 2022.

Figure 5 - Woodvale - Total Insoluble Matter (TIM) g/m2/month – Q4 2022

Red dotted line is the criteria used for the purpose of this report.

## 4.1.2 Mineral Content in Dust

The fraction of dust measured as ash content is displayed below in Figure 6. As expected, the increased levels in TIM (Figure 5) generally corresponds with total ash content in Figure 6.

Figure 6 - Woodvale - Total Ash Content g/m2/month – Q4 2022

To gain a general understanding of what proportion of the sample is ash content i.e., mineral dust, the ratio of total ash content and TIM has been plotted in Figure 7.

Figure 7 - Woodvale - Ash Content/TIM Ratio – Q4 2022

## 4.1.3 Metals/Metalloids in Dust

Graphical presentations of soluble and insoluble metals (arsenic, barium and manganese) are provided in Section 9 of this report. Arsenic, Barium and Manganese concentrations for Q4 2022 were within historical ranges and results being comparable to results reported in the CDM Smith Report for the first half of 2021 and in the ALS Reports for Q3/Q4 2021, and Q1/Q2 2022. Q3 2022 was not used for comparisons due to samples ashed.

To provide an understanding of total arsenic concentrations measured in dust relative to concentrations measured in soils within the region, total arsenic concentration has been displayed below Figure 8 in mg/kg. This is calculated by adding soluble and insoluble arsenic µg/m2/month dividing by the total solids g/m2/month.

This ratio has been plotted as this provides a value which could be compared to the figures quoted in the guideline, Contaminated Sites Management Series - Assessment Levels for Soil, Sediment and Water. The assessment level for soil for arsenic is 100 mg/kg for standard residential areas in this guideline.

Figure 8 - Woodvale - Total Arsenic/Total Solids mg/kg – Q4 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Total arsenic concentrations for Q4 ranged from 0 to 17.8 mg/kg with a mean of 4.8 mg/kg for Q4 2022.

Highest concentrations were recorded in October 2022, 17.8mg/kg at WVDD03 (N). This value is still well below the 100 mg/kg assessment level mentioned above. The high Tim reading recorded at WVDD01was not reflected in higher readings in total arsenic.

## 4.1.4 Woodvale Directional Dust Deposition

Results for directional dust deposition gauges are summarised in Table 2 - Woodvale Directional Dust Deposition Analysis Results, with Laboratory Analytical Results attached in Appendix B. As mentioned previously, for the purpose of continuity, the same approach for review of dust from the directional dust deposition gauges as in the CDM Smith Report has been applied here for Q4 2022 results.

The results from the northerly and southerly facing quadrants at the two directional dust deposition gauges WVDG03 (N) and WVDG05 (S) have been compared in Figure 9. Figure 9 shows the net gain or loss in dust deposition downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

To undertake this analysis, the Northern quadrant dust gauge at WVDG03 is assumed to measure dust coming onto the site in Northerly winds. This is then compared to the Northern quadrant dust gauge at WVDG05. The variance is then used to indicate dust contributions to the air from the site. A positive variance indicates an increase in dust to the atmosphere from the ponds. A negative variance indicates the ponds have removed more dust from the air than they contribute. This theoretical analysis is repeated for the site from each axis for directional dust gauge locations.

From the graph below (Figure 9) it can be seen that there is a small net gain in dust observed through the Q4 2022 monitoring period.

Such small gains can be expected and can be influenced by both the Woodvale site and environmental conditions. Note that there is agricultural farmland to the north, south and west of the Woodvale site and reserved parklands to the east of the Woodvale site, which can all contribute to the dust levels at these sampling locations.

Figure 9 - Woodvale - Directional Dust Deposition Results - Total Solids g/m2/month – Q4 2022 (North/South)

Similarly, the concentration of arsenic was compared using the same methodology as above, and it was found that low net gains and losses occurred for 3 of the 6 values recorded during Q4 2022.

Figure 10 presents the net gain and loss in total arsenic. Downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

Figure 10 - Woodvale - Directional Dust Results – Arsenic (mg)/dust(kg) – Q4 2022 (North/South)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 10- No recorded high figures were seen in any of the two directional gauges or six depositional dust gauges.

Figure 11 below displays the net gain or loss both upwind and downwind from east and west and no clear pattern is identified during Q4 2022 with both gains and losses occurring throughout the period.

Figure 11 - Woodvale - Directional Dust Deposition Results - Total Solids g/m2/month – Q4 2022 (East/West)

Figure 12 presents the net gains and losses in total arsenic downwind of the site compared to upwind of the site, for both east and west facing quadrants.

For the purpose of providing an understanding of total arsenic concentrations measured in dust relative to arsenic concentrations measured in soils in the assessment region, total arsenic is presented as mg/kg (based on the sum of soluble and insoluble arsenic µg/m2.month divided by total solids g/ m2.month).

Figure 12 - Woodvale - Directional Dust Results - Arsenic (mg/dustkg) – Q4 2022 (East/West)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## Review of Quality Assurance and Quality Control

Field sampling was undertaken in accordance with the following Standards:

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

Sampling methodology and locations are consistent with sampling previously undertaken to maintain continuity and comparability of results.

ALS Environmental Laboratory has NATA accreditation for the following Ambient Air Parameters:

ALS currently does not have NATA accreditation for metals analysis of ambient air but is in the process of gaining accreditation. Nevertheless, ALS is operating as though this accreditation is in place and expects to finalise the accreditation process in 2022.

Internal laboratory quality control measures note the following:

- No Method Blank value outliers occurred.

- No laboratory control outliers occurred.

- No Matrix Spike (MS) or Matrix Spike Duplicate (MSD) results reported.

- No laboratory duplicate results reported.

- No holding time outliers occurred.

- No quality control sample frequency outliers occurred (based on NEPM 2013 B3 quality control specification).

# Summary of Results

Results are discussed in detail throughout the relevant sections. In summary, the overall results for Q4 2022 were comparable to previous results.

The key findings for Q4 2022 are as follows:

There were seven instances where the dust level exceeded the 4 g/m2/month criteria used in this report.  One of these instances occurred at the Background site (WVDD07 (BG)) and is not attributed to the Woodvale Evaporation Ponds.

- WVDD01 (S), during October 2022 (14.8 g/m2/month)

- WVDD03 (N), during October 2022 (4.1 g/m2/month)

- WVDD06 (W), during October 2022 (9.7 g/m2/month)

- WVDD01(S), during November 2022 (28.7 g/m2/month)

- WVDD06 (W), during November 2022 (9.9 g/m2/month)

- WVDD01 (S), during December 2022 (8.9 g/m2/month)

- WVDD07(BG), during December 2022 (7.6 g/m2/month)

When the data from all source monitoring sites except WVDD07 (BG) (the background reference location) were averaged for Q4 2022, an average of 5.7g/m2/month was recorded. This value is above the 4 g/m2/month criteria and the inferred background plus 2 g/m2/month criteria in the Mining PEM. The average of 5.7g/m2/month is biased by the high figure from WVDD001result of 28.7 g/m2/month. The results at the WVDV07 (BG) site were also recorded to exceed during December 2022.

- Results for Barium and Manganese were higher at WVDD01 during October for in soluble fraction. Manganese in soluble fraction recorded higher results at WVDD06 during October and November in comparison to the background site.

- The control site results from Q4 2022 confirm the previous observation that the arsenic concentration levels recorded in the dust samples from the site are generally slightly higher than the background arsenic levels in the area but is still much lower than the Contaminated Sites Guideline criteria.

- The results from the directional dust monitoring gauges showed no clear patterns during the Q4 period.

- Total arsenic concentrations for Q4 ranged from 0 to 17.8 mg/kg with a mean of 4.8 mg/kg for Q4 2022.

# Further Considerations

DEECA(ERR) have advised that they will continue to monitor the site with regular visits and ensure that a range of dust control measures are applied during the current care and maintenance phase of this rehabilitation project. DEECA(ERR) is also reviewing the effectiveness of dust control measures and monitoring on an ongoing basis and adopting a continuous improvement approach to dust minimisation and monitoring.

The installation of an onsite weather monitoring station has been completed that will help ALS and DEECA(ERR) in future analysis by providing wind and windspeed information.

# References

- ALS Hydrographics Pty Ltd (2021), Woodvale Dust Monitoring Q3 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2021), Woodvale Dust Monitoring Q4 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q1 2022 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q2 2022 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q3 2022 Report. (ALS Report)

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Deposited Matter - Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Impinged Matter - Gravimetric Method. (AS/NZS 3580.10.2)

- CDM Smith (2021), Woodvale Evaporation Ponds - Dust Monitoring Report - January to June 2021. (CDM Smith Report)

- Department of Conservation (2010), Contaminated Sites Management Series, Assessment Levels for Soil, Sediment and Water, February 2010. (Contaminated Sites Assessment Levels)

- EPA Vic Publication 1191 (2007), Protocol for Environmental Management - Mining and Extractive Industries. (Mining PEM)

- EPA NSW Publication (2017), Approved Methods for the Modelling and Assessment of Air Pollutants in NSW, January 2017.

- EPA Vic Publication 1961 (2022), Guideline for Assessing and Minimising Air Pollution in Victoria, February 2022. (Assessing and Minimising Guideline)

- EPA Vic Publication 440.1 (2002), A Guide to the Sampling and Analysis of Air Emissions and Air Quality, December 2002. (Sampling and Analysis Guideline)

- Kralcopic Pty Ltd (2018), Quarterly Environmental Monitoring Report - Woodvale Dust Monitoring, October to December 2018. (Kralcopic Report)

- National Environment Protection Council (1998), National Environment Protection (Ambient Air Quality) Measure, July 2018. (NEPM AAQ)

- Peter J Ramsay and Associates (2018), Woodvale Evaporation Ponds Complex, Environmental Audit of the Risk of any Possible Harm or Detriment to the Environment Pursuant to Part IXD (Section 53V) of the Environment Protection Act 1970, 29 June 2018. (Environmental Audit Report)

- Victorian Government (2001), Statement of Environment Protection Policy (Air Quality Management), Victorian Government Gazette, No S 240, 21 December 2001. (SEPP AQM)

- Victorian Government (2016), State Environment Protection Policy (Ambient Air Quality), Original Policy including Two Variations, Victorian Gazette Nos S19, S240 and G30, published on 9 February 1999, 21 December 2001 and 28 July 2016, respectively. (SEPP AAQ)

- Victorian Government (2021), Environment Reference Standard, under the Environment Protection Act 2017, Victorian Government Gazette, No. S 245, 26 May 2021. (ERS)

# Tables

Table 1 - Woodvale Dust Deposition Analysis Results Q4, 2022

|  |  |  |  |  | Arsenic | Barium | Manganese | Total Solids | Total Solids (mg) | Total Insoluble Matter | Total Insoluble Matter (mg) | Total Soluble Matter | Total Soluble Matter (mg) | Combustible Matter | Combustible Matter (mg) | Ash Content | Ash Content (mg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ug/m2 month | ug/m2 month | ug/m2 month | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg |
| EQL |  |  |  |  |  |  |  |  | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 |
| Description | Location | Quarter | Month | Lab Report Number |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WVDD01 (Insoluble) | WVDD01 | Q4 2022 | Oct-22 | EM2221964-057_V1 | 40.7 | 160 | 1210 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-057_V1 | 33.5 | 1900 | 2260 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-057 | 6.0 | 383 | 285 |  |  |  |  |  |  |  |  |  |  |
| WVDD01 (Soluble) | WVDD01 | Q4 2022 | Oct-22 | EM2221964-029_V1 | 99.9 | 2280 | 3120 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-029_V1 | 118 | 966 | 795 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-029 | 24.9 | 228 | 945 |  |  |  |  |  |  |  |  |  |  |
| WVDD01 (Ash) | WVDD01 | Q4 2022 | Oct-22 | EM2221964-001__V1 | 36.5 | 1630 | 1140 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-001_V1 | 33.5 | 1900 | 2260 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-001 | 6.0 | 383 | 250 |  |  |  |  |  |  |  |  |  |  |
| WVDD01 | WVDD01 | Q4 2022 | Oct-22 | EM2221964-001_V1 |  |  |  | 23.2 | 424 | 14.8 | 270 | 8.4 | 154 | 10.7 | 194 | 4.1 | 76 |
|  |  |  | Nov-22 | EM2223849-001_V1 |  |  |  | 44.4 | 732 | 28.7 | 473 | 15.7 | 259 | 24.5 | 404 | 4.2 | 69 |
|  |  |  | Dec-22 | EM2300219-001 |  |  |  | 15.1 | 275 | 8.9 | 162 | 6.2 | 113 | 8.6 | 156 | 0.3 | 6 |
| WVDD02 (Insoluble) | WVDD02 | Q4 2022 | Oct-22 | EM2221964-058_V1 | 2.56 | 52 | 50.9 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-058_V1 | 4.37 | 110 | 151 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-058 | 3.0 | 47.3 | 47.3 |  |  |  |  |  |  |  |  |  |  |
| WVDD02 (Soluble) | WVDD02 | Q4 2022 | Oct-22 | EM2221964-030_V1 | 0.05 | 108 | 203 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-030_V1 | 49.9 | 264 | 1430 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-030 | 6.4 | 90.0 | 266 |  |  |  |  |  |  |  |  |  |  |
| WVDD02 (Ash) | WVDD02 | Q4 2022 | Oct-22 | EM2221964-002 | 2.27 | 52 | 31.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-002 | 3.91 | 110 | 128 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-002 | 3.0 | 47.3 | 45.5 |  |  |  |  |  |  |  |  |  |  |
| WVDD02 | WVDD02 | Q4 2022 | Oct-22 | EM2221964-002_V1 |  |  |  | 8.1 | 149 | 1.4 | 26 | 6.7 | 123 | 1.2 | 22 | 0.2 | 4 |
|  |  |  | Nov-22 | EM2223849-002 |  |  |  | 15.9 | 263 | 2.7 | 45 | 13.2 | 218 | 2.5 | 41 | 0.2 | 4 |
|  |  |  | Dec-22 | EM2300219-002 |  |  |  | 6.4 | 118 | 2.2 | 41 | 4.2 | 77 | 2.1 | 40 | 0.1 | 2 |
| WVDD03 (Insoluble) | WVDD03 | Q4 2022 | Oct-22 | EM2221964-059_V1 | 19.6 | 560 | 318 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-059_V1 | 2.84 | 138 | 56.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-059 | 4.0 | 70.5 | 53.6 |  |  |  |  |  |  |  |  |  |  |
| WVDD03 (Soluble) | WVDD03 | Q4 2022 | Oct-22 | EM2221964-031_V1 | 67.4 | 646 | 2330 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-031_V1 | 4.41 | 853 | 432 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-031 | 47.0 | 389 | 414 |  |  |  |  |  |  |  |  |  |  |
| WVDD03 (Ash) | WVDD03 | Q4 2022 | Oct-22 | EM2221964-003_V1 | 19.6 | 560 | 318 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-003_V1 | 2.84 | 138 | 47.6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-003 | 4.0 | 70.5 | 52.1 |  |  |  |  |  |  |  |  |  |  |
| WVDD03 | WVDD03 | Q4 2022 | Oct-22 | EM2221964-003_V1 |  |  |  | 4.9 | 89 | 4.1 | 75 | 0.8 | 14 | 3.0 | 54 | 1.1 | 21 |
|  |  |  | Nov-22 | EM2223849-003_V1 |  |  |  | 8.5 | 141 | 1.3 | 22 | 7.2 | 119 | 1.2 | 21 | 0.1 | 2 |
|  |  |  | Dec-22 | EM2300219-003 |  |  |  | 5.2 | 94 | 3.1 | 56 | 2.1 | 38 | 3 | 54 | 0.1 | 2 |
| WVDD04 (Insoluble) | WVDD04 | Q4 2022 | Oct-22 | EM2221964-060_V1 | 1.51 | 33.2 | 36.5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-060_V1 | 5.22 | 88.9 | 122 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-060 | 10.5 | 59.5 | 55.6 |  |  |  |  |  |  |  |  |  |  |
| WVDD04 (Soluble) | WVDD04 | Q4 2022 | Oct-22 | EM2221964-032_V1 | 3.4 | 61.6 | 143 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-032_V1 | 30.4 | 154 | 759 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-032 | 7.3 | 102 | 395 |  |  |  |  |  |  |  |  |  |  |
| WVDD04 (Ash) | WVDD04 | Q4 2022 | Oct-22 | EM2221964-004_V1 | 1.41 | 33.2 | 26.3 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-004_V1 | 5.22 | 88.9 | 122 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-004 | 10.5 | 59.5 | 42.2 |  |  |  |  |  |  |  |  |  |  |
| WVDD04 | WVDD04 | Q4 2022 | Oct-22 | EM2221964-004_V1 |  |  |  | 2.5 | 46 | 1.3 | 24 | 1.2 | 22 | 1.2 | 22 | 0.1 | 2 |
|  |  |  | Nov-22 | EM2223849-004_V1 |  |  |  | 9.0 | 149 | 2.7 | 45 | 6.3 | 104 | 2.4 | 39 | 0.3 | 6 |
|  |  |  | Dec-22 | EM2300219-004 |  |  |  | 4.1 | 76 | 2.7 | 50 | 1.4 | 26 | 2.6 | 48 | 0.1 | 2 |
| WVDD05 (Insoluble) | WVDD05 | Q4 2022 | Oct-22 | EM2221964-061_V1 | 1.7 | 22.1 | 32.7 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-061_V1 | 3.07 | 84.2 | 144 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-061 | 3.5 | 61.3 | 45.8 |  |  |  |  |  |  |  |  |  |  |
| WVDD05 (Soluble) | WVDD05 | Q4 2022 | Oct-22 | EM2221964-033_V1 | 0.33 | 48.9 | 135 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-033_V1 | 13.6 | 152 | 852 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-033 | 4.07 | 86.0 | 268 |  |  |  |  |  |  |  |  |  |  |
| WVDD05 (Ash) | WVDD05 | Q4 2022 | Oct-22 | EM2221964-005_V1 | 1.7 | 22.1 | 20.9 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-005_V1 | 3.07 | 84.2 | 144 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-005 | 3.5 | 61.3 | 45.8 |  |  |  |  |  |  |  |  |  |  |
| WVDD05 | WVDD05 | Q4 2022 | Oct-22 | EM2221964-005_V1 |  |  |  | 1.9 | 34 | 1.0 | 18 | 0.9 | 16 | 1.0 | 18 | 0.1 | 2 |
|  |  |  | Nov-22 | EM2223849-005_V1 |  |  |  | 7.9 | 130 | 3.2 | 52 | 4.7 | 78 | 2.9 | 48 | 0.3 | 4 |
|  |  |  | Dec-22 | EM2300219-005 |  |  |  | 2.3 | 44 | 2.2 | 41 | 0.1 | 3.0 | 2.1 | 39 | 0.1 | 2 |
| WVDD06 (Insoluble) | WVDD06 | Q4 2022 | Oct-22 | EM2221964-062_V1 | 36 | 78.1 | 478 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-062_V1 | 50.3 | 1480 | 778 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-062 | 3.1 | 73.5 | 82.2 |  |  |  |  |  |  |  |  |  |  |
| WVDD06 (Soluble) | WVDD06 | Q4 2022 | Oct-22 | EM2221964-034_V1 | 84.7 | 1010 | 4640 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-034_V1 | 136 | 1210 | 5100 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-034 | 36.0 | 227 | 1120 |  |  |  |  |  |  |  |  |  |  |
| WVDD06 (Ash) | WVDD06 | Q4 2022 | Oct-22 | EM2221964-006_V1 | 27.6 | 781 | 478 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-006_V1 | 50.3 | 1480 | 778 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-006 | 3.1 | 73.5 | 82.2 |  |  |  |  |  |  |  |  |  |  |
| WVDD06 | WVDD06 | Q4 2022 | Oct-22 | EM2221964-006_V1 |  |  |  | 20.7 | 379 | 9.7 | 178 | 11 | 201 | 7 | 129 | 2.7 | 49 |
|  |  |  | Nov-22 | EM2223849-006_V1 |  |  |  | 19.5 | 321 | 9.9 | 163 | 9.6 | 158 | 6.8 | 112 | 3.1 | 51 |
|  |  |  | Dec-22 | EM2300219-006 |  |  |  | 4.3 | 80 | 2.6 | 48 | 1.7 | 32 | 2.5 | 45 | 0.1 | 3 |
| WVDD07BG (Insoluble) | WVDD07BG | Q4 2022 | Oct-22 | EM2221964-063_V1 | 1.56 | 33.2 | 57.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-063_V1 | 2.55 | 64.8 | 72.6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-063 | 9.5 | 325 | 383 |  |  |  |  |  |  |  |  |  |  |
| WVDD07BG (Soluble) | WVDD07BG | Q4 2022 | Oct-22 | EM2221964-035_V1 | 1.98 | 102 | 206 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-035_V1 | 2.13 | 179 | 1020 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-035 | 7.3 | 183 | 998 |  |  |  |  |  |  |  |  |  |  |
| WVDD07BG (Ash) | WVDD07BG | Q4 2022 | Oct-22 | EM2221964-007_V1 | 1.5 | 33.2 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Nov-22 | EM2223849-007_V1 | 2.15 | 64.8 | 55.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dec-22 | EM2300219-007 | 9.5 | 325 | 383 |  |  |  |  |  |  |  |  |  |  |
| WVDD07BG | WVDD07BG | Q4 2022 | Oct-22 | EM2221964-007_V1 |  |  |  | 4.7 | 85 | 1.3 | 23 | 3.4 | 62 | 1.2 | 21 | 0.1 | 2 |
|  |  |  | Nov-22 | EM2223849-007_V1 |  |  |  | 8 | 132 | 1.6 | 27 | 6.4 | 105 | 1.4 | 24 | 0.2 | 3 |
|  |  |  | Dec-22 | EM2300219-007 |  |  |  | 12.7 | 232 | 7.6 | 139 | 5.1 | 93 | 7.1 | 131 | 0.4 | 8 |

Table 2 - Woodvale Directional Dust Deposition Analysis Results Q4, 2022

|  |  |  | Arsenic | Barium | Manganese | Total Solids | Total Solids (mg) | Total Insoluble Matter | Total Insoluble Matter (mg) | Total Soluble Matter | Total Soluble Matter (mg) | Combustible Matter | Combustible Matter (mg) | Ash Content | Ash Content (mg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ug/m2 month | ug/m2 month | ug/m2 month | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg |
| EQL |  |  |  |  |  |  | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 |
| Description | Month | Lab Report Number |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WVDG03E (Insoluble) | Oct-22 | EM2221964-070 | 1.1 | 35.5 | 35.1 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-070 | 0.38 | 53.5 | 13.9 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-070 | 0.70 | 80.3 | 25.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG03E (Soluble) | Oct-22 | EM2221964-042 | 2.72 | 76.9 | 221 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-042 | 0.28 | 28.4 | 14.6 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-042 | 1.8 | 36.5 | 45.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG03E (Ash) | Oct-22 | EM2221964-014 | 1.1 | 35.5 | 35.1 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-014 | 0.38 | 50.3 | 10.7 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-014 | 0.70 | 50.3 | 19.9 |  |  |  |  |  |  |  |  |  |  |
| WVDG03E | Oct-22 | EM2221964-014 |  |  |  | 1.6 | 26 | 1.3 | 21 | 0.3 | 5 | 1.3 | 21 | 0.1 | 2 |
|  | Nov-22 | EM2223849-014 |  |  |  | 1.4 | 21 | 0.8 | 12 | 0.6 | 9 | 0.8 | 12 | 0.1 | 2 |
|  | Dec-22 | EM2300219-014 |  |  |  | 2.9 | 47 | 2.7 | 43 | 0.2 | 4 | 2.7 | 42 | 0.1 | 2 |
| WVDG03N (Insoluble) | Oct-22 | EM2221964-069 | 0.5 | 211 | 21 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-069 | 0.21 | 893 | 12.4 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-069 | 0.60 | 2074 | 28.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG03N (Soluble) | Oct-22 | EM2221964-041 | 2.47 | 115 | 47.2 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-041 | 1.29 | 103 | 13.3 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-041 | 1.2 | 130 | 42.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG03N (Ash) | Oct-22 | EM2221964-013 | 0.5 | 175 | 10.6 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-013 | 0.21 | 893 | 12.4 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-013 | 0.20 | 149 | 4.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG03N | Oct-22 | EM2221964-013 |  |  |  | 2.0 | 31 | 1.0 | 16 | 1.0 | 15 | 1.0 | 16 | 0.1 | 2 |
|  | Nov-22 | EM2223849-013 |  |  |  | 2 | 29 | 1.2 | 17 | 0.8 | 12 | 1.2 | 17 | 0.1 | 2 |
|  | Dec-22 | EM2300219-013 |  |  |  | 2.7 | 44 | 2.5 | 40 | 0.2 | 4 | 2.5 | 39 | 0.1 | 2 |
| WVDG03S (Insoluble) | Oct-22 | EM2221964-071 | 1.6 | 32.5 | 40.1 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-071 | 0.15 | 25.7 | 10.2 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-071 | 2.30 | 47.1 | 37.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG03S (Soluble) | Oct-22 | EM2221964-043 | 5.73 | 53.6 | 211 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-043 | 0.5 | 27.1 | 15.1 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-043 | 1.15 | 23.7 | 54.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG03S (Ash) | Oct-22 | EM2221964-015 | 1.6 | 32.5 | 38.6 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-015 | 0.15 | 23.9 | 7.37 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-015 | 0.7 | 31.5 | 15.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG03S | Oct-22 | EM2221964-015 |  |  |  | 2.1 | 32 | 1.3 | 20 | 0.8 | 12 | 1.3 | 20 | 0.1 | 2 |
|  | Nov-22 | EM2223849-015 |  |  |  | 1 | 14 | 0.8 | 12 | 0.2 | 2 | 0.8 | 12 | 0.1 | 2 |
|  | Dec-22 | EM2300219-015 |  |  |  | 2.8 | 44 | 2.6 | 41 | 0.2 | 3 | 2.5 | 40 | 0.1 | 2 |
| WVDG03W (Insoluble) | Oct-22 | EM2221964-072 | 0.5 | 21.7 | 31.5 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-072 | 4.83 | 49.1 | 36 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-072 | 9.2 | 37.7 | 29.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG03W (Soluble) | Oct-22 | EM2221964-044 | 0.57 | 8.08 | 17.1 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-044 | 0.3 | 4.05 | 4.22 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-044 | 1.15 | 27.6 | 30.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG03W (Ash) | Oct-22 | EM2221964-016 | 0.5 | 18.3 | 23.8 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-016 | 3.54 | 49.1 | 30.4 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-016 | 1.9 | 30.6 | 23.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG03W | Oct-22 | EM2221964-016 |  |  |  | 1.4 | 22 | 1.3 | 21 | 0.1 | 2 | 0.1 | 21 | 0.1 | 2 |
|  | Nov-22 | EM2223849-016 |  |  |  | 2.2 | 31 | 2.1 | 30 | 0.1 | 2 | 1.9 | 28 | 0.2 | 2 |
|  | Dec-22 | EM2300219-016 |  |  |  | 3 | 48 | 2.8 | 44 | 0.2 | 4 | 2.7 | 42 | 0.1 | 2 |
| WVDG05E (Insoluble) | Oct-22 | EM2221964-078 | 2.7 | 119 | 117 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-078 | 0.28 | 22.6 | 12.7 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-078 | 5.46 | 23.5 | 24.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG05E (Soluble) | Oct-22 | EM2221964-050 | 6.77 | 271 | 111 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-050 | 0.31 | 18.6 | 12.3 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-050 | 0.60 | 18.0 | 41.8 |  |  |  |  |  |  |  |  |  |  |
| WVDG05E (Ash) | Oct-22 | EM2221964-022 | 2.7 | 119 | 116 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-022 | 0.28 | 19.2 | 7.99 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-022 | 0.84 | 23.5 | 24.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG05E | Oct-22 | EM2221964-022 |  |  |  | 3 | 48 | 1.7 | 27 | 1.3 | 21 | 1.4 | 22 | 0.3 | 5 |
|  | Nov-22 | EM2223849-022 |  |  |  | 1.6 | 24 | 1.6 | 23 | 0.1 | 2 | 1.5 | 22 | 0.1 | 2 |
|  | Dec-22 | EM2300219-022 |  |  |  | 1 | 15 | 0.9 | 14 | 0.1 | 2 | 0.8 | 13 | 0.1 | 2 |
| WVDG05N (Insoluble) | Oct-22 | EM2221964-077 | 0.79 | 17.8 | 23.9 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-077 | 0.93 | 31.4 | 9.35 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-077 | 0.35 | 21.3 | 14.8 |  |  |  |  |  |  |  |  |  |  |
| WVDG05N (Soluble) | Oct-22 | EM2221964-049 | 0.31 | 0.65 | 1.02 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-049 | 0.59 | 24.4 | 80.9 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-049 | 0.60 | 22.5 | 30.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG05N (Ash) | Oct-22 | EM2221964-021 | 0.69 | 17.8 | 23.9 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-021 | 0.28 | 31.1 | 8.1 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-021 | 0.35 | 21.3 | 14.8 |  |  |  |  |  |  |  |  |  |  |
| WVDG05N | Oct-22 | EM2221964-021 |  |  |  | 0.9 | 15 | 0.9 | 15 | 0.1 | 2 | 0.9 | 15 | 0.1 | 2 |
|  | Nov-22 | EM2223849-021 |  |  |  | 2.5 | 36 | 1.3 | 18 | 1.2 | 18 | 1.2 | 17 | 0.1 | 2 |
|  | Dec-22 | EM2300219-021 |  |  |  | 2.1 | 34 | 1.6 | 26 | 0.5 | 8 | 1.6 | 25 | 0.1 | 2 |
| WVDG05S (Insoluble) | Oct-22 | EM2221964-079 | 1.1 | 27 | 52.2 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-079 | 0.35 | 26.2 | 13.7 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-079 | 0.70 | 21.3 | 19.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG05S (Soluble) | Oct-22 | EM2221964-051 | 1.42 | 14.5 | 86.8 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-051 | 0.05 | 17.2 | 23.3 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-051 | 0.40 | 19.0 | 30.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG05S (Ash) | Oct-22 | EM2221964-023 | 1.1 | 27 | 52.2 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-023 | 0.15 | 21.7 | 11.1 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-023 | 0.70 | 21.3 | 19.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG05S | Oct-22 | EM2221964-023 |  |  |  | 1.4 | 22 | 1.1 | 18 | 0.3 | 4 | 1 | 17 | 0.1 | 2 |
|  | Nov-22 | EM2223849-023 |  |  |  | 2 | 28 | 1.9 | 27 | 0.1 | 2 | 1.9 | 27 | 0.1 | 2 |
|  | Dec-22 | EM2300219-023 |  |  |  | 1 | 17 | 0.7 | 11 | 0.3 | 6 | 0.7 | 11 | 0.1 | 2 |
| WVDG05W (Insoluble) | Oct-22 | EM2221964-080 | 3.9 | 226 | 59.3 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-080 | 0.24 | 27.3 | 11.3 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-080 | 0.46 | 22.7 | 11.8 |  |  |  |  |  |  |  |  |  |  |
| WVDG05W (Soluble) | Oct-22 | EM2221964-052 | 0.73 | 29.7 | 13..2 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-052 | 0.05 | 13.9 | 3.24 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-052 | 0.50 | 18.0 | 26.3 |  |  |  |  |  |  |  |  |  |  |
| WVDG05W (Ash) | Oct-22 | EM2221964-024 | 3.52 | 226 | 53.8 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-024 | 0.24 | 27.3 | 9.52 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-024 | 0.30 | 20.6 | 8.82 |  |  |  |  |  |  |  |  |  |  |
| WVDG05W | Oct-22 | EM2221964-024 |  |  |  | 2.6 | 41 | 2.5 | 39 | 0.1 | 2 | 2 | 32 | 0.5 | 7 |
|  | Nov-22 | EM2223849-024 |  |  |  | 2 | 29 | 2 | 28 | 0.1 | 2 | 1.9 | 27 | 0.1 | 2 |
|  | Dec-22 | EM2300219-024 |  |  |  | 2.6 | 42 | 2.5 | 40 | 0.1 | 2 | 2.5 | 40 | 0.1 | 2 |
| WVDG04E (Insoluble) | Oct-22 | EM2221964-074 | 1.1 | 39.7 | 46.5 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-074 | 0.21 | 25.2 | 18.2 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-074 | 0.50 | 21.2 | 19.3 |  |  |  |  |  |  |  |  |  |  |
| WVDG04E (Soluble) | Oct-22 | EM2221964-046 | 0.7 | 14.4 | 33.9 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-046 | 0.23 | 24.0 | 67.8 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-046 | 0.40 | 22.0 | 34.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG04E (Ash) | Oct-22 | EM2221964-018 | 1.1 | 31.9 | 46 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-018 | 0.08 | 24.3 | 9.83 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-018 | 0.50 | 18.0 | 17.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG04E | Oct-22 | EM2221964-018 |  |  |  | 1.3 | 21 | 1.1 | 18 | 0.2 | 3 | 1 | 17 | 0.1 | 2 |
|  | Nov-22 | EM2223849-018 |  |  |  | 1.7 | 24 | 1.7 | 24 | 0.1 | 2 | 1.7 | 24 | 0.1 | 2 |
|  | Dec-22 | EM2300219-018 |  |  |  | 2.9 | 46 | 2.7 | 42 | 0.2 | 4 | 2.7 | 42 | 0.1 | 2 |
| WVDG04N (Insoluble) | Oct-22 | EM2221964-073 | 1.03 | 35.1 | 34.9 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-073 | 0.62 | 29.0 | 14.8 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-073 | 0.40 | 19.1 | 13.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG04N (Soluble) | Oct-22 | EM2221964-045 | 1.33 | 46.6 | 71.3 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-045 | 0.20 | 4.59 | 7.18 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-045 | 0.40 | 19.9 | 28.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG04N (Ash) | Oct-22 | EM2221964-017 | 1.02 | 35.1 | 30.4 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-017 | 0.62 | 29.0 | 14.4 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-017 | 0.19 | 11.6 | 7.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG04N | Oct-22 | EM2221964-017 |  |  |  | 1.6 | 25 | 1.3 | 21 | 0.3 | 4 | 1.2 | 20 | 0.1 | 2 |
|  | Nov-22 | EM2223849-017 |  |  |  | 1.7 | 24 | 1.7 | 24 | 0.1 | 2 | 1.7 | 24 | 0.1 | 2 |
|  | Dec-22 | EM2300219-017 |  |  |  | 2.9 | 46 | 2.6 | 41 | 0.3 | 5 | 2.6 | 41 | 0.1 | 2 |
| WVDG04S (Insoluble) | Oct-22 | EM2221964-075 | 1.69 | 63 | 74.1 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-075 | 0.35 | 27.7 | 15.6 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-075 | 0.80 | 18.3 | 58.3 |  |  |  |  |  |  |  |  |  |  |
| WVDG04S (Soluble) | Oct-22 | EM2221964-047 | 1.13 | 37 | 57.6 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-047 | 0.05 | 21.8 | 15.1 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-047 | 1.0 | 18.5 | 37.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG04S (Ash) | Oct-22 | EM2221964-019 | 1.47 | 40.8 | 74.1 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-019 | 0.35 | 23.0 | 15.6 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-019 | 0.80 | 18.3 | 58.3 |  |  |  |  |  |  |  |  |  |  |
| WVDG04S | Oct-22 | EM2221964-019 |  |  |  | 1.7 | 27 | 1.5 | 23 | 0.2 | 4 | 1.4 | 22 | 0.1 | 2 |
|  | Nov-22 | EM2223849-019 |  |  |  | 1.6 | 23 | 1.5 | 22 | 0.1 | 2 | 1.5 | 22 | 0.1 | 2 |
|  | Dec-22 | EM2300219-019 |  |  |  | 2.8 | 44 | 2.3 | 36 | 0.5 | 8 | 2.3 | 36 | 0.1 | 2 |
| WVDG04W (Insoluble) | Oct-22 | EM2221964-076 | 2.2 | 31.3 | 102 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-076 | 6.53 | 52.6 | 23.1 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-076 | 5.40 | 18.6 | 18.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG04W (Soluble) | Oct-22 | EM2221964-048 | 0.94 | 27.7 | 57.8 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-048 | 1.93 | 11.0 | 34.1 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-048 | 1.2 | 19.5 | 19.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG04W (Ash) | Oct-22 | EM2221964-020 | 2.2 | 31.3 | 102 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-020 | 6.53 | 38.6 | 23.1 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-020 | 1.14 | 18.6 | 18.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG04W | Oct-22 | EM2221964-020 |  |  |  | 1.6 | 25 | 1.3 | 20 | 0.3 | 5 | 1.2 | 18 | 0.1 | 2 |
|  | Nov-22 | EM2223849-020 |  |  |  | 1.8 | 25 | 1.4 | 20 | 0.4 | 5 | 1.4 | 20 | 0.1 | 2 |
|  | Dec-22 | EM2300219-020 |  |  |  | 2.3 | 36 | 2.2 | 35 | 0.1 | 2 | 2.2 | 34 | 0.1 | 2 |
| WVDG06E (Insoluble) | Oct-22 | EM2221964-082 | 1.93 | 31 | 28.8 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-082 | 0.18 | 62.3 | 9.15 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-082 | 0.80 | 20.5 | 18.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG06E (Soluble) | Oct-22 | EM2221964-054 | 26.7 | 32.2 | 149 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-054 | 0.16 | 23.3 | 14 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-054 | 3.2 | 22.1 | 43.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG06E (Ash) | Oct-22 | EM2221964-026 | 1.43 | 31 | 28.8 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-026 | 0.05 | 27.3 | 9.15 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-026 | 0.80 | 20.5 | 13.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG06E | Oct-22 | EM2221964-026 |  |  |  | 2.4 | 38 | 1.6 | 25 | 0.8 | 13 | 1.5 | 23 | 0.1 | 2 |
|  | Nov-22 | EM2223849-026 |  |  |  | 1.3 | 19 | 1.2 | 17 | 0.1 | 2 | 1.1 | 16 | 0.1 | 2 |
|  | Dec-22 | EM2300219-026 |  |  |  | 3.8 | 60 | 3.1 | 49 | 0.7 | 11 | 3 | 48 | 0.1 | 2 |
| WVDG06N (Insoluble) | Oct-22 | EM2221964-081 | 6.08 | 51 | 41.1 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-081 | 0.05 | 43.6 | 8.03 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-081 | 1.20 | 31.5 | 23.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG06N (Soluble) | Oct-22 | EM2221964-053 | 1.59 | 17.7 | 38 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-053 | 0.05 | 17.3 | 6.98 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-053 | 0.50 | 19.0 | 41.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG06N (Ash) | Oct-22 | EM2221964-025 | 4.47 | 51 | 32.6 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-025 | 0.09 | 28.5 | 7.44 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-025 | 1.20 | 31.5 | 23.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG06N | Oct-22 | EM2221964-025 |  |  |  | 1.7 | 27 | 1.6 | 25 | 0.1 | 2 | 1.4 | 22 | 0.2 | 3 |
|  | Nov-22 | EM2223849-025 |  |  |  | 2.3 | 32 | 2.2 | 31 | 0.1 | 2 | 2.2 | 31 | 0.1 | 2 |
|  | Dec-22 | EM2300219-025 |  |  |  | 2.5 | 39 | 2.4 | 38 | 0.1 | 2 | 2.3 | 37 | 0.1 | 2 |
| WVDG06S (Insoluble) | Oct-22 | EM2221964-083 | 0.9 | 17.2 | 37.7 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-083 | 0.05 | 26.1 | 9.69 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-083 | 1.11 | 26.8 | 33.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG06S (Soluble) | Oct-22 | EM2221964-055 | 0.28 | 4.93 | 8.2 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-055 | 0.05 | 15.6 | 8.89 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-055 | 3.8 | 41.3 | 44.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG06S (Ash) | Oct-22 | EM2221964-027 | 0.81 | 17.2 | 27.0 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-027 | 0.05 | 4.06 | 0.61 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-027 | 1.10 | 24.3 | 31.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG06S | Oct-22 | EM2221964-027 |  |  |  | 1.3 | 22 | 1.3 | 21 | 0.1 | 2 | 1.2 | 20 | 0.1 | 2 |
|  | Nov-22 | EM2223849-027 |  |  |  | 1.1 | 17 | 1.1 | 16 | 0.1 | 2 | 1.1 | 16 | 0.1 | 2 |
|  | Dec-22 | EM2300219-027 |  |  |  | 3.6 | 57 | 3.1 | 49 | 0.5 | 8 | 3 | 48 | 0.1 | 2 |
| WVDG06W (Insoluble) | Oct-22 | EM2221964-084 | 0.92 | 15.9 | 24.1 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-084 | 0.22 | 39.1 | 14.6 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-084 | 2.70 | 40.8 | 49.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG06W (Soluble) | Oct-22 | EM2221964-056 | 2.25 | 35 | 145.0 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-056 | 0.05 | 19.9 | 9.79 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-056 | 2.80 | 18.8 | 36.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG06W (Ash) | Oct-22 | EM2221964-028 | 0.92 | 15.9 | 21.6 |  |  |  |  |  |  |  |  |  |  |
|  | Nov-22 | EM2223849-028 | 0.21 | 37.5 | 9.8 |  |  |  |  |  |  |  |  |  |  |
|  | Dec-22 | EM2300219-028 | 2.70 | 40.8 | 49.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG06W | Oct-22 | EM2221964-028 |  |  |  | 2.5 | 38 | 1.5 | 23 | 1 | 15 | 1.5 | 23 | 0.1 | 2 |
|  | Nov-22 | EM2223849-028 |  |  |  | 1.7 | 25 | 1.7 | 24 | 0.1 | 2 | 1.7 | 24 | 0.1 | 2 |
|  | Dec-22 | EM2300219-028 |  |  |  | 3.1 | 48 | 2.7 | 42 | 0.4 | 6 | 2.6 | 41 | 0.1 | 2 |

# Supplementary Data

Graphs for Metals/Metalloids in Insoluble and Soluble Fractions including trends are presented below.

## Arsenic

Figure 13 - Woodvale - Arsenic in Soluble Fraction – Q4 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph

Figure 14 - Woodvale - Arsenic in Insoluble Fraction – Q4 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 15 - Woodvale - Arsenic in Soluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 16 - Woodvale - Arsenic in Insoluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Arsenic lab results unable to be calculated left as blank

## Barium

Figure 17 - Woodvale - Barium in Soluble Fraction – Q4 2022

Figure 18 - Woodvale - Barium in Insoluble Fraction – Q4 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 19 - Woodvale - Barium in Soluble Fraction Year Review (2021-22)

Figure 20 - Woodvale - Barium in Insoluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph. The larger bolder asterisks represent the entire months records as less than

## Manganese

Figure 21 - Woodvale - Manganese in Soluble Fraction – Q4 2022

Figure 22 - Woodvale - Manganese in Insoluble Fraction – Q4 2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 23 - Woodvale - Manganese in Soluble Fraction Year Review (2021-22)

Figure 24 - Woodvale - Manganese in Insoluble Fraction Year Review (2021-22)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## Trend Analysis Graphs

Figure 25 - Woodvale - Total Insoluble Matter (TIM) - 2021-2022

Figure 25 shows TIM for all months in 2021 and 2022. The review of Q4 2022 data shows slightly higher trends across WVDD01, WVDD06 which recorded higher than the background site. In Q4 2022, WVDD01recorded elevated levels higher than any other site for October, November, and December. This may have been influenced by environmental conditions including but not limited to strong wind conditions. The overall average from all monitoring locations was higher being 5.69 g/m2/month for Q4 2022 (excluding the background control site), compared to 4.1 g/m2/month for quarter 3 in 2022 During the Q4 2022 field visits, ALS observed no variation in site conditions whilst undertaking monthly site visits which could have contributed to higher levels recorded.

Figure 26 - Woodvale - Total Ash Content - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 26 shows the Total Ash Content for all months in 2021and 2022 with Q4 monthly figures being increasing during the past 6 months. December 2022 results were very similar to that recorded in November 2021.

Figure 27 - Woodvale - Ash Content/TIM Ratio - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 27 shows all results collected and sampled for 2021 and 2022.

Figure 28 - Woodvale - Total Arsenic/Total Solids - 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Could not be calculated as per other months due to lab samples results – August 2022

Figure 28 above presents the ratio of Total Arsenic/Total Solids. Q3 Arsenic results for August have been given an    over the sample as lab results were compromised as a result of being totally ashed. For more details refer to Q3 report. .

Figure 29 - Woodvale - DDG Total Solids Comparison North/South - 2021-2022

Figure 29 above displays the results from 2021 and 2022, no definitive pattern is apparent throughout this period with small net gains and losses from both the upwind and downwind quadrants being observed. The southerly facing quadrant does have greater gains in 17 of 24 months and the northerly facing quadrant having gains in 11 of 24 months. However, the overall gains from the site are showing to be very low levels.

Figure 30 - Woodvale - Arsenic/Total Solids Comparison - North/South 2021-2022

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

No results for Arsenic (insoluble) for August 2022

Figure 30 shows the net gains and losses in total arsenic concentration for the north and south quadrants from Jan 2021 – Dec 2022. No definitive pattern is apparent in this period with net gains and losses from both the upwind and downwind quadrants being observed.

Figure 31 - Woodvale - DDG Total Solids Comparison East/West - 2021-2022

Figure 31 above displays the net gain or loss both upwind and downwind from east and west for total solids, and no clear pattern is identified above during the 12 months with both gains and losses occurring throughout the period.

Figure 32 - Woodvale - Arsenic/Total Solids Comparison - East/West - 2021-2022

Figure 322 shows the net gains and losses in total arsenic concentration for the east/west quadrants from the previous 12 months. No definitive pattern is apparent in this period with net gains and losses from both the upwind and downwind quadrants being observed.

# Appendix A – Supplementary Information

# Appendix B – Laboratory Reports

Environmental

CERTIFICATE OF ANALYSIS

Work Order	: EM2221964

Page	: 1 of 19

Amendment

Client

Contact

Address

: 1

: ALS WATER AND HYDROGRAPHICS PTY LTD

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Bronwyn Sheen

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +6138549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 07-Nov-2022 10:40 |
| Order number | : ---- | Date Analysis Commenced | : 15-Nov-2022 |
| C-O-C number Sampler Site | : ---- : RO : ---- | Issue Date | : 28-Feb-2023 20:26 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Dilani Fernando	Laboratory Coordinator	Melbourne Inorganics, Springvale, VIC

Eric Chau	Metals Team Leader	Melbourne Inorganics, Springvale, VIC

R I G H T  S O L U T I O N S | R I G H T  P A R T N E R

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- EA120-EA142: EM2221964 #1; Sample was observed to be very turbid.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- Amendment (28/02/2023): This report has been amended following a change to the metal (EG020TUG) results reported for sample EM2221964 #1-#28 due to a transcription error. All details are recorded in client query 23MECC076 and a full investigation will be detailed in corrective action request 23MEC033.

- Sampling Period: 30/09/2022-31/10/2022.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 10:35 | 31-Oct-2022 10:50 | 31-Oct-2022 11:15 | 31-Oct-2022 11:25 | 31-Oct-2022 11:45 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-001 | EM2221964-002 | EM2221964-003 | EM2221964-004 | EM2221964-005 |
|  |  |  | Result 4.1 | Result 0.2 | Result 1.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 76 10.7 | 4 1.2 | 21 3.0 | <2 1.2 | <2 1.0 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 194 8.4 | 22 6.7 | 54 0.8 | 22 1.2 | 18 0.9 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 154 14.8 | 123 1.4 | 14 4.1 | 22 1.3 | 16 1.0 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 270 23.2 | 26 8.1 | 75 4.9 | 24 2.5 | 18 1.9 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 424 36.5 | 149 2.27 | 89 <19.6 | 46 1.41 | 34 <1.70 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <1630 | <52.0 | <560 | <33.2 | <22.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 1140 | 31.5 | <318 | 26.3 | 20.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD06 | WVDD007BG | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 10:15 | 31-Oct-2022 09:45 | 31-Oct-2022 13:05 | 31-Oct-2022 12:55 | 31-Oct-2022 13:00 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-006 | EM2221964-007 | EM2221964-008 | EM2221964-009 | EM2221964-010 |
|  |  |  | Result 2.7 | Result 0.1 | Result <0.1 | Result <0.1 | Result 0.2 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 49 7.0 | 2 1.2 | <2 0.9 | <2 0.9 | 4 1.3 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 129 11.0 | 21 3.4 | 15 5.3 | 17 1.0 | 23 0.9 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 201 9.7 | 62 1.3 | 97 0.9 | 18 0.9 | 16 1.5 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 178 20.7 | 23 4.7 | 16 6.2 | 17 1.9 | 27 2.4 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 379 27.6 | 85 1.49 | 113 <4.80 | 35 <6.60 | 43 17.7 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <781 | <33.2 | <27.4 | <26.0 | <117 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <478 | 30.0 | 29.8 | 30.1 | 54.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 12:45 | 31-Oct-2022 12:35 | 31-Oct-2022 11:15 | 31-Oct-2022 11:15 | 31-Oct-2022 11:15 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-011 | EM2221964-012 | EM2221964-013 | EM2221964-014 | EM2221964-015 |
|  |  |  | Result <0.1 | Result <0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | <2 ---- | <2 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 1.1 | ---- 1.0 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 20 ---- | 19 ---- | ---- 1.0 | ---- 1.3 | ---- 1.3 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.8 | ---- <0.1 | 16 ---- | 21 ---- | 20 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 14 ---- | <2 ---- | ---- 1.0 | ---- 0.3 | ---- 0.8 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 1.1 | ---- 1.0 | 15 ---- | 5 ---- | 12 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 20 ---- | 19 ---- | ---- 1.0 | ---- 1.3 | ---- 1.3 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 1.9 | ---- 1.0 | 16 ---- | 21 ---- | 20 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 34 ---- | 20 ---- | ---- 2.0 | ---- 1.6 | ---- 2.1 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- 3.02 | ---- <5.20 | 31 <0.50 | 26 <1.10 | 32 <1.60 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <34.0 | <75.5 | 175 | <35.5 | <32.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 29.3 | <35.1 | 10.6 | <35.1 | 38.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 11:15 | 31-Oct-2022 11:25 | 31-Oct-2022 11:25 | 31-Oct-2022 11:25 | 31-Oct-2022 11:25 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-016 | EM2221964-017 | EM2221964-018 | EM2221964-019 | EM2221964-020 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result 0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 <0.1 | <2 1.2 | <2 1.0 | <2 1.4 | <2 1.2 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 21 0.1 | 20 0.3 | 17 0.2 | 22 0.2 | 18 0.3 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 1.3 | 4 1.3 | 3 1.1 | 4 1.5 | 5 1.3 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 21 1.4 | 21 1.6 | 18 1.3 | 23 1.7 | 20 1.6 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 22 <0.50 | 25 1.02 | 21 <1.10 | 27 1.47 | 25 <2.20 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 18.3 | <35.1 | 31.9 | 40.8 | <31.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 23.8 | 30.4 | 46.0 | <74.1 | <102 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG06N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 11:45 | 31-Oct-2022 11:45 | 31-Oct-2022 11:45 | 31-Oct-2022 11:45 | 31-Oct-2022 10:15 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-021 | EM2221964-022 | EM2221964-023 | EM2221964-024 | EM2221964-025 |
|  |  |  | Result <0.1 | Result 0.3 | Result <0.1 | Result 0.5 | Result 0.2 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.9 | 5 1.4 | <2 1.0 | 7 2.0 | 3 1.4 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 15 <0.1 | 22 1.3 | 17 0.3 | 32 0.1 | 22 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.9 | 21 1.7 | 4 1.1 | 2 2.5 | 2 1.6 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 15 0.9 | 27 3.0 | 18 1.4 | 39 2.6 | 25 1.7 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 15 0.69 | 48 <2.70 | 22 <1.10 | 41 3.52 | 27 4.47 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <17.8 | <119 | <27.0 | <226 | <51.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <23.9 | 116 | <52.2 | 53.8 | 32.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06E | WVDG06S | WVDG06W | WVDD01 Soluble metals | WVDD02 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 10:15 | 31-Oct-2022 10:15 | 31-Oct-2022 10:15 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2221964-026 | EM2221964-027 | EM2221964-028 | EM2221964-029 | EM2221964-030 |
|  |  |  | Result 0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.5 | <2 1.2 | <2 1.5 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 23 0.8 | 20 <0.1 | 23 1.0 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 13 1.6 | <2 1.3 | 15 1.5 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 25 2.4 | 21 1.3 | 23 2.5 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 38 1.43 | 22 0.81 | 38 <0.92 | ---- 99.9 | ---- <0.05 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <31.0 | <17.2 | <15.9 | 2280 | 108 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <28.8 | 27.0 | 21.6 | 3120 | 203 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble metals | WVDD04 Soluble metals | WVDD05 Soluble metals | WVDD06 Soluble metals | WVDD007BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-031 | EM2221964-032 | EM2221964-033 | EM2221964-034 | EM2221964-035 |
|  |  |  | Result 67.4 | Result 3.40 | Result 0.33 | Result 84.7 | Result 1.98 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 646 | 61.6 | 48.9 | 1010 | 102 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 2330 | 143 | 135 | 4640 | 206 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble metals | KF2 Soluble metals | KF3 Soluble metals | KF4 Soluble metals | KF5BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-036 | EM2221964-037 | EM2221964-038 | EM2221964-039 | EM2221964-040 |
|  |  |  | Result 2.19 | Result 8.26 | Result 12.8 | Result 4.59 | Result 2.89 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 136 | 40.4 | 65.8 | 136 | 55.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 200 | 124 | 160 | 151 | 163 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N Soluble metals | WVDG03E Soluble metals | WVDG03S Soluble metals | WVDG03W Soluble metals | WVDG04N Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-041 | EM2221964-042 | EM2221964-043 | EM2221964-044 | EM2221964-045 |
|  |  |  | Result 2.47 | Result 2.72 | Result 5.73 | Result 0.57 | Result 1.33 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 115 | 76.9 | 53.6 | 8.08 | 46.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 47.2 | 221 | 211 | 17.1 | 71.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E Soluble metals | WVDG04S Soluble metals | WVDG04W Soluble metals | WVDG05N Soluble metals | WVDG05E Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-046 | EM2221964-047 | EM2221964-048 | EM2221964-049 | EM2221964-050 |
|  |  |  | Result 0.70 | Result 1.13 | Result 0.94 | Result 0.31 | Result 6.77 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 14.4 | 37.0 | 27.7 | 0.65 | 271 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 33.9 | 57.6 | 57.8 | 1.02 | 111 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble metals | WVDG05W Soluble metals | WVDG06N Soluble metals | WVDG06E Soluble metals | WVDG06S Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-051 | EM2221964-052 | EM2221964-053 | EM2221964-054 | EM2221964-055 |
|  |  |  | Result 1.42 | Result 0.73 | Result 1.59 | Result 26.7 | Result 0.28 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 14.5 | 29.7 | 17.7 | 32.2 | 4.93 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 86.8 | 13.2 | 38.0 | 149 | 8.20 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06W Soluble metals | WVDD01 Insoluble metals | WVDD02 Insoluble metals | WVDD03 Insoluble metals | WVDD04 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-056 | EM2221964-057 | EM2221964-058 | EM2221964-059 | EM2221964-060 |
|  |  |  | Result 2.25 | Result 40.7 | Result 2.56 | Result <19.6 | Result 1.51 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 35.0 | <160 | <52.0 | <560 | <33.2 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 145 | 1210 | 50.9 | <318 | 36.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD05 Insoluble metals | WVDD06 Insoluble metals | WVDD007BG Insoluble metals | KF1 Insoluble metals | KF2 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-061 | EM2221964-062 | EM2221964-063 | EM2221964-064 | EM2221964-065 |
|  |  |  | Result <1.70 | Result 36.0 | Result 1.56 | Result <4.80 | Result <6.60 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <22.1 | <78.1 | <33.2 | <27.4 | <26.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 32.7 | <478 | 57.1 | 34.3 | 43.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 Insoluble metals | KF4 Insoluble metals | KF5BG Insoluble metals | WVDG03N Insoluble metals | WVDG03E Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-066 | EM2221964-067 | EM2221964-068 | EM2221964-069 | EM2221964-070 |
|  |  |  | Result 20.3 | Result 39.4 | Result <5.20 | Result <0.50 | Result <1.10 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <117 | <34.0 | <75.5 | 211 | <35.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 78.9 | 44.1 | <35.1 | 21.0 | <35.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Insoluble metals | WVDG03W Insoluble metals | WVDG04N Insoluble metals | WVDG04E Insoluble metals | WVDG04S Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-071 | EM2221964-072 | EM2221964-073 | EM2221964-074 | EM2221964-075 |
|  |  |  | Result <1.60 | Result <0.50 | Result 1.03 | Result <1.10 | Result 1.69 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <32.5 | 21.7 | <35.1 | 39.7 | 63.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 40.1 | 31.5 | 34.9 | 46.5 | <74.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Insoluble metals | WVDG05N Insoluble metals | WVDG05E Insoluble metals | WVDG05S Insoluble metals | WVDG05W Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-076 | EM2221964-077 | EM2221964-078 | EM2221964-079 | EM2221964-080 |
|  |  |  | Result <2.20 | Result 0.79 | Result <2.70 | Result <1.10 | Result 3.90 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <31.3 | <17.8 | <119 | <27.0 | <226 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <102 | <23.9 | 117 | <52.2 | 59.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06N Insoluble metals | WVDG06E Insoluble metals | WVDG06S Insoluble metals | WVDG06W Insoluble metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | 31-Oct-2022 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2221964-081 | EM2221964-082 | EM2221964-083 | EM2221964-084 | -------- |
|  |  |  | Result 6.08 | Result 1.93 | Result 0.90 | Result <0.92 | ---- ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <51.0 | <31.0 | <17.2 | <15.9 | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 41.1 | <28.8 | 37.7 | 24.1 | ---- |

Environmental

CERTIFICATE OF ANALYSIS

Work Order	: EM2223849

Page	: 1 of 19

Amendment

Client

Contact

Address

: 1

: ALS WATER AND HYDROGRAPHICS PTY LTD

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Bronwyn Sheen

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +6138549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 30-Nov-2022 16:15 |
| Order number | : ---- | Date Analysis Commenced | : 06-Dec-2022 |
| C-O-C number Sampler Site | : ---- : ROHAN OLIVER : ALS KERANG DUST | Issue Date | : 02-Mar-2023 18:41 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Arenie Vijayaratnam	Senior Inorganic Chemist	Melbourne Inorganics, Springvale, VIC

Eric Chau	Metals Team Leader	Melbourne Inorganics, Springvale, VIC

R I G H T  S O L U T I O N S | R I G H T  P A R T N E R

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- EM2223849 #1,6: Sample was observed to be turbid.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- Amendment (02/03/2023): This report has been amended following a change to the EG020TUG arsenic result reported for sample EM2223849 #27, 47, 51, 52, 53, 55, 56, 81 & 83 and barium results for #55 due to a transcription error. All details are recorded in client query 23MECC080 and a full investigation will be detailed in corrective action request 23MEC008.

- Dust sampled from 31/10/22 to 28/11/22.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- EM2223849 #1-15, 18-19, 21-28: Sample container was received with no liquid. 100ml of DI water was used to dissolve contents, perform analysis and for calculation of results.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 10:40 | 28-Nov-2022 10:50 | 28-Nov-2022 11:05 | 28-Nov-2022 11:25 | 28-Nov-2022 11:40 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-001 | EM2223849-002 | EM2223849-003 | EM2223849-004 | EM2223849-005 |
|  |  |  | Result 4.2 | Result 0.2 | Result <0.1 | Result 0.3 | Result 0.3 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 69 24.5 | 4 2.5 | <2 1.2 | 6 2.4 | 4 2.9 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 404 15.7 | 41 13.2 | 21 7.2 | 39 6.3 | 48 4.7 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 259 28.7 | 218 2.7 | 119 1.3 | 104 2.7 | 78 3.2 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 473 44.4 | 45 15.9 | 22 8.5 | 45 9.0 | 52 7.9 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 732 <33.5 | 263 3.91 | 141 <2.84 | 149 <5.22 | 130 <3.07 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <1900 | <110 | <138 | <88.9 | <84.2 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <2260 | 128 | 47.6 | <122 | <144 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD06 | WVDD007BG | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 10:10 | 28-Nov-2022 09:50 | 28-Nov-2022 13:10 | 28-Nov-2022 12:55 | 28-Nov-2022 13:00 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-006 | EM2223849-007 | EM2223849-008 | EM2223849-009 | EM2223849-010 |
|  |  |  | Result 3.1 | Result 0.2 | Result 0.2 | Result 0.1 | Result 0.2 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 51 6.8 | 3 1.4 | 2 1.4 | 2 1.3 | 3 1.1 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 112 9.6 | 24 6.4 | 24 4.3 | 21 4.8 | 18 4.6 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 158 9.9 | 105 1.6 | 71 1.6 | 79 1.4 | 76 1.3 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 163 19.5 | 27 8.0 | 26 5.9 | 23 6.2 | 21 5.9 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 321 <50.3 | 132 2.15 | 97 <26.4 | 102 51.3 | 97 <15.9 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <1480 | <64.8 | <83.2 | <82.8 | <73.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <778 | 55.1 | 55.2 | 65.1 | 70.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 12:45 | 28-Nov-2022 12:30 | 28-Nov-2022 11:05 | 28-Nov-2022 11:05 | 28-Nov-2022 11:05 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-011 | EM2223849-012 | EM2223849-013 | EM2223849-014 | EM2223849-015 |
|  |  |  | Result 0.2 | Result <0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | 4 ---- | <2 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 1.9 | ---- 1.2 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 30 ---- | 21 ---- | ---- 1.2 | ---- 0.8 | ---- 0.8 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 9.4 | ---- 5.3 | 17 ---- | 12 ---- | 12 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 155 ---- | 88 ---- | ---- 0.8 | ---- 0.6 | ---- 0.2 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 2.1 | ---- 1.3 | 12 ---- | 9 ---- | 2 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 34 ---- | 22 ---- | ---- 1.2 | ---- 0.8 | ---- 0.8 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 11.5 | ---- 6.6 | 17 ---- | 12 ---- | 12 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 189 ---- | 110 ---- | ---- 2.0 | ---- 1.4 | ---- 1.0 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- <32.8 | ---- 3.13 | 29 <0.21 | 21 <0.38 | 14 <0.15 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <211 | 55.1 | <893 | 50.3 | 23.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <159 | 46.4 | <12.4 | 10.7 | 7.37 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 11:05 | 28-Nov-2022 11:25 | 28-Nov-2022 11:25 | 28-Nov-2022 11:25 | 28-Nov-2022 11:25 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-016 | EM2223849-017 | EM2223849-018 | EM2223849-019 | EM2223849-020 |
|  |  |  | Result 0.2 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 2 1.9 | <2 1.7 | <2 1.7 | <2 1.5 | <2 1.4 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 28 0.1 | 24 <0.1 | 24 <0.1 | 22 0.1 | 20 0.4 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 2.1 | <2 1.7 | <2 1.7 | <2 1.5 | 5 1.4 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 30 2.2 | 24 1.7 | 24 1.7 | 22 1.6 | 20 1.8 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 31 3.54 | 24 <0.62 | 24 0.08 | 23 <0.35 | 25 <6.53 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <49.1 | <29.0 | 24.3 | 23.0 | 38.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 30.4 | 14.4 | 9.83 | <15.6 | <23.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG06N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 11:40 | 28-Nov-2022 11:40 | 28-Nov-2022 11:40 | 28-Nov-2022 11:40 | 28-Nov-2022 10:10 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-021 | EM2223849-022 | EM2223849-023 | EM2223849-024 | EM2223849-025 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.2 | <2 1.5 | <2 1.9 | <2 1.9 | <2 2.2 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 17 1.2 | 22 <0.1 | 27 0.1 | 27 <0.1 | 31 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 18 1.3 | <2 1.6 | <2 1.9 | <2 2.0 | <2 2.2 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 18 2.5 | 23 1.6 | 27 2.0 | 28 2.0 | 31 2.3 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 36 0.28 | 24 <0.28 | 28 0.15 | 29 <0.24 | 32 0.09 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 31.1 | 19.2 | 21.7 | <27.3 | 28.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 8.10 | 7.99 | 11.1 | 9.52 | 7.44 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06E | WVDG06S | WVDG06W | WVDD01 Soluble metals | WVDD02 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Nov-2022 10:10 | 28-Nov-2022 10:10 | 28-Nov-2022 10:10 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2223849-026 | EM2223849-027 | EM2223849-028 | EM2223849-029 | EM2223849-030 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.1 | <2 1.1 | <2 1.7 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 16 0.1 | 16 <0.1 | 24 <0.1 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 2 1.2 | <2 1.1 | <2 1.7 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 17 1.3 | 16 1.1 | 24 1.7 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 19 0.05 | 17 ---- | 25 0.21 | ---- 118 | ---- 49.9 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | ---- | <0.050 | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 27.3 | 4.06 | 37.5 | 966 | 264 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <9.15 | 0.61 | 9.80 | 795 | 1430 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble metals | WVDD04 Soluble metals | WVDD05 Soluble metals | WVDD06 Soluble metals | WVDD007BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-031 | EM2223849-032 | EM2223849-033 | EM2223849-034 | EM2223849-035 |
|  |  |  | Result 4.41 | Result 30.4 | Result 13.6 | Result 136 | Result 2.13 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 853 | 154 | 152 | 1210 | 179 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 432 | 759 | 852 | 5100 | 1020 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble metals | KF2 Soluble metals | KF3 Soluble metals | KF4 Soluble metals | KF5BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-036 | EM2223849-037 | EM2223849-038 | EM2223849-039 | EM2223849-040 |
|  |  |  | Result 6.50 | Result 16.4 | Result 2.61 | Result 23.9 | Result 4.53 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 208 | 128 | 142 | 336 | 179 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 462 | 298 | 237 | 708 | 331 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N Soluble metals | WVDG03E Soluble metals | WVDG03S Soluble metals | WVDG03W Soluble metals | WVDG04N Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-041 | EM2223849-042 | EM2223849-043 | EM2223849-044 | EM2223849-045 |
|  |  |  | Result 1.29 | Result 0.28 | Result 0.50 | Result 0.30 | Result 0.20 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 103 | 28.4 | 27.1 | 4.05 | 4.59 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 13.3 | 14.6 | 15.1 | 4.22 | 7.18 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E Soluble metals | WVDG04S Soluble metals | WVDG04W Soluble metals | WVDG05N Soluble metals | WVDG05E Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-046 | EM2223849-047 | EM2223849-048 | EM2223849-049 | EM2223849-050 |
|  |  |  | Result 0.23 | Result ---- | Result 1.93 | Result 0.59 | Result 0.31 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | ---- | <0.050 | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 24.0 | 21.8 | 11.0 | 24.4 | 18.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 67.8 | 15.1 | 34.1 | 80.9 | 12.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble metals | WVDG05W Soluble metals | WVDG06N Soluble metals | WVDG06E Soluble metals | WVDG06S Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-051 | EM2223849-052 | EM2223849-053 | EM2223849-054 | EM2223849-055 |
|  |  |  | Result ---- | Result ---- | Result ---- | Result 0.16 | Result ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | <0.050 | <0.050 | <0.050 | ---- | <0.050 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 17.2 | 13.9 | 17.3 | 23.3 | 15.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 23.3 | 3.24 | 6.98 | 14.0 | 8.89 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06W Soluble metals | WVDD01 Insoluble metals | WVDD02 Insoluble metals | WVDD03 Insoluble metals | WVDD04 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-056 | EM2223849-057 | EM2223849-058 | EM2223849-059 | EM2223849-060 |
|  |  |  | Result ---- | Result <33.5 | Result 4.37 | Result <2.84 | Result <5.22 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | <0.050 | ---- | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 19.9 | <1900 | <110 | <138 | <88.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 9.79 | <2260 | 151 | 56.1 | <122 |

Environmental

CERTIFICATE OF ANALYSIS

Work Order	: EM2300219

Page	: 1 of 19

Client

Contact

Address

: ALS WATER AND HYDROGRAPHICS PTY LTD

# : ROHAN OLIVER

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Bronwyn Sheen

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +6138549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 11-Jan-2023 10:35 |
| Order number | : ---- | Date Analysis Commenced | : 18-Jan-2023 |
| C-O-C number Sampler Site | : ---- : RO : ---- | Issue Date | : 25-Jan-2023 15:57 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Dilani Fernando	Laboratory Coordinator	Melbourne Inorganics, Springvale, VIC

R I G H T  S O L U T I O N S | R I G H T  P A R T N E R

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- Sampling Period:28/11/2022-29/12/2022.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- EM2300219 #13-28; Sample container was received with no liquid. 100ml of DI water was used to dissolve contents, perform analysis and for calculation of results.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 12:20 | 29-Dec-2022 12:30 | 29-Dec-2022 12:35 | 29-Dec-2022 13:00 | 29-Dec-2022 13:25 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-001 | EM2300219-002 | EM2300219-003 | EM2300219-004 | EM2300219-005 |
|  |  |  | Result 0.3 | Result <0.1 | Result 0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 6 8.6 | <2 2.1 | 2 3.0 | <2 2.6 | <2 2.1 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 156 6.2 | 40 4.2 | 54 2.1 | 48 1.4 | 39 0.1 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 113 8.9 | 77 2.2 | 38 3.1 | 26 2.7 | 3 2.2 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 162 15.1 | 41 6.4 | 56 5.2 | 50 4.1 | 41 2.3 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 275 <6.00 | 118 <3.00 | 94 <4.00 | 76 <10.5 | 44 <3.50 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <383 | <47.3 | <70.5 | <59.5 | <61.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 250 | 45.5 | 52.1 | 42.2 | <45.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD06 | WVDD007BG | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 11:50 | 29-Dec-2022 11:35 | 29-Dec-2022 15:15 | 29-Dec-2022 15:40 | 29-Dec-2022 15:25 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-006 | EM2300219-007 | EM2300219-008 | EM2300219-009 | EM2300219-010 |
|  |  |  | Result 0.1 | Result 0.4 | Result 0.2 | Result 0.2 | Result 0.2 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 3 2.5 | 8 7.1 | 3 2.3 | 3 2.9 | 3 0.8 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 45 1.7 | 131 5.1 | 43 2.3 | 54 1.2 | 15 0.6 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 32 2.6 | 93 7.6 | 42 2.5 | 23 3.1 | 11 1.0 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 48 4.3 | 139 12.7 | 46 4.8 | 57 4.3 | 18 1.6 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 80 <3.10 | 232 <9.50 | 88 7.27 | 80 209 | 29 <73.0 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <73.5 | <325 | <68.6 | <96.1 | <81.4 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <82.2 | <383 | 61.3 | 70.5 | 63.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 15:30 | 29-Dec-2022 15:00 | 29-Dec-2022 12:40 | 29-Dec-2022 12:45 | 29-Dec-2022 12:50 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-011 | EM2300219-012 | EM2300219-013 | EM2300219-014 | EM2300219-015 |
|  |  |  | Result 0.1 | Result 0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | 2 ---- | 2 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 0.8 | ---- 0.9 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 14 ---- | 17 ---- | ---- 2.5 | ---- 2.7 | ---- 2.5 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.1 | ---- 3.8 | 39 ---- | 42 ---- | 40 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | <2 ---- | 69 ---- | ---- 0.2 | ---- 0.2 | ---- 0.2 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.9 | ---- 1.0 | 4 ---- | 4 ---- | 3 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 16 ---- | 19 ---- | ---- 2.5 | ---- 2.7 | ---- 2.6 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 1.0 | ---- 4.8 | 40 ---- | 43 ---- | 41 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 17 ---- | 88 ---- | ---- 2.7 | ---- 2.9 | ---- 2.8 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- 27.7 | ---- 7.20 | 44 0.20 | 47 <0.70 | 44 0.70 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <57.4 | <61.6 | 149 | 50.3 | 31.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 49.3 | 103 | 4.00 | 19.9 | 15.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 12:55 | 29-Dec-2022 13:05 | 29-Dec-2022 13:10 | 29-Dec-2022 13:15 | 29-Dec-2022 13:20 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-016 | EM2300219-017 | EM2300219-018 | EM2300219-019 | EM2300219-020 |
|  |  |  | Result 0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 2 2.7 | <2 2.6 | <2 2.7 | <2 2.3 | <2 2.2 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 42 0.2 | 41 0.3 | 42 0.2 | 36 0.5 | 34 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 4 2.8 | 5 2.6 | 4 2.7 | 8 2.3 | <2 2.2 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 44 3.0 | 41 2.9 | 42 2.9 | 36 2.8 | 35 2.3 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 48 1.90 | 46 0.19 | 46 <0.50 | 44 <0.80 | 36 1.14 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 30.6 | 11.6 | 18.0 | <18.3 | <18.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 23.7 | 7.40 | 17.7 | <58.3 | <18.4 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG06N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 13:30 | 29-Dec-2022 13:35 | 29-Dec-2022 13:40 | 29-Dec-2022 13:45 | 29-Dec-2022 12:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-021 | EM2300219-022 | EM2300219-023 | EM2300219-024 | EM2300219-025 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.6 | <2 0.8 | <2 0.7 | <2 2.5 | <2 2.3 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 25 0.5 | 13 0.1 | 11 0.3 | 40 0.1 | 37 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 8 1.6 | <2 0.9 | 6 0.7 | 2 2.5 | <2 2.4 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 26 2.1 | 14 1.0 | 11 1.0 | 40 2.6 | 38 2.5 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 34 <0.35 | 15 0.84 | 17 <0.70 | 42 0.30 | 39 <1.20 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <21.3 | <23.5 | <21.3 | 20.6 | <31.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <14.8 | <24.0 | <19.7 | 8.82 | <23.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06E | WVDG06S | WVDG06W | WVDD01 Soluble metals | WVDD02 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 12:05 | 29-Dec-2022 11:55 | 29-Dec-2022 12:10 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2300219-026 | EM2300219-027 | EM2300219-028 | EM2300219-029 | EM2300219-030 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 3.0 | <2 3.0 | <2 2.6 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 48 0.7 | 48 0.5 | 41 0.4 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 11 3.1 | 8 3.1 | 6 2.7 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 49 3.8 | 49 3.6 | 42 3.1 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 60 <0.80 | 57 1.10 | 48 <2.70 | ---- 24.9 | ---- 6.40 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <20.5 | 24.3 | <40.8 | 228 | 90.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 13.6 | 31.2 | <49.0 | 945 | 266 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble metals | WVDD04 Soluble metals | WVDD05 Soluble metals | WVDD06 Soluble metals | WVDD07BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-031 | EM2300219-032 | EM2300219-033 | EM2300219-034 | EM2300219-035 |
|  |  |  | Result 47.0 | Result 7.30 | Result 4.07 | Result 36.0 | Result 7.30 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 389 | 102 | 86.0 | 227 | 183 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 414 | 395 | 268 | 1120 | 998 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble metals | KF2 Soluble metals | KF3 Soluble metals | KF4 Soluble metals | KF5BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-036 | EM2300219-037 | EM2300219-038 | EM2300219-039 | EM2300219-040 |
|  |  |  | Result 4.60 | Result 66.7 | Result 17.1 | Result 6.92 | Result 30.7 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 94.6 | 105 | 70.2 | 153 | 175 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 328 | 382 | 225 | 276 | 2521 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N Soluble metals | WVDG03E Soluble metals | WVDG03S Soluble metals | WVDG03W Soluble metals | WVDG04N Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-041 | EM2300219-042 | EM2300219-043 | EM2300219-044 | EM2300219-045 |
|  |  |  | Result 1.20 | Result 1.80 | Result 1.15 | Result 1.15 | Result 0.40 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 130 | 36.5 | 23.7 | 27.6 | 19.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 42.0 | 45.0 | 54.0 | 30.0 | 28.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E Soluble metals | WVDG04S Soluble metals | WVDG04W Soluble metals | WVDG05N Soluble metals | WVDG05E Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-046 | EM2300219-047 | EM2300219-048 | EM2300219-049 | EM2300219-050 |
|  |  |  | Result 0.40 | Result 1.00 | Result 1.20 | Result 0.60 | Result 0.60 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 22.0 | 18.5 | 19.5 | 22.5 | 18.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 34.0 | 37.2 | 19.5 | 30.5 | 41.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble metals | WVDG05W Soluble metals | WVDG06N Soluble metals | WVDG06E Soluble metals | WVDG06S Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-051 | EM2300219-052 | EM2300219-053 | EM2300219-054 | EM2300219-055 |
|  |  |  | Result 0.40 | Result 0.50 | Result 0.50 | Result 3.20 | Result 3.80 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 19.0 | 18.0 | 19.0 | 22.1 | 41.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 30.0 | 26.3 | 41.2 | 43.4 | 44.2 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06W Soluble metals | WVDD001 Insoluble metals | WVDD002 Insoluble metals | WVDD003 Insoluble metals | WVDD004 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-056 | EM2300219-057 | EM2300219-058 | EM2300219-059 | EM2300219-060 |
|  |  |  | Result 2.80 | Result <6.00 | Result <3.00 | Result <4.00 | Result <10.5 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 18.8 | <383 | <47.3 | <70.5 | <59.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 36.2 | 285 | 47.3 | 53.6 | 55.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD005 Insoluble metals | WVDD006 Insoluble metals | WVDD007BG Insoluble metals | KF1 Insoluble metals | KF2 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-061 | EM2300219-062 | EM2300219-063 | EM2300219-064 | EM2300219-065 |
|  |  |  | Result <3.50 | Result <3.10 | Result <9.50 | Result 8.40 | Result 367 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <61.3 | <73.5 | <325 | <68.6 | <96.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <45.8 | <82.2 | <383 | 82.7 | 141 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 Insoluble metals | KF4 Insoluble metals | KF5BG Insoluble metals | WVDG03N Insoluble metals | WVDG03E Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-066 | EM2300219-067 | EM2300219-068 | EM2300219-069 | EM2300219-070 |
|  |  |  | Result <73.0 | Result 40.0 | Result 8.20 | Result 0.60 | Result <0.70 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <81.4 | <57.4 | <61.6 | 2074 | 80.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 92.7 | 92.6 | 154 | 28.7 | 25.2 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Insoluble metals | WVDG03W Insoluble metals | WVDG04N Insoluble metals | WVDG04E Insoluble metals | WVDG04S Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-071 | EM2300219-072 | EM2300219-073 | EM2300219-074 | EM2300219-075 |
|  |  |  | Result 2.30 | Result 9.20 | Result 0.40 | Result <0.50 | Result <0.80 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 47.1 | 37.7 | 19.1 | 21.2 | <18.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 37.0 | 29.4 | 13.4 | 19.3 | <58.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Insoluble metals | WVDG05N Insoluble metals | WVDG05E Insoluble metals | WVDG05S Insoluble metals | WVDG05W Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-076 | EM2300219-077 | EM2300219-078 | EM2300219-079 | EM2300219-080 |
|  |  |  | Result 5.40 | Result <0.35 | Result 5.46 | Result <0.70 | Result 0.46 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <18.6 | <21.3 | <23.5 | <21.3 | 22.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <18.4 | <14.8 | <24.0 | <19.7 | 11.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06N Insoluble metals | WVDG06E Insoluble metals | WVDG06S Insoluble metals | WVDG06W Insoluble metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | 29-Dec-2022 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2300219-081 | EM2300219-082 | EM2300219-083 | EM2300219-084 | -------- |
|  |  |  | Result <1.20 | Result <0.80 | Result 1.11 | Result <2.70 | ---- ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <31.5 | <20.5 | 26.8 | <40.8 | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <23.5 | 18.7 | 33.6 | <49.0 | ---- |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD05 Insoluble metals | WVDD06 Insoluble metals | WVDD007BG Insoluble metals | KF1 Insoluble metals | KF2 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-061 | EM2223849-062 | EM2223849-063 | EM2223849-064 | EM2223849-065 |
|  |  |  | Result <3.07 | Result <50.3 | Result 2.55 | Result <26.4 | Result 77.5 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <84.2 | <1480 | <64.8 | <83.2 | <82.8 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <144 | <778 | 72.6 | 85.1 | 113 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 Insoluble metals | KF4 Insoluble metals | KF5BG Insoluble metals | WVDG03N Insoluble metals | WVDG03E Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-066 | EM2223849-067 | EM2223849-068 | EM2223849-069 | EM2223849-070 |
|  |  |  | Result <15.9 | Result <32.8 | Result 4.44 | Result <0.21 | Result <0.38 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <73.1 | <211 | 78.1 | <893 | 53.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 112 | <159 | 65.8 | <12.4 | 13.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Insoluble metals | WVDG03W Insoluble metals | WVDG04N Insoluble metals | WVDG04E Insoluble metals | WVDG04S Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-071 | EM2223849-072 | EM2223849-073 | EM2223849-074 | EM2223849-075 |
|  |  |  | Result <0.15 | Result 4.83 | Result <0.62 | Result 0.21 | Result <0.35 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 25.7 | <49.1 | <29.0 | 25.2 | 27.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 10.2 | 36.0 | 14.8 | 18.2 | <15.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Insoluble metals | WVDG05N Insoluble metals | WVDG05E Insoluble metals | WVDG05S Insoluble metals | WVDG05W Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-076 | EM2223849-077 | EM2223849-078 | EM2223849-079 | EM2223849-080 |
|  |  |  | Result <6.53 | Result 0.93 | Result <0.28 | Result 0.35 | Result <0.24 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 52.6 | 31.4 | 22.6 | 26.2 | <27.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <23.1 | 9.35 | 12.7 | 13.7 | 11.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG06N Insoluble metals | WVDG06E Insoluble metals | WVDG06S Insoluble metals | WVDG06W Insoluble metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | 07-Dec-2022 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2223849-081 | EM2223849-082 | EM2223849-083 | EM2223849-084 | -------- |
|  |  |  | Result ---- | Result 0.18 | Result ---- | Result 0.22 | ---- ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | <0.050 | ---- | <0.050 | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 43.6 | 62.3 | 26.1 | 39.1 | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 8.03 | <9.15 | 9.69 | 14.6 | ---- |