Department of Energy, Environment and Climate Action

Woodvale dust monitoring report q2 2023

28 th august 2023

CERTIFICATE OF APPROVAL FOR ISSUE OF DOCUMENTS

Client:	Department of Energy, Environment and Climate Action

Project Title:	Woodvale Dust Monitoring Report Q2 2023

Title:		Woodvale Dust Monitoring Report Q2 2023

Project: 	MV214940

Document Status:	Version 1- Approved

Date of Issue:		 28/08/2023

|  | Position | Name | Initials | Date |
| --- | --- | --- | --- | --- |
| Prepared by: | Northern Victorian Regional Manager | Rohan Oliver | RO | 8/08/2023 |
| Technical Review by: | Victorian Data Manager | Karen Bootland | KB | 14/08/2023 |
| Client: | Department of Energy, Environment and Climate Action (DEECA) | Ben Gazi | DEECA | 28/08/2023 |

For further information on this report, contact:

Name:		Rohan Oliver
Title:		Northern Victorian Regional Manager
Address:	94 Koondrook Rd, Kerang VIC 3579
E-mail:		Rohan.Oliver@alsglobal.com

© ALS Water and Hydrographics Pty Ltd

This report is prepared for the Client, Department of Energy, Environment and Climate Action for the purpose of monitoring dust at the former Bendigo Mine Woodvale site which is currently in care and maintenance. An unrestricted copy of this report will be shared with the public via the Earth Resources Regulation website.

Disclaimer

This document has been prepared for the Client above and is to be used only for the purposes for which it was commissioned. While all due skill and attention has been taken in collecting, validating and providing the data contained in this report, ALS shall not be liable in any way for loss of any kind including damages, costs, interest, loss of profits or special loss or damage, arising from any error, inaccuracy, incompleteness or other defect in this information. No warranty is given as to its suitability for any other purpose.

ALS Water and Hydrographics Pty Ltd ABN 94 105 060 320

ALS Australia Pty Ltd trading as ALS Hydrographics

Table of Contents

Executive Summary	5

1.	Introduction	7

2.	Background	7

2.1	Scope and Objective	8

2.2	Regulatory Environment	9

2.3	What is Dust	10

3.	Measurement of Dust	11

3.1	Site Locations	11

3.2	Deposited Matter	12

3.3	Directional Deposited Dust Monitoring	13

3.4	Metals/Metalloids Monitoring in Dust	15

3.5	Ash Content	15

3.6	Dust Monitoring Standards	15

3.7	Dust Monitoring Program	16

4.	Dust Deposition Results	17

4.1	Woodvale Evaporation Ponds – Q2 2023	17

4.1.1 Deposition of Total Insoluble Matter	17

4.1.2 Mineral Content in Dust	18

4.1.3 Metals/Metalloids in Dust	19

4.1.4 Woodvale Directional Dust Deposition	20

4.2	Review of Quality Assurance and Quality Control	23

5.	Weather Station Results Woodvale Q2	24

5.1	Stacked data plot April 2023	24

5.2	Wind Rose plot April 2023	25

5.3	Stacked data plot May 2023	26

5.4	Stacked data plot May 2023	27

5.5	Stacked data plot June 2023	28

5.6	Wind Rose plot June 2023	29

6.	Summary of Results	30

7.	Further Considerations	30

8.	References	31

9.	Tables	33

10.	Supplementary Data	54

10.1	Arsenic	55

10.2	Barium	57

10.3	Manganese	59

10.4	Trend Analysis Graphs	61

11.	Appendix A – Supplementary Information	69

12.	Appendix B – Laboratory Reports	72

Table of Figures

Figure 1 - Monitoring Locations at Woodvale Ponds	11

Figure 2 - Control Site Location WVDD07 (BG)	11

Figure 3 - Dust Deposition Gauge WVDD01	13

Figure 4 - Directional Dust Deposition Gauge WVDG08	14

Figure 5 - Woodvale - Total Insoluble Matter (TIM) g/m2/month – Q2 2023	17

Figure 6 - Woodvale - Total Ash Content g/m2/month – Q2 2023	18

Figure 7 - Woodvale - Ash Content/TIM Ratio – Q2 2023	18

Figure 8 - Woodvale - Total Arsenic/Total Solids mg/kg – Q2 2023	19

Figure 9 - Woodvale - Directional Dust Deposition Results - Total Solids g/m2/month – Q2 2022 (North/South)	20

Figure 10 - Woodvale - Directional Dust Results – Arsenic (mg)/dust(kg) – Q2 2022 (North/South)	21

Figure 11 - Woodvale - Directional Dust Deposition Results - Total Solids g/m2/month – Q2 2023 (East/West)	21

Figure 12 - Woodvale - Directional Dust Results - Arsenic (mg/dust kg) – Q2 2023 (East/West)	22

Figure 13 - Stacked data plot April 2023, 407810A Weather Station	24

Figure 14 - Woodvale Wind Rose plot April 2023	25

Figure 15 - Stacked data plot May 2023, 407810A Weather Station	26

Figure 16 - Woodvale Wind Rose plot May 2023	27

Figure 17 - Stacked data plot June 2023, 407810A Weather Station	28

Figure 18 - Woodvale Wind Rose plot June 2023	29

Figure 19 - Woodvale - Arsenic in Soluble Fraction – Q2 2023	55

Figure 20 - Woodvale - Arsenic in Insoluble Fraction – Q2 2023	55

Figure 21 - Woodvale - Arsenic in Soluble Fraction Year Review (2022-23)	56

Figure 22 - Woodvale - Arsenic in Insoluble Fraction Year Review (2022-23)	56

Figure 23 - Woodvale - Barium in Soluble Fraction – Q2 2023	57

Figure 24 - Woodvale - Barium in Insoluble Fraction – Q2 2023	57

Figure 25 - Woodvale - Barium in Soluble Fraction Year Review (2022-23)	58

Figure 26 - Woodvale - Barium in Insoluble Fraction Year Review (2022-23)	58

Figure 27 - Woodvale - Manganese in Soluble Fraction – Q2 2023	59

Figure 28 - Woodvale - Manganese in Insoluble Fraction – Q2 2023	59

Figure 29 - Woodvale - Manganese in Soluble Fraction Year Review (2022-23)	60

Figure 30 - Woodvale - Manganese in Insoluble Fraction Year Review (2022-23)	60

Figure 31 - Woodvale - Total Insoluble Matter (TIM) - 2022-2023	61

Figure 32 - Woodvale - Total Ash Content - 2022-2023	62

Figure 33 - Woodvale - Ash Content/TIM Ratio - 2022-2023	63

Figure 34 - Woodvale - Total Arsenic/Total Solids - 2022-2023	64

Figure 35 - Woodvale - DDG Total Solids Comparison North/South - 2022-2023	65

Figure 36 - Woodvale - Arsenic/Total Solids Comparison - North/South 2022-2023	66

Figure 37 - Woodvale - DDG Total Solids Comparison East/West - 2022-2023	67

Figure 38 - Woodvale - Arsenic/Total Solids Comparison - East/West - 2022-2023	68

Table of Tables

Table 1 - Woodvale Dust Deposition Analysis Results Q2, 2023	33

Table 2 - Woodvale Directional Dust Deposition Analysis Results Q2, 2023	41

# Executive Summary

ALS was engaged by the Department of Energy, Environment and Climate Action (DEECA), Earth Resources Regulator (ERR) to undertake dust monitoring at the Woodvale Evaporation Ponds from the start of January 2021. This report provides results and analysis of data collected from sampling in Q2 April - June 2023.

Department of Energy, Environment and Climate Action (DEECA)inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, when owner Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, DEECA engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust, and in particular the arsenic content of the dust.

This report analyses the data gathered in Q2 2023 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results. The results of dust monitoring in the first half of 2021 were published in a report by CDM Smith. Since Q3 2021, ALS has completed the collection of samples/monitoring, analysis of lab results (Springvale ALS Laboratory - NATA accredited) and drafted the dust monitoring reports.

The Woodvale Evaporation Ponds are located 13 km north of Bendigo. The ponds were formerly used to hold groundwater extracted from dewatering historic underground mine workings at Kangaroo Flat. The site is currently in care and maintenance. DEECA is continuing to monitor the site by regular site visits, observing weather conditions, and proactively applying a range of dust control measures. These measures include covering ponds with water, vegetation cover, mulch application and keeping a water/polymer spray truck on standby, should the need arise to use such control measures.  ALS field staff observed that all monitoring sites where in good working order with no obvious variations to sites that would reflect abnormalities in results.

The original scope of dust monitoring works at Woodvale was to operate and maintain six depositional dust gauges and two directional dust deposition gauges. ALS has added a further three monitoring sites to the dust monitoring program at Woodvale, i.e. one control site to obtain background data, and two directional dust deposition gauges at the east and west of the Woodvale site, to provide more complete data from directional dust monitoring. These newly added sites have been in operation since 1 October 2021.

The purpose of dust monitoring is to assure that dust issues are being minimised and managed effectively and to gather baseline data and report exceedances when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will inform decision making on rehabilitation requirements and measures required to ensure works meet air quality standards (where available).

The results for Q2 2023 recorded consistent results to historical records recorded in 2022 and were comparative to previous reports supplied (Kralcopic Report in 2018), by CDM Smith Report for the first half of 2021 and all ALS Reports.

As done previously in historic reports, the following questions are answered for Q2 2023:

- Did the dust deposition rates recorded in Q2 2023, exceed the adopted dust deposition criteria of 4g/m2/month, measured as Total Insoluble Matter (TIM)?

In Q2 2023, there were two instances where the dust levels were over the 4 g/m2/month criteria used in this report as follows:

- WVDD01(S), during April 2023 (17.4 g/m2/month)

- WVDD07(BG), during May 2023 (10.1 g/m2/month)

However, it should be noted that one of these occurrences was at the background site WVDD07 (BG) and therefore cannot be attributed to the Woodvale Evaporation Ponds. When the data from all source monitoring sites except WVDD07 (BG) (the background reference location) were averaged for Q2 2023, an average of 1.8 g/m2/month was recorded. This value is below the 4 g/m2/month criteria and the inferred background plus 2 g/m2/month criteria in the Mining PEM.

The dust levels recorded at WVDD01(S) exceeded during April which was most likely influenced by agricultural activities located nearby to the monitoring sites. Wind Rose data for April shows predominantly southerly winds during the period. This indicates that potential dust levels can be influenced by the surrounding land activities as more northerly winds would influence across the evaporation ponds.

WVDD06 was relocated on 1/3/2023 and results for the relocated site now known as WVDD08 can be found within this report.

The surrounding land use to the north, south and west is predominantly agricultural farmland with reserved parklands to the east of the Woodvale site. ERR conduct daily site inspections to ensure dust control measures (water cover, dampness, foliage) are adequate.

- Are dust deposition rates recorded in Q2 2023 comparable to available historic results in this area?

Overall, results for Q2 were lower than the previous quarter and only one result at WVDD01 during Q2 in April recorded above the 4.0 g/m2/month.

Results for Q2 2023 were within guidelines set in the PEM with the only exception WVDD01 (April). The results were similar to the same Q1 of 2023 and can be seen in Figure 25 - Woodvale - Total Insoluble Matter (TIM) - 2022-2023

- Are arsenic concentrations recorded in deposited dust in Q2 2023, comparable to available historic results in this area?

Yes, they are comparable. Arsenic results were consistent throughout the quarter with only one exception recorded at WVDD01 in April. This can be reviewed in section 10.1 of this report.

A high reading was recorded in April 2023 with a result of 120ug/m2 at WVDD01 as analysed by the laboratory. When calculating monthly assessment levels, Soluble Arsenic, Insoluble Arsenic and Total Solids are calculated to give an average result of 8.7mg/kg at WVDD01 (S). This value is still well below the 100 mg/kg assessment level as referenced in the EPA Publication 1191 – Protocol for Environmental Management – Mining and Extractive Industries (Mining PEM).

It is noted that in addition to the results above, the directional dust deposition gauge results for Q2 2023 showed that, there is where small overall net gains for south facing gauges in dust, although this is not represented as an increase in total solids/arsenic concentrations as can be seen in Figure 10.

# Introduction

ALS was engaged by the Earth Resource Regulator in the Department of Jobs, Precincts and Regions (DJPR) to undertake dust monitoring at the Woodvale Evaporation Ponds Complex from the start of January 2021. On 1 January 2023 ERR joined a new Government department, the Department of Energy, Environment and Climate Action (DEECA). The ongoing engagement of ALS is now with DEECA Earth Resources Regulator (DEECA(ERR)). This report provides results and analysis of data collected from sampling in Q2 2023. ALS undertook the field component of maintaining dust gauges and replacing sample containers, as well as laboratory analysis of samples collected.  Samples were collected from locations specified by DJPR on a routine monthly basis. These locations are displayed in Section 3.1 of this report.

# Background

In April 2021, ERR inherited the responsibility for care and maintenance and rehabilitation of the former Bendigo Gold Mine sites, as its owner Kralcopic Pty Ltd went into liquidation. Prior to inheriting this responsibility, ERR engaged ALS to undertake depositional dust monitoring at the former Bendigo Mine sites, i.e. Kangaroo Flat and Woodvale, to proactively address ongoing community concerns relating to dust, and in particular the arsenic content of the dust. This report analyses the data gathered in Q2 2023 for the Woodvale site by comparing the levels to accepted standards (where available) and previous dust monitoring results published in:

- ALS Report for Q3 2021 - Present

- CDM Smith Report for the first half of 2021

- Kralcopic Report for Q4 2018.

All reports are referenced in this document. To maintain continuity and comparability of results, this document has been prepared in a similar format to the CDM Smith Report.

The Woodvale Evaporation Ponds are located 13 km north of Bendigo. The ponds were formerly used to hold groundwater extracted from dewatering historic underground mines including Kangaroo Flat and New Moon mines. The site is currently in care and maintenance while DEECA develop the rehabilitation plan for the site. DEECA is continuing to monitor the site by regular site visits, observing weather conditions, and proactively applying a range of dust control measures. Dust suppression measures include ensuring water or vegetation cover, application of mulch and keeping a water/polymer spray truck on standby, should the need arise to use such control measures.

To maintain continuity and comparability of data, monitoring was undertaken at the same locations as previously, and similar methodology and analysis used. This report builds on the previous results and provides baseline information to potentially inform decisions around some aspects of site rehabilitation.

## Scope and Objective

The scope of dust monitoring works at Woodvale is to operate and maintain seven depositional dust gauges and four directional dust deposition gauges including gathering samples and sending off to the laboratory for analysis, analysing the data from the laboratory and producing quarterly dust monitoring reports consolidating results and comparing to historic and guideline levels.

All depositional dust gauges except for the background gauge (WVDD07 (BG)) are located at their historic locations with the following naming conventions since the dates specified below:

| - WVDD01 (S) Jan 2021 - WVDD02 (N) Jan 2021 - WVDD03 (N) Jan 2021 - WVDD04 (E) Jan 2021 | - WVDD05 (S) Jan 2021 - WVDD06 (W) Jan 2021 - WVDD07 (BG) Sep 2021 - WVDD08 (W) Mar 2023 |  |
| --- | --- | --- |

The location of the six depositional dust gauges at the Woodvale site can be seen in Figure 1. The location of the background depositional dust gauge is in Figure 2 which serves as a control site. A new depositional dust gauge WVDD08, was installed during March which saw the decommissioning of gauge WVDD06.

Similarly, four directional dust deposition gauges were installed since the dates specified below at the following locations:

- WVDG03 (N) Jan 2021

- WVDG04 (E) Oct 2021

- WVDG05 (S) Jan 2021

- WVDG06 (W) Oct 2021

These directional gauges are located next to the depositional dust gauges: WVDD03 (N), WVDD04 (E), WVDD05 (S) and WVDD006 (W).

Laboratory analysis requirements were provided by DEECA and were in line with historical requirements during the previous monitoring program undertaken by Kralcopic to meet its legislative requirements. This alignment maintains continuity and comparability of results.

The purpose of dust monitoring is to assure that dust issues are being minimised and managed effectively and to gather baseline data when the site is in care and maintenance, including testing the effectiveness of dust control measures. These results will potentially inform decisions around some aspects of site rehabilitation and any measures required to ensure works meet air quality standards (where available).

As done previously, the following considerations were included in assessment of Q2 2023 data:

- Did the dust deposition rates recorded in Q2 2023, exceed the adopted dust deposition criteria of 4g/m2/month, measured as Total Insoluble Matter (TIM)?

- Are dust deposition rates recorded in Q2 2023, comparable to available historic results in this area?

- Are arsenic concentrations recorded in deposited dust in Q2 2023, comparable to available historic results in this area?

## Regulatory Environment

The Environment Protection Authority (EPA) has overarching responsibility for administering legislation related to the air quality across Victoria. The EPA administers the Environment Protection Act 2017 and the Environment Protection Regulations 2021 under which Environmental Reference Standards (ERS) and related Guidelines are provided.

The purpose of the ERS is to support the protection of human health and the environment from harms posed by pollution and waste. The ERS does this by providing benchmarks to assess and report on environmental conditions in the whole or any part of Victoria.

The ERS seeks to achieve this purpose by:

- identifying environmental values to be achieved or maintained in the whole or any part of Victoria; and

- specifying indicators and objectives to be used to measure, determine or assess whether those environmental values are being achieved, maintained or threatened.

The ERS provides guidance, objectives and levels on values where the environment may be impacted and incorporates the Ambient Air Quality National Environment Protection Measure (NEPM AAQ).

The main guidance document used for the purpose of this report is EPA Publication 1191 – Protocol for Environmental Management – Mining and Extractive Industries (Mining PEM). The criteria in the Mining PEM  used for this report has been carried over to the EPA’s latest Guideline for Assessing and Minimising Air Pollution in Victoria, Publication 1961, February 2022. As stated in PEM, Results of monitoring should not exceed 4g/m2/month (no more than 2g/m2/month above background) as a monthly average.

Under the new EPA legislation, duty holders have responsibility under the general environmental duty to apply controls to eliminate or minimise risks to human health and the environment as far as reasonably practicable. This means that the primary focus is on applying effective elimination, minimisation and control measures. Monitoring is for the purpose of determining the effectiveness of control measures.

## What is Dust

Dust is typically not classified due to its composition but based on its particle size, as follows:

- Deposited matter - any particles that fall out of suspension in the atmosphere.

- Total Suspended Particles (TSP) - particles suspended or entrained in the air. Typically, this is particles of 30 µm (0.03 mm) equivalent aerodynamic diameter or less. Larger particles tend not to become suspended.

- PM10 - particles 10 µm equivalent aerodynamic diameter or less.

- PM2.5 - particles 2.5 µm equivalent aerodynamic diameter or less.

Dust particle size is an important consideration influencing dispersion and transport in the atmosphere and potential effects on human health. Human activities (e.g. energy use, transport, industrial activities etc) can affect the air quality, in particular airborne dust.

Potential sources of dust are:

- natural sources such as dust storms, agricultural dust, bushfires, vegetation, pollen and fungi; and

- anthropogenic sources such as mines sites, industry, roads and vehicles, construction sites, domestic and diffuse sources (CDM Smith, 2021).

# Measurement of Dust

## Site Locations

Locations of depositional dust gauges are as presented in Figure 1 and Figure 2. Directional depositional dust gauges WVDG03 (N), WVDG04 (E), WVDG05 (S) and WVDG06 (W) have been installed next to the respective depositional gauges as mentioned above.

Figure 1 - Monitoring Locations at Woodvale Ponds

Figure 2 - Control Site Location WVDD07 (BG)

Samples were collected in accordance with the Standard AS/NZS 3580.10.1 - Methods for sampling and analysis of ambient air - Determination of particulate matter - Deposited matter - Gravimetric method and ALS Dust Sampling and Depositional Dust Gauge Work Procedure MAT-MAP1000 (See Appendix A).

ALS Laboratory in Springvale analysed the samples for the following:

- Total deposited matter

- Total ash content

- Metals/metalloids, in the soluble, insoluble and ash content fraction of dust.

## Deposited Matter

The Standard (AS/NZS 3580.10.1:2016) sets out a method (used in this report) for sampling particulate matter that is deposited from the atmosphere. This Standard also sets out procedures for the gravimetric (i.e. by weighing) determination of the mass deposition rate of insoluble/soluble solids, ash/combustible matter, and total solids from ambient air.

The method provides an estimate of the mean surface concentration of deposited matter settling from the air over a sampling period, typically of one month. Particulate matter deposition rates of 0.1 g/m2/month and above may be determined using a monthly sampling period. The sample obtained by the sampling procedure specified may be subjected to physical or chemical analysis.

Over a given sampling period, particles that settle from the ambient air are collected in a vessel (see sampling bottle in Figure 3 below) and retained together with any rainwater. The sample is passed through a sieve to remove any extraneous matter (e.g. leaves, insects), and the sieved sample containing the deposited matter is transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of the dried insoluble solids is gravimetrically determined.

The ash and combustible matter content are determined by loss on ignition of the insoluble solids. Soluble solids are determined from the filtrate. The total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass deposition rate of deposited matter is then calculated from the mass of solids obtained, the funnel cross-sectional area and the exposure period. (AS/NZS 3580.10:2016)

New depositional gauges were installed as part of a renewals program for Woodvale. Gauges were replaced at all locations to meet standards for dust collection.

Figure 3 - Dust Deposition Gauge WVDD01

## Directional Deposited Dust Monitoring

The Standard (AS/NZS 3580.10.2:2013) states that over a given period, particles which impinge on the vertical, internal surface of four collecting cylinders of the directional dust gauge are sampled in collecting pots at the bases of the cylinders. Refer to Figure 4.

The samples are passed through a sieve to remove any extraneous matter (e.g. leaves, insects), with the sieved sample containing the impinged matter subsequently transferred to a filtration apparatus. The insoluble and soluble materials are separated by filtration, and the mass of dried insoluble solids determined gravimetrically. Soluble solids are determined by evaporation of the filtrate and gravimetric analysis.

The ash and combustible matter content are determined by loss on combustion of the insoluble solids. Total solids are obtained by the addition of the insoluble solids and the soluble solids. The mass flux rate of impinged matter for each collecting cylinder is calculated from the mass of solids obtained, the slot cross-sectional area and the exposure period.

A directional dust gauge consists of four vertical collecting cylinders, each with a slot of dimensions 45 mm by 340 mm, commencing approximately 10 mm from the top of the cylinder. The edges of each slot shall be at right angles to the width of the slot and not radial (see Figure 4). The cylinders shall be mounted at 90o to each other with the slots facing outwards. An easily detachable collecting pot shall be fitted to the bottom of each collecting cylinder. The combined length of a collecting cylinder and its attached collecting pot shall be at least 640 mm, with the collecting pot having a capacity of approx. 1 L.

The collecting cylinders and pots shall be constructed of chemically resistant material such as polyvinyl chloride. The external diameter of the collecting cylinders shall be 75 mm. The collecting cylinders shall be located with their axes at the corners of a 150 mm square mounting plate. (AS/NZS 3580.10.2:2013)

Figure 4 - Directional Dust Deposition Gauge WVDG08

## Metals/Metalloids Monitoring in Dust

The inclusion of the analysis of metals/metalloids in the dust samples assist in identify possible sources of contamination if present.

Analysis for insoluble matter, soluble matter and ash content was undertaken for the following:

- Arsenic

- Barium

- Manganese

It is understood that there are no specific threshold values for metals and metalloids in deposited dust either in the former Mining Licence or within the ERS. (CDM Smith, 2021)

## Ash Content

Ash Content is the remaining material after the sample has been combusted in the laboratory. Ash content provides an indication of the mineral content (or soil dust) of the sample. The mineral content may be attributable to on site contributions, but may also be attributable to other sources such as agriculture, unsealed roads, etc.

## Dust Monitoring Standards

Results were compared against relevant dust deposition rate criteria described by EPA Publication 1191  (2007) as shown below which has been adopted in EPA Publication 1961 (2022):

The following points apply to the criteria:

- Results of monitoring should not exceed 4 g/m2/month (no more than 2 g/m2/month above background) as a monthly average.

- The 2 g/m2/month criteria are used when baseline data on deposited dust levels exist, while the 4 g/m2/month criteria is used when no baseline data exists.

- If dust levels exceed this value, then site management practices should be reviewed, and dust controls implemented to reduce dust levels to within these guidelines.

- The criteria refer to all sources of deposited matter (including sources from mines, agriculture, unsealed roads, etc) and cumulative impacts.

- The criteria states that in some cases, a mine may increase deposited dust levels by up to 2 g/m2/month. However, the total deposited dust level (including sources from mines, agriculture, unsealed roads, etc) must not exceed 4 g/m2/month.

Deposited matter (dust) can be used as an indicator of the effectiveness of site management practices and the potential for offsite nuisance (fugitive dust). Deposited dust monitoring, when conducted over a set time period, is useful for examining trends and evaluating deviation from long term trends with respect to site activities.

## Dust Monitoring Program

The current method of undertaking depositional dust monitoring is considered sufficient as the site is in care and maintenance i.e. no new pollution is being introduced to the site from industry. There is a considerable body of knowledge about the contamination level at the site, so all that is required at this stage is to determine whether this contamination is becoming air borne and becoming a health hazard or problem to the nearby landholders. Depositional dust monitoring provides a range of useful information to be able to understand what’s happening at this site and its surrounds.

Depositional dust monitoring can provide the following invaluable information:

- test the effectiveness of dust minimisation, control and management measures

- identify key problematic sources, or groups of sources on larger more complex sites

- identify where dust sensitivities may occur

- characterise temporal or spatial trends

- metallic/metalloid concentrations are useful for health effect considerations.

# Dust Deposition Results

## Woodvale Evaporation Ponds – Q2 2023

Dust deposition analysis results are presented in Table 1 - Woodvale Dust Deposition Analysis Results and the accompanying Laboratory Certificates of Analysis are presented in Appendix B - Laboratory Reports.

Where results were reported as “less than” (<) a specified number (laboratory reporting limit) the number reported was adopted for use in the graphical displays. This approach adopts a conservative value for the number being reported.

## 4.1.1 Deposition of Total Insoluble Matter

Total Insoluble Matter (TIM) met the recommended criteria of less than 4 g/m2/month on 19 of 21 samples during Q2 2023. See Figure 5 below.

Dust deposition rate exceeded the assessment criteria at the sites below:

- WVDD01(S), during April 2023 (17.4 g/m2/month)

- WVDD07(BG), during May 2023 (10.1 g/m2/month)

When the data from all source monitoring sites except WVDD07 (BG) (the background reference location) were averaged for Q2 2023, an average of 1.8 g/m2/month was recorded. This value is below the 4 g/m2/month criteria and the inferred background plus 2 g/m2/month criteria in the Mining PEM.

Figure 5 - Woodvale - Total Insoluble Matter (TIM) g/m2/month – Q2 2023

Red dotted line is the criteria used for the purpose of this report.

## 4.1.2 Mineral Content in Dust

The fraction of dust measured as ash content is displayed below in Figure 6. As expected, the increased levels in TIM (Figure 5) generally corresponds with total ash content in Figure 6.

Figure 6 - Woodvale - Total Ash Content g/m2/month – Q2 2023

To gain a general understanding of what proportion of the sample is ash content i.e., mineral dust, the ratio of total ash content and TIM has been plotted in Figure 7.

Figure 7 - Woodvale - Ash Content/TIM Ratio – Q2 2023

## 4.1.3 Metals/Metalloids in Dust

Graphical presentations of soluble and insoluble metals (arsenic, barium and manganese) are provided in Section 9 of this report. Arsenic, Barium and Manganese concentrations for Q2 2023 were consistent with historical ranges and results were comparable to results reported in the CDM Smith Report for the first half of 2021 and in the ALS Reports for Q3 2021 to present.

To provide an understanding of total arsenic concentrations measured in dust relative to concentrations measured in soils within the region, total arsenic concentration has been displayed below Figure 8 in mg/kg. This is calculated by adding soluble and insoluble arsenic µg/m2/month dividing by the total solids g/m2/month.

This ratio has been plotted as this provides a value which could be compared to the figures quoted in the guideline, Contaminated Sites Management Series - Assessment Levels for Soil, Sediment and Water. The assessment level for soil for arsenic is 100 mg/kg for standard residential areas in this guideline.

Figure 8 - Woodvale - Total Arsenic/Total Solids mg/kg – Q2 2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Total arsenic concentrations for Q2 ranged from 0 to 30.3 mg/kg with a mean of 7.5 mg/kg for Q2 2023 not including the background site.

Highest concentrations were recorded in May 2023, 30.3 mg/kg at WVDD02 (N). This value is still well below the 100 mg/kg assessment level mentioned above. The high Tim reading recorded at WVDD01was also reflected in higher readings in total arsenic.

## 4.1.4 Woodvale Directional Dust Deposition

Results for directional dust deposition gauges are summarised in Table 2 - Woodvale Directional Dust Deposition Analysis Results, with Laboratory Analytical Results attached in Appendix B. As mentioned previously, for the purpose of continuity, the same approach for review of dust from the directional dust deposition gauges as in the CDM Smith Report has been applied here for Q2 2023 results.

The results from the northerly and southerly facing quadrants at the two directional dust deposition gauges WVDG03 (N) and WVDG05 (S) have been compared in Figure 9. Figure 9 shows the net gain or loss in dust deposition downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

To undertake this analysis, the Northern quadrant dust gauge at WVDG03 is assumed to measure dust coming onto the site in Northerly winds. This is then compared to the Northern quadrant dust gauge at WVDG05. The variance is then used to indicate dust contributions to the air from the site. A positive variance indicates an increase in dust to the atmosphere from the ponds. A negative variance indicates the ponds have removed more dust from the air than they contribute. This theoretical analysis is repeated for the site from each axis for directional dust gauge locations.

From the graph below (Figure 9) it can be seen that there is both net gain and loss in dust observed through the Q2 2023 monitoring period.

Small gains as seen on the south facing gauges can be expected and may be influenced by the Woodvale site and environmental conditions. Note that there is agricultural farmland to the north, south and west of the Woodvale site and reserved parklands to the east of the Woodvale site, which can all contribute to the dust levels at these sampling locations.

Figure 9 - Woodvale - Directional Dust Deposition Results - Total Solids g/m2/month – Q2 2022 (North/South)

Similarly, the concentration of arsenic was compared using the same methodology as above, and it was found that low net gains and losses occurred for 1 of the 6 values recorded during Q2 2023.

Figure 10 presents the net gain and loss in total arsenic. Downwind of the site compared to upwind of the site, for both north and south facing quadrants in the two directional dust deposition gauges being considered here.

Figure 10 - Woodvale - Directional Dust Results – Arsenic (mg)/dust(kg) – Q2 2022 (North/South)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 10- No recorded high figures were seen in any of the two directional gauges or six depositional dust gauges.

Figure 11 below displays the net gain or loss both upwind and downwind from east and west and no clear pattern is identified during Q2 2023 with both gains and losses occurring throughout the period.

Figure 11 - Woodvale - Directional Dust Deposition Results - Total Solids g/m2/month – Q2 2023 (East/West)

Figure 12 presents the net gains and losses in total arsenic downwind of the site compared to upwind of the site, for both east and west facing quadrants.

For the purpose of providing an understanding of total arsenic concentrations measured in dust relative to arsenic concentrations measured in soils in the assessment region, total arsenic is presented as mg/kg (based on the sum of soluble and insoluble arsenic µg/m2.month divided by total solids g/ m2.month).

Figure 12 - Woodvale - Directional Dust Results - Arsenic (mg/dust kg) – Q2 2023 (East/West)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## Review of Quality Assurance and Quality Control

Field sampling was undertaken in accordance with the following Standards:

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Deposited Matter – Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter – Impinged Matter – Gravimetric Method. (AS/NZS 3580.10.2)

Sampling methodology and locations are consistent with sampling previously undertaken to maintain continuity and comparability of results.

ALS Environmental Laboratory has NATA accreditation for the following Ambient Air Parameters:

ALS currently does not have NATA accreditation for metals analysis of ambient air but is in the process of gaining accreditation. Nevertheless, ALS is operating as though this accreditation is in place and expects to finalise the accreditation process in 2022.

Internal laboratory quality control measures note the following:

- No Method Blank value outliers occurred.

- No laboratory control outliers occurred.

- No Matrix Spike (MS) or Matrix Spike Duplicate (MSD) results reported.

- No laboratory duplicate results reported.

- No holding time outliers occurred.

- No quality control sample frequency outliers occurred (based on NEPM 2013 B3 quality control specification).

# Weather Station Results Woodvale Q2

During February 2023, ALS installed a new weather station at the Woodvale site. The purpose for the weather station installation at Woodvale is to provide data for Rainfall, Wind Direction, Wind Speed, Air Temperature, Relative Humidity and Solar radiation.

The new Weather Station at the Woodvale site is known as Site Number - 407810A Woodvale weather station. Data for these parameters are reported below and includes April, May and June 2023 data. RF totals for April 2023 recorded 56.4mm, May 2023 23.0mm and June 2023 up to 29/06/2023 88.6mm. The maximum wind velocity measured in meters/second was recorded on the 17/06/2023 with a recorded value of 8.83 M/s. An exceedance of dust over 4 g/m2/month criteria also occurred during June at the background site and during April.

## Stacked data plot April 2023

Figure 13 - Stacked data plot April 2023, 407810A Weather Station

## Wind Rose plot April 2023

Figure 14 - Woodvale Wind Rose plot April 2023

## Stacked data plot May 2023

Figure 15 - Stacked data plot May 2023, 407810A Weather Station

## Stacked data plot May 2023

Figure 16 - Woodvale Wind Rose plot May 2023

## Stacked data plot June 2023

Figure 17 - Stacked data plot June 2023, 407810A Weather Station

## Wind Rose plot June 2023

Figure 18 - Woodvale Wind Rose plot June 2023

# Summary of Results

Results are discussed in detail throughout the relevant sections. In summary, the overall results for Q2 2023 were comparable to previous results.

The key findings for Q2 2023 are as follows:

There were two instances where the dust level exceeded the 4 g/m2/month criteria used in this report.  One of these instances occurred at the Background site (WVDD07 (BG)) and is not attributed to the Woodvale Evaporation Ponds.

- WVDD01(S), during April 2023 (17.4 g/m2/month)

- WVDD07(BG), during May 2023 (10.1 g/m2/month)

When the data from all source monitoring sites excluding WVDD07 (BG) were averaged for Q2 2023, an average of 1.8 g/m2/month was recorded. This value is below the 4 g/m2/month criteria, and the inferred background plus 2 g/m2/month criteria in the Mining PEM.

Total arsenic concentrations for Q2 ranged from 0 to 30.3 mg/kg with a mean of 7.5 mg/kg for Q2 2023 not including the background site.

Highest concentrations were recorded in May 2023, 30.3 mg/kg at WVDD02 (N). This value is still well below the 100 mg/kg assessment level mentioned above. The high TIM reading recorded at WVDD01was also reflected in higher readings in total arsenic.

- The control site results from Q2 2023 confirm the previous observation that the arsenic concentration levels recorded in the dust samples from the site are generally slightly higher than the background arsenic levels in the area but is still much lower than the Contaminated Sites Guideline criteria.

- Results for Barium in soluble fraction were slightly higher in June for WVDD01 and WVDD04 and for WVDD01 in April in comparison to the background site. Manganese recorded higher results for soluble fraction at WVDD01 during April 2023 which was also the case in March 2023 at WVDD01.

- The results from the directional dust monitoring gauges showed some small net gains to the south facing gauges during the Q2 period, although this is not represented as an increase in total solids/arsenic concentrations.

# Further Considerations

DEECA(ERR) have advised that they will continue to monitor the site with regular visits and ensure that a range of dust control measures are applied during the current care and maintenance phase of this rehabilitation project. DEECA(ERR) is also reviewing the effectiveness of dust control measures and monitoring on an ongoing basis and adopting a continuous improvement approach to dust minimisation and monitoring.

# References

- ALS Hydrographics Pty Ltd (2021), Woodvale Dust Monitoring Q3 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2021), Woodvale Dust Monitoring Q4 2021 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q1 2022 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q2 2022 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q3 2022 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q4 2022 Report. (ALS Report)

- ALS Hydrographics Pty Ltd (2022), Woodvale Dust Monitoring Q1 2023 Report. (ALS Report)

- Australian Standard AS/NZS 3580.10.1 (2016), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Deposited Matter - Gravimetric Method. (AS/NZS 3580.10.1)

- Australian Standard AS/NZS 3580.10.2 (2013), Methods for Sampling and Analysis of Ambient Air - Determination of Particulate Matter - Impinged Matter - Gravimetric Method. (AS/NZS 3580.10.2)

- CDM Smith (2021), Woodvale Evaporation Ponds - Dust Monitoring Report - January to June 2021. (CDM Smith Report)

- Department of Conservation (2010), Contaminated Sites Management Series, Assessment Levels for Soil, Sediment and Water, February 2010. (Contaminated Sites Assessment Levels)

- EPA Vic Publication 1191 (2007), Protocol for Environmental Management - Mining and Extractive Industries. (Mining PEM)

- EPA NSW Publication (2017), Approved Methods for the Modelling and Assessment of Air Pollutants in NSW, January 2017.

- EPA Vic Publication 1961 (2022), Guideline for Assessing and Minimising Air Pollution in Victoria, February 2022. (Assessing and Minimising Guideline)

- EPA Vic Publication 440.1 (2002), A Guide to the Sampling and Analysis of Air Emissions and Air Quality, December 2002. (Sampling and Analysis Guideline)

- Kralcopic Pty Ltd (2018), Quarterly Environmental Monitoring Report - Woodvale Dust Monitoring, October to December 2018. (Kralcopic Report)

- National Environment Protection Council (1998), National Environment Protection (Ambient Air Quality) Measure, July 2018. (NEPM AAQ)

- Peter J Ramsay and Associates (2018), Woodvale Evaporation Ponds Complex, Environmental Audit of the Risk of any Possible Harm or Detriment to the Environment Pursuant to Part IXD (Section 53V) of the Environment Protection Act 1970, 29 June 2018. (Environmental Audit Report)

- Victorian Government (2001), Statement of Environment Protection Policy (Air Quality Management), Victorian Government Gazette, No S 240, 21 December 2001. (SEPP AQM)

- Victorian Government (2016), State Environment Protection Policy (Ambient Air Quality), Original Policy including Two Variations, Victorian Gazette Nos S19, S240 and G30, published on 9 February 1999, 21 December 2001 and 28 July 2016, respectively. (SEPP AAQ)

- Victorian Government (2021), Environment Reference Standard, under the Environment Protection Act 2017, Victorian Government Gazette, No. S 245, 26 May 2021. (ERS)

# Tables

Table 1 - Woodvale Dust Deposition Analysis Results Q2, 2023

|  |  |  |  |  | Arsenic | Barium | Manganese | Total Solids | Total Solids (mg) | Total Insoluble Matter | Total Insoluble Matter (mg) | Total Soluble Matter | Total Soluble Matter (mg) | Combustible Matter | Combustible Matter (mg) | Ash Content | Ash Content (mg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ug/m2 month | ug/m2 month | ug/m2 month | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg |
| EQL |  |  |  |  |  |  |  |  | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 |
| Description | Location | Quarter | Month | Lab Report Number |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WVDD01 (Insoluble) | WVDD01 (Insoluble) | Q2 2023 | Apr-23 | EM2307753-056 | 83.50 | 91.2 | 2130 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-057 | 8.40 | 3010 | 88.3 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-057 | 0.94 | 73.4 | 32.4 |  |  |  |  |  |  |  |  |  |  |
| WVDD01 (Soluble) | WVDD01 (Soluble) | Q2 2023 | Apr-23 | EM2307753-029 | 120.10 | 1030 | 2140 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-029 | 42.10 | 220 | 855 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-029 | 2.60 | 959 | 169 |  |  |  |  |  |  |  |  |  |  |
| WVDD01 (Ash) | WVDD01 (Ash) | Q2 2023 | Apr-23 | EM2307753-001 | 83.50 | 3010 | 2130 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-001 | 6.02 | 91.2 | 88.3 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-001 | 0.94 | 35.7 | 23.6 |  |  |  |  |  |  |  |  |  |  |
| WVDD01 | WVDD01 | Q2 2023 | Apr-23 | EM2307753-001 |  |  |  | 23.5 | 401 | 17.4 | 297 | 6.1 | 104 | 11.3 | 193 | 6.1 | 104 |
|  |  |  | May-23 | EM2310060-001 |  |  |  | 7.4 | 144 | 1.3 | 26 | 6.1 | 118 | 1 | 21 | 0.3 | 5 |
|  |  |  | Jun-23 | EM2311983-001 |  |  |  | 1.9 | 32 | 0.1 | 2 | 1.8 | 31 | 0.1 | 2 | 0.1 | 2 |
| WVDD02 (Insoluble) | WVDD02 (Insoluble) | Q2 2023 | Apr-23 | EM2307753-057 | 1.92 | 21.3 | 56.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-058 | 5.58 | 34.5 | 27.3 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-058 | 1.36 | 82.1 | 29.2 |  |  |  |  |  |  |  |  |  |  |
| WVDD02 (Soluble) | WVDD02 (Soluble) | Q2 2023 | Apr-23 | EM2307753-030 | 8.34 | 198 | 573 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-030 | 9.59 | 55.9 | 140 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-030 | 1.95 | 119.0 | 130 |  |  |  |  |  |  |  |  |  |  |
| WVDD02 (Ash) | WVDD02 (Ash) | Q2 2023 | Apr-23 | EM2307753-002 | 1.92 | 34.5 | 39.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-002 | 1.57 | 21.3 | 18.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-002 | 1.24 | 35.6 | 18.7 |  |  |  |  |  |  |  |  |  |  |
| WVDD02 | WVDD02 | Q2 2023 | Apr-23 | EM2307753-002 |  |  |  | 5.1 | 87 | 1.5 | 26 | 3.6 | 61 | 1.4 | 24 | 0.1 | 2 |
|  |  |  | May-23 | EM2310060-002 |  |  |  | 0.5 | 9 | 0.4 | 8 | 0.1 | 2 | 0.3 | 6 | 0.1 | 2 |
|  |  |  | Jun-23 | EM2311983-002 |  |  |  | 0.9 | 16 | 0.2 | 4 | 0.7 | 12 | 0.1 | 2 | 0.1 | 2 |
| WVDD03 (Insoluble) | WVDD03 (Insoluble) | Q2 2023 | Apr-23 | EM2307753-058 | 6.39 | 26.4 | 219 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-059 | 4.38 | 217 | 27.9 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-059 | 8.02 | 98.5 | 119 |  |  |  |  |  |  |  |  |  |  |
| WVDD03 (Soluble) | WVDD03 (Soluble) | Q2 2023 | Apr-23 | EM2307753-031 | 25.90 | 281 | 1560 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-031 | 16.80 | 98.3 | 304 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-031 | 36.00 | 245 | 932 |  |  |  |  |  |  |  |  |  |  |
| WVDD03 (Ash) | WVDD03 (Ash) | Q2 2023 | Apr-23 | EM2307753-003 | 6.39 | 217 | 219 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-003 | 0.93 | 26.4 | 24 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-003 | 4.29 | 98.5 | 67.7 |  |  |  |  |  |  |  |  |  |  |
| WVDD03 | WVDD03 | Q2 2023 | Apr-23 | EM2307753-003 |  |  |  | 5.5 | 94 | 2.2 | 37 | 3.3 | 57 | 1.8 | 30 | 0.4 | 7 |
|  |  |  | May-23 | EM2310060-003 |  |  |  | 1.3 | 25 | 0.3 | 6 | 1 | 19 | 0.2 | 5 | 0.1 | 2 |
|  |  |  | Jun-23 | EM2311983-003 |  |  |  | 6.5 | 111 | 0.8 | 14 | 5.7 | 97 | 0.5 | 9 | 0.3 | 4 |
| WVDD04 (Insoluble) | WVDD04 (Insoluble) | Q2 2023 | Apr-23 | EM2307753-059 | 3.02 | 18.9 | 49.4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-060 | 4.35 | 39.9 | 24.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-060 | 8.56 | 303 | 220 |  |  |  |  |  |  |  |  |  |  |
| WVDD04 (Soluble) | WVDD04 (Soluble) | Q2 2023 | Apr-23 | EM2307753-032 | 5.53 | 78.4 | 232 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-032 | 8.86 | 40.5 | 106 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-032 | 61.70 | 1510 | 2350 |  |  |  |  |  |  |  |  |  |  |
| WVDD04 (Ash) | WVDD04 (Ash) | Q2 2023 | Apr-23 | EM2307753-004 | 2.07 | 39.9 | 44.3 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-004 | 0.58 | 18.9 | 18.6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-004 | 6.65 | 303 | 188 |  |  |  |  |  |  |  |  |  |  |
| WVDD04 | WVDD04 | Q2 2023 | Apr-23 | EM2307753-004 |  |  |  | 2.3 | 38 | 0.9 | 15 | 1.4 | 23 | 0.8 | 14 | 0.1 | 2 |
|  |  |  | May-23 | EM2310060-004 |  |  |  | 1.0 | 19 | 0.3 | 6 | 0.7 | 13 | 0.3 | 5 | 0.1 | 2 |
|  |  |  | Jun-23 | EM2311983-004 |  |  |  | 9.6 | 164 | 2.3 | 40 | 7.3 | 124 | 1.7 | 30 | 0.6 | 10 |
| WVDD05 (Insoluble) | WVDD05 (Insoluble) | Q2 2023 | Apr-23 | EM2307753-060 | 3.59 | 23.1 | 57.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-061 | 4.38 | 53.4 | 32 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-061 | 0.96 | 21.2 | 25 |  |  |  |  |  |  |  |  |  |  |
| WVDD05 (Soluble) | WVDD05 (Soluble) | Q2 2023 | Apr-23 | EM2307753-033 | 3.20 | 140 | 293 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-033 | 5.14 | 49.8 | 128 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-033 | 5.10 | 55.4 | 126 |  |  |  |  |  |  |  |  |  |  |
| WVDD05 (Ash) | WVDD05 (Ash) | Q2 2023 | Apr-23 | EM2307753-005 | 1.96 | 53.4 | 36 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-005 | 0.70 | 23.1 | 17.9 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-005 | 0.58 | 21.2 | 10.9 |  |  |  |  |  |  |  |  |  |  |
| WVDD05 | WVDD05 | Q2 2023 | Apr-23 | EM2307753-005 |  |  |  | 2.5 | 43 | 1.5 | 25 | 1 | 18 | 1.4 | 24 | 0.1 | 2 |
|  |  |  | May-23 | EM2310060-005 |  |  |  | 1.3 | 26 | 0.3 | 6 | 1 | 20 | 0.1 | 3 | 0.2 | 3 |
|  |  |  | Jun-23 | EM2311983-005 |  |  |  | 1.1 | 18 | 0.1 | 2 | 1 | 17.0 | 0.1 | 2 | 0.1 | 2 |
| WVDD07BG (Insoluble) | WVDD07 BG (Insoluble) | Q2 2023 | Apr-23 | EM2307753-061 | 2.44 | 260 | 67.2 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-062 | 14.20 | 2.44 | 803 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-062 | 1.48 | 32.7 | 38.1 |  |  |  |  |  |  |  |  |  |  |
| WVDD07BG (Soluble) | WVDD07 BG (Soluble) | Q2 2023 | Apr-23 | EM2307753-034 | 3.89 | 86.8 | 560 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-034 | 29.60 | 326 | 2020 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-034 | 3.06 | 75.8 | 230 |  |  |  |  |  |  |  |  |  |  |
| WVDD07BG (Ash) | WVDD07 BG (Ash) | Q2 2023 | Apr-23 | EM2307753-006 | 2.44 | 74.6 | 65.9 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-006 | 8.81 | 260 | 760 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-006 | 1.21 | 32.5 | 34.7 |  |  |  |  |  |  |  |  |  |  |
| WVDD07BG | WVDD07BG | Q2 2023 | Apr-23 | EM2307753-006 |  |  |  | 3.2 | 55 | 1.6 | 27 | 1.6 | 28 | 1.4 | 24 | 0.2 | 3 |
|  |  |  | May-23 | EM2310060-006 |  |  |  | 19.6 | 380 | 10.1 | 196 | 9.5 | 184 | 8.8 | 172 | 1.3 | 24 |
|  |  |  | Jun-23 | EM2311983-006 |  |  |  | 0.2 | 4 | 0.2 | 4 | 0.1 | 2 | 0.1 | 2 | 0.2 | 4 |
| WVDD08 (Insoluble) | WVDD08 (Insoluble) | Q2 2023 | Apr-23 | EM2307753-062 | 2.23 | 38.9 | 60.8 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-063 | 5.92 | 52.3 | 58.2 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-063 | 1.92 | 30.2 | 34.5 |  |  |  |  |  |  |  |  |  |  |
| WVDD08 (Soluble) | WVDD08 (Soluble) | Q2 2023 | Apr-23 | EM2307753-035 | 2.52 | 110 | 236 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-035 | 5.78 | 326 | 166 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-035 | 4.94 | 92.4 | 167 |  |  |  |  |  |  |  |  |  |  |
| WVDD08 (Ash) | WVDD08 (Ash) | Q2 2023 | Apr-23 | EM2307753-007 | 2.23 | 52.3 | 42.4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | May-23 | EM2310060-007 | 2.58 | 38.9 | 32.4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Jun-23 | EM2311983-007 | 1.17 | 24.2 | 16.8 |  |  |  |  |  |  |  |  |  |  |
| WVDD08 | WVDD08 | Q2 2023 | Apr-23 | EM2307753-007 |  |  |  | 2.7 | 46 | 1.1 | 19 | 1.6 | 27 | 1 | 17 | 0.1 | 2 |
|  |  |  | May-23 | EM2310060-007 |  |  |  | 19.6 | 380 | 10.1 | 196 | 9.5 | 184 | 8.8 | 172 | 1.3 | 24 |
|  |  |  | Jun-23 | EM2311983-007 |  |  |  | 1.4 | 23 | 0.4 | 6 | 1 | 17 | 0.3 | 4 | 0.1 | 2 |

Table 2 - Woodvale Directional Dust Deposition Analysis Results Q2, 2023

|  |  |  | Arsenic | Barium | Manganese | Total Solids | Total Solids (mg) | Total Insoluble Matter | Total Insoluble Matter (mg) | Total Soluble Matter | Total Soluble Matter (mg) | Combustible Matter | Combustible Matter (mg) | Ash Content | Ash Content (mg) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ug/m2 month | ug/m2 month | ug/m2 month | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg | g/m2 month | mg |
| EQL |  |  |  |  |  |  | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 | 0.1 | 1 |
| Description | Month | Lab Report Number |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WVDG03E (Insoluble) | Apr-23 | EM2307753-076 | 1.04 | 13 | 27.6 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-070 | 7.08 | 14.2 | 13.6 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-078 | 16.50 | 17.5 | 18.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG03E (Soluble) | Apr-23 | EM2307753-049 | 3.88 | 8.2 | 15.1 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-042 | 10.4 | 13.2 | 20.4 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-050 | 0.135 | 0.936 | 1.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG03E (Ash) | Apr-23 | EM2307753-022 | 0.85 | 13 | 27.6 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-014 | 4.15 | 14.2 | 12.8 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-022 | 0.50 | 9.41 | 8.43 |  |  |  |  |  |  |  |  |  |  |
| WVDG03E | Apr-23 | EM2307753-022 |  |  |  | 1.4 | 20 | 1.4 | 20 | 0.1 | 2 | 1.3 | 19 | 0.1 | 2 |
|  | May-23 | EM2310060-014 |  |  |  | 0.5 | 8 | 0.3 | 5 | 0.2 | 3 | 0.1 | 2 | 0.1 | 2 |
|  | Jun-23 | EM2311983-022 |  |  |  | 0.8 | 11 | 0.6 | 9 | 0.2 | 2 | 0.6 | 9 | 0.1 | 2 |
| WVDG03N (Insoluble) | Apr-23 | EM2307753-021 | 0.75 | 19.5 | 24.6 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-069 | 4.5 | 35.1 | 16.5 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-077 | 0.36 | 12.7 | 11.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG03N (Soluble) | Apr-23 | EM2307753-048 | 0.8 | 18.4 | 35.7 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-041 | 0.977 | 4.98 | 4.42 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-049 | 0.318 | 6.26 | 12.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG03N (Ash) | Apr-23 | EM2307753-021 | 0.75 | 19.5 | 24.6 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-013 | 4.5 | 35.1 | 16.3 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-021 | 0.22 | 12.2 | 7.9 |  |  |  |  |  |  |  |  |  |  |
| WVDG03N | Apr-23 | EM2307753-021 |  |  |  | 1.8 | 26 | 1.6 | 23 | 0.2 | 3 | 1.5 | 22 | 0.1 | 2 |
|  | May-23 | EM2310060-013 |  |  |  | 0.6 | 11 | 0.4 | 7 | 0.2 | 4 | 0.3 | 5 | 0.1 | 2 |
|  | Jun-23 | EM2311983-021 |  |  |  | 0.4 | 7 | 0.3 | 5 | 0.1 | 2 | 0.3 | 4 | 0.1 | 2 |
| WVDG03S (Insoluble) | Apr-23 | EM2307753-077 | 0.49 | 11.6 | 12.90 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-071 | 8.96 | 13.9 | 16.3 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-079 | 0.37 | 10.8 | 9.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG03S (Soluble) | Apr-23 | EM2307753-050 | 0.31 | 9.8 | 21.2 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-043 | 8.12 | 6.69 | 16.4 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-051 | 1.06 | 5.8 | 16.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG03S (Ash) | Apr-23 | EM2307753-023 | 0.49 | 11.6 | 12.10 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-015 | 8.96 | 13.9 | 16.3 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-023 | 0.193 | 6.47 | 7.69 |  |  |  |  |  |  |  |  |  |  |
| WVDG03S | Apr-23 | EM2307753-023 |  |  |  | 1.5 | 21 | 1.4 | 20 | 0.1 | 2 | 1.3 | 19 | 0.1 | 2 |
|  | May-23 | EM2310060-015 |  |  |  | 0.5 | 8 | 0.4 | 6 | 0.1 | 2 | 0.4 | 6 | 0.1 | 2 |
|  | Jun-23 | EM2311983-023 |  |  |  | 0.6 | 8 | 0.5 | 7 | 0.1 | 2 | 0.5 | 7 | 0.1 | 2 |
| WVDG03W (Insoluble) | Apr-23 | EM2307753-078 | 1.07 | 13.1 | 15.7 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-072 | 2.92 | 8.04 | 8.42 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-080 | 0.284 | 9.75 | 7.73 |  |  |  |  |  |  |  |  |  |  |
| WVDG03W (Soluble) | Apr-23 | EM2307753-051 | 1.01 | 9.2 | 23.2 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-044 | 8.46 | 4.35 | 10.8 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-052 | 0.078 | 5.89 | 33.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG03W (Ash) | Apr-23 | EM2307753-025 | 0.56 | 13.10 | 11.60 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-016 | 0.968 | 7.79 | 5.46 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-024 | 0.164 | 6.04 | 4.59 |  |  |  |  |  |  |  |  |  |  |
| WVDG03W | Apr-23 | EM2307753-025 |  |  |  | 1 | 15 | 0.9 | 14 | 0.1 | 2 | 0.8 | 13 | 0.1 | 2 |
|  | May-23 | EM2310060-016 |  |  |  | 0.3 | 6 | 0.3 | 5 | 0.1 | 2 | 0.3 | 4 | 0.1 | 2 |
|  | Jun-23 | EM2311983-024 |  |  |  | 0.7 | 10 | 0.6 | 9 | 0.1 | 2 | 0.6 | 9 | 0.1 | 2 |
| WVDG05E (Insoluble) | Apr-23 | EM2307753-068 | 0.71 | 13.3 | 22.1 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-078 | 1.34 | 10.6 | 5.71 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-070 | 0.393 | 15 | 11.0 |  |  |  |  |  |  |  |  |  |  |
| WVDG05E (Soluble) | Apr-23 | EM2307753-041 | 0.7 | 12.7 | 63.2 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-050 | 7.22 | 5.47 | 9.02 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-042 | 0.44 | 16.7 | 8.12 |  |  |  |  |  |  |  |  |  |  |
| WVDG05E (Ash) | Apr-23 | EM2307753-014 | 0.7 | 12.7 | 63.2 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-022 | 1.09 | 10.6 | 5.32 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-014 | 0.192 | 7.23 | 4.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG05E | Apr-23 | EM2307753-014 |  |  |  | 1.3 | 20 | 1.2 | 18 | 0.1 | 2 | 1.1 | 17 | 0..1 | 2 |
|  | May-23 | EM2310060-022 |  |  |  | 0.4 | 7 | 0.3 | 5 | 0.1 | 2 | 0.2 | 3 | 0.1 | 2 |
|  | Jun-23 | EM2311983-014 |  |  |  | 0.6 | 10 | 0.5 | 8 | 0.1 | 2 | 0.4 | 7 | 0.1 | 2 |
| WVDG05N (Insoluble) | Apr-23 | EM2307753-067 | 0.75 | 32.4 | 10.7 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-076 | 1.71 | 13.1 | 12.8 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-069 | 0.277 | 19.6 | 12.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG05N (Soluble) | Apr-23 | EM2307753-040 | 0.45 | 31 | 12.41 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-049 | 7.24 | 3.5 | 16.1 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-041 | 0.20 | 7.21 | 1.36 |  |  |  |  |  |  |  |  |  |  |
| WVDG05N (Ash) | Apr-23 | EM2307753-040 | 0.45 | 31 | 12.4 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-021 | 1.06 | 13.1 | 12.8 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-013 | 0.052 | 3.2 | 2.86 |  |  |  |  |  |  |  |  |  |  |
| WVDG05N | Apr-23 | EM2307753-013 |  |  |  | 1.6 | 23 | 1.5 | 22 | 0.1 | 2 | 1.5 | 22 | 0.1 | 2 |
|  | May-23 | EM2310060-021 |  |  |  | 0.70 | 12 | 0.5 | 8 | 0.2 | 4 | 0.4 | 6 | 0.1 | 2 |
|  | Jun-23 | EM2311983-013 |  |  |  | 0.6 | 9 | 0.6 | 9 | 0.1 | 2 | 0.5 | 7 | 0.1 | 2 |
| WVDG05S (Insoluble) | Apr-23 | EM2307753-069 | 0.44 | 13 | 10.5 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-079 | 1.26 | 12.9 | 10.2 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-071 | 0.20 | 14.2 | 8.42 |  |  |  |  |  |  |  |  |  |  |
| WVDG05S (Soluble) | Apr-23 | EM2307753-042 | 0.09 | 6.6 | 12.3 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-051 | 7.91 | 3.76 | 11.8 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-043 | 0.25 | 10.2 | 5.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG05S (Ash) | Apr-23 | EM2307753-042 | 0.09 | 6.6 | 12.3 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-023 | 0.541 | 8.5 | 38.3 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-015 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WVDG05S | Apr-23 | EM2307753-015 |  |  |  | 1.2 | 18 | 1.1 | 17 | 0.1 | 2 | 1.1 | 17 | 0..1 | 2 |
|  | May-23 | EM2310060-023 |  |  |  | 0.4 | 7 | 0.3 | 5 | 0.1 | 2 | 0.3 | 4 | 0.1 | 2 |
|  | Jun-23 | EM2311983-015 |  |  |  | 0.6 | 9 | 0.6 | 9 | 0.1 | 2 | 0.5 | 7 | 0.1 | 2 |
| WVDG05W (Insoluble) | Apr-23 | EM2307753-070 | 0.6 | 16.5 | 12.5 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-080 | 1.26 | 12.9 | 10.2 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-072 | 0.195 | 8.75 | 7.92 |  |  |  |  |  |  |  |  |  |  |
| WVDG05W (Soluble) | Apr-23 | EM2307753-043 | 0.32 | 12.6 | 17.2 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-052 | 8.85 | 3.66 | 11.1 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-044 | 0.05 | 0.2 | 0.24 |  |  |  |  |  |  |  |  |  |  |
| WVDG05W (Ash) | Apr-23 | EM2307753-043 | 0.32 | 12.6 | 17.2 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-024 | 0.831 | 12.9 | 10.2 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-016 | 0.10 | 4 | 2.76 |  |  |  |  |  |  |  |  |  |  |
| WVDG05W | Apr-23 | EM2307753-016 |  |  |  | 1.1 | 17 | 1.1 | 16 | 0.1 | 2 | 1.1 | 16 | 0.1 | 2 |
|  | May-23 | EM2310060-024 |  |  |  | 0.5 | 8 | 0.4 | 7 | 0.1 | 2 | 0.4 | 7 | 0.1 | 2 |
|  | Jun-23 | EM2311983-016 |  |  |  | 0.5 | 7 | 0.5 | 7 | 0.1 | 2 | 0.5 | 7 | 0.1 | 2 |
| WVDG04E (Insoluble) | Apr-23 | EM2307753-080 | 4.79 | 109 | 50.4 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-073 | 2.75 | 16.3 | 15.7 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-082 | 0.37 | 9.19 | 9.27 |  |  |  |  |  |  |  |  |  |  |
| WVDG04E (Soluble) | Apr-23 | EM2307753-026 | 4.79 | 109 | 50.4 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-046 | 17.2 | 23.1 | 114 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-054 | 0.57 | 3.0 | 5.7 |  |  |  |  |  |  |  |  |  |  |
| WVDG04E (Ash) | Apr-23 | EM2307753-053 | 2.68 | 23 | 62.2 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-018 | 1.81 | 13.8 | 18.9 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-026 | 0.22 | 7.1 | 7.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG04E | Apr-23 | EM2307753-026 |  |  |  | 2.2 | 34 | 1.6 | 24 | 0.6 | 10 | 1.2 | 19 | 0.4 | 5 |
|  | May-23 | EM2310060-018 |  |  |  | 1.3 | 22 | 0.5 | 8 | 0.8 | 14 | 0.4 | 7 | 0.1 | 2 |
|  | Jun-23 | EM2311983-026 |  |  |  | 0.6 | 10 | 0.6 | 9 | 0.1 | 2 | 0.6 | 9 | 0.1 | 2 |
| WVDG04N (Insoluble) | Apr-23 | EM2307753-079 | 1.97 | 34 | 68.2 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-073 | 2.75 | 16.3 | 15.7 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-081 | 0.94 | 12.7 | 17.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG04N (Soluble) | Apr-23 | EM2307753-052 | 0.94 | 10.8 | 29.7 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-045 | 7.73 | 10.3 | 46 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-053 | 0.49 | 6.9 | 16.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG04N (Ash) | Apr-23 | EM2307753-052 | 0.94 | 10.8 | 29.7 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-017 | 1.81 | 16.3 | 13.7 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-025 | 0.439 | 12.7 | 11.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG04N | Apr-23 | EM2307753-025 |  |  |  | 1.2 | 17 | 1.1 | 16 | 0.1 | 2 | 1 | 15 | 0.1 | 2 |
|  | May-23 | EM2310060-017 |  |  |  | 0.6 | 10 | 0.3 | 5 | 0.3 | 5 | 0.1 | 2 | 0.2 | 3 |
|  | Jun-23 | EM2311983-025 |  |  |  | 0.6 | 8 | 0.5 | 7 | 0.1 | 2 | 0.5 | 7 | 0.1 | 2 |
| WVDG04S (Insoluble) | Apr-23 | EM2307753-081 | 1.08 | 11.7 | 12.8 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-075 | 1.89 | 7.66 | 10.3 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-083 | 1.07 | 34.6 | 29.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG04S (Soluble) | Apr-23 | EM2307753-054 | 0.72 | 6.7 | 21.6 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-047 | 1.82 | 2.11 | 7.69 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-055 | 1.0 | 10.4 | 7.81 |  |  |  |  |  |  |  |  |  |  |
| WVDG04S (Ash) | Apr-23 | EM2307753-053 | 0.72 | 6.7 | 21.6 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-019 | 0.604 | 7.7 | 7.4 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-027 | 1.07 | 34.6 | 22.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG04S | Apr-23 | EM2307753-027 |  |  |  | 0.9 | 13 | 0.8 | 12 | 0.1 | 2 | 0.8 | 12 | 0.1 | 2 |
|  | May-23 | EM2310060-019 |  |  |  | 1 | 16 | 0.5 | 8 | 0.5 | 8 | 0.4 | 7 | 0.1 | 2 |
|  | Jun-23 | EM2311983-027 |  |  |  | 0.6 | 9 | 0.5 | 8 | 0.1 | 2 | 0.5 | 8 | 0.1 | 2 |
| WVDG04W (Insoluble) | Apr-23 | EM2307753-082 | 4.33 | 97.1 | 56.2 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-076 | 4.66 | 19.8 | 25.9 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-084 | 0.65 | 18.9 | 11 |  |  |  |  |  |  |  |  |  |  |
| WVDG04W (Soluble) | Apr-23 | EM2307753-055 | 4.96 | 46.4 | 47.30 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-048 | 19.1 | 25.1 | 137 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-056 | 0.657 | 3.24 | 6.78 |  |  |  |  |  |  |  |  |  |  |
| WVDG04W (Ash) | Apr-23 | EM2307753-055 | 4.96 | 46.4 | 47.3 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-020 | 4.66 | 19.8 | 23.5 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-028 | 0.65 | 18.9 | 10.9 |  |  |  |  |  |  |  |  |  |  |
| WVDG04W | Apr-23 | EM2307753-028 |  |  |  | 1.8 | 26 | 1.3 | 19 | 0.5 | 7 | 1 | 15 | 0.3 | 4 |
|  | May-23 | EM2310060-020 |  |  |  | 1.2 | 20 | 0.5 | 9 | 0.7 | 11 | 0.4 | 8 | 0.1 | 2 |
|  | Jun-23 | EM2311983-028 |  |  |  | 0.5 | 9 | 0.5 | 8 | 0.1 | 2 | 0.5 | 8 | 0.1 | 2 |
| WVDG08N (Insoluble) | Apr-23 | EM2307753-071 | 1.21 | 19.9 | 19.8 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-081 | 1 | 10.5 | 25.7 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-073 | 0.364 | 20.8 | 25.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG08N (Soluble) | Apr-23 | EM2307753-044 | 0.57 | 11.5 | 22.5 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-053 | 7.88 | 2.27 | 9.92 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-045 | 1.05 | 14.8 | 117 |  |  |  |  |  |  |  |  |  |  |
| WVDG08N (Ash) | Apr-23 | EM2307753-017 | 0.57 | 19.3 | 12.5 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-025 | 0.441 | 10.5 | 25.7 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-017 | 0.338 | 13 | 22.2 |  |  |  |  |  |  |  |  |  |  |
| WVDG08N | Apr-23 | EM2307753-017 |  |  |  | 1.1 | 16 | 1.1 | 16 | 0.1 | 2 | 1.1 | 16 | 0.1 | 2 |
|  | May-23 | EM2310060-025 |  |  |  | 0.3 | 6 | 0.2 | 4 | 0.1 | 2 | 0.1 | 2 | 0.1 | 2 |
|  | Jun-23 | EM2311983-017 |  |  |  | 0.6 | 9 | 0.4 | 6 | 0.2 | 3 | 0.1 | 2 | 0.1 | 2 |
| WVDG08E (Insoluble) | Apr-23 | EM2307753-072 | 0.58 | 18.6 | 19.7 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-082 | 1.34 | 16 | 17.1 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-074 | 0.6 | 16.2 | 16.9 |  |  |  |  |  |  |  |  |  |  |
| WVDG08E (Soluble) | Apr-23 | EM2307753-045 | 0.36 | 8.2 | 36.6 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-054 | 5.86 | 5.51 | 28.8 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-046 | 0.953 | 7.21 | 18.9 |  |  |  |  |  |  |  |  |  |  |
| WVDG08E (Ash) | Apr-23 | EM2307753-018 | 0.58 | 18.6 | 17.1 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-026 | 0.922 | 16 | 12.4 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-018 | 0.6 | 16.2 | 14 |  |  |  |  |  |  |  |  |  |  |
| WVDG08E | Apr-23 | EM2307753-018 |  |  |  | 1.5 | 22 | 1.1 | 17 | 0.4 | 5 | 1 | 16 | 0.1 | 2 |
|  | May-23 | EM2310060-026 |  |  |  | 0.5 | 8 | 0.4 | 6 | 0.1 | 2 | 0.3 | 5 | 0.1 | 2 |
|  | Jun-23 | EM2311983-018 |  |  |  | 0.6 | 9 | 0.5 | 7 | 0.1 | 2 | 0.4 | 6 | 0.1 | 2 |
| WVDG08S (Insoluble) | Apr-23 | EM2307753-073 | 2.21 | 21.6 | 25 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-083 | 2.99 | 20.8 | 29.1 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-075 | 0.515 | 15.2 | 17.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG08S (Soluble) | Apr-23 | EM2307753-046 | 1.36 | 19.3 | 42.7 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-055 | 2.14 | 2.12 | 9.12 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-047 | 0.433 | 2.51 | 8.91 |  |  |  |  |  |  |  |  |  |  |
| WVDG08S (Ash) | Apr-23 | EM2307753-019 | 2.14 | 21.6 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-027 | 2.99 | 20.8 | 22.7 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-019 | 0.445 | 15.2 | 9.4 |  |  |  |  |  |  |  |  |  |  |
| WVDG08S | Apr-23 | EM2307753-019 |  |  |  | 1.5 | 23 | 1.3 | 19 | 0.2 | 4 | 1.2 | 18 | 0.1 | 2 |
|  | May-23 | EM2310060-027 |  |  |  | 0.6 | 9 | 0.4 | 6 | 0.2 | 3 | 0.3 | 5 | 0.1 | 2 |
|  | Jun-23 | EM2311983-019 |  |  |  | 0.4 | 7 | 0.4 | 6 | 0.1 | 2 | 0.4 | 6 | 0.1 | 2 |
| WVDG08W (Insoluble) | Apr-23 | EM2307753-074 | 1.8 | 22.3 | 22.6 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-084 | 4.65 | 25.9 | 41.4 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-076 | 1.04 | 15.1 | 25.5 |  |  |  |  |  |  |  |  |  |  |
| WVDG08W (Soluble) | Apr-23 | EM2307753-047 | 0.6 | 2.9 | 11 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-056 | 6.32 | 9.36 | 50.3 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-049 | 0.318 | 6.26 | 12.6 |  |  |  |  |  |  |  |  |  |  |
| WVDG08W(Ash) | Apr-23 | EM2307753-020 | 1.3 | 22.3 | 22.6 |  |  |  |  |  |  |  |  |  |  |
|  | May-23 | EM2310060-028 | 3.39 | 25.9 | 40.2 |  |  |  |  |  |  |  |  |  |  |
|  | Jun-23 | EM2311983-020 | 0.908 | 15.1 | 24.1 |  |  |  |  |  |  |  |  |  |  |
| WVDG08W | Apr-23 | EM2307753-020 |  |  |  | 1.4 | 20 | 1.3 | 19 | 0.1 | 2 | 1.2 | 18 | 0.1 | 2 |
|  | May-23 | EM2310060-028 |  |  |  | 0.5 | 9 | 0.1 | 2 | 0.4 | 7 | 0.1 | 2 | 0.1 | 2 |
|  | Jun-23 | EM2311983-020 |  |  |  | 1 | 15 | 0.5 | 8 | 0.5 | 7 | 0.5 | 8 | 0.1 | 2 |

# Supplementary Data

Graphs for Metals/Metalloids in Insoluble and Soluble Fractions including trends are presented below.

## Arsenic

Figure 19 - Woodvale - Arsenic in Soluble Fraction – Q2 2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph

Figure 20 - Woodvale - Arsenic in Insoluble Fraction – Q2 2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 21 - Woodvale - Arsenic in Soluble Fraction Year Review (2022-23)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 22 - Woodvale - Arsenic in Insoluble Fraction Year Review (2022-23)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Arsenic lab results unable to be calculated left as blank

## Barium

Figure 23 - Woodvale - Barium in Soluble Fraction – Q2 2023

Figure 24 - Woodvale - Barium in Insoluble Fraction – Q2 2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 25 - Woodvale - Barium in Soluble Fraction Year Review (2022-23)

Figure 26 - Woodvale - Barium in Insoluble Fraction Year Review (2022-23)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph. The larger bolder asterisks represent the entire months records as less than

## Manganese

Figure 27 - Woodvale - Manganese in Soluble Fraction – Q2 2023

Figure 28 - Woodvale - Manganese in Insoluble Fraction – Q2 2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 29 - Woodvale - Manganese in Soluble Fraction Year Review (2022-23)

Figure 30 - Woodvale - Manganese in Insoluble Fraction Year Review (2022-23)

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

## Trend Analysis Graphs

Figure 31 - Woodvale - Total Insoluble Matter (TIM) - 2022-2023

Figure 25 shows TIM for all months in 2022 and 2023. The review of Q2 2023 data shows slightly higher trends during June although these higher results were also seen at the background site. The overall average from all monitoring locations was higher being 1.8 g/m2/month for Q2 2023 (excluding the background control site), compared to 1.9 g/m2/month for quarter 1, 2023.

Figure 32 - Woodvale - Total Ash Content - 2022-2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 26 shows the Total Ash Content for all months in 2022 and Q1/Q2 2023. Q2 monthly figures were slightly higher than those presented in Q1 2023 when averaged over the quarter.

Figure 33 - Woodvale - Ash Content/TIM Ratio - 2022-2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 27 shows all results collected and sampled for 2022 and 2023.

Figure 34 - Woodvale - Total Arsenic/Total Solids - 2022-2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Could not be calculated as per other months due to lab samples results – August 2022

Figure 28 above presents the ratio of Total Arsenic/Total Solids. Arsenic results for August have been given an     over the sample as lab results were compromised as a result of being totally ashed. For more details refer to Q3 2022 report. .

Figure 35 - Woodvale - DDG Total Solids Comparison North/South - 2022-2023

Figure 29 above displays the results from 2022 and 2023, no definitive pattern is apparent throughout this period with small net gains and losses from both the upwind and downwind quadrants being observed.

Figure 36 - Woodvale - Arsenic/Total Solids Comparison - North/South 2022-2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

No results for Arsenic (insoluble) for August 2022

Figure 30 shows the net gains and losses in total arsenic concentration for the north and south quadrants from Jan 2022 – June 2023. No definitive pattern is apparent in this period with net gains and losses from both the upwind and downwind quadrants being observed.

Figure 37 - Woodvale - DDG Total Solids Comparison East/West - 2022-2023

Figure 31 above displays the net gain or loss both upwind and downwind from east and west for total solids, and no clear pattern is identified above during the 18 months with both gains and losses occurring throughout the period.

Figure 38 - Woodvale - Arsenic/Total Solids Comparison - East/West - 2022-2023

An asterisk (*) has been placed over the samples which recorded as less than (<) the value x displayed on the graph.

Figure 32 shows the net gains and losses in total arsenic concentration for the east/west quadrants from the previous months. No definitive pattern is apparent in this period with net gains and losses from both the upwind and downwind quadrants being observed.

# Appendix A – Supplementary Information

# Appendix B – Laboratory Reports

CERTIFICATE OF ANALYSIS

Work Order	: EM2307753

Page	: 1 of 19

Amendment

Client

Contact

Address

: 2

: ALS WATER AND HYDROGRAPHICS PTY LTD

: ROHAN OLIVER

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Customer Services EM

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +61 3 8549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 03-May-2023 10:20 |
| Order number | : ---- | Date Analysis Commenced | : 15-May-2023 |
| C-O-C number Sampler Site | : ---- : ROHAN OLIVER : ---- | Issue Date | : 25-Jul-2023 23:23 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 81 : 81 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Dilani Fernando	Laboratory Coordinator	Melbourne Inorganics, Springvale, VIC

right solutions. right partner.

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- Sample labelled "KF5BG" received broken on receipt

- Sampling Date: 30/03/23 - 28/04/23

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- The laboratory was unable to perform the prescribed analyses or report results on sample KF5BG due to broken bottle upon receipt.

- Amendment (25/07/2023): This report has been amended as a result of misinterpretation of sample identification numbers (IDs) for samples #7, 17-20, 35, 45-47, 62, 71-74. All analysis results are as per the previous report.

- Amendment (25/07/2023):This report has been amended following the request from Rohan Oliver to include an additional comment for broken sample, KF5BG.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- EA120i-EA142i: EM2307753 #13-19, 20-24, 27; Sample container was received with no liquid. 100ml of DI water was used to dissolve contents, perform analysis and for calculation of results.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 10:00 | 28-Apr-2023 10:10 | 28-Apr-2023 10:25 | 28-Apr-2023 10:45 | 28-Apr-2023 11:00 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2307753-001 | EM2307753-002 | EM2307753-003 | EM2307753-004 | EM2307753-005 |
|  |  |  | Result 6.1 | Result 0.1 | Result 0.4 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 104 11.3 | <2 1.4 | 7 1.8 | <2 0.8 | <2 1.4 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 193 6.1 | 24 3.6 | 30 3.3 | 14 1.4 | 24 1.0 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 104 17.4 | 61 1.5 | 57 2.2 | 23 0.9 | 18 1.5 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 297 23.5 | 26 5.1 | 37 5.5 | 15 2.3 | 25 2.5 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 401 <83.5 | 87 <1.92 | 94 <6.39 | 38 2.07 | 43 1.96 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <3010 | <34.5 | <217 | <39.9 | <53.4 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <2130 | 39.1 | <219 | 44.3 | 36.0 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD007BG | WVDD08 | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 09:45 | 28-Apr-2023 11:40 | 28-Apr-2023 13:15 | 28-Apr-2023 13:20 | 28-Apr-2023 12:50 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2307753-006 | EM2307753-007 | EM2307753-008 | EM2307753-009 | EM2307753-010 |
|  |  |  | Result 0.2 | Result 0.1 | Result 0.4 | Result 0.1 | Result 0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 3 1.4 | 2 1.0 | 6 1.1 | 2 0.8 | <2 0.8 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 24 1.6 | 17 1.6 | 20 2.1 | 14 0.9 | 13 2.2 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 28 1.6 | 27 1.1 | 36 1.5 | 16 0.9 | 38 0.9 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 27 3.2 | 19 2.7 | 26 3.6 | 16 1.8 | 15 3.1 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 55 <2.44 | 46 <2.23 | 62 <7.78 | 32 <18.9 | 53 <3.75 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <74.6 | <52.3 | <191 | <50.3 | 30.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 65.9 | 42.4 | <60.8 | 66.4 | 27.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | WVDG05N | WVDG05E | WVDG05S | WVDG05W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 12:35 | 28-Apr-2023 11:00 | 28-Apr-2023 11:00 | 28-Apr-2023 11:00 | 28-Apr-2023 11:00 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2307753-011 | EM2307753-013 | EM2307753-014 | EM2307753-015 | EM2307753-016 |
|  |  |  | Result 0.2 | Result ---- | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | 3 ---- | ---- <0.1 | ---- <0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 0.6 | <2 ---- | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 11 ---- | ---- 1.5 | ---- 1.1 | ---- 1.1 | ---- 1.1 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 1.2 | 22 ---- | 17 ---- | 17 ---- | 16 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 21 ---- | ---- 0.1 | ---- 0.1 | ---- 0.1 | ---- <0.1 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.8 | <2 ---- | 2 ---- | <2 ---- | <2 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 14 ---- | ---- 1.5 | ---- 1.2 | ---- 1.1 | ---- 1.1 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 2.0 | 22 ---- | 18 ---- | 17 ---- | 16 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 35 ---- | ---- 1.6 | ---- 1.3 | ---- 1.2 | ---- 1.1 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- <8.04 | 23 0.63 | 20 <0.71 | 18 <0.44 | 17 <0.60 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <53.4 | 24.9 | <13.3 | <13.0 | <16.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 45.3 | 10.4 | 19.6 | 10.2 | <12.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG08N | WVDG08E | WVDG08S | WVDG08W | WVDG03N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 11:40 | 28-Apr-2023 11:40 | 28-Apr-2023 11:40 | 28-Apr-2023 11:40 | 28-Apr-2023 10:25 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2307753-017 | EM2307753-018 | EM2307753-019 | EM2307753-020 | EM2307753-021 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.1 | <2 1.0 | <2 1.2 | <2 1.2 | <2 1.5 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 16 <0.1 | 16 0.4 | 18 0.2 | 18 0.1 | 22 0.2 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 1.1 | 5 1.1 | 4 1.3 | <2 1.3 | 3 1.6 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 16 1.1 | 17 1.5 | 19 1.5 | 19 1.4 | 23 1.8 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 16 0.57 | 22 <0.58 | 23 2.14 | 20 1.30 | 26 <0.75 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 19.3 | <18.6 | <21.6 | <22.3 | <19.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 12.5 | 17.1 | 20.0 | <22.6 | 24.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03E | WVDG03S | WVDG03W | WVDG04N | WVDG04E |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 10:25 | 28-Apr-2023 10:25 | 28-Apr-2023 10:25 | 28-Apr-2023 10:45 | 28-Apr-2023 10:45 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2307753-022 | EM2307753-023 | EM2307753-024 | EM2307753-025 | EM2307753-026 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result 0.4 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 1.3 | <2 1.3 | <2 0.8 | <2 1.0 | 5 1.2 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 19 <0.1 | 19 0.1 | 13 0.1 | 15 0.1 | 19 0.6 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 1.4 | <2 1.4 | <2 0.9 | <2 1.1 | 10 1.6 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 20 1.4 | 20 1.5 | 14 1.0 | 16 1.2 | 24 2.2 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 20 0.85 | 21 <0.49 | 15 0.56 | 17 <1.97 | 34 <4.79 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <13.0 | <11.6 | <13.1 | <34.0 | <109 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <27.6 | 12.1 | 11.6 | <68.2 | <50.4 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04S | WVDG04W | WVDD01 Soluble Metals | WVDD02 Soluble Metals | WVDD03 Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 10:45 | 28-Apr-2023 10:45 | 28-Apr-2023 10:00 | 28-Apr-2023 10:10 | 28-Apr-2023 10:25 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2307753-027 | EM2307753-028 | EM2307753-029 | EM2307753-030 | EM2307753-031 |
|  |  |  | Result <0.1 | Result 0.3 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.8 | 4 1.0 | ---- ---- | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 12 0.1 | 15 0.5 | ---- ---- | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.8 | 7 1.3 | ---- ---- | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 12 0.9 | 19 1.8 | ---- ---- | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 13 <1.08 | 26 3.66 | ---- 120.1 | ---- 8.34 | ---- 25.9 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <11.7 | <97.1 | 1030 | 198 | 281 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 11.4 | 45.1 | 2140 | 573 | 1560 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD04 Soluble Metals | WVDD05 Soluble Metals | WVDD007BG Soluble Metals | WVDD08 Soluble Metals | KF1 Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 10:45 | 28-Apr-2023 11:00 | 28-Apr-2023 09:45 | 28-Apr-2023 11:40 | 28-Apr-2023 13:15 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-032 | EM2307753-033 | EM2307753-034 | EM2307753-035 | EM2307753-036 |
|  |  |  | Result 5.53 | Result 3.20 | Result 3.89 | Result 2.52 | Result 6.64 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 78.4 | 140 | 86.8 | 110 | 136 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 232 | 293 | 560 | 236 | 238 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF2 Soluble Metals | KF3 Soluble Metals | KF4 Soluble Metals | WVDG05N Soluble Metals | WVDG05E Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 13:20 | 28-Apr-2023 12:50 | 28-Apr-2023 12:35 | 28-Apr-2023 11:00 | 28-Apr-2023 11:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-037 | EM2307753-038 | EM2307753-039 | EM2307753-040 | EM2307753-041 |
|  |  |  | Result 16.2 | Result 51.4 | Result 4.64 | Result 0.45 | Result 0.70 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 72.6 | 80.7 | 151 | 31.0 | 12.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 516 | 323 | 176 | 12.4 | 63.2 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble Metals | WVDG05W Soluble Metals | WVDG08N Soluble Metals | WVDG08E Soluble Metals | WVDG08S Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 11:00 | 28-Apr-2023 11:00 | 28-Apr-2023 11:40 | 28-Apr-2023 11:40 | 28-Apr-2023 11:40 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-042 | EM2307753-043 | EM2307753-044 | EM2307753-045 | EM2307753-046 |
|  |  |  | Result 0.09 | Result 0.32 | Result 0.57 | Result 0.36 | Result 1.36 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 6.60 | 12.6 | 11.5 | 8.20 | 19.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 12.3 | 17.2 | 22.5 | 36.6 | 42.7 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG08W Soluble Metals | WVDG03N Soluble Metals | WVDG03E Soluble Metals | WVDG03S Soluble Metals | WVDG03W Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 11:40 | 28-Apr-2023 10:25 | 28-Apr-2023 10:25 | 28-Apr-2023 10:25 | 28-Apr-2023 10:25 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-047 | EM2307753-048 | EM2307753-049 | EM2307753-050 | EM2307753-051 |
|  |  |  | Result 0.60 | Result 0.80 | Result 3.88 | Result 0.31 | Result 1.01 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 2.90 | 18.4 | 8.20 | 9.80 | 9.20 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 11.0 | 35.7 | 15.1 | 21.2 | 23.2 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04N Soluble Metals | WVDG04E Soluble Metals | WVDG04S Soluble Metals | WVDG04W Soluble Metals | WVDD01 Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 10:45 | 28-Apr-2023 10:45 | 28-Apr-2023 10:45 | 28-Apr-2023 10:45 | 28-Apr-2023 10:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-052 | EM2307753-053 | EM2307753-054 | EM2307753-055 | EM2307753-056 |
|  |  |  | Result 0.94 | Result 2.68 | Result 0.72 | Result 4.96 | Result <83.5 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 10.8 | 23.0 | 6.70 | 46.4 | <3010 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 29.7 | 62.2 | 21.6 | 47.3 | <2130 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD02 Insoluble Metals | WVDD03 Insoluble Metals | WVDD04 Insoluble Metals | WVDD05 Insoluble Metals | WVDD007BG Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 10:10 | 28-Apr-2023 10:25 | 28-Apr-2023 10:45 | 28-Apr-2023 11:00 | 28-Apr-2023 09:45 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-057 | EM2307753-058 | EM2307753-059 | EM2307753-060 | EM2307753-061 |
|  |  |  | Result <1.92 | Result <6.39 | Result 3.02 | Result 3.59 | Result <2.44 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <34.5 | <217 | <39.9 | <53.4 | <74.6 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 56.1 | <219 | 49.4 | 57.1 | 67.2 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD08 Insoluble Metals | KF1 Insoluble Metals | KF2 Insoluble Metals | KF3 Insoluble Metals | KF4 Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 11:40 | 28-Apr-2023 13:15 | 28-Apr-2023 13:20 | 28-Apr-2023 12:50 | 28-Apr-2023 12:35 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-062 | EM2307753-063 | EM2307753-064 | EM2307753-065 | EM2307753-066 |
|  |  |  | Result <2.23 | Result <7.78 | Result <18.9 | Result <3.75 | Result <8.04 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <52.3 | <191 | <50.3 | 34.7 | <53.4 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 60.8 | <60.8 | 87.3 | 52.8 | 54.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N Insoluble Metals | WVDG05E Insoluble Metals | WVDG05S Insoluble Metals | WVDG05W Insoluble Metals | WVDG08N Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 11:00 | 28-Apr-2023 11:00 | 28-Apr-2023 11:00 | 28-Apr-2023 11:00 | 28-Apr-2023 11:40 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-067 | EM2307753-068 | EM2307753-069 | EM2307753-070 | EM2307753-071 |
|  |  |  | Result 0.75 | Result <0.71 | Result <0.44 | Result <0.60 | Result 1.21 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | 32.4 | <13.3 | <13.0 | <16.5 | 19.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 10.7 | 22.1 | 10.5 | <12.5 | 19.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG08E Insoluble Metals | WVDG08S Insoluble Metals | WVDG08W Insoluble Metals | WVDG03N Insoluble Metals | WVDG03E Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 11:40 | 28-Apr-2023 11:40 | 28-Apr-2023 11:40 | 28-Apr-2023 10:25 | 28-Apr-2023 10:25 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-072 | EM2307753-073 | EM2307753-074 | EM2307753-075 | EM2307753-076 |
|  |  |  | Result <0.58 | Result 2.21 | Result 1.80 | Result <0.75 | Result 1.04 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <18.6 | <21.6 | <22.3 | <19.5 | <13.0 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 19.7 | 25.0 | <22.6 | 26.4 | <27.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Insoluble Metals | WVDG03W Insoluble Metals | WVDG04N Insoluble Metals | WVDG04E Insoluble Metals | WVDG04S Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 10:25 | 28-Apr-2023 10:25 | 28-Apr-2023 10:45 | 28-Apr-2023 10:45 | 28-Apr-2023 10:45 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-077 | EM2307753-078 | EM2307753-079 | EM2307753-080 | EM2307753-081 |
|  |  |  | Result <0.49 | Result 1.07 | Result <1.97 | Result <4.79 | Result <1.08 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <11.6 | <13.1 | <34.0 | <109 | <11.7 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 12.9 | 15.7 | <68.2 | <50.4 | 12.8 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Insoluble Metals | ---- | ---- | ---- | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 28-Apr-2023 10:45 | ---- | ---- | ---- | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2307753-082 | -------- | -------- | -------- | -------- |
|  |  |  | Result 4.33 | ---- ---- | ---- ---- | ---- ---- | ---- ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <97.1 | ---- | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | 56.2 | ---- | ---- | ---- | ---- |

CERTIFICATE OF ANALYSIS

Work Order	: EM2310060

Page	: 1 of 20

Client

Contact

Address

: ALS WATER AND HYDROGRAPHICS PTY LTD

: ROHAN OLIVER

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Customer Services EM

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +61 3 8549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 02-Jun-2023 10:05 |
| Order number | : ---- | Date Analysis Commenced | : 21-Jun-2023 |
| C-O-C number Sampler Site | : ---- : ROHAN OLIVER : ALS KERANG DUST | Issue Date | : 27-Jun-2023 17:38 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Eric Chau	Metals Team Leader	Melbourne Inorganics, Springvale, VIC

right solutions. right partner.

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- Sampling Period: 28/4/23-31/5/23

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- Sample exposure period is 33 days which is outside the typical exposure period of 30+/-2 days as per AS3580.10.1

- Sample container was received with no liquid. 100ml of DI water was used to dissolve contents, perform analysis and for calculation of results.

- Sample exposure period is 33 days which is outside the typical exposure period of 30 +/- 2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 10:50 | 31-May-2023 11:00 | 31-May-2023 11:45 | 31-May-2023 12:00 | 31-May-2023 12:25 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2310060-001 | EM2310060-002 | EM2310060-003 | EM2310060-004 | EM2310060-005 |
|  |  |  | Result 0.3 | Result <0.1 | Result <0.1 | Result <0.1 | Result 0.2 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 5 1.0 | <2 0.3 | <2 0.2 | <2 0.3 | 3 0.1 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 21 6.1 | 6 0.1 | 5 1.0 | 5 0.7 | 3 1.0 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 118 1.3 | <2 0.4 | 19 0.3 | 13 0.3 | 20 0.3 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 26 7.4 | 8 0.5 | 6 1.3 | 6 1.0 | 6 1.3 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | 144 6.02 | 9 1.57 | 25 0.934 | 19 0.581 | 26 0.704 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <91.2 | <21.3 | <26.4 | <18.9 | <23.1 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <88.3 | ---- | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | ---- | 18.1 | 24.0 | 18.6 | 17.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD007BG | WVDD08 | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 10:30 | 31-May-2023 11:15 | 31-May-2023 14:15 | 31-May-2023 13:50 | 31-May-2023 13:55 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2310060-006 | EM2310060-007 | EM2310060-008 | EM2310060-009 | EM2310060-010 |
|  |  |  | Result 1.3 | Result <0.1 | Result <0.1 | Result <0.1 | Result 0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 24 8.8 | <2 0.4 | <2 0.4 | <2 0.3 | 2 0.9 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 172 9.5 | 7 1.5 | 8 1.8 | 6 1.5 | 17 4.8 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 184 10.1 | 30 0.4 | 35 0.5 | 29 0.4 | 94 1.0 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 196 19.6 | 8 1.9 | 9 2.3 | 8 1.9 | 19 5.8 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 380 ---- | 38 ---- | 44 ---- | 37 <10.9 | 113 ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 8.81 | 2.58 | 2.78 | ---- | 5.85 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <260 | <38.9 | ---- | <41.1 | <77.9 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | ---- | ---- | 21.8 | ---- | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | ---- | ---- | ---- | ---- | <105 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 760 | 32.4 | 30.1 | 51.4 | ---- |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 13:40 | 31-May-2023 13:30 | 31-May-2023 11:45 | 31-May-2023 11:45 | 31-May-2023 11:45 |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2310060-011 | EM2310060-012 | EM2310060-013 | EM2310060-014 | EM2310060-015 |
|  |  |  | Result <0.1 | Result 0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | <2 ---- | <2 ---- | ---- 0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 0.5 | ---- 0.3 | 2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 11 ---- | 5 ---- | ---- 0.3 | ---- <0.1 | ---- 0.4 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 1.1 | ---- 1.8 | 5 ---- | <2 ---- | 6 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 21 ---- | 35 ---- | ---- 0.2 | ---- 0.2 | ---- 0.1 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.6 | ---- 0.4 | 4 ---- | 3 ---- | 2 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 12 ---- | 7 ---- | ---- 0.4 | ---- 0.3 | ---- 0.4 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 1.7 | ---- 2.2 | 7 ---- | 5 ---- | 6 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 33 ---- | 42 ---- | ---- 0.6 | ---- 0.5 | ---- 0.5 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | ---- ---- | ---- ---- | 11 <4.50 | 8 ---- | 8 <8.96 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 2.38 | 4.22 | ---- | 4.15 | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <35.9 | <38.0 | <35.1 | <14.2 | <13.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG03N | WVDG03E | WVDG03S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 13:40 | 31-May-2023 13:30 | 31-May-2023 11:45 | 31-May-2023 11:45 | 31-May-2023 11:45 |
| Compound CAS Number LOR Unit EG020T: Total Metals by ICP-MS - Continued |  |  | EM2310060-011 | EM2310060-012 | EM2310060-013 | EM2310060-014 | EM2310060-015 |
|  |  |  | Result ---- | Result ---- | Result ---- | Result ---- | Result <16.3 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month |  |  |  |  |  |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 66.9 | 86.8 | 16.3 | 12.8 | ---- |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03W | WVDG04N | WVDG04E | WVDG04S | WVDG04W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 11:45 | 31-May-2023 12:00 | 31-May-2023 12:00 | 31-May-2023 12:00 | 31-May-2023 12:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2310060-016 | EM2310060-017 | EM2310060-018 | EM2310060-019 | EM2310060-020 |
|  |  |  | Result <0.1 | Result 0.2 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.3 | 3 0.1 | <2 0.4 | <2 0.4 | <2 0.4 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 4 <0.1 | 2 0.3 | 7 0.8 | 7 0.5 | 8 0.7 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.3 | 5 0.3 | 14 0.5 | 8 0.5 | 11 0.5 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 5 0.3 | 5 0.6 | 8 1.3 | 8 1.0 | 9 1.2 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 6 ---- | 10 ---- | 22 ---- | 16 ---- | 20 <4.66 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 0.968 | 1.81 | 1.81 | 0.604 | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | <16.3 | <13.8 | <7.66 | <19.8 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 7.79 | ---- | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | ---- | ---- | <18.9 | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 5.46 | 13.7 | ---- | 7.40 | 23.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N | WVDG05E | WVDG05S | WVDG05W | WVDG08N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 12:25 | 31-May-2023 12:25 | 31-May-2023 12:25 | 31-May-2023 12:25 | 31-May-2023 11:15 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2310060-021 | EM2310060-022 | EM2310060-023 | EM2310060-024 | EM2310060-025 |
|  |  |  | Result 0.1 | Result 0.1 | Result <0.1 | Result <0.1 | Result 0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.4 | 2 0.2 | <2 0.3 | <2 0.4 | 2 0.1 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 6 0.2 | 3 0.1 | 4 0.1 | 7 0.1 | 2 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 4 0.5 | 2 0.3 | 2 0.3 | <2 0.4 | 2 0.2 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 8 0.7 | 5 0.4 | 5 0.4 | 7 0.5 | 4 0.3 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | 12 1.06 | 7 1.09 | 7 0.541 | 8 0.831 | 6 0.441 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <13.1 | <10.6 | <8.50 | <12.9 | <10.5 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <12.8 | ---- | <38.3 | <10.2 | <25.7 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | ---- | 5.32 | ---- | ---- | ---- |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG08E | WVDG08S | WVDG08W | WVDD01 Soluble Metals | WVDD02 Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 11:15 | 31-May-2023 11:15 | 31-May-2023 11:15 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2310060-026 | EM2310060-027 | EM2310060-028 | EM2310060-029 | EM2310060-030 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.3 | <2 0.3 | <2 <0.1 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 5 0.1 | 5 0.2 | <2 0.4 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 2 0.4 | 3 0.4 | 7 0.1 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 6 0.5 | 6 0.6 | 2 0.5 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 8 ---- | 9 <2.99 | 9 ---- | ---- ---- | ---- ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 0.922 | ---- | 3.39 | 42.1 | 9.59 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <16.0 | <20.8 | <25.9 | ---- | ---- |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | ---- | ---- | ---- | 220 | 55.9 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 12.4 | 22.7 | 40.2 | 855 | 140 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble Metals | WVDD04 Soluble Metals | WVDD05 Soluble Metals | WVDD007BG Soluble Metals | WVDD08 Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2310060-031 | EM2310060-032 | EM2310060-033 | EM2310060-034 | EM2310060-035 |
|  |  |  | Result 16.8 | Result 8.86 | Result 5.14 | Result 29.6 | Result 5.78 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 98.3 | 40.5 | 49.8 | 326 | 64.0 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 304 | 106 | 128 | 2020 | 166 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble Metals | KF2 Soluble Metals | KF3 Soluble Metals | KF4 Soluble Metals | KF5BG Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2310060-036 | EM2310060-037 | EM2310060-038 | EM2310060-039 | EM2310060-040 |
|  |  |  | Result 76.1 | Result 96.4 | Result 80.8 | Result 60.9 | Result 65.6 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 52.0 | 60.6 | 177 | 103 | 79.1 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 152 | 189 | 288 | 205 | 230 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N Soluble Metals | WVDG03E Soluble Metals | WVDG03S Soluble Metals | WVDG03W Soluble Metals | WVDG04N Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2310060-041 | EM2310060-042 | EM2310060-043 | EM2310060-044 | EM2310060-045 |
|  |  |  | Result 0.977 | Result 10.4 | Result 8.12 | Result 8.46 | Result 7.73 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 4.98 | 13.2 | 6.69 | 4.35 | 10.3 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 4.42 | 20.4 | 16.4 | 10.8 | 46.0 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E Soluble Metals | WVDG04S Soluble Metals | WVDG04W Soluble Metals | WVDG05N Soluble Metals | WVDG05E Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2310060-046 | EM2310060-047 | EM2310060-048 | EM2310060-049 | EM2310060-050 |
|  |  |  | Result 17.2 | Result 1.82 | Result 19.1 | Result 7.24 | Result 7.22 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 23.1 | 2.11 | 25.1 | 3.50 | 5.47 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 114 | 7.69 | 137 | 16.1 | 9.02 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Soluble Metals | WVDG05W Soluble Metals | WVDG08N Soluble Metals | WVDG08E Soluble Metals | WVDG08S Soluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2310060-051 | EM2310060-052 | EM2310060-053 | EM2310060-054 | EM2310060-055 |
|  |  |  | Result 7.91 | Result 8.85 | Result 7.88 | Result 5.86 | Result 2.14 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 3.76 | 3.66 | 2.27 | 5.51 | 2.12 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 11.8 | 11.1 | 9.92 | 28.8 | 9.12 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG08W Soluble Metals | WVDD01 Insoluble Metals | WVDD02 Insoluble Metals | WVDD03 Insoluble Metals | WVDD04 Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2310060-056 | EM2310060-057 | EM2310060-058 | EM2310060-059 | EM2310060-060 |
|  |  |  | Result 6.32 | Result 8.40 | Result 5.58 | Result 4.38 | Result 4.35 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | <91.2 | <21.3 | <26.4 | <18.9 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 9.36 | ---- | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | ---- | <88.3 | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 50.3 | ---- | 27.3 | 27.9 | 24.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD05 Insoluble Metals | WVDD007BG Insoluble Metals | WVDD08 Insoluble Metals | KF1 Insoluble Metals | KF2 Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2310060-061 | EM2310060-062 | EM2310060-063 | EM2310060-064 | EM2310060-065 |
|  |  |  | Result ---- | Result ---- | Result ---- | Result ---- | Result <10.9 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 4.38 | 14.2 | 5.92 | 5.73 | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <23.1 | <260 | <38.9 | ---- | <41.1 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | ---- | ---- | ---- | 25.8 | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 32.0 | 803 | 58.2 | 57.1 | 63.5 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 Insoluble Metals | KF4 Insoluble Metals | KF5BG Insoluble Metals | WVDG03N Insoluble Metals | WVDG03E Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2310060-066 | EM2310060-067 | EM2310060-068 | EM2310060-069 | EM2310060-070 |
|  |  |  | Result ---- | Result ---- | Result ---- | Result <4.50 | Result ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 9.34 | 4.81 | 5.83 | ---- | 7.08 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <77.9 | <35.9 | <38.0 | <35.1 | <14.2 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <105 | ---- | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | ---- | 80.0 | 112 | 16.5 | 13.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Insoluble Metals | WVDG03W Insoluble Metals | WVDG04N Insoluble Metals | WVDG04E Insoluble Metals | WVDG04S Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2310060-071 | EM2310060-072 | EM2310060-073 | EM2310060-074 | EM2310060-075 |
|  |  |  | Result <8.96 | Result ---- | Result ---- | Result ---- | Result ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | ---- | 2.92 | 2.75 | 2.58 | 1.89 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <13.9 | ---- | <16.3 | <13.8 | <7.66 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | ---- | 8.04 | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <16.3 | ---- | ---- | <18.9 | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | ---- | 8.42 | 15.7 | ---- | 10.3 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Insoluble Metals | WVDG05N Insoluble Metals | WVDG05E Insoluble Metals | WVDG05S Insoluble Metals | WVDG05W Insoluble Metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2310060-076 | EM2310060-077 | EM2310060-078 | EM2310060-079 | EM2310060-080 |
|  |  |  | Result <4.66 | Result ---- | Result ---- | Result ---- | Result ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | ---- | 1.71 | 1.34 | 1.32 | 1.26 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <19.8 | <13.1 | <10.6 | <8.50 | <12.9 |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | ---- | <12.8 | ---- | <38.3 | <10.2 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 25.9 | ---- | 5.71 | ---- | ---- |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG08N Insoluble Metals | WVDG08E Insoluble Metals | WVDG08S Insoluble Metals | WVDG08W Insoluble Metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | 31-May-2023 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2310060-081 | EM2310060-082 | EM2310060-083 | EM2310060-084 | -------- |
|  |  |  | Result ---- | Result ---- | Result <2.99 | Result ---- | ---- ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 1.00 | 1.34 | ---- | 4.65 | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <10.5 | <16.0 | <20.8 | <25.9 | ---- |
| ø Manganese 7439-96-5 | 0.05 | µg/m².month | <25.7 | ---- | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | ---- | 17.1 | 29.1 | 41.4 | ---- |

CERTIFICATE OF ANALYSIS

Work Order	: EM2311983

Page	: 1 of 20

Amendment

Client

Contact

Address

: 1

: ALS WATER AND HYDROGRAPHICS PTY LTD

: ROHAN OLIVER

: 94 KERANG-KOONDROOK ROAD KERANG 3579

Laboratory

Contact

Address

: Environmental Division Melbourne

: Customer Services EM

: 4 Westall Rd Springvale VIC Australia 3171

| Telephone | : ---- | Telephone | : +61 3 8549 9600 |
| --- | --- | --- | --- |
| Project | : MV214940 | Date Samples Received | : 03-Jul-2023 10:30 |
| Order number | : ---- | Date Analysis Commenced | : 05-Jul-2023 |
| C-O-C number Sampler Site | : ---- : R.Oliver : ---- | Issue Date | : 25-Jul-2023 13:03 |
| Quote number No. of samples received No. of samples analysed | : ME/968/20 : 84 : 84 |  |  |

This report supersedes any previous report(s) with this reference. Results apply to the sample(s) as submitted, unless the sampling was conducted by ALS. This document shall not be reproduced, except in full.

This Certificate of Analysis contains the following information:

- General Comments

- Analytical Results

Additional information pertinent to this report will be found in the following separate attachments: Quality Control Report, QA/QC Compliance Assessment to assist with Quality Review and Sample Receipt Notification.

Signatories

This document has been electronically signed by the authorized signatories below. Electronic signing is carried out in compliance with procedures specified in 21 CFR Part 11.

Signatories

Position

Accreditation Category

Eric Chau	Metals Team Leader	Melbourne Inorganics, Springvale, VIC

right solutions. right partner.

General Comments

The analytical procedures used by ALS have been developed from established internationally recognised procedures such as those published by the USEPA, APHA, AS and NEPM.	In house developed procedures are fully validated and are often at the client request.

Where moisture determination has been performed, results are reported on a dry weight basis.

Where a reported less than (<) result is higher than the LOR, this may be due to primary sample extract/digestate dilution and/or insufficient sample for analysis.

Where the LOR of a reported result differs from standard LOR, this may be due to high moisture content, insufficient sample (reduced weight employed) or matrix interference.

When sampling time information is not provided by the client, sampling dates are shown without a time component. In these instances, the time component has been assumed by the laboratory for processing purposes.

Where a result is required to meet compliance limits the associated uncertainty must be considered. Refer to the ALS Contract for details.

Key :

CAS Number = CAS registry number from database maintained by Chemical Abstracts Services. The Chemical Abstracts Service is a division of the American Chemical Society. LOR = Limit of reporting

^ = This result is computed from individual analyte detections at or above the level of reporting ø = ALS is not NATA accredited for these tests.

~ = Indicates an estimated value.

- ALS is not NATA accredited for the analysis of metals in dust deposition gauge.

- Dust analysis as per AS3580.10.1-2016. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in deposition units e.g., g/m².mth where the sampling procedure is not NATA accredited. ALS Mudgee laboratory is NATA accredited for dust sampling, therefore ALS Mudgee reported deposition units are accredited.

- Amendment (25/07/2023): This report has been amended as a result of misinterpretation of sample identification numbers (IDs) for samples #7, 17-20, 35, 45-48, 63, 73-76. All analysis results are as per the previous report.

- Sampling Period 31/05/2023 to 29/06/2023.

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.1

- Sample exposure period is within the typical exposure period of 30+/-2 days as per AS3580.10.2

- Directional dust analysis as per AS3580.10.2-2013. Samples passed through a 1mm sieve prior to analysis. NATA accreditation does not apply for results reported in g/m².mth as sampling data was provided by the client.

- For dust analysis, the Limit of Reporting (LOR) referenced in the reports for deposited matter parameters represents the reporting increment rather than reporting limit.

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD01 | WVDD02 | WVDD03 | WVDD04 | WVDD05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2311983-001 | EM2311983-002 | EM2311983-003 | EM2311983-004 | EM2311983-005 |
|  |  |  | Result <0.1 | Result 0.1 | Result 0.3 | Result 0.6 | Result <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.1 | 2 0.1 | 4 0.5 | 10 1.7 | <2 <0.1 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | <2 1.8 | 2 0.7 | 9 5.7 | 30 7.3 | <2 1.0 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 31 0.1 | 12 0.2 | 97 0.8 | 124 2.3 | 17 0.1 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | <2 1.9 | 4 0.9 | 14 6.5 | 40 9.6 | <2 1.1 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 32 <0.94 | 16 ---- | 111 ---- | 164 ---- | 18 ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | ---- | 1.24 | 4.29 | 6.65 | 0.578 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | ---- | <98.5 | <303 | <21.2 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 35.7 | 35.6 | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 23.6 | 18.7 | 67.7 | 188 | 10.9 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD007BG | WVDD08 | KF1 | KF2 | KF3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2311983-006 | EM2311983-007 | EM2311983-008 | EM2311983-009 | EM2311983-010 |
|  |  |  | Result 0.2 | Result <0.1 | Result 0.2 | Result <0.1 | Result 0.2 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 4 <0.1 | <2 0.3 | 4 0.3 | <2 <0.1 | 2 1.0 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | <2 <0.1 | 4 1.0 | 5 4.1 | <2 0.5 | 17 7.3 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.2 | 17 0.4 | 69 0.5 | 8 0.1 | 124 1.2 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 4 0.2 | 6 1.4 | 9 4.6 | <2 0.6 | 20 8.5 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 4 ---- | 23 ---- | 78 ---- | 9 <2.07 | 144 <4.39 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 1.21 | 1.17 | 3.97 | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <32.5 | ---- | ---- | ---- | <53.1 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | ---- | 24.2 | 25.4 | 13.1 | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 34.7 | 16.8 | 73.6 | 11.6 | 43.0 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG05N | WVDG05E | WVDG05S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] |
| Compound CAS Number EA120: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2311983-011 | EM2311983-012 | EM2311983-013 | EM2311983-014 | EM2311983-015 |
|  |  |  | Result <0.1 | Result <0.1 | Result ---- | Result ---- | Result ---- |
| Ash Content (mg) ---- EA120I: Ash Content Ash Content ---- | 2 0.1 | mg g/m².month | <2 ---- | <2 ---- | ---- 0.1 | ---- <0.1 | ---- <0.1 |
| Ash Content (mg) ---- EA125: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | ---- 0.2 | ---- <0.1 | <2 ---- | <2 ---- | <2 ---- |
| Combustible Matter (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | 3 ---- | <2 ---- | ---- 0.5 | ---- 0.4 | ---- 0.2 |
| Combustible Matter (mg) ---- EA139: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.1 | ---- 2.0 | 7 ---- | 7 ---- | 4 ---- |
| Total Soluble Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 2 ---- | 35 ---- | ---- <0.1 | ---- 0.1 | ---- 0.1 |
| Total Soluble Matter (mg) ---- EA141: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | ---- 0.2 | ---- 0.1 | <2 ---- | 2 ---- | 2 ---- |
| Total Insoluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 3 ---- | <2 ---- | ---- 0.6 | ---- 0.5 | ---- 0.3 |
| Total Insoluble Matter (mg) ---- EA142: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | ---- 0.3 | ---- 2.1 | 9 ---- | 8 ---- | 5 ---- |
| Total Solids (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 5 ---- | 36 ---- | ---- 0.6 | ---- 0.6 | ---- 0.4 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | ---- 2.29 | ---- 1.15 | 9 0.052 | 10 0.192 | 7 0.165 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | <20.9 | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 14.9 | ---- | 3.20 | 7.23 | 4.90 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF4 | KF5BG | WVDG05N | WVDG05E | WVDG05S |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] |
| Compound CAS Number LOR Unit EG020T: Total Metals by ICP-MS - Continued |  |  | EM2311983-011 | EM2311983-012 | EM2311983-013 | EM2311983-014 | EM2311983-015 |
|  |  |  | Result 16.9 | Result 19.9 | Result 2.86 | Result 4.58 | Result 3.50 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month |  |  |  |  |  |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05W | WVDG08N | WVDG08E | WVDG08S | WVDG08W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2311983-016 | EM2311983-017 | EM2311983-018 | EM2311983-019 | EM2311983-020 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.5 | <2 <0.1 | <2 0.4 | <2 0.4 | <2 0.5 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 7 <0.1 | <2 0.2 | 6 0.1 | 6 <0.1 | 8 0.5 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.5 | 3 0.4 | 2 0.5 | <2 0.4 | 7 0.5 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 7 0.5 | 6 0.6 | 7 0.6 | 6 0.4 | 8 1.0 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 7 ---- | 9 ---- | 9 <0.60 | 7 ---- | 15 ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 0.098 | 0.338 | ---- | 0.445 | 0.908 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | ---- | <16.2 | <15.2 | <15.1 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 4.00 | 13.0 | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 2.76 | 22.2 | 14.0 | 9.40 | 24.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03N | WVDG03E | WVDG03S | WVDG03W | WVDG04N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2311983-021 | EM2311983-022 | EM2311983-023 | EM2311983-024 | EM2311983-025 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 | Result <0.1 |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.3 | <2 0.6 | <2 0.5 | <2 0.6 | <2 0.5 |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 4 0.1 | 9 0.2 | 7 0.1 | 9 0.1 | 7 0.1 |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | 2 0.3 | 2 0.6 | <2 0.5 | <2 0.6 | <2 0.5 |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 5 0.4 | 9 0.8 | 7 0.6 | 9 0.7 | 7 0.6 |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.050 | mg µg/m².month | 7 0.224 | 11 0.502 | 8 0.193 | 10 0.164 | 8 0.439 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | ---- | ---- | ---- | <12.7 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 12.2 | 9.41 | 6.47 | 6.04 | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 7.94 | 8.43 | 7.69 | 4.59 | 11.1 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04E | WVDG04S | WVDG04W | WVDD01 Soluble metals | WVDD02 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | [29-Jun-2023] | [29-Jun-2023] | [29-Jun-2023] | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EA120I: Ash Content Ash Content ---- | LOR 0.1 | Unit g/m².month | EM2311983-026 | EM2311983-027 | EM2311983-028 | EM2311983-029 | EM2311983-030 |
|  |  |  | Result <0.1 | Result <0.1 | Result <0.1 | Result ---- | Result ---- |
| Ash Content (mg) ---- EA125I: Combustible Matter Combustible Matter ---- | 2 0.1 | mg g/m².month | <2 0.6 | <2 0.5 | <2 0.5 | ---- ---- | ---- ---- |
| Combustible Matter (mg) ---- EA139I: Total Soluble Matter Total Soluble Matter ---- | 2 0.1 | mg g/m².month | 9 <0.1 | 8 0.1 | 8 <0.1 | ---- ---- | ---- ---- |
| Total Soluble Matter (mg) ---- EA141I: Total Insoluble Matter Total Insoluble Matter ---- | 2 0.1 | mg g/m².month | <2 0.6 | <2 0.5 | <2 0.5 | ---- ---- | ---- ---- |
| Total Insoluble Matter (mg) ---- EA142I: Total Solids Total Solids ---- | 2 0.1 | mg g/m².month | 9 0.6 | 8 0.6 | 8 0.5 | ---- ---- | ---- ---- |
| Total Solids (mg) ---- EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | 2 0.05 | mg µg/m².month | 10 ---- | 9 <1.07 | 9 <0.65 | ---- ---- | ---- ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 0.220 | ---- | ---- | 2.60 | 1.95 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | <34.6 | <18.9 | ---- | ---- |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 7.13 | ---- | ---- | 959 | 119 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 7.10 | 22.2 | 10.9 | 169 | 130 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD03 Soluble metals | WVDD04 Soluble metals | WVDD05 Soluble metals | WVDD007BG Soluble metals | WVDD08 Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2311983-031 | EM2311983-032 | EM2311983-033 | EM2311983-034 | EM2311983-035 |
|  |  |  | Result 36.0 | Result 61.7 | Result 5.10 | Result 3.06 | Result 4.94 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 245 | 1510 | 55.4 | 75.8 | 92.4 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 932 | 2350 | 126 | 230 | 167 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF1 Soluble metals | KF2 Soluble metals | KF3 Soluble metals | KF4 Soluble metals | KF5BG Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2311983-036 | EM2311983-037 | EM2311983-038 | EM2311983-039 | EM2311983-040 |
|  |  |  | Result 3.44 | Result 16.4 | Result 12.7 | Result 8.16 | Result 5.58 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 163 | 76.0 | 157 | 136 | 66.2 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 325 | 93.9 | 482 | 156 | 185 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05N Soluble metals | WVDG05E Soluble metals | WVDG05S Soluble metals | WVDG05W Soluble metals | WVDG08N Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2311983-041 | EM2311983-042 | EM2311983-043 | EM2311983-044 | EM2311983-045 |
|  |  |  | Result 0.204 | Result 0.437 | Result 0.250 | Result <0.050 | Result 1.05 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 7.21 | 16.7 | 10.2 | 0.200 | 14.8 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 1.36 | 8.12 | 5.10 | 0.240 | 117 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG08E Soluble metals | WVDG08S Soluble metals | WVDG08W Soluble metals | WVDG03N Soluble metals | WVDG03E Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2311983-046 | EM2311983-047 | EM2311983-048 | EM2311983-049 | EM2311983-050 |
|  |  |  | Result 0.953 | Result 0.433 | Result 4.80 | Result 0.318 | Result 0.135 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 7.21 | 2.51 | 35.6 | 6.26 | 0.936 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 18.9 | 8.91 | 105 | 12.6 | 1.53 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG03S Soluble metals | WVDG03W Soluble metals | WVDG04N Soluble metals | WVDG04E Soluble metals | WVDG04S Soluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2311983-051 | EM2311983-052 | EM2311983-053 | EM2311983-054 | EM2311983-055 |
|  |  |  | Result 1.06 | Result 0.078 | Result 0.488 | Result 0.569 | Result 0.990 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 5.80 | 5.89 | 6.19 | 3.03 | 10.4 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 16.6 | 33.6 | 16.5 | 5.66 | 7.81 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04W Soluble metals | WVDD01 Insoluble metals | WVDD02 Insoluble metals | WVDD03 Insoluble metals | WVDD04 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2311983-056 | EM2311983-057 | EM2311983-058 | EM2311983-059 | EM2311983-060 |
|  |  |  | Result ---- | Result <0.94 | Result ---- | Result ---- | Result ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 0.657 | ---- | 1.36 | 8.02 | 8.56 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | ---- | ---- | <98.5 | <303 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 3.24 | 73.4 | 82.1 | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 6.78 | 32.4 | 29.2 | 119 | 220 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDD05 Insoluble metals | WVDD007BG Insoluble metals | WVDD08 Insoluble metals | KF1 Insoluble metals | KF2 Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2311983-061 | EM2311983-062 | EM2311983-063 | EM2311983-064 | EM2311983-065 |
|  |  |  | Result ---- | Result ---- | Result ---- | Result ---- | Result <2.07 |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 0.958 | 1.48 | 1.92 | 5.16 | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <21.2 | <32.7 | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | ---- | ---- | 30.2 | 33.2 | 17.2 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 25.0 | 38.1 | 34.5 | 111 | 17.6 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | KF3 Insoluble metals | KF4 Insoluble metals | KF5BG Insoluble metals | WVDG05N Insoluble metals | WVDG05E Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2311983-066 | EM2311983-067 | EM2311983-068 | EM2311983-069 | EM2311983-070 |
|  |  |  | Result <4.39 | Result ---- | Result ---- | Result ---- | Result ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | ---- | 4.56 | 1.33 | 0.277 | 0.393 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <53.1 | ---- | <20.9 | ---- | ---- |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | ---- | 28.1 | ---- | 19.6 | 15.0 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 44.4 | 27.8 | 27.5 | 12.4 | 11.0 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG05S Insoluble metals | WVDG05W Insoluble metals | WVDG08N Insoluble metals | WVDG08E Insoluble metals | WVDG08S Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2311983-071 | EM2311983-072 | EM2311983-073 | EM2311983-074 | EM2311983-075 |
|  |  |  | Result ---- | Result ---- | Result ---- | Result <0.60 | Result ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 0.195 | 0.195 | 0.364 | ---- | 0.515 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | ---- | ---- | ---- | <16.2 | <15.2 |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | 14.2 | 8.75 | 20.8 | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 8.42 | 7.92 | 25.1 | 16.9 | 17.4 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG08W Insoluble metals | WVDG03N Insoluble metals | WVDG03E Insoluble metals | WVDG03S Insoluble metals | WVDG03W Insoluble metals |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.050 | Unit µg/m².month | EM2311983-076 | EM2311983-077 | EM2311983-078 | EM2311983-079 | EM2311983-080 |
|  |  |  | Result 1.04 | Result 0.355 | Result 16.5 | Result 0.365 | Result 0.284 |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <15.1 | ---- | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | ---- | 12.7 | 17.5 | 10.8 | 9.75 |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 25.5 | 11.2 | 18.1 | 9.14 | 7.73 |

Analytical Results

| Sub-Matrix: DUST Sample ID (Matrix: AIR) |  |  | WVDG04N Insoluble metals | WVDG04E Insoluble metals | WVDG04S Insoluble metals | WVDG04W Insoluble metals | ---- |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling date / time |  |  | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | 29-Jun-2023 00:00 | ---- |
| Compound CAS Number EG020T: Total Metals by ICP-MS ø Arsenic 7440-38-2 | LOR 0.05 | Unit µg/m².month | EM2311983-081 | EM2311983-082 | EM2311983-083 | EM2311983-084 | -------- |
|  |  |  | Result ---- | Result ---- | Result <1.07 | Result <0.65 | ---- ---- |
| ø Arsenic 7440-38-2 | 0.050 | µg/m².month | 0.938 | 0.365 | ---- | ---- | ---- |
| ø Barium 7440-39-3 | 0.05 | µg/m².month | <12.7 | ---- | <34.6 | <18.9 | ---- |
| ø Barium 7440-39-3 | 0.050 | µg/m².month | ---- | 9.19 | ---- | ---- | ---- |
| ø Manganese 7439-96-5 | 0.050 | µg/m².month | 17.4 | 9.27 | 29.4 | 11.0 | ---- |