The Gippsland groundwater model

Technical report

June 2015

Authors: Craig Beverly (DEDJTR), Mark Hocking (DEDJTR), Xiang Cheng (DEDJTR), Cassady O’Neil (DEDJTR), Ralf Schroers (DEDJTR) and Simon Baker (DELWP). Acknowledgements The authors acknowledge Jane McInnes (DEDJTR) and Terry McLean (DEDJTR) for their contribution to data collation, quality control, modelling and map production. Independent review was provided by: Professor Rae Mackay, Director, Geotechnical and Hydrogeological Engineering Group, Federation University Australia Professor Dayanthi Nugegoda, Ecotoxicology, School of Applied Science, RMIT University Professor Craig Simmons, Director, National Centre for Groundwater Research and Training, Flinders University.

Contents

Figures	6

Tables	10

Executive summary	11

1	Introduction	13

1.1	Project background	13

1.2	Project objectives	14

1.3	Model specification	14

1.4	Project duration	15

1.5	Existing models	15

2	Hydrogeological setting	29

2.1	Study area	29

2.2	Climate	31

2.3	Topography	37

2.4	Land use	41

2.5	Soil	43

2.6	Hydrology	45

2.7	Flooding	48

2.8	Surface water and groundwater interactions	53

2.9	Hydrogeology	61

2.10	Current groundwater development	65

2.11	Dominant aquifers	66

2.12	Schematic conceptualisation	66

3	Numerical model design	69

3.1	Modelling approach	69

3.2	Data sources	69

3.3	Groundwater model layer extents	70

3.4	Groundwater model layer boundary conditions	72

3.5	Groundwater model layer parameter attribution	72

3.6	Model complexity	94

3.7	Allowance for temperature and density	95

3.8	Allowance for faults	95

3.9	Model recharge	95

3.10	Model evapotranspiration	95

3.11	Model rivers and drains	96

3.12	Model groundwater extractions	99

3.13	Model simulation period	108

3.14	Model stress periods	108

3.15	Steady-state model conditions	108

3.16	Transient model initial conditions	108

4	Model calibration criteria	109

4.1	Key groundwater observation bores	111

4.2	Baseflow estimates	113

4.3	Manual calibration	113

4.4	Automated calibration	113

4.5	Sensitivity analysis	119

5	Steady-state model calibration	120

5.1	Mass balance	123

5.2	Calibration statistics	123

5.3	Depth to watertable and potentiometric surfaces	125

6	Transient model calibration	129

6.1	Calibrated model aquifer parameterisation	129

6.2	Mass balance	130

6.3	Calibration statistics	131

6.4	Spatial residual error	133

7	Model sensitivity	138

8	Model assumptions	141

9	Model fit for purpose	142

10	Scenario modelling	142

10.1	Scenario future climate projects	142

10.2	Scenario development conditions	144

10.3	Scenario model recharge and evapotranspiration rates	149

10.4	Scenario modelling stress periods	149

10.5	Scenario model boundary conditions	149

10.6	Scenario model rewetting setting	149

10.7	Scenario reporting bores	149

10.8	Scenario reporting regions	149

10.9	Scenario reporting attributes	149

10.10	Scenario results	150

10.11	Recovery scenario and results	162

Conclusions and recommendations	164

Glossary	166

References	169

Appendix A: Baseflow estimates	175

Appendix B: Gippsland Region recharge	188

Appendix C: Calibration bores	192

Appendix D: Modelled layer thicknesses	217

Appendix E: Model boundary conditions	233

Appendix F: Groundwater extraction reporting regions	249

Appendix G: Hydrogeological units	256

Appendix H: Time-series simulated versus observed water level data	290

# Figures

Figure 1 Map of Victoria, with the hatched area the showing extent of Gippsland region model 
described in this report	13

Figure 2: Grouping of hydrostratigraphic units incorporated into the IRM model layer structure.	16

Figure 3: Location of active cells and IRM model domain.	17

Figure 4: IRM boundary conditions for Layer 1.	18

Figure 5: Domain and boundary conditions for the ecoMarkets East Gippsland groundwater model  .	20

Figure 6: Domain and boundary conditions for the ecoMarkets East Gippsland groundwater models.	22

Figure 7: Locality map of the CSIRO large-scale model domain.	24

Figure 8: Conceptual model layers adopted in the CSIRO large-scale model .	25

Figure 9: Latrobe Valley finite difference model domain and boundary conditions .	26

Figure 10: Extent of the 2014 Victorian Gippsland Groundwater model.	30

Figure 11: Zones of influence attributed to climate stations within the study area.	32

Figure 12: Interpolated mean annual rainfall (mm/yr) for the period 1957–2013.	33

Figure 13: Interpolated average daily minimum and maximum temperature for the period 1957–2013.	34

Figure 14: Interpolated average daily solar radiation for the period 1957–2013.	35

Figure 15: Potential mean annual evaporation.	36

Figure 16: Elevation of ground surface based on a 20 m digital elevation model	38

Figure 17: Aspect derived from the 20 m digital elevation model..	39

Figure 18: Slope derived from the 200 m DEM.	40

Figure 19: Land use classified into broad groupings.	42

Figure 20: Spatial pattern of the key soil classifications in the Gippsland region.	44

Figure 21: Major rivers of the Gippsland region.	47

Figure 22: Location and extent of 1-in-100 year flooding events in the Gippsland area .	49

Figure 23: Location and extent of major recent flooding events.	50

Figure 24: Surface-groundwater connectivity in the Gippsland region.	55

Figure 25: Groundwater hydrographs of three nested bores located in MID west of Sale plotted 
against daily stream water level of Latrobe River at Rosedale (gauge station no. 226228).	59

Figure 26: Groundwater hydrographs of a transect of  bores located near Mitchell River west of 
Bairnsdale plotted  against daily stream water level of Mitchell River at Rose.	59

Figure 27: Groundwater hydrographs of two nested bores located near Snowy River west of Orbost 
plotted  against daily stream water level of Snowy River at Jarrahmond.	60

Figure 28: Groundwater hydrographs of three nested bores located in the Macalister Irrigation District 
north of Sale plotted against daily stream water level of Avon River at Stratford.	60

Figure 29: Stylised conceptualisation of the dominant water balance components within the 
Gippsland region .	67

Figure 30: Schematic flow conceptualisation of the Seaspray depression showing the Baragwanath 
anticline  and typical water quality variation offshore	68

Figure 31: Modelled layer 1 attribution source layer.	86

Figure 32: Modelled layer 2 attribution source layer.	86

Figure 33: Modelled layer 3 attribution source layer.	86

Figure 34: Modelled layer 4 attribution source layer.	86

Figure 35: Modelled layer 5 attribution source layer.	87

Figure 36: Modelled layer 6 attribution source layer.	87

Figure 37: Modelled layer 7 attribution source layer.	87

Figure 38: Modelled layer 8 attribution source layer.	87

Figure 39: Modelled layer 9 attribution source layer.	88

Figure 40: Modelled layer 10 attribution source layer.	88

Figure 41: Modelled layer 11 attribution source layer.	88

Figure 42: Modelled layer 12 attribution source layer.	88

Figure 43: Modelled layer 13 attribution source layer.	89

Figure 44: Modelled layer 14 attribution source layer.	89

Figure 45: Modelled layer 15 attribution source layer.	89

Figure 46: Modelled layer 16 attribution source layer.	89

Figure 47: Modelled layer 17 attribution source layer.	90

Figure 48: Modelled layer 18 attribution source layer.	90

Figure 49: Modelled layer 19 attribution source layer.	90

Figure 50: Modelled layer 20 attribution source layer.	90

Figure 51: Modelled layer 21 attribution source layer.	91

Figure 52: Modelled layer22 attribution source layer.	91

Figure 53: Modelled layer 23 attribution source layer.	91

Figure 54: Modelled layer 24 attribution source layer.	91

Figure 55: Modelled layer 25 attribution source layer.	92

Figure 56: Modelled layer26 attribution source layer.	92

Figure 57: Modelled layer 27 attribution source layer.	92

Figure 58: Modelled layer 28 attribution source layer.	92

Figure 59: Modelled layer 29 attribution source layer.	93

Figure 60: Modelled layer 30 attribution source layer.	93

Figure 61: Drainage channels and monitoring sites in the Macalister Irrigation District.	97

Figure 62: Mean monthly flow at key gauges monitored by SRW in the Macalister Irrigation District.	98

Figure 63: Annual flow at key gauges monitored by SRW in the Macalister Irrigation District.	99

Figure 64:  Annual total extraction by contribution incorporated into the model.	101

Figure 65: State water account reporting regions 	103

Figure 66: Location of all extractions including S&D bores.	104

Figure 67: Location of all extraction bores excluding S&D bores.	104

Figure 68: Location of extraction bores in modelled layer 2.	105

Figure 69: Location of extraction bores in modelled layer 3.	105

Figure 70: Location of extraction bores in modelled layer 4.	105

Figure 71: Location of extraction bores in modelled layer 5.	105

Figure 72: Location of extraction bores in modelled layer 6.	106

Figure 73: Location of extraction bores in modelled layer 9.	106

Figure 74: Location of extraction bores in modelled  layer 17.	106

Figure 75: Location of extraction bores in modelled layer 18.	106

Figure 76: Location of extraction bores in modelled layer 19.	107

Figure 77: Location of extraction bores in modelled layer 23.	107

Figure 78: Location of extraction bores in modelled layer 30.	107

Figure 79: Location of observation bores used for model calibration.	112

Figure 80: Calibration zones for model layer 1.	114

Figure 81: Calibration zones for model layer 2.	114

Figure 82: Calibration zones for model layer 3.	114

Figure 83: Calibration zones for model layer 4.	114

Figure 84: Calibration zones for model layer 5.	114

Figure 85: Calibration zones for model layer 6.	114

Figure 86: Calibration zones for model layer 7.	115

Figure 87: Calibration zones for model layer 8.	115

Figure 88: Calibration zones for model layer 9.	115

Figure 89: Calibration zones for model layer 10.	115

Figure 90: Calibration zones for model layer 11.	115

Figure 91: Calibration zones for model layer 12.	115

Figure 92: Calibration zones for model layer 13.	116

Figure 93: Calibration zones for model layer 14.	116

Figure 94: Calibration zones for model layer 15.	116

Figure 95: Calibration zones for model layer 16.	116

Figure 96: Calibration zones for model layer 17.	116

Figure 97: Calibration zones for model layer 18.	116

Figure 98: Calibration zones for model layer 19.	117

Figure 99: Calibration zones for model layer 20.	117

Figure 100: Calibration zones for model layer 21.	117

Figure 101: Calibration zones for model layer 22.	117

Figure 102: Calibration zones for model layer 23.	117

Figure 103: Calibration zones for model layer 24.	117

Figure 104: Calibration zones for model layer 25.	118

Figure 105: Calibration zones for model layer 26.	118

Figure 106: Calibration zones for model layer 27.	118

Figure 107: Calibration zones for model layer 28.	118

Figure 108: Calibration zones for model layer 29.	118

Figure 109: Calibration zones for model layer 30.	118

Figure 110: Steady-state equivalent annual recharge.	121

Figure 111: Steady-state equivalent annual potential groundwater evapotranspiration .	122

Figure 112: Observed versus computed steady-state heads 	124

Figure 113: Residual error (observed head – modelled head) (m) for each steady-state calibration bore.	124

Figure 114: Simulated 1970 unconfined aquifer potentiometric surface 	126

Figure 115: Simulated 1970 steady-state depth to watertable.	127

Figure 116: 1990 watertable depth within the West Gippsland CMA region.	128

Figure 117: Transient model mass balance components .	131

Figure 118: Number of observation points throughout the simulation period.	132

Figure 119: Temporal trace of RMS error throughout simulation period.	132

Figure 120: Temporal trace of mean residual error throughout simulation period.	133

Figure 121: Temporal trace of mean absolute error throughout the simulation period.	133

Figure 122: Spatial mean residual water level error for 1980 based on observed and predicted heads.	134

Figure 123: Spatial mean residual water level error for 1990 based on observed and predicted heads.	135

Figure 124: Spatial mean residual water level error for 2000 based on observed and predicted heads.	136

Figure 125: Spatial mean residual water level error  for 2010 based on observed and predicted heads.	137

Figure 126: Sensitivity of the steady-state model to variations in recharge.	139

Figure 127: Sensitivity of the steady-state model to variations in potential groundwater 
evapotranspiration rate.	139

Figure 128: Sensitivity of the steady-state model to variations in river conductance.	140

Figure 129: Sensitivity of the steady-state model to variations in drain conductance.	140

Figure 130: Cumulative residual rainfall traces for each climate station within the study domain 
and showing 2006,  selected as the future climate scenario.	143

Figure 131: Spatial extents of the prospect fields (1), sub-regional (2) and regional (3) boundaries.	145

Figure 132: Sub-regional tight and shale and coal seam gas extents.	148

Figure 133: “Coal seam gas affected area”: Scenario 1 total drawdown area on shallow watertable 
and T1 coal due to coal seam gas development.	152

Figure 134: “Coal seam gas drawdown”: Scenario 1 average drawdown in the impacted shallow 
watertable and T1 coal due to coal seam gas development.	152

Figure 135: “Tight and shale gas affected area”: Scenario 2 total drawdown area on shallow 
watertable and Strzelecki Group due to tight and shale gas development.	153

Figure 136: “Tight and shale gas drawdown” Scenario 2 average drawdown in the impacted shallow watertable and Strzelecki Group due to tight and shale gas development	153

Figure 137: Simulated coal seam gas extraction volume  under maximum (Scenario 1) and 50% 
(Scenario 6) conditions.	154

Figure 138: Simulated tight and shale gas extraction volume under maximum (Scenario 2) and 50% (Scenario 4)  conditions.	154

Figure 139: Scenario 1 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown 
compared  to baseline at 2042.	155

Figure 140: Scenario 2 shallow watertable (top) and tight and shale gas aquifer (bottom) drawdown compared to baseline at 2042.	156

Figure 141: Scenario 3 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown  
compared  to baseline at 2042.	157

Figure 142: Scenario 4 shallow watertable (top) and tight and shale gas aquifer (bottom) drawdown  compared to baseline at 2042.	158

Figure 143: Scenario 5 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown 
compared  to baseline at 2042.	159

Figure 144: Scenario 6 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown 
compared to baseline at 2042.	160

Figure 145: Scenario 7 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown 
compared  to baseline at 2042.	161

Figure 146: Preliminary unstructured grid developed for the study area.	165

Figure 147: Zoomed in area within the preliminary unstructured grid highlighting the construct 
of the model  and localised grid refinement adjacent to surface features.	165

# Tables

Table 1: Summary of key features of the top five ranked models by SKM (2011a) and the CSIRO model currently under development.	28

Table 2: Land use per CMA, calculated as a percentage of the area of each CMA.	43

Table 3: Extents of historical flood events .	51

Table 4: Modelled flood extents and dates.	52

Table 5: Estimated baseflow index (BFI) in 51 unregulated catchments in the Gippsland region .	56

Table 6: Sources of calibration data used.	70

Table 7: Groundwater model layers.	71

Table 8: Values of specific yield, specific storage and lateral hydraulic conductivity assigned to each modelled layer in the Gippsland model.	73

Table 9: Summary of Confidence Level Classification criteria adopted in the Australian Groundwater Modelling Guidelines .	94

Table 10:  Gippsland Model appraisal based on the abbreviated guideline 	94

Table 11: Gauges monitoring drain from the Macalister Irrigation District .	98

Table 12: State Water Accounts .	100

Table 13: Modelled extractions within each GMA and WSPA reporting region 	101

Table 14: Statistical measure of model performance.	110

Table 15: Steady-state mass balance.	123

Table 16: Steady-state model calibration statistics.	123

Table 17: Calibrated model aquifer parameterisation	129

Table 18: Transient model calibration statistics.	131

Table 19: Key model assumptions.	141

Table 20: Onshore natural gas development data 	146

Table 21: Basic description of the primary development scenarios.	147

Table 22: Scenario configurations.	150

Table 23: Scenario shallow watertable impacts relative to baseline at 2042.	150

Table 24: Scenario coal seam gas impacts relative to baseline at 2042.	151

Table 25: Scenario tight and shale gas impacts relative to baseline at 2042.	151

Table 26: Summary of recovery results by modelled layer.	163

- Executive summary

This report describes the conceptualisation, development, calibration and application of the Gippsland groundwater model.  This model has been developed to investigate and understand the potential impacts of future onshore gas developments and to understand the possible impacts of a potential onshore natural gas industry on groundwater and surface waters within the Gippsland region.

The groundwater model was required to be conceptualised, constructed and calibrated within a six month period to provide information for a screening study. During this time data acquisition, quality control and data conversion tasks were undertaken.  The model adopts a uniform spatial resolution of 400 m and includes representation of the dominant aquifers and aquitards.  The model layer structure and attribution are based on a combination of (1) new stratigraphic mapping and interpretation developed by Geological Survey of Victoria, (2) the Victorian Aquifer Framework (VAF), (3) previous groundwater model data, and (4) existing maps and cross-sections.  The model comprises 30 layers, coal seams specified as discrete layers and Cretaceous sediments of the Strzelecki formation in discrete sub-layers.  Key improvements compared to previous Gippsland region groundwater models include (1) new interpreted data sets that extend to the catchment boundaries, (2) finer grid resolution, (3) the incorporation of offshore stratigraphic information as developed by CSIRO and Geological Survey of Victoria, and (4) incorporation of a basement layer representing the rocks underlying the sedimentary basin and extending to the upland regions of the Gippsland region where the basement outcrops.

The model has been used to quantify groundwater flow and groundwater head levels within specified aquifers under historical conditions.  Analysis of simulated groundwater heads indicates that the calibrated model satisfies the designated model specifications. The groundwater model has the capacity to be used to assess the relative difference in predicted groundwater level changes between the proposed scenarios.  It must be noted that it is not appropriate to use the model to assess the absolute water level or water balance under either calibration or scenario conditions.

Seven hypothetical tight and shale gas and coal seam gas (CSG) development scenarios were considered under a dry future climate.  Each scenario was designed to estimate the likely water usage of onshore gas development under varying well field designs and configurations.  Details of the scenarios are provided in Section 10.2 and Tables 20 and 21, and provide for a potential gas development area, target depth, well field design and other necessary modelling estimates. Additional scenarios with reduced scale of development (reduced volume of pumping for hypothetical gas development) were run to supplement understanding of model behaviour.

Simulation predictions were reported relative to a baseline state in which quarterly groundwater abstractions were fixed at averaged equivalent quarterly values for the period 2003 to 2012; all other conditions were as applied in the 2001–2012 model calibration/verification period.

Predictive scenario results suggest that the most substantial impacts on the shallow watertable are associated with coal seam gas development, whereas tight and shale gas developments have a negligible impact.  Broadscale development of prospective coal seam gas areas is predicted to create an average drawdown in the shallow watertable of 10 m and impact 193 000 ha.  The impacted area is predicted to reduce to 170 000 ha with an average drawdown of 8 m under a 50% volume coal seam gas development

condition. Alternatively, under full entitlement pumping extraction and 50% volume coal seam gas development conditions the impacted area is 208 000 ha.  In contrast, the maximum area affected under tight and shale gas extraction is estimated to be less than 1000 ha, and the effect on shallow watertable levels is very small.

Other key observations include:

For each gas type, the relevant impacted area is increasing and has not reached an equilibrium condition by the end of the 30 year scenario period.

For coal seam gas extractions, the impact on the shallow watertable occurs within 3 months following commencement of depressurisation.

For tight and shale gas development, the impacts on the shallow watertable first begin 23 months following commencement of depressurisation.

All predictive scenario results are considered conservative and represent the likely maximum impacts of onshore gas development on the shallow watertable, aquifer potentiometric surfaces, aquifer heads and drawdowns.  The conservative results are due to the following conditions:

- a prolonged dry future climate

- full development of the prospective gas field at commencement of gas extraction

- relatively high vertical hydraulic conductivities assigned to a some modelled layers when compared to other studies, though within reported limits.

In order to allow for recent history of water use in Gippsland groundwater use was averaged from the 2002–2012 extraction information, which is greater than the long-term average due to the drought conditions of the previous decade. No allowance for localised changes in aquifer hydraulic conductivity from gas and storage coefficients due to depressurisation processes.

Density head correction adjustments were also made to all groundwater observation bore data. Due to data limitations the model does not explicitly simulate non-isothermal processes, density-driven flows, contaminant flow or subsidence.

This groundwater model provides the capacity to evaluate and compare groundwater management scenarios and to assess the likely impacts of future climate and pumping regimes on groundwater resources within the Gippsland region.  The model was conceptualised and calibrated at a design level that meets the requirements of a moderate complexity regional scale groundwater model as defined in the Murray-Darling Basin Commission Groundwater Flow Modelling Guidelines (Middlemis, 2000; Middlemis et al., 2000) and more recently meets the requirements of a Class 2 model confidence level classification as defined in the National Water Commission Australian Groundwater Modelling Guidelines (Barnett et al., 2012).  Within the scope of the project this provided sufficient confidence for this model to be used in the Gippsland region onshore natural gas water science studies.

Importantly, it is considered that while the developed groundwater model cannot be used predictively to quantify the water balance or water levels at the regional scale under specific development scenarios, it is appropriate for comparison of different scenarios for volumetric changes to the water balance or water levels against a baseline condition.

# Introduction

This report details the conceptualisation, development and calibration of a split steady-state transient groundwater model representing the Victorian Gippsland region located in Victoria’s south-east as shown in Figure 1. The model incorporates both the highland areas north of the basin and extends offshore to beyond the current oil and gas extraction platforms.  The report also details the design and evaluation of seven onshore hypothetical tight and shale gas and coal seam gas development scenarios.

.

Figure 1: Map of Victoria, with the hatched area the showing extent of the Gippsland region model described in this report

Project background

The Gippsland groundwater model was developed for the whole Gippsland region (incorporating onshore and offshore areas) and was developed using:

- available geological and groundwater data (including the Victorian Aquifer Framework mapping) and

- data obtained from universities, energy companies, water authorities and major licensed water users.

The calibrated groundwater model was designed to enable an assessment of the regional scale impacts on groundwater and surface water due to gas development.  It can also assess the impacts of key water uses such as coal mine developments and offshore oil and natural gas extraction, although variations in these uses were not investigated.  The modelling accounts for all existing groundwater usage (onshore and offshore) and integrated farming systems land use to best estimate the current water balance in the region.  Mine pumping uses existing pumping wells, and pumping is set to achieve target pressures. Offshore extraction is explicitly modelled, using equivalent pumping volumes at the relevant offshore platforms.

Hypothetical coal seam gas, and tight and shale gas development scenarios have been used to determine the assess the potential impact a gas extraction industry may have on water resources.  The model is intended to be used to compare different scenarios for changes to the water balance and water levels against a baseline condition.  Absolute changes in the water balance, while informative, are difficult at this regional scale.

Project objectives

The Gippsland Groundwater Model was developed to help quantify the potential groundwater and surface water impacts of possible onshore gas developments in the Gippsland region.  The model will be used to quantify impacts arising from potential developments (both individually and cumulatively) including:

the drawdown in groundwater heads

the reduction in baseflow to rivers that drain the basin

the reduction in water availability to groundwater-dependent ecosystems.

Model specification

Minimum specifications for the groundwater model and associated outputs are:

- uniform finite difference grid of 400 metre cell size

- model domain represents the entire Victorian Gippsland region and extends offshore to adequately represent oil and gas extractions

- 30 modelled layers representing the major geological units and consistent with existing documented hydrostratigraphic units

- 822 groundwater monitoring observation bores used for calibration

- 8175 groundwater pumping bores representing licensed pumps, domestic and stock usage and offshore oil and gas platforms

- recharge estimates (steady-state and transient) generated using the Catchment Analysis Tool (CAT) farming system models and provided by DEDJTR

- allowance for episodic flood recharge events

- groundwater evaporation extinction depth to be based on land use

- groundwater evaporation rate to account for unsaturated vegetation evapotranspiration rates such that the summed saturated and unsaturated evaporation does not exceed potential evaporation as calculated from meteorological data

- a normalised (scaled) RMS of less than 5% for steady-state based on matching groundwater head observation data

- a normalised (scaled) RMS of less than 10% for the transient model based on matching groundwater head observation data at selected and agreed sites

- existing mapped depth to watertable and groundwater baseflow estimates to be considered

- a transient split calibration/validation period of 10 and 13 years respectively (1990–1999 and 
2000–2012)

- a sensitivity analysis to assess the variability of modelled outputs to variations in key model input parameters

- catchment-scale model water balance error of less than 2%

- all significant catchment water balance features to be considered and reported

- the source and a statement of quality of all input data sets to be reported

- accuracy in drawdown outputs within 5 to 10 m

an annual time step for the simulation period 1970–1989 and thereafter monthly time steps for the simulation period 1990–2012 and beyond, which was considered to be sufficiently small to determine adverse impacts during periods of low surface water flow but also mindful of the impact on model 
run time.

Project duration

The duration of the project to conceptualise, develop and calibrate the groundwater model of the Gippsland region as detailed in this report was from July 2014 to December 2014.  Data acquisition, quality control, data conversion and review were undertaken within the same timeframe.

Existing models

A review of the existing groundwater management tools developed for the Gippsland Basin (SKM, 2011a) commissioned by the then Victorian Department of Sustainability and Environment (DSE) identified 19 models ranging in complexity from impact assessment frameworks to distributed physics-based numerical models.  The models reviewed were:

- Stage 1 (Fraser 1980) and review of Stage 1 report (Golder Associates, 1990)

- Stage 2 (Evans, 1983)

- Stage 3 (Bolger, 1987)

- Reservoir simulation of the Gippsland Basin (Henzell et al., 1985)

Loy Yang (Bolger 1990; Golder Associates, 1990)

- Latrobe Valley Resource Model — Update (Golder Associates, 1991)

- Latrobe Valley Resource Model — Update (Golder Associates, 1992a)

- Latrobe Valley Resource Model — Update (Golder Associates, 1992b)

- Latrobe Valley Resource Model — Update (Golder Associates, 1992c)

- Stage 4 (GeoEng, 1994)

- Stage 5 (GeoEng, 1996a)

- Gippsland Basin (SKM, 1995)

- Gippsland Basin (SKM, 1996)

- Yarram sub-regional model (SKM, 1999)

- Gippsland Basin (Nahm, 2002)

- Sale WSPA Groundwater Model (SKM, 2011a)

- Integrated Resource Model (IRM) of the Gippsland Basin (Schaeffer, 2008)

ecoMarkets (GHD, 2008a; 2010a; 2010b; 2010c).

The review later included the DPI Gippsland Basin Model (Beverly et al., 2012) which was derived from the IRM Gippsland Basin model developed by Schaeffer (2008).  Most recently CSIRO has developed a Latrobe Valley Coal Model (LVCM) to assess the likely water management issues associated with potential coal seam gas extractions in the Latrobe Valley (Freij-Ayoub et al., 2011a).  Summarised below are the key features of those models considered to have sufficient extent, grid resolution, design focus and attribution relevant to this study.

## 1.5.1	IRM model

The Integrated Resource Model (IRM) was developed by Schaeffer (2008) to simulate the impacts of different groundwater abstractions and multiple pumping and artificial recharge scenarios on groundwater levels within the Gippsland Basin.  The model represented eighteen layers comprising the unconfined surface layer, the basement layer, eight regional aquifers and eight regional aquitards (Figure 2).  Time dependent general head boundaries were assigned to the eastern and southern regions of the model domain, with no flow boundaries assigned to the western and northern regions of the model (Figure 3 and Figure 4).  Spatially variable conductivities were assigned based on lithological analysis and structural regions.  Recharge was assigned to sub-cropping aquifers and incorporated groundwater extractions accounted for offshore pumping, coal mine dewatering and irrigation in the Yarram region.

Calibration of this model was based on matching simulated heads and groundwater trends with observation bore data for the period 1960–1999.  The error criterion was 10 m with model validation based on the period 2000–2004.

The recommendations for future improvement of this model as reported by SKM (2011a) included improved calibration of the unconfined aquifer, refinement of the recharge input and extension of the eastern and southern boundaries to reduce boundary effects when assessing the impacts of artificial recharge and pumping adjacent to these boundaries.

Figure 2: Grouping of hydrostratigraphic units incorporated into the IRM model layer structure (sourced from 
Schaeffer, 2008).

Figure 3: Location of active cells and IRM model domain (sourced from Schaeffer, 2008).

Figure 4: IRM boundary conditions for Layer 1 (source:Schaeffer, 2008).

## 1.5.2 	ecoMarket models

The ecoMarket groundwater models (GHD, 2008a; 2010a) were developed to assess potential impacts on depth to the water table and baseflows for a range of land use change scenarios. Two models were developed with identical specifications: one for the West Gippsland Catchment Management Authority (CMA) region and one for the East Gippsland CMA region.  Both models extend offshore to the freshwater–seawater interface. The offshore extent of the model is based on structural and stratigraphic mapping.  Each model was constructed in MODFLOW (Harbaugh, A.W., 2005) using a 200 by 200 m grid and represented seven layers.

In the case of the East Gippsland model, transient head boundaries were used offshore so the effects of offshore oil and gas pumping could be simulated at the southern model boundary (Figure 5). Stream cells were used over a large proportion of the high ground area to simulate the streams and rivers, whereas river cells were used to simulate lakes. General head boundary cells were set at an average sea level (0.1 mAHD) across the upper most active layer in all offshore areas.  The peer reviewer concluded these boundary conditions were appropriate (PB, 2010a). However, it is noteworthy that the steady-state model for this area did not converge and consequently had a large water balance error.  That is, an adequate steady-state calibration was not achieved for the East Gippsland ecoMarkets groundwater model, which has implications for the robustness of the associated transient model.

A review of the East Gippsland model identified that the ability of the model to simulate the actual groundwater flow systems was constrained by a lack of data to define those flow systems over the majority of the modelled area (PB, 2010a). Consequently, the usefulness of this model is effectively limited to a proportion of the lowland areas. That part of the model covering the elevated areas representing the Mesozoic basement outcrop is not reliable and is considered to be of limited use. Even in the lowland 
areas, the model has significant limitations regarding its primary purpose. The limitations take into account regard the effects of land use changes, which are likely to be poorly predicted because in many areas the computed heads, gradients, trends in heads, and responses to stresses do not match the observed data. 
As reported by PB (2010a), more effort would be required in the calibration of localised areas for the East Gippsland ecoMarkets groundwater model to be fit for purpose in all areas for which groundwater level data are available.

Figure 5: Domain and boundary conditions for the ecoMarkets East Gippsland groundwater model (source: GHD, 2010c).  
.

In the case of the ecoMarkets West Gippsland groundwater model, the model extends to the onshore topographic divides across which no-flow boundaries are assigned (Figure 6).  The eastern boundary extends into East Gippsland along the Mitchell River, and the western boundary extends to the offshore saltwater interface, along which a no-flow boundary is assigned that forces groundwater outflow to climb along the interface and discharge far out to sea. The offshore extent of the model is based on structural and stratigraphic mapping.

The shoreline is simulated in Layer 1, only with a density corrected head of 0.1 mAHD. Stream cells representing third order and higher order streams are used over a large proportion of the high ground area, whereas first and second order streams are represented as drain cells.  River boundaries are also used to represent surface water bodies on the lowlands.  Time-varying drain cells with high conductance values (10 000 m2/d) were used to represent the dewatering of coal mines.  A reviewer (PB, 2010b) noted that, while this is normal practice, it was not clear why this approach was not successful in reducing adjacent heads to observed levels, given that some heads were identified as being 100 m too high.

Steady-state was based on 1970s pre-development water levels and 1957–2005 average recharge.  While a steady-state solution was achieved, no comparative groundwater level contour maps were shown to demonstrate the degree of agreement with the observed groundwater level contour maps.  The transient model calibration period was from 1970 to 1985, with monthly stress periods.  A warm-up period was run from 1960 to 1969 to provide better initial conditions (than steady-state heads) in a system disturbed by mining and characterised by declining heads.  Verification was undertaken for the period 1986 to 1990.  From the statistics and other calibration evidence, PB (2010b) concluded that the model was fit for the purpose as specified for the ecoMarkets study, which focused on the watertable aquifer but could not be used reliably in deeper layers where coal-mining effects are evident in observed datasets.

The peer review of the West Gippsland model by PB (2010b) highlighted that the West Gippsland model was unique among the ecoMarkets models in having to simulate extensive dewatering associated with coal mining. The review identified that depressurisation caused by coal mining was poorly simulated and consequently the groundwater model could not be recommended for broader water resource management purposes.  The review stated that the model might be suitable for this purpose in areas very distant from the mines.  In conclusion, the review stated that there is uncertainty over the usefulness of the West Gippsland ecoMarkets groundwater model for the water resource management of deeper aquifers, given the poor simulation of mine depressurisation.

Figure 6: Domain and boundary conditions for the ecoMarkets East Gippsland groundwater models (source: GHD 2010b).

## 1.5.3 	CSIRO model

The CSIRO Gippsland Basin model was developed in collaboration with DPI Victoria (Geological Survey of Victoria) to assess coal seam gas water management in the Gippsland Basin. The objective of the study was to build and show the capability CSIRO has in assessing the impact of producing coal seam gas on aquifer pressure heads. The study also provided information related to the risk of land subsidence adjacent to coal seam gas production wells. The area selected for the study was a region in the Gippsland Basin with abundant coal and mining activity.

The developed model qualitatively predicted the amount of recovered methane and water from single or multiple coal seams which could be separated by either an aquifer or a fine grained clayey-silty material typically considered to be an aquitard. The model coupled geomechanics and fluid flow and as such was capable of predicting possible land subsidence associated with reservoir depletion due to coal seam methane production. The modelling approach used the FLAC3D axisymmetric finite difference software, which assumes single-phase flow under which water and desorbed gas is modelled as one fluid. That is, the desorbed methane is treated as a chemical species carried with the fluid whereby methane transport is controlled by water advection and its diffusion in the water. This model simulates the simultaneous production of two coal seams 50 m and 44 m thick separated by 12 m of another layer of sediments (Freij-Ayoub et al., 2011b). The modelling explores the influence of coal permeability on the quantities of produced methane and water in a wellbore.

In addition to the bore modelling, the CSIRO study developed a numerical model based on Schaeffer (2008) to assess the impact of economically viable methane gas extractions on the hydraulic heads in aquifers underlying coal seams in the Latrobe Valley (Figure 7). This large-scale study used MODFLOW, which is limited to single phase water production. As such the predicted impact of extracted methane on the hydraulic head in the produced or adjacent layers was considered negligible at the modelled basin-wide scale (Freij-Ayoub et al., 2011b). The MODFLOW model considered a domain of 291 km by 186 km with a uniform grid resolution of 1 km, and adopted a 10-layered simplified stratigraphic representation of the Gippsland Basin as shown in Figure 8. Of note was that the model assumed that the head is hydrostatic.  Constant head boundary conditions were assigned on the assumption that the edge-effect boundary impacts associated with the scenario modelling would be negligible. The domain and boundary conditions are shown in Figure 9.  In addition to the large-scale basin-wide model, a smaller scale model was developed specifically for the Latrobe Valley with a domain of 40 km by 20 km with a uniform grid resolution of 100 m (Freij-Ayoub et al., 2011b). This model estimated the impact of coal seam gas extractions on aquifer heads in the Latrobe Valley.  In this case, the extraction estimates were predicted using the COMET3 modelling package.

Figure 7: Locality map of the CSIRO large-scale model domain (source: CSIRO unpublished data).

Figure 8: Conceptual model layers adopted in the CSIRO large-scale model (source: CSIRO, unpublished data.).

Figure 9: Latrobe Valley finite difference model domain and boundary conditions (source: CSIRO, unpublished data).

## 1.5.4 	Summary of existing groundwater model attributes

A summary of the relevant model attributes and considerations required for this study is presented in 
Table 1.  For each of the top five ranked models as reported by SKM (2011a) and including the CSIRO (Freij-Ayoub et al., 2011a) and DPI (Beverly et al., 2012) models developed in 2012, a score is assigned based on the original ranking procedure adopted in SKM (2011a) using a Multi Criteria Assessment (MCA) approach involving the following steps:

definition of a set of criteria

assignment of weightings to the various criteria

identification of performance objectives for the agreed criteria

scoring and ranking of the alternatives.

An aggregated performance rating of 1 to 3 was used in which 1 refers to does not meet the set of criteria, 2 satisfies more than 50% of the criteria and 3 satisfies all criteria  It is noteworthy that the SKM (2011a) study compiled results from two independent reviewers.

In general, the MCA analysis has identified that no existing groundwater management tool meets all the model specifications required for this project.  Specifically no existing model has sufficient spatial extent, vertical resolution or model grid as outlined in the project specification.

Table 1: Summary of key features of the top five ranked models by SKM (2011a) and the CSIRO model currently under development.

|  | Yarram sub-regional (SKM 1999) | Gippsland Basin (Nahm 2002) | Sale WSPA (SKM 2008) | Integrated Resource IRM (Schaeffer, 2008) | ecoMarkets (GHD, 2008a, 2010a) | CSIRO (Freij-Ayoub et al., 2011a) | DPI (Beverly et al., 2012) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Extent focus | onshore | offshore | onshore | onshore, offshore | onshore | offshore | onshore |
| Inclusion of offshore extractions | 1 | 2 | 1 | 3 | 1 | 3 |  |
| Representation of within-Basin structural features | 2 | 2 | 2 | 2 | 2 | 2 |  |
| Number of layers | 4 | 7 | 4 | 18 | 7 | 10 | 18 |
| Grid resolution | 500–2000 m | ? | 1 km | 1 km | 200 m | 1 km | 200 m |
| Representation of major aquifers and aquitards | 1 | 2 | 1 | 3 | 2 | 3 | 3 |
| Representation of both onshore and offshore hydrogeology | 2 | 2 | 0 | 3 | 2 | 3 | 3 |
| Representation of appropriate boundary conditions | 1 | 2 | 1 | 3 | 3 | 3 | 3 |
| Accurate representation of groundwater/surface water interface | 1 | 2 | 2 | 2 | 1 | 2 | 3 |
| Ability to represent abstraction and injection wells | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| Ability to determine extraction impacts on environment/users | 2 | 2 | 2 | 3 | 2 | 3 | 3 |
| Ability to identify shallow water tables | 2 | 2 | 2 | 3 | 2 | 3 | 3 |
| Ability to simulate steady-state conditions | 3 | ? | 3 | 3 | 2 | 3 | 3 |

Hydrogeological setting

Study area

The proposed extent of the Gippsland Groundwater model is shown in Figure 10 and includes the East and West Gippsland CMA regions and significant offshore area.  The model domain is defined to the north by the Great Dividing Range, to the west by the Latrobe and Tarwin river catchments, and to the east by the New South Wales – Victoria border and Thurra River catchment. The southern limit of Wilsons Promontory defines the boundary offshore. In total the model covers an area of 6,698,000 ha of which 3,629,000 ha exists onshore and 3,069,000 ha offshore.

The model domain was selected to provide regional scale assessment and cumulative impacts, as distinct from fine scale site specific local impact assessment.  Importantly the domain extent ensures that boundary condition impacts on regions of interest associated with existing mine activity and likely areas of potential resource development are minimised.  Key elements of the domain are as follows:

## 2.1.1 	Spatial extent

The spatial extent of the model domain is shown in Figure 10. It illustrates the following:

- Offshore extents of the aquifers are extended to model offshore extraction and to avoid boundary condition influences.

- North-western and western boundaries are defined according to the full extents of the surface water catchments, thereby including all recharge zones to the pre-Tertiary.  This enables consideration and evaluation of future of climate change scenarios.

- North-eastern boundary is defined based on the full extent of the surface water catchments, and bounded by the New South Wales – Victoria border.  It was considered prudent to limit the model domain to the state border on the basis of data acquisition.  In addition it was considered that the model domain terminating at the New South Wales – Victoria border was sufficiently large to capture the primary recharge extents that impact the study region.

The south-western boundary follows the Victorian coastline along the Tarwin Groundwater Catchment outline.

The model domain covers key river basins, including the Latrobe, Thomson, Macalister, Avon, Mitchell, Tambo, Snowy, Tarwin, Agnes and Tarra Rivers. It includes four Water Supply Protection Areas (Denison, Sale, Wy Yung and Yarram) and eight Groundwater Management Areas (Giffard, Leongatha, Moe, Orbost, Rosedate, Stratford, Tarwin, Wa-De-Lock).

## 2.1.2 	Vertical extent

The vertical scale of the model was designed to incorporate conventional, tight, shale and brown coal gas structures while enabling the calibration of pre-tertiary base flows.  Key design features included the incorporation of:

- Mesozoic sediments (Strzelecki group) greater than 8km thick in the offshore basin (primarily incorporated for tight and shale gas scenarios)

- Tertiary layers based on Victorian Aquifer Framework (SKM 2011b; GHD 2012a), which groups hydrogeological units into major hydrostratigraphic units (all layers were explicitly modelled to allow for this.)

- coal seams based on stratigraphic information sourced from Jansen & Maher (2003).

Figure 10: Extent of the 2014 Victorian Gippsland groundwater model.

Climate

The climate in the Gippsland region is temperate and generally humid. However, there is considerable variation across the region.  In the Strzelecki Ranges and Great Dividing Range, annual rainfall can be as high as 1500 mm, averaging over 1000 mm, and includes snow falls on the higher peaks during winter. In lower levels east of the Snowy River, mean annual rainfall is typically about 900–950 mm.  Rain shadows occur in the river valleys and on the plains in the central region. In the area around Sale, annual rainfall can be less than 600 mm. Mean maximum temperatures in lower areas range from 24 °C in January to 15 °C in July.  In the highlands of the Baw Baw Plateau and the Errinundra Plateau, mean temperatures range from a maximum of 18 °C to a minimum of 8 °C, and in winter mean minima in these areas can be as low as –4 °C, leading to heavy snowfalls that often isolate the Errinundra Plateau between June and October.

For this study, daily climate information was obtained from data from the Queensland Department of Science, Information Technology, Innovation and the Arts SILO Climate data website (DSITI, 2015) for each of the 199 climate stations with meteorological data for the period 1957–2013 that are located within the study area (Figure 11).  For a given climate station, daily climate data is a combination of original measurements and rectified data to remove any gaps in the record using interpolation methods discussed in Jeffrey et al. (2001).  To account for sparsely located climate stations within the study area, daily rainfall, temperature, evaporation and solar radiation data were scaled to each solution point within the study area according to interpolated mean annual spatial layers created using the ANUClim software (Hutchinson 2001).  This approach combines a DEM and temporal climatic data to generate a smoothed climate surface.  Daily meteorological data assigned to each solution point within the study area was therefore a function of the data from the 199 climate stations, landscape position and topography.  The patch point shape file identifying the extent of influence of each climate station is shown in Figure 11, whereas Figure 12, 
Figure 13, Figure 14 and Figure 15 show the mean annual rainfall, average daily temperature, average 
daily radiation and mean average potential evaporation for the period 1957–2012 across the study 
area, respectively.

Figure 11: Zones of influence attributed to climate stations within the study area.

Figure 12: Interpolated mean annual rainfall (mm/yr) for the period 1957–2013.

Figure 13: Interpolated average daily minimum (top) and maximum temperature (bottom) (ºC) 
for the period 1957–2013.

Figure 14: Interpolated average daily solar radiation (MJ/m2) for the period 1957–2013.

Figure 15: Potential mean annual evaporation (mm/yr).

Topography

The Gippsland region extends from the Great Dividing Range in the north to the Gippsland Lakes and Wilsons Promontory in the south and from the Latrobe River catchment and Strzelecki Ranges in the west to the New South Wales border in the east.  The topography in the Gippsland region is highly varied (Figure 16, Figure 17 and Figure 18).  The northern half is dominated by forested river valleys and mountains in the Great Dividing Range. Mt Wellington (1632 m) is a landmark at the southern end of the Snowy Range, a long ridge that extends south from Mt Howitt to form the divide between the Macalister and Wonnangatta Rivers, and on which Mt Reynard (1737 m) is the highest point south of the Great Divide.  Most major rivers in the Gippsland region originate in the Great Dividing Range, including the Latrobe, Thomson, Macalister, Mitchell, Tambo, Nicholson and Snowy Rivers.

Much of the southern part of the Gippsland region is covered by the Eastern Plains, ranging from undulating rises to almost level plains.  The surficial sediments are mostly alluvial and range in age from Quaternary to Recent. These mainly comprise sediments derived from the uplands to the north. The youngest sediments are the flood plains, swamps and morasses associated with the present rivers and streams. Between Traralgon and Bairnsdale, mostly north of the Latrobe River, are extensive Quaternary terraces associated with the earlier rivers and streams. A number of terraces have been identified; the earliest is suggested to be the result of the Kosciusko Uplift. Later terraces are attributed to sea level changes associated with glacial periods that occurred during the Quaternary.  The most widespread terrace is believed to date from the Early Quaternary and is a high-level terrace extending from Stratford to Bairnsdale, in which some areas are mantled by sand dunes.

The Strzelecki Ranges is dominated by low volcanic hills stretching from Warragul to Thorpdale and Leongatha. The highest peak is Mount Tassie (approximately 740 m). The ranges were originally covered by a mosaic of wet forest and temperate rainforest. The native vegetation was extensively cleared for agriculture in the late 19th and early 20th centuries.

Most of East Gippsland is mountainous and wooded and is managed as a national park. The Buchan Caves near Orbost are one of the few examples of cavernous limestone in Gippsland.

Figure 16: Elevation of ground surface (mAHD) based on a 20 m digital elevation model (source: Gallant et al., 2011).

Figure 17: Aspect derived from the 20 m digital elevation model.  Aspect is measured as the angle in degrees from north.

Figure 18: Slope derived from the 200 m DEM.

Land use

Land use varies greatly throughout the Gippsland region (Figure 19).  The majority of arable land used for farming consists of dryland pasture and is used for grazing beef cattle and sheep as well as some dryland cropping. The high country is mostly used for beef and sheep production.  Sheep production for both wool and prime lambs was historically a major enterprise within the hill areas. However, there has been a steep decline in sheep numbers due to the presence of wild dogs, drought, and labour shortages.  The land use in the foothills and plains is predominantly sheep and beef, with some cropping enterprises.

The most significant irrigation area is the Macalister Irrigation District around Maffra, which is predominantly used for dairy production. Small pockets of irrigated agriculture (mainly horticulture and dairy) occur in the more fertile river flats around Orbost, Genoa, Cann River, Yarram and Bairnsdale. In these regions irrigated agriculture uses both surface water and groundwater.

Land use data for this study were classified using the Australia Land Use Mapping (ALUM) classification Version 6 (BRS 2006).  The ALUM taxa describe land cover against which management strategies need to be specified. A total of 49 land use classifications were adopted in this study.  In the case of cropping enterprises, crop rotations and cropping history were incorporated into the management scripts used in the catchment modelling. Based on the scale and resolution of the spatial land use mapping, more than 98% of land use in the study area is agricultural, with the remainder consisting of conservation areas, urban or built infrastructure and water bodies.

The following is a description of the land uses as they are defined for West Gippsland and East Gippsland Catchment Management Authority areas and as summarised in Table 2.

Tree cover accounts for 43% and 33% of the land use in the West Gippsland and East Gippsland regions respectively, including all natural conservation and reserve lands, managed natural resources and a range of production forestry uses. Most of this is concentrated in the northern and eastern highlands, Strzelecki Ranges and Wilsons Promontory.  Smaller patches are scattered across the central and coastal areas.  Softwood production areas are distinguished in the modelling, however they account for less than 7% and 17% of the total area in the West Gippsland and East Gippsland regions respectively, occurring in isolated patches. Forest nursery and some other woody plant land uses are also separated, and they comprise less than 0.1% of the area.

Grazing, dryland production systems, pasture mosaics and a range of other rural residential classes are grouped into a general “Annual Pasture” class which accounts for 49% and 64% of the area in the West Gippsland and East Gippsland regions respectively.  A large proportion of this class occurs in the northwest and along the southern slopes of the study adjacent to the uplands forests.

Cropping includes cereals, legumes, shrub fruits and berries, seasonal and intensive horticulture, fruits and shade/glass houses.  This broad group covers less than 1% of the study area and occurs most heavily in the west of the study area, although some is scattered across the central area.  In contrast, irrigated crops (4% in West Gippsland and 2% in East Gippsland) occur only in the central region around Maffra and in the fertile river flats around Orbost, Genoa, Cann River, Yarram and Bairnsdale.

Degraded lands (<0.1%), urban development (1%) and water bodies (1.4%) make up the remaining land uses.  By comparison they comprise small areas scattered across the study area.

Figure 19: Land use classified into broad groupings.

Table 2: Land use per CMA, calculated as a percentage of the area of each CMA.

| Description | West Gippsland | East Gippsland |
| --- | --- | --- |
| Remnant trees | 35.81 | 16.39 |
| Softwood | 7.46 | 16.66 |
| Degraded pasture | 0.78 | 0.33 |
| Annual pasture | 48.76 | 63.78 |
| Irrigated pasture | 3.82 | 1.72 |
| Perennial horticulture | 0.19 | 0.18 |
| Seasonal horticulture | 0.10 | 0.11 |
| Cropping | 0.24 | 0.00 |
| Urban | 1.13 | 0.42 |
| Water | 1.70 | 0.42 |

Soil

The Gippsland region has a great diversity of soil types that reflect differences in parent material, topography, climate, organic content and age (e.g. degree of weathering). Soils in the ranges are generally of granitic origin and have sandy loam to sandy clay loam textures. The foothills of the ranges include podosol type soils with a “coffee rock” layer at depth in the profile. Soils on the coastal plains around Bairnsdale largely originate from material deposited by streams and from dune forming processes. Most 
soils contain alluvial sediments and are texture contrast soils (soils with a loamy or sandy loam surface overlying a heavy clay subsoil). Textures vary from loamy sands to sandy loams and tend to be moderately to strongly acidic.

For this modelling study, soil data were derived by merging broadscale 1 : 250,000 land classification survey data and 1 : 25,000 soil attribute coverage (Smith 2002). The merged spatial soil layer was attributed using the Factual Key of Northcote (1979) at the Principal Profile Form (PPF) level to classify different soil types. Within the study area 169 different soil types were identified and spatially assigned. The spatially assigned key soil classifications are presented in Figure 20. The soil attribution through depth for each soil type was based on published data sources (McKenzie et al., 2000), field observations and pseudo-transfer functions (van Genuchten et al., 1991) and includes soil water characteristics, bulk densities, hydraulic properties and impedance properties specified for each soil layer modelled. Additionally each soil required specification of erodability, erosion and soil evaporation attributes.

Figure 20: Spatial pattern of the key soil classifications in the Gippsland region.

Hydrology

The Gippsland region includes the South Gippsland, Latrobe, Thomson, Macalister, Avon, Mitchell, Tambo and Nicholson, Snowy and East Gippsland river basins (Figure 21). The Latrobe, Thomson-Macalister, Avon, Tambo, Nicholson and Mitchell rivers flow into the Gippsland Lakes, while the Tarwin, Snowy and Cann rivers flow into small estuaries or to the sea. The Tarwin, Agnes and Tarra Rivers flow steeply from the southern face of the Strzelecki Ranges to the coast. The Gippsland Lakes form the largest estuarine lake system in Australia. Along with Ninety Mile Beach they are major features of the region. Dunes and wetlands are common around the lakes and along the coast. There are also some large reservoirs such as Blue Rock, Thomson and Glenmaggie, and extensive channel and drain networks in the Macalister Irrigation District.

Latrobe River

The Latrobe Basin includes the Latrobe, Tanjil, Tyers, Moe, Morwell and Traralgon river systems. The Latrobe River rises at an elevation of approximately 750 m due west of Noojee. Its headwaters, together with the Ada, Loch and Toorongo rivers, drain the eastern and southern slopes of the Yarra Ranges, which form a southern extension of the Great Dividing Range. Approximately 70 km from its source, the Latrobe River emerges from the foothills onto its floodplain. The river flows through the Latrobe Valley and discharges into Lake Wellington, at the western end of the Gippsland Lakes. The Latrobe River is the highest contributor of freshwater inflows to the Gippsland Lakes contributing 44% of mean annual inflow. The Latrobe River system has been most affected by regulation and extraction, with a 33% reduction of inflows to Lake Wellington (Tilleard et al., 2009).

The Latrobe Valley houses two major water storages: Blue Rock Lake (capacity 208 GL) and Moondarra Reservoir (capacity 30 GL). Another much smaller storage, Lake Narracan, is situated on the main stem of the Latrobe River at Yallourn near Moe. Blue Rock and Lake Narracan are primarily used to supply Yallourn, Loy Yang A and Loy Yang B power stations.

The environmental condition of the Latrobe River varies from excellent in the headwaters to moderate and poor condition in the mid – lower reaches below the storages. Freshwater flows from the Latrobe Basin are critical for sustaining the health of the Ramsar-listed Gippsland Lakes, which underpin the region’s tourism industry.

Mitchell River

The Mitchell River has been identified as a heritage river and as one of two iconic rivers in Victoria.  This is largely because of its size, being the largest unregulated river in Victoria, and because it supports a wide range of environmental and social values.  Land use in the upper catchment includes sections of the Alpine National Park and the Mitchell River National Park.

The Mitchell River system originates on the southern slopes of the Great Dividing Range, with the Wonnangatta and Dargo Rivers being the two major rivers in this area, and forming the Mitchell River downstream at their confluence.  The Mitchell River discharges to Lake King within the Gippsland Lakes.  Other surface water systems include the Humfray, Dargo, Wongungarra and Wonnangatta rivers, Wentworth Creek and the Gippsland Lakes.

The river system is highly valued by the community for urban and rural water supply, recreation and its contribution to the Gippsland Lakes.  The Mitchell River provides about one third of the total flow to the Lakes system on average.

Geologically 70% of the catchment consists of fractured rock systems in Palaeozoic-aged metasediments and intrusive rocks. Small areas of contained alluvial valley sequences occur in the highland valleys together with layered systems within Tertiary-aged volcanics. The southern third of the catchment contains the regional aquifer systems of the Gippsland Basin sequence (SKM, 2012a).

Thomson–Macalister system

The Thomson River flows from the Dividing Range and joins the Latrobe River south of Sale. The Macalister River flows from the Alpine National Park in the Snowy Ranges and joins the Thomson River between Maffra and Sale.  The Thomson and Macalister rivers then continue towards and contribute flows to the Ramsar-listed Gippsland Lakes.  The Thomson River above Cowwarr Weir is listed as a Heritage River.

There are two major storages within the Thomson–Macalister system, namely Lake Glenmaggie situated on the Macalister River and the Thomson Reservoir on the upper reaches of the Thomson River.  Lake Glenmaggie is the main source of supply for the Macalister Irrigation District, the largest irrigation area south of the Great Dividing Range.  The Thomson Dam provides water to Melbourne as well as the Macalister Irrigation District.

However, both the Thomson and Macalister Rivers are amongst the most stressed rivers in Victoria due to the altered flow regimes through regulation and over allocation of water for irrigation and consumptive use.  As such river management is considered to have the greater impact on mitigating river stress than groundwater/baseflow management.  The Thomson and Macalister Rivers directly below the Thomson Dam and Lake Glenmaggie respectively have reduced annual flows and reversed seasonality as a result of the reservoirs, with high flows in January and February and low flows during the winter months.  This altered flow regime has significant impacts on the breeding and migration cues for fish. The dams and weirs are also a barrier to fish movement.  Poor water quality in the rivers is affecting water supplies for consumptive use and recreation, as well as the Gippsland Lakes.

Avon River

The Avon River rises on the slopes of Mount Wellington in the eastern highlands and flows south to Lake Wellington.  A significant portion of the upper catchment is protected in the Avon Wilderness Area (295 km2).  The upper catchment is steep, heavily forested and largely inaccessible. The channel is stable and confined by bedrock.  The upper reaches of the Avon River including Turton and Dolodrook Rivers and Ben Cruachan Creek have been listed as Natural Catchment Areas.

The Lower Avon catchment below the Valencia Creek confluence flows through a topographically flat area which has been predominantly cleared for agriculture.  The Avon River above Stratford and Freestone Creek has experienced dramatic widening since European settlement and now flow through wide mobile gravel beds.  The Perry River joins the Avon approximately 1 km upstream from where the Avon discharges into Lake Wellington.

In the lower part of the Avon catchment, both surface water and groundwater are utilised for irrigation.  The river has experienced low flows over the past decade and as a consequence, frequent irrigation restrictions have been imposed (Jones et al., 2009).

Snowy River

The Snowy River originates on the slopes of Mount Kosciuszko, draining the eastern slopes of the Snowy Mountains in New South Wales, before flowing through the Alpine National Park and the Snowy River National Park in Victoria and emptying into Bass Strait.  The river flow was drastically reduced in the mid 20th century after the construction of four large dams (Guthega, Island Bend, Eucumbene and Jindabyne) and many smaller diversion structures in its headwaters in New South Wales, as part of the Snowy Mountains Scheme.

Figure 21: Major rivers of the Gippsland region.

Flooding

Flooding contributes to episodic recharge and was considered in the Gippsland groundwater modelling study.  For example, in early June 2012 much of Gippsland experienced heavy rainfall, causing flooding across a number of municipalities, including Latrobe, Wellington and East Gippsland. Hydrograph analysis revealed that groundwater levels responded to this event.

For this study flood data was sourced from the Victorian Flood Database and included information regarding location and extent of a 1-in-100 year event (Figure 22) and historical flood events. Figure 23 shows the location and extent of recent major floods and Table 3 presents the magnitude of each flooding event in the past 100 years (including area directly affected and duration).

Although damage caused by some recent flooding events (e.g. June 2012) to towns and communities was widespread in the Gippsland region, the areas directly affected by flooding were relatively small compared to total area of the region.  The extent of the 1-in-100 year flooding event only covers 4.8% of the total area of the Gippsland region.   In the last 100 years, flooding events mainly occurred in small areas around the lower reaches of the Mitchell, Thomson, Latrobe and Snowy Rivers.  The area directly affected by flooding was generally less than 1% of the Gippsland region.  The largest flood event occurred in June 2012 and covered 1.4% of the Gippsland region.  Given the areas affected by historic flooding events were relatively small, it is believed that the impact of flooding on groundwater recharge is insignificant at the regional scale, but likely to be significant at the local scale.

By examining groundwater hydrographs across the Gippsland region, a noticeable impact of flooding on groundwater recharge is only evidenced in the shallow aquifer systems well connected to the lower reaches of the Thomson, Mitchell and Latrobe rivers. It was found that recharge spikes in groundwater hydrographs of some shallow bores near the rivers are aligned to high river flow events.

Stream gauge station

Figure 22: Location and extent of 1-in-100 year flooding events in the Gippsland region (source: Victorian Flood Database).

Stream gauge station

Figure 23: Location and extent of major recent flooding events (source: the Victorian Flood Database).

Table 3: Extents of historical flood events (sourced from the Victorian Flood Database).

| Date | Duration (days) | Area affected (ha) | % of total catchment area |
| --- | --- | --- | --- |
| Jan-1919 | Not known | 2717 | 0.1 |
| Jan-1920 | Not known | 1129 | 0.0 |
| Oct-1923 | Not known | 208 | 0.0 |
| Jan-1934 | Not known | 7527 | 0.2 |
| Dec-1934 | Not known | 13361 | 0.4 |
| May-1968 | 1 | 544 | 0.0 |
| Jun-1969 | 1 | 644 | 0.0 |
| Feb-1971 | Not known | 1666 | 0.0 |
| Sep-1974 | Not known | 42 | 0.0 |
| Jan-1977 | Not known | 11323 | 0.3 |
| Jul-1977 | 3 | 20248 | 0.5 |
| Jan-1978 | Not known | 2884 | 0.1 |
| May-1978 | 5 | 14947 | 0.4 |
| Jun-1978 | 5 | 12086 | 0.3 |
| Jul-1978 | 1 | 3709 | 0.1 |
| Jan-1985 | Not known | 740 | 0.0 |
| Dec-1985 | 1 | 6427 | 0.2 |
| Nov-1988 | 1 | 7061 | 0.2 |
| Apr-1990 | 27 | 32563 | 0.9 |
| Oct-1990 | 1 | 3278 | 0.1 |
| Oct-1991 | 1 | 169 | 0.0 |
| Sep-1993 | 16 | 2155 | 0.1 |
| Jun-1998 | 3 | 6364 | 0.2 |
| Nov-1998 | 1 | 949 | 0.0 |
| Jun-2007 | 4 | 23308 | 0.6 |
| Jul-2011 | Not known | 36 | 0.0 |
| Mar-2012 | 8 | 9023 | 0.2 |
| Jun-2012 | 3 | 54912 | 1.4 |
| 1-in-100 year extent | N/A | 181490 | 4.8 |

Table 4: Modelled flood extents and dates.

| Flood event date | Recorded dates |
| --- | --- |
| 2012June_ext | 6/6, 7/6 |
| 2012March_ext | 3/3, 9/3,10/3 |
| 2011July_ext | 1/7 |
| 2007June | 26/6, 29/9 |
| 1998Nov_ext | 14/11 |
| 1998June_ext | 23/6, 24/6, 25/6 |
| 1993Sep_ext | 1/9, 14/9, 15/9, 16/9 |
| 1991Oct_ext | 3/10 |
| 1990Oct_ext | 11/10 |
| 1990April_ext | 1/4, 22/4, 23/4, 25/4, 27/4 |
| 1988Nov_ext | 18/11 |
| 1985Dec_ext | 12/12 |
| 1985Jan_ext | 1/1 |
| 1978July_ext | 9/7 |
| 1978June_ext | 1/6, 4/6, 5/6 |
| 1978May_ext | 21/5, 22/5, 25/5 |
| 1978Jan_ext | 1/1 |
| 1977July_ext | 1/7, 27/7 |
| 1977Jan_ext | 1/1 |
| 1974Sep_ext | 1/9 |
| 1971Feb_ext | 1/2 |
| 1969June_ext | 4/6 |

Surface water and groundwater interactions

Groundwater generally interacts with surface water through various processes and pathways. The development of groundwater often has impacts on major streams (and vice versa).  As such it is necessary to manage groundwater and surface water resources in combination.  This requires an understanding of the interconnectivity and processes underpinning surface water and groundwater interactions.  In the context of water resource management it is important to understand and account for surface-groundwater water interaction when considering issues, including:

double accounting of water resources

impacts of groundwater pumping on stream flow, particularly flow depletion

surface water requirements for downstream users

water requirements for environmental purposes (e.g. floodplain, stream, wetland ecosystems)

health of groundwater dependent ecosystems (GDEs)

conjunctive resource management strategy development and water allocation regime

salinity impacts on water quality, salt loads, and ecosystem health

management for climate variation/change and its impacts on groundwater–surface water systems.

To better inform the numerical groundwater model developed for the Gippsland region, a brief assessment of surface-groundwater water interaction across the study area was undertaken, based on available literature (e.g. DSE 2012, SKM 2012a, 2012b; Hofmann, 2011) and analysis of groundwater and surface water information. DSE (2012) collated a state wide dataset of groundwater and surface water interaction from numerous investigations across Victoria.  The dataset described groundwater and surface water interaction in four broad classes: neutral/losing, gaining, variable and unclassified (Figure 24). SKM (2012a) undertook baseflow separation analysis for 180 stream gauges on unregulated rivers in Victoria. This included 51 gauges in the Gippsland region.  The baseflow separation analysis was undertaken on historical river flow records up to 2012 and utilised a filter parameter of 0.98.  The results of the analysis for the 51 stream gauges in the Gippsland region is summarised in Table 5.

The groundwater and surface water interaction in the main rivers in the Gippsland regions is summarised below.

In the East Gippsland catchment it is believed that the shallow aquifers are well connected to the rivers and all river reaches are generally gaining.  The annual average base flow indices (BFI) are high, ranging from 0.72 to 0.79 (DSE, 2012; SKM, 2012b).  As there are few groundwater monitoring bores in the catchment, these gaining conditions are not evidenced by a groundwater hydrograph.

In the Latrobe catchment, the river reaches in the upper part of the catchment (upstream of Traralgon) are generally gaining with high annual average BFI ranging from 0.71 to 0.79 (DSE, 2012; SKM, 2012b).  The surface water – groundwater interaction in the lower part of the catchment is not well understood.  Analysis of groundwater and stream flow data indicated that the flux exchange between the river and shallow aquifers is temporally and spatially variable.  The river reaches near the Macalister Irrigation District (MID) might be dominated by gaining condition due to elevated watertable in the MID (Figure 25).  Deep aquifers (e.g. Latrobe Group) are generally artesian and poorly connected to the river.  Total licensed groundwater use in the catchment is estimated to be 14.4 GL/yr and extractions for mine dewatering are approximately twice this volume (SKM, 2012a). SKM (2012a) conclude that the impacts of current rates of groundwater extraction on the streamflow in the Latrobe River are low.

In the Mitchell catchment the river reaches in the upper part of the catchment are generally gaining with  annual average BFI ranging from 0.66 to 0.76 (DSE, 2012; SKM, 2012b).  The surface-groundwater interaction in the lower part of the catchment was investigated by Monash University and SKM (2012a) using hydrogeochemistry and radon as a tracer (Hofmann 2011) and using an analytical modelling approach respectively.  There is a significant interaction between groundwater and surface water in the Mitchell River floodplain area.  Hofmann (2011) found that these river reaches have gaining and losing sections and these invert depending on the flow conditions of the river.  This is supported by groundwater hydrographs of the bores near the river (Figure 26).  SKM (2012a) found that a high proportion of the Mitchell River catchment water yield is derived from the higher altitude areas in the upper catchment.  Flows at Bairnsdale are much lower than those upstream and the Mitchell River becomes a losing stream as it emerges from the ranges (SKM, 2012a). SKM (2012a) estimated that the volume of discharge lost from the Mitchell River due to groundwater extraction is negligible when compared to the cumulative yearly discharge, but considerable during periods of low flow.  During low flows periods, it is estimated that groundwater extraction can lead to a 13% reduction in streamflow.  Groundwater levels are therefore considered to have a significant influence on the flow and aquatic habitat condition of the Mitchell River during low flow periods.

In the Snowy catchment most river reaches are gaining with annual average BFI ranging from 0.69 to 0.79 (DSE, 2012; SKM, 2012a).  The surface water – groundwater interaction in the lower part of the catchment is not well understood.  Analysis of limited groundwater and stream flow data indicated that the flux exchange between the river and aquifers is variable (Figure 27).

In the South Gippsland catchment, based on stream flow analysis, most river reaches are gaining, with annual average BFI ranging from 0.64 to 0.78 (DSE, 2012; SKM, 2012a).

In the Tambo catchment  the river reaches in the upper part of the catchment are generally gaining, with annual average BFI ranging from 0.69 to 0.77 (DSE, 2012; SKM, 2012a).  The surface water-groundwater interaction in the lower part of the catchment was investigated by Unland (2013) using hydrogeochemistry tracer techniques.  Unland (2013) found that these river reaches have gaining and losing sections and these invert depending on the flow conditions of the river.

In the Thomson catchment (including the Avon catchment) the river reaches in the upper part of the catchment are generally gaining, with annual average BFI ranging from 0.64 to 0.73 (DSE, 2012; SKM, 2012a).  The river reaches in the MID are also dominated by gaining condition due to elevated watertable (Figure 28).  The surface water-groundwater interaction in the lower part of the catchment was investigated by Monash University using hydrogeochemistry and radon as a tracer (Hofmann 2011). Hofmann (2011) found that these river reaches have gaining and losing sections and these invert depending on the flow conditions of the river.  There is significant groundwater/surface water interaction along the Avon River.  SKM (2008b) reported that the Avon River is strongly gaining along its main stem and along most tributaries.  During periods of average rainfall, the groundwater baseflow component of stream flow is approximately 
24–36% of average annual flow in the Avon River and 17–25% of average annual flow in Freestone Creek.  Groundwater entitlement in the catchment is 13.8 GL/yr and current use is approximately 3.6 GL/yr.  
SKM (2008b) suggests that current groundwater use reduces streamflow by approximately 3.4 GL/yr.  
Based on these estimates it is suggested that if groundwater usage increases to full entitlement (assuming pumping infrastructure can support such usage) then groundwater extractions may have a significant impact on streamflow.

Main towns

Stream gauge station

Figure 24: Surface-groundwater connectivity in the Gippsland region (source:  data compiled by DSE in 2012).

Table 5: Estimated average baseflow index (BFI) (by season and annual) in 51 unregulated catchments in the Gippsland region (sourced from SKM 2012b).

| Gauge ID | Gauge name | River basin | BFI Summer | BFI Autumn | BFI Winter | BFI Spring | BFI Annual |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 221201 | Cann River (West Branch) @ Weeragua | East Gippsland | 0.81 | 0.83 | 0.71 | 0.74 | 0.77 |
| 221204 | Thurra River @ Point Hicks | East Gippsland | 0.79 | 0.78 | 0.61 | 0.7 | 0.72 |
| 221207 | Errinundra River @ Errinundra | East Gippsland | 0.83 | 0.86 | 0.72 | 0.75 | 0.79 |
| 221208 | Wingan River @ Wingan Inlet National Park | East Gippsland | 0.82 | 0.79 | 0.6 | 0.71 | 0.73 |
| 221209 | Cann River (East Branch) @ Weeragua | East Gippsland | 0.81 | 0.8 | 0.67 | 0.72 | 0.75 |
| 221210 | Genoa River @ The Gorge | East Gippsland | 0.8 | 0.78 | 0.67 | 0.72 | 0.74 |
| 221211 | Combienbar River @ Combienbar | East Gippsland | 0.82 | 0.84 | 0.72 | 0.74 | 0.78 |
| 221212 | Bemm River @ Princes Highway | East Gippsland | 0.83 | 0.86 | 0.71 | 0.75 | 0.79 |
| 226008 | Tyers River West Branch @ Morgans Mill | Latrobe River | 0.83 | 0.83 | 0.7 | 0.71 | 0.77 |
| 226012 | Tanjil River East Branch @ Tanjil Bren | Latrobe River | 0.79 | 0.77 | 0.69 | 0.61 | 0.72 |
| 226016 | Waterhole Creek @ Morwell | Latrobe River | 0.9 | 0.82 | 0.68 | 0.73 | 0.78 |
| 226017 | Jacobs Creek @ Otooles | Latrobe River | 0.79 | 0.81 | 0.61 | 0.64 | 0.71 |
| 226204 | Latrobe River @ Willow Grove | Latrobe River | 0.86 | 0.87 | 0.72 | 0.73 | 0.8 |
| 226217 | Latrobe River @ Hawthorn Bridge | Latrobe River | 0.85 | 0.87 | 0.69 | 0.72 | 0.78 |
| 226219 | Tooronga River @ Noojee | Latrobe River | 0.85 | 0.88 | 0.7 | 0.72 | 0.79 |
| 226220 | Loch River @ Noojee | Latrobe River | 0.87 | 0.89 | 0.68 | 0.73 | 0.79 |
| 226226 | Tanjil River @ Tanjil Junction | Latrobe River | 0.83 | 0.87 | 0.67 | 0.65 | 0.75 |
| 226407 | Morwell River @ Boolarra | Latrobe River | 0.85 | 0.82 | 0.58 | 0.66 | 0.72 |
| 226410 | Traralgon Creek @ Koornalla | Latrobe River | 0.84 | 0.81 | 0.55 | 0.64 | 0.71 |
| 224205 | Dargo River @ Dargo (Upper Site) | Mitchell River | 0.79 | 0.8 | 0.57 | 0.53 | 0.67 |
| 224209 | Cobbannah Creek @ Near Bairnsdale | Mitchell River | 0.67 | 0.69 | 0.66 | 0.68 | 0.67 |
| 224213 | Dargo River @ Lower Dargo Road | Mitchell River | 0.79 | 0.83 | 0.48 | 0.56 | 0.66 |
| 224214 | Wentworth River @ Tabberabbera | Mitchell River | 0.78 | 0.86 | 0.7 | 0.7 | 0.76 |
| 222202 | Brodribb River @ Sardine Creek | Snowy River | 0.83 | 0.86 | 0.71 | 0.75 | 0.79 |
| 222206 | Buchan River @ Buchan | Snowy River | 0.77 | 0.83 | 0.6 | 0.61 | 0.7 |
| 222210 | Deddick River @ Deddick (Caseys) | Snowy River | 0.79 | 0.83 | 0.76 | 0.77 | 0.74 |
| 222213 | Suggan Buggan River @ Suggan Buggan | Snowy River | 0.77 | 0.84 | 0.57 | 0.58 | 0.69 |
| 222216 | Murrindal River @ Basin Road (Buchan) | Snowy River | 0.76 | 0.75 | 0.62 | 0.65 | 0.7 |
| 222217 | Rodger River @ Jacksons Crossing | Snowy River | 0.79 | 0.85 | 0.67 | 0.71 | 0.75 |
| 227203 | Franklin River @ Henwoods Bridge | South Gippsland | 0.84 | 0.79 | 0.61 | 0.67 | 0.73 |
| 227210 | Bruthen Creek @ Carrajung Lower | South Gippsland | 0.82 | 0.83 | 0.63 | 0.68 | 0.74 |
| 227213 | Jack River @ Jack River | South Gippsland | 0.83 | 0.84 | 0.61 | 0.68 | 0.74 |
| 227220 | Greig Creek @ Mumfords | South Gippsland | 0.84 | 0.85 | 0.67 | 0.68 | 0.76 |
| 227223 | Macks Creek @ Richards | South Gippsland | 0.84 | 0.81 | 0.68 | 0.7 | 0.76 |
| 227225 | Tarra River @ Fischers | South Gippsland | 0.84 | 0.86 | 0.68 | 0.73 | 0.78 |
| 227226 | Tarwin River East Branch @ Dumbalk North | South Gippsland | 0.85 | 0.79 | 0.52 | 0.62 | 0.69 |
| 227227 | Wilkur Creek @ Leongatha | South Gippsland | 0.84 | 0.74 | 0.45 | 0.6 | 0.64 |
| 227228 | Tarwin River East Branch @ Mirboo | South Gippsland | 0.85 | 0.76 | 0.54 | 0.62 | 0.69 |
| 227236 | Powlett River @ D/S Foster Creek Junction | South Gippsland | 0.9 | 0.8 | 0.42 | 0.6 | 0.67 |
| 227237 | Franklin River @ Toora | South Gippsland | 0.85 | 0.8 | 0.55 | 0.65 | 0.71 |
| 223204 | Nicholson River @ Deptford | Tambo River | 0.78 | 0.8 | 0.71 | 0.73 | 0.75 |
| 223206 | Tambo River @ Bindi | Tambo River | 0.78 | 0.84 | 0.57 | 0.58 | 0.69 |
| 223207 | Timbarra River @ Timbarra | Tambo River | 0.81 | 0.85 | 0.71 | 0.69 | 0.76 |
| 223211 | Haunted Stream @ Stirling | Tambo River | 0.78 | 0.83 | 0.72 | 0.67 | 0.75 |
| 223212 | Timbarra River @ D/S Of Wilkinson Creek | Tambo River | 0.79 | 0.86 | 0.69 | 0.69 | 0.76 |
| 223215 | Haunted Stream @ Hells Gate | Tambo River | 0.77 | 0.86 | 0.71 | 0.73 | 0.77 |
| 225019 | North Cascade Creek @ Thomson Valley Road | Thomson River | 0.8 | 0.77 | 0.7 | 0.62 | 0.64 |
| 225217 | Barkly River @ Glencairn | Thomson River | 0.81 | 0.83 | 0.45 | 0.57 | 0.66 |
| 225218 | Freestone Creek @ Briagalong | Thomson River | 0.73 | 0.78 | 0.64 | 0.68 | 0.71 |
| 225219 | Macalister River @ Glencairn | Thomson River | 0.8 | 0.81 | 0.42 | 0.53 | 0.64 |
| 225230 | Glenmaggie Creek @ The Gorge | Thomson River | 0.75 | 0.8 | 0.67 | 0.71 | 0.73 |

Figure 25: Groundwater hydrographs of three nested bores located in MID west of Sale plotted against daily stream water level of Latrobe River at Rosedale (gauge station no. 226228).

Figure 26: Groundwater hydrographs of a transect of  bores located near Mitchell River west of Bairnsdale plotted against daily stream water level of Mitchell River at Rose (gauge station no. 224203).

Figure 27: Groundwater hydrographs of two nested bores located near Snowy River west of Orbost plotted 
against daily stream water level of Snowy River at Jarrahmond (gauge station no. 222200).

Figure 28: Groundwater hydrographs of three nested bores located in the Macalister Irrigation District north of Sale plotted against daily stream water level of Avon River at Stratford (gauge station no. 225201).

Hydrogeology

The surface and sub-surface geology is diverse and complex in the region.  Outcropping Cretaceous sedimentary bedrocks (Strzelecki Group and Strzelecki Ranges) occur in the uplands in the southwest, while Silurian to Devonian sedimentary, metamorphic and igneous bedrock outcrops in the uplands in the west and north.  The bedrocks underlie a sequence of up to 900 m of Upper Cretaceous to Quaternary marine and terrestrial siliclastic sediments, volcanics, coal measures and carbonate sediments in the Gippsland Basin which extends more than 80 km offshore.  Structurally, the Gippsland Basin consists of fault-bounded uplifted and down-thrown blocks, and monoclines and anticlines which produced a series of depressions separated by bedrock “highs”.  These diverse geological units and complex structures result in a large number of aquifer systems across the model domain. Hydrogeology in the model domain was investigated and described in detail in several previous studies (Schaeffer 2008; GHD, 2008a, 2010b, 2012a, 2012b).  Table 7 summaries the hydrostratigraphic units defined across the modelling area. The main aquifer systems in the modelling area are summarised below.

2.9.1	Quaternary alluvium and Haunted Hill Gravel

The Quaternary alluvial units are relatively thin but extensive across the plains of the Gippsland Basin.  Sand and gravel beds form largely unconfined aquifers, usually from 5 to 15 m thick.  Hydraulic conductivity ranges from 0.1 to more than 50 m/day (Schaeffer, 2008; GHD, 2008a, 2010a).  Specific yield ranges from 0.04 to 0.25 in the unconfined aquifers while specific storage ranges from 1  10–5 to 1  10–4 m–1 in the semi-confined or confined condition (Walker & Mollica, 1990; Schaeffer, 2008; GHD, 2008a, 2010a).  The Haunted Hill Gravel is composed of sands, gravels and clays and conformably overlies the older Tertiary units across most of the Gippsland Basin and the East Gippsland coastal plain.  Records from the Haunted Hill Formation in the Bairnsdale area show hydraulic conductivity of up to 100 m/day and a specific yield of 0.1 (Schaeffer, 2008). The limited lateral extent of many of these units and usually thin vertical extent means that transmissivity and aquifer yields are variable but generally low (i.e. less than 5 L/s). These upper units are not as heavily utilised as the deeper aquifers due to their lower yield potential.

2.9.2	Boisdale Formation Aquifer

The Boisdale Formation extends across much of the Gippsland Basin, except between Stratford and Bairnsdale north of the Princes Highway. The formation is an Upper Tertiary terrestrial sequence that primarily consists of sands, silts and clays. It can be subdivided into an upper clay unit (the Nuntin Clay Member), and a lower sand unit (the Wurruk Sand Member), the latter forming the Boisdale Formation aquifer. SKM (2006) mapped (and subsequently modelled) the aquifer as extending and narrowing to as far west as Traralgon. The lithological interpretations of Schaeffer (2008) indicate only minor, patchy Boisdale Formation Aquifer towards Traralgon. The aquifer thins to absence in the Lakes Entrance area. The Boisdale Formation also occurs in the onshore part of the Seaspray Depression, extending from the Lake Wellington area in the northeast to near Yarram in the southwest.

The Boisdale Formation Aquifer is confined to semiconfined by either the Nuntin Clay Member (i.e. upper clay), or by clays within the overlying Haunted Hill Formation (Leonard, 2003).   The aquifer is highly permeable, particularly around Sale, where it is used as a source for urban water supply.  To the south and east of Sale, the Boisdale Formation merges laterally into calcareous sands of the Jemmys Point Formation. This transition is thought to occur close to the existing shoreline in the Seaspray Depression, but in this area the Boisdale Formation is known to interfinger with the upper parts of the Jemmys Point Formation (SKM, 2001).  The Boisdale Formation Aquifer is considered to be in direct hydraulic connection with the Morwell Formation Aquifer System (MFAS) (described in Section 2.9.6) further west in the Latrobe Valley via the Balook Formation.

Walker and Mollica (1990) reported a hydraulic conductivity for the Boisdale Formation Aquifer (at Sale, just inside the eastern boundary of West Gippsland CMA region) of 24 m/day (Schaeffer (2008) suggested up to 30 m/d), and a storage coefficient of 1  10–4 (Schaeffer also suggested Ss up to 1  10–2 m–1 depending on the degree of confinement).  SKM (2006) report transmissivities ranging from 470 to 1300 m2/day, and storage coefficients ranging from 1  10–4 to 5  10–4.  Yields of 5–20 L/s are reported for the Boisdale Formation.

SKM (2006) reported that the majority of recharge occurs along the northern extent of the aquifer, beneath the floodplains of the Thomson and Macalister Rivers west of Maffra, and south of Sale, along the northern flank of the Baragwanath Anticline. Walker and Mollica (1990) suggested that a significant proportion of recharge to the Boisdale Formation Aquifer may be derived from rainfall and stream infiltration via the Haunted Hill and Quaternary Formations where the aquifer subcrops at shallow depths, near its margins. Groundwater flow is from the west towards offshore areas in the southeast. Groundwater quality is less than 500 mg/L total dissolved solids (TDS) around Sale and in the north and west, increasing to 500–1500 mg/L TDS in the south and southeast.  Around Lakes Entrance it increases to 1500–3000 mg/L TDS.

2.9.3	Jemmys Point Formation Aquifer

The Jemmys Point Formation is an Upper Tertiary aged marine sequence primarily overlain by, and distal to, the Boisdale Formation (Walker and Mollica, 1990).  It is thought to have coarse-grained facies in its upper horizon, and a marlier lower horizon with properties similar to those underlying marine carbonates of the Tambo River Formation, Bairnsdale Limestone Member, Wuk Wuk Marl and Gippsland Limestone.  Lithology between the Boisdale sand aquifer (LSU), sandy facies of Jemmys Point Formation and the Lake Welling Formation is very similar and good hydraulic connection between them is highly likely.  GHD (2010b) considers the Boisdale sand aquifer (LSU), sandy facies of Jemmys Point Formation and the Lake Welling Formation as a single continuous aquifer unit.

The sandier facies of the Jemmy’s Point Formation are believed to have similar hydraulic properties to its lateral equivalent Boisdale Formation (Walker and Mollica, 1990) while the clay-rich horizons are a much poorer aquifer, with properties similar to those of the Gippsland Limestone and other marine carbonates in this area.  Hydraulic conductivity in these marls and limestones is thought to be about an order of magnitude lower (2–10 m/d) with storage coefficients ranging from 10–5 to 10–6 (Schaeffer, 2008; GHD, 2008a, 2010a).

2.9.4	Balook Formation Aquifer

The Balook Formation is a Mid-Cenozoic aged unit and represents a transition zone in depositional environment, from a largely terrestrial sequence in the west and north, to a marine sequence in the south and east. Brumley et al. (1981) and later publications discuss the importance of the Balook Formation barrier sand sequence to regional groundwater flow. This is due to the formation being a largely sandy deposit that crosses much of the vertical sedimentary pile, and therefore provides hydraulic connection between deep aquifers (Traralgon Formation Aquifer System (TFAS) and MFAS) and shallow unconfined to semiconfined aquifers (Haunted Hill Formation, Boisdale) over a large section of the Gippsland Basin.  The Balook Formation also provides hydraulic connection between the MFAS and its marine equivalents of the Seaspray Group (Gippsland Limestone and Lakes Entrance Formations), although the average hydraulic conductivity of these lateral equivalents is thought to be relatively low (Schaeffer, 2008), and therefore groundwater could be expected to preferentially migrate upwards via the Balook Formation.

Brumley et al. (1981) reported an estimated transmissivity for the Balook Formation of 1000 m2/day based upon (unreferenced) relatively low hydraulic conductivities of 2 to 7 m/day, and a 500 m average formation thickness.  Higher hydraulic conductivities (up to 57 m/day) were reported by Schaeffer (2008) and Walker and Mollica (1990).  Specific yield ranges from 0.04 to 0.1 in the aquifers under unconfined condition while specific storage ranges from 1  10–6 to 1  10–3 m–1 in the aquifers under semi-confined or confined condition (Walker and Mollica, 1990; Schaeffer, 2008; GHD, 2010b).

Groundwater quality mapping (DCNR, 1995) indicates groundwater within the Balook Formation (and associated formations) is typically 500–1000 mg/L TDS.

2.9.5 	Childers Formation / Thorpdale Volcanics / Yarragon Formation Aquifers

These units are Early Cenozoic in age. The, Lower Morwell Formation (M2) and Childers Formation Aquifer lateral equivalent (see Section 2.9.6 below) is found in the Moe Basin and the Tarwin Basin respectively.  In the Tarwin Basin, it is considered an insignificant aquifer, having relatively low hydraulic conductivity, and limited thickness (Pratt, 1985).  Pratt (1985) also reported that it is essentially in direct hydraulic connection with the Thorpdale Volcanics, with minor locally confined conditions.  For this reason the Childers Formation is lumped together with the Thorpdale Volcanics in GHD’s Gippsland ecoMarket models (GHD, 2010a, 2010b).

The Thorpdale Volcanics form the primary aquifer in the Tarwin Basin, and are considered to be in direct hydraulic connection with the overlying Haunted Hill Formation (Pratt, 1985).  A hydraulic conductivity for the Thorpdale Volcanics of 0.03 to 1 m/day and a specific storage of 1  10–6 to 1  10–4 m–1 were reported (Schaeffer, 2008; GHD, 2010b); these two units form a largely unconfined aquifer.  Based on limited data, Pratt (1985) reported that most groundwater flow in the Tarwin Basin is from the west and northwest to the southeast, where the Koorooman Fault largely forces groundwater to discharge upward into the Tarwin River as baseflow.  Pratt (1985) also reported that recharge largely occurs via rainfall infiltration to outcropping Haunted Hills Formation, Thorpdale Volcanics, and Quaternary units.  Pratt (1985) estimated around 10,000ML/year total annual recharge (6% of mean annual rainfall), using the throughflow method, and suggested that much of this discharges to the Tarwin River.  An urban water supply has recently been developed in the Volcanics for Leongatha.

In the Tarwin Basin, groundwater quality in the Childers Formation / Thorpdale Volcanics Aquifer ranges from 200 mg/L TDS to 2,000 mg/L TDS, and tends to decrease at greater aquifer depths (Pratt, 1985).

In the Moe Basin, the Yarragon Formation, a lateral equivalent of the Latrobe Valley’s Yallourn Formation overlies the Thorpdale Volcanics.  The Yarragon Formation forms an aquifer towards its base, although the waterbearing sands and gravels are intermittent in extent, primarily found in the south of the basin (GHD, 2010a; Brumley and Holdgate, 1983).  A lateral facies change from sandy to clayey sediments occurs around the Moe monocline, approximately 5 km west of Moe (Brumley and Holdgate, 1983). Brumley and Holdgate (1983) state that discharge occurs from the highly confined Yarragon Formation by upwards leakage through the confining beds in the east of the basin, up into the watertable aquifer, and presumably the Latrobe River.  Groundwater flows from the north-western and southern basin margins towards the east, where basement topography rises and narrows (around Moe, on the Haunted Hills Block). This, in combination with the lateral facies change, restricts groundwater flow eastwards out of the basin and into the Latrobe Valley (Brumley and Holdgate, 1983).

The Thorpdale Volcanics also forms a significant confined to semiconfined aquifer, with greater reported transmissivity than the Yarragon and Childers Formations (GHD, 2010a). The volcanics form a major aquifer in the west of the basin, where it is at lesser depths, and outcropping over large areas (Brumley and Holdgate, 1983).

The Childers Formation forms a minor confined aquifer in the Moe Basin, primarily due to its limited extent, thickness, and depth of burial (Brumley and Holdgate, 1983).

Walker and Mollica (1990) considered that recharge occurs primarily via direct rainfall and stream infiltration into outcropping units along the southern basin margin, and in the northwest. They also suggested that the majority of discharge is to streams and rivers via upward vertical leakage, with none into the Latrobe Valley via subsurface flow. Recharge was estimated to be around 10,000 ML/year in the Moe Basin. In the Moe Basin, groundwater quality varies from less than 500 mg/L TDS to 1,000 mg/L TDS (DCNR, 1995).

2.9.6	Morwell Formation Aquifer System (MFAS)

Within the MFAS, there are two regionally extensive aquifers, the M1B Aquifer, and the M2 (or M2C) Aquifer. Corresponding coal units (the M1B Coal and M2 Coals) confine both aquifers.  The M2 Aquifer (and its lateral equivalents) extends from the Moe Basin in the west (where it is termed the Childers Formation), to around Rosedale, where the Morwell Formation (including the M1B) merges into the Balook Formation barrier sand sequence.  However, the M2 Aquifer equivalent effectively passes beneath, but is in direct connection with, the Balook Formation, extending further east to the Bairnsdale – Lakes Entrance area, where it is named the Seaspray Sands (Schaeffer, 2008). The M2 Aquifer equivalents also extend offshore.  The Childers Formation (M2 equivalent) occurs in the Tarwin Basin (in the southwest).  The portions of the MFAS overlying the M2 Aquifer laterally transition into relatively poor aquifers on the east side of the Balook Formation, namely into the Lakes Entrance Limestone, Gippsland Limestone and Wuk Wuk Marl.

Schaeffer (2008) reported a hydraulic conductivity of 22 m/day and a storage coefficient of 1  10–3 for the M1B Aquifer, and 57 m/day and 3  10–4 for the M2 Aquifer (although these are based upon pumping tests at the Hazelwood Mine, outside the East Gippsland CMA region). Tests conducted at Loy Yang Mine, also in the West Gippsland CMA region, indicate an M2 Aquifer hydraulic conductivity of 2 to 7 m/day, and storage coefficient of 10–6 to 10–4 (Thatcher, 1976), but Schaeffer (2008) notes that the M2 is less sandy at Loy Yang compared to Hazelwood, and grades back into sands and gravels to the east of Loy Yang.

Groundwater salinity within the MFAS varies from less than 500 mg/L TDS to 1,000 mg/L TDS in the north and west, becoming increasingly saline (up to 3500 mg/L TDS) towards coastal areas in the south and east (DCNR, 1995). The presence of fresher groundwater in the north and west indicates that these are the primary areas of recharge, probably via rainfall infiltration into subcropping MFAS along the northern margins of the basin, and surrounding the Strzelecki Ranges in the west.  Some of this recharge is also likely to be derived from infiltration of stream leakage and overland flow from the surrounding hills.  There appears to be more recharge along the northern margins of the basin into the MFAS compared to the TFAS, based upon groundwater salinity mapping (DCNR, 1995).

Groundwater flow within the MFAS is in an easterly and south-easterly direction, from the recharge areas of the west and north, towards offshore areas.  As for the TFAS, discharge is thought to ultimately occur in offshore areas via the oil and gas production wells, and/or via vertical seepage over large areas, or directly into Bass Canyon, where the Latrobe Group are thought to outcrop (Holdgate and Gallagher, 2003; Nahm, 2002).  The MFAS is dewatered in the Latrobe Valley coalmines, and used as a source for cooling water for the power stations, which locally influence flow directions.

2.9.7	Traralgon Formation Aquifer System (TFAS)

Within the Latrobe Valley, the TFAS has been subdivided into two confining beds and two aquifers: the T1 Coal, overlying and confining the T1 Aquifer, and the T2 Coal, which overlies and confines the T2 Aquifer.  Equivalents of these units are regionally extensive in the Gippsland Basin, however the T2 Coal is limited in thickness and extent, and the T1 Coal therefore forms the primary confining unit for the TFAS.  The TFAS is up to 100 m thick in the Latrobe Valley, and thickens in an offshore direction to more than 2000 m.

Aquifer tests throughout the onshore Gippsland Basin indicate hydraulic conductivity for the TFAS of 7.5 to 88 m/day, and storage coefficients of 2.5  10–5 to 4  10–4 (Brumley et al, 1981; SKM 1999).

Groundwater salinity within the TFAS varies from less than 500 mg/L TDS to 1000 mg/L TDS, becoming increasingly saline towards the east (DCNR, 1995).  The comparatively fresh groundwater quality in the west indicates that recharge to the TFAS primarily occurs in this area, around the base of the Strzelecki Ranges, and along the Baragwanath Anticline, the subsurface continuation of the Strzelecki Range, which extends towards Sale. Evidence for this inference also includes mapped TFAS outcrop (including the Carrajung Volcanics) around the flanks of the Strzelecki Range (Douglas et al., 1971).  It is thought that recharge is derived directly from infiltrating rainfall and from leakage from streams crossing the TFAS where it outcrops or subcrops (Brumley et al., 1981; Walker and Mollica, 1990). Greater recharge is made possible to the TFAS over the Baragwanath Anticline compared to areas where the TFAS is more deeply buried within the sedimentary pile because it is brought closer to the land surface by basement topography and subcrops at shallow depth.

Groundwater flow is generally from the recharge areas in the west, offshore to the east and southeast.  CSIRO report that while the original offshore discharge was predominantly  through vertical leakage and from the continental shelf, since 1995 discharge is predominantly through extraction (Hatton et al, 2004) or directly into Bass Canyon, where the Latrobe Group are thought to outcrop (Holdgate and Gallagher, 2003; Nahm, 2002).  Localised flow directions vary, particularly around the Latrobe Valley coalmines where the TFAS is dewatered at Loy Yang. Flow directions are also complicated by features such as the Baragwanath Anticline and Rosedale Monocline/Fault, which direct flow from the west in an easterly direction until the anticline plunges to sufficient depth to allow flow towards offshore areas in the southeast (Schaeffer, 2008).  Schaeffer (2008) and Underschultz et al. (2006) state that faults in the Gippsland Basin primarily act as flow barriers rather than conduits.

2.9.8	Bedrock Aquifers

While not transmitting great quantities of water in Gippsland, the Palaeozoic and Strzelecki bedrock geology is known to maintain baseflows in the many rivers and streams in the upland parts of the region.  Most of the outcropping bedrock comprises Siluro-Devonian metasediments, Cretaceous sediments, and Devonian granites.  Groundwater flow through the bedrock aquifers is typically via fractures rather than pore space, although the distinction between flow media becomes blurred in highly weathered bedrock.  Hydraulic conductivities for the bedrock aquifers are typically low ranging from less than 0.0001 to 1 m/day and storage coefficients range from less than 3  10–6 to 5  10–4 (Batu, 1998; Shugg & Harris, 1975; Szabo, 1979; Leonard, 2006). Bore yields are typically less than 1 L/s but are highly variable over short distances, and there is similarly wide variation in groundwater salinity (Heislers, 1993). Yields are typically lower in granitic bedrock (less than 0.5 L/s) and as such these aquifers are typically only used for stock and domestic supply.

Current groundwater development

Groundwater resources in the tertiary formations of the Gippsland Basin have been developed since 1970, firstly in the offshore basin with the exploitation of oil and gas resources, and in the Latrobe Valley to allow the extensive coal mining associated with power station operations.  This has resulted in extensive regional depressurisation of the MFAS and the TFAS, and the local dewatering of these aquifer systems around 
the mines.

The system is consequently far from equilibrated and unlikely to establish equilibrium in the foreseeable future.  In the shallow system, water levels are relatively stable in the areas east of Sale and the 
Gippsland Lakes.

Dominant aquifers

The dominant aquifers in the Gippsland region are summarised below.

Quaternary deposits. The quaternary aquifers primarily consist of coarse sand and gravel along the river valleys and floodplains of the major rivers and dune deposits in coastal areas.  They are generally unconfined.  Permeability ranges from low to high.

Upper Cenozoic aquifer.  This aquifer is primarily comprised of various fluvial deposits (Wurruk Sand) in the Boisdale Formation and some unnamed Tertiary sands.  It occurs in many parts of the floodplain in West Gippsland.  It is generally greater than 50 m thick with highly variable permeability.  It is generally semiconfined or unconfined depending on the permeability of overlying materials.

Upper Mid-Cenozoic aquifer. This aquifer primarily consists of sands and gravel in the Balook Formation, Morwell Formation and Alberton Formation. Relatively permeable members of Hazelwood Formation, Gippsland Limestone and Lakes Entrance Formation also form parts of this aquifer.  The aquifer is buried up to 1000 m and is dominantly semiconfined to confined.

Lower Mid-Cenozoic aquifer. Seaspray Sand forms the main part of this aquifer.  It mainly occurs around the Morwell area in West Gippsland and is generally confined with moderate permeability.

Lower Cenozoic aquifer. This aquifer primarily consists of the Childers Formation, M2, Traralgon Formation and Burong formations.  These formations mainly occur around the Latrobe Valley.  The aquifer is mainly semiconfined to confined with moderate permeability.

Tertiary Basalts. They include Thorpdale Volcanics, Carrajung Volcanics and Older Volcanics. They occur around the Warragul, Thorpdale and Leongatha areas.  Their permeability ranges from low to moderate depending on the density of fracturing.  This aquifer is generally confined with little outcrop.

Palaeozoic metasediments and Strzelecki Group.  Fractured rock aquifers in fresh rock with variable permeability depending on the density of fracturing.

Additionally, there are a number of well-defined aquitards in the region, specifically:

Nuntin Clay in the Boisdale Formation.  This occurs beneath the Quaternary deposits and covers the majority of the floodplains.

Clay/coal seams. These aquitards occur in various formations (e.g. Yallourn, Yarragon, Morwell, Traralgon and Burong formations) in the northwest of the Gippsland Basin.

Upper Gippsland Limestone. This occurs in the east and centre of the Gippsland Basin.

Schematic conceptualisation

A schematic conceptual understanding of the water balance and hydrogeology within the Gippsland region is presented in Figure 29. The important water balance features considered within the model include:

groundwater recharge

groundwater inflow and outflow

groundwater abstraction

groundwater evaporation

groundwater – surface water interaction.

Figure 29: Stylised conceptualisation of the dominant water balance components within the Gippsland region.

The regional groundwater flow conceptualisation considers the Tertiary aquifers in the Gippsland region as behaving as three distinct flow systems underlain by the regional basement aquifer of the Strzelecki Group.  The most significant of the Tertiary aquifers are the Moe sub-basin, Latrobe Valley, Seaspray depression (in which the catchments of the Thomson, Latrobe and Mitchell rivers occur) and Southern Terrace (which includes the catchment of the Tarra River).

Flow in the northern terrace and Seaspray depression aquifers is dominantly west to east, with limited lateral interconnectivity due to both faulting and the Baragwanath Anticline (see Figure 30).  Flow in the southern terrace is topographically driven off the southern slopes of the Strzelecki ranges.

Hatton et al (2004) noted the widespread declines in aquifer pressures in the Seaspray depression ‘are clearly associated to some large but geographically variable degree with offshore oil and gas production’. These declines and the cone of depression associated with the Loy Yang and Hazelwood mine dewatering are distinctly disconnected responses.  The other basins are the Tarwin Basin (included in the model domain) which is effectively a disconnected basin flowing south, and the Westernport Basin flowing westerly towards Westernport Bay (and is not included in the model domain).

The conceptual model considers flows in the exposed meta-sediments in the high-relief areas of the Strzelecki and Eastern Highlands are primarily local flow systems discharging to nearby streams with some recharge to the basement rocks.  Regional scale flows occur in deeper tertiary units where recharge primarily occurs on the margins of basins where units sub-crop or outcrop, dominated by rainfall (GHD, 2010d).

Coal, silt and clay layers are effective aquitards over the region.  However, the complex depositional history results in regional scale connections between aquifers within coal seams.  The Balook Formation in particular provides a significant connection between the Morwell and Traralgon formation aquifers and the Boisdale Aquifer.

The aquifers of the Latrobe Formation contain good quality water for many kilometres offshore indicating this groundwater is formation water associated with deposition and a result of long term flow from onshore.  However near the seaward margin much higher salinities are encountered which are considered to pre-date the onset of oil and gas production and represent formation water.

Figure 30: Schematic flow conceptualisation of the Seaspray depression showing the Baragwanath anticline 
and typical water quality variation offshore (source: DNRE unpublished data).

Numerical model design

Modelling approach

The modelling tasks involved using an unsaturated biophysical catchment model to generate daily evapotranspiration demands and recharge estimates.  This data was then used as input into a distributed groundwater model.  As such the simulation procedure was uncoupled.  This approach was considered to better represent farming systems and farm practices due to the construct of the unsaturated biophysical catchment model, functions of which are not commonly available in commercial groundwater models.  Importantly this approach enables (1) finer resolution representation of farm management units to be captured and integrated into coarser resolution groundwater models, and (2) a holistic water balance that describes both soil-water-plant interactions and groundwater dynamics.  In addition to the unsaturated and groundwater modelling frameworks, customised software was also developed to assist in the pre- and post-processing of input data and simulation outputs.  Each modelling approach is described below.

## 3.1.1 	Unsaturated modelling software

Modelling of the unsaturated zone was undertaken using the Catchment Analysis Tool (CAT) (Beverly et al., 2005, Beverley, 2009).  The CAT model utilises a suite of farming system models linked within a catchment framework with allowance for landscape connectivity and connection to a distributed, multi-layered groundwater model.  The farm-scale models account for position in the landscape (topography, soil type, aspect and slope), climate, land use and land management and simulates water balance, nutrient transport and production on a daily time step.

## 3.1.2 	Groundwater modelling software

Two groundwater modelling frameworks have been deployed, namely:

Modflow-2005:  A uniform rectilinear grid of the model domain was constructed and incorporated into the Modflow-2005 finite difference modelling software.  This model has provided predictions at a scale consistent with previous regional scale assessments.

Groundwater Vistas:  The commercial software version of Modflow-2005 embedded in Groundwater Vistas was used to independently check results derived using the source code version of the Modflow-2005 finite difference modelling software. This was deemed necessary as Groundwater Vistas has implemented modifications to the solver algorithms enabling convergence to be met based on outer iterations only.

Both models were developed using the same spatial and temporal data inputs.

## 3.1.3 	Customised software

Numerous software programs were developed to automate data manipulation, undertake quality assurance, interrogate model input data sets and process simulation outputs.

Data sources

The model layer structure that define the surfaces that separate overlying hydrogeological units, and the initial attribution of these layers, were based on a combination of (1) stratigraphic mapping and interpretation (see Appendix G), (2) the Victorian Aquifer Framework (VAF) (SKM, 2011b; GHD, 2012a), (3) the DPI Gippsland Basin groundwater model (Beverly et al., 2012), (4) work of Schaeffer (2008), and (5) published maps and cross-sections as reported in previous studies including the ecoMarket model reports and reviews.

Time series calibration information was sourced from on-line databases as summarised in Table 6. An assessment of the confidence of each key data set is also included based on criteria that considered:

expertise of those groups responsible for data collection and collation

processes for consistency of field measurement

processes used to quality assure data

data management and custodianship

degree of acceptance of data robustness by other groups and end-users.

Table 6: Sources of calibration data used.

| Time-series data | Source | Confidence in data |
| --- | --- | --- |
| Groundwater level | Victorian Water Measurement Information System (http://data.water.vic.gov.au/monitoring.htm) | High |
| Surface water level | Victorian Water Measurement Information System (http://data.water.vic.gov.au/monitoring.htm) | High |
| Baseflow | SKM reports and derived estimates from Victorian Water Measurement Information System (http://data.water.vic.gov.au/monitoring.htm) | Moderate |
| Flood mapping | Department of Environmental and Primary Industries |  |
| Groundwater extraction (onshore) | Southern-Rural Water, Water, Department of Environmental and Primary Industries | Moderate to High; some data has been inferred and some actual |
| Groundwater extraction (offshore) | Volume equivalent groundwater extractions associated with offshore oil and gas operations were sourced from Hatton, et al (2004) and Varma, et al (2012) | Moderate |
| Mine floor elevations | Department of Environmental and Primary Industries | Moderate |
| Climatic data | Bureau of Meteorology (http://www.bom.gov.au/climate/) | High |

Groundwater model layer extents

The model consists of 30 layers and includes coal seams similar to those represented in the IRM model in addition to a basement layer that extends to the upland regions of the Gippsland region.  Key refinements to the previous DEPI Gippsland Basin groundwater model include (1) new interpreted data layers that extend to the catchment boundaries, (2) finer grid resolution, (3) the incorporation of offshore aquifer stratigraphic information as developed by CSIRO and Geological Survey of Victoria and (4) incorporation of a basement layer representing the upland outcropped regions of the Gippsland region.

All dominant coal seams were specified as individual layers.

Table 7 summarises the grouping of coal and interseams into modelled layers.  Model layers 23 to 29 represent the Strzelecki Group.  Rather than allocating this as one layer, it was necessary to split this geological unit into six model layers to accommodate the likely scenarios which require depressurisation within parts of the formation.

Table 7: Groundwater model layers.

| Model layer | VAF name | Coal name | Comment |
| --- | --- | --- | --- |
| 1 |  |  | Marine water thickness |
| 2 | 101 |  | Quaternary |
| 3 | 102 |  | Haunted Hill Formation |
| 4 | 103 |  | Nuntin clay |
| 5 | 105 |  | Boisdale Formation |
| 6 | 106 |  | Jemmy’s Point Formation and upper Hazelwood Formation |
| 7 | 106 | Yallourn Coal Seam | Y, Y1a, Y1b, Y2, Y1; y_all |
| 8 | 106 | Yallourn Aquifer & interseam | Hazelwood Formation; y_all floor & M1a_all top |
| 9 | 107 | Lower M2 interseam, | Balook Formation Tambo River, Wuk Wuk Marl, Gippsland Limestone |
| 10 |  | M1A coal | Yarragon Formation, M10, M1a, M1b2, ML, M12; M1a_all |
| 11 |  | Morwell 1A interseam/aquifer | M1a_all_floor and M1b_top |
| 12 |  | Morwell 1B coal | M1b, M1b1, M1b2, ML, M12 |
| 13 |  | Morwell 1B interseam | Floor M1b_all & M2_all top |
| 14 |  | Morwell 2 | M2, M2A, M2B coal; M2_all |
| 15 | 108 |  | Lakes Entrance Formation |
| 16 | 109 |  | M2c aquifer/Seaspray Sands |
| 17 | 112 |  | Thorpdale volcanics |
| 18 | 111 |  | Upper Latrobe Group |
| 19 | 111 | T1 coal | TP, T1, TRU, TRM, TRL |
| 20 | 111 | T1 interseam | Floor T1_all & Top T2_all |
| 21 | 111 | T2 coal |  |
| 22 | 111 | T2 interseam | Lower Latrobe Group; T2_all floor |
| 23 |  |  | Strzelecki top 500 m |
| 24 |  |  | Strzelecki 500–1000 m |
| 25 |  |  | Strzelecki 1–2 km |
| 26 |  |  | Strzelecki 2–3 km |
| 27 |  |  | Strzelecki 3–4 km |
| 28 |  |  | Strzelecki 4–6 km |
| 29 |  |  | Strzelecki >6 km |
| 30 | 114 |  | Palaeozoic basement |

Groundwater model layer boundary conditions

Groundwater boundary conditions are features which influence groundwater flow.  In a groundwater model context, boundary conditions are constraints imposed onto the model to reflect an area which is influenced by external features (such as wells, rivers, no-flow barriers, etc.).  Groundwater flow boundary conditions considered in this study are listed below.

No flow boundary – represents locations where groundwater does not flow and/or the aquifer is absent; such features include groundwater divides (specified flow boundary type).

Well boundary – represents locations where fluxes are applied to the model (on a layer-by-layer basis).  They are used to represent groundwater extraction from stock, domestic, industrial bores and from groundwater pumping in offshore oil and gas fields.

River boundary – represents a head dependent boundary condition where groundwater can either recharge or discharge into/from the model based upon a specified head elevation, the model-predicted head in neighbouring cells and a specified boundary conductance term.  Rivers in this model have been grouped as either major or minor rivers.  Major rivers represent primary tributaries and were assigned a width and depth of 20 metres with a stage of 4 metres from the base.  Minor rivers were considered as significant tributaries and assigned a width and depth of 10 metres and a stage of 2 metres from the base.  In the absence of river bed elevation data, the base elevation was assumed to be surface elevation less 5 metres for major rivers and surface elevation less 2 metres for minor rivers.

Drain boundary – represents a head dependent boundary condition where water is removed from the model depending on the specified head elevation, the predicted head in neighbouring cells and the specified boundary conductance term.

Constant head boundary (time constant specified head) – represents flows into or out of the 
model domain where groundwater connects or interacts with features (and the ocean) outside the 
model domain.

Appendix F illustrates the locations of nominated boundary conditions for each modelled layer.

Groundwater model layer parameter attribution

Parameter ranges presented in Table 8 are sourced from literature and previous numerical modelling and show that there is considerable variability in both the hydraulic conductivity and storage estimates.  Initial model parameter attribution is also summarised in Table 8, this attribution is based upon the perceived likely value of the aquifer based upon the pre-existing studies cited.  Initial model parameters are based on the average aquifer values, and by definition within the range cited, for the particular hydrogeological unit that the layer represents.

The depositional history of the basin is dominated by fluvial, deltaic, marginal marine and open marine environments (Schaeffer, 2008, after others).  Typically vertical hydraulic conductivity is approximated as an order of magnitude less than lateral conductivity, and as an initial calibration value, Kz values were set to 0.1 of the Kxy value, and the final, calibrated, value of this ratio is provided in Table 17.

Table 8: Values of specific yield (Sy), specific storage (Ss) and lateral hydraulic conductivity (Kxy) assigned to each modelled layer 
in the Gippsland model.

| Model layer | VAF no. | Coal name | Comment | VAF HGU | Layers (Schaeffer 2008) | Aquifer type1 | Ss (m–1) 2,3 | Sy2,3 | Kxy (m/d) 2,3 | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Marine water thickness | N/A | N/A | N/A | (1.0 X 10-5) | (1.0) | (100) | Initial values in this study |
| 2 | 101 |  | Quaternary | Various Aeolian deposits, various fluvial, lacustrine, alluvial and colluvial sediments | 1 | UC | 2.5 X 10-5 | 0.15 | 0.01 to 10.26 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.02 to 0.05 | 0.04 to 0.08 | 0.1 to 1 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 1.0 X 10-4 | 0.04 to 0.25 | 2 to 50 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | N/A | 0.001 to 0.05 | 1.0 X 10-6 to 100 | Dahlhaus et al. (2004) |
|  |  |  |  |  |  |  | 2.4 X 10-6 | N/A | 59 | Mollica (1991) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.07) | (2.0) | Initial values in this study |
| 3 | 102 |  | Haunted Hill Formation | Haunted Hill Formation, Eagle Point Sand | 1 | UC | 1.0 X 10-5 | 0.1 to 0.15 | 0.01 to 10.26 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.02 to 0.05 | 0.04 to 0.08 | 1 to 10 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 2 to2.01 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | N/A | 0.001 to 0.05 | 1.0 X 10-6 to 100 | Dahlhaus et al. (2004) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.1) | (2.01) | Initial values in this study |
| 4 | 103 |  | Nuntin clay | Boisdale Fm (Nuntin Clay), Jemmys Point Fm Sale Grp | 1 | C/UC | 1.0 X 10-6 to 1.0 X 10-5 | 0.005 to 0.1 | 0 to 0.5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.001 | 0.02 | 0.2 to 0.5 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.1 to 0.23 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.04) | (0.23) | Initial values in this study |
| 5 | 105 |  | Boisdale Formation | Boisdale Fm (Wurruk Sand), Jemmys Point Fm, Unnamed Tertiary Sands, Gravels and Clays | 1 | C/UC | 1.0 X 10-5 to 1.0 X 10-3 | 0.1 to 0.2 | 1 to 30 | Schaeffer (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 5.31 | Nahm (1977) |
|  |  |  |  |  |  |  | N/A | N/A | 6.5 | Nahm & Reid (1979a) |
|  |  |  |  |  |  |  | N/A | 0.1 | 0.1 to 1 | SKM (1999) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | N/A | 5 to 24 | Walker and Mollica (1990) |
|  |  |  |  |  |  |  | 0.001 | 0.02 | 15 to 25 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 2 to 12.38 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-4 to 5.0 X 10-4 | N/A | 4.7 to 13 | SKM (2006) |
|  |  |  |  |  |  |  | (1.0 X 10-4) | (0.1) | (12.38) | Initial values in this study |
| 6 | 106 |  | Jemmy’s Point Formation and upper Hazelwood Formation | Jemmy’s Point Formation and upper Hazelwood Formation | 1 | C/UC | 1.0 X 10-5 to 5.0 X 10-4 | 0.1 to 0.2 | 0.2 to 13 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.001 | 0.02 | 0.2 to 0.5 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 1.0 X 10-6 to 1.0 X 10-5 | 0.1 | 2 to 10 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | (2.0 X 10-5) | (0.1) | (0.23) | Initial values in this study |
| 7 | 106 | Yallourn Coal Seam | Y, Y1a, Y1b, Y2, Y1; y_all | Yallourn Formation | 2 | C/UC (Aquitard) | 2.5 X 10-5 to 2.1 X 10-4 | 0.001 to 0.05 | 2.0 X 10-6 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 0.005 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | (2.0 X 10-5) | (0.02) | (0.0002) | Initial values in this study |
| 8 | 106 | Yallourn Aquifer & interseam | Hazelwood Formation; all floor & M1a_all top | Hazelwood Formation, Yallourn Formation | 3 | C/UC | 1.0 X 10-6 to 2.5 X 10-5 | 0.05 to 0.1 | 0.2 to 8 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.001 | 0.02 | 0.2 to 0.5 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 2 to 2.44 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.2 to 0.5 | SKM (1999) |
|  |  |  |  |  |  |  | 6.8 X 10-5 | N/A | 8 | Blake (1972) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.1) | (2.44) | Initial values in this study |
| 9 | 107, 108 | Lower M2 interseam | Balook Formation Tambo River, Wuk Wuk Marl, Gippsland Limestone, | Balook Fm, LVG: Yarragon Fm, Alberton Fm, Tambo River Fm, Gippsland Limestone, Middle Lakes Entrance Fm | 5, 9 | C/UC | 1.0 X 10-6 to 1.0 X 10-4 | 0.04-0.06 | 2 to 57 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0008 | 0.05 | 1.5 to 2 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 1 to 7.5 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | N/A | N/A | 10 to 58 | Reid (1985) |
|  |  |  |  |  |  |  | 1.0 X 10-5 | 0.06 | 1 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.5 to 8 | SKM (1999) |
|  |  |  |  |  |  |  | N/A | N/A | 2 to 7 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 1 X 10-5 to 09.0 X 10-4 | N/A | 2 to 30 | Walker & Mollica (1990) |
|  |  |  |  |  |  |  | (5.0 X 10-6) | (0.05) | (3.53) | Initial values in this study |
| 10 |  | M1A coal | Yarragon Formation, M10, M1a, M1b2, ML, M12; M1a_all | Yarragon Formation, Upper Gippsland Limestone | 4 | C/UC (Aquitard) | 1.0 X 10-5 to 1.0 X 10-4 | 0.001 to 0.05 | 2.0 X 10-6 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.0005) | Initial values in this study |
| 11 |  | Morwell 1A interseam/aquifer | M1a_all_floor & M1b_top | Morwell Formation, Middle Gippsland Limestone | 5 | C/UC | 1.0 X 10-6 to 1.0 X 10-4 | 0.05 to 0.1 | 7.7 X 10-5 to 8 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04 to 0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 2 to 2.44 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.28 to 0.5 | SKM (1999) |
|  |  |  |  |  |  |  | 1.2 X 10-5 | N/A | 0.27 | Nahm (1972) |
|  |  |  |  |  |  |  | (5.0 X 10-6) | (0.1) | (2.44) | Initial values in this study |
| 12 |  | Morwell 1B coal | M1b, M1b1, M1b2, ML, M12 | Morwell Formation / Morwell seams, Lower Gippsland Limestone | 6 | C/UC (Aquitard) | 1.0 X 10-6 to 5.0 X 10-4 | 0.001-0.05 | 2.0 X 10-6 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04-0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | N/A | N/A | 0.0001 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | (5.0 X 10-6) | (0.05) | (0.0015) | Initial values in this study |
| 13 |  | Morwell 1B interseam | Floor M1b_all & M2_all top | Morwell Formation / Morwell seams, Upper Lakes Entrance Formation | 7 | C/UC | 9.35 X 10-9 to 1 X 10-4 | 0.06-0.1 | 2.3 X 10-4 to 40.39 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04-0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.97-1 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-6 to 9.4 X 10-9 | N/A | 0.08 to 4.07 | Nahm (1977) |
|  |  |  |  |  |  |  | 0.003 | N/A | 2.5 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 0.001 | N/A | 10.83 | Fraser (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 4.48 to 47.68 | Barton (1971) |
|  |  |  |  |  |  |  | 1.0 X 10-6 to 1.0 X 10-4 | N/A | 2 to 7 | Thatcher (1976) |
|  |  |  |  |  |  |  | 2.5 X 10-5 | 0.06 | 0.2 to 5 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.5 to 8 | SKM (1999) |
|  |  |  |  |  |  |  | N/A | N/A | 6.91 | Golder Brawner (unpubl. data) |
|  |  |  |  |  |  |  | (4.0 X 10-6) | (0.1) | (0.97) | Initial values in this study |
| 14 |  | Morwell 2 | M2, M2A, M2B coal; M2_all | Morwell Formation / Morwell seams / Middle Lakes Entrance Formation | 8, 10 | C/UC (Aquitard) | 8.8 X 10-7 to 9.2 X 10-4 | 0.001 to 0.05 | 1.0 X 10-5 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04 to 0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.1 to 0.42 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow and LeCain (1993) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.42) | Initial values in this study |
| 15 | 108 |  | Lakes Entrance Formation | Lakes Entrance Fm /Morwell Formation | 8, 10, 12 | C/UC (Aquitard) | 3.4 X 10-7 to 1.0 X 10-4 | 0.001 to 0.05 | 1.0 X 10-5 to 0.1 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.0002 | 0.02 | 0.04 to 0.05 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.1 to 0.42 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | N/A | N/A | 0.03 | Thatcher (1976) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.05) | Initial values in this study |
| 16 | 109 |  | M2c aquifer/ Seaspray Sands | LVG: M2C aquifer, Seaspray Sand, Lower Lakes Entrance Fm, Seaspray Sands | 11, 13 | C/UC | 1 X 10-6 to 4.7 X 10-4 | 0.03 to 0.1 | 6.4 X 10-4 to 76.06 | Schaeffer (2008); |
|  |  |  |  |  |  |  | 0.0008 | 0.05 | 1.5 to 2 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.1 to 1.91 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 6.3 X 10-6 to 2.2 X 10-5 | N/A | 2 to 48 | Nahm (1973a, 1973b, 1977) |
|  |  |  |  |  |  |  | 4.0 X 10-4 | N/A | 5.7 to 32.35 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 5.0 X 10-5 to 2.5 X 10-4 | N/A | 0.1 to 1.11 | Thatcher (1976) |
|  |  |  |  |  |  |  | N/A | N/A | 0.22 to 19.38 | Geo-Eng (1993, 1996a, 2001) |
|  |  |  |  |  |  |  | N/A | N/A | 7.49 | Reid (1985) |
|  |  |  |  |  |  |  | N/A | N/A | 23.23 | Barton (1971) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 1.0 X 10-3 | 0.015 to 0.06 | 0.2 to 8 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 0.5 to 8 | SKM (1999) |
|  |  |  |  |  |  |  | 3.0 X 10-4 | N/A | 13.6 | Fraser (1980) |
|  |  |  |  |  |  |  | (3.0 X 10-5) | (0.1) | (1.63) | Initial values in this study |
| 17 | 112 |  | Thorpdale volcanics | Thorpdale Volcanics | 7, 9, 11 | C/UC | 1.0 X 10-6 to 4.7 X 10-4 | 0.015 to 0.1 | 0.03 to 6.4 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.04 | 0.05 | 0.2 to 1 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.51 to 2 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 5.0 X 10-5 to 2.5 X 10-4 | N/A | 0.1 to 1.11 | Thatcher (1976) |
|  |  |  |  |  |  |  | 1.7 X 10-7 to 3.5 X 10-5 | N/A | 7.49 to 23.6 | Reid (1985a; 1985b) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 1.0 X 10-3 | 0.015 to 0.06 | 0.2 to 8 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 8 | SKM (1999) |
|  |  |  |  |  |  |  | 6.3 X 10-6 to 2.2 X 10-5 | N/A | 2 to 48 | Nahm & Reid (1979a, 1979b) |
|  |  |  |  |  |  |  | N/A | <0.05 | 0.001 to 100 | Dahlhaus et al. (2004) |
|  |  |  |  |  |  |  | 5.0 X 10-5 | N/A | 6 to 35.6 | Pratt (1985) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.1) | (0.51) | Initial values in this study |
| 18 | 111 |  | Upper Latrobe Group | Childers Fm, M2 / M2C aquifer (when basal aquifer) | 13 | C/UC | 1.0 X 10-5 to 5.6 X 10-2 | 0.03 to 0.1 | 1.6 X 10-3 to 32.35 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.04 | 0.05 | 0.2 to 1 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 0.76 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.5 X 10-6 to 4.0 X 10-5 | N/A | 1 to 13.84 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 1.1 X 10-4 | N/A | 0.25 to 7.3 | Thatcher (1976) |
|  |  |  |  |  |  |  | N/A | N/A | 2.34 to 76.06 | Geo-Eng (1993, 1996b, 2001) |
|  |  |  |  |  |  |  | 1.7 X 10-7 to 3.5 X 10-5 | N/A | 7.49 to 23.6 | Reid (1985a, 1985b) |
|  |  |  |  |  |  |  | 1.0 X 10-4 to 1.0 X 10-3 | 0.015 to 0.06 | 0.8 to 8 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 8 | SKM (1999) |
|  |  |  |  |  |  |  | 3.0 X 10-4 | N/A | 13.6 | Fraser (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 2.0 | Nahm (1974) |
|  |  |  |  |  |  |  | 6.3 X 10-6 to 2.2 X 10-5 | N/A | 2 to 48 | Nahm & Reid (1979a, 1979b) |
|  |  |  |  |  |  |  | 5.0 X 10-5 | N/A | 6 to 35.6 | Pratt (1985) |
|  |  |  |  |  |  |  | (1.0 X 10-4) | (0.1) | (1.63) | Initial values in this study |
| 19 | 111 | T1 coal | TP, T1, TRU, TRM, TRL | Traralgon Fm/ Burong Fm, Carrajung Volcanics | 14 | C/UC (Aquitard) | 2.9 X 10-7 to 6.9 X 10-5 | 0.001 to 0.05 | 1 X 10-6 to 5.0 X 10-3 | Schaeffer (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.0025) | Initial values in this study |
| 20 | 111 | T1 interseam | Floor T1_all & Top T2_all | Traralgon Fm/ Burong Fm | 15 | C/UC | 2.5 X 10-6 to 9.8 X 10-4 | 0.015 to 0.1 | 1.6 X 10-3 to 48.26 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.04 | 0.05 | 1.1 to 2 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 1 to 2.02 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-6 | N/A | 4.07 | Nahm (1977) |
|  |  |  |  |  |  |  | 1.0 X 10-6 to 7.5 X 10-6 | N/A | 1.1 to 6.83 | Nahm & Reid (1979b, 1979c, 1979d, 1979e) |
|  |  |  |  |  |  |  | 2.5 X 10-5 to 4.0 X 10-4 | N/A | 2 to 88 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | N/A | N/A | 6.33 to 48.26 | Geo-Eng (1993, 2001) |
|  |  |  |  |  |  |  | 1.0 X 10-4 to 5.0 X 10-4 | 0.015 to 0.06 | 3 to 10 | Golder Associates (1990) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 8 | SKM (1999) |
|  |  |  |  |  |  |  | (1.0 X 10-4) | (0.1) | (2.02) | Initial values in this study |
| 21 | 111 | T2 coal |  | Traralgon Fm/ Burong Fm | 16 | C/UC (Aquitard) | 1.7 X 10-7 to 2.4 X 10-5 | 0.001 to 0.05 | 1 X 10-6 to 5 X 10-3 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 8.30 X 10-7 to 5 X 10-6 | N/A | 1.1 X 10-6 to 3.56 X10-6 | SKM (2001) |
|  |  |  |  |  |  |  | N/A | N/A | 0.015 to 0.1 | PDA (2006) |
|  |  |  |  |  |  |  | 3.0 X 10-6 to 5.0 X 10-4 | N/A | 0.0005 to 1.36 | USQ (2011) |
|  |  |  |  |  |  |  | 0.0001 | 0.005 | 0.04 | Aquaterra (2008) |
|  |  |  |  |  |  |  | N/A | N/A | 6 X 10-5 to 1.8 X 10-1 | Harlow & LeCain (1993) |
|  |  |  |  |  |  |  | (1.0 X 10-5) | (0.02) | (0.0025) | Initial values in this study |
| 22 | 111 | T2 interseam | Lower Latrobe Group; T2_all floor | Lower Latrobe Group; T2_all floor & Traralgon Fm/Burong Fm, Honeysuckle Gravels, Yarram Fm | 17 | C/UC | 1.1 X 10-7 to 4.9 X 10-5 | 0.015 to 0.1 | 1.6 X 10-7 to 24.65 | Schaeffer (2008) |
|  |  |  |  |  |  |  | 0.04 | 0.05 | 1.1 to 2 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.1 | 1 to 2.02 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | 1.0 X 10-5 to 4.0 X 10-4 | N/A | 4.27 to 88 | Brumley et al. (1981) |
|  |  |  |  |  |  |  | 1.0 X 10-6 | N/A | 4.02 | Thompson (1968) |
|  |  |  |  |  |  |  | 1.0 X 10-4 | 0.1 | 8 | SKM (1999) |
|  |  |  |  |  |  |  | (7.0 X 10-6) | (0.1) | (2.02) | Initial values in this study |
| 23 | 114 |  | Strzelecki 500 m; >0–500 m |  | 18 | C/UC | 4.5 X 10-6 to 1.0 X 10-5 | N/A | 1 X 10-10 to 1.0 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004-0.005 | 0.005-0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | N/A | 0.02 to 0.1 | 1.0 X 10-4 to 1 | Dahlhaus et al. (2004) |
|  |  |  |  |  |  |  | 2.2 X 10-5 | N/A | 0.02 to 1 | Shugg & Harris (1975) |
|  |  |  |  |  |  |  | 3.6 X 10-5 | N/A | 0.3 | Szabo (1979) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.02) | (0.002) | Initial values in this study |
| 24 | 114 |  | Strzelecki 500 m; 500–1000 m |  | 18 | C/UC | 4.5 X 10-6 to 1 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004-0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.02) | (0.002) | Initial values in this study |
| 25 | 114 |  | Strzelecki 1 km; 1000–2000 m |  | 18 | C/UC | 4.5 X 10-6 to 1 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1 X 10-6) | (0.02) | (0.0004) | Initial values in this study |
| 26 | 114 |  | Strzelecki 1 km; 2000–3000 m |  | 18 | C/UC | 4.5 X 10-6 to 1.0 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.02) | (0.0004) | Initial values in this study |
| 27 | 114 |  | Strzelecki 1km; 3000-4000m |  | 18 | C/UC | 4.5 X 10-6 to 1 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3.0 X 10-6 to 5.0 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.02) | (0.0004) | Initial values in this study |
| 28 | 114 |  | Strzelecki 1km; 4000-6000m |  | 18 | C/UC | 4.5 X 10-6 to 1.0 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1.0 X 10-6) | (0.01) | (0.0004) | Initial values in this study |
| 29 |  |  | Strzelecki 4km; >6000m |  | 18 | C/UC | 4.5 X 10-6 to 1.0 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a, 2010b) |
|  |  |  |  |  |  |  | (1 X 10-6) | (0.01) | (0.0004) | Initial values in this study |
| 30 |  |  | Palaeozoic basement 200m thick |  | 18 |  | 4.5 X 10-6 to 1 X 10-5 | N/A | 1 X 10-10 to 1 X 10-5 | Schaeffer (2008) |
|  |  |  |  |  |  |  | <3 X 10-6 to 5 X 10-3 | 0.03 to 0.1 | 2.6 X 10-9 to 2.6 | Domenico & Schwartz (1990); Batu (1998); Todd (1980) |
|  |  |  |  |  |  |  | N/A | N/A | 0.001 to 0.3 | Leonard (2006) |
|  |  |  |  |  |  |  | 0.00004 to 0.005 | 0.005 to 0.02 | 0.001 to 0.008 | GHD (2008a, 2008b) |
|  |  |  |  |  |  |  | 0.01 | 0.02 | 0.0001 to 0.008 | GHD (2010a; 2010b) |
|  |  |  |  |  |  |  | (1 X 10-6) | (0.02) | (0.0004) | Initial values in this study |

1. UC = unconfined, C/UC = confined/unconfined.

2. The ranges of aquifer parameter values were obtained from the publications listed in the table.

3. The aquifer parameter values in bracket are the initial values for the groundwater model in the study. They are primarily sourced from the previously calibrated groundwater models.

4. A Kx to Kz ratio of 10:1 was assumed prior to calibration.

The model has been developed by adopting a regular gridding of 400 metres, totalling 740,768 solution points comprised of 633 rows and 905 columns in 30 layers.  In total the model contains 8,290,157 
active cells.

In recognition that most pseudo-three dimensional groundwater models assume connection between adjacent modelled aquifers, a layered approach was applied for the assignment and attribution where the model layers, which must extend over the full model area, are assigned to an aquifer that may be absent in some areas.

The approach sets a minimum thickness of 2 metres for all layers.  In locations where a cell is assigned to a model layer for an aquifer which is not present, the model cell is set to a minimum thickness of 2 metres and the aquifer attributes are the same as the next underlying active layer. The underlying aquifer thickness is reduced by the 2m thickness.  If several sequential layers are absent, this process is applied to all “missing” aquifers. The application of this approach results in the artificially infilled spatial data sets presented in Figure 31 to 59 inclusive; the resultant thickness of each modelled layer is presented in Appendix D.

| Figure 31: Modelled layer 1 attribution source layer. | Figure 32: Modelled layer 2 attribution source layer. |
| --- | --- |
| Figure 33: Modelled layer 3 attribution source layer. | Figure 34: Modelled layer 4 attribution source layer. |
| Figure 35: Modelled layer 5 attribution source layer. | Figure 36: Modelled layer 6 attribution source layer. |
| Figure 37: Modelled layer 7 attribution source layer. | Figure 38: Modelled layer 8 attribution source layer. |

| Figure 39: Modelled layer 9 attribution source layer. | Figure 40: Modelled layer 10 attribution source layer. |
| --- | --- |
| Figure 41: Modelled layer 11 attribution source layer. | Figure 42: Modelled layer 12 attribution source layer. |
| Figure 43: Modelled layer 13 attribution source layer. | Figure 44: Modelled layer 14 attribution source layer. |
| Figure 45: Modelled layer 15 attribution source layer. | Figure 46: Modelled layer 16 attribution source layer. |

| Figure 47: Modelled layer 17 attribution source layer. | Figure 48: Modelled layer 18 attribution source layer. |
| --- | --- |
| Figure 49: Modelled layer 19 attribution source layer. | Figure 50: Modelled layer 20 attribution source layer. |

| Figure 51: Modelled layer 21 attribution source layer. | Figure 52: Modelled layer22 attribution source layer. |
| --- | --- |
| Figure 53: Modelled layer 23 attribution source layer. | Figure 54: Modelled layer 24 attribution source layer. |

| Figure 55: Modelled layer 25 attribution source layer. | Figure 56: Modelled layer26 attribution source layer. |
| --- | --- |
| Figure 57: Modelled layer 27 attribution source layer. | Figure 58: Modelled layer 28 attribution source layer. |

| Figure 59: Modelled layer 29 attribution source layer. | Figure 60: Modelled layer 30 attribution source layer. |
| --- | --- |

.

Model complexity

The complexity of the Gippsland region groundwater model is consistent with the “Impact Assessment” class described by Middlemis et al. (2000).  Based on a model appraisal the developed model conceptualisation is on average assumed to have moderate complexity and is suitable for predicting the impacts of proposed developments or management policies. This is further supported using the more recent Australian Groundwater Modelling Guidelines (Barnett et al., 2012) which adopts a Confidence Level Classification.  The Confidence Level Classification is a cornerstone of the national guidelines and is used to indicate the reliability of model predictions based on a number of criteria related to the available data with which the model is conceptualised and calibrated, the manner and accuracy of calibration and the manner in which the predictions are formulated.  The assessment criteria are summarised in Table 9.  Three confidence level classes are defined with Class 1 being the lowest confidence and Class 3 the highest.  On assessing the various relevant criteria as reported in Table 10, it is considered that the model should be targeted as Class 2 which is appropriate for assessing regional groundwater impacts.  Factors limiting higher confidence include the limited availability of data in the offshore parts of the model domain and the absence of any historic coal seam gas or tight and shale gas developments and hence predictive scenarios include stresses and associated impacts that have yet to be demonstrated.

Table 9: Summary of Confidence Level Classification criteria adopted in the Australian Groundwater Modelling Guidelines (Barnett et al., 2012).

Table 10: Gippsland Model appraisal based on the abbreviated guideline (Table 9).

| Class | Data | Calibration | Prediction | Indicators |
| --- | --- | --- | --- | --- |
| 1 |  |  |  | X Timeframes > 10 stresses |
| 2 | X Some | X Partial performance | X Timeframe > calibration |  |
| 3 |  |  |  |  |

Allowance for temperature and density

The model does not explicitly simulate non-isothermal processes, density driven flows, contaminant flow or subsidence. As such solute transport and water quality has not been explicitly modelled. It is acknowledged that temperature variation over the depth range considered, both onshore and offshore, is large as are the off-shore salinity variations.  Density gradients offshore are known to significantly influence pressure distributions resulting in flows towards the shoreline that are difficult to approximate using a constant density model. There are limitations to this approach to ignoring density variations at the coast and off-shore but there was insufficient time to address the significance of these.  However, the assumption of constant density is not considered to significantly compromise the onshore predictions of the shallow system response for the purpose of this study.

It is noteworthy that data used in CSIRO modelling that accounted for density (to assess potential for seawater intrusion) and temperature (due to geothermal resources in the basin) has been used in the calibration process.  Specifically the predevelopment and 2005 simulated potentiometric surfaces derived by CSIRO modelling for the Latrobe Group have been adjusted to remove the density and temperature impacts.  These adjusted surfaces were then used in the calibration procedures.  Given the limited available data this approach was deemed appropriate and pragmatic. Density head correction adjustments were also made to all groundwater observation bore data.

Allowance for faults

Geological faults and other geological discontinuities have not been explicitly modelled. Analysis of the stratigraphic data identified connection between the offshore and onshore units. Additionally, the properties were estimated to be continuous and not truncated. Faults were therefore considered to the extent that they were incorporated in the model geometry.

Model recharge

Dryland rainfall recharge, irrigation recharge and flood induced episodic recharge have been incorporated into the groundwater model from predictions derived using the CAT unsaturated catchment model (Beverly et al., 2005; Beverley, 2009a).  Recharge derived from irrigation has been estimated based on management scripts representing irrigation practices adopted by each irrigated agricultural industry.  These management scripts are assigned to irrigated land units within the land use layer and describe irrigation triggers, application volumes and frequency of irrigation events.  Flood induced episodic recharge has been simulated by maintaining saturation in the surface soil layer in all regions within mapped flood extents.  The duration of inundation was arbitrarily set as 5 days, unless specific information to the contrary was available.  The spatially averaged annual recharge is summarised in Appendix C.

Model evapotranspiration

In Modflow the evapotranspiration (ET) module simulates ET from shallow watertables based on a root extinction depth (m) and a maximum groundwater extraction rate (mm/yr).  The extinction depth identifies the watertable depth below natural surface below which no ET will occur; see for example Shah et al. (2007).  In the current application, the extinction depth was based on the vegetation root depth assigned in the various farming system models used to estimate recharge within the CAT modelling framework as reported in Beverly et al. (2011).  The extinction depths vary from 800 mm for annual pasture to 7 m for forests.  The maximum evapotranspiration rate was calculated as the difference between potential daily ET and estimated daily vegetative ET as predicted using the CAT modelling framework.  This approach ensures that the sum of estimated daily vegetative ET from the unsaturated zone and the assigned maximum groundwater ET from the saturated zone cannot exceed daily potential ET calculated from meteorological data.

Model rivers and drains

A total of 197 river gauges were identified within the model area representing all of the major rivers.  Daily gauge level data was sourced from the Victorian Department of Environment, Land, Water and Planning Water Measurement Information System (WMIS, 2015). A list of the river gauges is provided in Appendix A.

Only main stems of the major rivers were included in the model.  These river reaches were identified using the DEPI hydro25 spatial data set (DEPI, 2014).  The river classification was used to vary river incision depth (depth below the ground surface as defined by the digital elevation model) and width attributes.  In the absence of recorded stage height information, river classification was used to estimate river stage heights.  A total of 22,573 river cells are included in the model.  Fifty-one gauges were selected to calibrate the catchment modelling framework in unregulated catchments based on Base Flow Indexes and observed stream flows.

Drainage channels and man-made drainage features in the Macalister Irrigation District (MID) were included in the model based on available drainage network mapping (Figure 61).  This information was sourced from Southern Rural Water (SRW) and the DEPI Corporate Spatial Data library.  Drainage cells are assigned to the uppermost cells within the model to capture groundwater discharge processes.  Drain cells in Modflow can only act as groundwater discharge points and as such those cells outside drainage channels will be characterised as having a bed elevation equivalent to ground surface elevation.  A total of 410,504 drainage cells are incorporated in the model.  Apart from 3 river gauges sourced from the WMIS, SRW also has 15 gauges (Figure 61 and Table 11) monitored drainage from the MID.  The measurements commenced between 1997 and 2005.  Of the 15 gauges, six were selected to calibrate the catchment modelling framework based on observed discharge.  The mean monthly discharge and annual discharge at these six gauges are presented in Figure 62 and Figure 63, respectively.

Figure 61: Drainage channels and monitoring sites in the Macalister Irrigation District.

Table 11: Gauges monitoring drain from the Macalister Irrigation District (sourced from SWR).

| Site code | Site name | Eastings_z55 | Northings_z55 | Data Record |  | Mean Annual Discharge (ML/yr) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | From | to |  |
| 225248A | Boggy Creek | 5793110 | 493086 | 2001 | 2013 | 3710 |
| 225731A | Boisdale No 1 Drain | 5807657 | 499378 | 2001 | 2013 | 623 |
| 225732A | Bundalaguah | 5790315 | 500025 | 2001 | 2013 | 1696 |
| 225735A | CG1 | 5781567 | 478177 | 2001 | 2013 | 2106 |
| 225729A | CG2 | 5779780 | 489665 | 1999 | 2013 | 4960 |
| 225709A | CG3 | 5779400 | 492200 | 1997 | 2013 | 5431 |
| 225728A | CG4 | 5784308 | 499554 | 2000 | 2013 | 4161 |
| 225734A | CG6 | 5790162 | 493141 | 2001 | 2013 | 1955 |
| 225733A | Heyfield Ext | 5793889 | 491102 | 2001 | 2013 | 2685 |
| 225730A | Lake Wellington Main Drain | 5785744 | 515613 | 1997 | 2013 | 6133 |
| 225745A | Lake Wellington No 1 Drain | 5783956 | 513485 | 2004 | 2013 | 1366 |
| 225245A | Newry Creek | 5801277 | 495980 | 2000 | 2013 | 4251 |
| 225746A | Newry No 1 | 5800909 | 493696 | 2004 | 2013 | 388 |
| 225251A | Nuntin Creek | 5791751 | 509946 | 2001 | 2013 | 1799 |
| 225250A | Serpentine Main Drain | 5794788 | 496506 | 2000 | 2013 | 3203 |

Figure 62: Mean monthly flow (ML/month) at key gauges monitored by SRW in the Macalister Irrigation District.

Figure 63: Annual flow (ML/yr) at key gauges monitored by SRW in the Macalister Irrigation District.

Model groundwater extractions

Time series groundwater extraction data was sourced from various independent data sets, namely:

Southern Rural Water – This data includes groundwater bore locations, bore construction date, entitlement and metered usage.

GHD mine models – Pre-existing simulation model well files sourced to provide site specific groundwater extractions associated with Latrobe Valley coal mines (GHD 2011a, GHD 2011b).

EcoMarkets data – This data set captured the groundwater usage information incorporated in the East Gippsland and West Gippsland groundwater models developed by GHD Pty Ltd (GHD 2010a, 2010b) as part of the ecoMarkets project commissioned by DSE in 2008.

Stock and domestic – Victorian state-wide stock and domestic data was provided by DSE and included location, completion date, top of screen and annual extraction volume.

Offshore extractions – Volume equivalent groundwater extractions associated with offshore oil and gas operations were sourced from Hatton, et al. (2004) and Varma, et al (2012).

All bores were assigned to aquifers according to the following criteria (in order of preference):

as defined by the source of any historical abstraction data

based upon bore construction (screen depth) information in conjunction with assigned GMU

based upon bore construction (screen or bore depth) information

bores without any GMU or construction data were assigned to the upper most (watertable) aquifer.

East Gippsland groundwater extraction estimates for the period 1970 to 1990 were derived based on the relative change as observed in the West Gippsland extraction for the same period with allowance for bore construction dates and entitlement volumes.  This was required as limited monitored groundwater usage data was available for East Gippsland bores for the period 1970 to 1990.  The process assigned the West Gippsland annualised usage trend derived for the period 1970 to 1990 to those East Gippsland bores with missing data.  Bore construction dates were used to ensure extraction were only assigned post construction.  Importantly all abstractions within the model domain were forced to exactly match the Water Account usage volumes for the 2003-2012 reporting periods as summarised in Table 12.

The final allocation and composition of groundwater extraction bores incorporated in the model and as associated to various Groundwater Management Areas (GMA) and Water Supply Protection Areas (WSPA) is summarised Table 13.  Figure 64 shows the specified annual extraction volumes and source that were compiled into the model groundwater extraction input file.

Table 12: State Water Accounts (usage ML).

| GMA/WSPA | 2003–04 | 2004–05 | 2005–06 | 2006–07 | 2007–08 | 2008-09 | 2009–10 | 2010–11 | 2011–12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Denison | 15,224 | 6,500 | 6,680 | 10,152 | 6,147 | 8,385 | 7,987 | 3,695 | 2,992 |
| Giffard | 2,719 | 2,520 | 3,260 | 3,719 | 3,205 | 3,662 | 1,717 | 865 | 845 |
| Leongatha | 648 | 515 | 441 | 625 | 600 | 344 | 158 | 31 | 72 |
| Moe | 1,098 | 1,084 | 990 | 1,447 | 1,414 | 1,081 | 1,095 | 191 | 330 |
| Orbost | 464 | 270 | 350 | 540 | 490 | 578 | 333 | 95 | 0 |
| Rosedale | 15,457 | 9,920 | 10,860 | 7,539 | 10,678 | 11,540 | 11,009 | 7,543 | 7,739 |
| Sale | 14,680 | 7,680 | 10,450 | 13,358 | 9,504 | 11,185 | 11,094 | 7,164 | 6,324 |
| Stratford | 27,355 | 17,230 | 17,690 | 19,182 | 24,099 | 26,897 | 27,896 | 24,904 | 26,042 |
| Tarwin | 18 | 14 | 12 | 12 | 2 | 6 | 6 | 9 | 15 |
| Wa De Lock | 12,095 | 9,403 | 8,059 | 10,509 | 7,194 | 9,517 | 10,386 | 4,832 | 3,767 |
| Wy Yung | 2,438 | 790 | 1,110 | 1,895 | 631 | 1,024 | 798 | 309 | 347 |
| Yarram | 12,205 | 8,100 | 11,070 | 16,009 | 12,048 | 13,911 | 11,778 | 6,882 | 6,740 |

Table 13: Modelled extractions within each GMA and WSPA reporting region.

|  | Depth upper (m) | Depth lower (m) | Licensed pumps | Mine pumps | S&D | Total pumps |
| --- | --- | --- | --- | --- | --- | --- |
| Denison WSPA | 0 | 25 | 183 | 0 | 118 | 301 |
| Giffard GMA | 500 | 200 | 16 | 0 | 152 | 168 |
| Leongatha GMA | 0 | basement | 27 | 0 | 130 | 157 |
| Moe GMA | 25 | basement | 67 | 0 | 249 | 316 |
| Orbost GMA | 20 | 45 | 11 | 0 | 13 | 24 |
| Rosedale GMA Zone 1 | 50 | 150 | 20 | 48 | 19 | 87 |
| Rosedale GMA Zone 2 | 25 | 350 | 229 | 0 | 506 | 735 |
| Rosedale GMA Zone 3 | 200 | 350 | 15 | 0 | 11 | 26 |
| Sale WSPA | 25 | 200 | 413 | 0 | 1011 | 1424 |
| Stratford GMA Zone 1 | 150 | Basement | 1 | 8 | 7 | 16 |
| Stratford GMA Zone 2 | 350 | Basement | 5 | 0 | 15 | 24 |
| Tarwin GMA | 0 | 25 | 1 | 0 | 23 | 24 |
| WaDeLock GMA | 0 | 25 | 301 | 0 | 323 | 627 |
| Wy Yung WSPA | 0 | 25 | 110 | 0 | 71 | 181 |
| Yarram WSPA Zone 1 | 0 | basement | 102 | 0 | 510 | 612 |
| Yarram WSPA Zone 2 | 200 | basement | 6 | 0 | 17 | 23 |

Figure 64: Annual total extraction (ML/yr) by contribution incorporated into the model.

Figure 65: State water account reporting regions (separate figures to avoid overlapping)

Figure 65 (cont’d): State water account reporting regions (separate figures to avoid overlapping)

Figure 66: Location of all extractions including S&D bores.

Figure 67: Location of all extraction bores excluding S&D bores.

| Figure 68: Location of extraction bores in modelled layer 2. | Figure 69: Location of extraction bores in modelled layer 3. |
| --- | --- |
| Figure 70: Location of extraction bores in modelled layer 4. | Figure 71: Location of extraction bores in modelled layer 5. |

| Figure 72: Location of extraction bores in modelled layer 6. | Figure 73: Location of extraction bores in modelled layer 9. |
| --- | --- |
| Figure 74: Location of extraction bores in modelled layer 17. | Figure 75: Location of extraction bores in modelled layer 18. |
| Figure 76: Location of extraction bores in modelled layer 19. | Figure 77: Location of extraction bores in modelled layer 23. |
| Figure 78: Location of extraction bores in modelled layer 30. |  |

Model simulation period

Groundwater recharge in southern Australia is strongly correlated with annual rainfall.  This is particularly true when the seasonal rainfall pattern follows that established by the long-term trend.  In this condition most rainfall occurs during cold wet winters and to a lesser extent in hot dry summers.

When considering the steady-state condition it is useful to consider the hypothetical condition that has stable climate and land use over time.  This allows for an appreciation of a quasi-equilibrium condition that is ultimately realised between surface water and groundwater.  This approach was adopted as the basis of a steady-state groundwater model.

Choosing a year that best represents the climatic circumstances for the simulation objectives is an essential part of the steady-state assessment. This decision was influenced by the availability of appropriate calibration data.  This approach generally allows for the selection of a year that was outside the bounds of a longer term shift in annual rainfall distribution.  The selected steady-state condition was based on 1970 rainfall and assumed no groundwater extractions.  This initial state was selected to represent predevelopment conditions as post-1970 historic groundwater pumping in the region (on and offshore) have not achieved a quasi-equilibrium response as based on available groundwater hydrograph trends.

The transient simulation period was 1970 to 2012.  This period captures both pre-mine development and a range of varying climatic conditions, including above average wet and dry sequences.  The 1970 starting date enables the incorporation of historic groundwater extraction data into the model and provides sufficient lead time for the groundwater model to minimise the impact of initial conditions on model predictions associated with the period of interest, namely the calibration/validation period of 2000 to 2012.

Model stress periods

The transient calibration period has been formulated with annual stress periods for the period 1970–1990, and monthly stress periods throughout the 1991–2012 calibration/verification periods.  The model stress periods were based on adequately capturing historical offshore and offshore extractions, specifically Latrobe Valley coal mine and offshore oil and gas production wells.  To this end, pre-1990 stress periods were assigned as annual within which twelve time-steps were adopted.  For the more recent post-1990 calibration/verification period the stress period was based on representing seasonal water level oscillation and extractions data.  Following consultation with ecologists it was subsequently agreed that post-1990 monthly time-steps were required to determine the potential impacts of water changes on water dependent ecosystems.  It is also noted that daily data was used in the generation of stress period recharge and potential groundwater evapotranspiration rates.

Steady-state model conditions

The steady-state calibration model represents the 1970 Latrobe Valley pre-development conditions.  On the basis that steady-state predictions reflect long-term equilibrium conditions assuming constant groundwater inputs and stresses throughout time, no groundwater extractions were assigned during the steady-state simulation.

Transient model initial conditions

The initial conditions for the transient model were based on a steady-state solution representing the 1970 pre-development condition which assumed no groundwater pumping.

Model calibration criteria

The calibration procedure adopted a split calibration/verification approach in which the calibration period 
was 1990 to 1999 and the verification period was 2000 to 2012.  Model calibration was based on matching groundwater hydrograph response and groundwater discharge to streams.  In addition to these data sets, model predictions were also compared to mapped discharge extents in regions where this information 
was available.

Numerous statistical performance measures were applied to assess the goodness of fit model predicted and observed groundwater heads as summarised in Table 14.  Analysis includes the coefficient of determination (CD) and the Nash–Sutcliffe index (NSI) (Nash and Sutcliffe, 1970).  Each coefficient is defined by the number of observations n, measured data at time i, (Yi), and the corresponding modelled prediction at time i (Xi).  The CD value is a measure of the relationship between observed and simulated values whereas the NSI value indicates how well the plot of the observed versus simulated values fit the 1:1 line.  If the CD and NSI values approach zero (or are negative), the model performance is considered to be unacceptable or poor.  If the values are equal to one, the model prediction is considered perfect (Middlemis et al., 2000).  
An NSI value of 0.6 is viewed as “satisfactory” whereas an NSI of 0.8 or higher is considered “good” (Chiew et al., 1993).

Both manual and automated calibration procedures were adopted in this study.  In the case of automated calibration three independent software packages were utilised, namely:

- Groundwater Vistas

- DOS version of PEST

- A simplex optimisation approach.

A common underpinning data set was used by each package to enable a comparison of results.  A zonal parameter distribution approach was employed.

Table 14: Statistical measure of model performance.

| Statistical Measure | Definition | Units | Description |
| --- | --- | --- | --- |
| Mean sum of squares |  | m | MSSQ is the average of the square of the differences between predicted values and observed values. |
| Root mean square (RMS) error |  | m | Root Mean Square (RMS) error is the sample standard deviation of the differences between predicted values and observed values. |
| Scaled root mean square error |  | % | Scales the RMS error to the range of the observed values and is expressed as a percentage. The SRMS is more meaningful than root mean square error as it is independent of scale. |
| Root mean fraction square error |  | % | RMFS represents the sample standard deviation of the differences between predicted values and observed values as a fraction of the observed value expressed as a percentage. |
| Scaled root mean fraction square error |  | % | Scales the RMFS error by the ratio of the mean observed value to the range of the observed values expressed as a percentage. |
| Mean sum of residual |  | m | MSR or residual mean is not an unbiased estimator of the error variance. |
| Scaled mean sum of residuals |  | % | As per Root Mean Square error but further scaled by the range of observation data and expressed as a percentage. |
| Coefficient of determination |  |  | Describes the causation of change in Y by changes in X. |
| Nash–Sutcliffe index |  |  | The NSI value is a measure of the relationship between observed and simulated values. |

Key groundwater observation bores

All available groundwater observation bores data was compiled and included in the model as calibration targets.  A total of 822 target bores are utilised for the transient calibration model whereas only 71 bores have data available to calibrate the predevelopment steady-state condition.  Figure 79 shows the location of the observation bores used for model calibration.  Bore hydrographs identified in SKM (2014) and GHD (2014b) are considered “high reliability” and were assigned a calibration weight of 5; all other observation bore data were assigned a calibration weight of 1.

Figure 79: Location of observation bores used for model calibration.

Baseflow estimates

Baseflow separation and analysis were conducted on all stream flow data within the study area to estimate the contribution of groundwater to streamflow.  The estimated groundwater discharge volumes to streams were then used as a calibration flux target within ungauged catchments.

Baseflow separation was estimated using the approach developed by the USGS and subsequently implemented in the BFI software.  Daily streamflow input data was sourced from the Victorian Water resources data Warehouse (http://www.vicwaterdata.net/vicwaterdata/home) and extracted as computed daily flows.  Periods with poor quality data and/or missing records were infilled to ensure a complete data record at each location.  The program implements a digital filter procedure and combines a local minimums approach with a recession slope test.  The program estimates the annual base-flow volume of unregulated rivers and streams and computes an annual base-flow index (BFI, the ratio of base flow to total flow volume for a given year) for multiple years of data at one or more gage sites.  Although the method may not yield the true base flow as might be determined by a more sophisticated analysis, the index has been found to be consistent and indicative of base flow, and thus may be useful for analysis of long term base-flow trends (Arnold et al., 2010).

As a comparison, the derived BFI estimates were compared to independent values based on the Lyne and Hollick digital filter method (SKM, 2012b) and recent GHD’s study (GHD, 2013).  The comparative values adopted a single consistent baseflow filter parameter of 0.98 applied to all sites thereby allowing a comparison from site to site on an equivalent basis.  Appendix A tabulates the BFI estimates derived in this project and those reported in the SKM study for all stream gauges within the study region.

Manual calibration

Initial model calibration was made by using a manual trial-and-error approach.  Due to the large number of solution points and the large number of model layers, it was found to be inefficient and difficult to make notable enhancements in the calibration process.  As a result automated calibration methods were adopted.

Automated calibration

The automated calibration approach utilised three independent software packages, namely:

- BeoPest implemented in Groundwater Vistas

- DOS version of PEST (Doherty, 2010)

- A simplex optimisation approach.

The same calibration targets were used by each package to enable a comparison of results.

The automated calibration procedure adopted a zonal approach.  The zone calibration approach was based on a series of parameter zones in which the horizontal and vertical hydraulic conductivities were modified in order to minimise the variation between observed data and modelled prediction.  A total of 36 zones based on geological extents and attributes were mapped as shown in Figures 80 to 109.

| Figure 80: Calibration zones for model layer 1. | Figure 81: Calibration zones for model layer 2. |
| --- | --- |
| Figure 82: Calibration zones for model layer 3. | Figure 83: Calibration zones for model layer 4. |
| Figure 84: Calibration zones for model layer 5. | Figure 85: Calibration zones for model layer 6. |

| Figure 86: Calibration zones for model layer 7. |  |  | Figure 87: Calibration zones for model layer 8. |  |
| --- | --- | --- | --- | --- |
| Figure 88: Calibration zones for model layer 9. |  |  | Figure 89: Calibration zones for model layer 10. |  |
| Figure 90: Calibration zones for model layer 11. |  |  | Figure 91: Calibration zones for model layer 12. |  |
| Figure 92: Calibration zones for model layer 13. |  |  | Figure 93: Calibration zones for model layer 14. |  |
| Figure 94: Calibration zones for model layer 15. |  |  | Figure 95: Calibration zones for model layer 16. |  |
| Figure 96: Calibration zones for model layer 17. |  |  | Figure 97: Calibration zones for model layer 18. |  |
| Figure 98: Calibration zones for model layer 19. |  |  |  | Figure 99: Calibration zones for model layer 20. |
| Figure 100: Calibration zones for model layer 21. |  |  |  | Figure 101: Calibration zones for model layer 22. |
| Figure 102: Calibration zones for model layer 23. |  |  |  | Figure 103: Calibration zones for model layer 24. |
| Figure 104: Calibration zones for model layer 25. | Figure 105: Calibration zones for model layer 26. |  |  |  |
| Figure 106: Calibration zones for model layer 27. | Figure 107: Calibration zones for model layer 28. |  |  |  |
| Figure 108: Calibration zones for model layer 29. | Figure 109: Calibration zones for model layer 30. |  |  |  |

Sensitivity analysis

A comprehensive analysis was undertaken to assess the sensitivity of key modelled outputs to variations in input data.  Modelled outputs considered included baseflow rates, saturated area and groundwater discharges (evapotranspiration, boundary fluxes and aquifer interflows).  The sensitivity analysis procedure involved altering by up to three orders of magnitude the calibrated input data sets (including horizontal hydraulic conductivities, specific storage, specific yield, vertical hydraulic conductivities, recharge, maximum evapotranspiration rates, river and boundary conductance terms) and recording the groundwater response relative to the calibrated condition.  Each input data set was systematically modified while maintaining 
all other input data.  In total, approximately 4200 simulations were undertaken to test the robustness of 
the groundwater model and the uniqueness of the combination of input data required to meet the 
calibration criteria.

Steady-state model calibration

The steady-state model assumed 1970 pre-development conditions in which there was no groundwater extractions and annual recharge and potential evapotranspiration rates are shown in Figure 110 and 
Figure 111 respectively.

Figure 110: Steady-state equivalent annual recharge (mm/yr).

Figure 111: Steady-state equivalent annual potential groundwater evapotranspiration (mm/yr).

Mass balance

Simulated steady-state water balance results for the active model domain are summarised in Table 15.  It is noteworthy that all domain cells remained saturated.  The percentage mass balance closure error reported by Modflow-2005 was 0.7%, which satisfies the prescribed target.

Table 15: Steady-state mass balance.

|  | Flow in ML/day | Flow out ML/day | Net flow ML/day | % Recharge |
| --- | --- | --- | --- | --- |
| Recharge | 9737.7 |  | 9737.7 |  |
| Evapotranspiration |  | 4002 | –4002 | 41 |
| Major rivers | 15 | 353 | –338 | 4 |
| Drains/streams |  | 3468 | –3468 | 36 |
| Constant head | 777 | 2632 | –1855 | 19 |

Results suggest that:

40% of groundwater recharge discharges to river and drains

41% of groundwater recharge is “lost” through evapotranspiration

19% of groundwater recharge is discharged as a net outflow across the model boundaries predominantly through flux offshore.

Calibration statistics

The calibration statistics for the steady-state groundwater model based on 71 observation points are summarised in Table 16.  These target statistics meet the calibration criteria as specified by the project.  The observed versus computed target values are shown in Figure 112 whereas the observed versus residual error is shown in Figure 113.

Table 16: Steady-state model calibration statistics.

| Statistic | Code | Unit | Steady-state |
| --- | --- | --- | --- |
| Mean sum of squares | MSSQ | m | 3.50 |
| Root mean square | RMS | m | 1.87 |
| Scale root mean square | SRMS | % | 5.34 |
| Root mean fraction square | RMFS | % | 10.24 |
| Scaled root mean fraction square | SRMFS | % | 0.60 |
| Sum of residuals | SR | m | 398.07 |
| Mean sum of residuals | MSR | m | 0.52 |
| Scaled mean sum of residuals | SMSR | % | 1.49 |
| Coefficient of determination | CD |  | 0.66 |
| Nash–Sutcliffe index | NSI |  | 0.69 |

Figure 112: Observed versus computed steady-state heads (mAHD).

Figure 113: Residual error (observed head – modelled head) (m) for each steady-state calibration bore.

Depth to watertable and potentiometric surfaces

The simulated 1970 unconfined potentiometric surface (watertable) and depth-to-watertable across the entire model domain is shown in Figure 114 and Figure 115 respectively.

Visual comparison of the Victorian SAFE depth to watertable map (Figure 116) with the steady-state simulated depth to watertable shows the watertable surface is reasonable within the alluvial systems, however in some upland locations the watertable appears in greater connection with surface features than presented in the Victorian Aquifer Framework (VAF) data.  This is not considered a significant issue as these areas are well beyond the zone of interest.

It must be noted that the VAF depth to watertable map was derived using a combination of terrain analysis and interpolated bore data, and in part on proximity to streams within the exposed basement areas.  Additionally, the VAF reflects the 1990 conditions whereas the simulated steady-state depth to watertable represents pre-development conditions.  As such, it is expected that that the simulated steady-state depth to watertable map would have a greater area of shallow watertable than reported in the VAF spatial layer.

Figure 114: Simulated 1970 unconfined aquifer potentiometric surface (mAHD).

Figure 115: Simulated 1970 steady-state depth to watertable (m).

Figure 116: 1990 watertable depth (m) within the West Gippsland CMA region.

Transient model calibration

Calibrated model aquifer parameterisation

The transient model calibration was based on a spilt calibration/verification approach in which parameter optimisation was applied for the period 1990-2000 following which verification was assessed over the subsequent last eleven years from 2001 to 2012.  Time constraints impacted on the progress of the transient calibration.  The calibration strategy was to produce modelled hydrographs that match the trends of the observation bores.  It was acknowledged that any elevation offsets between observed and predicted heads would be addressed as part of any future model refinement phase.

With the exception of zones 23-29 representing the Strzelecki Formation parameterisation, the post-calibration modelled aquifer parameterisation is summarised in Table 17.  The aquifer parameterisation attributed to zones 23-29 were revised following a review of the Strzelecki Formation parameterisation.  The model calibration results in the horizontal hydraulic conductivity for the uppermost Strzelecki units of the order 0.195 m/day and the vertical hydraulic conductivity be 0.0295 m/day which are inconsistent with the potential for this layer to act as a tight or shale gas reservoir. In order to better represent this potential a lower value was used for scenario purposes.

Table 17: Calibrated model aquifer parameterisation

| Zone | Aquifer | Kxy (m/d) | Kzz (m/d) | Specific Yield | Specific storage (m-1) | Kzz/Kxy |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Marine water thickness | 1.063 X 102 | 1.594 X 101 | 0.1 | 1 X 10-5 | 0.150 |
| 2 | Quaternary | 6.507 | 9.761 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 3 | Haunted Hill Formation | 3.203 | 4.804 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 4 | Nuntin clay | 3.333 | 5 X 10-1 | 0.04 | 1 X 10-5 | 0.150 |
| 5 | Boisdale Formation | 2.986 X 101 | 4.479 | 0.1 | 1 X 10-5 | 0.150 |
| 6 | Jemmys Point & Upper Hazelwood Formation | 8.890 X 10-2 | 1.334 X 10-2 | 0.1 | 1 X 10-5 | 0.150 |
| 7 | Yallourn Coal Seam | 2 X 10-3 | 3 X 10-4 | 0.1 | 1 X 10-5 | 0.150 |
| 8 | Yallourn Aquifer and interseam | 7.667 X 10-1 | 1.15 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 9 | Lower M2 interseam | 1.605 X 101 | 1.202 | 0.1 | 1 X 10-5 | 0.075 |
| 10 | M1A coal | 2.230 X 10-1 | 3.345 X 10-2 | 0.02 | 1 X 10-5 | 0.150 |
| 11 | Morwell 1A interseam/aquifer | 7.450 | 7.45 X 10-1 | 0.1 | 1 X 10-5 | 0.100 |
| 12 | Morwell 1B coal | 4.525 X 10-2 | 6.79 X 10-3 | 0.1 | 1 X 10-5 | 0.150 |
| 13 | Morwell 1B interseam | 9.691 | 1.602 X 10-1 | 0.1 | 1 X 10-5 | 0.017 |
| 14 | Morwell 2 | 4.108 X 10-1 | 6.162 X 10-2 | 0.1 | 1 X 10-5 | 0.150 |
| 15 | Lakes Entrance Formation | 9.572 X 10-2 | 8.860 X 10-3 | 0.05 | 1 X 10-5 | 0.093 |
| 16 | M2c aquifer/Seaspray sands | 6.119 | 3.902 X 10-1 | 0.1 | 1 X 10-5 | 0.064 |
| 17 | Thorpdale volcanics | 6.522 X 10-1 | 1.820 X 10-2 | 0.1 | 1 X 10-5 | 0.028 |
| 18 | Upper Latrobe Group | 2.350 | 3.524 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 19 | T1 coal | 1.92 X 10-3 | 2.9 X 10-4 | 0.02 | 1 X 10-5 | 0.151 |
| 20 | T1 interseam | 1.262 | 1.894 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 21 | T2 coal | 1.420 X 10-3 | 8 X 10-5 | 0.02 | 1 X 10-5 | 0.056 |
| 22 | T2 interseam | 8.862 X 10-1 | 9.566 X 10-2 | 0.1 | 1 X 10-5 | 0.108 |
| 23 | Strzelecki top 500m | 1 X 10-2 | 1 X 10-3 | 0.02 | 1 X 10-5 | 0.100 |
| 24 | Strzelecki 500-1000m | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 25 | Strzelecki 1km-2km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 26 | Strzelecki 2km-3km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 27 | Strzelecki 3km-4km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 28 | Strzelecki 4km-6km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 29 | Strzelecki >6km | 2 X 10-4 | 1 X 10-5 | 0.02 | 1 X 10-5 | 0.050 |
| 30 | Palaeozoic basement 200m thick | 1.012 X 10-1 | 1.518 X 10-2 | 0.02 | 1 X 10-5 | 0.150 |
| 31 | Lower M2 interseam | 8.747 | 1.312 | 0.1 | 1 X 10-5 | 0.150 |
| 32 | Lower M2 interseam | 6.077 | 6.273 X 10-1 | 0.1 | 1 X 10-5 | 0.103 |
| 33 | Upper Latrobe Group | 2.665 | 3.997 X 10-1 | 0.1 | 1 X 10-5 | 0.150 |
| 34 | Upper Latrobe Group | 2.930 | 3.163 X 10-1 | 0.12 | 1 X 10-5 | 0.108 |
| 35 | Palaeozoic basement 200m thick | 1.950 | 2.337 X 10-2 | 0.04 | 1 X 10-5 | 0.012 |
| 36 | Quaternary | 1.501 X 10-1 | 2.009 X 10-2 | 0.07 | 1 X 10-5 | 0.134 |

Mass balance

Simulated transient volumetric water balance components for the active model domain are shown in Figure 117.  Presented results clearly show the transition phase from the initial steady-state conditions to the calibration period commencing 1990.  Also of note is the lack of seasonal responses during the 1970 to 1990 simulation period during which annual stress periods have been assigned.

Figure 117: Transient model mass balance components (m3/day per timestep).

Calibration statistics

The calibration statistics for the transient groundwater model based on 88, 799 observation points are summarised in Table 18 for the transition, calibration and verification periods.  The transient calibration statistics suggest that there is a poor match between the modelled and observed data during the initial twenty years of simulation, which is also when the stress periods are large and the transition from the pseudo steady-state conditions would have the greatest impact on model predictions.  This suggests that the simulated pseudo steady-state solution describes the 1970 groundwater condition poorly.  Importantly the calibration statistics improve during the 1990-1999 calibration period as evidenced with an improved scaled RMS of 5.62% relative to the pre-calibration period scaled RMS of 8.82%.  The 2000–2010 verification period has improved calibration statistics with a scaled RMS of 1.42%, a mean sum of residuals of 5 m and a coefficient of determination of 0.9.  Key calibration criteria traces are presented in Figure 118 to Figure 121.

Simulated versus observed time-series traces are presented in Appendix H.

Table 18: Transient model calibration statistics.

| Name of statistics | Code | Unit | 1970–1989 | 1990–1999 | 2000–2010 |
| --- | --- | --- | --- | --- | --- |
| Number of observations |  |  | 16050 | 30158 | 42591 |
| Mean Sum of Squares | MSSQ | m | 876.30 | 354.53 | 231.78 |
| Root Mean Square | RMS | m | 29.60 | 18.83 | 15.22 |
| Scale Root Mean Square | SRMS | % | 8.82 | 5.62 | 1.42 |
| Root Mean Fraction Square | RMFS | % | 42186.37 | 5403.267 | 10633.07 |
| Scaled Root Mean Fraction Square | SRMFS | % | 3318.30 | 271.58 | 139.77 |
| Sum of Residuals | SR | m | 59826.58 | 221472.12 | 223509.96 |
| Mean Sum of Residuals | MSR | m | 3.73 | 7.34 | 5.25 |
| Scaled Mean Sum of Residuals | SMSR | % | 1.11 | 2.19 | 0.49 |
| Coefficient of Determination | CD |  | 0.45 | 0.68 | 0.90 |

Figure 118: Number of observation points throughout the simulation period.

Figure 119: Temporal trace of RMS error (%) throughout simulation period.

Figure 120: Temporal trace of mean residual error (m) throughout simulation period.

Figure 121: Temporal trace of mean absolute error (m) throughout the simulation period.

Spatial residual error

The spatial distributions of residual errors are presented in Figure 122, Figure 123, Figure 124 and Figure 125 for 1980, 1990, 2000 and 2010 respectively.  The mean residual water level is defined such that negative numbers refer to points where predicted groundwater head values are higher than observed and positive values indicate that predicted groundwater head values are lower than observed.

Results shown in Figures 121 to 124 suggest that the model over predicts the heads in the outcropped regions and under predicts the heads in the lower parts of the landscape.  The greatest errors are shown to be in the outcropping regions of the Strzelecki Formation suggesting that the model attribution associated with model layer 23 requires further analysis.  This is consistent with observations reported in Section 6.1 that considered the hydraulic conductivities attributed to zones 23–29 were on the lower bounds of previously reported values.

Figure 122: Spatial mean residual water level error (m) for 1980 based on observed and predicted heads.

Figure 123: Spatial mean residual water level error (m) for 1990 based on observed and predicted heads.

Figure 124: Spatial mean residual water level error (m) for 2000 based on observed and predicted heads.

Figure 125: Spatial mean residual water level error (m) for 2010 based on observed and predicted heads.

Model sensitivity

A sensitivity analysis was undertaken to assess the performance of the groundwater model simulation to changes in various input parameter values.  The model performance was assessed based on scaled RMS, coefficient of determination and NSI criteria (see Section 4) using observed and predicated calibration bore hydrograph responses.  A total of 4200 simulations were examined in detail, in which changes in recharge, groundwater evapotranspiration rates, river conductance, horizontal and vertical hydraulic conductivities, specific storage and specific yields were modified by three orders of magnitude from the calibrated values.  Although 4200 simulations were examined in detail, model performance results based on variations in recharge, groundwater evapotranspiration rates, river conductance, drain conductance and lateral hydraulic conductivities are presented for the steady-state in Figure 126 to Figure 129 respectively.

The sensitivity of the model to recharge estimates is shown in Figure 126.  This figure shows the variation in MAE, coefficient of determination and NSI criteria due to altered recharge.  The trajectory of each response curve reflects the solution uniqueness and stability of the model.  The flatter the curve, the less sensitive the output to the associated model attribute.

Results from the sensitivity analysis show the comparatively high sensitivity of simulated shallow watertable extent to variations in recharge and maximum groundwater evapotranspiration.  This is to be expected as these parameter changes are applied across large extents of the model, whereas hydraulic conductivity changes are applied on a zone by zone basis which may be of minor/limited areal extents.  A similar outcome applies to river conductance changes which impact localised processes and is limited by the density of the river network incorporated in the model.

The sensitivity of predicted baseflow to parameter change, the most significant parameters are (in order of significance) recharge, potential groundwater evapotranspiration rates and river conductance.  The least significant parameters are those associated with the deeper aquifers.

With respect to the modelled area of shallow watertable, the most significant parameters are (in order 
of significance) recharge, potential groundwater evapotranspiration rates and conductivities of the 
unconfined aquifers.

Based on this analysis, model uncertainty is most impacted by estimates of recharge and potential groundwater evapotranspiration rates.  The hydraulic conductivities of the unconfined and outcropping aquifers also have significant influence on predicted baseflow and watertable extent.

In general the sensitivity analysis show model parameters generally fit within type I or II classification as described by Middlemis et al. (2000).

Figure 126: Sensitivity of the steady-state model to variations in recharge.

Figure 127: Sensitivity of the steady-state model to variations in potential groundwater 
evapotranspiration rate.

Figure 128: Sensitivity of the steady-state model to variations in river conductance.

Figure 129: Sensitivity of the steady-state model to variations in drain conductance.

Model assumptions

The key assumptions underpinning the development of the Gippsland region groundwater model are summarised in Table 19. These assumptions provide a foundation for the development of the model; while there is always some level of uncertainty the best available data has been used to limit predictive errors. It is likely that in some areas detailed hydrogeological processes have not been fully captured in the conceptualisation.  Sub-catchment or site-specific investigations would be required to better define local variations and responses.

Table 19: Key model assumptions.

| Assumption | Basis |
| --- | --- |
| There is no dual porosity. | Dual porosity models are only applicable to solute transport models and are not meaningful for flow models. |
| Groundwater concentration gradients are acknowledged to have some impact on groundwater heads but have not been explicitly included in the simulation. | Data limitations make inclusion into a dispersion-convective transient model problematic and would require significant additional computational resources and time. |
| Groundwater temperature gradients are acknowledged to have some impact on groundwater heads but have not been explicitly included in the simulation. | As for density driven. |
| Bores used in the model have been correctly assigned to aquifers. | Significant data cleansing work used to preparing data sets, however minor inaccuracies may exist. |
| The digital elevation model (DEM) has adequate vertical resolution to allow reasonable calibration of the model and represents the land surface topology over the entire domain. | Self-evident. |
| Land use change data is limited in availability and extent and has not been incorporated in this study. | Development scenarios based on existing land use. |
| The model cell size is adequate to meet project objectives. | Self-evident. |

Model fit for purpose

Review of model calibration (spatial and temporal) data in combination with the sensitivity analysis results suggests the groundwater model has the capacity to be used to assess the relative difference in predicted groundwater level changes between the proposed scenarios.  It must be noted that it is not appropriate to use the model to assess the absolute water level or water balance under either calibration or scenario conditions.  If additional scenarios (e.g. any not presented within this report) were to be applied to this calibrated groundwater model, the suitability (e.g. the fit for purpose for each) would need to be assessed on an individual basis.

Scenario modelling

Scenario modelling considers combinations of the following conditions:

various climate scenarios, although not the same scenarios as modelled under the CSIRO Sustainable Yields Project

development phases: the model has the capacity to simulate tight and shale and coal seam gas developments continuing for up to 30 years into the future with and without offshore development.

The purpose of the modelling was to quantify the impacts of potential onshore gas projects. The design of the scenarios was to consider combinations of tight and shale gas and coal seam gas extractions within defined prospective development regions.  Note there is negligible/limited conventional gas.  The impacts of each predictive scenario were estimated based on a comparison with a defined baseline condition.

Scenario future climate projects

Future climate regimes commence at the end of the calibration/validation period and extend from 1 January 2013 to December 2042.  Prior to 1 January 2013 recorded daily meteorological data were used to derive recharge and groundwater evapotranspiration rate estimates as assigned during the model calibration/validation process.  The future climate scenarios were based on a “dry” climate period selected to be 2006.  Rainfall in 2006 was consistently close to the average for the 2003–2010 period concurrent with the drought, as shown in Figure 130.

All measured 2006 daily climate conditions were concatenated throughout the scenario period.  It is noteworthy that the interannual distributions were unaltered, including potential evapotranspiration and 
solar radiation.  Additionally it is important to note that no time varying climate change projections were applied, nor were the impacts of elevated carbon dioxide on plant performance considered.  The climate change scenario directly influences future recharge and potential groundwater evapotranspiration rate estimates only.

Figure 130: Cumulative residual rainfall traces for each climate station within the study domain and showing 2006, selected as the future climate scenario.

Scenario development conditions

The scenarios regarding potential onshore natural gas development focuses on onshore depressurisation due to potential future tight and shale gas and coal seam gas extractions.  Table 20 summarises the basic attributes of the primary development scenarios.

The areas for potential onshore natural gas projects are reported in Goldie Divko (2014) and shown in 
Figure 131.  The extent indicated by the “regional scale” boundary (Figure 131) is not representative of the likely development field; only the extent of the sub-cropping rock formation that has the potential to contain gas.  For potential impact assessment purposes, the “sub-regional” extent represents the maximum limit to the area in which large scale resource development may occur and is applied in the model as the development scenario.

Gas extraction (both coal seam and tight and shale) was modelled using the Modflow drain function which removes water from the aquifer at a rate proportional to the difference between the head in the aquifer and some fixed head or elevation (called the drain elevation) so long as the head in the aquifer is above that elevation. If, however, the aquifer head falls below the drain elevation, then the drain has no effect on the aquifer.  This approach does not enforce a pressure head elevation (which is unknown in this study) rather it assigns a potential head elevation and volumetric flux conductance based on aquifer properties. .  In this case the drain elevation is set to the potential head elevation.  In this way the time-varying pressure head elevation is not fixed but rather fluctuates based on the extracted volumetric flux defined as a function of drain conductance and difference in pressure head elevation to drain elevation.   It was considered that this approach was appropriate in preference to assigning the following alternative options:

- A well function which relies on an a-priori user defined extraction volume (m3/day).  A suitable time-varying extraction volume was unknown.

- A fixed head condition.  This assumes that the assigned head elevation is known.  Additionally the flux across this fixed head boundary could not be bounded by known extraction volume limits and therefore could result in unrealistic extracted volumes of water.

- An evaporation function.  The evaporation function is similar to a drain function except that the rate of extraction varies linearly from an uppermost elevation or “ET surface” and an “extinction depth” or cutoff depth below the “Et surface” elevation below which no extractions occur.

Figure 131: Spatial extents of the prospect fields (1), sub-regional (2) and regional (3) boundaries. Refer to Table 20 for data type attributes.

Table 20: Onshore natural gas development data (Goldie Divko, 2015).

|  | Development Type | Target formation / Depth (m below NS) | Total gas field area (km2) a Note (a) | GIIP (Tcf) | Target formation water pressure during gas extraction | Well pad/ well spacing | Flow rate (gas) mmcfpd | Flow rate (gas) cubic m/d | Porosity (%) Note (b) | Permeability (milli Darcy) | Water extraction (L/s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tight and shale | (1) Prospect/ field scale | 1400 | 8.6 | 0.89 | dry | 1 km |  |  | <10 | 1.7-5.7 | <0.5 |
|  |  | 1400–1800 | 18.1 | 0.78 | dry | 1 km | 0.6-1.2 | 16992–33984 | <10 | 0.2 | <0.5 |
|  |  | 1000–1500 | 10.8 | ? | dry | 1 km |  |  | <10 |  | <0.5 |
|  |  | 750–950 | 4.8 | ? | dry | 1 km |  |  | <10 | 0.35 | <0.5 |
|  |  | 1600 | 5.7 | ? | dry | 1 km |  |  | <10 |  | <0.5 |
|  | (2) Sub-regional | 1000–1800 | 1132 | ? | dry | 1 km |  |  | 2 |  | <0.5 |
|  | (3) Regional | 1000–2700 | 5374 | ? | dry | 1 km |  |  |  |  | <0.5 |
| Coal Seam Gas (brown coal) | (1) Prospect/ field scale | 692 | 8.4 | ? | 30 m above top of coal | 354 m |  |  |  |  |  |
|  | (2) Sub-regional | 400–800 | 438 | 3.7 | 30 m above top of coal | 354 m |  |  |  |  |  |
|  | (3) Regional | 200–1200 | 2793 | ? | 30 m above top of coal | 354 m |  |  |  |  |  |

Note (a): Area as per Figure 131

Note (b): Not specific yield

The development scenarios (Table 20) considered in this study focused on the sub-regional extents as shown in Figure 132 and adopted various configurations as summarised in Table 21.  Scenario 1 and 2 were modelled by assigning drain cells to either layer 23 (Scenario 1) or layer 19 (Scenario 2) respectively.  Drain bed hydraulic conductivities were initially assumed to be 50% of the horizontal hydraulic conductivity in which the drain cells are located and calibrated thereafter to match extraction volumes estimated by the New York State Department of Environmental Conservation (2009) for tight and shale gas and Sander and Connell (2014) for coal seam gas.  Accordingly for coal seam gas developments the extraction volume was assumed to be 146 ML/yr/bore at 400 m spacing at start-up and for tight and shale gas the extraction volume was assumed to range between 2 and 5 ML/yr/km2.  These estimates were further tested through a model sensitivity procedure.  The drain elevations for Scenario 1 were set to the top of the uppermost modelled layer representing the assigned formation whereas for Scenario 2 the drain elevation was set at 20 metres above the top of the uppermost modelled layer representing the assigned formation.

Table 21: Basic description of the primary development scenarios.

| Scenario | Latrobe Valley coal mines | Groundwater licence entitlement | Offshore oil and gas | Onshore natural gas project |
| --- | --- | --- | --- | --- |
| Scenario 0 (average) (baseline) | 70% of the mine licence groundwater entitlement | Average usage over 10 years | Last 10 years | None |
| Scenario 0 (maximum) (sensitivity) | 100% of the mine licence groundwater entitlement | As above | As above | As above |
| Scenario 1 (tight and shale) | As above | As above | As above | See Table 20 – “Tight and shale, sub-regional” |
| Scenario 2 (coal seam gas) | As above | As above | As above | See Table 20 – “CSG, sub-regional” |

Tight and shale gas sub-region

Figure 132: Sub-regional tight and shale and coal seam gas extents.

Scenario model recharge and evapotranspiration rates

For each scenario considered, a synthetic annualised recharge and potential groundwater evapotranspiration rate was applied to all modelled years post 2012; the recharge and potential groundwater evapotranspiration rate for the period 1970-2012 was assumed to be as applied during the model initialisation and calibration/validation phases.  The synthetic annual sequences were based on simulated daily recharge and evapotranspiration derived using the CAT model for 2006.

Scenario modelling stress periods

Projected development time extended from 1 January 2013 to 31 December 2042 throughout which a three monthly stress period was adopted for all scenario modelling.  A recovery scenario, in which all abstractions were set to zero, extends from 2042 to 2125 and similarly adopted a three-monthly stress period.

Scenario model boundary conditions

All boundary conditions assigned for the simulation period 1970-2012 remain unchanged.  Post 2012 all boundary conditions as assigned to the last stress period of the verification period were applied throughout the projected potential onshore natural gas scenario simulation.

Scenario model rewetting setting

Modflow does not allow a cell that goes “dry” to return to “wet” (i.e. return to a value with a head of pressure or water level). In order overcome the possibility of solution cells going dry in a particular layer, Modflow-2005 has several options to enable cells to rewet.  Two options are available to convert dry cells to partially saturated cells.  Option 1 enforces rewetting based only on the head in the cell below whereas option 2 nominates that wetting is based on the heads from the four surrounding horizontal cells and the cell below.  In this study all simulations adopt rewetting option 2.

Scenario reporting bores

The changes in groundwater trends for all observation bores were considered and reported separately for each scenario.  Simulated groundwater hydrographs for all observation bores have been compiled into a database for review and analysis.

Scenario reporting regions

The scenarios are reported for the entire region, specifically the change in depth-to-watertable and groundwater heads in specified aquifers relative to baseline conditions.

Scenario reporting attributes

The key scenario assessments were based on groundwater levels in the Tertiary aquifers and maintenance of productive base.  The criteria considered the following attributes:

stabilisation of groundwater levels (key environmental function)

stabilisation of extraction (productive base)

prevention of dewatering confined aquifers (productive base)

maintenance of environmental river flows (key environmental outcome).

Scenario results

The scenario results are presented as impacts relative to a baseline condition in which monthly groundwater abstractions were fixed at averaged equivalent monthly values for the period 2003 to 2012; all other conditions were as applied in the 2001-2012 verification period.  In all cases recharge and groundwater potential evapotranspiration rates were based on 2006 daily climate as reported in Section 11.1.  Scenarios 1 and 2 adopted the upper limit of abstractions as referenced in Section 11.2 (5 ML/yr/km2 for tight and shale gas and 146 ML/yr/bore at 400 m2 spacing for coal seam gas), whereas all other scenarios adopt 50% abstractions of either scenario 1 or scenario 2 as applicable with the exception of scenario 7.  Scenario 7 applied the upper limit of coal seam gas abstractions to only half the area identified for potential onshore natural gas projects.  The area from which abstractions were applied was defined based on a region centred on the shallowest cell within the potential onshore coal seam gas zone.  Table 22 summarises the start-up abstraction volumes and area over which the abstractions are applied for each scenario.

The shallow watertable impacts in metres relative to the baseline estimates at 2042 are summarised in 
Table 23 for each of the seven scenarios considered.  Also tabled are the maximum drawdown (m) and mean drawdown (m) within an impacted area.  The impacted area is defined based on a drawdown threshold of 0.2 m below which no impact is assumed.  The drawdown threshold reflects the groundwater model solver tolerance with a fourfold confidence factor assigned.  The corresponding coal seam gas and tight and shale gas impacts are summarised in Table 24 and 
Table 25 respectively.  Results suggest that the greatest impacts on the shallow watertable are associated with coal seam gas development.

Table 22: Scenario configurations.

| Scenario | Description | Abstraction | Applied area (ha) |
| --- | --- | --- | --- |
| 1 | CSG only | 146 ML/yr/400 m2 | Entire potential area |
| 2 | Tight and shale only | 5 ML/yr/km2 | Entire potential area |
| 3 | CSG only at 50% | 73 ML/yr/400 m2 | Entire potential area |
| 4 | Tight and shale only at 50% | 2.5 ML/yr/km2 | Entire potential area |
| 5 | CSG and Tight and shale at 50% | As per scenarios 3 & 4 | Entire potential area |
| 6 | CSG only at 50% plus licence entitlement | As per scenario 3 + LE | Entire potential area |
| 7 | CSG only over half potential area | 146 ML/yr/400 m2 | Half potential area |

Table 23: Scenario shallow watertable impacts (m) relative to baseline at 2042.

| Scenario | Description | Max drawdown (m) | Mean drawdown (m) | Impacted area (ha) |
| --- | --- | --- | --- | --- |
| 1 | CSG only | 47.8 | 10.4 | 193,120 |
| 2 | Tight and shale only | 1.9 | 1.4 | 736 |
| 3 | CSG only at 50% | 34.2 | 6.7 | 170,384 |
| 4 | Tight and shale only at 50% | 1.4 | 1.2 | 64 |
| 5 | CSG and Tight and shale at 50% | 34.3 | 6.4 | 180,080 |
| 6 | CSG only at 50% plus licence entitlement | 34.3 | 5.8 | 208,256 |
| 7 | CSG only over half potential area | 47.8 | 5.6 | 127,952 |

Table 24: Scenario coal seam gas impacts (m) relative to baseline at 2042.

| Scenario | Description | Max drawdown (m) | Mean drawdown (m) | Impacted area (ha) |
| --- | --- | --- | --- | --- |
| 1 | CSG only | 220.9 | 25.2 | 424,192 |
| 3 | CSG only at 50% | 149.3 | 6.2 | 303,408 |
| 5 | CSG and Tight and shale at 50% | 130.9 | 17.9 | 344,816 |
| 6 | CSG only at 50% plus licence entitlement | 130.8 | 17.2 | 362,640 |
| 7 | CSG only over half potential area | 175.5 | 18.5 | 270,320 |

Table 25: Scenario tight and shale gas impacts (m) relative to baseline at 2042.

| Scenario | Scenario | Max drawdown (m) | Mean drawdown (m) | Impacted area (ha) |
| --- | --- | --- | --- | --- |
| 2 | Tight and shale only | 30.8 | 4.1 | 147,904 |
| 4 | Tight and shale only at 50% | 30.6 | 2.4 | 116,528 |
| 5 | CSG and Tight and shale at 50% | 30.5 | 7.3 | 384,032 |

The response of the water table in the affected area, and the gas resource layer following initiation of coal seam gas extractions (Scenario 1) on the impacted area (ha) and the average drawdown (m) within the impacted area are shown in Figure 133 and Figure 134 respectively.  Each figure shows two traces, one representing the shallow watertable impacts, the other showing the impact within the coal seam gas extraction layer.

Similar response trajectories arising from the initiation of tight and shale gas extractions (Scenario 2) are shown in Figure 135 and Figure 136. The simulated extraction volumes under nominated maximum and 50% coal seam gas and tight and shale gas conditions are shown in Figure 137 and Figure 138 respectively.

Key observations are:

The trajectories of impacted area are increasing and have not reached an equilibrium condition by 2042.

Coal seam gas extractions impact on the shallow watertable within 3 months following commencement of extractions.

Tight and shale gas extractions impact on the shallow watertable after 23 months following commencement of extractions.  This lag is not observed under the coal seam gas scenarios.

It is acknowledged that time lags predicted for water table response are likely to be under-estimated (i.e. the model responds more rapidly than would be the case in reality) because aquitards are generally represented as single layers which are unable to simulate the gradual vertical migration of pressure responses within these units.

The difference in the spatial impacts on shallow watertable and the assumed gas extraction layer for each of the seven scenarios compared to the baseline scenario (no gas development) are shown in Figure 139 to Figure 145.  Also shown are the outlines of either the coal seam gas or tight and shale development zones as appropriate.

Figure 133: “Coal seam gas affected area”: Scenario 1 total drawdown area (ha) on shallow watertable and T1 coal due to coal seam gas development.

Figure 134: “Coal seam gas drawdown”: Scenario 1 average drawdown (m) in the impacted shallow watertable and T1 coal due to coal seam gas development.

Figure 135: “Tight and shale gas affected area”: Scenario 2 total drawdown area (ha) on shallow watertable and Strzelecki Group due to tight and shale gas development.

Figure 136: “Tight and shale gas drawdown” Scenario 2 average drawdown (m) in the impacted shallow watertable and Strzelecki Group due to tight and shale gas development

Figure 137: Simulated coal seam gas extraction volume (ML/3 months) under maximum (Scenario 1) and 50% (Scenario 6) conditions.

Figure 138: Simulated tight and shale gas extraction volume (ML/3 months) under maximum (Scenario 2) and 50% (Scenario 4) conditions.

Figure 139: Scenario 1 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown (m) compared to baseline at 2042.

Figure 140: Scenario 2 shallow watertable (top) and tight and shale gas aquifer (bottom) drawdown (m) compared to baseline at 2042.

Figure 141: Scenario 3 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown (m) compared to baseline at 2042.

Figure 142: Scenario 4 shallow watertable (top) and tight and shale gas aquifer (bottom) drawdown (m) compared to baseline at 2042.

Figure 143: Scenario 5 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown (m) compared to baseline at 2042.

Figure 144: Scenario 6 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown (m) compared to baseline at 2042.

Figure 145: Scenario 7 shallow watertable (top) and coal seam gas aquifer (bottom) drawdown (m) compared to baseline at 2042.

Recovery scenario and results

The possible long term implications of a gas resource development on the water resources needs to consider the effects of time lag for overlying aquifers.  For the coal seam gas development, Figures 132 and 133 show both the area impacted and the drawdown in the area continue to increase however the trend to an asymptote or stable trend appears to be established. For the tight and shale gas developments, Figures 134 and 135 indicate the shallow water table drawdown and area of impact have already reached near stable values, indicating further impacts after development are unlikely to show an increase. These figures also show the time lag for the system is relatively short, as the onset of drawdown for the water table begins within 3 months after coal seam gas extraction begins, and less than 2 years after the beginning of tight and shale gas extraction.

A recovery scenario was applied to the model to evaluate the impact of cessation of coal seam gas abstractions on groundwater observation bores.  The application extended Scenario 1 above for a further 100 years during which coal seam gas abstractions were set to zero, all other conditions remaining unaltered.  This scenario considered the time required for the model to reach equilibrium and the recovery of the system, assuming the same “baseline condition” (off-shore, mine and licensed extraction) continued throughout the 100 years after the end of coal seam gas development and using the 2006 climate series.

Equilibrium was tested by considering the trend in calibration bore levels.  The hydrograph gradient of “typical” bores over 5 time steps at (a) the end of the transient model period (2012 or if not available then the last 5 recorded observation points), (b) the end of the development scenario (2042) and (c) the end of the extended period (2142) was assessed for return to historical trend; recovery was defined as a more positive trend relative to the baseline trend.  Results based on 766 groundwater calibration bore hydrograph trends estimate that 74% of calibration bores return to baseline or improved trends following cessation of coal seam gas abstractions after 100 years.  A summary of the results for each modelled layer in which calibration groundwater observation bores are assigned is tabulated in Table 26

Table 26: Summary of recovery results by modelled layer.

| Layer | Total number of bores | %Recover | %Not Recovered |
| --- | --- | --- | --- |
| 2 | 198 | 64 | 36 |
| 3 | 288 | 66 | 34 |
| 4 | 8 | 88 | 13 |
| 5 | 27 | 70 | 30 |
| 6 | 4 | 100 | 0 |
| 7 | 1 | 100 | 0 |
| 8 | 1 | 100 | 0 |
| 9 | 50 | 98 | 2 |
| 10 | 4 | 75 | 25 |
| 11 | 14 | 86 | 14 |
| 12 | 8 | 100 | 0 |
| 13 | 4 | 75 | 25 |
| 14 | 5 | 80 | 20 |
| 15 | 1 | 100 | 0 |
| 16 | 10 | 90 | 10 |
| 17 | 13 | 77 | 23 |
| 18 | 69 | 84 | 16 |
| 19 | 6 | 83 | 17 |
| 20 | 10 | 100 | 0 |
| 21 | 7 | 100 | 0 |
| 22 | 8 | 100 | 0 |
| 23 | 15 | 100 | 0 |
| 30 | 15 | 93 | 7 |

- Conclusions and recommendations

The developed groundwater model has been shown to broadly quantify groundwater flow and match historical hydrograph trends.  The groundwater model is capable of predicting relative changes in water level heads and baseflow estimates.  The model is deemed appropriate to assess the potential regional cumulative impacts of gas developments and other groundwater users (including coal mines and offshore oil and gas extraction) on existing groundwater users and the environment against a baseline condition of current development.

Key observations arising from this study are as follows:

The need to recognise the assumptions and limitations associated with the model so as to ensure the appropriate use of the model and interpretation of model predictions.  This groundwater model may be used to consider relative changes in the catchment water balance.  However, the spatial accuracy of simulation results may not correlate well with gauged data due to the model construct and spatial resolution and therefore should be used with regards to these considerations.

Caution should be exercised when using the solver option available in Groundwater Vistas by enabling the solution to continue if the convergence criteria are met for only the outer iterations without satisfying all convergence tolerances.

The following recommendations could improve the robustness and confidence of the groundwater model predictions as follows:

Improve knowledge of groundwater abstraction across the region. This should extend to the compilation of basic information including the (1) location of pumping wells, (2) the groundwater volumes abstracted and (3) the nature of the aquifers from which abstractions are sourced.

Translate the groundwater model into the recently developed USGS UnStructured Grid (USG) MODFLOW application would likely increase accuracy in the representation of processes in those regions of steep hydraulic gradients (such as near mines, near connected surface features, adjacent to groundwater extraction points and in regions of stream/aquifer interaction) thereby improving groundwater predictions and water budget estimates.  A preliminary unstructured grid has been developed for the study region as shown in Figure 146 and Figure 147.

Further assess groundwater recharge estimates and associated hydrograph response would likely increase mass balance prediction accuracy.

Incorporate temperature and density correction functions.

Review model layers assigned to groundwater extraction bores.

Review the attribution of the model layer representing the uppermost Strzelecki Formation.  This recommendation is based on analysis of the spatial residual error presented in this report.

Figure 146: Preliminary unstructured grid developed for the study area.

Figure 147: Zoomed in area within the preliminary unstructured grid highlighting the construct of the model 
and localised grid refinement adjacent to surface features.

- Glossary

| Term | Meaning | Units (if applicable) |
| --- | --- | --- |
| ALUM | Australia Land Use Mapping |  |
| aquifer | rock or soil that readily transmits water |  |
| aquitard | rock or soil that transmits water very slowly |  |
| baseflow | contribution of surface water flow due attributed to groundwater | ML |
| BFI | Base flow index [0–1] representing the percentage of streamflow due to groundwater inflows |  |
| BoM | Bureau of Meteorology |  |
| CAT | Catchment Analysis Tool biophysical catchment model |  |
| CD | coefficient of determination |  |
| CMA | Catchment Management Authority |  |
| confined aquifer | an aquifer in which an impermeable rock or soil layer or layers prevents water from seeping into the aquifer vertically |  |
| COMET3 | A reservoir simulator for gas shale, shale oil, and coalbed methane (CBM) reservoirs provided by Advanced Resources International Inc. |  |
| constant head boundary | time constant specified head which represents flows into or out of the model domain where groundwater connects or interacts with features (and the ocean) outside the model domain | mAHD |
| CSG | coal seam gas |  |
| CSIRO | Commonwealth Scientific and Industrial Research Organisation |  |
| DEDJTR | Department of Economic Development, Jobs, Transport and Resources |  |
| DELWP | Department of Environment, Land, Water and Planning |  |
| DEM | digital elevation model defining surface elevations in mAHD |  |
| DEPI | Department of Environment and Primary Industries |  |
| drain boundary | Represents a head dependent boundary condition where water is removed from the model depending on the specified head elevation, the predicted head in neighbouring cells and the specified boundary conductance term. |  |
| drawdown | Reduction in groundwater head elevation relative to a nominated baseline condition. | m |
| DSE | former Department of Sustainability and Environment |  |
| Term | Meaning | Units (if applicable) |
| ET | water lost due to a combination of soil evaporation and vegetation transpiration | m |
| GA | Geoscience Australia |  |
| GDE | groundwater-dependent ecosystem |  |
| GMA | Groundwater Management Area |  |
| hydraulic head |  | m |
| hydraulic conductivity (Kxy) |  | m/day |
| IRM | Integrated Resource (groundwater) Model |  |
| LVCM | Latrobe Valley Coal Model |  |
| LVRM | Latrobe Valley Resource Model |  |
| mAHD | Metres Australian Height Datum | m |
| MDBA | Murray-Darling Basin Authority |  |
| MDBC | Murray-Darling Basin Commission |  |
| MID | Macalister Irrigation District |  |
| no flow boundary | Represents locations where groundwater does not flow and/or the aquifer is absent; such features include groundwater divides (specified flow boundary type). |  |
| NPA | National Partnership Agreement |  |
| NSI | Nash-Sutcliffe index [0-1] |  |
| permeability |  |  |
| PPF | Principal Profile Form used to describe soil classifications |  |
| porosity |  |  |
| potentiometric surface |  |  |
| quickflow | stream flow component attributed to surface runoff and sub-surface flow | ML |
| river boundary | Represents a head dependent boundary condition where groundwater can either recharge or discharge into/from the model based upon a specified head elevation, the model-predicted head in neighbouring cells and a specified boundary conductance term. |  |
| recharge rate | water that flows below the root zone and enters the groundwater | m/day |
| residual error |  |  |
| root mean fraction square (RMFS) |  | % |
| SAFE | Victorian Secure Allocation Future Entitlement |  |
| scaled mean sum of residuals (SMSR) |  | % |
| scale root mean square (SRMS) |  | % |
| scaled root mean fraction square (SRMFS) |  | % |
| Term | Meaning | Units (if applicable) |
| SKM | Sinclair Knight Merz Pty Ltd |  |
| specific yield (Sy) |  |  |
| specific storage (Ss) |  | m–1 |
| steady-state condition | Represents an equilibrium condition based on constant inputs in which time is indeterminate. |  |
| sum of residuals (Sr) |  | m |
| sum of squares (SSQ) |  | m2 |
| SVERS | Strengthening Victoria’s Earth Resources Sector Initiative |  |
| TDS | total dissolved solids | mg/L |
| transient | time-varying |  |
| transmissivity |  | m2/day |
| unconfined aquifer |  |  |
| VAF | Victorian Aquifer Framework |  |
| vertical hydraulic conductivity (Kz) |  |  |
| watertable | the “surface” where the groundwater level is balanced against atmospheric pressure | mAHD |
| WSPA | Water Supply Protection Area |  |
| well boundary | represents locations where fluxes are applied to the model (on a layer-by-layer basis); they are used to represent groundwater extraction from stock, domestic, industrial bores and from groundwater pumping in offshore oil and gas fields |  |
| yield |  | L/s |

- References

Aquaterra, 2008. Ashton Underground Mine Pikes Gully Seam groundwater modelling report. Report submitted to ACOL in support of the Longwall/Miniwall 5-9 SMP Application.

Arnold, J.G., Allen, P.M., Muttiah, R., Bernhardt, G., 2010. Automated base flow separation and recession analysis techniques.

Barnett, B., Townley, L.R., Post, V., Evans, R.E., Hunt, R.J., Peeters, L., Richardson, S., Werner, A.D., Knapton, A. and Boronkay, A., 2012.  Australian groundwater modelling guidelines, Waterlines report, National Water Commission, Canberra.

Barton, C.M., 1971. The Morwell "interseam" sands. Journal of the Geological Society of Australia 17(2): 191–204.

Batu, V., 1998. Aquifer Hydraulics: A comprehensive guide to hydrogeologic data analysis, John Wiley & Sons, New York.

Beverley, C., 2009. Models of the Catchment Analysis Tool (CAT1D Version 32), August 2009, Technical Reference Manual (ed. C. Beverly), Department of Primary Industries, Victoria.

Beverly, C., Bari, M., Christy, B., Hocking, M., Smettem, K., 2005. Predicted salinity impacts from land use change: a comparison between rapid assessment approaches and a detailed modelling framework. Australian Journal of Experimental Agriculture 45, 1453–1469.

Beverly, C., Hocking, M., Day, G., 2012. Understanding the potential significance of energy and earth resource development on water resources. Technical Report. Department of Primary Industries, Victoria, pp.66.

Blake, W.J.R., 1972. An investigation of a groundwater supply for the township of Trafalgar, Parish of Moe. Report No. 1972/3, Geological Survey of Victoria.

Bolger, P.F., 1987. Regional groundwater investigations of the Latrobe Valley — Stage III mathematical modelling studies. Geological Report 41, Coal Resources Division, Fuel Department, State Electricity Commission of Victoria.

Bolger, P., 1990. Mathematical modelling studies for predicting artesian dewatering requirements at Loy Yang Open Cut, Fuel Department, State Electricity Commission of Victoria.

BRS, 2006.  Guidelines for land use mapping in Australia: principles, procedures and definitions (3rd edn), Australia, Bureau of Rural Science, Canberra.

Brumley, J.C., Barton, C.M., Holdgate, G.R. and Reid, M.A., 1981. Regional groundwater investigation of the Latrobe Valley: 1976-1981. State Electricity Commission of Victoria and Victorian Department of Minerals and Energy, Melbourne.

Brumley, J.C., and Holdgate, G.R., 1983. An assessment of the confined groundwater resources of the Moe Swamp Basin, Victoria, Australia. International Conference on Groundwater and Man, vol. 1, pp. 41–50. Department of Resources and Energy, Canberra

Chiew, F.H.S., Stewardson, M.J., McMahon, T.A., 1993. Comparison of six rainfall-runoff modelling approaches. Journal of Hydrology 147, 1–36.

Constantine, A., 2001. Sedimentology, Stratigraphy and Palaeoenvironment of the Upper Jurassic – Lower Cretaceous Non-Marine Strzelecki Group, Gippsland Basin, Southern Australia, PhD Thesis, Monash University.

Dahlhaus, P.G., Heislers, D.S., Brewin, D., Leonard, J.L., Dyson, P.R., Cherry, D.P., 2004. Port Phillip and Westernport groundwater flow systems. A report prepared for the Port Phillip and Westernport Catchment Management Authority, Frankston, Victoria.

DCNR, 1995. Eastern Victoria Regional Aquifer Systems, Victorian Groundwater Beneficial Use Map Series. Department of Conservation and Natural Resources, Victoria.

DEPI, 2014. Watercourse Network 1:25,000 – Vicmap Hydro. Spatial Information Infrastructure (DSE), Department of Environment and Primary Industries, Victoria.

Doherty, J., 2010. PEST Model-independent parameter estimation, User Manual (5th edn). Watermark Numerical Computing.

Domenico, P.A. and F.W. Schwartz, 1990. Physical and chemical hydrogeology. John Wiley & Sons, New York.

Douglas, J.G., Spencer Jones, D., Wail, S., McInnes, D.W. 1971. Warragul 1:250 000 Scale Geological Map. Sheet SJ 5510. Geological Survey of Victoria.

DSE, 2012. Victorian groundwater-surface water interaction spatial data. Compiled for the Secure Allocation, Future Entitlements (SAFE) Project by the Department of Sustainability and Environment, Victoria.

DSITI 2015. Queensland Department of Science, Information Technology, Innovation and the Arts SILO Climate data, https://www.longpaddock.qld.gov.au/silo/. [15 May 2015]

Evans, R.S., 1983. Regional groundwater investigation of the Latrobe Valley, Stage 2 mathematical modelling studies. Fuel Department Report DD187. State Electricity Commission of Victoria (unpubl.).

Fraser, C.J., 1980. Regional groundwater investigation of the Latrobe Valley — Initial mathematical modelling studies Volume 1. Fuel Department Report DD 149. State Electricity Commission of Victoria (unpubl.).

Freij-Ayoub, R., Strand, J., Ahmed, S., 2011a. Coal seam gas water production management. Project Report EP114078. CSIRO Earth Science and Resource Engineering.

Freij-Ayoub, R., Strand, J., Ahmed, S., 2011b. A Numerical Model for the Prediction of Produced Methane and Water from Coal Seams and the Associated Ground Surface Subsidence. 4th International Conference GeoProc2011: Cross Boundaries Through THMC Integration, Perth, Western Australia, 6–9 July 2011.

Gallant, J.C., Dowling, T.I., Read, A.M., Wilson, N., Tickle, P., Inskeep, C., 2011. 1 second SRTM Derived Digital Elevation Models user guide. Geoscience Australia (www.ga.gov.au/topographic-mapping/digital-elevation-data.html).

Geo-Eng, 1993. Aquifer depressurisation report. Report No. 1204/1. Geo-Eng Pty Ltd. (unpubl.)

Geo-Eng, 1994. Estimates of long term pumping requirements for Loy Yang Mine. Report No. 1205/3. Geo-Eng Pty Ltd (unpubl.).

Geo-Eng, 1996a. Predictive modelling of the aquifer depressurisation requirements for Latrobe Valley Mines. Report 1205/10. Geo-Eng Pty Ltd (unpubl.).

Geo-Eng, 1996b. The Latrobe Valley regional groundwater model and the Loy Yang Mine groundwater model - A review of work completed to October 1996. Draft Report 1205/12. Geo-Eng Pty Ltd (unpubl.)

Geo-Eng, 1999. Aquifer depressurisation strategic assessment. Report prepared by Geo-Eng Pty Ltd for Loy Yang Power Mine (unpubl.).

Geo-Eng, 2000. Latrobe Valley regional groundwater and land surface monitoring report. Five year review as at June 2000. Report 1000/8505/99. Geo-Eng Pty Ltd.

Geo-Eng, 2001. Aquifer depressurisation strategic assessment. A unpubl. report prepared by Geo-Eng Pty Ltd.  Report No 1100/1204/21. Geo-Eng Pty Ltd.

GHD, 2006. Latrobe Valley regional groundwater and land level monitoring report. Five year review. Report prepared by GHD.

GHD, 2008a. East Gippsland CMA Groundwater Model, Interim steady-state model development report. Report prepared by GHD for Department of Sustainability and Environment, Victoria.

GHD, 2008b. Hydrogeological assessment to support licence application BCL 9030173. Report prepared by GHD for East Gippsland Water.

GHD, 2010a.  West Gippsland CMA Groundwater Model. Interim steady-state model development report.  Report prepared by GHD for the Department of Sustainability and Environment, Victoria.

GHD, 2010b. West Gippsland CMA Groundwater Model. Transient model development report. Report prepared by GHD for Department of Sustainability and Environment, Victoria.

GHD, 2010c.  East Gippsland CMA Groundwater Model. Transient model development report.  Report for the Dept. of Sustainability and Environment, Victoria.

GHD, 2010d.  West Gippsland CMA Groundwater Model. Transient model development report.  Report for the Dept. of Sustainability and Environment, Victoria.

GHD, 2011a. Latrobe Valley Coal Model completion report. Report prepared by GHD for Clean Coal Victoria, Department of Primary Industries, Victoria.

GHD, 2011b. Latrobe Valley regional groundwater and land level monitoring report. Five Year Review. Prepared by GHD.

GHD, 2012a. Victorian Aquifer Framework updates for seamless mapping of aquifer surfaces. Report prepared by GHD for for the Secure Allocation Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria.

GHD, 2012b. Development of State Wide 3D Aquifer surfaces. Report prepared by GHD for the Secure Allocation, Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria.

GHD, 2013. Groundwater Assessment – baseflow dependent rivers characterising groundwater contribution to baseflow dependent waterways.  Report prepared for the Department of Environment and Primary Industries, Victoria.

GHD, 2014a. Groundwater impacts and management for lignite mining in the Latrobe Valley. Report prepared for the Dept. of State Development, Business and Innovation, January 2014.

GHD, 2014b. Statewide groundwater level reporting. Report prepared by GHD for Department of Environment and Primary Industries, Victoria.

Golder Associates, 1990. Loy Yang regional groundwater model. Report No 89612362, prepared by Golder Associayes Pty Ltd for State Electricity Commission of Victoria (unpubl.).

Golder Associates, 1991. Groundwater model for Loy Yang and Morwell and open cuts, (Interim report). Report No. 91612011. Golder Associates Pty Ltd.

Golder Associates, 1992a. Update and recalibration of groundwater model — progress report. Report 92612275/02. Report prepared by Golder Associates Pty Ltd for State Electricity Commission of Victoria.

Golder Associates Pty Ltd, 1992b. Groundwater model for the Loy Yang and Morwell open cuts, Latrobe Valley, Victoria. Second Interim Report. Report No. 91612011.

Golder Associates Pty Ltd, 1992c. Groundwater Model for the Loy Yang and Morwell Open Cuts, Latrobe Valley, Victoria (Third Interim Report). 92612275/09, Geo Eng Pty Ltd.

Goldie Divko, L.M., 2015.  A review of natural gas prospectivity in the Gippsland Region. Report prepared for the Department of Economic Development, Jobs, Transport and Resources, Victoria.

Harbaugh, A.W., 2005, MODFLOW-2005, The U.S. Geological Survey modular ground-water model—the ground-water flow process. Techniques and Methods Report 6-A16. U.S. Geological Survey.

Harlow, G.E., LeCain, G.D., 1993. Hydraulic characteristics of, and ground-water flow in, coal-bearing rocks of southwestern Virginia. Water-supply Paper 2388. U.S. Geological Survey.

Hatton, T., Otto, C. and Underschultz, J. (2004) Falling water levels in the Latrobe Aquifer, Gippsland Basin: Determination of cause and recommendations for future work.  Prepared by CSIRO Petroleum Resources Division.

Heislers, D., 1993. Groundwater in the Central Victorian Highlands. Water Division, Department of Conservation and Natural Resources, Victoria.

Henzell, S.T., Irrgang, H.R., Jenssen, E.J., Mitchell, R.A.M., Morrell, G.O., Palmer, I.D., Seage, N.W., Hicks, G.J., Hordern, M.J., and Kable, C.W, 1985. Fortescue reservoir development and reservoir studies. The APEA Journal 25(1): 95-106.

Hofmann H., 2011. Understanding connectivity within groundwater systems and between groundwater and rivers, PhD Thesis, Monash University, Clayton, Victoria.

Holdgate, G.R., Gallagher, S.J., 2003. Tertiary. In: Birch, W.D. (ed.), 2003. Geology of Victoria, pp. 289–335. Geological Society of Australia Special Publication 23. Geological Society of Australia (Victoria Division).

Hutchinson, M.F., 2001. ANUClim Version 5.1. Available online at http://cres.anu.edu.au/outputs/anuclim.php (verified 29 July 2004).

Jansen, B. & Maher, S., 2003.  Digital geological model of the Latrobe Valley Coal Resource.  Report 2003/2. Geological Survey of Victoria (unpubl.).

Jeffrey, S., Carter, J., Moodie, K., Beswick, A., 2001. Using spatial interpolation to construct a comprehensive archive of Australian climate data. Environmental Modelling and Software 16: 309–330.

Jones, T., Flynn, T., Goode, A., 2009. Interconnected Systems: Groundwater and Surface Water Interaction In The Lower Avon Catchment, Gippsland. In proceedings of the Irrigation and Drainage Conference, Swan Hill, 18–21 October 2009.

Leonard, J.G., 2006. Hydrogeology of the Melbourne area. Australian Geomechanics 41(3) 63–74.

Leonard, J.G. 2003. Groundwater: a vital renewable resource. In: Birch, W.D. (ed.), 2003. Geology of Victoria, pp. 519–531. Geological Society of Australia Special Publication 23. Geological Society of Australia (Victoria Division).

McKenzie. N.J., Jacquier, D.W., Ashton, L.J., Cresswell, H.P., 2000. Estimation of Soil Properties Using the Atlas of Australian Soil, CSIRO Land and Water Technical Report 11/00. CSIRO Land and Water, Canberra

Middlemis, H., 2000.  Murray Darling Basin Commission Groundwater Flow Modelling Guidelines. Report prepared for the Murray Darling Basin Commission by Aquaterra Pty Ltd.

Middlemis, H., Merrick, N., Ross, J., 2000. Groundwater Modelling Guidelines. Report prepared for the Murray Darling Basin Commission by Aquaterra Pty Ltd.

Mollica, F.J., 1991. Results of pumping test carried out on Bore 3468-15060, Sale. Technical Note TN/5. Rural Water Commission of Victoria.

Nahm, G.Y., 1972. Report on the pumping test of Bore No. 12, Parish of Harrow. Report No. 1972/6 Geological Survey of Victoria (unpubl.).

Nahm, G.Y., 1973a. Report on Bore No. 5, Parish of Wy Yung.  Report No. 1973/71. Geological Survey of Victoria (unpubl.).

Nahm, G.Y., 1973b. Report on Bore Nos. 7, 8 and 9, Parish of Wy Yung. Report No. 1973/72. Geological Survey of Victoria (unpubl.).

Nahm, G.Y., 1974. Geology and groundwater resources of the Thorpdale area including aquifer test of Bore No. 46, Parish of Moe. Report No. 1974–36. Geological Survey of Victoria (unpubl.).

Nahm, G.Y., 1977. Groundwater resources in Gippsland. Report No. 1977/1, Geological Survey of Victoria.

Nahm, G.Y., 2002. The hydrogeology of the Gippsland Basin and its role in the genesis and accumulation of petroleum. PhD Thesis, University of Melbourne.

Nahm, G.Y., Reid, M.A., 1979a. Report on the aquifer tests of the D.M.E. and S.E.C. joint project (1), Bore No. 197, Parish of Willung. Report No. 1979/29. Geological Survey of Victoria (unpubl.).

Nahm, G.Y., Reid, M.A., 1979b. Report on the aquifer tests of the D.M.E. and S.E.C. joint project (3), Bore No. 96, Parish of Coolungoolun. Report No. 1979/58. Geological Survey of Victoria (unpubl.).

Nahm, G.Y., Reid, M.A., 1979c. Report on the aquifer tests of the D.M.E. and S.E.C. joint project (5), Bore No. 307, Parish of Rosedale. Report No. 1979/61. Geological Survey of Victoria (unpubl.).

Nahm, G.Y., Reid, M.A., 1979d. The second aquifer tests on Bore Nos. 17 and 19, Parish of Sale. Report No. 1979/70.1979/64. Geological Survey of Victoria (unpubl.).

Nahm, G.Y., Reid, M.A., 1979e. Report on the aquifer tests of the D.M.E. and S.E.C. joint project (6), Bore No. 286, Parish of Traralgon, Report No. 1979/70. Geological Survey of Victoria (unpubl.).

Nash, J.E., Sutcliffe, J.V., 1970. River flow forecasting through conceptual models, 1. A discussion of principles. Journal of Hydrology 10, 282–290.

New York State Department of Environmental Conservation, 2009. Draft Supplemental Generic Environmental Impact Statement (SGEIS) on the Oil, Gas, and Solution Mining Regulatory Program, Albany, New York, USA. Chapter 6. Potential Environmental Impacts. NYSDEC.

Northcote, K., 1979. A factual key for the recognition of Australian soils. Rellim Technical Publications.

Osbourne, C., 2014, Updates to the Latrobe Valley Coal Model. Prepared by Geological Survey of Victoria, Department of State Development and Business Innovation.

PB, 2010a. Review of West Gippsland CMA groundwater model. Transient Model Development Report. Prepared by Parsons Brinckerhoff for the Department of Sustainability and Environment, Victoria.

PB, 2010b. Review of East Gippsland CMA groundwater model. Transient model development report. Prepared by Parsons Brinckerhoff for the Department of Sustainability and Environment, Victoria.

PDA, 2006. Ashton Coal Mine Longwall Panels 1–4 Subsidence Management Plan; groundwater assessment. Report prepared by Peter Dundon and Associates Pty Ltd for Ashton Coal Operations Ltd.

Pratt M., 1985. Groundwater investigation — Shires of Bass and Phillip Island. Report No. 1985/48. Geological Survey Victoria (unpubl.).

Reid, M.A., 1985a. Completion Report on Sale 21, Sale town supply bore, Report No. 1985/57. Geological Survey Victoria (unpubl.).

Reid, M.A., 1985b. Narracan 3737 Latrobe Valley Groundwater Investigation, Report No. 1985/66. Geological Survey Victoria (unpubl.).

Sander, R. and Connell, L.D., 2014.  Potential impacts of CSG production on groundwater resources – field scale CSG modelling and impact assessment. CSIRO.

Schaeffer, J., 2008.  Scaling Point Based Aquifer Data for Developing Regional Groundwater Models: Application to the Gippsland Groundwater System. PhD thesis. The University of Melbourne.

Shah, N., Nachabe, M. and Ross, M. (2007), Extinction Depth and Evapotranspiration from Ground Water under Selected Land Covers. Groundwater, 45: 329–338. doi: 10.1111/j.1745-6584.2007.00302.x

Shugg, A., Harris, I.F., 1975. Evaluation of pumping test data of an unconfined fractured rock aquifer, Monbulk 3, Parish of Monbulk. Report No. 1975/47. Geological Survey Victoria (unpubl.).

SKM, 1995. Groundwater modelling of the Generation Victoria and Esso/BHP drawdown cones. Report prepared by Sinclair Knight Merz for the Department of Conservation and Natural Resources, Victoria (unpubl.).

SKM, 1996. Numerical modelling of declining groundwater levels in Gippsland. Report prepared by Sinclair Knight Merz for the Department of Conservation and Natural Resources, Victoria (unpubl.).

SKM, 1999. Yarram subregional model. Report prepared by Sinclair Knight Merz for the Department of Natural Resources and Environment, Victoria (unpubl).

SKM, 2001. Gippsland subsidence modelling — Yarram. Report prepared by Sinclair Knight Merz for the Department of Natural Resources and Environment, Victoria (unpubl).

SKM, 2006. Sale Water Supply Protection Area Groundwater Resource Appraisal. Report WC03438.1, prepared by Sinclair Knight Merz for Southern Rural Water.

SKM, 2008a. Sale WSPA Groundwater Model. Report prepared by Sinclair Knight Merz.

SKM, 2008b. Investigation of groundwater/surface water interaction in the Avon River catchment. A report prepared by Sinclair Knight Merz for the Western Gippsland Catchment Management Authority. Final Report. December 2008.

SKM, 2011a. Gippsland Basin Model Scoping Study. Review of groundwater management tools in the Gippsland Basin. Report prepared by Sinclair Knight Merz.

SKM, 2011b. A review of the Victorian Aquifer Framework. Report prepared by Sinclair Knight Merz for the Secure Allocation, Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria.

SKM, 2012a. Impacts of groundwater extraction on streamflow in selected catchments throughout Australia. Waterlines Report Series No 84.  Report prepared by Sinclair Knight Merz for the National Water Commission, Canberra.

SKM, 2012b. Baseflow separation and analysis. Report prepared by Sinclair Knight Merz for the Secure Allocation, Future Entitlements (SAFE) Project, Department of Sustainability and Environment, Victoria.

SKM, 2014. SOBN hydrograph review: State-wide groundwater hydrograph catalogue. Report prepared by by Sinclair Knight Merz for the Department of Environment and Primary Industries, Victoria.

SKM, GHD, 2009. Hydrogeological Mapping of Southern Victoria. July 2009. Report prepared by Sinclair Knight Merz and GHD for Southern Rural Water.

Smith, C., 2002. 1:250 000 Statewide Soil Attribute Coverage Documentation. Centre for Land Protection Research: Department of Primary Industries, Victoria.

Stuart-Smith, P., Gou, B., Cunningham, M. M., Prendergast, L., Monoury, S., 2010. Gippsland Basin Study Modules: Onshore & Offshore, Department of Primary Industries, Victoria. The Cooperative Research Centre for Greenhouse Gas Technologies, Canberra.

Szabo, L.G., 1979. Pumping test on Bore 2, Mount Franklin Mineral Water Spring. Scanex Minerals Pty Ltd, Melbourne.

Thatcher, R.C., 1976. Loy Yang Project aquifer characteristic from pump tests conducted during 1975. DD111, Fuel Department, Development and Planning Division, State Electricity Commission of Victoria (unpubl.).

Thompson, B.R., 1968. Longford Gippsland, Glenco South 80001 Esso BHP Water Bore No 2.

Tilleard, J.W., O’Connor, N., Boon, P.I., 2009. Understanding the environmental water requirements of the Gippsland Lakes system: Scoping Study, report by Moroka Pty Ltd, Ecos Environmental Consulting and Dodo Environmental for the East and West Gippsland Catchment Management Authorities.

Todd, D.K., 1980. Groundwater hydrology (2nd edn). John Wiley & Sons, New York.

Underschultz, J., Bailey, W., Freij-Ayoub, R., Otto, C., 2006. Wealth from oceans. Falling water levels in the Latrobe aquifer, Gippsland Basin. Offshore aquifer update, onshore fault seal analysis, and preliminary numerical simulation of coastal subsidence risk. National Research Flagships Report, CSIRO.

Unland, N.P., Cartwright, I., Andersen, M.S., Rau, G.C., Reed, J., Gilfedder, B.S., Atkinson, A.P., Hofmann, H., 2013. Investigating the spatio-temporal variability in groundwater and surface water interactions: a multi-technique approach. Hydrology and Earth Systems Science 17: 3437–3453.

USQ, 2011. Preliminary assessment of cumulative drawdown impacts in the Surat Basin associated with the coal seam gas industry. Investigation of parameters and features for a regional model of Surat Basin coal seam gas developments. University of Southern Queensland.

van Genuchten, M. Th., Leij, F. J., Yates, S. R., 1991. The RETC code for quantifying the hydraulic functions of unsaturated soils. Report No. EPA/600/2-91/065. R. S. Kerr Environmental Research Laboratory, U.S. Environmental Protection Agency, Ada, Oklahoma.

Varma, S., Karsten, M., Bekele, E. and Ciftci, B. (2012).  Hydrodrnamic modelling of the Gippsland Basin.  Prepared by CSIRO Petrolium and Geothermal Research Division, Perth in collaboration with the Department of Primary Industries, Victoria.  Unpublished report

Walker, G., Molica, F., 1990. Review of the Groundwater resources in the South East Region. A report for the Department of Water Resources Victoria. South East Region Water Management Strategy. Water Resources Management Report Series. No. 54.  Department of Water Resources, Victoria

WMIS, DELWP Water Management Information System, Water monitoring data, Available from <http://data.water.vic.gov.au/monitoring.htm>. [15 May 2015]

- Appendix A: Baseflow estimates

East Gippsland

| Site ID | Site name | Easting | Northing | Gauge zero (Ahd) | Site start date | Site cease date | Drainage area (km2) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 221001 | GENOA RIVER @ ROCKTON | 705825 | 5887218 | 414.72 | 26/05/1993 |  | 125.6 |  | 0.28 |
| 221201 | CANN RIVER (WEST BRANCH) @ WEERAGUA | 694682 | 5861473 | 153.96 | 21/02/1957 |  | 311 |  | 0.51 |
| 221202 | GENOA RIVER @ WANGARABELL | 721597.1 | 5858290.5 | 0 | 13/10/1927 | 30/06/1929 | 780 |  | 0.42 |
| 221203 | BETKA RIVER @ MALLACOOTA | 737895.1 | 5836091.4 | 0 | 26/01/1966 | 24/07/1972 | 117 |  | 0.37 |
| 221204 | THURRA RIVER @ POINT HICKS | 699200 | 5822200 | 0 | 17/05/1966 | 28/07/1978 | 345 |  | 0.43 |
| 221205 | BEMM RIVER @ BEMM RIVER | 671893.8 | 5821410.8 | 0 | 28/06/1966 | 5/06/1975 | 935 |  | 0.55 |
| 221206 | CANN RIVER @ NOORINBEE | 694397 | 5854712.4 | 0 | 31/05/1967 | 3/02/1971 | 541 |  | 0.43 |
| 221207 | ERRINUNDRA RIVER @ ERRINUNDRA | 669418 | 5853798 | 0 | 28/07/1971 |  | 162 |  | 0.65 |
| 221208 | WINGAN RIVER @ WINGAN INLET NATIONAL PARK | 719710.3 | 5825779.5 | 0 | 14/12/1982 |  | 420 |  | 0.39 |
| 221209 | CANN RIVER (EAST BRANCH) @ WEERAGUA | 695470 | 5863398 | 0 | 23/10/1972 |  | 154 |  | 0.36 |
| 221210 | GENOA RIVER @ THE GORGE | 723046 | 5856007 | 0 | 22/08/1972 |  | 837 |  | 0.34 |
| 221211 | COMBIENBAR RIVER @ COMBIENBAR | 675439 | 5854433 | 0 | 12/08/1974 |  | 179 |  | 0.48 |
| 221212 | BEMM RIVER @ PRINCES HIGHWAY | 667855 | 5836059 | 0 | 30/04/1975 |  | 725 |  | 0.59 |
| 221213 | GOOLENGOOK RIVER @ D/S OF ARTE RIVER JUNCTION | 663700 | 5846900 | 0 | 1/05/1978 | 18/12/1986 | 171 |  | 0.69 |
| 221214 | CANN RIVER @ D/S OF CANN RIVER | 688655 | 5836283 | 0 | 26/07/1979 |  | 675 |  | 0.44 |
| 221216 | GENOA RIVER @ D/S OF BIG FLAT CREEK JUNCTION | 722308.7 | 5858209.9 | 0 | 23/05/1967 | 10/12/1970 | 829 |  | 0.43 |
| 221217 | GENOA RIVER @ GIPSY POINT (WOOD'S JETTY) | 736900 | 5848600 | -1.3 | 8/07/1992 | 3/11/1998 | 0 |  |  |
| 221218 | BETKA RIVER @ MINERS TRACK | 732688 | 5835801 | 0 | 16/01/1996 |  | 0 |  | 0.29 |
| 221222 | HENSLEIGH CREEK U/S COMBIENBAR | 679158 | 5863650 | 0 | 18/05/2006 | 17/11/2010 | 0 |  |  |
| 221223 | COMBIENBAR RIVER @ TIGER SNAKE CK | 680158 | 5864336 | 0 | 18/05/2006 | 17/11/2010 | 0 |  |  |
| 221224 | CANN RIVER U/S CANN RIVER OFFTAKE | 690544 | 5843171 | 0 | 11/03/2009 |  | 0 |  | 0.52 |
| 221225 | BEMM RIVER U/S OF PUMPHOUSE | 671929 | 5822377 | 0 |  |  | 0 |  | 0.44 |
| 221800 | RAINGAUGE (GENOA RIVER) @ THE GORGE | 722715 | 5855916.4 | 0 | 22/08/1972 |  | 0 |  |  |

Latrobe Basin

| Site ID | Site name | Easting | Northing | Gauge zero (mAHD) | Site start date | Site cease date | Drainage area (km2) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 226005 | LATROBE RIVER @ THOMS BRIDGE | 448418 | 5775480 | 0 | 24/05/1960 |  | 2657 |  | 0.57 |
| 226006 | TYERS RIVER @ BOOLA | 448568.9 | 5781803 | 112.027 | 16/04/1958 |  | 293 |  | 0.44 |
| 226007 | TYERS RIVER @ BROWNS | 443476 | 5788600 | 169.359 | 17/08/1961 |  | 207 |  | 0.60 |
| 226008 | TYERS RIVER WEST BRANCH @ MORGANS MILL | 439256 | 5800120 | 0 | 27/04/1960 |  | 80 |  | 0.64 |
| 226012 | TANJIL RIVER EAST BRANCH @ TANJIL BREN | 429307.7 | 5812543.1 | 0 | 18/01/1961 | 11/08/1971 | 12 |  | 0.65 |
| 226016 | WATERHOLE CREEK @ MORWELL | 449045.2 | 5767997.4 | 60.56 | 22/08/1961 | 15/02/1982 | 41 |  | 0.42 |
| 226017 | JACOBS CREEK @ O'TOOLES | 446350 | 5788246 | 0 | 16/04/1962 |  | 36 |  | 0.43 |
| 226021 | NARRACAN CREEK @ MOE | 435969 | 5775832 | 0 | 26/06/1996 |  | 0 |  | 0.69 |
| 226023 | TRARALGON CREEK @ TRARALGON | 460064 | 5772541 | 32.673 | 18/09/1998 |  | 189 |  | 0.48 |
| 226027 | LATROBE RIVER AT SWING BRIDGE | 511200 | 5778084 | 0 | 7/05/2010 |  | 0 |  | 0.41 |
| 226028 | TYERS R @ PUMP HOUSE | 451503 | 5778560 | 0 | 5/04/2007 |  | 0 |  | 0.63 |
| 226033 | LATROBE RIVER @ SCARNES BRIDGE | 460763 | 5777041 | 21.185 | 20/12/1996 |  | 0 |  | 0.44 |
| 226039 | BILLY CREEK @ U/S OF OFFTAKE WEIR | 446116.2 | 5755155.9 | 0 | 2/03/1967 | 8/05/1968 | 22 |  | 0.57 |
| 226041 | LAKE WELLINGTON @ BULL BAY | 532876 | 5780415 | 0 | 25/02/1991 |  | 0 |  |  |
| 226202 | TYERS RIVER @ GOULD | 444451.2 | 5785659.5 | 0 | 19/05/1926 | 28/02/1941 | 220 |  | 0.59 |
| 226204 | LATROBE RIVER @ WILLOW GROVE | 426308 | 5784017 | 0 | 17/10/1966 |  | 580 |  | 0.74 |
| 226205 | LATROBE RIVER @ NOOJEE | 414035 | 5804037 | 0 | 1/05/1996 |  | 290 |  | 0.79 |
| 226206 | LATROBE RIVER @ APM MARYVALE | 452119.2 | 5774427.3 | 27.546 | 2/01/1946 | 3/05/1977 | 3002 |  | 0.62 |
| 226209 | MOE RIVER @ DARNUM | 412633 | 5770880 | 79.598 | 19/10/1960 |  | 214 |  | 0.48 |
| 226216 | TANJIL RIVER @ TANJIL SOUTH | 433560 | 5783395 | 69.245 | 5/04/1955 |  | 358 |  | 0.70 |
| 226217 | LATROBE RIVER @ HAWTHORN BRIDGE | 419490.7 | 5796547.9 | 0 | 2/06/1955 | 9/01/1989 | 440 |  | 0.73 |
| 226218 | NARRACAN CREEK @ THORPDALE | 428843 | 5763687 | 0 | 22/06/1955 |  | 66 |  | 0.75 |
| 226219 | TOORONGA RIVER @ NOOJEE | 415838.9 | 5810012 | 0 | 1/10/1924 | 1/07/1933 | 65 |  | 0.78 |
| 226220 | LOCH RIVER @ NOOJEE | 412644 | 5808497 | 0 | 25/04/1978 |  | 106 |  | 0.74 |
| 226222 | LATROBE RIVER @ NEAR NOOJEE (US ADA R JUNCT.) | 402650 | 5806658 | 272.304 | 10/05/1971 |  | 62 |  | 0.80 |
| 226223 | LATROBE RIVER @ NEERIM EAST | 414328.5 | 5800194.3 | 0 | 16/03/1972 | 8/01/1979 | 378 |  | 0.77 |
| 226224 | LATROBE RIVER @ ROSEDALE (ANABRANCH) | 481555 | 5777709 | 9.7 | 1/12/1936 |  | 4144 |  | 0.12 |
| 226226 | TANJIL RIVER @ TANJIL JUNCTION | 429155 | 5796100 | 0 | 25/05/1960 |  | 289 |  | 0.69 |
| 226227 | LATROBE RIVER @ KILMANY SOUTH | 492900 | 5778721 | 2.571 | 16/12/1976 |  | 4464 |  | 0.62 |
| 226228 | LATROBE RIVER @ ROSEDALE (MAIN STREAM) | 481748 | 5778351 | 9.7 | 1/12/1936 |  | 4144 |  | 0.62 |
| 226229 | TANJIL RIVER @ D/S OF BLUE ROCK DAM | 432764.2 | 5784646.8 | 75.315 | 11/05/1979 | 28/02/1982 | 352 |  | 0.71 |
| 226232 | TANJIL RIVER @ MOE-WALHALLA ROAD BRIDGE | 0 | 0 | 0 | 17/05/1990 | 5/06/1990 | 514 |  |  |
| 226233 | TANJIL RIVER @ U/S OF SERPENTINE CREEK | 435276.4 | 5783132.8 | 0 | 16/07/1992 | 12/12/2006 | 0 |  | 0.74 |
| 226235 | TYERS RIVER @ TYERS JUNCTION | 441506 | 5798197 | 0 | 20/08/2003 |  | 132.2 |  | 0.61 |
| 226244 | TYERS R EAST BRANCH @ CORRINGAL SCOUT CAMP | 441496 | 5798505 | 0 | 27/10/2004 | 18/09/2008 | 0 |  |  |
| 226245 | TYERS R WEST BRANCH U/S SOUTH FACE RD | 436327 | 5807354 | 0 | 1/11/2004 | 24/02/2009 | 0 |  |  |
| 226246 | TYERS R WEST BRANCH D/S SOUTH FACE RD | 436123 | 5807237 | 0 | 28/10/2004 | 24/02/2009 | 0 |  |  |
| 226247 | HOPE CK U/S SOUTHFACE ROAD BRIDGE | 434283 | 5810028 | 0 | 26/03/2007 | 24/02/2009 | 0 |  |  |
| 226248 | HOPE CK D/S SOUTHFACE ROAD BRIDGE | 434267 | 5809990 | 0 | 26/03/2007 | 24/02/2009 | 0 |  |  |
| 226400 | LATROBE RIVER @ YALLOURN | 437171.9 | 5776866.3 | 0 | 1/11/1923 |  | 1940 |  |  |
| 226402 | MOE DRAIN @ TRAFALGAR EAST | 431070 | 5774351 | 0 | 7/06/1957 |  | 622 |  |  |
| 226407 | MORWELL RIVER @ BOOLARRA | 439351 | 5748787 | 98.302 | 9/05/1972 |  | 114 |  |  |
| 226408 | MORWELL R @ YALLOURN | 444702 | 5770110.4 | 0 | 21/04/2004 |  | 622 |  |  |
| 226410 | TRARALGON CREEK @ KOORNALLA | 459050 | 5758231 | 0 | 9/07/1953 |  | 89 |  |  |
| 226415 | TRARALGON CREEK @ TRARALGON SOUTH (JONES RD) | 459805 | 5764127 | 60 | 27/05/1976 |  | 128 |  |  |
| 226602 | AREA 2 SITE1 @ DOWDS MORASS NTH | 515887 | 5777885 | -0.016 | 17/06/2003 | 24/09/2009 | 0 |  |  |
| 226603 | AREA 4 SITE 2 @ DOWDS MORASS STH | 516572 | 5776987 | -0.016 | 17/06/2003 |  | 0 |  |  |
| 226605 | LAKE VICTORIA AT MCLENNAN'S STRAIT | 539492.4 | 5787708 | 0 | 17/11/2010 |  | 0 |  |  |
| 226606 | LAKE VICTORIA AT LOCH SPORT | 551177 | 5789006 |  | 26/11/2010 |  |  |  |  |
| 226607 | MCMILLAN STRAIT AT PAYNESVILLE | 564019 | 5803744.2 |  | 21/10/2010 |  |  |  |  |
| 226608 | LAKE KING AT METUNG | 576509 | 5807127.8 |  | 17/11/2010 |  |  |  |  |
| 226609 | CUNNINGHAM ARM AT BULLOCK ISLAND | 585591 | 5806582.2 |  | 19/10/2010 |  |  |  |  |
| 226700 | LOY YANG OUTFALL TO TRARALGON CREEK | 0 | 0 | 0 | 17/12/1997 | 7/02/2000 | 0 |  |  |
| 226702 | MORWELL GROSS POLLUTION TRAP D/S CIRCULAR PIPE | 447900 | 5766700 | 0 | 1/06/1998 | 7/10/1998 | 0 |  |  |
| 226801 | RAINGAUGE (REPRESENTATIVE BASIN) @ LATROBE NO. 2 | 399410 | 5810382.1 | 0 | 19/01/1982 | 17/08/1988 | 0 |  |  |
| 226802 | RAINGAUGE (REPRESENTATIVE BASIN) @ LATROBE NO. 3 | 398477.4 | 5802787.5 | 0 | 19/01/1982 | 6/03/1986 | 0 |  |  |
| 226804 | RAINGAUGE (SITE 3) @ WALHALLA ST - NEWBOROUGH | 438000 | 5772500 | 0 | 6/12/1994 | 16/10/1995 | 0 |  |  |
| 226805 | RAINGAUGE (SITE 4) @ MOE INDOOR RECREATION CENTRE | 437018.3 | 5772948.1 | 0 | 7/12/1994 | 16/10/1995 | 0 |  |  |
| 226814 | RAIN GAUGE @ MT TASSIE | 461784 | 5750077 | 0 | 3/12/1998 |  | 0 |  |  |
| 226815 | RAIN GAUGE (TRARALGON CK) @ TRARALGON - EPA YARD | 458982 | 5772514 | 0 | 25/05/1999 |  | 0 |  |  |
| 226816 | RAIN GAUGE @ MT HOOGHLY | 453401 | 5750509 | 0 | 29/09/1999 | 15/05/2008 | 0 |  |  |
| 226817 | RAIN GAUGE @ LE ROY QUARRY | 457656 | 5750650 | 0 | 30/09/1999 | 31/10/2009 | 0 |  |  |
| 226818 | RAIN GAUGE @ BALOOK | 459952 | 5746341 | 0 | 10/06/1999 |  | 0 |  |  |
| 226819 | RAIN GAUGE @ CALIGNEE NORTH | 462063 | 5758780 | 0 | 25/05/1999 |  | 0 |  |  |
| 226825 | RAINGAUGE @ MOE SOUTH | 434231.7 | 5769648.9 |  | 6/10/2009 |  |  |  |  |
| 226826 | RAINGAUGE AT YARRAGON SOUTH | 420511 | 5763367.1 |  | 21/10/2009 |  |  |  |  |
| 226827 | RAINGAUGE @ THORPDALE PEAK | 431275 | 5760002.4 |  | 17/09/2009 |  |  |  |  |
| 226828 | RAINGAUGE @ JEERALANG SOUTH HALLAMS RD | 451008.4 | 5752916 |  | 6/10/2009 | 16/11/2011 |  |  |  |
| 226829 | RAINGAUGE AT JERRALANG, DOBBINS ROAD | 450883.6 | 5754699.5 |  | 29/01/2013 |  |  |  |  |

Mitchell Basin

| Site ID | Site name | Easting | Northing | Gauge zero (Ahd) | Site start date | Site cease date | Drainage area (km2) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 224200 | MITCHELL RIVER @ BAIRNSDALE | 552241 | 5813267 | 0 |  |  | 4425 |  |  |
| 224201 | WONNANGATTA RIVER @ WATERFORD | 514714 | 5850745 | 175.337 | 15/07/1976 |  | 1979 |  |  |
| 224203 | MITCHELL RIVER @ GLENALADALE | 533010 | 5820334 | 28.951 | 22/02/1991 |  | 3903 |  |  |
| 224205 | DARGO RIVER @ DARGO (UPPER SITE) | 524129.9 | 5861627.7 | 0 | 1/06/1953 | 26/06/1974 | 539 |  |  |
| 224206 | WONNANGATTA RIVER @ CROOKED RIVER | 507891 | 5859903 | 240.829 | 13/04/1977 |  | 1096 |  |  |
| 224207 | WONGUNGARRA RIVER @ GUYS | 508800 | 5862200 | 247.234 | 7/05/1953 | 10/01/1989 | 736 |  |  |
| 224208 | MITCHELL RIVER @ HOWITT DAM SITE | 532433.8 | 5827894.3 | 51.586 | 14/02/1969 | 22/12/1975 | 3761 |  |  |
| 224209 | COBBANNAH CREEK @ NEAR BAIRNSDALE | 530900 | 5831600 | 0 | 15/05/1970 | 1/07/1987 | 106 |  |  |
| 224210 | WONNANGATTA RIVER @ KINGSWELL BRIDGE (HAWKHURST) | 510185.8 | 5859581.7 | 224.768 | 12/02/1970 | 25/03/1981 | 1883 |  |  |
| 224213 | DARGO RIVER @ LOWER DARGO ROAD | 523752 | 5850152 | 172.824 | 24/05/1973 |  | 676 |  |  |
| 224214 | WENTWORTH RIVER @ TABBERABBERA | 534808 | 5850099 | 0 | 4/07/1974 |  | 443 |  |  |
| 224215 | MITCHELL RIVER @ ANGUSVALE (TABBERABBERA) | 529928 | 5838289 | 92.644 | 28/05/1975 | 30/09/2008 | 3430 |  |  |
| 224216 | CLIFTON CREEK @ WY YUNG | 554294.5 | 5816639.7 | 0 | 28/01/1977 | 4/01/1979 | 129 |  |  |
| 224217 | MITCHELL RIVER @ ROSEHILL | 550366 | 5814542 | 0 | 2/04/2003 |  | 4413 |  |  |
| 224220 | RAIN GAUGE (BOGGY CREEK) @ BULLUMWAAL | 548565.5 | 5830013.3 | 178.885 | 26/02/1991 |  | 83 |  |  |
| 224222 | MITCHELL RIVER U/S GLENALADALE PUMPHOUSE | 532443 | 5823351 | 0 | 18/12/2009 |  | 0 |  |  |

Snowy Basin

| Site Id | Site Name | Easting | Northing | Gauge Zero (Ahd) | Site Start Date | Site Cease Date | Drainage Area (Sq.km) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 222200 | SNOWY RIVER @ JARRAHMOND | 620071 | 5830901 | 0 | 30/06/1971 |  | 13421 |  |  |
| 222201 | SNOWY RIVER @ ORBOST | 627898 | 5825644.4 | -0.141 | 27/03/1997 |  | 13572 |  |  |
| 222202 | BRODRIBB RIVER @ SARDINE CREEK | 636752 | 5847240 | 78.591 | 16/06/1998 |  | 658 |  |  |
| 222203 | SNOWY RIVER @ MARLO JETTY | 634504.8 | 5815384.5 | -0.745 | 25/06/1934 | 18/01/1998 | 0 |  |  |
| 222204 | SNOWY RIVER @ BETE BELONG | 623487.8 | 5824589.9 | 0 | 21/10/1938 | 26/07/1949 | 13540 |  |  |
| 222205 | SNOWY RIVER @ NEWMERELLA-LOCHEND ROAD | 632611.9 | 5821910.2 | -0.479 | 1/03/1960 | 27/01/1983 | 13600 |  |  |
| 222206 | BUCHAN RIVER @ BUCHAN | 603711 | 5849459 | 74.85 | 27/03/1926 |  | 822 |  |  |
| 222207 | BUCHAN RIVER @ MURRINDAL | 609403 | 5848400.8 | 0 | 15/02/1951 | 21/11/1972 | 1204 |  |  |
| 222209 | SNOWY RIVER @ MCKILLOP BRIDGE | 625612 | 5894902 | 0 | 5/04/1967 |  | 10619 |  |  |
| 222210 | DEDDICK RIVER @ DEDDICK (CASEYS) | 626577.7 | 5894693.7 | 0 | 5/04/1973 |  | 857 |  |  |
| 222212 | SNOWY RIVER @ BASIN CREEK NEAR BUCHAN | 613458.7 | 5850738.1 | 0 | 4/05/1932 | 10/01/1934 | 11836 |  |  |
| 222213 | SUGGAN BUGGAN RIVER @ SUGGAN BUGGAN | 618048 | 5909278 | 0 | 25/06/1974 |  | 361 |  |  |
| 222214 | ROCKY RIVER @ NEAR ORBOST | 645109.5 | 5833372.7 | 0 | 8/12/1969 | 17/03/1978 | 20 |  |  |
| 222216 | MURRINDAL RIVER @ BASIN ROAD (BUCHAN) | 608500 | 5849900 | 0 | 4/02/1976 | 30/06/1987 | 302 |  |  |
| 222217 | RODGER RIVER @ JACKSONS CROSSING | 620346 | 5858744 | 0 | 3/06/1976 |  | 447 |  |  |
| 222218 | LITTLE RIVER @ WULGULMERANG | 616400 | 5897200 | 0 | 21/04/1977 | 18/07/1984 | 88 |  |  |
| 222219 | SNOWY RIVER @ D/S OF BASIN CREEK | 612580 | 5848984 | 0 | 12/12/1978 |  | 11964 |  |  |
| 222221 | BUCHAN RIVER @ EGW OFFTAKE | 603874 | 5852271 | 0 | 14/05/2009 |  | 0 |  |  |
| 222222 | ROCKY RIVER U/S OT THE WEIR | 645938 | 5834053 | 0 | 11/03/2009 |  | 0 |  |  |
| 222400 | MOYANGUL RIVER @ LOOKOUT NEAR TIN MINE | 613203.5 | 5941897.7 | 0 | 14/03/1955 | 14/11/1963 | 29 |  |  |
| 222401 | INGEEGOODBEE RIVER @ D/S OF TIN MINE HUTS | 613098.4 | 5933947.3 | 0 | 19/05/1955 | 14/11/1963 | 21 |  |  |
| 222403 | BUCHAN RIVER @ GLENMORE | 600499.4 | 5879843.5 | 0 | 27/01/1955 | 22/07/1969 | 513 |  |  |
| 222404 | MELLICK MUNJIE CREEK @ GILLINGALL | 598595.5 | 5877355.2 | 0 | 5/07/1955 | 22/07/1969 | 70 |  |  |

South Gippsland region

| Site ID | Site name | Easting | Northing | Gauge zero (Ahd) | Site start date | Site cease date | Drainage area (km2) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 227001 | MERRIMAN CREEK @ SEASPRAY | 514294.7 | 5754491 | 0 | 29/09/1966 | 17/06/1971 | 525 |  |  |
| 227200 | TARRA RIVER @ YARRAM | 471649 | 5734286 | 0 | 18/02/1976 |  | 215 |  |  |
| 227201 | BRUTHEN CREEK @ WOODSIDE | 488800 | 5735750 | 0 | 1/03/1946 | 30/09/1960 | 174 |  |  |
| 227202 | TARWIN RIVER @ MEENIYAN | 412165 | 5729150 | 0 | 22/06/1955 |  | 1067 |  |  |
| 227203 | FRANKLIN RIVER @ HENWOODS BRIDGE | 436475.8 | 5725293.9 | 0 | 3/12/1946 | 8/01/1985 | 12 |  |  |
| 227205 | MERRIMAN CREEK @ CALIGNEE SOUTH | 469833 | 5755048 | 172.448 | 13/12/1965 |  | 36 |  |  |
| 227210 | BRUTHEN CREEK @ CARRAJUNG LOWER | 477651 | 5750309 | 0 | 7/08/1952 |  | 18 |  |  |
| 227211 | AGNES RIVER @ TOORA | 445387 | 5722973 | 0 | 10/01/1957 |  | 67 |  |  |
| 227213 | JACK RIVER @ JACK RIVER | 459739 | 5735728 | 0 | 1/12/1962 |  | 34 |  |  |
| 227216 | ALBERT RIVER @ HIAWATHA (BELOW FALLS) | 453863.5 | 5735533.5 | 0 | 25/06/1964 | 23/02/1989 | 41 |  |  |
| 227217 | LILLYPILLY CREEK @ STAIRCASE | 442292 | 5680222.7 | 0 | 2/09/1965 | 15/01/1974 | 5 |  |  |
| 227219 | BASS RIVER @ LOCH | 388609 | 5753722 | 0 | 1/04/1966 |  | 52 |  |  |
| 227220 | GREIG CREEK @ MUMFORDS | 473100 | 5743600 | 0 | 6/05/1968 | 2/12/1998 | 25 |  |  |
| 227221 | BODMAN CREEK @ BRIDGES | 476725.7 | 5746055.5 | 0 | 7/05/1968 | 20/10/1978 | 15 |  |  |
| 227222 | SPRING CREEK @ BOWDENS | 475282.4 | 5742352.5 | 65.909 | 20/05/1968 | 13/06/1975 | 10 |  |  |
| 227223 | MACKS CREEK @ RICHARDS | 467870.1 | 5741586.5 | 0 | 24/04/1968 | 15/06/1987 | 19 |  |  |
| 227224 | WOMERAH CREEK @ TARR VALLEY ROAD | 462201 | 5741376.9 | 0 | 29/04/1968 | 20/12/1982 | 1 |  |  |
| 227225 | TARRA RIVER @ FISCHERS | 461190 | 5741983 | 0 | 24/04/1968 |  | 16 |  |  |
| 227226 | TARWIN RIVER EAST BRANCH @ DUMBALK NORTH | 426838 | 5738399 | 0 | 8/01/1969 |  | 127 |  |  |
| 227227 | WILKUR CREEK @ LEONGATHA | 408948 | 5750056 | 0 | 31/07/1970 |  | 106 |  |  |
| 227228 | TARWIN RIVER EAST BRANCH @ MIRBOO | 433155.3 | 5737488.6 | 0 | 30/04/1971 | 18/06/1987 | 43 |  |  |
| 227231 | BASS RIVER @ GLEN FORBES SOUTH | 370463 | 5741320 | 0 | 30/03/1973 |  | 233 |  |  |
| 227232 | LANCE CREEK U/S LANCE CREEK RESERVOIR | 383783 | 5738738 | 0 | 31/03/2008 |  | 14 |  |  |
| 227234 | SPRING CREEK @ BEAUMONT | 475276.8 | 5744201.8 | 0 | 3/04/1975 | 15/12/1982 | 7 |  |  |
| 227235 | MIDDLE CREEK @ TALL TIMBERS | 460742.6 | 5742294.6 | 0 | 27/04/1978 | 18/01/1989 | 9 |  |  |
| 227236 | POWLETT RIVER @ D/S FOSTER CREEK JUNCTION | 387508 | 5731353 | 0 | 18/05/1979 |  | 228 |  |  |
| 227237 | FRANKLIN RIVER @ TOORA | 439771 | 5724088 | 0 | 7/04/1983 |  | 75 |  |  |
| 227238 | FOSTER CREEK @ DAM SITE | 387050.1 | 5739746.7 | 0 | 8/06/1979 | 17/01/1989 | 55 |  |  |
| 227239 | MERRIMAN CREEK @ STRADBROKE WEST | 492658 | 5764652 | 71.868 | 18/11/1983 |  | 256 |  |  |
| 227240 | MERRIMAN CREEK @ PROSPECT ROAD SEASPRAY | 514320 | 5754179 | 2.544 | 26/08/1983 |  | 529 |  |  |
| 227242 | MERRIMAN CREEK @ SEASPRAY TOWNSHIP | 516203 | 5752529 | 0 | 6/03/1990 |  | 8 |  |  |
| 227243 | BRUTHEN CREEK @ D/S REEDY CREEK | 484778 | 5747810 | 79.91 | 13/05/1992 |  | 124 |  |  |
| 227244 | DEEP CREEK @ FOSTER | 432312 | 5725707 | 0 | 29/04/1993 |  | 0 |  |  |
| 227245 | LITTLE BASS RIVER @ POOWONG U/S LITTLE BASS RES. | 39050 | 5753576 | 0 | 18/05/1999 |  | 0 |  |  |
| 227246 | COALITION CREEK | 396827 | 5748087.4 | 0 | 8/06/2004 |  | 0 |  |  |
| 227248 | BELLVIEW CREEK U/S BELLVIEW RESERVOIR | 396367 | 5749105 | 0 | 18/05/1999 |  | 0 |  |  |
| 227249 | RUBY CREEK @ ARAWATA | 402379 | 5748848 | 0 | 23/07/2008 |  | 0 |  |  |
| 227251 | TARRA RIVER @ TARRA WEIR OFFTAKE | 463301 | 5741305 | 0 | 27/10/2004 |  | 0 |  |  |
| 227264 | COALITION CREEK @ LEONGATHA (SPENCERS ROAD BRIDGE) | 409001 | 5743754 | 0 | 21/10/2008 |  | 0 |  |  |
| 227265 | GOLDEN CREEK @ BLACK SWAMP ROAD | 428266 | 5711026 | 0 | 15/02/2008 | 18/11/2008 | 0 |  |  |
| 227266 | TARWIN RIVER @ KOONWARRA | 408996 | 5735402 | 0 | 22/09/2008 |  | 0 |  |  |
| 227270 | FOSTER CREEK AT KORUMBURRA | 394617.6 | 5745540 |  | 13/10/2011 |  |  |  |  |

Tambo Basin

| Site Id | Site Name | Easting | Northing | Gauge Zero (Ahd) | Site Start Date | Site Cease Date | Drainage Area (Sq.km) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 223202 | TAMBO RIVER @ SWIFTS CREEK | 564619 | 5875047 | 280.863 | 8/03/1977 |  | 943 |  |  |
| 223204 | NICHOLSON RIVER @ DEPTFORD | 561543 | 5839104 | 0 | 12/05/1961 |  | 287 |  |  |
| 223205 | TAMBO RIVER @ D/S OF RAMROD CREEK | 576710 | 5830147 | 15.452 | 9/06/1965 |  | 2681 |  |  |
| 223206 | TAMBO RIVER @ BINDI | 571424.2 | 5896694.3 | 0 | 8/08/1957 | 19/12/1974 | 401 |  |  |
| 223207 | TIMBARRA RIVER @ TIMBARRA | 592900 | 5869900 | 0 | 9/09/1957 | 4/06/1973 | 205 |  |  |
| 223208 | TAMBO RIVER @ BINDI (NEAR JUNCTION CREEK) | 568710 | 5887104 | 0 | 21/03/1974 | 14/07/2003 | 523 |  |  |
| 223209 | TAMBO RIVER @ BATTENS LANDING | 574796.1 | 5820757.9 | 0 | 26/01/1977 |  | 2781 |  |  |
| 223210 | NICHOLSON RIVER @ SARSFIELD | 562646 | 5823050 | 0 | 21/09/1977 |  | 471 |  |  |
| 223212 | TIMBARRA RIVER @ D/S OF WILKINSON CREEK | 594066 | 5855069 | 0 | 6/05/1982 |  | 438 |  |  |
| 223213 | TAMBO RIVER @ D/S OF DUGGAN CREEK | 578509 | 5904321 | 746.726 | 16/09/1987 |  | 96 |  |  |
| 223214 | TAMBO RIVER @ U/S OF SMITH CREEK | 582588 | 5909736 | 0 | 2/03/1989 |  | 32 |  |  |
| 223215 | HAUNTED STREAM @ HELLS GATE | 573015 | 5851389 | 153.892 | 8/02/1990 |  | 180 |  |  |
| 223216 | TAMBO RIVER U/S SWIFTS CK OFFTAKE | 563729 | 5877748 | 0 | 20/05/2009 |  | 0 |  |  |
| 223217 | NICHOLSON RIVER AT PUMP HOUSE | 564088 | 5821594 | 0 | 20/01/2011 |  | 0 |  |  |
| 223402 | TIMBARRA RIVER @ NUNNIONG PLAINS | 587028.3 | 5888208.6 | 0 | 22/06/1955 | 16/02/1960 | 16 |  |  |
| 223403 | TAMBO RIVER @ NUNNIONG PLAINS | 583943.9 | 5892462.9 | 0 | 21/06/1955 | 16/02/1960 | 39 |  |  |
| 223800 | RAINGAUGE (TAMBO RIVER) @ MOUNT ELIZABETH | 582300 | 5850700 | 0 | 15/01/1985 | 20/10/2004 | 0 |  |  |
| 223801 | RAIN GAUGE (TAMBO RIVER) @ MT ELIZABETH HELIPAD | 581909 | 5850809 | 0 | 20/10/2004 | 19/01/2011 | 0 |  |  |
| 223802 | RAINGAUGE AT MOUNT ELIZABETH SOMMERVILLE TRACK | 580733 | 5851999 |  | 19/01/2011 |  |  |  |  |

Thomson Basin

| Site Id | Site Name | Easting | Northing | Gauge Zero (Ahd) | Site Start Date | Site Cease Date | Drainage Area (Sq.km) | BFI GHD | BFI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 225019 | NORTH CASCADE CREEK @ THOMSON VALLEY ROAD | 441836.2 | 5815577.5 | 0 | 11/01/1962 | 6/06/1974 | 11 |  |  |
| 225105 | THOMSON RIVER @ NEWLAND ROAD | 427654.1 | 5819829.7 | 0 | 30/03/1954 | 1/05/1984 | 16 |  |  |
| 225114 | THOMSON RIVER @ D/S WHITELAWS CREEK | 436711 | 5825795 | 0 | 27/03/1987 |  | 155.3 |  |  |
| 225200 | THOMSON RIVER @ HEYFIELD | 480693 | 5795763 | 35.846 | 17/01/1991 |  | 1238 |  |  |
| 225201 | AVON RIVER @ STRATFORD | 506676 | 5797653 | 0 | 1/11/1976 |  | 1485 |  |  |
| 225204 | MACALISTER RIVER @ LAKE GLENMAGGIE (TAIL GAUGE) | 482885 | 5805021 | 47.015 | 28/09/1966 |  | 1891 |  |  |
| 225207 | THOMSON RIVER @ WALHALLA | 449064.3 | 5798590.9 | 0 | 9/03/1950 | 22/05/1952 | 875 |  |  |
| 225209 | MACALISTER RIVER @ LICOLA | 466762 | 5835198 | 2 | 1/08/1952 |  | 1233 |  |  |
| 225210 | THOMSON RIVER @ THE NARROWS | 447551 | 5805990 | 247.08 | 9/04/1957 |  | 518 |  |  |
| 225212 | THOMSON RIVER @ WANDOCKA | 489554 | 5792978 | 19.423 | 1/03/1977 |  | 1417 |  |  |
| 225213 | ABERFELDY RIVER @ BEARDMORE | 450135 | 5810238 | 305.552 | 27/06/1963 |  | 311 |  |  |
| 225216 | JORDAN RIVER @ ABERFELDY | 439747.8 | 5829099.3 | 0 | 4/10/1971 | 18/07/1972 | 124 |  |  |
| 225217 | BARKLY RIVER @ GLENCAIRN | 461700 | 5842800 | 263.1 | 12/05/1966 | 4/01/1989 | 248 |  |  |
| 225218 | FREESTONE CREEK @ BRIAGALONG | 508366.4 | 5815232.8 | 63.238 | 14/07/1975 |  | 309 |  |  |
| 225219 | MACALISTER RIVER @ GLENCAIRN | 461689 | 5847757 | 293.54 | 7/04/1967 |  | 570 |  |  |
| 225221 | MACALISTER RIVER @ STRINGYBARK CREEK | 470738 | 5819709 | 105.249 | 18/03/1968 |  | 1542 |  |  |
| 225222 | GLENMAGGIE CREEK @ SEATON (AUBREYS) | 471117.4 | 5803309.8 | 111.285 | 10/03/1970 | 18/12/1975 | 141 |  |  |
| 225223 | VALENCIA CREEK @ GILLIO ROAD | 499321 | 5822633 | 85.481 | 26/03/1991 |  | 195 |  |  |
| 225224 | AVON RIVER @ THE CHANNEL | 489868 | 5816150 | 72 | 12/07/1972 |  | 554 |  |  |
| 225225 | MACALISTER RIVER @ LAKE GLENMAGGIE (HEAD GAUGE) | 482418.6 | 5804322.1 |  | 26/01/1925 |  |  |  |  |
| 225228 | THOMSON RIVER @ COWWARR TIMBER WEIR H.G. | 469855.4 | 5794299.7 | -0.96 | 1/01/1958 |  | 1093 |  |  |
| 225230 | GLENMAGGIE CREEK @ THE GORGE | 469772 | 5803687 | 120.847 | 2/05/1975 |  | 139 |  |  |
| 225231 | THOMSON RIVER @ U/S OF COWWARR WEIR | 467065 | 5796878 | 69.4 | 1/04/1976 |  | 1080 |  |  |
| 225232 | THOMSON RIVER @ BUNDALAGUAH | 499308 | 5789148 | 5.094 | 3/11/1976 |  | 3538 |  |  |
| 225233 | PERRY RIVER @ PERRY BRIDGE | 523288.7 | 5793397.5 | -0.938 | 24/12/1976 | 19/11/1982 | 357 |  |  |
| 225234 | AVON RIVER @ CLYDEBANK (CHINN'S BRIDGE) | 515238 | 5791515 | 0 | 29/06/2004 |  | 1584 |  |  |
| 225236 | RAINBOW CREEK @ HEYFIELD | 480686 | 5793911 | 33.837 | 30/04/1992 |  | 0 |  |  |
| 225247 | MACALISTER RIVER @ RIVERSLEA | 498044.5 | 5791335.7 | 0 | 11/01/2001 |  | 0 |  |  |
| 225248 | BOGGY CREEK @ CORNWALLS ROAD | 492831 | 5793649.7 | 0 | 29/08/2008 |  | 0 |  |  |
| 225255 | AVON RIVER U/S VALENCIA CK JUNCTION | 498324 | 5813965 | 0 | 30/06/2004 |  | 0 |  |  |
| 225256 | MACALISTER R D/S MAFFRA (SMITHS BR.) | 498377 | 5793730 | 0 | 26/10/2005 |  | 0 |  |  |
| 225600 | LAKE WELLINGTON @ SALE | 519212 | 5781295.6 | 0 | 1/12/1973 | 1/07/1977 | 0 |  |  |
| 225703 | THOMSON RIVER DIVERSION CHANNEL @ COWWARR WEIR | 470010 | 5794250 | 0 | 5/02/2001 | 19/07/2007 | 0 |  |  |
| 225711 | LAKE WELLINGTON DRAIN @ 5 MILES 32 CHAIN MEASURING WEIR | 512208.2 | 5787288.3 | 0 | 12/11/1976 | 3/05/2001 | 0 |  |  |
| 225715 | CENTRAL GIPPSLAND DRAIN 3 @ NAMBROK (RD 1066M) | 488590 | 5786295 | 0 | 24/02/1980 | 15/04/2005 | 0 |  |  |
| 225716 | CENTRAL GIPPSLAND 1/2 DRAIN 3 @ DROP STRUCTURE (RD500) | 491500 | 5784700 | 0 | 24/02/1980 | 4/02/2004 | 0 |  |  |
| 225717 | CENTRAL GIPPSLAND 2/3 DRAIN @ US OF DRAIN 3 (RD 500M) | 491102 | 5783737 | 0 | 24/02/1980 | 15/04/2005 | 0 |  |  |
| 225721 | CENTRAL GIPPSLAND DRAIN NO 3/3 U/S OUTFALL | 487050 | 5788350 | 0 | 29/05/2000 | 13/02/2004 | 0 |  |  |
| 225722 | CENTRAL GIPPSLAND DRAIN NO 3 D/S NO3 O/FALL | 487100 | 5788300 | 0 | 29/05/2000 | 13/02/2004 | 0 |  |  |
| 225723 | CENTRAL GIPPSLAND DRAIN NO 1/3 @ D/S 11/4/1 JUNCTION | 488200 | 5784600 | 0 | 29/05/2000 | 13/02/2004 | 0 |  |  |
| 225724 | CHANNEL O/FALL 11/4/1 U/S JUNCTION CG DR 1/3 | 488150 | 5784500 | 0 | 29/05/2000 | 15/04/2005 | 0 |  |  |
| 225725 | CENTRAL GIPPSLAND DRAIN NO 3 D/S O/FALL 11/1 | 490400 | 5783500 | 0 | 26/05/2000 | 15/04/2005 | 0 |  |  |
| 225726 | CENTRAL GIPPSLAND DRAIN NO 3 D/S RAILWAY LINE | 491000 | 5781800 | 0 | 29/05/2000 | 15/04/2005 | 0 |  |  |
| 225727 | CENTRAL GIPPSLAND DRAIN NO 2/3 @ SOLDIERS RD | 490100 | 5785600 | 0 | 26/05/2000 | 13/02/2004 | 0 |  |  |
| 225737 | CENTRAL GIPPSLAND DRAIN NO 8/2 @ DENISON ROAD | 483978 | 5789779 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225738 | CENTRAL GIPPSLAND DRAIN NO 2 @ SALE-TOONGABBIE ROAD | 487500 | 5782800 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225739 | CENTRAL GIPPSLAND DRAIN NO 2/2 @ DESSENTS | 487311 | 5782836 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225740 | CENTRAL GIPPSLAND DRAIN NO 2 U/S TINAMBA-ROSEDALE ROAD | 487800 | 5781900 | 0 | 16/05/2002 | 15/04/2005 | 0 |  |  |
| 225741 | CENTRAL GIPPSLAND DRAIN NO 2 U/S NAMBROK ROAD | 485335 | 5786592 | 0 | 16/05/2002 | 15/04/2005 | 0 |  |  |
| 225742 | CENTRAL GIPPSLAND DRAIN NO 2 U/S SALE-COWWARR ROAD | 484488 | 5788744 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225743 | CENTRAL GIPPSLAND DRAIN NO 6/2 U/S DENISON ROAD | 483670 | 5788400 | 0 | 28/05/2002 | 15/04/2005 | 0 |  |  |
| 225747 | NOBLES O/FALL @ VALENCIA CK | 498563 | 5813297 | 0 | 24/02/2004 | 1/07/2005 | 0 |  |  |
| 225748 | TINAMBA MAIN O/FALL @ MENBURN PARK | 498214 | 5813767 | 0 | 25/02/2004 | 27/06/2005 | 0 |  |  |
| 225801 | RAIN GAUGE (MACALISTER RIVER) @ MURDERERS HILL | 460964 | 5810725 | 0 | 8/06/1970 |  | 0 |  |  |
| 225802 | RAIN GAUGE (MACALISTER RIVER) @ MOUNT TAMBORITHA | 472550 | 5853466 | 0 | 1/04/1986 |  | 0 |  |  |
| 225809 | RAIN GAUGE (AVON RIVER) @ MOUNT WELLINGTON | 487412 | 5850069 | 0 | 21/05/1997 |  | 0 |  |  |
| 225810 | RAIN GAUGE (AVON RIVER) @ REEVE KNOB | 500533.2 | 5846182 | 0 | 21/05/1997 |  | 0 |  |  |
| 225819 | RAINGAUGE @ MT USEFUL | 456353 | 5827872 | 0 | 27/05/2002 |  | 0 |  |  |
| 225823 | RAINGAUGE AT BLANKET HILL | 472533.6 | 5811892.1 |  | 26/07/2010 |  |  |  |  |
| 225824 | RAINGAUGE (MACALISTER RV) AT SNOWY RANGE | 0 | 0 |  | 22/02/2011 |  |  |  |  |
| 225825 | RAINGAUGE (MACALISTER RV) AT HIGH RIDGE | 0 | 0 |  |  |  |  |  |  |
| 225826 | RAINGAUGE (MACALISTER RV) AT MOUNT SUNDAY | 448995 | 5867264 |  | 6/04/2011 |  |  |  |  |

- Appendix B: Gippsland region recharge

Spatially averaged groundwater potential evapotranspiration and recharge for each modelled 
time period.

| Stress period | Period | Time step (ts) | Days per timestep | Rain (mm/ts) | G’water Evap’n (mm/ts) | Recharge (mm/ts) | Rech/Rain (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 01/01/1971 – 31/12/1971 | 365 | 365 | 1022 | 783 | 65 | 6.31 |
| 2 | 01/01/1972 – 31/12/1972 | 366 | 731 | 626 | 619 | 20 | 3.14 |
| 3 | 01/01/1973 – 31/12/1973 | 365 | 1096 | 1035 | 644 | 76 | 7.37 |
| 4 | 01/01/1974 – 31/12/1974 | 365 | 1461 | 1353 | 678 | 173 | 12.78 |
| 5 | 01/01/1975 – 31/12/1975 | 365 | 1826 | 1064 | 682 | 108 | 10.18 |
| 6 | 01/01/1976 – 31/12/1976 | 366 | 2192 | 914 | 641 | 78 | 8.49 |
| 7 | 01/01/1977 – 31/12/1977 | 365 | 2557 | 840 | 639 | 78 | 9.28 |
| 8 | 01/01/1978 – 31/12/1978 | 365 | 2922 | 1378 | 639 | 166 | 12.02 |
| 9 | 01/01/1979 – 31/12/1979 | 365 | 3287 | 677 | 615 | 49 | 7.29 |
| 10 | 01/01/1980 – 31/12/1980 | 366 | 3653 | 879 | 662 | 50 | 5.72 |
| 11 | 01/01/1981 – 31/12/1981 | 365 | 4018 | 945 | 671 | 71 | 7.51 |
| 12 | 01/01/1982 – 31/12/1982 | 365 | 4383 | 671 | 616 | 29 | 4.38 |
| 13 | 01/01/1983 – 31/12/1983 | 365 | 4748 | 1021 | 569 | 102 | 10.04 |
| 14 | 01/01/1984 – 31/12/1984 | 366 | 5114 | 965 | 649 | 91 | 9.39 |
| 15 | 01/01/1985 – 31/12/1985 | 365 | 5479 | 1143 | 613 | 113 | 9.85 |
| 16 | 01/01/1986 – 31/12/1986 | 365 | 5844 | 782 | 626 | 75 | 9.55 |
| 17 | 01/01/1987 – 31/12/1987 | 365 | 6209 | 828 | 631 | 59 | 7.13 |
| 18 | 01/01/1988 – 31/12/1988 | 366 | 6575 | 995 | 626 | 72 | 7.25 |
| 19 | 01/01/1989 – 31/12/1989 | 365 | 6940 | 1031 | 655 | 122 | 11.81 |
| 20 | 01/01/1990 – 31/03/1990 | 90 | 7030 | 145 | 179 | 3 | 1.77 |
| 21 | 01/04/1990 – 30/06/1990 | 91 | 7121 | 283 | 96 | 19 | 6.86 |
| 22 | 01/07/1990 – 30/09/1990 | 92 | 7213 | 308 | 114 | 50 | 16.25 |
| 23 | 01/10/1990 – 31/12/1990 | 92 | 7305 | 238 | 262 | 22 | 9.15 |
| 24 | 01/01/1991 – 31/03/1991 | 90 | 7395 | 199 | 194 | 4 | 1.90 |
| 25 | 01/04/1991 – 30/06/1991 | 91 | 7486 | 243 | 92 | 12 | 4.92 |
| 26 | 01/07/1991 – 30/09/1991 | 92 | 7578 | 359 | 118 | 64 | 17.82 |
| 27 | 01/10/1991 – 31/12/1991 | 92 | 7670 | 165 | 241 | 7 | 4.53 |
| 28 | 01/01/1992 – 31/03/1992 | 91 | 7761 | 193 | 191 | 6 | 3.01 |
| 29 | 01/04/1992 – 30/06/1992 | 91 | 7852 | 216 | 76 | 12 | 5.77 |
| 30 | 01/07/1992 – 30/09/1992 | 92 | 7944 | 303 | 111 | 40 | 13.34 |
| 31 | 01/10/1992 – 31/12/1992 | 92 | 8036 | 377 | 231 | 47 | 12.50 |
| 32 | 01/01/1993 – 31/03/1993 | 90 | 8126 | 248 | 217 | 17 | 6.89 |
| 33 | 01/04/1993 – 30/06/1993 | 91 | 8217 | 125 | 94 | 11 | 9.11 |
| 34 | 01/07/1993 – 30/09/1993 | 92 | 8309 | 339 | 112 | 49 | 14.55 |
| 35 | 0110/1993 – 31/12/1993 | 92 | 8401 | 304 | 247 | 31 | 10.14 |
| 36 | 01/01/1994 – 31/03/1994 | 90 | 8491 | 266 | 213 | 15 | 5.77 |
| 37 | 01/04/1994 – 30/06/1994 | 91 | 8582 | 192 | 107 | 14 | 7.45 |
| 38 | 01/07/1994 – 30/09/1994 | 92 | 8674 | 143 | 111 | 13 | 9.14 |
| 39 | 01/10/1994 – 31/12/1994 | 92 | 8766 | 243 | 249 | 16 | 6.59 |
| 40 | 01/01/1995 – 31/03/1995 | 90 | 8856 | 207 | 202 | 5 | 2.32 |
| 41 | 01/04/1995 – 30/06/1995 | 91 | 8947 | 252 | 87 | 25 | 9.94 |
| 42 | 01/07/1995 – 30/09/1995 | 92 | 9039 | 203 | 121 | 26 | 12.65 |
| 43 | 0110/1995 – 31/12/1995 | 92 | 9131 | 378 | 234 | 38 | 10.11 |
| 44 | 01/01/1996 – 31/03/1996 | 91 | 9222 | 230 | 220 | 10 | 4.50 |
| 45 | 01/04/1996 – 30/06/1996 | 91 | 9313 | 191 | 92 | 18 | 9.45 |
| 46 | 01/07/1996 – 30/09/1996 | 92 | 9405 | 304 | 122 | 45 | 14.71 |
| 47 | 01/10/1996 – 31/12/1996 | 92 | 9497 | 211 | 246 | 13 | 6.08 |
| 48 | 01/01/1997 – 31/03/1997 | 90 | 9587 | 141 | 178 | 3 | 1.97 |
| 49 | 01/04/1997 – 30/06/1997 | 91 | 9678 | 174 | 73 | 5 | 2.75 |
| 50 | 01/07/1997 – 30/09/1997 | 92 | 9770 | 148 | 113 | 14 | 9.68 |
| 51 | 01/10/1997 – 31/12/1997 | 92 | 9862 | 158 | 223 | 3 | 1.96 |
| 52 | 01/01/1998 – 31/03/1998 | 90 | 9952 | 131 | 143 | 1 | 0.73 |
| 53 | 01/04/1998 – 30/06/1998 | 91 | 10043 | 295 | 69 | 10 | 3.32 |
| 54 | 01/07/1998 – 30/09/1998 | 92 | 10135 | 196 | 111 | 26 | 13.50 |
| 55 | 01/10/1998 – 31/12/1998 | 92 | 10227 | 317 | 264 | 17 | 5.44 |
| 56 | 01/01/1999 – 31/03/1999 | 90 | 10317 | 210 | 219 | 5 | 2.36 |
| 57 | 01/04/1999 – 30/06/1999 | 91 | 10408 | 156 | 91 | 9 | 5.99 |
| 58 | 01/07/1999 – 30/09/1999 | 92 | 10500 | 164 | 113 | 17 | 10.26 |
| 59 | 0110/1999 – 31/12/1999 | 92 | 10592 | 203 | 207 | 6 | 2.78 |
| 60 | 01/01/2000 – 31/03/2000 | 91 | 10683 | 168 | 188 | 3 | 1.84 |
| 61 | 01/04/2000 – 30/06/2000 | 91 | 10774 | 248 | 89 | 17 | 6.67 |
| 62 | 01/07/2000 – 30/09/2000 | 92 | 10866 | 235 | 112 | 29 | 12.26 |
| 63 | 01/10/2000 – 31/12/2000 | 92 | 10958 | 211 | 237 | 21 | 9.83 |
| 64 | 01/01/2001 – 31/03/2001 | 90 | 11048 | 199 | 179 | 3 | 1.58 |
| 65 | 01/04/2001 – 30/06/2001 | 91 | 11139 | 217 | 90 | 16 | 7.53 |
| 66 | 01/07/2001 – 30/09/2001 | 92 | 11231 | 285 | 113 | 42 | 14.73 |
| 67 | 01/10/2001 – 31/12/2001 | 92 | 11323 | 309 | 237 | 29 | 9.26 |
| 68 | 01/01/2002 – 31/03/2002 | 90 | 11413 | 216 | 205 | 7 | 3.17 |
| 69 | 01/04/2002 – 30/06/2002 | 91 | 11504 | 267 | 105 | 24 | 8.81 |
| 70 | 01/07/2002 – 30/09/2002 | 92 | 11596 | 145 | 128 | 17 | 11.73 |
| 71 | 01/10/2002 – 31/12/2002 | 92 | 11688 | 156 | 231 | 3 | 2.14 |
| 72 | 01/01/2003 – 31/03/2003 | 90 | 11778 | 104 | 121 | 0 | 0.45 |
| 73 | 01/04/2003 – 30/06/2003 | 91 | 11869 | 169 | 93 | 5 | 2.68 |
| 74 | 01/07/2003 – 30/09/2003 | 92 | 11961 | 228 | 130 | 22 | 9.45 |
| 75 | 01/10/2003 – 31/12/2003 | 92 | 12053 | 244 | 236 | 14 | 5.78 |
| 76 | 01/01/2004 – 31/03/2004 | 91 | 12144 | 111 | 143 | 2 | 1.40 |
| 77 | 01/04/2004 – 30/06/2004 | 91 | 12235 | 229 | 86 | 11 | 4.69 |
| 78 | 01/07/2004 – 30/09/2004 | 92 | 12327 | 220 | 119 | 22 | 10.13 |
| 79 | 01/10/2004 – 31/12/2004 | 92 | 12419 | 262 | 252 | 16 | 6.05 |
| 80 | 01/01/2005 – 3103/2005 | 90 | 12509 | 223 | 203 | 7 | 2.98 |
| 81 | 01/04/2005 – 30/06/2005 | 91 | 12600 | 116 | 86 | 3 | 2.74 |
| 82 | 01/07/2005 – 30/09/2005 | 92 | 12692 | 298 | 115 | 29 | 9.63 |
| 83 | 01/10/2005 – 31/12/2005 | 92 | 12784 | 223 | 266 | 10 | 4.48 |
| 84 | 01/01/2006 – 31/03/2006 | 90 | 12874 | 114 | 161 | 1 | 0.98 |
| 85 | 01/04/2006 – 30/06/2006 | 91 | 12965 | 239 | 86 | 12 | 5.08 |
| 86 | 01/07/2006 – 30/09/2006 | 92 | 13057 | 174 | 123 | 13 | 7.38 |
| 87 | 01/10/2006 – 31/12/2006 | 92 | 13149 | 86 | 186 | 1 | 1.23 |
| 88 | 01/01/2007 – 31/03/2007 | 90 | 13239 | 190 | 157 | 1 | 0.37 |
| 89 | 01/04/2007 – 30/06/2007 | 91 | 13330 | 326 | 99 | 14 | 4.18 |
| 90 | 01/07/2007 – 30/09/2007 | 92 | 13422 | 188 | 126 | 33 | 17.44 |
| 91 | 01/10/2007 – 31/12/2007 | 92 | 13514 | 251 | 254 | 11 | 4.56 |
| 92 | 01/01/2008 – 31/03/2008 | 91 | 13605 | 172 | 193 | 4 | 2.11 |
| 93 | 01/04/2008 – 30/06/2008 | 91 | 13696 | 106 | 70 | 4 | 3.32 |
| 94 | 01/07/2008 – 30/09/2008 | 92 | 13788 | 199 | 117 | 18 | 8.82 |
| 95 | 01/10/2008 – 31/12/2008 | 92 | 13880 | 280 | 223 | 9 | 3.38 |
| 96 | 01/01/2009 – 31/03/2009 | 90 | 13970 | 81 | 154 | 2 | 2.10 |
| 97 | 01/04/2009 – 30/06/2009 | 91 | 14061 | 132 | 73 | 4 | 2.82 |
| 98 | 01/07/2009 – 30/09/2009 | 92 | 14153 | 252 | 133 | 21 | 8.18 |
| 99 | 01/10/2009 – 31/12/2009 | 92 | 14245 | 225 | 244 | 13 | 5.85 |
| 100 | 01/01/2010 – 31/03/2010 | 90 | 14335 | 251 | 195 | 6 | 2.35 |
| 101 | 01/04/2010 – 30/06/2010 | 91 | 14426 | 197 | 90 | 15 | 7.76 |
| 102 | 01/07/2010 – 30/09/2010 | 92 | 14518 | 205 | 106 | 30 | 14.53 |
| 103 | 01/10/2010 – 31/12/2010 | 92 | 14610 | 314 | 240 | 22 | 6.93 |
| 104 | 01/01/2011 – 31/03/2011 | 90 | 14700 | 293 | 198 | 18 | 6.18 |
| 105 | 01/04/2011 – 30/06/2011 | 91 | 14791 | 232 | 88 | 30 | 12.87 |
| 106 | 01/07/2011 – 30/09/2011 | 92 | 14883 | 278 | 117 | 38 | 13.71 |
| 107 | 01/10/2011 – 31/12/2011 | 92 | 14975 | 315 | 259 | 25 | 7.80 |
| 108 | 01/01/2012 – 31/03/2012 | 91 | 15066 | 335 | 216 | 22 | 6.41 |
| 109 | 01/04/2012 – 30/06/2012 | 91 | 15157 | 322 | 95 | 44 | 13.63 |
| 110 | 01/07/2012 – 30/09/2012 | 92 | 15249 | 202 | 125 | 32 | 15.76 |
| 111 | 01/10/2012 – 31/12/2012 | 92 | 15341 | 191 | 266 | 9 | 4.96 |

- Appendix C: Calibration bores

Summarised in Table F.1 are all the bores used to calibrate the groundwater model.  Included in the table are the layer allocations based on the VAF data (VAF Lay) and the corresponding layer allocation based on bore depth and surface elevation data (New Lay).  The top and bottom elevations of the VAF assigned layer (columns TopLay and BotLay respectively) are included for comparison with the associated bore depth.  The tabled information suggests that in most cases the VAF assigned bore layer varies from the depth based layer as reported by the last column of Table F.1.

Table C.1: Calibration bore attribution.

| Bore_ID | VAF Lay | Bore_x | Bore_y | Depth (m) | Surface (mAHD) | Depth (mAHD) | TopLay (mAHD) | BotLay (mAHD) | New Lay | Diff |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 114161 | 3 | 2568202 | 2315100 | 6 | 9.56 | 3.56 | 1.66 | -0.34 | 2 | yes |
| 114156 | 3 | 2568974 | 2314144 | 38 | 0 | -38 | -5.73 | -7.73 | 19 | yes |
| 114157 | 3 | 2568974 | 2314142 | 17 | 0 | -17 | -5.73 | -7.73 | 8 | yes |
| 114158 | 3 | 2568974 | 2314141 | 11 | 0 | -11 | -5.73 | -7.73 | 5 | yes |
| 94807 | 2 | 2568974 | 2314146 | 4 | 0 | -4 | -3.73 | -5.73 | 2 | no |
| 94814 | 3 | 2569007 | 2313783 | 9 | 12 | 3 | 3.16 | 1.01 | 3 | no |
| 94813 | 3 | 2570892 | 2310487 | 21 | 12.15 | -8.85 | 9.7 | 6.99 | 9 | yes |
| 94804 | 2 | 2571195 | 2311454 | 4 | 7.48 | 3.48 | 7.11 | 3.41 | 2 | no |
| 94816 | 3 | 2571454 | 2310076 | 12 | 5.97 | -6.03 | 8.43 | 6.27 | 9 | yes |
| 94815 | 2 | 2571634 | 2310081 | 1 | 5.97 | 4.97 | 16.82 | 8.43 | 4 | yes |
| 94803 | 3 | 2572449 | 2311040 | 6 | 3 | -3 | -0.63 | -2.63 | 4 | yes |
| 94802 | 2 | 2572605 | 2310763 | 4 | 6.99 | 2.99 | 4.9 | 0.85 | 2 | no |
| 113124 | 3 | 2572605 | 2310763 | 22 | 6.99 | -15.01 | 0.85 | -1.15 | 10 | yes |
| 113125 | 3 | 2572606 | 2310763 | 12 | 6.99 | -5.01 | 0.85 | -1.15 | 5 | yes |
| 75405 | 17 | 2572685 | 2329154 | 16 | 74.84 | 58.84 | 48 | 40.09 | 11 | yes |
| 75404 | 18 | 2574268 | 2332762 | 70 | 68.94 | -1.06 | 29.8 | 20.57 | 21 | yes |
| 61429 | 18 | 2574390 | 2323889 | 28 | 66 | 38 | 40.62 | 33.35 | 18 | no |
| 75566 | 18 | 2574974 | 2330073 | 29 | 59 | 30 | 23.75 | 20.93 | 14 | yes |
| 61430 | 18 | 2576510 | 2323093 | 25 | 86 | 61 | 41.25 | 38.08 | 8 | yes |
| 75399 | 17 | 2576679 | 2325405 | 16 | 55 | 39 | 30.79 | 12.22 | 12 | yes |
| 75400 | 17 | 2576892 | 2327769 | 20 | 38 | 18 | 5.79 | -5.95 | 10 | yes |
| 75401 | 18 | 2578498 | 2330054 | 26 | 32.87 | 6.87 | -0.9 | -4.75 | 14 | yes |
| 61184 | 30 | 2578770 | 2375435 | 38 | 174 | 136 | 58.57 | -141.43 | 23 | yes |
| 75565 | 18 | 2580601 | 2336533 | 68 | 94 | 26 | 94.26 | 78.82 | 22 | yes |
| 75403 | 18 | 2581386 | 2330524 | 84 | 38 | -46 | -6.19 | -14.54 | 22 | yes |
| 75563 | 18 | 2582856 | 2337821 | 29 | 73 | 44 | 59.78 | 48.99 | 19 | yes |
| 71148 | 18 | 2585325 | 2337641 | 79 | 46 | -33 | -14.24 | -19.08 | 21 | yes |
| 71149 | 18 | 2585360 | 2337644 | 65 | 46 | -19 | -14.24 | -19.08 | 18 | no |
| 100979 | 2 | 2596905 | 2296875 | 3 | 8.17 | 5.17 | 3.95 | -0.02 | 0 | yes |
| 100978 | 2 | 2597197 | 2297400 | 1 | 4.94 | 3.94 | -2.9 | -4.9 | 0 | yes |
| 100980 | 2 | 2597499 | 2296386 | 4 | 11.24 | 7.24 | 2.14 | -0.03 | 0 | yes |
| 100977 | 2 | 2598224 | 2296343 | 3 | 8.67 | 5.67 | 16.94 | -0.06 | 2 | no |
| 100975 | 2 | 2598284 | 2295929 | 3 | 11.09 | 8.09 | 12.05 | -0.04 | 2 | no |
| 100976 | 2 | 2598608 | 2296044 | 3 | 6.31 | 3.31 | 9.39 | -0.09 | 2 | no |
| 79775 | 18 | 2600542 | 2364722 | 141 | 75 | -66 | -32.4 | -45.44 | 20 | yes |
| 79774 | 30 | 2600569 | 2364712 | 0 | 75 | n/a | -189.08 | -389.08 | 30 | no |
| 110729 | 30 | 2601696 | 2376617 | 68 | 105 | 37 | 29.05 | -170.95 | 26 | yes |
| YG55 | 17 | 2602034 | 2367878 | 187 | 59.99 | -127.01 | -110.96 | -156.59 | 17 | no |
| 79784 | 18 | 2603345 | 2356315 | 21 | 252.83 | 231.83 | 249.6 | 229.92 | 18 | no |
| YG49 | 17 | 2604822 | 2368537 | 221 | 56 | -165 | -148.32 | -164.89 | 18 | yes |
| YG43 | 17 | 2605523 | 2370851 | 168 | 63 | -105 | -90.02 | -131.46 | 17 | no |
| YG34 | 10 | 2608727 | 2371814 | 58 | 68.1 | 10.1 | 32.18 | 27.29 | 14 | yes |
| 107971 | 17 | 2609634 | 2371458 | 111 | 78.97 | -32.03 | 20.67 | -58.85 | 17 | no |
| 107970 | 17 | 2609634 | 2371458 | 66 | 78.97 | 12.97 | 20.67 | -58.85 | 17 | no |
| 107972 | 18 | 2609981 | 2369233 | 226 | 65 | -161 | -139.37 | -141.37 | 28 | yes |
| 107973 | 17 | 2609981 | 2369233 | 136 | 65 | -71 | -52.9 | -139.37 | 17 | no |
| N3788 | 17 | 2611466 | 2370191 | 98 | 58 | -40 | -10.2 | -59.61 | 17 | no |
| 84155 | 18 | 2612123 | 2364811 | 156 | 133.83 | -22.17 | -5.59 | -7.59 | 26 | yes |
| N3789 | 23 | 2612432 | 2369327 | 216 | 75.39 | -140.61 | -73.07 | -75.07 | 30 | yes |
| N3787 | 12 | 2613479 | 2368347 | 222 | 86 | -136 | -40.22 | -54.95 | 30 | yes |
| 110731 | 18 | 2613789 | 2357471 | 69 | 90.11 | 21.11 | 39.36 | 10.53 | 18 | no |
| N3726 | 23 | 2614021 | 2367934 | 197 | 103.98 | -93.02 | -52.08 | -54.08 | 30 | yes |
| 84156 | 18 | 2614468 | 2365921 | 209 | 163.05 | -45.95 | -50.33 | -52.33 | 17 | yes |
| N3607 | 17 | 2614508 | 2361004 | 97 | 131.69 | 34.69 | 56.98 | 54.98 | 19 | yes |
| N3615 | 14 | 2614602 | 2359409 | 69 | 89.21 | 20.21 | 57.85 | 55.85 | 18 | yes |
| N3799 | 23 | 2614973 | 2366950 | 236 | 140.35 | -95.65 | -39.56 | -41.56 | 30 | yes |
| N3567 | 17 | 2615132 | 2361777 | 135 | 139.88 | 4.88 | 40.84 | 38.84 | 21 | yes |
| Y135 | 10 | 2615627 | 2350900 | 330 | 79.89 | -250.11 | -106.88 | -108.88 | 19 | yes |
| N3270 | 13 | 2615782 | 2358819 | 49 | 66 | 17 | 39.35 | 25.36 | 17 | yes |
| N3288 | 10 | 2615836 | 2357563 | 49 | 75 | 26 | 48.16 | 46.16 | 14 | yes |
| N3694 | 14 | 2615904 | 2357479 | 75 | 75 | 0 | 32.37 | 24.17 | 18 | yes |
| N5952 | 14 | 2616110 | 2360737 | 144 | 87 | -57 | -14.87 | -19.87 | 18 | yes |
| N4558 | 14 | 2616386 | 2362578 | 178 | 88.73 | -89.27 | -64.61 | -81.05 | 15 | yes |
| N3263 | 14 | 2616411 | 2359248 | 102 | 67.94 | -34.06 | 11.07 | 4.84 | 18 | yes |
| N4651 | 9 | 2616643 | 2362376 | 194 | 79 | -115 | 30.23 | 28.23 | 15 | yes |
| N3780 | 17 | 2616695 | 2361984 | 200 | 81 | -119 | -114.94 | -130.54 | 17 | no |
| N3369 | 10 | 2616696 | 2361995 | 145 | 81 | -64 | 33.37 | 28.84 | 14 | yes |
| N4652 | 9 | 2616749 | 2363272 | 219 | 80.36 | -138.64 | 31.76 | 29.76 | 16 | yes |
| N3570 | 23 | 2616929 | 2358071 | 283 | 62 | -221 | -211.1 | -482.18 | 23 | no |
| N3271 | 14 | 2617255 | 2359681 | 120 | 59 | -61 | -45.73 | -65.57 | 14 | no |
| N2491 | 14 | 2617641 | 2358877 | 117 | 52 | -65 | -22.33 | -41.69 | 16 | yes |
| Y122 | 17 | 2617726 | 2351085 | 410 | 74 | -336 | -278.86 | -283.34 | 20 | yes |
| N4541 | 9 | 2618455 | 2361561 | 228 | 49 | -179 | -11.47 | -13.47 | 16 | yes |
| H1095 | 14 | 2618694 | 2357662 | 106 | 72 | -34 | 21.44 | 12.17 | 17 | yes |
| H1502 | 10 | 2618826 | 2354188 | 347 | 73 | -274 | -102.74 | -148.07 | 18 | yes |
| M3787 | 3 | 2619925 | 2364195 | 10 | 44.93 | 34.93 | 35.04 | 31.35 | 3 | no |
| Y152 | 23 | 2620417 | 2350817 | 645 | 96.97 | -548.03 | -533.59 | -705.45 | 23 | no |
| M2758 | 14 | 2620950 | 2363802 | 276 | 69 | -207 | -184.49 | -211.95 | 14 | no |
| H1320 | 23 | 2621603 | 2355955 | 571 | 82 | -489 | -474.01 | -974.01 | 23 | no |
| H1691 | 18 | 2622148 | 2352910 | 429 | 105 | -324 | -202.01 | -259.54 | 20 | yes |
| M3282 | 9 | 2623325 | 2364252 | 250 | 72 | -178 | -30.31 | -32.31 | 14 | yes |
| TE1694 | 17 | 2623554 | 2369626 | 355 | 33.99 | -321.01 | -249.75 | -259.32 | 30 | yes |
| H1726 | 22 | 2623761 | 2351447 | 365 | 154.81 | -210.19 | -181.26 | -219.72 | 22 | no |
| H1719 | 20 | 2624946 | 2354097 | 352 | 112 | -240 | -183.63 | -239.99 | 21 | yes |
| H1348 | 12 | 2625037 | 2356496 | 643 | 109 | -534 | -130.55 | -216.32 | 23 | yes |
| H1632 | 17 | 2625505 | 2358509 | 703 | 88.1 | -614.9 | -519.68 | -521.68 | 22 | yes |
| H1631 | 12 | 2625507 | 2358524 | 298 | 88.1 | -209.9 | -197.54 | -269.22 | 12 | no |
| H1333 | 16 | 2625515 | 2358500 | 600 | 88.1 | -511.9 | -509.26 | -519.68 | 16 | no |
| M3101 | 17 | 2625832 | 2362004 | 694 | 62 | -632 | -489.94 | -496.25 | 23 | yes |
| M3054 | 14 | 2626343 | 2368322 | 357 | 57.25 | -299.75 | -266.75 | -307.76 | 14 | no |
| M942 | 7 | 2626659 | 2367886 | 529 | 43 | -486 | -27.79 | -124.35 | 20 | yes |
| T487 | 9 | 2627182 | 2355413 | 303 | 116.99 | -186.01 | 1.97 | -1.73 | 15 | yes |
| M3190 | 14 | 2627377 | 2365373 | 463 | 47.41 | -415.59 | -341.02 | -456.66 | 14 | no |
| 110724 | 9 | 2627413 | 2313451 | 119 | 9 | -110 | -43.98 | -55.23 | 14 | yes |
| T426 | 12 | 2628599 | 2357891 | 202 | 106.98 | -95.02 | -85.74 | -126.72 | 12 | no |
| BB195 | 23 | 2629104 | 2370602 | 480 | 43 | -437 | -359.07 | -460.48 | 23 | no |
| T493 | 10 | 2629164 | 2363251 | 267 | 60.3 | -206.7 | -210.32 | -246.92 | 8 | yes |
| T494 | 23 | 2629164 | 2363251 | 712 | 60.3 | -651.7 | -618.72 | -1118.7 | 23 | no |
| T454 | 23 | 2630325 | 2359681 | 644 | 100.97 | -543.03 | -544.64 | -1044.6 | 22 | yes |
| T440 | 14 | 2630346 | 2359679 | 294 | 100.97 | -193.03 | -174.39 | -293.37 | 14 | no |
| BB196 | 9 | 2630715 | 2369875 | 360 | 35.98 | -324.02 | -71.88 | -90.01 | 15 | yes |
| T256 | 23 | 2631780 | 2357668 | 536 | 159.39 | -376.61 | -358.35 | -858.35 | 23 | no |
| T485 | 23 | 2632831 | 2367857 | 704 | 44 | -660 | -598.85 | -1098.8 | 23 | no |
| TS37 | 23 | 2632959 | 2370884 | 607 | 34 | -573 | -557.39 | -856.45 | 23 | no |
| T489 | 23 | 2633154 | 2360587 | 627 | 120.34 | -506.66 | -439.08 | -939.08 | 23 | no |
| T445 | 18 | 2633252 | 2363852 | 632 | 87 | -545 | -451.29 | -473.66 | 22 | yes |
| T466 | 23 | 2633307 | 2364240 | 722 | 77.06 | -644.94 | -574.96 | -1075 | 23 | no |
| T442 | 23 | 2633472 | 2354615 | 149 | 143.63 | -5.37 | -32.83 | -532.83 | 21 | yes |
| T495 | 23 | 2633534 | 2355971 | 242 | 122.24 | -119.76 | -146.93 | -646.93 | 22 | yes |
| T496 | 19 | 2633534 | 2355973 | 182 | 122.24 | -59.76 | 47.83 | 25.14 | 21 | yes |
| LY3298 | 23 | 2634759 | 2355782 | 243 | 84 | -159 | -108.99 | -608.99 | 23 | no |
| LY3299 | 20 | 2634760 | 2355786 | 115 | 84 | -31 | 34.89 | -13.07 | 21 | yes |
| LY3055 | 20 | 2634887 | 2362019 | 420 | 55 | -365 | -323.31 | -366.79 | 20 | no |
| T433 | 12 | 2634984 | 2366881 | 353 | 39.76 | -313.24 | -287.33 | -308.72 | 13 | yes |
| T491 | 20 | 2635423 | 2362597 | 529 | 62.98 | -466.02 | -385.32 | -419.78 | 22 | yes |
| LY2477 | 23 | 2635544 | 2357010 | 226 | 82.31 | -143.69 | -150.18 | -650.18 | 22 | yes |
| TS40 | 11 | 2636291 | 2371312 | 330 | 31.6 | -298.39 | -168.16 | -305.8 | 11 | no |
| 110721 | 9 | 2637094 | 2324642 | 25 | 26 | 1 | 11.19 | -13.5 | 9 | no |
| LY3118 | 23 | 2637659 | 2364370 | 607 | 90 | -517 | -510.39 | -1010.4 | 23 | no |
| LY3119 | 10 | 2637670 | 2364367 | 150 | 90 | -60 | -79.2 | -129.89 | 8 | yes |
| TS38 | 20 | 2637732 | 2372083 | 575 | 30 | -545 | -507.3 | -527.89 | 21 | yes |
| TS32 | 17 | 2639294 | 2375777 | 430 | 54 | -376 | -302.87 | -338.98 | 23 | yes |
| LY2268 | 14 | 2639531 | 2367013 | 451 | 51 | -400 | -352.69 | -442.46 | 14 | no |
| LY2676 | 20 | 2639563 | 2367251 | 705 | 51 | -654 | -571.26 | -606.51 | 22 | yes |
| TS42 | 23 | 2640827 | 2372258 | 621 | 31 | -590 | -578.57 | -1078.6 | 23 | no |
| LY2472 | 23 | 2642202 | 2363504 | 570 | 60 | -510 | -500.11 | -1000.1 | 23 | no |
| 76074 | 30 | 2643006 | 2367953 | 0 | 31 | n/a | -2530.2 | -2730.2 | 30 | no |
| 110722 | 9 | 2643027 | 2367954 | 0 | 31 | n/a | -177.81 | -185.53 | 9 | no |
| TB212 | 23 | 2643128 | 2354433 | 193 | 186.11 | -6.89 | 35.64 | -464.36 | 23 | no |
| TN25 | 30 | 2643402 | 2381160 | 433 | 70.83 | -362.17 | -477.9 | -677.9 | 23 | yes |
| TS34 | 30 | 2643796 | 2378073 | 514 | 52 | -462 | -1051.7 | -1251.7 | 23 | yes |
| TB214 | 12 | 2644238 | 2359737 | 117 | 91.87 | -25.13 | 63.6 | -22.36 | 13 | yes |
| 76079 | 18 | 2644285 | 2368211 | 703 | 33.71 | -669.29 | -569.95 | -571.95 | 21 | yes |
| TB213 | 12 | 2644367 | 2360412 | 147 | 88 | -59 | 56.17 | -38.24 | 14 | yes |
| TB220 | 12 | 2644636 | 2358912 | 256 | 110.4 | -145.6 | 7.73 | -89.14 | 19 | yes |
| LY2678 | 23 | 2644987 | 2366319 | 691 | 63 | -628 | -639.33 | -1139.3 | 22 | yes |
| LY2310 | 14 | 2644989 | 2366328 | 347 | 63 | -284 | -264.49 | -339.06 | 14 | no |
| TB205 | 19 | 2645186 | 2357946 | 189 | 153.35 | -35.65 | 56.21 | -37.66 | 19 | no |
| TB198 | 20 | 2645269 | 2358285 | 199 | 167.84 | -31.16 | -22.09 | -76.9 | 20 | no |
| TB176 | 20 | 2645685 | 2359864 | 528 | 119.92 | -408.08 | -308.82 | -348.52 | 22 | yes |
| TB165 | 9 | 2645751 | 2359904 | 271 | 119.92 | -151.08 | 78.42 | 76.42 | 15 | yes |
| TB167 | 22 | 2645828 | 2354613 | 196 | 217.38 | 21.38 | 55.43 | -88.28 | 22 | no |
| LY1967 | 23 | 2646204 | 2364366 | 656 | 77 | -579 | -619.58 | -1119.6 | 22 | yes |
| LY1979 | 14 | 2646328 | 2364358 | 400 | 77 | -323 | -310.14 | -373.82 | 14 | no |
| LY2883 | 3 | 2646335 | 2364355 | 71 | 77 | 6 | 50.74 | 31.28 | 5 | yes |
| LY2809 | 9 | 2646402 | 2364342 | 456 | 77 | -379 | -122.91 | -124.91 | 15 | yes |
| LY2810 | 20 | 2646418 | 2364339 | 658 | 77 | -581 | -472.35 | -521.43 | 22 | yes |
| 105220 | 18 | 2646432 | 2327644 | 329 | 27.7 | -301.3 | -223.96 | -225.96 | 21 | yes |
| TS36 | 23 | 2647000 | 2372700 | 743 | 29 | -714 | -717.22 | -1217.2 | 22 | yes |
| LY2269 | 14 | 2647013 | 2368728 | 403 | 44.36 | -358.64 | -331.64 | -408.88 | 14 | no |
| TN24 | 30 | 2647139 | 2386406 | 358 | 62 | -296 | -214.75 | -414.75 | 30 | no |
| R330 | 23 | 2647174 | 2361701 | 665 | 85.25 | -579.75 | -563.82 | -1063.8 | 23 | no |
| TS43 | 20 | 2648186 | 2375355 | 644 | 27.98 | -616.02 | -550.06 | -570.42 | 23 | yes |
| 105222 | 18 | 2648660 | 2325903 | 318 | 24 | -294 | -173.26 | -230.01 | 20 | yes |
| 105221 | 18 | 2649379 | 2327361 | 185 | 40.77 | -144.23 | -43.06 | -45.06 | 21 | yes |
| 45759 | 3 | 2650353 | 2378495 | 20 | 41 | 21 | 29.06 | 21.69 | 4 | yes |
| 45760 | 3 | 2650353 | 2378495 | 12 | 41 | 29 | 29.06 | 21.69 | 3 | no |
| 103811 | 8 | 2650359 | 2378494 | 78 | 41 | -37 | -103.25 | -134.25 | 6 | yes |
| 147173 | 18 | 2650869 | 2325257 | 238 | 16 | -222 | -217.72 | -219.72 | 19 | yes |
| R324 | 9 | 2651453 | 2369213 | 400 | 35 | -365 | -119.14 | -139.12 | 14 | yes |
| R340 | 9 | 2651482 | 2369201 | 558 | 35 | -523 | -119.14 | -139.12 | 15 | yes |
| R344 | 3 | 2651644 | 2362903 | 451 | 71.78 | -379.22 | 84.51 | 69.33 | 23 | yes |
| 147174 | 9 | 2652295 | 2317015 | 156 | 4.72 | -151.28 | -216.4 | -218.4 | 7 | yes |
| 110726 | 18 | 2652400 | 2324979 | 520 | 19 | -501 | -259.87 | -261.87 | 21 | yes |
| 45757 | 3 | 2653706 | 2377948 | 13 | 29 | 16 | 21.81 | 13.81 | 3 | no |
| 45756 | 3 | 2653706 | 2377948 | 28 | 29 | 1 | 21.81 | 13.81 | 5 | yes |
| WN47 | 9 | 2654438 | 2372380 | 452 | 60 | -392 | -174.54 | -196.14 | 13 | yes |
| WL196 | 23 | 2654621 | 2358697 | 402 | 135.54 | -266.46 | -216.36 | -716.36 | 23 | no |
| WL197 | 21 | 2654640 | 2358703 | 358 | 135.54 | -222.46 | -133.56 | -147.24 | 23 | yes |
| 45758 | 2 | 2655668 | 2374732 | 6 | 23 | 17 | 34.07 | 11.35 | 2 | no |
| R343 | 22 | 2656046 | 2364512 | 358 | 87.86 | -270.14 | -200.25 | -305.38 | 22 | no |
| 103822 | 4 | 2657145 | 2377374 | 91 | 23 | -68 | -9.06 | -129.65 | 4 | no |
| 103820 | 8 | 2658261 | 2384698 | 64 | 37 | -27 | -27.73 | -43.22 | 7 | yes |
| 45764 | 3 | 2658276 | 2375990 | 20 | 21 | 1 | 12.46 | -45.73 | 3 | no |
| 45765 | 3 | 2658276 | 2375990 | 12 | 21 | 9 | 12.46 | -45.73 | 3 | no |
| WN52 | 20 | 2658441 | 2379797 | 703 | 28 | -675 | -577.22 | -608.74 | 23 | yes |
| DN58 | 9 | 2659538 | 2372849 | 572 | 31 | -541 | -158.8 | -219.8 | 15 | yes |
| 45761 | 3 | 2660163 | 2375868 | 24 | 23 | -1 | 11.04 | -15.53 | 3 | no |
| 45762 | 3 | 2660163 | 2375868 | 11 | 23 | 12 | 11.04 | -15.53 | 2 | yes |
| 45763 | 2 | 2660163 | 2375868 | 4 | 23 | 19 | 34.82 | 11.04 | 2 | no |
| R323 | 11 | 2660757 | 2366250 | 219 | 109.16 | -109.84 | 54.83 | 4.51 | 14 | yes |
| 89809 | 18 | 2661060 | 2370220 | 804 | 15 | -789 | -708.65 | -710.65 | 22 | yes |
| R325 | 12 | 2661080 | 2370219 | 373 | 15 | -358 | -324.71 | -357.05 | 13 | yes |
| 145094 | 5 | 2661499 | 2332102 | 0 | 57.12 | n/a | 6.56 | -17.14 | 5 | no |
| DN56 | 9 | 2662121 | 2373826 | 434 | 42 | -392 | -133.49 | -135.49 | 14 | yes |
| 58937 | 18 | 2662747 | 2379792 | 710 | 23.89 | -686.11 | -674.39 | -676.39 | 19 | yes |
| HP207 | 21 | 2663024 | 2361507 | 359 | 165.04 | -193.96 | -107.14 | -121.52 | 23 | yes |
| S51 | 23 | 2663451 | 2353016 | 362 | 196.51 | -165.49 | -165.84 | -665.84 | 22 | yes |
| HP221 | 19 | 2664291 | 2362982 | 327 | 161.24 | -165.76 | 17.32 | -7.26 | 23 | yes |
| 59308 | 4 | 2664623 | 2381390 | 65 | 23 | -42 | 2.46 | -89.14 | 4 | no |
| HP196 | 23 | 2665014 | 2362962 | 339 | 154.74 | -184.26 | -143.19 | -643.19 | 23 | no |
| HP204 | 10 | 2665364 | 2369310 | 313 | 17.16 | -295.84 | -157.11 | -167.82 | 14 | yes |
| 92118 | 18 | 2665789 | 2349930 | 214 | 154.47 | -59.53 | -28.7 | -30.7 | 19 | yes |
| DN341 | 22 | 2665917 | 2381168 | 692 | 25.5 | -666.5 | -675.35 | -697.84 | 21 | yes |
| 58934 | 5 | 2666053 | 2374972 | 122 | 17 | -105 | -82.96 | -109.81 | 5 | no |
| 145090 | 5 | 2666243 | 2324016 | 0 | 12 | n/a | -52.3 | -79.06 | 5 | no |
| HP220 | 19 | 2667289 | 2368318 | 302 | 41.45 | -260.55 | -219.6 | -245.22 | 20 | yes |
| HP177 | 21 | 2667300 | 2365125 | 183 | 130.59 | -52.41 | -24.93 | -47.56 | 22 | yes |
| HP188 | 21 | 2667311 | 2365094 | 229 | 130.59 | -98.41 | -24.93 | -47.56 | 23 | yes |
| HP189 | 21 | 2667314 | 2365075 | 216 | 130.59 | -85.41 | -24.93 | -47.56 | 23 | yes |
| 95482 | 5 | 2667529 | 2388040 | 67 | 26 | -41 | -30.89 | -42.92 | 5 | no |
| 67442 | 30 | 2669134 | 2369326 | 120 | 19.06 | -100.94 | -3067.4 | -3267.4 | 8 | yes |
| 105134 | 9 | 2669366 | 2376001 | 0 | 31.72 | n/a | -143.23 | -206.54 | 9 | no |
| 110725 | 18 | 2670480 | 2376025 | 668 | 32 | -636 | -556.96 | -558.96 | 22 | yes |
| 67441 | 18 | 2671405 | 2368736 | 889 | 39.1 | -849.9 | -628.01 | -639.27 | 22 | yes |
| C97 | 21 | 2671536 | 2359557 | 254 | 118.72 | -135.28 | 2.25 | -41.48 | 22 | yes |
| 105132 | 3 | 2671954 | 2376198 | 28 | 31 | 3 | 21.05 | -24.92 | 3 | no |
| 105196 | 5 | 2672137 | 2378998 | 123 | 18 | -105 | -91.06 | -103.21 | 6 | yes |
| C93 | 21 | 2672162 | 2363449 | 126 | 142.33 | 16.33 | 29.92 | -4.53 | 21 | no |
| 105547 | 5 | 2673156 | 2374585 | 154 | 13 | -141 | -112.64 | -128.83 | 6 | yes |
| C96 | 21 | 2673608 | 2361845 | 222 | 152.61 | -69.39 | 17.12 | -17.52 | 22 | yes |
| 98028 | 9 | 2673618 | 2404058 | 55 | 52 | -3 | 33.72 | 27.5 | 14 | yes |
| C94 | 23 | 2673622 | 2361830 | 233 | 152.61 | -80.39 | -76.9 | -576.9 | 23 | no |
| 104536 | 18 | 2673691 | 2328292 | 993 | 2 | -991 | -834.63 | -836.63 | 21 | yes |
| BDL11 | 2 | 2674741 | 2382600 | 0 | 12 | n/a | 9.97 | 7.97 | 2 | no |
| 52752 | 9 | 2675691 | 2390117 | 115 | 32 | -83 | -47.92 | -102.93 | 9 | no |
| 51532 | 30 | 2676358 | 2405156 | 0 | 49 | n/a | -39.38 | -239.38 | 30 | no |
| 105548 | 5 | 2677006 | 2374317 | 145 | 24 | -121 | -104.08 | -126.04 | 5 | no |
| 52754 | 18 | 2677284 | 2377910 | 879 | 8 | -871 | -760.31 | -762.31 | 21 | yes |
| 52753 | 5 | 2677730 | 2379523 | 169 | 17 | -152 | -104.62 | -115.14 | 9 | yes |
| 110172 | 2 | 2679710 | 2404836 | 3 | 53 | 50 | 39.31 | 37.31 | 0 | yes |
| 64835 | 18 | 2679945 | 2359719 | 233 | 78.46 | -154.54 | -66.05 | -68.05 | 20 | yes |
| 110171 | 3 | 2679975 | 2404783 | 6 | 53 | 47 | 37.31 | 33.59 | 0 | yes |
| 92176 | 9 | 2680120 | 2396546 | 73 | 35.43 | -37.57 | -31.77 | -57.61 | 9 | no |
| 92175 | 18 | 2681697 | 2401558 | 87 | 67.8 | -19.2 | -12.4 | -35.13 | 18 | no |
| 145092 | 5 | 2682235 | 2344458 | 0 | 27 | n/a | -56.05 | -73.52 | 5 | no |
| 110166 | 3 | 2682626 | 2406700 | 6 | 65 | 59 | 60.56 | 58.56 | 3 | no |
| 110167 | 3 | 2682829 | 2406593 | 5 | 65 | 60 | 60.56 | 58.56 | 3 | no |
| 110168 | 3 | 2683040 | 2406454 | 6 | 64 | 58 | 54.51 | 52.51 | 0 | yes |
| 90149 | 5 | 2683141 | 2370724 | 143 | 3.88 | -139.12 | -108.83 | -122.79 | 7 | yes |
| G46 | 19 | 2683676 | 2364817 | 200 | 59.58 | -140.42 | -13.29 | -56.53 | 22 | yes |
| WWK7 | 23 | 2683864 | 2357055 | 799 | 51.5 | -747.5 | -721.54 | -1221.5 | 23 | no |
| 86669 | 5 | 2684013 | 2386136 | 85 | 9 | -76 | -73.44 | -81.29 | 5 | no |
| 92177 | 5 | 2684210 | 2394150 | 38 | 49 | 11 | 11.1 | -3.26 | 5 | no |
| 86464 | 5 | 2685541 | 2378892 | 105 | 8 | -97 | -99.36 | -111.68 | 4 | yes |
| 86670 | 4 | 2685542 | 2378884 | 59 | 8 | -51 | -19.53 | -99.36 | 4 | no |
| 86726 | 3 | 2685548 | 2378915 | 12 | 8 | -4 | 0.57 | -19.53 | 3 | no |
| G54 | 19 | 2685958 | 2367270 | 234 | 54 | -180 | -113.42 | -162.36 | 20 | yes |
| 90148 | 5 | 2688532 | 2374035 | 106 | 3 | -103 | -81.79 | -92.85 | 6 | yes |
| 105483 | 18 | 2688625 | 2351378 | 1029 | 28 | -1001 | -736.02 | -738.02 | 21 | yes |
| 105484 | 5 | 2688656 | 2351346 | 52 | 28 | -24 | -20.12 | -47 | 5 | no |
| 145091 | 5 | 2692353 | 2345871 | 0 | 1.99 | n/a | -47.37 | -63.28 | 5 | no |
| 109044 | 5 | 2693425 | 2391702 | 0 | 44.84 | n/a | -11.44 | -40.29 | 5 | no |
| G66 | 22 | 2693472 | 2368179 | 801 | 6 | -795 | -688.26 | -895.73 | 22 | no |
| 90400 | 15 | 2694503 | 2373976 | 787 | 0.73 | -786.27 | -682.64 | -840.38 | 15 | no |
| 90366 | 16 | 2694529 | 2374029 | 830 | 0.73 | -829.27 | -840.38 | -842.87 | 15 | yes |
| 86465 | 5 | 2694549 | 2380502 | 112 | 4.99 | -107.01 | -91.65 | -114.1 | 5 | no |
| 86466 | 5 | 2694551 | 2380500 | 112 | 4.99 | -107.01 | -91.65 | -114.1 | 5 | no |
| 77947 | 8 | 2700775 | 2386061 | 107 | 30.17 | -76.83 | -91.3 | -100.86 | 6 | yes |
| 77945 | 18 | 2700814 | 2386079 | 775 | 30.17 | -744.83 | -721.9 | -723.9 | 19 | yes |
| 105478 | 9 | 2708356 | 2411556 | 48 | 39 | -9 | -15.12 | -17.12 | 5 | yes |
| 105479 | 2 | 2708360 | 2411571 | 13 | 39 | 26 | 29 | -3.12 | 2 | no |
| 105392 | 2 | 2709725 | 2411335 | 22 | 37 | 15 | 30.5 | -16.12 | 2 | no |
| 50876 | 5 | 2709899 | 2370039 | 57 | 5 | -52 | -50.62 | -110.61 | 5 | no |
| 110177 | 9 | 2709953 | 2411714 | 39 | 37 | -2 | -28.12 | -30.12 | 2 | yes |
| 145093 | 5 | 2710101 | 2362746 | 0 | 10 | n/a | -34.85 | -74.67 | 5 | no |
| 105477 | 2 | 2710822 | 2410369 | 3 | 28.23 | 25.23 | 23.91 | -10.07 | 0 | yes |
| 56545 | 15 | 2711368 | 2394903 | 43 | 46.67 | 3.67 | -371.08 | -424.47 | 4 | yes |
| 105476 | 2 | 2711423 | 2411015 | 6 | 32 | 26 | 30.24 | -1.97 | 2 | no |
| 56541 | 2 | 2711424 | 2409624 | 0 | 32 | n/a | 37.01 | 27.78 | 2 | no |
| 56546 | 2 | 2711451 | 2409610 | 5 | 32 | 27 | 37.01 | 27.78 | 3 | yes |
| 56548 | 9 | 2712147 | 2409756 | 6 | 30 | 24 | 0.79 | -4.03 | 2 | yes |
| 56540 | 9 | 2712152 | 2409786 | 3 | 30 | 27 | 0.79 | -4.03 | 2 | yes |
| 111800 | 9 | 2712155 | 2409797 | 58 | 30 | -28 | 0.79 | -4.03 | 14 | yes |
| 56539 | 9 | 2712249 | 2410310 | 5 | 31 | 26 | -3.78 | -7.43 | 0 | yes |
| 56536 | 9 | 2713306 | 2409284 | 4 | 26 | 22 | 9.28 | 2.13 | 2 | yes |
| 56537 | 9 | 2713402 | 2409814 | 5 | 26 | 21 | 0.4 | -4.14 | 2 | yes |
| 56550 | 9 | 2713411 | 2409842 | 6 | 26 | 20 | 0.4 | -4.14 | 2 | yes |
| 56538 | 9 | 2713520 | 2410442 | 5 | 27 | 22 | -2.57 | -6.09 | 2 | yes |
| 56533 | 2 | 2714924 | 2409073 | 3 | 23 | 20 | 22.69 | -5.37 | 2 | no |
| 56534 | 2 | 2715048 | 2409763 | 5 | 24 | 19 | 22.14 | -17.1 | 2 | no |
| 56535 | 2 | 2715143 | 2410293 | 3 | 23 | 20 | 20.95 | -7.22 | 2 | no |
| 90615 | 5 | 2716462 | 2376783 | 65 | 8 | -57 | -60.44 | -89.95 | 4 | yes |
| 56531 | 2 | 2716561 | 2408898 | 3 | 21 | 18 | 20.33 | -27.94 | 2 | no |
| 56532 | 2 | 2716677 | 2409530 | 4 | 23 | 19 | 19.94 | -33.21 | 2 | no |
| 140692 | 5 | 2717298 | 2389460 | 69 | 26 | -43 | -20.03 | -90.37 | 5 | no |
| 56744 | 2 | 2718291 | 2409242 | 4 | 19.95 | 15.95 | 19.03 | -3.13 | 2 | no |
| 110165 | 2 | 2718609 | 2408008 | 4 | 18 | 14 | 19.2 | -7.09 | 2 | no |
| 56530 | 2 | 2719538 | 2406888 | 4 | 17 | 13 | 19.89 | 3.52 | 2 | no |
| 56528 | 2 | 2719629 | 2407381 | 2 | 18 | 16 | 19 | -4.99 | 2 | no |
| 56529 | 2 | 2719718 | 2407957 | 7 | 18 | 11 | 18.9 | -7.89 | 2 | no |
| 105480 | 2 | 2720462 | 2407322 | 3 | 17 | 14 | 18 | -3.82 | 2 | no |
| 80760 | 2 | 2721827 | 2406046 | 2 | 12 | 10 | 17 | 0.79 | 2 | no |
| 80761 | 2 | 2722005 | 2406657 | 4 | 15 | 11 | 17.96 | -19.54 | 2 | no |
| 80762 | 2 | 2722108 | 2407287 | 2 | 15 | 13 | 17.84 | -5.37 | 2 | no |
| 140279 | 2 | 2723314 | 2405478 | 6 | 11 | 5 | 18.25 | -4.03 | 2 | no |
| 65762 | 5 | 2723813 | 2388361 | 71 | 25 | -46 | -23.71 | -92.66 | 5 | no |
| 140281 | 15 | 2727673 | 2405062 | 0 | 8 | n/a | -110.67 | -127.08 | 15 | no |
| 140280 | 3 | 2728606 | 2406724 | 11 | 4.22 | -6.78 | -1.79 | -22 | 3 | no |
| 105733 | 18 | 2730124 | 2408060 | 149 | 53 | -96 | -97.33 | -117.05 | 17 | yes |
| 46968 | 5 | 2730196 | 2394362 | 36 | 37 | 1 | -7.97 | -50.85 | 4 | yes |
| 105725 | 30 | 2730404 | 2413138 | 0 | 28 | n/a | -532.58 | -732.58 | 30 | no |
| 47063 | 18 | 2730849 | 2396570 | 553 | 36 | -517 | -502.22 | -507.36 | 20 | yes |
| 105728 | 18 | 2731009 | 2408220 | 132 | 30 | -102 | -69.75 | -81.19 | 20 | yes |
| 65764 | 5 | 2731255 | 2389067 | 45 | 0 | -45 | -41.48 | -80.4 | 5 | no |
| 105730 | 18 | 2732144 | 2408798 | 83 | 36 | -47 | -19.54 | -30.56 | 20 | yes |
| 144468 | 3 | 2797767 | 2417344 | 15 | 6 | -9 | 9.69 | 4.37 | 7 | yes |
| 144469 | 30 | 2798169 | 2419147 | 31 | 3.14 | -27.86 | -47.19 | -247.19 | 20 | yes |
| 115733 | 30 | 2798901 | 2418392 | 29 | 12 | -17 | -46.03 | -246.03 | 15 | yes |
| 144467 | 15 | 2799610 | 2415561 | 32 | 14 | -18 | -20.92 | -22.92 | 13 | yes |
| 115734 | 15 | 2801774 | 2416059 | 27 | 9 | -18 | -18.89 | -20.89 | 14 | yes |
| 115732 | 15 | 2804218 | 2414545 | 29 | 7 | -22 | -24.46 | -26.46 | 13 | yes |
| 41973 | 3 | 2660288 | 2376718 | 13.71 | 22 | 8.29 | 12.93 | 2.45 | 3 | no |
| 42001 | 2 | 2656732 | 2377395 | 10.05 | 23 | 12.95 | 27.18 | 16.01 | 3 | yes |
| 42002 | 3 | 2663477 | 2376112 | 14.32 | 19 | 4.68 | 11.7 | -23.04 | 3 | no |
| 42003 | 3 | 2666460 | 2375529 | 10.66 | 23.95 | 13.29 | 15.47 | -0.6 | 3 | no |
| 42004 | 3 | 2655966 | 2377510 | 14.63 | 26 | 11.37 | 17.64 | -22.75 | 3 | no |
| 42005 | 2 | 2659365 | 2376894 | 12.49 | 23 | 10.51 | 34.08 | 13.2 | 3 | yes |
| 42006 | 3 | 2658709 | 2377052 | 16.15 | 25 | 8.85 | 14.34 | 2.49 | 3 | no |
| 42007 | 3 | 2665085 | 2375827 | 17.37 | 21 | 3.63 | 12.13 | -22.33 | 3 | no |
| 42020 | 3 | 2660710 | 2376652 | 18.59 | 22 | 3.41 | 12.81 | 1.35 | 3 | no |
| 42026 | 3 | 2657303 | 2377286 | 19.2 | 24 | 4.8 | 14.15 | -3.73 | 3 | no |
| 42027 | 3 | 2656397 | 2377442 | 18.89 | 22 | 3.11 | 16.75 | -33.32 | 3 | no |
| 42028 | 3 | 2657317 | 2378300 | 18.89 | 21 | 2.11 | 18.33 | -2.54 | 3 | no |
| 42038 | 3 | 2665681 | 2379023 | 13.71 | 21 | 7.29 | 18.75 | 6.25 | 3 | no |
| 42039 | 3 | 2658760 | 2378541 | 18.28 | 23 | 4.72 | 19.12 | -15.77 | 3 | no |
| 42041 | 3 | 2665377 | 2377422 | 15.84 | 21 | 5.16 | 17 | 1.51 | 3 | no |
| 42045 | 2 | 2655864 | 2377547 | 10.97 | 26 | 15.03 | 27.12 | 17.64 | 3 | yes |
| 42051 | 4 | 2664100 | 2379330 | 44 | 22 | -22 | -1.06 | -102.19 | 4 | no |
| 42058 | 3 | 2663738 | 2377525 | 20.42 | 20 | -0.42 | 17.25 | -0.61 | 3 | no |
| 42059 | 3 | 2654903 | 2385150 | 25.9 | 44.6 | 18.7 | 35.33 | 25.33 | 4 | yes |
| 42063 | 3 | 2657676 | 2379947 | 17.37 | 28 | 10.63 | 21.78 | 13.42 | 4 | yes |
| 42066 | 3 | 2667692 | 2380777 | 23.16 | 20.9 | -2.26 | 18.3 | 2.21 | 4 | yes |
| 42067 | 3 | 2664453 | 2381417 | 32.61 | 23 | -9.61 | 19.2 | 2.46 | 4 | yes |
| 42068 | 3 | 2662883 | 2381711 | 20.42 | 26 | 5.58 | 20.04 | -29.95 | 3 | no |
| 42069 | 3 | 2661361 | 2381995 | 19.2 | 31.1 | 11.9 | 20.08 | 1.41 | 3 | no |
| 42070 | 3 | 2659404 | 2382358 | 14.93 | 30 | 15.07 | 22.79 | 10.44 | 3 | no |
| 42071 | 3 | 2657925 | 2382929 | 12.19 | 36 | 23.81 | 27.12 | 17.42 | 3 | no |
| 42072 | 3 | 2656373 | 2383538 | 15.24 | 37 | 21.76 | 29.85 | 20.25 | 3 | no |
| 42073 | 9 | 2655126 | 2384033 | 85 | 40 | -45 | -43.81 | -74.33 | 9 | no |
| 42074 | 3 | 2656599 | 2384879 | 14.02 | 39 | 24.98 | 30.11 | 21.66 | 3 | no |
| 42075 | 3 | 2658253 | 2384589 | 17.67 | 37 | 19.33 | 28.51 | 20.1 | 4 | yes |
| 42076 | 3 | 2659861 | 2384312 | 13.1 | 32 | 18.9 | 25.19 | 15.77 | 3 | no |
| 42077 | 3 | 2661654 | 2383970 | 19.2 | 35 | 15.8 | 21.73 | 12.47 | 3 | no |
| 42078 | 3 | 2663670 | 2383687 | 18.89 | 28.99 | 10.1 | 19 | -9.47 | 3 | no |
| 42079 | 3 | 2665061 | 2384478 | 18.59 | 28 | 9.41 | 18 | -0.62 | 3 | no |
| 42080 | 2 | 2666693 | 2384226 | 10.05 | 28 | 17.95 | 19.95 | 17.95 | 3 | yes |
| 42081 | 3 | 2668281 | 2383852 | 14.93 | 26.24 | 11.31 | 12.78 | 0.14 | 3 | no |
| 42083 | 3 | 2669022 | 2379228 | 15.24 | 20 | 4.76 | 17 | 0.55 | 3 | no |
| 42084 | 3 | 2667005 | 2377117 | 15.24 | 21 | 5.76 | 16.53 | 5.54 | 3 | no |
| 42085 | 2 | 2656914 | 2386440 | 10.66 | 42 | 31.34 | 34.9 | 30.43 | 2 | no |
| 42086 | 3 | 2658488 | 2386159 | 18.89 | 37.7 | 18.81 | 26.62 | 21.66 | 4 | yes |
| 42087 | 3 | 2660108 | 2385873 | 18.89 | 37 | 18.11 | 23.27 | 18.41 | 4 | yes |
| 42088 | 3 | 2661859 | 2385103 | 18.89 | 34 | 15.11 | 20.95 | 16.13 | 4 | yes |
| 42095 | 3 | 2662487 | 2379615 | 22.25 | 26 | 3.75 | 22.95 | -63.88 | 3 | no |
| 42108 | 2 | 2660954 | 2379913 | 10.97 | 27 | 16.03 | 30.01 | 20.39 | 3 | yes |
| 42117 | 3 | 2661090 | 2378288 | 18.89 | 22.22 | 3.33 | 19.61 | -11.72 | 3 | no |
| 42118 | 3 | 2662146 | 2377826 | 14.1 | 20 | 5.9 | 19.19 | -3.53 | 3 | no |
| 42121 | 3 | 2670362 | 2377738 | 11.27 | 20.98 | 9.71 | 16.48 | -7.34 | 3 | no |
| 42122 | 3 | 2671190 | 2381876 | 14.32 | 23 | 8.68 | 14.8 | -7.23 | 3 | no |
| 42123 | 2 | 2672604 | 2380876 | 9.14 | 18 | 8.86 | 12.99 | 10.99 | 3 | yes |
| 42128 | 3 | 2672289 | 2378975 | 13.41 | 18 | 4.59 | 13.11 | -21.44 | 3 | no |
| 42134 | 2 | 2665666 | 2374594 | 11.35 | 18 | 6.65 | 16.79 | 11.33 | 3 | yes |
| 42135 | 2 | 2665652 | 2374445 | 6.75 | 18 | 11.25 | 16.51 | 10.74 | 2 | no |
| 42136 | 2 | 2666359 | 2374612 | 9.8 | 17 | 7.2 | 16.03 | 11.14 | 3 | yes |
| 42137 | 2 | 2667385 | 2374192 | 9.8 | 18 | 8.2 | 18.82 | 12.87 | 3 | yes |
| 42181 | 5 | 2664919 | 2374924 | 110 | 17 | -93 | -65.8 | -89.72 | 6 | yes |
| 45962 | 18 | 2643354 | 2317957 | 229.3 | 2 | -227.3 | -178.77 | -180.77 | 21 | yes |
| 50867 | 3 | 2708672 | 2372152 | 6.55 | 5 | -1.55 | -2.05 | -21.08 | 2 | yes |
| 50868 | 2 | 2708672 | 2372152 | 2.8 | 5 | 2.2 | 2.19 | -2.05 | 0 | yes |
| 50870 | 3 | 2709643 | 2369543 | 23 | 10 | -13 | 1.25 | -13.49 | 3 | no |
| 50871 | 3 | 2708309 | 2372760 | 21.3 | 0.1 | -21.2 | -2.69 | -20.93 | 4 | yes |
| 50872 | 3 | 2709643 | 2369543 | 23 | 10 | -13 | 1.25 | -13.49 | 3 | no |
| 50877 | 2 | 2708309 | 2372760 | 4.75 | 0.1 | -4.65 | 1.05 | -2.69 | 3 | yes |
| 50878 | 2 | 2708309 | 2372760 | 1.9 | 0.1 | -1.8 | 1.05 | -2.69 | 2 | no |
| 50891 | 2 | 2708794 | 2371999 | 3 | 3 | 0 | 2.83 | -2.06 | 2 | no |
| 51535 | 3 | 2683008 | 2406070 | 20 | 69 | 49 | 53.86 | 51.03 | 4 | yes |
| 51539 | 2 | 2682716 | 2405765 | 10.4 | 63 | 52.6 | 53.76 | 51.76 | 2 | no |
| 51575 | 2 | 2682229 | 2405778 | 10.66 | 61 | 50.34 | 51.09 | 49.09 | 2 | no |
| 51590 | 3 | 2682263 | 2405901 | 13.41 | 61 | 47.59 | 49.09 | 47.09 | 3 | no |
| 51592 | 2 | 2682001 | 2405356 | 28 | 60 | 32 | 48.89 | 46.89 | 10 | yes |
| 52872 | 3 | 2680268 | 2383579 | 25 | 15 | -10 | 9.99 | -26.28 | 3 | no |
| 52873 | 3 | 2677763 | 2383982 | 20 | 20 | 0 | 9.91 | -23.45 | 3 | no |
| 52874 | 3 | 2679068 | 2380040 | 20 | 16 | -4 | 9.24 | -22.87 | 3 | no |
| 52875 | 3 | 2679451 | 2381595 | 17.5 | 15 | -2.5 | 9.77 | -24.31 | 3 | no |
| 52877 | 3 | 2675601 | 2387655 | 14.87 | 15.71 | 0.84 | 15.61 | -0.36 | 3 | no |
| 52878 | 2 | 2675601 | 2387655 | 5.95 | 15.71 | 9.76 | 18.15 | 15.61 | 3 | yes |
| 52879 | 3 | 2680560 | 2386750 | 13.97 | 21 | 7.03 | 12.84 | -29.28 | 3 | no |
| 52880 | 2 | 2680560 | 2386750 | 10.02 | 21 | 10.98 | 18.59 | 12.84 | 3 | yes |
| 52882 | 3 | 2677763 | 2383982 | 12.69 | 20 | 7.31 | 9.91 | -23.45 | 3 | no |
| 52883 | 2 | 2677763 | 2383982 | 5.42 | 20 | 14.58 | 15.01 | 9.91 | 2 | no |
| 52885 | 2 | 2680268 | 2383579 | 7.08 | 15 | 7.92 | 14.8 | 9.99 | 3 | yes |
| 52886 | 2 | 2680268 | 2383579 | 4.99 | 15 | 10.01 | 14.8 | 9.99 | 2 | no |
| 52888 | 3 | 2679068 | 2380040 | 13.34 | 16 | 2.66 | 9.24 | -22.87 | 3 | no |
| 52889 | 3 | 2679068 | 2380040 | 10.78 | 16 | 5.22 | 9.24 | -22.87 | 3 | no |
| 52890 | 2 | 2679068 | 2380040 | 5.09 | 16 | 10.91 | 11.24 | 9.24 | 2 | no |
| 52891 | 3 | 2676431 | 2380996 | 13.86 | 11 | -2.86 | 6.98 | -23.61 | 3 | no |
| 52892 | 2 | 2676431 | 2380996 | 4.89 | 11 | 6.11 | 9.06 | 6.98 | 3 | yes |
| 52893 | 2 | 2679451 | 2381595 | 10.18 | 15 | 4.82 | 11.77 | 9.77 | 3 | yes |
| 52894 | 2 | 2679451 | 2381595 | 5.42 | 15 | 9.58 | 11.77 | 9.77 | 3 | yes |
| 56497 | 18 | 2680887 | 2368715 | 774.14 | 27 | -747.14 | -501.13 | -554.29 | 23 | yes |
| 56547 | 9 | 2711758 | 2409462 | 13 | 30 | 17 | 8.55 | 2.22 | 4 | yes |
| 56549 | 9 | 2712564 | 2409113 | 12 | 31 | 19 | 3.72 | -3.28 | 2 | yes |
| 56551 | 9 | 2713298 | 2409256 | 14 | 26 | 12 | 9.28 | 2.13 | 7 | yes |
| 56552 | 2 | 2718110 | 2408193 | 10 | 19 | 9 | 19.27 | -11.97 | 2 | no |
| 56553 | 2 | 2718243 | 2408935 | 11 | 19.95 | 8.95 | 19.03 | -3.13 | 2 | no |
| 58935 | 9 | 2666399 | 2375524 | 862 | 17 | -845 | -183.86 | -185.86 | 23 | yes |
| 59328 | 2 | 2664419 | 2381449 | 5.36 | 22.82 | 17.46 | 21.32 | 19.32 | 3 | yes |
| 59329 | 3 | 2664567 | 2382021 | 10.05 | 24 | 13.95 | 19.05 | 0.32 | 3 | no |
| 59330 | 2 | 2664567 | 2382021 | 5.41 | 24 | 18.59 | 21.05 | 19.05 | 3 | yes |
| 59331 | 2 | 2665106 | 2384501 | 4.99 | 28 | 23.01 | 20 | 18 | 0 | yes |
| 59336 | 2 | 2657303 | 2377286 | 6.17 | 24 | 17.83 | 28.98 | 14.15 | 2 | no |
| 59337 | 2 | 2657339 | 2377269 | 3.02 | 24 | 20.98 | 28.98 | 14.15 | 2 | no |
| 60082 | 9 | 2643050 | 2326627 | 30.5 | 21 | -9.5 | -37.39 | -39.39 | 6 | yes |
| 66881 | 9 | 2621756 | 2359451 | 100 | 14 | -86 | -1.56 | -3.56 | 15 | yes |
| 76860 | 3 | 2669776 | 2396182 | 20.13 | 33 | 12.87 | 29.16 | 26.24 | 7 | yes |
| 76888 | 3 | 2666044 | 2395310 | 25 | 38 | 13 | 33.58 | 27.9 | 7 | yes |
| 76891 | 3 | 2669776 | 2396182 | 10.08 | 33 | 22.92 | 29.16 | 26.24 | 5 | yes |
| 76892 | 2 | 2669776 | 2396182 | 5.75 | 33 | 27.25 | 33.08 | 29.16 | 3 | yes |
| 76894 | 2 | 2665829 | 2397604 | 5 | 40 | 35 | 39.87 | 31.37 | 2 | no |
| 76895 | 2 | 2666044 | 2395310 | 10.3 | 38 | 27.7 | 35.58 | 33.58 | 4 | yes |
| 76896 | 2 | 2666044 | 2395310 | 5.45 | 38 | 32.55 | 35.58 | 33.58 | 3 | yes |
| 77914 | 2 | 2700547 | 2381991 | 10.7 | 1.1 | -9.6 | 4.4 | -2.21 | 3 | yes |
| 77915 | 3 | 2705710 | 2383644 | 21 | 10.81 | -10.19 | 4.71 | -24.78 | 3 | no |
| 77918 | 3 | 2705591 | 2382981 | 19 | 10 | -9 | 1.12 | -23.36 | 3 | no |
| 77919 | 3 | 2705591 | 2382981 | 10.01 | 10 | -0.01 | 1.12 | -23.36 | 3 | no |
| 77920 | 2 | 2705591 | 2382981 | 6.65 | 10 | 3.35 | 7.69 | 1.12 | 2 | no |
| 77921 | 3 | 2705632 | 2383248 | 18.75 | 10 | -8.75 | 1.12 | -23.36 | 3 | no |
| 77922 | 2 | 2705632 | 2383248 | 12.02 | 10 | -2.02 | 7.69 | 1.12 | 3 | yes |
| 77924 | 3 | 2705470 | 2382325 | 19 | 3.58 | -15.42 | -6.09 | -20.77 | 3 | no |
| 77925 | 2 | 2705470 | 2382325 | 10.31 | 3.58 | -6.73 | 4.74 | -6.09 | 3 | yes |
| 77926 | 2 | 2705470 | 2382325 | 5.63 | 3.58 | -2.05 | 4.74 | -6.09 | 2 | no |
| 77927 | 3 | 2705284 | 2381449 | 20.97 | 1.6 | -19.37 | -8.65 | -19.72 | 3 | no |
| 77928 | 2 | 2705284 | 2381449 | 10.23 | 1.6 | -8.63 | 1.8 | -8.65 | 2 | no |
| 77929 | 2 | 2705284 | 2381449 | 5.49 | 1.6 | -3.89 | 1.8 | -8.65 | 2 | no |
| 77930 | 3 | 2702941 | 2383982 | 21.28 | 22.68 | 1.4 | 13.11 | -23.18 | 3 | no |
| 77933 | 3 | 2702676 | 2383292 | 21.28 | 17.66 | -3.62 | 3.09 | -19.4 | 3 | no |
| 77936 | 3 | 2702580 | 2382756 | 21.7 | 4 | -17.7 | -1.02 | -18.41 | 3 | no |
| 77937 | 3 | 2702580 | 2382756 | 9.7 | 4 | -5.7 | -1.02 | -18.41 | 3 | no |
| 77938 | 2 | 2702580 | 2382756 | 4.91 | 4 | -0.91 | 9.6 | -1.02 | 2 | no |
| 77939 | 3 | 2702476 | 2382096 | 21.14 | 1 | -20.14 | -6.67 | -16.77 | 4 | yes |
| 77940 | 2 | 2702476 | 2382096 | 10.27 | 1 | -9.27 | 5.59 | -6.67 | 3 | yes |
| 77941 | 2 | 2702476 | 2382096 | 4.92 | 1 | -3.92 | 5.59 | -6.67 | 2 | no |
| 77942 | 3 | 2702281 | 2381024 | 21.35 | 1 | -20.35 | -7.94 | -14.64 | 4 | yes |
| 77943 | 2 | 2702281 | 2381024 | 10.27 | 1 | -9.27 | 2.14 | -7.94 | 3 | yes |
| 77944 | 2 | 2702281 | 2381024 | 4.92 | 1 | -3.92 | 2.14 | -7.94 | 2 | no |
| 80866 | 2 | 2721083 | 2407535 | 10 | 18 | 8 | 18 | -7.35 | 2 | no |
| 86653 | 3 | 2697822 | 2382551 | 19.37 | 1.69 | -17.68 | -2.8 | -15.49 | 4 | yes |
| 86654 | 3 | 2695174 | 2379864 | 20 | 2.42 | -17.58 | -2.05 | -19.29 | 3 | no |
| 86655 | 3 | 2695174 | 2379864 | 9.23 | 2.42 | -6.81 | -2.05 | -19.29 | 3 | no |
| 86656 | 3 | 2691898 | 2378707 | 21 | 4 | -17 | -1.17 | -20.33 | 3 | no |
| 86657 | 2 | 2691898 | 2378707 | 4.77 | 4 | -0.77 | 2.68 | -1.17 | 2 | no |
| 86658 | 3 | 2690815 | 2379555 | 20 | 3 | -17 | -0.17 | -21.44 | 3 | no |
| 86659 | 2 | 2690815 | 2379555 | 5.4 | 3 | -2.4 | 5.31 | -0.17 | 3 | yes |
| 86660 | 3 | 2688409 | 2379916 | 17.26 | 5 | -12.26 | 0.01 | -21.91 | 3 | no |
| 86661 | 2 | 2688409 | 2379916 | 4.77 | 5 | 0.23 | 6.8 | 0.01 | 2 | no |
| 86662 | 3 | 2687427 | 2380215 | 20.5 | 8 | -12.5 | -0.05 | -22.74 | 3 | no |
| 86663 | 2 | 2687427 | 2380215 | 5.26 | 8 | 2.74 | 7.75 | -0.05 | 2 | no |
| 86664 | 3 | 2692667 | 2380142 | 10.25 | 6 | -4.25 | -1.91 | -21.28 | 3 | no |
| 86665 | 2 | 2692667 | 2380142 | 5.6 | 6 | 0.4 | 5.13 | -1.91 | 2 | no |
| 86666 | 3 | 2695267 | 2384265 | 19.74 | 1.33 | -18.41 | -3.31 | -18.98 | 3 | no |
| 86667 | 2 | 2695267 | 2384265 | 5.65 | 1.33 | -4.32 | 3.5 | -3.31 | 3 | yes |
| 86725 | 4 | 2685548 | 2378915 | 50 | 8 | -42 | -19.53 | -99.36 | 4 | no |
| 86727 | 2 | 2685548 | 2378915 | 8 | 8 | 0 | 5 | 0.57 | 3 | yes |
| 86736 | 3 | 2697822 | 2382551 | 13.45 | 1.69 | -11.76 | -2.8 | -15.49 | 3 | no |
| 86737 | 2 | 2697822 | 2382551 | 4.53 | 1.69 | -2.84 | 2.6 | -2.8 | 3 | yes |
| 86739 | 3 | 2682856 | 2379347 | 12.45 | 10 | -2.45 | 5.99 | -20.27 | 3 | no |
| 86740 | 2 | 2682856 | 2379347 | 5.66 | 10 | 4.34 | 7.99 | 5.99 | 3 | yes |
| 86741 | 3 | 2694215 | 2383036 | 16.49 | 1.65 | -14.84 | -7.77 | -21.12 | 3 | no |
| 86742 | 2 | 2694215 | 2383036 | 9.33 | 1.65 | -7.68 | 3.86 | -7.77 | 2 | no |
| 86743 | 2 | 2694215 | 2383036 | 5.5 | 1.65 | -3.85 | 3.86 | -7.77 | 2 | no |
| 86744 | 3 | 2689996 | 2382237 | 25.25 | 7 | -18.25 | -3.17 | -26.23 | 3 | no |
| 86745 | 2 | 2689996 | 2382237 | 9.73 | 7 | -2.73 | 6.73 | -3.17 | 2 | no |
| 86746 | 2 | 2689996 | 2382237 | 5.23 | 7 | 1.77 | 6.73 | -3.17 | 2 | no |
| 86747 | 3 | 2685658 | 2384116 | 17.4 | 11 | -6.4 | 0.01 | -31.08 | 3 | no |
| 86748 | 2 | 2685658 | 2384116 | 6.51 | 11 | 4.49 | 9 | 0.01 | 2 | no |
| 86749 | 2 | 2685658 | 2384116 | 6.45 | 11 | 4.55 | 9 | 0.01 | 2 | no |
| 89810 | 30 | 2651492 | 2369190 | 702.02 | 35 | -667.02 | -3440.4 | -3640.4 | 23 | yes |
| 89818 | 3 | 2658117 | 2369716 | 22.55 | 18 | -4.55 | 10.15 | 6.4 | 6 | yes |
| 89841 | 2 | 2657221 | 2369554 | 9.3 | 17 | 7.7 | 17.45 | 11.15 | 4 | yes |
| 89842 | 2 | 2657221 | 2369554 | 5.32 | 17 | 11.68 | 17.45 | 11.15 | 2 | no |
| 89845 | 3 | 2658117 | 2369716 | 30 | 18 | -12 | 10.15 | 6.4 | 6 | yes |
| 89850 | 2 | 2660328 | 2370169 | 4.75 | 13.94 | 9.19 | 18.32 | 9.69 | 3 | yes |
| 90138 | 18 | 2694513 | 2374002 | 1049.8 | 0.73 | -1049.03 | -845.36 | -847.36 | 22 | yes |
| 90357 | 3 | 2691134 | 2372094 | 21.6 | 0.02 | -21.58 | -4.62 | -28.95 | 3 | no |
| 90358 | 2 | 2691134 | 2372094 | 10.34 | 0.02 | -10.32 | 2 | -4.62 | 3 | yes |
| 90359 | 2 | 2691134 | 2372094 | 6.57 | 0.02 | -6.55 | 2 | -4.62 | 3 | yes |
| 90360 | 3 | 2691200 | 2375245 | 20 | 3 | -17 | -1.18 | -44.15 | 3 | no |
| 90361 | 2 | 2691200 | 2375245 | 5.84 | 3 | -2.84 | 2.24 | -1.18 | 3 | yes |
| 90423 | 3 | 2683852 | 2373197 | 21.17 | 13 | -8.17 | 0.11 | -39.77 | 3 | no |
| 90424 | 3 | 2684537 | 2374124 | 21.5 | 11 | -10.5 | 0.55 | -42.78 | 3 | no |
| 90428 | 3 | 2686816 | 2373735 | 14.07 | 7 | -7.07 | -0.03 | -55.51 | 3 | no |
| 90429 | 2 | 2686816 | 2373735 | 10.34 | 7 | -3.34 | 5 | -0.03 | 3 | yes |
| 90430 | 2 | 2686816 | 2373735 | 5.12 | 7 | 1.88 | 5 | -0.03 | 2 | no |
| 90431 | 3 | 2688593 | 2373773 | 21.55 | 3 | -18.55 | -2.66 | -63.71 | 3 | no |
| 90432 | 2 | 2688493 | 2373775 | 10.71 | 3 | -7.71 | 3.89 | -2.66 | 3 | yes |
| 90433 | 2 | 2688493 | 2373775 | 5.3 | 3 | -2.3 | 3.89 | -2.66 | 2 | no |
| 90434 | 3 | 2688607 | 2374448 | 22.04 | 4 | -18.04 | -2.02 | -59.4 | 3 | no |
| 90435 | 2 | 2688607 | 2374448 | 8.15 | 4 | -4.15 | 3.98 | -2.02 | 3 | yes |
| 90436 | 2 | 2688607 | 2374448 | 4.44 | 4 | -0.44 | 3.98 | -2.02 | 2 | no |
| 90437 | 3 | 2694679 | 2374109 | 15.7 | 0.73 | -14.97 | -2.32 | -29.1 | 3 | no |
| 90438 | 3 | 2694679 | 2374109 | 11.05 | 0.73 | -10.32 | -2.32 | -29.1 | 3 | no |
| 90439 | 3 | 2694679 | 2374109 | 6.32 | 0.73 | -5.59 | -2.32 | -29.1 | 3 | no |
| 90614 | 18 | 2716462 | 2376783 | 1246.4 | 8 | -1238.39 | -1014.5 | -1016.5 | 22 | yes |
| 92296 | 4 | 2680164 | 2396489 | 33 | 38 | 5 | 10.63 | -21.78 | 4 | no |
| 92297 | 3 | 2680164 | 2396489 | 15 | 38 | 23 | 25.57 | 10.63 | 3 | no |
| 94805 | 3 | 2569818 | 2312072 | 11.5 | 6.9 | -4.6 | 7.44 | 4.91 | 7 | yes |
| 94806 | 3 | 2569395 | 2313647 | 10 | 5 | -5 | -5.49 | -7.5 | 2 | yes |
| 94808 | 3 | 2568601 | 2314320 | 10 | 8.95 | -1.05 | 4.41 | 2.18 | 5 | yes |
| 94809 | 3 | 2568284 | 2313546 | 11.5 | 13.08 | 1.58 | 7.71 | 5.37 | 5 | yes |
| 94810 | 3 | 2568745 | 2313304 | 22 | 18.73 | -3.27 | 1.9 | -0.2 | 5 | yes |
| 94811 | 3 | 2568572 | 2313057 | 26.5 | 18.73 | -7.77 | 1.9 | -0.2 | 7 | yes |
| 95196 | 18 | 2660434 | 2394286 | 96 | 75.09 | -20.91 | 34.87 | 32.87 | 23 | yes |
| 95401 | 2 | 2672432 | 2385695 | 5.22 | 23 | 17.78 | 13.05 | 11.05 | 0 | yes |
| 95485 | 4 | 2667519 | 2388050 | 34 | 26 | -8 | 11.47 | -30.89 | 4 | no |
| 95486 | 3 | 2667529 | 2388049 | 15 | 26 | 11 | 15.43 | 11.47 | 4 | yes |
| 95487 | 3 | 2661349 | 2388017 | 41 | 42 | 1 | 30.82 | 27.22 | 7 | yes |
| 95488 | 9 | 2662140 | 2392315 | 28 | 36.74 | 8.74 | 17.58 | 11.8 | 10 | yes |
| 95489 | 8 | 2665364 | 2393361 | 33 | 38.06 | 5.06 | 10.66 | 5.47 | 9 | yes |
| 95491 | 2 | 2665364 | 2393361 | 8.81 | 38.06 | 29.25 | 34.96 | 29.64 | 3 | yes |
| 95492 | 2 | 2665364 | 2393361 | 6.51 | 38.06 | 31.55 | 34.96 | 29.64 | 2 | no |
| 95493 | 2 | 2669865 | 2390902 | 8.39 | 29 | 20.61 | 23.25 | 20.52 | 2 | no |
| 95494 | 2 | 2669865 | 2390902 | 5.49 | 29 | 23.51 | 23.25 | 20.52 | 0 | yes |
| 95495 | 2 | 2671771 | 2395185 | 8.35 | 27 | 18.65 | 27.4 | 23.75 | 5 | yes |
| 95496 | 2 | 2671771 | 2395185 | 5.48 | 27 | 21.52 | 27.4 | 23.75 | 4 | yes |
| 95497 | 3 | 2669041 | 2386192 | 17.4 | 26 | 8.6 | 13.78 | 4.02 | 3 | no |
| 95498 | 2 | 2669041 | 2386192 | 8.35 | 26 | 17.65 | 15.78 | 13.78 | 0 | yes |
| 95499 | 2 | 2669041 | 2386192 | 5.48 | 26 | 20.52 | 15.78 | 13.78 | 0 | yes |
| 95500 | 3 | 2665108 | 2386529 | 19.53 | 27.45 | 7.92 | 16.89 | -27.41 | 3 | no |
| 95501 | 2 | 2665108 | 2386529 | 7.69 | 27.45 | 19.76 | 18.89 | 16.89 | 0 | yes |
| 95502 | 2 | 2665108 | 2386529 | 4.89 | 27.45 | 22.56 | 18.89 | 16.89 | 0 | yes |
| 95503 | 3 | 2661349 | 2388017 | 18.69 | 42 | 23.31 | 30.82 | 27.22 | 5 | yes |
| 95504 | 2 | 2662140 | 2392315 | 8.87 | 36.74 | 27.87 | 40.74 | 29.58 | 3 | yes |
| 95505 | 2 | 2662140 | 2392315 | 5.45 | 36.74 | 31.29 | 40.74 | 29.58 | 2 | no |
| 96560 | 18 | 2635010 | 2366948 | 583 | 34.78 | -548.22 | -518.07 | -546.7 | 19 | yes |
| 98114 | 8 | 2674804 | 2400464 | 26 | 35 | 9 | 12.82 | 6.85 | 8 | no |
| 98115 | 3 | 2672955 | 2397251 | 42 | 73 | 31 | 50.49 | 40.73 | 4 | yes |
| 98119 | 3 | 2679319 | 2389598 | 19.2 | 21 | 1.8 | 11.61 | -19.48 | 3 | no |
| 98120 | 2 | 2679319 | 2389598 | 10.8 | 21 | 10.2 | 18.99 | 11.61 | 3 | yes |
| 98121 | 2 | 2679319 | 2389598 | 4.68 | 21 | 16.32 | 18.99 | 11.61 | 2 | no |
| 98122 | 2 | 2679053 | 2391234 | 9.63 | 19 | 9.37 | 18.17 | 13.35 | 3 | yes |
| 98123 | 2 | 2679053 | 2391234 | 4.24 | 19 | 14.76 | 18.17 | 13.35 | 2 | no |
| 98124 | 2 | 2675572 | 2397024 | 5.31 | 30 | 24.69 | 31.02 | 27.85 | 4 | yes |
| 98125 | 3 | 2674794 | 2400454 | 14.01 | 35 | 20.99 | 30.77 | 28.77 | 6 | yes |
| 98126 | 3 | 2674794 | 2400454 | 10.88 | 35 | 24.12 | 30.77 | 28.77 | 6 | yes |
| 98127 | 2 | 2674794 | 2400454 | 3.81 | 35 | 31.19 | 38.88 | 30.77 | 2 | no |
| 103582 | 9 | 2653126 | 2384365 | 460.33 | 61 | -399.33 | -47.98 | -77.14 | 20 | yes |
| 103583 | 18 | 2651685 | 2375860 | 727.67 | 21 | -706.67 | -617.23 | -619.23 | 23 | yes |
| 103734 | 3 | 2653116 | 2375225 | 20 | 18 | -2 | 8.08 | -1.97 | 4 | yes |
| 103825 | 2 | 2653051 | 2375271 | 10.44 | 18 | 7.56 | 18.1 | 8.08 | 3 | yes |
| 103826 | 2 | 2653116 | 2375225 | 4.86 | 18 | 13.14 | 18.1 | 8.08 | 2 | no |
| 103828 | 2 | 2655864 | 2377547 | 7.47 | 26 | 18.53 | 27.12 | 17.64 | 2 | no |
| 104537 | 4 | 2673518 | 2327830 | 39 | 4 | -35 | -14.28 | -31.29 | 5 | yes |
| 105199 | 3 | 2674346 | 2378582 | 20.46 | 14 | -6.46 | 9.44 | -25.41 | 3 | no |
| 105200 | 2 | 2674334 | 2378590 | 9.4 | 14 | 4.6 | 11.91 | 9.44 | 3 | yes |
| 105201 | 2 | 2674346 | 2378582 | 5.43 | 14 | 8.57 | 11.91 | 9.44 | 3 | yes |
| 109039 | 9 | 2698908 | 2386140 | 121.92 | 4 | -117.92 | -98.96 | -168.22 | 9 | no |
| 110199 | 2 | 2689066 | 2378191 | 9 | 2 | -7 | 3.81 | 0.04 | 3 | yes |
| 110201 | 2 | 2688604 | 2380983 | 8.3 | 7 | -1.3 | 8.53 | -0.04 | 3 | yes |
| 110206 | 3 | 2688274 | 2379098 | 13.4 | 4 | -9.4 | -0.02 | -20.8 | 3 | no |
| 110208 | 3 | 2688163 | 2378590 | 10.95 | 4 | -6.95 | -0.04 | -20.53 | 3 | no |
| 110720 | 18 | 2659138 | 2338232 | 333 | 92.79 | -240.21 | -116.22 | -118.22 | 21 | yes |
| 110727 | 9 | 2600232 | 2365476 | 193.3 | 69.46 | -123.84 | -19.58 | -32.9 | 16 | yes |
| 110728 | 3 | 2600200 | 2365480 | 80 | 69.46 | -10.54 | 60.55 | 47.19 | 8 | yes |
| 110732 | 9 | 2651454 | 2369213 | 400.2 | 35 | -365.2 | -119.14 | -139.12 | 14 | yes |
| 110733 | 9 | 2661081 | 2370219 | 373.2 | 15 | -358.2 | -189.05 | -272.22 | 13 | yes |
| 110749 | 3 | 2695267 | 2384265 | 10.47 | 1.33 | -9.14 | -3.31 | -18.98 | 3 | no |
| 110976 | 9 | 2710822 | 2410345 | 116 | 28.23 | -87.77 | -22.07 | -24.29 | 23 | yes |
| 110978 | 9 | 2708309 | 2411580 | 74.5 | 39 | -35.5 | -15.12 | -17.12 | 19 | yes |
| 112982 | 2 | 2688703 | 2369493 | 9 | 11 | 2 | 6.75 | -2.19 | 2 | no |
| 112983 | 2 | 2688703 | 2369493 | 5.1 | 11 | 5.9 | 6.75 | -2.19 | 2 | no |
| 112984 | 3 | 2688480 | 2368397 | 12.95 | 19 | 6.05 | 12.04 | 1.79 | 3 | no |
| 112987 | 3 | 2694523 | 2370272 | 13.3 | 1 | -12.3 | -2.2 | -34.72 | 3 | no |
| 112988 | 3 | 2694523 | 2370272 | 10.25 | 1 | -9.25 | -2.2 | -34.72 | 3 | no |
| 112989 | 2 | 2694523 | 2370272 | 5.05 | 1 | -4.05 | 1.24 | -2.2 | 3 | yes |
| 112991 | 2 | 2677658 | 2377230 | 11.17 | 5 | -6.17 | 7.59 | -0.93 | 3 | yes |
| 112992 | 2 | 2677658 | 2377230 | 4.78 | 5 | 0.22 | 7.59 | -0.93 | 2 | no |
| 112993 | 3 | 2679253 | 2372193 | 13.46 | 5 | -8.46 | -1.29 | -30.79 | 3 | no |
| 112994 | 2 | 2679253 | 2372193 | 4.77 | 5 | 0.23 | 8.09 | -1.29 | 2 | no |
| 112995 | 3 | 2672975 | 2373426 | 20.45 | 9 | -11.45 | 6 | -48.26 | 3 | no |
| 112996 | 3 | 2650006 | 2374078 | 19.32 | 25 | 5.68 | 9.94 | 0.82 | 3 | no |
| 112997 | 3 | 2652942 | 2367743 | 9.37 | 37 | 27.63 | 21.54 | 17.7 | 0 | yes |
| 112998 | 3 | 2648550 | 2363533 | 22.05 | 61 | 38.95 | 40.67 | 26.92 | 3 | no |
| 113000 | 3 | 2665385 | 2369383 | 21.69 | 11.98 | -9.71 | 9.78 | -37.35 | 3 | no |
| 113668 | 30 | 2571699 | 2310853 | 50 | 11.32 | -38.68 | -742.42 | -942.42 | 23 | yes |
| 113669 | 3 | 2571699 | 2310854 | 19 | 11.32 | -7.68 | 7.61 | 5.61 | 10 | yes |
| 114155 | 30 | 2571699 | 2310855 | 32 | 11.32 | -20.68 | -742.42 | -942.42 | 17 | yes |
| 114159 | 3 | 2568204 | 2315100 | 50 | 9.56 | -40.44 | 1.66 | -0.34 | 23 | yes |
| 114160 | 3 | 2568203 | 2315099 | 23 | 9.56 | -13.44 | 1.66 | -0.34 | 10 | yes |
| 115218 | 2 | 2684708 | 2384637 | 14.3 | 16 | 1.7 | 10.49 | 2.74 | 3 | yes |
| 115220 | 3 | 2687560 | 2378473 | 15.7 | 7 | -8.7 | -0.07 | -19.73 | 3 | no |
| 115221 | 3 | 2684421 | 2379069 | 14.1 | 9 | -5.1 | 3.3 | -20.31 | 3 | no |
| 115223 | 3 | 2686471 | 2376614 | 14.6 | 8 | -6.6 | 0.16 | -22.78 | 3 | no |
| 115225 | 3 | 2686471 | 2376614 | 20.6 | 8 | -12.6 | 0.16 | -22.78 | 3 | no |
| 115226 | 3 | 2687049 | 2376502 | 16.7 | 7 | -9.7 | 0.12 | -27.43 | 3 | no |
| 115245 | 2 | 2687049 | 2376502 | 8.9 | 7 | -1.9 | 5.01 | 0.12 | 3 | yes |
| 115630 | 3 | 2681968 | 2379501 | 13.33 | 11 | -2.33 | 7.31 | -21.2 | 3 | no |
| 115631 | 3 | 2681856 | 2378923 | 12.91 | 12.98 | 0.07 | 7.95 | -20.58 | 3 | no |
| 115937 | 3 | 2664427 | 2381049 | 20.5 | 26 | 5.5 | 19.64 | -6.78 | 3 | no |
| 115939 | 3 | 2661734 | 2384545 | 20.5 | 33 | 12.5 | 21.05 | 14.9 | 4 | yes |
| 115940 | 3 | 2667702 | 2380907 | 20.5 | 20.9 | 0.4 | 18.3 | 2.21 | 4 | yes |
| 115941 | 3 | 2667877 | 2381778 | 20.5 | 23 | 2.5 | 18.78 | 1.67 | 3 | no |
| 119103 | 3 | 2654568 | 2320729 | 0 | 3 | n/a | -0.1 | -29.35 | 3 | no |
| 119104 | 2 | 2654573 | 2320724 | 0 | 3 | n/a | 7.18 | -0.1 | 2 | no |
| 119105 | 2 | 2654573 | 2320719 | 0 | 3 | n/a | 7.18 | -0.1 | 2 | no |
| 119106 | 2 | 2654559 | 2320689 | 0 | 3 | n/a | 7.18 | -0.1 | 2 | no |
| 119107 | 2 | 2654568 | 2320675 | 0 | 3 | n/a | 7.18 | -0.1 | 2 | no |
| 119108 | 2 | 2656313 | 2319385 | 0 | 5 | n/a | 2.75 | -2.45 | 2 | no |
| 119109 | 3 | 2656307 | 2319382 | 0 | 5 | n/a | -2.45 | -47.77 | 3 | no |
| 119110 | 3 | 2656423 | 2323075 | 0 | 12.13 | n/a | 6.2 | -18.69 | 3 | no |
| 119113 | 3 | 2665743 | 2321355 | 0 | 8 | n/a | 0.03 | -30.19 | 3 | no |
| 119114 | 2 | 2665745 | 2321356 | 0 | 8 | n/a | 3.76 | 0.03 | 2 | no |
| 119115 | 3 | 2667011 | 2326948 | 0 | 16 | n/a | 9.09 | -13.19 | 3 | no |
| 119116 | 2 | 2667424 | 2323704 | 0 | 8 | n/a | 5.58 | -0.24 | 2 | no |
| 119117 | 3 | 2667422 | 2323704 | 0 | 8 | n/a | -0.24 | -27.87 | 3 | no |
| 119118 | 2 | 2662140 | 2321449 | 0 | 7 | n/a | 8.56 | 0.01 | 2 | no |
| 119119 | 2 | 2662139 | 2321453 | 0 | 7 | n/a | 8.56 | 0.01 | 2 | no |
| 119120 | 2 | 2661150 | 2321142 | 0 | 5 | n/a | 6.64 | 0.01 | 2 | no |
| 119121 | 2 | 2661149 | 2321138 | 0 | 5 | n/a | 6.64 | 0.01 | 2 | no |
| 119122 | 2 | 2672405 | 2328230 | 0 | 2 | n/a | 5.68 | -0.19 | 2 | no |
| 119123 | 3 | 2672405 | 2328228 | 0 | 2 | n/a | -0.19 | -11.56 | 3 | no |
| 119132 | 2 | 2665655 | 2322353 | 0 | 7 | n/a | 4.76 | 0.02 | 2 | no |
| 119136 | 3 | 2645306 | 2320962 | 0 | 13 | n/a | 9.5 | 1.25 | 3 | no |
| 119137 | 3 | 2649317 | 2332391 | 0 | 58.81 | n/a | 46.22 | 44.22 | 3 | no |
| 119138 | 3 | 2649029 | 2332819 | 0 | 50 | n/a | 47.1 | 45.1 | 3 | no |
| 119892 | 3 | 2689529 | 2379798 | 12.2 | 7.3 | -4.9 | 0.02 | -21.79 | 3 | no |
| 120793 | 3 | 2692906 | 2373778 | 25 | 4 | -21 | -1.5 | -48.88 | 3 | no |
| 120794 | 3 | 2694222 | 2373580 | 20.5 | 3.99 | -16.51 | -1.57 | -42.03 | 3 | no |
| 120795 | 3 | 2694065 | 2373223 | 20.5 | 2.53 | -17.97 | -2.02 | -48.77 | 3 | no |
| 120796 | 3 | 2693678 | 2379595 | 14.5 | 2.45 | -12.05 | -1.69 | -19.13 | 3 | no |
| 120799 | 3 | 2693873 | 2378891 | 20.5 | 0.98 | -19.52 | -0.49 | -16.9 | 4 | yes |
| 120800 | 3 | 2693714 | 2378464 | 20.5 | 3 | -17.5 | -1.57 | -15.9 | 4 | yes |
| 120801 | 3 | 2692396 | 2378572 | 13 | 3.99 | -9.01 | -1.83 | -19.97 | 3 | no |
| 120802 | 3 | 2692561 | 2379289 | 12 | 0 | -12 | -1.02 | -19.91 | 3 | no |
| 120997 | 3 | 2693233 | 2384648 | 18.1 | 2.23 | -15.87 | -3.95 | -22.48 | 3 | no |
| 121802 | 3 | 2709866 | 2386171 | 23.5 | 20.96 | -2.54 | 11.46 | -21.26 | 3 | no |
| 121803 | 3 | 2712227 | 2383520 | 25 | 3 | -22 | -5.46 | -23.46 | 3 | no |
| 121804 | 3 | 2706711 | 2384376 | 20.5 | 18 | -2.5 | 9.37 | -26.33 | 3 | no |
| 121805 | 3 | 2713407 | 2388988 | 25 | 14 | -11 | 14.52 | -9.51 | 4 | yes |
| 121806 | 3 | 2718562 | 2384778 | 16 | 10 | -6 | -7.28 | -24.99 | 2 | yes |
| 121807 | 3 | 2715586 | 2387942 | 19.5 | 15 | -4.5 | 8.06 | -14.61 | 3 | no |
| 121808 | 3 | 2725593 | 2389414 | 23.5 | 14 | -9.5 | 9.83 | -4.8 | 4 | yes |
| 121809 | 3 | 2719211 | 2388067 | 20.5 | 29 | 8.5 | 13.79 | -9.24 | 3 | no |
| 121810 | 3 | 2723768 | 2388272 | 20.5 | 25 | 4.5 | 17.31 | -8.01 | 3 | no |
| 122449 | 2 | 2710505 | 2385647 | 3 | 5 | 2 | 6.66 | -0.69 | 2 | no |
| 122450 | 3 | 2710505 | 2385647 | 14.5 | 5 | -9.5 | -0.69 | -20.3 | 3 | no |
| 122451 | 2 | 2713007 | 2384714 | 6.1 | 3 | -3.1 | 3.99 | -4.77 | 2 | no |
| 122452 | 2 | 2712403 | 2380704 | 7 | 4 | -3 | 1.02 | -1.9 | 3 | yes |
| 122453 | 2 | 2716187 | 2382226 | 2 | 4 | 2 | 1 | -2.86 | 0 | yes |
| 122454 | 2 | 2722295 | 2387202 | 5 | 5 | 0 | 16.81 | -3.02 | 2 | no |
| 122455 | 2 | 2722295 | 2387202 | 9 | 5 | -4 | 16.81 | -3.02 | 3 | yes |
| 122456 | 3 | 2722295 | 2387202 | 20 | 5 | -15 | -3.02 | -10.5 | 4 | yes |
| 122457 | 2 | 2718869 | 2386073 | 5 | 6 | 1 | 10.82 | -7.6 | 2 | no |
| 122458 | 2 | 2718869 | 2386073 | 10 | 6 | -4 | 10.82 | -7.6 | 2 | no |
| 122459 | 3 | 2718869 | 2386073 | 16 | 6 | -10 | -7.6 | -18.77 | 3 | no |
| 122461 | 3 | 2711719 | 2388674 | 16.79 | 6 | -10.79 | 9.87 | -13.53 | 3 | no |
| 122462 | 3 | 2711719 | 2388674 | 9.74 | 6 | -3.74 | 9.87 | -13.53 | 3 | no |
| 122463 | 3 | 2711719 | 2388674 | 20.99 | 6 | -14.99 | 9.87 | -13.53 | 4 | yes |
| 122464 | 3 | 2711724 | 2388924 | 20 | 6 | -14 | 9.87 | -13.53 | 4 | yes |
| 122465 | 3 | 2711070 | 2388737 | 20 | 5.49 | -14.51 | 6.86 | -16.4 | 3 | no |
| 122466 | 3 | 2710709 | 2388244 | 20.43 | 10 | -10.43 | 2.69 | -18.51 | 3 | no |
| 122467 | 3 | 2713574 | 2388885 | 10.8 | 6 | -4.8 | 1.59 | -10.61 | 3 | no |
| 122468 | 2 | 2713572 | 2388785 | 5.48 | 6 | 0.52 | 15.89 | 1.59 | 3 | yes |
| 122469 | 2 | 2713570 | 2388685 | 5.13 | 6 | 0.87 | 15.89 | 1.59 | 3 | yes |
| 122470 | 3 | 2723300 | 2387431 | 16.03 | 5 | -11.03 | -1.9 | -11.58 | 3 | no |
| 122471 | 2 | 2723300 | 2387431 | 9.49 | 5 | -4.49 | 17.05 | -1.9 | 3 | yes |
| 122472 | 2 | 2723300 | 2387431 | 5.51 | 5 | -0.51 | 17.05 | -1.9 | 2 | no |
| 122473 | 3 | 2723820 | 2388371 | 20.83 | 25 | 4.17 | 17.31 | -8.01 | 3 | no |
| 123721 | 3 | 2666401 | 2388173 | 13 | 28 | 15 | 20 | 5.89 | 3 | no |
| 126436 | 3 | 2680435 | 2380824 | 35 | 14 | -21 | 9 | -23.44 | 3 | no |
| 126437 | 3 | 2683019 | 2380474 | 16.5 | 11 | -5.5 | 6.57 | -21.25 | 3 | no |
| 127134 | 3 | 2691340 | 2376422 | 18.6 | 3 | -15.6 | -0.98 | -26.57 | 3 | no |
| 127135 | 3 | 2691388 | 2376792 | 17.4 | 5 | -12.4 | -1 | -23.31 | 3 | no |
| 127137 | 3 | 2691496 | 2377180 | 17 | 3 | -14 | -0.99 | -21.34 | 3 | no |
| 127144 | 3 | 2691546 | 2377639 | 14 | 3 | -11 | -0.92 | -20.57 | 3 | no |
| 127302 | 3 | 2672127 | 2378746 | 14.5 | 16 | 1.5 | 13.25 | -22.33 | 3 | no |
| 127303 | 3 | 2672568 | 2378922 | 11 | 16 | 5 | 12.42 | -23.8 | 3 | no |
| 127611 | 3 | 2711712 | 2388364 | 15 | 6.49 | -8.51 | -1.48 | -14.91 | 3 | no |
| 127612 | 2 | 2711712 | 2388364 | 5 | 6.49 | 1.49 | 9.79 | -1.48 | 2 | no |
| 127613 | 2 | 2692110 | 2383070 | 5 | 1 | -4 | 4.99 | -6.32 | 2 | no |
| 127614 | 2 | 2693807 | 2383355 | 5 | 0 | -5 | 3.86 | -7.5 | 2 | no |
| 127615 | 3 | 2693807 | 2383355 | 15 | 0 | -15 | -7.5 | -21.61 | 3 | no |
| 127616 | 3 | 2693467 | 2378149 | 15 | 1.99 | -13.01 | -0.26 | -16.74 | 3 | no |
| 127617 | 3 | 2693467 | 2378149 | 5 | 1.99 | -3.01 | -0.26 | -16.74 | 3 | no |
| 127618 | 3 | 2692431 | 2375009 | 15 | 3 | -12 | -1.43 | -35.6 | 3 | no |
| 127619 | 3 | 2692431 | 2375009 | 5 | 3 | -2 | -1.43 | -35.6 | 3 | no |
| 127620 | 3 | 2692110 | 2383070 | 14 | 1 | -13 | -6.32 | -24.35 | 3 | no |
| 127621 | 3 | 2694724 | 2372669 | 15 | 1.65 | -13.35 | -2.67 | -49.33 | 3 | no |
| 127622 | 3 | 2694724 | 2372669 | 5 | 1.65 | -3.35 | -2.67 | -49.33 | 3 | no |
| 127623 | 3 | 2687929 | 2370740 | 15 | 0 | -15 | -4.04 | -25.34 | 3 | no |
| 127624 | 2 | 2687929 | 2370740 | 5 | 0 | -5 | 2.35 | -4.04 | 3 | yes |
| 127625 | 3 | 2690899 | 2369227 | 15 | 6.94 | -8.06 | -1.46 | -11.45 | 3 | no |
| 127626 | 2 | 2690899 | 2369227 | 5 | 6.94 | 1.94 | 1.22 | -1.46 | 0 | yes |
| 127896 | 3 | 2681312 | 2380655 | 16 | 14 | -2 | 8 | -23.1 | 3 | no |
| 127897 | 3 | 2680684 | 2380769 | 16 | 13 | -3 | 8.85 | -23.38 | 3 | no |
| 127898 | 3 | 2679957 | 2380904 | 17.5 | 15 | -2.5 | 9 | -23.5 | 3 | no |
| 127899 | 3 | 2680219 | 2379107 | 19 | 14 | -5 | 8.9 | -21.64 | 3 | no |
| 127996 | 3 | 2691992 | 2379361 | 13 | 3 | -10 | -1.19 | -20.96 | 3 | no |
| 127997 | 3 | 2691564 | 2380430 | 11.5 | 4 | -7.5 | -1.5 | -21.97 | 3 | no |
| 128028 | 3 | 2648483 | 2373659 | 13 | 25 | 12 | 11.38 | 2.9 | 2 | yes |
| 128029 | 3 | 2646285 | 2372498 | 20.5 | 27 | 6.5 | 19.16 | 8.09 | 4 | yes |
| 128030 | 3 | 2644525 | 2372823 | 15 | 29 | 14 | 20.08 | 12.95 | 3 | no |
| 128031 | 3 | 2641280 | 2373330 | 19 | 34 | 15 | 28.93 | 19.56 | 4 | yes |
| 128033 | 3 | 2641613 | 2374433 | 14.5 | 38 | 23.5 | 30.94 | 23.18 | 3 | no |
| 129031 | 3 | 2664660 | 2382406 | 12.5 | 25 | 12.5 | 19 | -1.58 | 3 | no |
| 129032 | 3 | 2664851 | 2383490 | 12 | 28 | 16 | 18 | -1.21 | 3 | no |
| 130367 | 3 | 2673705 | 2387669 | 11 | 20 | 9 | 14.93 | 5.93 | 3 | no |
| 130368 | 3 | 2674095 | 2384349 | 10 | 16 | 6 | 9.97 | -9.13 | 3 | no |
| 130369 | 2 | 2668751 | 2391936 | 11.6 | 31 | 19.4 | 27.74 | 22.9 | 3 | yes |
| 130370 | 2 | 2671345 | 2393012 | 10 | 26 | 16 | 25.2 | 19.94 | 3 | yes |
| 130371 | 2 | 2669928 | 2394623 | 10 | 31 | 21 | 29.66 | 23 | 3 | yes |
| 130372 | 2 | 2671415 | 2388238 | 11.75 | 26 | 14.25 | 19.04 | 17.04 | 3 | yes |
| 130373 | 2 | 2662613 | 2395537 | 7 | 45 | 38 | 44.15 | 31.61 | 2 | no |
| 130374 | 2 | 2667404 | 2389723 | 11 | 29.99 | 18.99 | 20.92 | 18.92 | 2 | no |
| 131246 | 3 | 2675496 | 2383379 | 10.5 | 13 | 2.5 | 9.88 | -18.28 | 3 | no |
| 131249 | 3 | 2679167 | 2393348 | 13 | 23 | 10 | 18.95 | -4.87 | 3 | no |
| 131250 | 2 | 2676821 | 2395499 | 10 | 28 | 18 | 26.36 | 21.36 | 4 | yes |
| 131253 | 3 | 2666038 | 2389942 | 10 | 28 | 18 | 22.73 | 20.73 | 5 | yes |
| 131255 | 2 | 2674784 | 2403816 | 6.5 | 44 | 37.5 | 37.83 | 35.83 | 2 | no |
| 131257 | 2 | 2668746 | 2395008 | 11 | 32 | 21 | 30.7 | 26.82 | 4 | yes |
| 131259 | 3 | 2684843 | 2382953 | 14 | 13 | -1 | 2.08 | -29.14 | 3 | no |
| 135289 | 2 | 2682217 | 2383238 | 10 | 15 | 5 | 12.15 | 9.04 | 3 | yes |
| 135416 | 3 | 2684086 | 2377895 | 17.5 | 11 | -6.5 | 2.5 | -20.9 | 3 | no |
| 136519 | 3 | 2678617 | 2383364 | 18 | 18 | 0 | 9.9 | -25.01 | 3 | no |
| 136529 | 3 | 2678970 | 2380154 | 20.5 | 16 | -4.5 | 9 | -23.29 | 3 | no |
| 136531 | 3 | 2678981 | 2380174 | 21 | 16 | -5 | 9 | -23.29 | 3 | no |
| 136533 | 3 | 2678125 | 2381843 | 19 | 17 | -2 | 9.1 | -24.3 | 3 | no |
| 136537 | 3 | 2683171 | 2378175 | 10.97 | 13 | 2.03 | 5.01 | -19.93 | 3 | no |
| 136539 | 3 | 2681699 | 2383359 | 24 | 13 | -11 | 9.66 | -26.34 | 3 | no |
| 136540 | 3 | 2685888 | 2380789 | 14.5 | 8 | -6.5 | 0.52 | -23.4 | 3 | no |
| 136718 | 3 | 2693543 | 2376046 | 20.5 | 2.93 | -17.57 | -1.88 | -17.49 | 4 | yes |
| 136719 | 2 | 2693553 | 2376046 | 5.9 | 2.93 | -2.97 | 1 | -1.88 | 3 | yes |
| 136999 | 3 | 2665923 | 2381163 | 20 | 25.5 | 5.5 | 20.09 | 7.94 | 4 | yes |
| 137476 | 2 | 2678696 | 2383792 | 7.5 | 18 | 10.5 | 15.19 | 10.02 | 2 | no |
| 137478 | 3 | 2681548 | 2383312 | 14 | 15 | 1 | 9.79 | -26.04 | 3 | no |
| 137479 | 2 | 2681553 | 2383312 | 7 | 15 | 8 | 12.99 | 9.79 | 3 | yes |
| 137481 | 3 | 2678679 | 2380090 | 20 | 15 | -5 | 8.67 | -22.95 | 3 | no |
| 137539 | 3 | 2678696 | 2383782 | 18 | 18 | 0 | 10.02 | -25.05 | 3 | no |
| 139365 | 2 | 2654096 | 2374994 | 18 | 19 | 1 | 22.15 | 9.95 | 3 | yes |
| 139366 | 3 | 2651180 | 2374394 | 20 | 18 | -2 | 9.62 | 0.81 | 4 | yes |
| 139367 | 3 | 2653588 | 2375115 | 14 | 17 | 3 | 8.9 | -2.59 | 3 | no |
| 139370 | 3 | 2650900 | 2375697 | 20 | 25 | 5 | 15.63 | 8.15 | 4 | yes |
| 139374 | 3 | 2655426 | 2376387 | 30 | 22 | -8 | 11.35 | -31.02 | 3 | no |
| 139379 | 3 | 2665260 | 2386009 | 19 | 27 | 8 | 16.63 | -12.6 | 3 | no |
| 139380 | 3 | 2660191 | 2386345 | 17 | 35 | 18 | 23.3 | 19.29 | 4 | yes |
| 139381 | 3 | 2656390 | 2387026 | 15 | 39 | 24 | 30.19 | 25.47 | 4 | yes |
| 140201 | 3 | 2667171 | 2387227 | 13.7 | 24 | 10.3 | 19.78 | 0.75 | 3 | no |
| 140202 | 3 | 2677244 | 2373096 | 15 | 4 | -11 | -3.91 | -22.9 | 3 | no |
| 140204 | 3 | 2676701 | 2372493 | 13 | 3 | -10 | -4.19 | -31.01 | 3 | no |
| 140213 | 3 | 2666872 | 2387253 | 16 | 24 | 8 | 19.78 | 0.75 | 3 | no |
| 140277 | 3 | 2678206 | 2371734 | 17 | 8 | -9 | -5.65 | -32.03 | 3 | no |
| 140691 | 5 | 2709255 | 2389826 | 80 | 17 | -63 | -30.43 | -56.65 | 7 | yes |
| 143099 | 3 | 2654794 | 2389638 | 22 | 46 | 24 | 42.25 | 36.28 | 6 | yes |
| 143100 | 2 | 2654794 | 2389638 | 6 | 46 | 40 | 46 | 42.25 | 3 | yes |
| 143101 | 2 | 2655890 | 2389618 | 8.3 | 42 | 33.7 | 47.21 | 35.72 | 3 | yes |
| 143102 | 3 | 2655895 | 2389615 | 21 | 42 | 21 | 35.72 | 31.18 | 6 | yes |
| 143103 | 3 | 2653131 | 2389533 | 8 | 56 | 48 | 45.68 | 41.51 | 0 | yes |
| 143104 | 3 | 2652817 | 2389800 | 8.3 | 54 | 45.7 | 48.77 | 44.86 | 3 | no |
| 143747 | 3 | 2686847 | 2383581 | 15 | 3.49 | -11.51 | -0.57 | -30.24 | 3 | no |
| 143748 | 2 | 2685426 | 2384541 | 10 | 5 | -5 | 9.01 | 0.11 | 3 | yes |
| 143749 | 3 | 2684009 | 2386122 | 9 | 7.69 | -1.31 | 5.56 | -31.49 | 3 | no |
| 144943 | 3 | 2668684 | 2396519 | 11 | 33 | 22 | 28.47 | 25.37 | 5 | yes |
| 145207 | 3 | 2681633 | 2375102 | 27 | 14 | -13 | 0.64 | -22.26 | 3 | no |
| 145214 | 2 | 2681633 | 2375102 | 27 | 14 | -13 | 6.98 | 0.64 | 3 | yes |
| 145215 | 3 | 2680897 | 2377188 | 19.5 | 13.93 | -5.57 | 1.8 | -20.49 | 3 | no |
| 145216 | 2 | 2680897 | 2377188 | 19.5 | 13.93 | -5.57 | 9.3 | 1.8 | 3 | yes |
| 145217 | 3 | 2683658 | 2376258 | 20 | 13 | -7 | 1.71 | -24.76 | 3 | no |
| 145218 | 2 | 2683658 | 2376258 | 20 | 13 | -7 | 8.13 | 1.71 | 3 | yes |
| 145219 | 3 | 2682544 | 2377315 | 22 | 11 | -11 | 3.11 | -21.03 | 3 | no |
| 145220 | 2 | 2682544 | 2377315 | 22 | 11 | -11 | 9 | 3.11 | 3 | yes |
| WRK059121 | 18 | 2642696 | 2334157 | 23 | 71.73 | 48.73 | 19.65 | 12.54 | 3 | yes |
| Sole1 | 18 | 2853712 | 2368609 | n/a | 0 | n/a | -795.3 | -1376.6 | 18 | no |
| Scallop1 | 18 | 2814360 | 2359237 | n/a | 0 | n/a | -1697.4 | -2131.5 | 18 | no |
| Grunter1 | 18 | 2807716 | 2353253 | n/a | 0 | n/a | -1987.6 | -2463.6 | 18 | no |
| Patricia1 | 18 | 2802636 | 2380240 | n/a | 0 | n/a | -701.47 | -727.65 | 18 | no |
| Flounder1 | 18 | 2799462 | 2348982 | n/a | 0 | n/a | -2000.3 | -2640.5 | 18 | no |
| Tuna1 | 18 | 2799392 | 2364629 | n/a | 0 | n/a | -1324.7 | -1662.4 | 18 | no |
| SpermWhale1 | 18 | 2795214 | 2377488 | n/a | 0 | n/a | -822.87 | -847 | 18 | no |
| Kahawai1 | 18 | 2795163 | 2364658 | n/a | 0 | n/a | -1403.5 | -1766.4 | 18 | no |
| Mackerel1 | 18 | 2792860 | 2330670 | n/a | 0 | n/a | -2551 | -3272 | 18 | no |
| Halibut2 | 18 | 2790885 | 2339980 | n/a | 0 | n/a | -2342 | -3122.3 | 18 | no |
| Anemone1A | 18 | 2789397 | 2299109 | n/a | 0 | n/a | -2597.8 | -2933.1 | 18 | no |
| Marlin2 | 18 | 2788238 | 2358496 | n/a | 0 | n/a | -2070.2 | -2589.1 | 18 | no |
| Cobia1 | 18 | 2786606 | 2333302 | n/a | 0 | n/a | -2443.4 | -3141.7 | 18 | no |
| Turrum7 | 18 | 2785594 | 2354766 | n/a | 0 | n/a | -1676.6 | -2286.1 | 18 | no |
| Sunfish2 | 18 | 2784391 | 2368653 | n/a | 0 | n/a | -1683.8 | -1944.4 | 18 | no |
| Turrum5 | 18 | 2780180 | 2356691 | n/a | 0 | n/a | -1411.7 | -1846.7 | 18 | no |
| Kingfish9 | 18 | 2774207 | 2314644 | n/a | 0 | n/a | -2314 | -2987.9 | 18 | no |
| Kingfish8 | 18 | 2766768 | 2319041 | n/a | 0 | n/a | -2289.3 | -3010.8 | 18 | no |
| Snapper1 | 18 | 2763929 | 2362938 | n/a | 0 | n/a | -1172.8 | -1709 | 18 | no |
| Snapper | 18 | 2763929 | 2362938 | n/a | 0 | n/a | -1172.8 | -1709 | 18 | no |
| WestMoonfish1 | 18 | 2760997 | 2368277 | n/a | 0 | n/a | -1523.4 | -1946.5 | 18 | no |
| ZaneGrey1 | 18 | 2760502 | 2321223 | n/a | 0 | n/a | -2297.7 | -2989.3 | 18 | no |
| Bream1 | 18 | 2743954 | 2327976 | n/a | 0 | n/a | -1878.8 | -2595.4 | 18 | no |
| Omeo | 18 | 2736696 | 2317734 | n/a | 0 | n/a | -2280.9 | -2512.5 | 18 | no |
| Seahorse1 | 18 | 2734125 | 2364019 | n/a | 0 | n/a | -1406 | -1453.3 | 18 | no |
| Barracouta3 | 18 | 2728797 | 2350406 | n/a | 0 | n/a | -1001.5 | -1463.7 | 18 | no |
| Bullseye | 18 | 2723620 | 2320362 | n/a | 0 | n/a | -2100.8 | -2267 | 18 | no |
| WestWhiptail1 | 18 | 2719026 | 2350092 | n/a | 0 | n/a | -1172.4 | -1229.2 | 18 | no |
| Blenny1 | 18 | 2710713 | 2333805 | n/a | 0 | n/a | -1243.7 | -1592.6 | 18 | no |
| Dolphin1 | 18 | 2707487 | 2332052 | n/a | 0 | n/a | -1242.1 | -1650.7 | 18 | no |
| Pearch2 | 18 | 2703421 | 2323090 | n/a | 0 | n/a | -1161.1 | -1273.7 | 18 | no |
| Kyarra1A | 18 | 2690428 | 2311385 | n/a | 0 | n/a | -964.2 | -1010.9 | 18 | no |
| TommyRuff1 | 18 | 2686766 | 2319025 | n/a | 0 | n/a | -952.69 | -1114.6 | 18 | no |
| Drummer1 | 18 | 2783672 | 2331495 | n/a | 0 | n/a | -2460.2 | -3137.7 | 18 | no |
| Fortescue3 | 18 | 2785434 | 2340855 | n/a | 0 | n/a | -2407.3 | -3210 | 18 | no |
| Barracouta5 | 18 | 2732789 | 2352535 | n/a | 0 | n/a | -1052.7 | -1485.1 | 18 | no |
| Barracouta4 | 18 | 2737315 | 2355029 | n/a | 0 | n/a | -1097.6 | -1508.5 | 18 | no |
| Whiptail1A | 18 | 2720415 | 2350188 | n/a | 0 | n/a | -1149.7 | -1206.1 | 18 | no |
| Tarwhine1 | 18 | 2720950 | 2341321 | n/a | 0 | n/a | -1418.5 | -1826.4 | 18 | no |
| Seahorse2 | 18 | 2728764 | 2363182 | n/a | 0 | n/a | -1409.6 | -1450.5 | 18 | no |
| West Seahorse | 18 | 2729819 | 2363297 | n/a | 0 | n/a | -1414.9 | -1457.3 | 18 | no |
| Luderick1 | 18 | 2737252 | 2337069 | n/a | 0 | n/a | -1826.8 | -2499.2 | 18 | no |
| Wirrah1 | 18 | 2746783 | 2364504 | n/a | 0 | n/a | -1505.6 | -2058.9 | 18 | no |
| Wirrah2 | 18 | 2747519 | 2365142 | n/a | 0 | n/a | -1491.9 | -2021.7 | 18 | no |
| Whiting1 | 18 | 2752555 | 2359104 | n/a | 0 | n/a | -1265.5 | -1880.2 | 18 | no |
| Cod1 | 18 | 2760055 | 2345032 | n/a | 0 | n/a | -2096.8 | -2698.3 | 18 | no |
| Snapper3 | 18 | 2761498 | 2361584 | n/a | 0 | n/a | -1281.6 | -1813.1 | 18 | no |
| Snapper5 | 18 | 2761883 | 2360486 | n/a | 0 | n/a | -1279.3 | -1830.8 | 18 | no |
| Veilfin1 | 18 | 2762314 | 2338728 | n/a | 0 | n/a | -1939.3 | -2519.7 | 18 | no |
| Moonfish1 | 18 | 2763787 | 2368350 | n/a | 0 | n/a | -1379.7 | -1722.7 | 18 | no |
| Moonfish2 | 18 | 2764969 | 2368408 | n/a | 0 | n/a | -1332.3 | -1683.9 | 18 | no |
| West Kingfish | 18 | 2770563 | 2318779 | n/a | 0 | n/a | -2247.7 | -2999.5 | 18 | no |
| Turrum6 | 18 | 2777935 | 2358316 | n/a | 0 | n/a | -1492.2 | -1967.5 | 18 | no |
| Remora1 | 18 | 2779805 | 2367419 | n/a | 0 | n/a | -2187.9 | -2447.5 | 18 | no |
| Kingfish1 | 18 | 2779609 | 2318261 | n/a | 0 | n/a | -2297.3 | -3094.4 | 18 | no |
| Sunfish1 | 18 | 2782979 | 2368800 | n/a | 0 | n/a | -1752.3 | -1997.1 | 18 | no |
| Turrum1 | 18 | 2784149 | 2361923 | n/a | 0 | n/a | -2069.4 | -2462 | 18 | no |
| Turrum2 | 18 | 2784501 | 2357221 | n/a | 0 | n/a | -1538 | -2039.1 | 18 | no |
| Turrum3 | 18 | 2784501 | 2355320 | n/a | 0 | n/a | -1579.7 | -2174 | 18 | no |
| Marlin4 | 18 | 2786130 | 2357629 | n/a | 0 | n/a | -1838.3 | -2359.6 | 18 | no |
| Trumpeter1 | 18 | 2792616 | 2338145 | n/a | 0 | n/a | -2477.7 | -3188.3 | 18 | no |

- Appendix D: Modelled layer thicknesses

- Appendix E: Model boundary conditions

formation

Jemmys

Sands (VAF 109)

Volcanics

Strzelecki Group  0 – 500 m

Strzelecki Group  500 – 1000 m

Strzelecki Group  1000 – 2000 m

Strzelecki Group  2000 – 3000 m

Strzelecki Group  3000 – 4000 m

Strzelecki Group  4000 – 6000 m

Strzelecki Group  >6000 m

- Appendix F: Groundwater extraction reporting regions

- Appendix G: Hydrogeological units

HGU 1 – Represents marine waters throughout the offshore portion of the modelled area.

The unit top is mean sea level and the unit bottom is the sea floor. Marine waters extend beyond the modelled area. Marine Water is clipped to the model extent in the south and east and the coastline in the northwest. Bathymetry is from Whiteway (2009).

The thickness of Marine Water ranges from 2 m at the coastline to 455 m in the southeast corner.

HGU 2 – Represents the Quaternary Alluvium and includes Fluvial, lacustrine, alluvial and colluvial sediments. Quaternary Alluvium overlies the Haunted Hill Formation in the valleys and the Strzelecki Group and Palaeozoic Basement in the highlands.

The QA HGU is based on the aquifer QA from (GHD 2012; SKM 2009). QA extends to the Gippsland coast.

The thickness of Quaternary Alluvium ranges from 2 m to 105 m in small isolated pockets.

HGU 3 – Represents the Upper Tertiary Quaternary Alluvium and includes the Haunted Hill Formation.

The UTQA HGU is based on the aquifer UTQA from (GHD 2012; SKM 2009). UTQA was extended offshore by re-gridding using bathymetry and a 1 km buffer zone. UTQA was modified to account for the offshore pinching out of the unit Haunted Hill Formation. The unit extends to the 10m bathymetric contour.

The thickness of UTQA ranges from 2 m to 175 m near Lake Wellington.

HGU 4 - Represents the Upper Tertiary Quaternary Aquitard which includes the Haunted Hills Gravel, Eagle Point Sand and Boisdale Formation (Nuntin Clay Member).

The UTQD HGU is based on the aquitard UTQD from (GHD 2012; SKM 2009). UTQD was extended offshore by re-gridding with bathymetry and using a 15 km buffer zone. UTQD was modified to account for the offshore pinching out of the unit. The surface extends to the 40m bathymetric contour.

The thickness of UTQD ranges from 2 m to 157 m west of Sale.

HGU 5 - Represents the Upper Tertiary Aquifer which includes the Boisdale (Wurruk Sand Member).

The UTAF HGU is based on the aquifer UTAF from (GHD 2012; SKM 2009). UTAF was extended offshore by re-gridding with Bathymetry and using a 20 km buffer zone. UTAF was modified to account for the offshore pinching out of the Wurruk Sand Member. The surface extends to the 45m bathymetric contour.

The thickness of UTAF ranges from 2 m to 141 m beneath Lake Victoria.

HGU 6 - Represents the Upper Tertiary Aquitard and includes the Hazelwood Formation, Yallourn Formation and Jemmys Point Formation.

The UTD HGU is based on the aquitard UTD from (GHD 2012; SKM 2009). UTD was extended offshore by re-gridding with Bathymetry and using a 40 km buffer zone. UTD was modified to account for the offshore pinching out of the Jemmys Point Formation.  The surface extends to the 55 m bathymetric contour.

The thickness of UTD ranges from 2 m to 200 m southwest of Rosedale.

HGU 7 – Represents the Yallourn Coal seams (Y Coal) and includes the Yallourn Y1A, Y1B and Y2 coal seams.

Coal seam isopachs and extents were determined from the Latrobe Valley coal model Yallourn Coal seams (Jansen & Maher 2003; GHD 2011b; Osbourne et al., 2014). These isopachs have been incorporated into UTD (6).

The thickness of Y Coal ranges from 2m to 228m north of Rosedale.

HGU 8 – Represents the Yallourn interseam. Y Interseam represents the sediments below the Yallourn Coal floor and above the Morwell Coal roof.

This interseam was incorporated into UTD (6).

The thickness of Y Interseam ranges from 2m to 160m beneath Hazelwood.

HGU 9 – Represents the Upper Mid-Tertiary Aquifer and Aquitard. This includes the Morwell, Balook and Tambo River Formations and the Gippsland Limestone.

The UMT HGU is based on the aquifer UMTA and aquitard UMTD from (GHD 2012; SKM 2009). To merge UMTA and UMTD the boundary was re-gridded to smooth conflicts. This occurred through the area of overlap between UMTA and UMTD. UMT was extended offshore by re-gridding with bathymetry and using a 40km buffer zone. The buffer was designed to account for the offshore pinching out of the Jemmys Point Formation. UMT was modified to account for the significant volume of the Gippsland Limestone. Proximal to corner inlet onshore, the UMT was re-gridded to align with petroleum wells and remove conflicts with Strzelecki. The re-gridding occurred over an area approximately 20kmx20km. The UMT extends beyond the study area where it has been clipped. The thickness of UMT ranges from 2m to 428m offshore central deep.

HGU 10 - Represents the Morwell 1A Coal Seam (M1A).

Coal seam isopachs and extents were determined from the Latrobe Valley coal model M1A seams (Jansen & Maher 2003; GHD 2011b; Osbourne et al., 2014). M1A was incorporated into UMT (9).

The M1A Coal thickness ranges from 2m to 228m east of Traralgon.

HGU 11 - Represents the interseam between the Morwell 1A Coal floor and the Morwell 1B Coal roof.

The M1A interseam was incorporated into UMT (9).

The thickness of the M1A Interseam ranges from 2m to 252m south of Rosedale.

HGU 12 - Represents the Morwell 1B (M1B) coal seams.

Coal seam isopachs and extents were determined from the Latrobe Valley coal model M1B seams (Jansen & Maher 2003; GHD 2011b; Osbourne et al., 2014). M1B was incorporated into UMT 9.

The M1B Coal thickness ranges from 2 m to 252 m proximal to Morwell.

HGU 13 - Represents the Morwell 1B interseam between the Morwell 1B coal floor and Morwell 2 coal roof.

M1a Interseam was incorporated within UMT 9.

The thickness of the M1B Interseam ranges from 2m to 252m proximal to Rosedale.

HGU 14 - Represents the Morwell 2 Coal Seam (M2).

Coal seam isopachs and extents were determined from M2 coals, (Jansen & Maher 2003; GHD 2011b; Osbourne et al., 2014). M2 was incorporated into UMT (9).

The M2 Coal thickness ranges from 2m to 239m east of Rosedale.

HGU 15 - Represents the Lakes Entrance Formation. The Lakes Entrance Formation thickness ranges from 2m to 605m offshore in the central deep.

The LEF HGU is based on the top Lakes Entrance Formation from McLean & Blackburn (2013).  LEF was extended onshore by re-gridding with petroleum well control. The surface extends to the terminal effective seal proposed by Blevin et al. (2013) although it is acknowledged that sealing characteristics will extend onshore beyond this line.

The Lakes Entrance Formation thickness ranges from 2m to 605m offshore in the central deep.

HGU 16 - Represents the Lower Mid Tertiary Aquifer and contains the Morwell 2C aquifer and Seaspray Sands.

The LMTA HGU is based on the aquifer LMTA from (GHD 2012; SKM 2009). LMTA isopachs and extent were determined and were then incorporated above HGU 18. No offshore extension was modelled for this aquifer.

The LMTA thickness ranges from 2m to 123m proximal to Longford.

HGU 17 – Represents the Lower Tertiary Basalts.

The LTB HGU is based on the aquifer LTB from (GHD 2012; SKM 2009). LTB was not extended offshore.

The Lower Tertiary Basalts range in thickness ranges from 2m to 163m proximal to Yarragon.

HGU 18 - Represents the Lower Tertiary Aquifer and contains the Traralgon coals, Cobia Subgroup, Halibut Subgroup, Golden Beach Subgroup, and Emperor Subgroup.

The LTA HGU is based on the aquifer LTA from (GHD 2012; SKM 2009). LTA was extended offshore by re-gridding with top La Trobe (McLean & Blackman 2013).  Well intersections used to better constrain top LTA were applicable. This has particularly the case onshore proximal to 90 mile beach. The unit extends to the model extent in the east. In the southeast the LTA extent is based upon the interpretations from Blevin et al. (2013).

The Latrobe Aquifer ranges in thickness from 2m to 1192m offshore in the central deep.

HGU 19 - Represents the Traralgon T0 and T1 coal seams and T0 inter-burden (T1 Coal).

Coal isopachs and extents were determined for TP, TRU, TRM and TRL from the Latrobe Valley Coal Model (Jansen & Maher 2003; GHD 2011b; Osbourne et al., 2014). Additional Coal isopachs were determined for T0 and T1 from Holdgate et al. (2000). Coal model isopachs were aggregated and extended offshore by merging with T0/T1 isopachs. Depth control was derived the coal model and petroleum wells where available. T1 Coal was incorporated into LTA 18.

The T1 Coal ranges in thickness from 2 m to 133 m near Yarram and Longford.

HGU 20 - Represents the Traralgon T1 aquifer and includes the T1 interseam.

T1 Interseam isopachs and extents were determined by subtracting T2 Coal roof from T1 Coal floor. Petroleum wells were used for depth control where available. T1 Interseam was incorporated into LTA 18.

The T1 Interseam ranges in thickness from 2 m to 562 m offshore from Lake Wellington.

HGU 21 – Represents the Traralgon T2 coal seams and minor interseams (T2 Coal).

Coal isopachs and extents were determined for T2A, T2B from the Latrobe Valley Coal Model (Jansen & Maher 2003; GHD 2011b; Osbourne et al., 2014). Additional Coal isopachs for the T2 coals were taken from Holdgate et al, (2000). Coal model isopachs were aggregated and extended offshore by merging with T2 isopachs. Depth control was derived from the coal model and petroleum wells where available. T2 Coal was incorporated into LTA 18.

The T2 coals range in thickness from 2 m to 448 m with the thicker coals tracing the Baragwaneth Anticline and thinner coals north of the Rosedale Fault.

HGU 22 - Represents the Traralgon T2 inter-burden (aquifer) below the T2 coal seams.

T2 Interseam isopachs and extents were determined by subtracting LTA top from T2 Coal floor. Petroleum wells were used for depth control where available. T2 Interseam was incorporated into LTA 18.

The T2 Interseam ranges in thickness from 2 m to 2645 m offshore where top Strzelecki Group is deeper.

HGU 23 – Represents the upper 500 m of the Strzelecki Group and includes the Wombat gas fields near Seaspray and the historical Black Coal mines at Wonthaggi.

Areas where the Palaeozoic Basement outcrops were clipped from BSE, GHD (2012), to produce the Strzelecki HGU onshore. This surface was then blended with the offshore top Strzelecki (McLean and Blackburn, 2013) using a 10km feathering distance. The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped.

Strzelecki ranges in thickness from 2 m to 624 m.

HGU 24 – Represents from 500 m to 1000 m depth within the Strzelecki Group.

The Strzelecki Group was arbitrarily subdivided by subtracting a 500 m thick slab from Strzelecki (23). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped.

Strzelecki1 ranges in thickness from 2 m to 545 m.

HGU 25 – Represents from 1000 m to 2000 m depth within the Strzelecki Group.

The Strzelecki Group was arbitrarily subdivided by subtracting a 1000m thick slab from Strzelecki1 (23). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped.

Strzelecki2 ranges in thickness from 2 m to 1000 m.

HGU 26 – Represents from 2000 m to 3000 m depth within the Strzelecki Group.

The Strzelecki Group was arbitrarily subdivided by subtracting a 1000m thick slab from Strzelecki2 (25). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped.

Strzelecki3 ranges in thickness from 2 m to 1000 m.

HGU 27 – Represents from 3000 m to 400 m depth within the Strzelecki Group.

The Strzelecki Group was arbitrarily subdivided by subtracting a 500m thick slab from Strzelecki3 (26). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped.

Strez4 ranges in thickness from 2 m to 1000 m.

HGU 28 – Represents from 4000 m to 7000 m depth within the Strzelecki Group.

The Strzelecki Group was arbitrarily subdivided by subtracting a 3000 m thick slab from Strzelecki4 (27). The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped.

Strzelecki5 ranges in thickness from 2 m to 3134 m.

HGU 29 – Represents from 7000 m to Palaeozoic Basement depth within the Strzelecki Group.

The Strzelecki Group was arbitrarily subdivided and Strzelecki6 represents the remaining Strzelecki Group not already incorporated into overlying layers. The unit extends to the intersection with Palaeozoic Basement (30) at which point it has been clipped.

Strzelecki6 ranges in thickness from 266 m to 4000 m.

HGU 30 – Represents the Palaeozoic Basement.

Internal GSV grids were used as the foundation for the Palaeozoic (30) HGU. The Palaeozoic surface was then modified in the following ways: BSE (GHD 2012; SKM 2009) was clipped to the extent of Palaeozoic outcrop and merged with Palaeozoic basement; Basement intersections from Constantine (2009) were used to constrain Palaeozoic basement depth; Strzelecki intersections from (Constantine 2009) were used to limit the upward extent; Interpretations based on isostatic gravity anomalies were used to mould the geometry of troughs; offshore basement surfaces from McLean & Blackman (2013) and Stuart-Smith et al. (2010) were merged together.

The Palaeozoic HGU ranges in thickness from 2 m to 262 m.

| Model layer | Name | Inputs | Geology - Comment | References |
| --- | --- | --- | --- | --- |
| 1 | Marine Water | Bathymetry | Marine water thickness | Whiteway (2009) |
| 2 | QA | VAF 101 | Quaternary | GHD (2012) SKM (2009) |
| 3 | UTQA | VAF 102; -10m elevation contour | Haunted Hill Formation | GHD (2012) SKM (2009) |
| 4 | UTQD | VAF 103 | Nuntin clay | GHD (2012) SKM (2009) |
| 5 | UTAF | VAF 105 | Boisdale Formation | GHD (2012) SKM (2009) |
| 6 | UTD | VAF 106 | Jemmys Point Formation and upper Hazelwood Formation | GHD (2012) SKM (2009) |
| 7 | Y COAL | VAF 106 Yallourn Coal (isopach) | Yallourn coal seams; 1,1a, 1b and 2 | GHD (2012) SKM (2009) GHD (2011) |
| 8 | Y Interseam | VAF 106 Yallourn Coal Interseam | y_all floor & M1a_all top | GHD (2012) SKM (2009) GHD (2011) |
| 9 | UMT A&D | UMTA 107 UMTD 108 | Balook Formation Tambo River, Wuk Wuk Marl, Gippsland Limestone, Morwell Coals | GHD (2012) SKM (2009) |
| 10 | M1A COAL | UMTA 107 Morwell 1A coal (isopach) | Yarragon Formation, M10, M1a, M1b2, ML, M12; M1a_all | GHD (2012) SKM (2009) GHD (2011) |
| 11 | M1A Interseam | UMTA 107 Morwell 1A Interseam | M1a_all_floor & M1b_top | GHD (2012) SKM (2009) GHD (2011) |
| 12 | M1B COAL | UMTA 107 Morwell 1B coal (isopach) | M1b, M1b1, M1b2, ML, M12 | GHD (2012) SKM (2009) GHD (2011) |
| 13 | M1B Interseam | UMTA 107 Morwell 1B Interseam | Floor M1b_all & M2_all top | GHD (2012) SKM (2009) GHD (2011) |
| 14 | M2 COAL | UMTA 107 Morwell 2 coal (isopach) | M2, M2A, M2B coal; M2_all | GHD (2012) SKM (2009) GHD (2011) |
| 15 | LEF | LEF Top; LEF Terminal Seal; LEF well picks | Lake Entrance Formation | McLean & Blackburn. (2013); (Blevin et al, 2013); Goldie Divko pers. comm. (2014) |
| 16 | LMTA | LMTA 109 Morwell 2 interseam, | M2c aquifer/Seaspray sands | GHD (2012) SKM (2009) |
| 17 | LTB | LTB 112 | Thorpdale Volcanics | GHD (2012) SKM (2009) |
| 18 | LTA | LTA 111 Top La Trobe Petroleum wells | Upper Latrobe Group | GHD (2012) SKM (2009) McLean & Blackburn (2013) |
| 19 | T1 COAL | LTA 111 T0 & T1 Coal Petroleum wells | TP, T1, TRU, TRM, TRL | GHD (2012) SKM (2009) GHD (2011) |
| 20 | T1 Interseam | LTA 111 T1-T2 interseam Petroleum wells | Floor T1_all & Top T2_all | GHD (2012) SKM (2009) GHD (2011) |
| 21 | T2 COAL | LTA 111 T2 coal Petroleum wells | T2 coal seams | GHD (2012) SKM (2009) GHD (2011) Osbourne et al. (2014) |
| 22 | T2 Interseam | LTA 111 T2 interseam Petroleum wells | Lower Latrobe Group; T2_all floor & | GHD (2012) SKM (2009) GHD (2011) Osbourne et al. (2014) |
| 23 | STRZ | BSE 114 | Strzelecki 500 m; >0–500 m | GHD (2012) SKM (2009) McLean & Blackburn (2013) |
| 24 | STRZ1 | BSE 114 | Strzelecki 500 m; >500–1000 m | GHD (2012) SKM (2009) McLean & Blackburn (2013) |
| 25 | STRZ2 | BSE 114 | Strzelecki 1 km; >1000–2000 m | GHD (2012) SKM (2009) McLean & Blackburn (2013) |
| 26 | STRZ3 | BSE 114 | Strzelecki 2 km; >2000–3000 m | GHD (2012) SKM (2009) McLean & Blackburn (2013) |
| 27 | STRZ4 | BSE 114 | Strzelecki 2 km; >3000–4000 m | GHD (2012) SKM (2009) McLean & Blackburn (2013) |
| 28 | STRZ5 | BSE 114 | Strzelecki 3 km; >3000–7000 m | GHD (2012) SKM (2009) McLean & Blackburn (2013) |
| 29 | STRZ6 | BSE 114 | Strzelecki; >7000 m | GHD (2012) SKM (2009) McLean & Blackburn (2013) |
| 30 | PALEO | BSE 114 | Palaeozoic basement 200 m thick | GHD (2012) SKM (2009) McLean & Blackburn (2013) Stuart-Smith et al. (2010) Constantine (2009) |

- Appendix H: Time-series simulated versus observed 
water level data

© The State of Victoria Department of Environment, Land, Water & Planning 2015

This work is licensed under a Creative Commons Attribution 4.0 International licence. You are free to re-use the work under that licence, on the condition that you credit the State of Victoria as author. The licence does not apply to any images, photographs or branding, including the Victorian Coat of Arms, the Victorian Government logo and the Department of Environment, Land, Water and Planning (DELWP) logo. To view a copy of this licence, visit http://creativecommons.org/licenses/by/4.0/

ISBN 978-1-74146-620-1 (pdf)

Accessibility

If you would like to receive this publication in an alternative format, please telephone the DELWP Customer Service  Centre on 136186, email customer.service@delwp.vic.gov.au or via the National Relay Service on 133 677  www.relayservice.com.au. This document is also available on the internet at www.onshoregas.vic.gov.au

Disclaimer

This publication may be of assistance to you but the State of Victoria and its employees do not guarantee that the publication is without flaw of any kind or is wholly appropriate for your particular purposes and therefore disclaims all liability for any error, loss or other consequence which may arise from you relying on any information in this publication.